3GPP TSG RAN WG1 #107bis-e		                                   					       												                  R1-2200763

e-Meeting, January 17th – 25th, 2022

Source:	Ad-Hoc Chair (AT&T)

Title:	Session Notes of AI 8.15.2

Agenda Item:	8.15.2

Document for: 	Endorsement

#### 8.15.2	UE features for supporting NR from 52.6 GHz to 71 GHz

[107bis-e-R17-UE-features-52-71GHz-01] Email discussion UE features for supporting NR from 52.6 GHz to 71 GHz – Ralf (AT&T)

- 1st check point: January 20

- Final check point: January 25

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 24. NR_ext_to_71GHz | 24-1 | Basic FR2-2 DL support | 1. Support reception of 120kHz subcarrier spacing for DL data and control channels, SSB, and reference signals in FR2-2 for non-initial access |  | Yes | N/A | FR2-2 is not supported | [per UE][per band] | N/A | N/A | N/A |  | Optional with capability signalling A UE that supports FR2-2 must indicate this FG is supported |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 24. NR_ext_to_71GHz | 24-1a | Basic FR2-2 UL support | 1. PRACH with 120KHz SCS and length 139 2. Support transmission of 120kHz subcarrier spacing for UL data and control channels and reference signals in FR2-2 | [24-1] | Yes | N/A | UL in FR2-2 is not supported | [per UE][per band] | N/A | N/A | N/A |  | Optional with capability signalling [A UE that supports FR2-2 must indicate this FG is supported] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 24. NR_ext_to_71GHz | 24-1c | Multi-RB support PUCCH format 0/1/4 for 120 kHz in FR2-2 [with/without shared spectrum channel access] | 1. Support multi-RB PUCCH format 4 for 120 kHz 2. Support multi-RB PUCCH format 0/1 for 120 kHz | [24-1a] | Yes | N/A | Multi-RB support PUCCH format 0/1/4 for 120 kHz in FR2-2 is not supported | Per band | N/A | N/A | N/A |  | Optional with capability signalling [A UE that supports [24-1a/24-2/FR2-2] must indicate this FG is supported] This FG is only supported in bands under PSD limitation in shared spectrum operation |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 24. NR_ext_to_71GHz | 24-4c | Multi-RB PUCCH format 0/1/4 for 480 kHz in FR2-2 [with/without shared spectrum channel access] | Support multi-RB PUCCH format 0/1/4 for 480 kHz | 24-4a | Yes | N/A | Multi-RB PUCCH format 0/1/4 for 480 kHz in FR2-2 is not supported | Per band | N/A | N/A | N/A |  | Optional with capability signalling This FG is only supported in bands under PSD limitation in shared spectrum operation |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 24. NR_ext_to_71GHz | 24-5c | Multi-RB PUCCH format 0/1/4 for 960 kHz in FR2-2 [with/without shared spectrum channel access] | Support multi-RB PUCCH format 0/1/4 for 960 kHz | 24-5a | Yes | N/A | Multi-RB PUCCH format 0/1/4 for 960 kHz in FR2-2 is not supported | Per band | N/A | N/A | N/A |  | Optional with capability signalling This FG is only supported in bands under PSD limitation in shared spectrum operation |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 24. NR_ext_to_71GHz | 24-1b | Wideband PRACH for 120 kHz in FR2-2 [with/without shared spectrum channel access] | Enhanced PRACH design for operation by adopting a single long ZC sequence, with ZC sequence equal to 1151 for 120kHz and ZC sequence equal to 571 for 120kHz | [24-1a] | Yes | N/A | Wideband PRACH for 120 kHz in FR2-2 is not supported | Per band | N/A | N/A | N/A | FFS: whether to split this FG for SA and DC | Optional [with/without]capability signalling [Note: This FG is only supported in bands for shared spectrum operation] [A UE that supports 24-2 must indicate this FG is supported] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 24. NR_ext_to_71GHz | 24-4b | Wideband PRACH for 480 kHz in FR2-2 [with/without shared spectrum channel access] | PRACH with 480KHz and length 571 | 24-4a | Yes | N/A | Wideband PRACH for 480 kHz in FR2-2 is not supported | Per band | N/A | N/A | N/A | FFS: whether to split this FG for SA and DC [Agreement: Do not support PRACH length L=571, 1151 for 960kHz PRACH and at least L =1151 for 480kHz PRACH] | Optional with capability signalling [Note: This FG is only supported in bands for shared spectrum operation] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 24. NR_ext_to_71GHz | 24-4 | 480KHz SCS support for DL | 1. 480KH SCS for DL data and control channels, SSB, and reference signal reception in FR2-2 for non-initial access 2. Multiple-slot PDCCH monitoring for 480KHz with X=4 slots (Xs,Ys) = (4,1) FFS: 3. Multi- PDSCH scheduling by single DCI for the operation with 480 kHz SCS and corresponding HARQ enhancements 4. Within the Ys = 1 slot, monitoring of type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS with a maximum of two monitoring spans per slot according to FG 3-5b with set2 = (4, 3) and (7, 3) symbols where set2 is defined in FG3-5b (FFS: Monitoring capability within slots of type 1 CSS without dedicated RRC configuration and type0, 0A, and 2 CSS) 5. Processing one unicast DCI scheduling DL and one unicast DCI scheduling UL per slot group of Xs slots per scheduled CC for FDD (This supersedes corresponding component of FG 3-5b) 6. Processing one unicast DCI scheduling DL and 2 unicast DCI scheduling UL per slot group of Xs slots per scheduled CC for TDD (This supersedes corresponding component 6 of FG 3-5b) | 24-1 | Yes | N/A | 480KHz SCS for DL is not supported | [Per UE/band] | N/A | N/A | N/A | FFS: component description without a reference to other R15 FGs | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 24. NR_ext_to_71GHz | 24-5 | 960KHz SCS support for DL | 1. 960KHz SCS for DL data and control channels, SSB, and reference signal reception in FR2-2 for non-initial access 2. Multiple-slot PDCCH monitoring for 960KHz with (Xs,Ys)=(8,1) slots FFS: 3. Multi-PDSCH scheduling by single DCI for the operation with 960 kHz SCS and corresponding HARQ enhancements 3. Within the Ys = 1 slot, monitoring of type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS according to FG 3-5b with set1 = (7, 3) symbols where set1 is defined in FG3-5b (FFS: Monitoring capability within slots of type 1 CSS without dedicated RRC configuration and type0, 0A, and 2 CSS) 4. Processing one unicast DCI scheduling DL and one unicast DCI scheduling UL per slot group of Xs slots per scheduled CC for FDD (This supersedes corresponding component of FG 3-5b) 5. Processing one unicast DCI scheduling DL and 2 unicast DCI scheduling UL per slot group of Xs slots per scheduled CC for TDD (This supersedes corresponding component 6 of FG 3-5b) | 24-1 | Yes | N/A | 960KHz SCS support for DL is not supported | [Per UE/band] | N/A | N/A | N/A | FFS: component description without a reference to other R15 FGs | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 24. NR_ext_to_71GHz | 24-4f | Enhanced PDCCH monitoring for 480KHz in FR2-2 | [1.) Multiple-slot PDCCH monitoring for 480KHz with (Xs,Ys)=(2,1)] 1.) Multiple-slot PDCCH monitoring for 480KHz with (Xs,Ys)=[(4,2)] slots 2.) Within each of the Ys = 2 slots, monitoring of type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS in the first 3 OFDM symbols of each slot as in according to FG 3-1 (FFS: Monitoring capability within slots of type 1 CSS without dedicated RRC configuration and type0, 0A, and 2 CSS) | 24-4 | Yes | N/A | Enhanced PDCCH monitoring for 480KHz in FR2-2 is not supported | Per band | N/A | N/A | N/A | Component 1 candidate values: [one or more of] {[(2,1),] (4,2) } Note: If (2,1) is not agreed, this FG will have no component candidate values and the component 1 description will be updated from (Xs,Ys) to (Xs,Ys)=(4,2) similar to FG 24-4 and 24-5 FFS: component description without a reference to other R15 FGs | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 24. NR_ext_to_71GHz | 24-5f | Enhanced PDCCH monitoring for 960KHz | 1.) Multiple-slot PDCCH monitoring for 960KHz with (Xs,Ys)=(4,1) 2.) Multiple-slot PDCCH monitoring for 960KHz with (Xs,Ys)= (4,2) 1.) Multiple-slot PDCCH monitoring for 960KHz with (Xs,Ys)=(8,4) slots 2.) Within each of the Ys = 2 or 4 slots, monitoring of type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS in the first 3 OFDM symbols of each slot as in according to FG 3-1 (FFS: Monitoring capability within slots of type 1 CSS without dedicated RRC configuration and type0, 0A, and 2 CSS) | 24-5 | Yes | N/A | Enhanced PDCCH monitoring for 960KHz is not supported | Per band | N/A | N/A | N/A | Component 1 candidate values: one or more of {(4,1), (4,2), (8,4)} FFS: component description without a reference to other R15 FGs | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

R1-2200050	Rel-17 UE features for extension to 71 GHz	Huawei, HiSilicon

R1-2200099	Discussions on UE features for NR operation from 52.6GHz to 71GHz	vivo

R1-2200217	UE features for supporting NR from 52.6 GHz to 71 GHz	Samsung

R1-2200247	Views on Rel-17 UE features for supporting NR in FR2-2	NTT DOCOMO, INC.

R1-2200266	Discussion on UE features for 52.6 to 71GHz	ZTE, Sanechips

R1-2200312	UE features for NR from 52.6 Ghz to 71 Ghz	Qualcomm Incorporated

Late submission

R1-2200330	Discussion on UE feature for FR2-2	OPPO

R1-2200390	Discussion on UE capability for extending NR up to 71 GHz	Intel Corporation

R1-2200408	UE features for extending current NR operation to 71 GHz	Ericsson

R1-2200431	Views on Rel-17 Beyond 52.6 GHz UE features	Apple

R1-2200543	Views on UE features for supporting NR from 52.6 GHz to 71 GHz	MediaTek Inc.

R1-2200582	Discussion on UE features for NR above 52.6 GHz	LG Electronics

R1-2200623	On UE features for supporting NR from 52.6 GHz to 71 GHz	Nokia, Nokia Shanghai Bell