3GPP TSG RAN WG1 Meeting #108-e			R1-2202344

e-Meeting, February 21st – March 3rd, 2022

Agenda Item:	8.6.1.1

Source: 	LG Electronics

Title: 	Aspects related to the reduced maximum UE bandwidth of RedCap

Document for:	Discussion and decision

- Introduction

As of RAN#90-e meeting, the WI titled “Support of reduced capability NR devices” was approved [1]. The WI objectives are copied below from latest version of the WID [2] for convenience. Related to the reduced UE bandwidth of RedCap, it is noted that as of RAN#91-e meeting, the maximum bandwidth of an FR1 RedCap UE during and after initial access is 20 MHz. That is, no wider bandwidth such as 40MHz is further considered for RedCap UEs.

## 4 Objective ### 4.1 Objective of Core part WI This WI has the following objectives: - Specify support for the following UE complexity reduction features [RAN1, RAN2, RAN4]: - Reduced maximum UE bandwidth: - Maximum bandwidth of an FR1 RedCap UE during and after initial access is 20 MHz. - Maximum bandwidth of an FR2 RedCap UE during and after initial access is 100 MHz - …

- Discussion

In this contribution, we present our views on the remaining open issues related to the reduced maximum UE bandwidth of RedCap.

  - Separate initial DL BWP for RedCap

Whether to transmit NCD-SSB in the separate initial DL BWP configured for RedCap UEs for random access and/or paging has been controversial since RAN1#106-e meeting. It is basically considered as a trade-off b/w SSB overhead and UE power consumption/complexity. The SSB is needed for selection of the PRACH resources and the monitoring occasions for paging. If a RedCap UE has to receive the SSB by RF retuning, it would take additional power consumption and complexity. So UE vendors tend to prefer the solution that a RedCap UE may assume the transmission of SSBs for both random access and paging on the separate initial DL BWP for RedCap. On the other hand, there are some others including NW operators that have shown their concerns on the overhead of the NCD-SSB if provided in the separate initial DL BWP.

After a lengthy discussion, the following agreement was made for FR1 during RAN1#107-e meeting.

Agreement (107-e) - For FR1, - For a separate initial DL BWP (if it does not include CD-SSB and the entire CORESET#0) from RAN1 perspective, - If it is configured for random access while not for paging in idle/inactive mode, RedCap UE does NOT expect it to contain SSB/CORESET#0/SIB. - Note: RAN1 assumes REDCAP UE performing Random access in the separate DL BWP does not need to monitor paging in a BWP containing CORESET#0 - Working assumption: If it is configured for paging, RedCap UE expects it to contain NCD-SSB for serving cell but not CORESET#0/SIB from RAN1 perspective - For an RRC-configured active DL BWP in connected mode (if it does not include CD-SSB and the entire CORESET#0) from RAN1 perspective, - A RedCap UE supporting mandatory FG 6-1 (but not optional FG 6-1a) expects it to contain NCD-SSB for serving cell but not CORESET#0/SIB - A RedCap UE can indicate the following as optional capability: - Not need NCD-SSB: A RedCap UE can in addition optionally support relevant operation based on for CSI-RS (working assumption) and/or FG 6-1a by reporting optional capabilities. - Note: if a separate initial/RRC configured DL BWP is configured to contain the entire CORESET#0, CD-SSB is expected by RedCap UE. - Note: The network may choose to configure SSB or MIB-configured CORESET#0 or SIB1 to be within the respective DL BWP. - Note: If a separate SIB-configured initial DL BWP for RedCap UEs contains the entire CORESET#0, the RedCap UE shall use the bandwidth and location of the CORESET#0 in DL during initial access. - Note: NCD-SSB periodicity is not required to be configured the same as that of CD-SSB - Note: Periodicity of NCD-SSB shall be not less than periodicity of CD-SSB

Regarding the separate initial DL BWP for RedCap, one of the remaining open issues captured in the SR is whether to support the combination of the separate initial DL BWP and the BWP#0 configuration option 1 for RedCap in connected mode, and if supported then whether further clarification is needed or not.

According to the agreement above, for an RRC-configured active DL BWP in connected mode (if it does not include CD-SSB and the entire CORESET#0), a RedCap UE supporting mandatory FG 6-1 (but not optional FG 6-1a) expects it to contain NCD-SSB for serving cell but not CORESET#0/SIB.

For BWP#0 configuration option 1, the BWP#0 is not considered to be an RRC-configured BWP, so the agreement above does not guarantee the transmission of the NCD-SSB in the BWP#0 per se. On the other hand, for BWP#0 configuration option 2, as the BWP#0 is considered to be an RRC-configured BWP, the NCD-SSB is transmitted in the BWP#0 according to the agreement.

A RedCap UE can also optionally support the case where the NCD-SSB transmission not needed for RedCap UEs in the RRC-configured active DL BWP in connected mode by reporting optional capabilities of relevant operation based on CSI-RS (working assumption) and/or FG 6-1a. In this case, for both BWP#0 configuration option 1 and option 2, a RedCap UE may NOT expect NCD-SSB in the separate initial DL BWP.

With the clarifications above, we think we could support the combination of the separate initial DL BWP and the BWP#0 configuration option 1. Or, if we could not make any agreement on not supporting the combination, then our understanding is that the combination is allowed. So, our preference is either to not further discuss this issue, or to make a conclusion out of the following proposal.

Proposal 1: Allow the combination of BWP#0 configuration option 1 and separate initial DL BWP for RedCap.

Regarding the separate initial DL BWP for RedCap UEs, a working assumption was made in RAN1#106b-e meeting.

Working Assumption: (106b-e) - For a cell that allows a RedCap UE to access, network can configure a separate initial DL BWP for RedCap UEs in SIB. - Working assumption: It can be used during initial access - It can be used after initial access. - It is no wider than the maximum RedCap UE bandwidth. - FFS: It is always configured if the initial DL BWP for non-RedCap UEs is wider than the maximum RedCap UE bandwidth. - This applies to both TDD and FDD (including FD FDD and HD FDD) cases. - Working assumption: It applies at least after initial access for FR1 when MIB configured CORESET#0 is included

After another round of the discussion on the separate initial DL BWP, following agreement was made in RAN1#107-e meeting.

Agreement (107-e) - For both FR1 and FR2, for a cell that allows a RedCap UE to access, network can configure a separate initial DL BWP for RedCap UEs in SIB. At least the case when the separate initial DL BWP includes CD-SSB and the entire CORESET#0 is supported - It can be used in idle/inactive mode (including paging) and during and after initial access, when applicable - It is no wider than the maximum RedCap UE bandwidth. - This applies to both TDD and FDD (including FD FDD and HD FDD) cases.

According to the agreement, it is not an invalid configuration that an initial DL BWP for non-RedCap UEs is larger than max RedCap UE bandwidth but a separate initial DL BWP is not configured for RedCap UEs. Clarification of UE behaviour in this case is captured in the SR as one of the remaining open issues of RedCap WI. In our view, if the separate initial DL BWP for RedCap UEs is not configured when an initial DL BWP for non-RedCap UEs is larger than max RedCap UE bandwidth, a RedCap UE may assume the MIB-configured CORESET#0 bandwidth as the initial DL BWP. In this case, same as for the non-RedCap UEs, the center frequencies b/w the separate initial UL BWP and the MIB-configured CORESET#0 may or may not be aligned.

Alternatively, RedCap UE may assume as the initial DL BWP the SIB-configured initial DL BWP for non-RedCap UEs if configured. In this case, the center frequencies b/w the separate initial UL BWP and the SIB-configured initial DL BWP for non-RedCap UEs may or may not be aligned.

To minimize frequency retuning for RedCap UEs in the case above, applying some restrictions on the possible configurations of the separate initial UL BWP for RedCap to align the center frequencies between the separate initial UL BWP and the MIB-configured CORESET#0. However, we tend to think this is an issue that is neither critical nor essential. Therefore, this discussion should only be pursued only if time allows, and if there is no clear consensus, then there is no restriction on the center frequencies of the initial UL BWP and the MIB-configured CORESET#0 as long as the bandwidth spanned by the initial UL BWP and the MIB-configured CORESET#0 together does not exceed the max RedCap UE bandwidth.

Proposal 2: If the separate initial DL BWP for RedCap UEs is not configured when the initial DL BWP for non-RedCap UEs is larger than max RedCap UE bandwidth, RedCap UEs may assume the MIB-configured CORESET#0 bandwidth as the initial DL BWP

- In this case, same as for the non-RedCap UEs, center frequencies b/w the separate initial UL BWP and the MIB-configured CORESET#0 may or may not be aligned.

Proposal 3: There is no restriction on the center frequencies of the initial UL BWP and the MIB-configured CORESET#0 as long as the bandwidth spanned by the initial UL BWP and the MIB-configured CORESET#0 together does not exceed the max RedCap UE bandwidth

In idle/inactive mode, the separate initial DL BWP for RedCap can be used for random access and paging for RedCap UEs. The SI messages for RedCap can also be transmitted on the separate initial DL BWP of RedCap UEs.

For random access in idle/inactive mode, CORESET/CSS for random access can be configured in the separate initial DL BWP for RedCap UEs. If it is not configured, then a RedCap UE may need to switch the separate initial DL BWP for RedCap to the initial DL BWP for non-RedCap, and use the MIB-configured CORESET#0 and SS MOs in that initial DL BWP for non-RedCap during the random access. Similarly, for paging in idle/inactive mode, CORESET/CSS for paging can be configured in the separate initial DL BWP for RedCap UEs. If it is not configured, then a RedCap UE may need to switch the separate initial DL BWP for RedCap to the initial DL BWP for non-RedCap, and use the MIB-configured CORESET#0 and SS MOs in that initial DL BWP for non-RedCap for paging reception. The CORESET/CSS for random access or paging can be configured in the PDCCH-ConfigCommon IE in BWP-DownlinkCommon (or in BWP-DownlinkCommon-R).

Proposal 4: If CORESET/CSS for random access is configured in the separate initial DL BWP for RedCap UEs, RedCap UEs use the CORESET/CSS for random access configured in the separate initial DL BWP for RedCap UEs.

  - If it is not configured, RedCap UEs use the MIB-configured CORESET#0 and SS MO for random access.

Proposal 5: If CORESET/CSS for paging is configured in the separate initial DL BWP for RedCap UEs, RedCap UEs use the CORESET/CSS for paging configured in the separate initial DL BWP for RedCap UEs.

  - If it is not configured, RedCap UEs use the MIB-configured CORESET#0 and SS MO for paging

The separate initial DL BWP for RedCap doesn’t always have to contain the MIB-configured CORESET#0 or SIB1. Same for the RRC-configured DL BWP. However, in both cases, the network can still configure the initial DL BWP to include the MIB-configured CORESET#0 or SIB1. It shall not be mandatory for a RedCap UE to always monitor the CORESET#0 to acquire SI update notification in connected mode if it requires the RF retuning. The SI update notification in connected mode can still be acquired if CORESETs/CSSs for paging and system information acquisition are configured in the separate initial DL BWP for RedCap UEs. And the updated system information can also be provided to a RedCap UE through dedicated signaling using the RRCReconfiguration message as in Rel-15.

Proposal 6: Regarding CORESET#0 and SIB1 in idle/inactive/connected mode for RedCap UEs in FR1,

- If a separate initial DL BWP for RedCap UEs is configured, then the UE shall not expect it to contain MIB-configured CORESET#0 or SIB1.

  - Note: The network may configure MIB-configured CORESET#0 or SIB1 to be within the separate initial DL BWP.

- If an RRC-configured DL BWP is configured, then the UE shall not expect it to contain MIB-configured CORESET#0 or SIB1.

  - Note: The network may configure MIB-configured CORESET#0 or SIB1 to be within the RRC-configured DL BWP.

- In connected mode, a RedCap UE is not required to monitor CORESET#0 for SI update notification.

  - Notes: SI update notification can be supported if CORESETs/CSSs for paging and system information acquisition are configured in the separate initial DL BWP for RedCap UEs. The updated system information can also be provided to a RedCap UE through dedicated signaling using the RRCReconfiguration message as in Rel-15.

  - Separate initial UL BWP for RedCap

For the separate initial UL BWP for RedCap, the following agreement was made in RAN1#106b-e meeting.

Agreement: (106b-e) - For a cell that allows a RedCap UE to access, network can configure a separate initial UL BWP for RedCap UEs in SIB - It can be used both during and after initial access. - It is no wider than the maximum RedCap UE bandwidth. - It is always configured if the initial UL BWP for non-RedCap UEs is wider than the maximum RedCap UE bandwidth - This applies to both TDD and FDD (including FD FDD and HD FDD) cases

So, for the separate initial UL BWP itself, we don’t see much remaining issues other than the configuration details.

It was agreed to align the center frequencies of the initial DL and UL BWPs for RedCap UEs in TDD in FR1 during random access as well as those of the non-initial DL and UL BWPs.

Agreement: (106b-e) For FR1, - For TDD, center frequencies are assumed to be the same for the initial DL (FFS: if it does not include CD-SSB and the entire CORESET#0) and UL BWPs used during random access for RedCap UEs. - FFS: For Option 1 and Option 2, whether the case that the center frequencies are different is also supported, and whether RedCap UE can expect CD-SSB and CORESET#0 in this case - For TDD, center frequencies are assumed to be the same for non-initial DL and UL BWPs with the same BWP id for a RedCap UE.

For the FFS in the main bullet, if the initial DL BWP for RedCap UEs includes CD-SSB and the entire CORESET#0, but the center frequency of it is not aligned with that of the initial UL BWP, then the network should configure a separate initial UL BWP for RedCap UEs for center frequency alignment. If this is mandated for the network, then the FFS is not needed. As all these discussions some depend on the decision on the separate initial DL BWP, we can leave this FFS until we make a decision on the initial DL BWP for RedCap UEs.

Taking into account the payload size limit of SIB1 (The maximum SIB1 or SI message size is 2976 bits), it would be useful to define the UE behavior when the separate initial DL BWP is not configured while the separate initial UL BWP is configured. If the separate initial DL BWP for RedCap UEs is not configured, then the RedCap UEs may assume the MIB-configured CORESET#0 bandwidth as the initial DL BWP. In this case, center frequencies b/w the separate initial UL BWP and the initial DL BWP may not be aligned. To minimize RF retuning for RedCap UEs, applying some restrictions on the possible configuration of the separate initial UL BWP can be considered. That is, a RedCap UE may expect the center frequency of the (separate) initial UL BWP to be aligned with that of the separate initial DL BWP if configured, or the MIB-configured CORESET#0 otherwise.

  - RACH occasions

As one of the aspects related to the reduced maximum UE bandwidth of RedCap, the case where the total frequency span of the FDMed ROs for PRACH transmission does not fit in the maximum RedCap UE bandwidth has been discussed. This case can happen, for example, if the RedCap UEs with 20 MHz maximum UE bandwidth in FR1 is configured with 8-FDMed ROs for the preamble formats with 30 kHz subcarrier spacing or for the preamble format 3 with 5 kHz subcarrier spacing.

Agreement (106-e) Confirm the following working assumption from RAN1#105-e regarding RACH occasions. - For enabling/supporting that the RACH occasion (RO) associated with the best SSB falls within the RedCap UE bandwidth, support separate initial UL BWP for RedCap UEs (which is not expected to exceed the maximum RedCap UE bandwidth), and this separate initial UL BWP for RedCap includes ROs for RedCap UEs. - Note: these ROs can be dedicated for RedCap UEs or shared with non-RedCap UEs.

In the separate initial UL BWP for RedCap UEs, RedCap UEs can be configured with the PRACH configurations (e.g., ROs) either dedicated to RedCap UEs or shared with non-RedCap UEs. And the PRACH configurations in the separate initial UL BWP for RedCap UEs shall guarantee the RACH occasion associated with the best SSB will fall within the maximum RedCap UE bandwidth by gNB configuration.

If ROs are configured for the separate initial UL BWP for RedCap, RedCap UEs perform the random access procedure on the separate initial UL BWP for RedCap. If a separate initial DL BWP is configured with the CORESET/CSS for random access, then the RedCap UEs also need to switch to the separate initial DL BWP for Msg2/4 reception. If ROs are not configured for the separate initial UL BWP for RedCap, the RedCap UEs switch the separate initial UL BWP for RedCap to the initial UL BWP for non-RedCap to perform the random access procedure. If a separate initial DL BWP is configured with the CORESET/CSS for random access, then the UE may also need to switch the separate initial DL BWP for RedCap to the initial DL BWP for non-RedCap e.g., to align the center frequencies b/w the initial UL and DL BWPs.

  - PUSCH/PUCCH transmissions during initial access

As another aspect related to the reduced maximum UE bandwidth of RedCap, a potential issue related to the frequency hopping for Msg3 PUSCH and PUCCH for Msg4 HARQ feedback during initial access procedure has been discussed and the following agreements were made.

Agreement (#106b-e) - In case a separate initial UL BWP is configured for RedCap UEs, it is supported that the network can enable/disable intra-slot PUCCH frequency hopping within the separate initial UL BWP in the PUCCH resource for HARQ feedback for Msg4/MsgB for RedCap UEs. - The frequency hopping is enabled/disabled at least via SIB. Agreement (#106b-e) - FFS: What specification changes (if any) are needed to support that the network can enable/disable intra-slot PUCCH frequency hopping (FH) within the separate initial UL BWP in the PUCCH resource for HARQ feedback for Msg4/MsgB for RedCap - FFS: Whether any specification changes are needed and desired in order to support multiplexing of non-FH and FH PUCCH transmissions in PUCCH resources. Agreement (#107-e) - When the frequency hopping for the RedCap PUCCH resources (for HARQ feedback for Msg4/MsgB) is deactivated, - Each PUCCH resource is mapped to a single PRB. - What side[(s)] of the RedCap UL BWP center frequency to which PUCCH resources are mapped is[/are] configurable by the network, including SIB-configurable [additional] offset (with no more than [4] candidate values) from edge using the existing equations for determining the PRB index of the PUCCH transmission as a starting point. - RedCap and non-RedCap can be configured with the same or different PUCCH resource set indices (see TS 38.213 Table 9.2.1-1).

According to the agreements so far, a separate initial UL BWP can be configured for RedCap and the intra-slot PUCCH frequency hopping (FH) within the separate initial UL BWP in the PUCCH resource for HARQ feedback for Msg4/MsgB for RedCap can be enabled/disabled via SIB1. If the frequency hopping is disabled, then each PUCCH resource is mapped to a single PRB. With the agreements so far and using the existing equations and procedures for determining the PRB index of the PUCCH transmission, at the minimum the following is considered to be the procedure for determining the PRB index of the PUCCH transmission when the FH is disabled.

- A PUCCH resource set index (see TS 38.213 Table 9.2.1-1) and a cell-specific [additional] PRB offset are provided by pucch-ResourceCommonRedCap

  - RedCap and non-RedCap can be configured with the same or different PUCCH resource set indices

- FH for the PUCCH transmission is disabled if disable-FH-PUCCH is provided in PUCCH-ConfigCommonRedCap

- If the FH for PUCCH transmission is disabled (if disable-FH-PUCCH is provided in PUCCH-ConfigCommonRedCap), then PRB index of the PUCCH transmission is determined by the PRB index of the PUCCH transmission in the first frequency hop which is determined as specified in TS 38.213.

Proposal 7: If the intra-slot PUCCH frequency hopping within the separate initial UL BWP in the PUCCH resource for HARQ feedback for Msg4/MsgB for RedCap UEs is disabled, then the PRB index of the PUCCH transmission is determined by the PRB index of the PUCCH transmission in the first frequency hop which is determined as specified in TS 38.213.

With the above procedure, gNB can transmit the PUCCH without FH either at the lower edge of the initial UL BWP by using the PUCCH resource indices rPUCCH from 0 to 7 or at the upper edge using the  rPUCCH from 8 to 15 or at both edges using all the  rPUCCH from 0 to 15.

As an enhancement, but with some spec changes, supporting all the 16 PUCCH resources (same as in non-RedCap UEs) even in the case of the PUCCH transmission only in the single edge of the initial UL BWP can be considered. To avoid resource fragmentation in the initial UL BWP, all the PRB indices for the PUCCH transmission need to be consecutive in either lower or upper edge. To achieve this, one of the two alternatives can be considered.

[Alt.A1]

Reuse the existing equations for determining the PRB index for the PUCCH transmission with NBWPsize replaced by the new parameter NBWP, Vsize defined by the following equation.

NBWP, Vsize= 2×RBBWP, Roffset+8NCS

where RBBWP, Roffset is the PRB offset for RedCap to replace or add to the PRB offset RBBWPoffset in TS 38.213 Table 9.2.1-1, and NCS is the total number of initial cyclic shift indexes in the set of initial cyclic shift indexes.

Alt.A1 would result in the following changes in the spec (highlighted in red). WP,  for RedCap to replace or add to the PRB offset in hlighted in red).ransmission and use the minimum the following should be

If rPUCCH8=0 and a UE is provided a PUCCH resource by pucch-ResourceCommon and is not provided useInterlacePUCCH-PUSCH in BWP-UplinkCommon

  - the UE determines the PRB index of the PUCCH transmission as RBBWP, Roffset+rPUCCHNCS, where NCS is the total number of initial cyclic shift indexes in the set of initial cyclic shift indexes

  - the UE determines the initial cyclic shift index in the set of initial cyclic shift indexes as rPUCCHmodNCS

If rPUCCH8=1 and a UE is provided a PUCCH resource by pucch-ResourceCommon and is not provided useInterlacePUCCH-PUSCH in BWP-UplinkCommon

  - the UE determines the PRB index of the PUCCH transmission as NBWP, Vsize-1-RBBWP, Roffset-rPUCCH-8NCS

  - the UE determines the initial cyclic shift index in the set of initial cyclic shift indexes as

[Alt.A2]

Or alternatively, the PRB index of PUCCH transmission can be incremented with the increasing rPUCCH starting from the RBBWP, Roffset.

Alt.A2 could be implemented in the spec as follows.

If a UE is provided a PUCCH resource by pucch-ResourceCommon and is not provided useInterlacePUCCH-PUSCH in BWP-UplinkCommon

  - the UE determines the PRB index of the PUCCH transmission as RBBWP, Roffset+rPUCCHNCS, where NCS is the total number of initial cyclic shift indexes in the set of initial cyclic shift indexes

  - the UE determines the initial cyclic shift index in the set of initial cyclic shift indexes as rPUCCHmodNCS

Proposal 8: Down-select b/w Alt.A1 and Alt.A2 proposed in Clause 2.4 for determining the PRB index of the PUCCH transmission if enhancement on common PUCCH transmission is needed.

If the separate initial UL BWP for RedCap is configured at the upper edge of the initial UL BWP for non-RedCap, then two alternatives can be considered.

[Alt.B1]

The RBBWP, Roffset can still indicate the PRB offset for RedCap. This would require large values of the RBBWP, Roffset but has the benefit of handling this case by a single RRC parameter, that is without introducing additional RRC parameter to indicate which side of the initial UL BWP the PRBs of the PUCCH transmission are mapped on.

Alt.B1 could be implemented in the spec as follows. (Same as Alt.A2, but with large RBBWP, Roffset)

If a UE is provided a PUCCH resource by pucch-ResourceCommon and is not provided useInterlacePUCCH-PUSCH in BWP-UplinkCommon

  - the UE determines the PRB index of the PUCCH transmission as RBBWP, Roffset+rPUCCHNCS, where NCS is the total number of initial cyclic shift indexes in the set of initial cyclic shift indexes

  - the UE determines the initial cyclic shift index in the set of initial cyclic shift indexes as rPUCCHmodNCS

[Alt.B2]

An alternative to handle this case is to introduce a flag that indicates the edge/side on which the PRB of PUCCH transmission is mapped. If the flag indicates the PUCCH is transmitted on the upper edge of the separate initial UL BWP for RedCap, the RBBWP, Roffset would indicate the PRB offset from the upper edge (highest PRB index) of the separate initial UL BWP for RedCap to the PRB corresponding to the lowest PUCCH resource index (i.e., rPUCCH = 0). This could be implemented in the spec as follows.

If a UE is provided a PUCCH resource by pucch-ResourceCommon and is not provided useInterlacePUCCH-PUSCH in BWP-UplinkCommon

  - the UE determines the PRB index of the PUCCH transmission as RBBWP, Roffset-rPUCCHNCS, where NCS is the total number of initial cyclic shift indexes in the set of initial cyclic shift indexes

  - the UE determines the initial cyclic shift index in the set of initial cyclic shift indexes as rPUCCHmodNCS

Proposal 9: For the case where the separate initial UL BWP for RedCap is configured at the upper edge of the initial UL BWP for non-RedCap, down-select b/w Alt.B1 and Alt.B2 proposed in Clause 2.4 for determining the PRB index of the PUCCH transmission if enhancement on common PUCCH transmission is deemed necessary.

The newly introduced parameters such as [additional] PRB offset for RedCap and a flag to indicate the edge/side on which the PRB of PUCCH transmission is mapped are configured via system information (e.g., SIB1).

Potential remaining issues on the PUCCH transmission during initial access would be whether to support the time-domain OCC for user multiplexing in the separate initial UL BWP for RedCap UEs. Given the limited time for discussion on this relatively new issue, this topic can be left for PUCCH enhancement in general or deferred to a later release. Another issue is to enable the per-hop sequence (group) hopping for PUCCH for interference randomization even when the intra-slot PUCCH frequency hopping is disabled in the separate initial UL BWP for RedCap UEs. We tend to think this is kind of an optimization issue and therefore should be okay to be addressed later if there is a consensus on the enhancement in that direction. The simplest solution would be separately enable/disable via SIB1 the intra-slot frequency hopping and the PUCCH sequence (group) if supported. The same solution can apply to the time-domain OCC if supported.

- Conclusion

In this contribution, we presented our views on the remaining open issues related to the reduced maximum UE bandwidth of RedCap.

Proposal 1: Allow the combination of BWP#0 configuration option 1 and separate initial DL BWP for RedCap.

Proposal 2: If the separate initial DL BWP for RedCap UEs is not configured when the initial DL BWP for non-RedCap UEs is larger than max RedCap UE bandwidth, RedCap UEs may assume the MIB-configured CORESET#0 bandwidth as the initial DL BWP

- In this case, same as for the non-RedCap UEs, center frequencies b/w the separate initial UL BWP and the MIB-configured CORESET#0 may or may not be aligned.

Proposal 3: There is no restriction on the center frequencies of the initial UL BWP and the MIB-configured CORESET#0 as long as the bandwidth spanned by the initial UL BWP and the MIB-configured CORESET#0 together does not exceed the max RedCap UE bandwidth

Proposal 4: If CORESET/CSS for random access is configured in the separate initial DL BWP for RedCap UEs, RedCap UEs use the CORESET/CSS for random access configured in the separate initial DL BWP for RedCap UEs.

  - If it is not configured, RedCap UEs use the MIB-configured CORESET#0 and SS MO for random access.

Proposal 5: If CORESET/CSS for paging is configured in the separate initial DL BWP for RedCap UEs, RedCap UEs use the CORESET/CSS for paging configured in the separate initial DL BWP for RedCap UEs.

  - If it is not configured, RedCap UEs use the MIB-configured CORESET#0 and SS MO for paging

Proposal 6: Regarding CORESET#0 and SIB1 in idle/inactive/connected mode for RedCap UEs in FR1,

- If a separate initial DL BWP for RedCap UEs is configured, then the UE shall not expect it to contain MIB-configured CORESET#0 or SIB1.

  - Note: The network may configure MIB-configured CORESET#0 or SIB1 to be within the separate initial DL BWP.

- If an RRC-configured DL BWP is configured, then the UE shall not expect it to contain MIB-configured CORESET#0 or SIB1.

  - Note: The network may configure MIB-configured CORESET#0 or SIB1 to be within the RRC-configured DL BWP.

- In connected mode, a RedCap UE is not required to monitor CORESET#0 for SI update notification.

  - Notes: SI update notification can be supported if CORESETs/CSSs for paging and system information acquisition are configured in the separate initial DL BWP for RedCap UEs. The updated system information can also be provided to a RedCap UE through dedicated signaling using the RRCReconfiguration message as in Rel-15.

Proposal 7: If the intra-slot PUCCH frequency hopping within the separate initial UL BWP in the PUCCH resource for HARQ feedback for Msg4/MsgB for RedCap UEs is disabled, then the PRB index of the PUCCH transmission is determined by the PRB index of the PUCCH transmission in the first frequency hop which is determined as specified in TS 38.213.

Proposal 8: Down-select b/w Alt.A1 and Alt.A2 proposed in Clause 2.4 for determining the PRB index of the PUCCH transmission if enhancement on common PUCCH transmission is needed.

Proposal 9: For the case where the separate initial UL BWP for RedCap is configured at the upper edge of the initial UL BWP for non-RedCap, down-select b/w Alt.B1 and Alt.B2 proposed in Clause 2.4 for determining the PRB index of the PUCCH transmission if enhancement on common PUCCH transmission is deemed necessary.

- References

RP-202933	New WID on support of reduced capability NR devices, 			Ericsson

RP-210918	Revised WID on support of reduced capability NR devices, 		Ericsson