3GPP TSG RAN WG1 #109-e			R1-2203661

e-Meeting, May 9th – 20th, 2022

Agenda Item:	9.6.1

Source:	China Telecom

Title:	Discussion on potential solutions to further reduce UE complexity

Document for:	Discussion

Introduction

The study item on further NR RedCap UE complexity reduction was approved in [1]. The objective is to further reduce the complexity of RedCap devices, the following should be studied:

- Study further UE complexity reduction techniques based on Rel-17 evaluation methodology in TR 38.875 [RAN1]

  - Consider network impact, coexistence of Rel-17 and Rel-18 RedCap and non-RedCap UEs in a cell, UE impact, specification impact

  - Potential solutions, which may complement each other, for reducing device complexity are focusing on:

    - UE bandwidth reduction to 5MHz in FR1,

      - Possibly in combination with relaxed UE processing timeline for PDSCH and/or PUSCH and/or CSI

    - reduced UE peak data rate in FR1,

      - Possibly including restricted bandwidth for PDSCH and/or PUSCH

      - Possibly in combination with relaxed UE processing timeline for PDSCH and/or PUSCH and/or CSI

- Notes:

  - Rel-15 SSB should be reused and L1 changes minimized.

  - Operation in BWP with/without SSB and without/with RF retuning should be considered.

  - It is not precluded that some solutions for FR1 can be applied to FR2 in WI stage.

  - Aim to define a single Rel-18 RedCap UE type for further UE complexity reduction.

This contribution provides views on potential solutions to further reduce UE complexity.

Reduced UE complexity

Aspects related to reduced maximum UE bandwidth

In Rel-17 NR RedCap work item, several use cases for RedCap UEs were mentioned, such as industrial wireless sensors, video surveillance, wearables, etc. And the justification in [1] mentioned that the enhancements can be considered to improve the support for these use cases and to further expand RedCap into a new range of use cases such as smart grid. In Rel-17, the maximum RedCap UE bandwidth is reduced to 20MHz for FR1 and 100MHz for FR2, aiming to reduce the cost and UE complexity. However, for industrial sensors and environmental monitoring, some user requirements are characterized by a large number of terminals, small single service data and many service times, which are sensitive to cost but do not require a large bandwidth. In these cases, 20MHz for FR1 is still a bit large. Hence, it is necessary to support a smaller bandwidth for low-tier devices between existing LPWA UEs and the capabilities of Rel-17 RedCap UEs, such as 5MHz or 10MHz.

If the maximum RedCap UE bandwidth is agreed to reduce to 5MHz or 10MHz in FR1, some critical issues need to be considered. One issue is that the entire SSB or CORESET#0 may exceed the maximum RedCap UE bandwidth in some NR bands. Hence, the coexistence of Rel-17 and Rel-18 RedCap and non-RedCap UEs should be further studied and identified, which would bring access problems. Then we have the following proposals.

Proposal 1: Support maximum RedCap UE bandwidth reduction to 5MHz or 10MHz in FR1.

Proposal 2: Further study the coexistence of Rel-17 and Rel-18 RedCap and non-RedCap UEs.

Aspects related to UE processing time relaxation

In Rel-17 NR RedCap study item, some evaluation results and analysis on UE processing time relaxation were provided in TR 38.875 [2]. However, such objective was not included in the final Rel-17 RedCap WI. The relaxed UE processing time in terms of N1 and N2 potentially reduces UE complexity by allowing a longer time for the processing of PDCCH and PDSCH and preparing PUSCH and PUCCH. Hence, we prefer to support UE processing time relaxation in terms of N1 and N2 for Rel-18 RedCap UEs. The specific detailed values of N1 and N2 can be further decided with considering the cost and UE complexity.

Proposal 3: Support UE processing time relaxation in terms of N1 and N2 for Rel-18 RedCap UEs.

- FFS: The values of N1 and N2

Aspects related to RedCap UE type

Regarding the user requirements of peak data rate, there is a big difference between Rel-17 and Rel-18 RedCap UEs. For Rel-18 RedCap UEs, the target peak data rate is 10Mbps, while the target peak data rate for Rel-17 RedCap UEs is 2Mbps to 150Mbps. Hence, it is hard for higher layer to guarantee QoS flow and latency within only one UE type. Meanwhile, the reduction of RedCap UE bandwidth is bound to bring some changes on configuration and UE capabilities. Therefore, it is better to define a single Rel-18 RedCap UE type for further UE complexity reduction, which is benefit for resource allocation by network. Hence, we have the following proposal.

Proposal 4: A single Rel-18 RedCap UE type should be supported for further UE complexity reduction.

Conclusion

In this contribution, we provide the following proposals:

Proposal 1: Support maximum RedCap UE bandwidth reduction to 5MHz or 10MHz in FR1.

Proposal 2: Further study the coexistence of Rel-17 and Rel-18 RedCap and non-RedCap UEs.

Proposal 3: Support UE processing time relaxation in terms of N1 and N2 for Rel-18 RedCap UEs.

- FFS: The values of N1 and N2

Proposal 4: A single Rel-18 RedCap UE type should be supported for further UE complexity reduction.

- References

RP-213661, “New SID on Study on further NR RedCap UE complexity reduction”, Ericsson, 3GPP TSG RAN #94e, December 2021.

3GPP TR 38.875 v17.0.0, “Study on support of reduced capability NR devices (Release 17)”, March 2021.