3GPP TSG RAN WG1 #109	R1- 2204587

e-Meeting, 16 – 27 May, 2022

Agenda item:		8.16.2

Source:	Nokia, Nokia Shanghai Bell

Title:	On UE features for supporting NR from 52.6 GHz to 71 GHz

Document for:		Discussion and Decision

# Introduction

In RAN1#108-e the RAN1 UE features have been further developed [1, 2]. In this contribution we present our views on the UE features for supporting NR from 52.6 GHz to 71 GHz.

# Discussion

Detailed comments on UE features are listed below. For reference, the latest feature list from RAN1#108-e is available in the Annex.

- 24-1c/5c/5c, 24-1b/4b:

  - Implement the following agreement has been reached in RAN#95-e [3]:

    - Replace the notes under FGs 24-1c, 24-4c and 24-5c for multi-RB PUCCH, and replace the bracketed notes under FGs 24-1b and 24-4b for wideband PRACH, with “This FG is only supported when PSD limitation applies within FR2-2 based on the regional regulations”

- 24-8/9:

  - RAN1#108-e agreed on a per band FG 26-5 (NTN) on Increasing the number of HARQ processes, with following candidate values for UL and DL, respectively:

    - Candidate component values for (X,Y): {(16,32),(32,16),(32,32)}

  - We do not see any technical reason why such per band indication would be suitable for NTN but unsuitable for FR2-2. In RAN1#108-e some companies argued that FSPC would be needed for the sake of limiting the number of carriers where the feature would be supported. In our view this is an inappropriate usage of FSPC signaling, as a mere limitation on the number of carriers where FGs 24-8/9 could be configured would suffice, while not stealing deployment flexibility from the gNB to choose the carriers where the UEs are operating, and without adding unnecessary overhead and complexity to the capability signalling.

  - We propose Per band indication for these FGs.

Proposal: Consider the observations and modifications proposed above for the next version of the corresponding RAN1 UE features list.

# Conclusion

In this contribution we have presented our views on UE features for supporting NR from 52.6 GHz to 71 GHz, with the following proposal:

Proposal: Consider the observations and modifications proposed above for the next version of the corresponding RAN1 UE features list.

# References

- Chair’s Notes RAN1#108-e, Mar. 2022

- R1-2202929, ”Updated RAN1 UE features list for Rel-17 NR after RAN1 #108-e　including remaining RAN1 issues”, Moderators (AT&T, NTT DOCOMO, INC.), Mar. 2022

- RP-220889, “Moderator's summary for discussion [95e-29-R17-52.6-71GHz-WI],” RAN1 Vice-Chair (Huawei), Mar. 2022

# Annex

(See next page)

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 24. NR_ext_to_71GHz | 24-1 | Basic FR2-2 DL support | 1. Support reception of 120kHz subcarrier spacing for DL data and control channels, SSB, and reference signals in FR2-2 for non-initial access |  | Yes | N/A | FR2-2 is not supported | per band | N/A | N/A | N/A | A UE that supports FR2-2 must indicate this FG is supported | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-1a | Basic FR2-2 UL support | 1. PRACH with 120KHz SCS and length 139 2. Support transmission of 120kHz subcarrier spacing for UL data and control channels and reference signals in FR2-2 | 24-1 | Yes | N/A | UL in FR2-2 is not supported | per band | N/A | N/A | N/A |  | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-1b | Wideband PRACH for 120 kHz in FR2-2 | Enhanced PRACH design for operation by adopting a single long ZC sequence, with ZC sequence equal to 1151 for 120kHz and ZC sequence equal to 571 for 120kHz | 24-1a | Yes | N/A | Wideband PRACH for 120 kHz in FR2-2 is not supported | Per band | N/A | N/A | N/A | [A UE that supports FG 24-2 must indicate this FG is supported] [Note: This FG is only supported in bands for shared spectrum operation] | Optional withcapability signalling |
| 24. NR_ext_to_71GHz | 24-1c | Multi-RB support PUCCH format 0/1/4 for 120 kHz in FR2-2 | 1. Support multi-RB PUCCH format 4 for 120 kHz 2. Support multi-RB PUCCH format 0/1 for 120 kHz | 24-1a | Yes | N/A | Multi-RB support PUCCH format 0/1/4 for 120 kHz in FR2-2 is not supported | Per band | N/A | N/A | N/A | [A UE that supports [24-1a/24-2/FR2-2] must indicate this FG is supported] This FG is only supported in bands under PSD limitation in shared spectrum operation | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-1d | Multiple PDSCH scheduling by single DCI for 120kHz in FR2-2 | 1. Multi-PDSCH scheduling by single DCI for the operation with 120 kHz SCS 2. HARQ enhancements for both type 1 and type 2 HARQ codebook for supporting multi-PDSCH scheduling with singe DCI | 24-1 | Yes | N/A | Multiple PDSCH scheduling by single DCI for 120kHz is not supported in FR2-2 | Per band | N/A | N/A | N/A |  | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-1f | Multiple PDSCH scheduling by single DCI for 120kHz in FR2-1 | 1. Multi-PDSCH scheduling by single DCI for the operation with 120 kHz SCS 2. HARQ enhancements for both type 1 and type 2 HARQ codebook for supporting multi-PDSCH scheduling with singe DCI |  | Yes | N/A | Multiple PDSCH scheduling by single DCI for 120kHz is not supported in FR2-1 | Per band | N/A | N/A | N/A |  | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-1e | Multiple PUSCH scheduling by single DCI for 120kHz in FR2-2 | 1. Multi-PUSCH scheduling by single DCI for the operation with 120 kHz SCS | 24-1a | Yes | N/A | Multiple PUSCH scheduling by single DCI for 120kHz is not supported in FR2-2 | Per band | N/A | N/A | N/A |  | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-1g | Multiple PUSCH scheduling by single DCI for 120kHz in FR2-1 | 1. Multi-PUSCH scheduling by single DCI for the operation with 120 kHz SCS with non-contiguous allocation |  | Yes | N/A | Multiple PUSCH scheduling by single DCI for 120kHz is not supported in FR2-1 with non-contiguous allocation | Per band | N/A | N/A | N/A |  | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-2 | 120KHz SSB support for initial access in FR2-2 | 1. Support 120KHz SSB for initial access in FR2-2 | 24-1, 24-1a | N/A | N/A | 120KHz SSB based initial access in FR2-2 is not supported | per band | N/A | N/A | N/A |  | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-3 | 480KHz SSB support for initial access in FR2-2 | 1. Support 480KHz SSB for initial in FR2-2 | 24-2, 24-4, 24-4a | N/A | N/A | 480KHz SSB for initial access in FR2-2 is not supported | per band | N/A | N/A | N/A |  | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-4 | 480KHz SCS support for DL | 1. 480KHz SCS for DL data and control channels, SSB, and reference signal reception in FR2-2 for non-initial access 2. Multiple-slot PDCCH monitoring for 480KHz with (Xs,Ys) = (4,1) 3. Multi- PDSCH scheduling by single DCI for the operation with 480 kHz SCS and corresponding HARQ enhancements 4. Within the Ys = 1 slot (with Xs=4), monitoring of type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS with a maximum of two monitoring spans per slot with a span duration of Y symbols and a minimum gap of X symbols between the start of two spans, where (X,Y) = (4, 3) and (7, 3) are supported 5. Processing one unicast DCI scheduling DL and one unicast DCI scheduling UL per slot group of Xs slots per scheduled CC for FDD 6. Processing one unicast DCI scheduling DL and 2 unicast DCI scheduling UL per slot group of Xs slots per scheduled CC for TDD [7. For type 1 CSS without dedicated RRC configuration and for type 0, 0A, and 2 CSS, the monitoring occasion can be any OFDM symbol(s) of each slot of the slot group, with the monitoring occasions for any of Type 1- CSS without dedicated RRC configuration, or Types 0, 0A, or 2 CSS configurations within a single span of three consecutive OFDM symbols within each slot of the slot group.] | 24-1 | Yes | N/A | 480KHz SCS for DL is not supported | Perband | N/A | N/A | N/A |  | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-4a | 480KHz SCS support for UL | 1. PRACH with 480KHz and length 139 2. 480KHz SCS for UL data and control channels and reference signal transmission in FR2-2 3. Multi-PUSCH scheduling by single DCI for the operation with 480 kHz SCS | 24-1a, 24-4 | Yes | N/A | 480KHz SCS support for UL is not supported | Per band | N/A | N/A | N/A |  | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-4b | Wideband PRACH for 480 kHz in FR2-2 | PRACH with 480KHz and length 571 | 24-4a | Yes | N/A | Wideband PRACH for 480 kHz in FR2-2 is not supported | Per band | N/A | N/A | N/A | [Note: This FG is only supported in bands for shared spectrum operation] | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-4c | Multi-RB PUCCH format 0/1/4 for 480 kHz in FR2-2 | Support multi-RB PUCCH format 0/1/4 for 480 kHz | 24-4a | Yes | N/A | Multi-RB PUCCH format 0/1/4 for 480 kHz in FR2-2 is not supported | Per band | N/A | N/A | N/A | This FG is only supported in bands under PSD limitation in shared spectrum operation | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-4f | Enhanced PDCCH monitoring for 480KHz in FR2-2 | 1. Multiple-slot PDCCH monitoring for 480KHz with (Xs,Ys)=(4,2) 2.) Within each of the Ys = 2 slots (with Xs=4), monitoring of type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS in the first 3 OFDM symbols of each slot | 24-4 | Yes | N/A | Enhanced PDCCH monitoring for 480KHz in FR2-2 is not supported | Per band | N/A | N/A | N/A |  | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-5 | 960KHz SCS support for DL | 1. 960KHz SCS for DL data and control channels, SSB, and reference signal reception in FR2-2 for non-initial access 2. Multiple-slot PDCCH monitoring for 960KHz with (Xs,Ys)=(8,1) 3. MultiPDSCH scheduling by single DCI for the operation with 960 kHz SCS and corresponding HARQ enhancements 4. Within the Ys = 1 slot (with Xs=8), monitoring of type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS with a span duration of Y symbols and a minimum gap of X symbols between the start of two spans, where (X,Y)= (7, 3) is supported 5. Processing one unicast DCI scheduling DL and one unicast DCI scheduling UL per slot group of Xs slots per scheduled CC for FDD 6. Processing one unicast DCI scheduling DL and 2 unicast DCI scheduling UL per slot group of Xs slots per scheduled CC for TDD [7. For type 1 CSS without dedicated RRC configuration and for type 0, 0A, and 2 CSS, the monitoring occasion can be any OFDM symbol(s) of each slot of the slot group, with the monitoring occasions for any of Type 1- CSS without dedicated RRC configuration, or Types 0, 0A, or 2 CSS configurations within a single span of three consecutive OFDM symbols within each slot of the slot group.] | 24-1 | Yes | N/A | 960KHz SCS support for DL is not supported | Per band | N/A | N/A | N/A |  | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-5a | 960KHz SCS support for UL | 1. PRACH with 960KHz and length 139 2. 960KHz SCS for UL data and control channels and reference signal transmission in FR2-2 3. Multi-PUSCH scheduling by single DCI for the operation with 960 kHz SCS | 24-1a, 24-5 | Yes | N/A | 960KHz SCS support for UL is not supported | Per band | N/A | N/A | N/A |  | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-5c | Multi-RB PUCCH format 0/1/4 for 960 kHz in FR2-2 | Support multi-RB PUCCH format 0/1/4 for 960 kHz | 24-5a | Yes | N/A | Multi-RB PUCCH format 0/1/4 for 960 kHz in FR2-2 is not supported | Per band | N/A | N/A | N/A | This FG is only supported in bands under PSD limitation in shared spectrum operation | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-5f | Enhanced PDCCH monitoring for 960KHz in FR2-2 | 1. Multiple-slot PDCCH monitoring for 960KHz with (Xs,Ys) 2.) Within each of the Ys = 2 (with Xs=4) or Ys = 4 (with Xs=8) slots, monitoring of type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS in the first 3 OFDM symbols of each slot or within the Ys = 1 (with Xs=4) slot, monitoring of type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS with a span duration of Y symbols and a minimum gap of X symbols between the start of two spans, where (X,Y) = (7, 3) | 24-5 | Yes | N/A | Enhanced PDCCH monitoring for 960KHz is not supported | Per band | N/A | N/A | N/A | Component 1 candidate values: one or more of {(4,1), (4,2), (8,4)} | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-6 | Type 1 channel access procedure in uplink for FR2-2 with shared spectrum channel access | 1. Support Type 1 channel access procedure [2. Support LBT performed per carrier/BWP bandwidth] | 24-1a | Yes | N/A | Type 1 channel access procedure in uplink for FR2-2 with shared spectrum channel access is not supported | per band | N/A | N/A | N/A | A UE that supports FR2-2 must indicate this FG is supported when required by regulation | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-7 | Type 2 channel access procedure in uplink for FR2-2 with shared spectrum channel access | 1. Support Type 2 channel access procedure [2. Support LBT performed per carrier/BWP bandwidth] | 24-1a, 24-6 | Yes | N/A | Type 2 channel access procedure in uplink for FR2-2 with shared spectrum channel access is not supported | per band | N/A | N/A | N/A | A UE that supports FR2-2 must indicate this FG is supported when required by regulation | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-8 | 32 DL HARQ processes for FR 2-2 | Support 32 HARQ processes in DL for 120/480/960 kHz | 24-1 | Yes | N/A | 32 DL HARQ processes for FR 2-2 is not supported | FFS | N/A | N/A | N/A | A UE supporting 32 maximum number of HARQ processes for 480/960 kHz SCS for DL shall support 32 as the maximum number of HARQ processes for 120 kHz SCS for DL in FR2-2 | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-9 | 32 UL HARQ processes for FR 2-2 | Support 32 HARQ processes in UL for 120/480/960 kHz | 24-1 | Yes | N/A | 32 DL HARQ processes for FR 2-2 is not supported | FFS | N/A | N/A | N/A | A UE supporting 32 maximum number of HARQ processes for 480/960 kHz SCS for DL shall support 32 as the maximum number of HARQ processes for 120 kHz SCS for DL in FR2-2 | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-10 | Reduced beam switching time delay | Support of reduced beam switching time delay d = 56 symbols for 480 kHz SCS |  | Yes | N/A | Reduced beam switching time delay d = 56 symbols is not supported for 480kHz SCS | per band | N/A | N/A | N/A | If this capability is not reported and the UE supports both FG 24-4 and 24-5, the default value of 112 symbols is assumed | Optional with capability signalling |