3GPP TSG RAN WG1 #109-e	R1-2204882

e-Meeting, May 9th – 20th, 2022

Agenda Item:	9.7.2

Source:	Ericsson

Title:	Network energy saving techniques

Document for:	Discussion and Decision

# Introduction

In RAN#94, it was agreed to start a Rel 18 SI on network energy savings (latest WID in [1]), with the following objectives.

- Definition of a base station energy consumption model [RAN1]

- Adapt the framework of the power consumption modelling and evaluation methodology of TR38.840 to the base station side, including relative energy consumption for DL and UL (considering factors like PA efficiency, number of TxRU, base station load, etc), sleep states and the associated transition times, and one or more reference parameters/configurations.

- Definition of an evaluation methodology and KPIs [RAN1]

- The evaluation methodology should target for evaluating system-level network energy consumption and energy savings gains, as well as assessing/balancing impact to network and user performance (e.g. spectral efficiency, capacity, UPT, latency, handover performance, call drop rate, initial access performance, SLA assurance related KPIs), energy efficiency, and UE power consumption, complexity. The evaluation methodology should not focus on a single KPI, and should reuse existing KPIs whenever applicable; where existing KPIs are found to be insufficient new KPIs may be developed as needed.

Note: WGs will decide KPIs to evaluate and how.

- Study and identify techniques on the gNB and UE side to improve network energy savings in terms of both BS transmission and reception, which may include:

- How to achieve more efficient operation dynamically and/or semi-statically and finer granularity adaptation of transmissions and/or receptions in one or more of network energy saving techniques in time, frequency, spatial, and power domains, with potential support/feedback from UE, and potential UE assistance information [RAN1, RAN2]

- Information exchange/coordination over network interfaces [RAN3]

Note: Other techniques are not precluded

The study should prioritize idle/empty and low/medium load scenarios (the exact definition of such loads is left to the study), and different loads among carriers and neighbor cells are allowed.

In this paper, we provide input for the third objective, i.e., potential techniques on gNB and UE side which can improve network energy savings, with the emphasis that the underlying techniques should prioritize No-, Low- and Medium-load scenarios.

Such techniques include efficient dynamic and/or semi-static adaptation of transmission/reception operations in time, frequency, spatial and power domains including potential information exchange between various network nodes and assistance information from UEs. Some of such RAN1 related techniques are discussed in the current paper.

# Discussion

In this contribution we present various techniques in different domains that could aid in network energy savings. The discussion is mainly relevant to the baseband unit and the radio related equipment which are the components that contribute most to the total consumed network energy. Until an established gNB power model has been agreed in RAN1, these techniques are discussed on a general and qualitative level. As outlined, some of these may be suitable for specific types of scenarios, deployments, and configurations.

## Spatial domain energy saving techniques

In gNBs with active antenna systems, the energy consumed by the multitude of transceiver chains stands for a major part of total consumed network energy. It is quite common that such gNBs are equipped with 64 and above Tx/Rx chains, and more such chains are foreseen in the future, especially at higher frequencies. For efficient beam management, this also results in a higher number of reference signal transmissions such as CSI-RSs which in turn, due to excessive number of radio wakeups, is quite energy consuming. Not all UEs in connected mode benefit from all these transmissions and transceiver chains being active, e.g., when they are close to the gNB, or when the load is low or medium and thus the capacity which is provided by the higher number of antennas is underutilized. It is therefore highly desirable for the gNB to be able to dynamically turn on/off unnecessary circuits depending on a gNB’s load, UE’s link quality and throughput demands.

It shall be noted that having the proper input in the network for optimal decision for turning back ON the ideal transceiver chains is just as important as being able to dynamically turn them OFF. For this point, we think that the UEs could assist in efficient decision making. Even though such assistance is already possible in the form of CSI reporting framework, it is not efficient enough. Excessive CSI reporting/polling for the sake of link assessment involves lots of signaling which is quite energy consuming both for the UE and for the network. For this we think that techniques that help in signaling reduction should be studied. For example, rather than configuring the UE with high-rate periodic CSI transmissions/reports, or high-rate polling of aperiodic reports, RAN1 should study configuring specific criteria in the UE instead. The UE as such would also save energy as they would only measure on the reference signals when necessary and send reports when the criteria are fulfilled.

Additionally, a gNB antenna setup reconfiguration may make the current CSI-RS configurations invalid, and thus an RRC reconfiguration which additionally leads to an energy overhead and latency for both the network and the UE. We should note that it currently is possible to apply gNB antenna setup adaptation, e.g., by muting some ports, remapping of ports to other antennas and so on. Nevertheless, some of these are rather slow processes in part because of current framework, and may impact the gNB scheduler and UE throughput. Therefore, it is also benefits UE to assist the network in choosing an antenna adaptation scheme such that the overall performance is not impacted, and also the UE does not have to consume additional power because of excessive measurement and reporting, potential retransmissions, and RRC reconfigurations. Such UE assistance need not necessary only be in the form of RRC UE Assistance which is relevant for slow-rate scheduling and link adaptations but could also be through more efficient CSI reporting/feedback for faster adaptations.

- A need for increasing number of transceiver chains is foreseen in gNBs in the future, especially at higher frequencies.

- For efficient beam management, increased number of transceiver chains results in a higher number of energy consuming components and reference signal transmissions.

- Higher number of antennas results in a high energy consumption even in low to medium load scenarios.

- Excessive CSI reporting/polling for turning on/off transceiver chains is quite energy consuming both for the UE and for the network.

- Changes in gNB port to antenna mapping may require reference signal reconfiguration.

- Reference signal reconfigurations via RRC is slow and leads to excessive energy consumption.

- Study and identify techniques that allow fast CSI-RS reconfigurations without increasing UE measurement and processing complexity.

- Study and identify techniques for signaling reduction in CSI reporting framework.

- Study and identify techniques including UE slow- and fast-rate feedback for efficient port reconfigurations.

## Time domain energy saving techniques

Next, we discuss the energy saving techniques in time domain, which are basically techniques that allow the network to apply one of the appropriate sleep modes depending on the available inactive time (or sleep opportunity). The inactive time in this case means the time over which the gNB does not need to transmit or receive. A gNB at least on a serving PCell needs to transmit specific signals and messages periodically, e.g., SSBs, SIBs, periodic TRS and if configured periodic CSI-RS, and so on. There may also be frequent paging transmissions including associated PEI transmissions for potential UE energy savings in idle mode. Furthermore, the gNB needs to at least listen to PRACH for potential UEs accessing the cell even if no other UL grants are given. In essence, NR is designed very lean, e.g., SSB periodicity of 20ms, and as such, ideally if there is nothing to transmit or receive between two SSB transmissions, the gNB should be able to go to light sleep and save significant amount of energy. Nevertheless, this is rendered difficult considering the other periodic or configured activities between the two SSB transmissions. Therefore, it is important to minimize the number of such operations between two SSB transmissions to the absolute necessity in order for the network to be able to go to deeper sleep than micro sleep.

For example, a TRS is typically needed when the UE is expected to receive a PDSCH scheduled with a higher order modulation. Furthermore, it is needed when there is data to be delivered for the UE, as such it makes sense that TRS is transmitted only when it is needed.

In addition to minimizing the number of frequent transmissions in between two SSBs, it is important to also minimize the number of periodic receptions, e.g., PRACH occasions. Currently, it is possible to configure the PRACH occasions with longer periodicities, nevertheless, this can impact the initial access delay or latency even for UEs which are camped on the cell. To dynamically change the PRACH occasions between lower and higher periodicities, the gNB and UE has to go through a cumbersome and energy consuming SI update which prevents faster dynamic adaptation of PRACH occasions. Therefore, it is beneficial for both network energy savings as well as UE power consumption and latency, to look for PRACH configuration enhancements such that PRACH occasions configuration can be adapted dynamically without the need for SI update.

Moreover, for the gNB to be able to gain the maximum sleep opportunity, it should be possible to align transmissions as much as possible with the SSB transmissions. E.g., if a gNB is only serving one UE in connected mode, if the gNB aligns the data transmissions to the UE with the SSB transmissions (e.g., of 20ms periodicity), then it can go to light sleep in between and save energy. Nevertheless, a gNB can do this if it is aware that for example such a UE can tolerate a delay of 20 ms or more. Therefore, the more information a gNB can obtain from the UE, the more it can help the scheduler such that it achieves gNB energy savings while the UE is also happy with the underlying latency.

- Frequent Rx/Tx activities (e.g., periodic TRS or PRACH occasions) at low-moderate loads increases the network energy consumption.

- Study and identify techniques minimizing periodic reference signal transmissions, e.g., enabling fully aperiodic TRS for FR1 and FR2 when needed.

- Study and identify techniques which enable dynamic adaptation of PRACH occasions according to the need.

- Study and identify techniques in which the UE can assist the network in optimizing its scheduling to maximize its sleep opportunities.

## Power domain energy saving techniques

Currently the DL transmission power is typically fixed by the gNB for all UEs in a cell.  UEs in good coverage may experience very high SINR levels. For these UEs, even with the highest MCS, the received power may still be higher than required for the specific scenario. For such scenarios, it is useful in terms of energy consumption to reduce the gNB transmission power specifically for the UEs in good coverage. Such reduction would however mean that the transmission power of e.g. PDSCH would change dynamically according to UE channel quality. One issue with changing PDSCH power levels dynamically is that currently the power offset between PDSCH and CSI-RS is notified to UE via RRC signaling. When transmission power of PDSCH changes, if the transmission power of CSI-RS does not change accordingly, e.g., due to that this CSI-RS is shared with more than one UE, this power offset between PDSCH and CSI-RS will change as well. Then this will have impact on CSI feedback from UE as the power offset between PDSCH and CSI-RS is not the same as that notified in RRC signaling. One way is to let gNB compensate for e.g., L1-RSRP and/or CQI reports according to the transmission power difference between actual and the nominal settings. However, such compensation at gNB may not be very accurate. Change of transmission power may in addition to CQI and L1-RSRP, also affect the RI or PMI from the UE for which the gNB may have major difficulties to compensate. Furthermore, change of transmission power may also affect some UE internal receiver settings. Therefore, it is necessary to study methods that can enable PDSCH transmission power changes dynamically.

- Lowering the gNB output power for UEs in good coverage may have very limited impact on throughput.

- UEs need to be aware of PDSCH power offset changes in relation to reference signals, otherwise the CSI reports and UE internal receiver settings may become invalid.

- PDSCH power offsets to reference signals (SSS, CSI-RS) is configured via RRC signalling.

- Study and identify techniques which can enable gNB dynamic power adaptation.

## Frequency/carrier domain energy saving techniques

Energy saving techniques in frequency domain can be divided to techniques which saves on e.g., reduced BW and techniques which saves energy in Scells.

First, we discuss the BW adaptation techniques. On one hand, the network can reduce the energy consumption by reducing the BW at the symbol level, nevertheless, this can also mean that when there is a burst of data, a longer time is necessary to transmit the data. Therefore, there is a trade-off where and when BW adaptation can be useful. Current BWP adaptation provides a good tool for the network to adapt both its own BW as well as UE’s operating BW. Nevertheless, there are some limitations. E.g., the current BWP switching delays in some cases seem unnecessarily long, particularly when the RF characteristics of the BWP remain the same. For certain transmissions, the network can adapt its BW quite fast, and UE can also benefit from this adaptation, e.g., if the network reduces the BW for transmitting PDCCH to save energy, so does the UE. There are however some transmissions, such as reference signals, that are expected to fill the entire configured BWP even if the data related transmission is adapted to smaller BW by the network. In order to also apply BW adaptation for these, the network needs to reconfigure the BWP. Nevertheless, the current delays may prevent the network from applying the adaptation on a faster scale. The current delay means that the network needs to stay in micro-sleep upto 3ms until the UE changes the BWP to start transmission, which prevents the network from going to deeper sleep modes. Additionally, when the network adapts its BW, it makes sense to change the BWP for all the UEs simultaneously so they are also made aware of the changes and can also benefit from it.

Next, we discuss energy savings potential on Scells. Current standards provide good tools for the network to reduce energy consumption on the Scells both for itself and also for the UE, e.g., by deactivating SCells, SCell dormancy, etc. Nevertheless, the network still transmits reference signals such as SSB or TRS or CSI-RS on Scells even if the UE can acquire the necessary synchronization from other cells, e.g., as in the case of SSB less intra-band CA. Transmission of less RSs on Scells can reduce the network energy consumption, and particularly if there is not much activity on the cell, provide a longer sleep time. Therefore, it is necessary to study techniques which can optimize application of reference signals on Scells.

- BW adaptation at the network can potentially save energy at both network and UE side.

- Study potential of reducing the BW adaptation delays for Rel18 UEs.

- Study group-common or cell-specific BWP switching.

- Study techniques which optimize reference signal transmissions over Scells in terms of network energy savings.

# Conclusion

In previous sections, the following observations and proposals were made:

Observation 1	A need for increasing number of transceiver chains is foreseen in gNBs in the future, especially at higher frequencies.

Observation 2	For efficient beam management, increased number of transceiver chains results in a higher number of energy consuming components and reference signal transmissions.

Observation 3	Higher number of antennas results in a high energy consumption even in low to medium load scenarios.

Observation 4	Excessive CSI reporting/polling for turning on/off transceiver chains is quite energy consuming both for the UE and for the network.

Observation 5	Changes in gNB port to antenna mapping may require reference signal reconfiguration.

Observation 6	Reference signal reconfigurations via RRC is slow and leads to excessive energy consumption.

Observation 7	Frequent Rx/Tx activities (e.g., periodic TRS or PRACH occasions) at low-moderate loads increases the network energy consumption.

Observation 8	Lowering the gNB output power for UEs in good coverage may have very limited impact on throughput.

Observation 9	UEs need to be aware of PDSCH power offset changes in relation to reference signals, otherwise the CSI reports and UE internal receiver settings may become invalid.

Observation 10	PDSCH power offsets to reference signals (SSS, CSI-RS) is configured via RRC signalling.

Observation 11	BW adaptation at the network can potentially save energy at both network and UE side.

Proposal 1	Study and identify techniques that allow fast CSI-RS reconfigurations without increasing UE measurement and processing complexity.

Proposal 2	Study and identify techniques for signaling reduction in CSI reporting framework.

Proposal 3	Study and identify techniques including UE slow- and fast-rate feedback for efficient port reconfigurations.

Proposal 4	Study and identify techniques minimizing periodic reference signal transmissions, e.g., enabling fully aperiodic TRS for FR1 and FR2 when needed.

Proposal 5	Study and identify techniques which enable dynamic adaptation of PRACH occasions according to the need.

Proposal 6	Study and identify techniques in which the UE can assist the network in optimizing its scheduling to maximize its sleep opportunities.

Proposal 7	Study and identify techniques which can enable gNB dynamic power adaptation.

Proposal 8	Study potential of reducing the BW adaptation delays for Rel18 UEs.

Proposal 9	Study group-common or cell-specific BWP switching.

Proposal 10	Study techniques which optimize reference signal transmissions over Scells in terms of network energy savings.

# References

RP-220297, 3GPP TSG RAN Meeting #95e, Electronic Meeting, March 17-23, 2022; Revised SI: Study on network energy savings for NR, Huawei