3GPP TSG RAN WG1 #109-e 	R1-2205016

e-Meeting, May 9th – 20th, 2022

Agenda item:	9.1.2

Source: 	Qualcomm Incorporated

Title: 	CSI enhancements for high/medium UE velocities and Coherent-JT

Document for:	Discussion/Decision

# Introduction

In Rel-18 MIMO WID [1], two objectives related to CSI enhancement were scoped:

- Study, and if justified, specify CSI reporting enhancement for high/medium UE velocities by exploiting time-domain correlation/Doppler-domain information to assist DL precoding, targeting FR1, as follows:

  - Rel-16/17 Type-II codebook refinement, without modification to the spatial and frequency domain basis

  - UE reporting of time-domain channel properties measured via CSI-RS for tracking

- Study, and if justified, specify enhancements of CSI acquisition for Coherent-JT targeting FR1 and up to 4 TRPs, assuming ideal backhaul and synchronization as well as the same number of antenna ports across TRPs, as follows:

  - Rel-16/17 Type-II codebook refinement for CJT mTRP targeting FDD and its associated CSI reporting, taking into account throughput-overhead trade-off

  - ……

  - Note: the maximum number of CSI-RS ports per resource remains the same as in Rel-17, i.e. 32

In this contribution, we discuss aspects related to these two items.

# Time-domain CSI for high/medium velocity UE

## (F)eType-II codebook refinement with time-domain correlation

### Usage and general approach

Prior to Rel-18, no time-domain correlation / Doppler-domain information was introduced into the codebook design. A general assumption is that, channel can be assumed constant within a certain time period after a CSI was measured and reported. While this works well with slow varying channel, it can be more challenging under fast fading environment (e.g. due to high/medium UE velocity). More frequent CSI updates can be needed to assist DL precoding adapted to the varying channel, which also brings larger reporting overhead.

As channel’s time correlation decreases more with higher UE velocity, the bottleneck becomes CSI latency, regardless of how frequently it is reported. For example, consider a CSI-RS-to-report timeline Z2' = 69 symbols = 4.9 slots (30kHz SCS), in addition with a duration of 1 slot PUSCH to convey the CSI report, we can assume a total of 6 slots (3 msec) as CSI report latency. Figure 1 shows how channel’s time correlation degrades with the increase of UE velocity. At the latency of 6 slots, while low velocity (3km/h) UE can still have near-constant channel with time correlation close to 1, the delayed CSI for high/medium UE can already be outdated. (Assumption for Figure 1 is Jakes model: Time correlation Ehnhn+T=J02πfdT; Doppler spectrum is in U-shape, i.e. larger power around fdmax and fdmin)

Observation 1: Two issues exist for CSI reporting under fast fading channel environment: (1) Larger overhead with frequent report; (2) CSI outdating due to report latency.

Figure 1. Time correlation for {10,100,200,400}Hz Doppler – {3, 30, 60, 120}km/h @3.6GHz carrier.

To address the above issues, this sub-objective is to study Rel-16/17 Type-II codebook refinement for high/medium UE velocities, by exploiting time-domain correlation / Doppler-domain information to assist DL precoding. Firstly, write the precoder of (F)eType-II in a time-varying sense over instances n (n=0,1,…):

- Rel-16/17: W=W1×W2×WfH

- Rel-18: Wn=W1×W2n×WfH, where SD/FD bases selection can be assumed constant within a short time period since UE location is not supposed to change much

Therefore, for the above two issues: (1) Overhead, and (2) CSI outdating, the target becomes: (1) Time-domain compression for overhead reduction, i.e. how to represent a time series of precoders Wns or coefficient matrices W2ns with less overhead; (2) How to predict channel with the previously observed outdated channel measurements.

A well-known time-domain compression is to represent a series of channel measurements in Doppler-domain. Regarding whether channel prediction is part of the time-domain codebook, it can be classified with two general approaches:

- Approach 1: Codebook only comprises observed CSI-RS measurements

- Approach 2: Codebook comprises both observation and extrapolation

For example in Figure 2, a number of Nob CSI-RS observations are labeled with time indices 0,1,…,Nob-1, while the extrapolations can be up to length N4, with indices Nob,…,N4-1. For the above general approach 1, N4=Nob, while for approach 2, N4>Nob.

Figure 2. Illustration of time-domain compression based on Rel-16 eType-II refinement

A high-level comparison is provided in Table 1 for the two general approaches.

Table 1. Comparison of two general approaches regarding whether codebook comprises extrapolation

|  | Approach 1 | Approach 2 |
| --- | --- | --- |
| Support of prediction | gNB implementation | UE feedback |
| Input for prediction | Quantized precoder Wns | Either measured channel Hns or precoder Wns (unquantized) |
| Other pros./cons. | Possibly more accurate time-domain compression (no need to fit extrapolations) | - Unawareness of future interference for prediction - Higher complexity for UE, e.g. MMSE-extrapolation, more SVD operations with the extrapolated channel |

Besides the above two general approaches, another approach 0 for channel prediction can already possibly be supported under the framework of current standard, as shown in Figure 3. The CSI report series over time can be triggered either by different DCIs (each as aperiodic report), or in a semi-persistent manner, and the prediction is up to gNB implementation – similar as approach 1. The main difference between approach 0 and 1 is just whether or not compressed in time-domain.

Figure 3. Current standard allowable approach 0: Prediction with CSI series by gNB implementation

The pros./cons. of the above three general approaches need to be investigated at the starting point of this study.

Proposal 1: Pros./cons. regarding the following three general approaches need to be studied:

- Approach 0: Legacy Rel-16/17 Type-II reporting, up to gNB implementation for prediction;

- Approach 1: Reporting with only observed CSI-RS instances: Compression-only at UE side, while prediction at gNB side;

- Approach 2: Reporting with both observations and extrapolations: Compression + prediction at UE side

### Evaluation

Some preliminary link-level evaluation results are shown in this section, aiming at providing some initial insights on the problems to tackle. Under a system of 30kHz SCS @4GHz carrier, a maximum Doppler of 100Hz is assumed, which corresponds to a medium UE velocity of 27km/h. Other parameters for the simulation setup are listed in Appendix.

Time-domain parameters for the evaluation are illustrated in Figure 4. A CSI-RS observation window of 8msec (16 slots) is used with 4 CSI-RS instances (observations), and the time interval between two instances is 2msec (4 slots). Assuming a report latency of 3msec and a report periodicity of 16msec, comparison with baseline (delayed Rel-16 eType-II) is based on the duration of [3, 19] msec after the last CSI-RS instance in the observation window.

Figure 4. Time-domain illustration for evaluation setup

For the time-domain compression, DFT basis is utilized.

RMMSE-based extrapolation is applied first before TD compression. In this evaluation, channel Hns are extrapolated.

For simplicity, a noise&interference-free ideal environment is assumed.

The performance is shown in Figure 5 CDF, where the precoded channel with delay-free eigen precoder is normalized as 0 dB. The curves are for the precoding gains of different methods over the delay-free eigen precoder (< 0 dB), including baseline: delayed Rel-16 eType-II, as well as eType-II-Doppler. The later method further divides into two: Depending on whether the input from “future time” is based on estimation (RMMSE-extrapolated), or simply in a genie way (assuming the future time are also “observed” as input for TD compression).

For the 4 segments of gain labeled as (1) to (4) in Figure 5, some intuitive understanding can be interpreted as:

- SD/FD compression/quantization loss of Rel-16 eType-II;

- TD compression/quantization loss of eType-II-Doppler;

- Prediction loss of RMMSE-based extrapolation (w.r.t. genie observations in future) – or saying in another way, this represents performance improvement potentials if implemented with a more advanced prediction algorithm than simply using RMMSE;

- Gain of RMMSE-based prediction over baseline: Delayed Rel-16 eType-II

Figure 5. Performance under noise/interference-free ideal environment

Gains of eType-II-Doppler over baseline are summarized in Table 2.

Table 2. Gain of eType-II-Doppler over delayed Rel-16 eType-II

|  | Extrapolation with estimated Doppler spectrum – segment (4) | Genie “extrapolation” – segment (4) + (3) |
| --- | --- | --- |
| 10% CDF | 1.7 dB | 3.6 dB |
| 50% CDF | 0.4 dB | 0.9 dB |

From the results, it can be observed that, for more severe channel fading, the extrapolation gain can be larger. This can also be verified with precoding gain sampled over time, as in Figure 6. With eType-II-Doppler CSI, the precoding gain samples of eType-II-Doppler becomes smoother and has lower probabilities to degrade severely with a deep fading, which could be beneficial for throughput with link adaptation easier to track.

Figure 6. Precoding gain samples of eType-II-Doppler

Observation 2: Certain performance gain of eType-II-Doppler can be observed over delayed Rel-16 eType-II: 1.7dB @10% CDF, 0.4dB @50% CDF, under ideal environment w/o noise or interference.

Proposal 2: Study and evaluate the performance of (F)eType-II-Doppler CSI.

### UE impact on CSI-RS buffering

For an aperiodic (F)eType-II-Doppler CSI report configured with a periodic CSI-RS (P-CSI-RS), UE needs to be prepared for an aperiodic report potentially triggered any time. This may require UE buffer a series of recently received CSI-RS instances, and can dramatically increase the requirement of UE memory and cause wasted power consumption, especially when operating at a large bandwidth.

To avoid the above issue, aperiodic/SPS CSI-RS can be used, which works in a causal way since Rel-15, i.e. all CSI-RS instances occur no earlier than the slot of the triggering PDCCH. Similarly, P-CSI-RS for (F)eType-II-Doppler CSI may also be considered to work in a causal way.

Proposal 3: Study mechanisms requiring no extra CSI-RS buffer for UE to support (F)eType-II-Doppler CSI, w.r.t. legacy CSIs.

## Doppler reporting based on TRS measurement

Firstly, some potential use cases for Doppler reporting based on TRS measurement are listed below:

- Channel pre-compensation

- Channel prediction

- Assist time-correlation dependent scheduling/configuration e.g. CSI update periodicity, DMRS time density

Channel pre-compensation based on TRS measured Doppler report was discussed in Rel-17 HST-SFN without consensus, which is 2-TRP case. As for a more general sTRP case, channel pre-compensation may have no advantage over legacy baseline: DL tracking by UE implementation, since this “report + pre-compensation” procedure introduces extra latency.

For channel prediction, it has overlap target as the other sub-objective of this study as discussed in Section 2.1. An intuitive approach may be, using TRS-measured TD correlation to assist the prediction of legacy SD/FD CSI measured via CSI-RS. However, the main limitation of TRS is single-port, which makes it difficult to obtain the per-beam Doppler characteristics. Generally speaking, it is expected to provide no better performance than the method based on CSI-RS observations.

For assisting gNB scheduling/configuration whose performance depends on channel’s time-correlation, two examples are given here. The first example is for CSI update periodicity, as discussed at the start of Section 2.1.1. Without Doppler reporting, typically gNB identifies whether CSI is outdated based on HARQ-ACK, which can consume additional latency between failed PDSCH and updated CSI. Base on the Doppler reporting, gNB may determine an estimated CSI outdating period and trigger CSI update more efficiently. A second example is DMRS pattern configured/scheduled with different time densities (PDSCH/PUSCH/PUCCH). For higher Doppler, a larger number of DMRS symbols can be beneficial for time-domain channel estimation.

Proposal 4: Investigate use cases of Doppler reporting based on TRS measurement.

# (F)eType-II codebook refinement for CJT mTRP

## Scenario

Prior to Rel-18, two most relevant works with CJT mTRP are:

- Rel-15 Type-I multi-panel, under the scenario 1A in Figure 7

- Rel-16/17 NCJT, with either co-located or distributed TRPs, as scenario 1B or 2 respectively in Figure 7

Figure 7. Scenarios of CJT mTRP to consider in Rel-18

For CJT mTRP, co-located scenario 1A/1B is more practical than distributed scenario 2, in the sense of satisfying the objective assumption “ideal backhaul and synchronization.” For co-located RRUs (TRPs/panels), they can share a same local oscillator (LO) and thus have ideal synchronization. However, for a cost-practical deployment of distributed RRUs, each RRU have its individual LO. According to [2], the oscillator drift can be 0.05ppm or 0.1ppm (Table 6.5.1.2-1 in [2]), which means a maximum phase error of 126° even within a short period of only 10 msec: 2π×700×106×0.05×10-6×10×10-3=126°, where a low-band of only 700MHz carrier is assumed. Besides, phase error increases with a higher carrier frequency. Thus for scenario 2, ideal synchronization is difficult to be satisfied even with ideal backhaul.

Observation 3: It is impractical for distributed TRPs to satisfy objective assumption of “ideal synchronization.”

How does the distributed TRPs achieve synchronization is up to network implementation and is out of the scope of the study objective. Therefore, scenario 2 should be deprioritized in this study.

As from evaluation perspective, all the three scenarios can be supported under the modelling of TR 38.901 [3]: Scenario 1A can be supported by antenna modelling comprising MgNg panels, while scenario 1B can be modelled as inter-sector cells.

Proposal 5: Study and evaluate Rel-16/17 Type-II CJT mTRP under the following scenarios:

- Scenario 1A: Co-located TRPs/panels with a same boresight orientation;

- Scenario 1B: Co-located TRPs/panels with different boresight orientations (e.g. inter-sector)

- Scenario 2 (deprioritized): Distributed TRPs

## Codebook structure

For Rel-16/17 Type-II codebook structure to support mTRP, one simplest way is just to add one intermediate dimension (TRP-dimension) in spatial-domain, since TRP is just a group of antenna ports. Main modification is associated with SD basis matrix W1, e.g. with selected SD bases for each TRP arranged diagonally:

WTRP#AWTRP#B=W1,A00W1,B×W2×WfH

For other part of the precoder, to reduce the standard efforts, it can reuse existing (F)eType-II as much as possible.

Proposal 6: For F(e)Type-II CJT mTRP codebook, minimize modification to existing precoder by mainly enhancing W1 structure.

Figure 8. Illustration of CJT mTRP codebook based on Rel-16 eType-II refinement

From UE perspective, one main challenge is the CSI processing complexity regarding the potentially increased number of ports with up to 4 TRPs. If the maximum 32-port is maintained for a report, UE complexity can be expected comparable to Rel-16 eType-II. For low FDD bands typically targeted by this technique (e.g. 700MHz), 32-port can be enough for up to 4 TRPs, each with 8 ports.

Proposal 7: Restrict the total number of ports for (F)eType-II-CJT mTRP CSI report no larger than 32.

## CSI-RS resource configuration

For the two relevant previous works, the respective CSI-RS resource configuration is

- Rel-15 Type-I MP: Port level for each TRP/panel, implicitly as consecutive ports within one CSI-RS resource, as in Table 3

- Rel-16/17 NCJT: CSI-RS resource level for each TRP, configured as two resource groups within a resource set for 2 TRPs

Table 3. An example of 32-port CSI-RS for 4-panel Type-I MP

| Panel index | 0 | 1 | 2 | 3 |
| --- | --- | --- | --- | --- |
| Port index | 0,…,7 | 8,…,15 | 16,…,23 | 24,…,31 |

Proposal 8: Investigate potential standard efforts for defining CMR for CJT TRPs: (1) On port level, or, (2) on CSI-RS resource level

# Conclusion

In this contribution, we discuss issues related to CSI enhancement for Doppler-domain information and CJT mTRP. Based on the observations:

Observation 1: Two issues exist for CSI reporting under fast fading channel environment: (1) Larger overhead with frequent report; (2) CSI outdating due to report latency.

Observation 2: Certain performance gain of eType-II-Doppler can be observed over delayed Rel-16 eType-II: 1.7dB @10% CDF, 0.4dB @50% CDF, under ideal environment w/o noise or interference.

Observation 3: It is impractical for distributed TRPs to satisfy objective assumption of “ideal synchronization.”

We propose:

Proposal 1: Pros./cons. regarding the following three general approaches need to be studied:

- Approach 0: Legacy Rel-16/17 Type-II reporting, up to gNB implementation for prediction;

- Approach 1: Reporting with only observed CSI-RS instances: Compression-only at UE side, while prediction at gNB side;

- Approach 2: Reporting with both observations and extrapolations: Compression + prediction at UE side

Proposal 2: Study and evaluate the performance of (F)eType-II-Doppler CSI.

Proposal 3: Study mechanisms requiring no extra CSI-RS buffer for UE to support (F)eType-II-Doppler CSI, w.r.t. legacy CSIs.

Proposal 4: Investigate use cases of Doppler reporting based on TRS measurement.

Proposal 5: Study and evaluate Rel-16/17 Type-II CJT mTRP under the following scenarios:

- Scenario 1A: Co-located TRPs/panels with a same boresight orientation;

- Scenario 1B: Co-located TRPs/panels with different boresight orientations (e.g. inter-sector)

- Scenario 2 (deprioritized): Distributed TRPs

Proposal 6: For F(e)Type-II CJT mTRP codebook, minimize modification to existing precoder by mainly enhancing W1 structure.

Proposal 7: Restrict the total number of ports for (F)eType-II-CJT mTRP CSI report no larger than 32.

Proposal 8: Investigate potential standard efforts for defining CMR for CJT TRPs: (1) On port level, or, (2) on CSI-RS resource level

# References

- RP-213598, New WID: MIMO Evolution for Downlink and Uplink

- TS 38.104, Base Station (BS) radio transmission and reception

- TR 38.901, Study on channel model for frequencies from 0.5 to 100 GHz

# Appendix: Simulation setup for (F)eType-II Doppler CSI

| General |  |
| --- | --- |
| Carrier frequency | 4GHz |
| SCS | 30kHz |
| Tx Antenna | 32 (1x16x2) |
| Rx Antenna | 4 |
| Bandwidth | 20MHz (48 RBs) |
| Channel | CDL-C |
| Delay spread | 100nsec |
| Doppler | 100Hz (UE velocity 27km/h) |
| Codebook parameter |  |
| Subband size | 4 RBs |
| #SD bases (L) | 4 |
| #FD bases (M) | 3 (p=1/4 for rank 1) |
| Rank | Fixed rank 1 |
| #NZC (K0) | 12 (β=1/2) |
| #TD bases (S) | 2 |
| Time-domain parameter |  |
| CSI-RS periodicity | 2msec (4 slots) |
| Observation window | 8msec (4 CSI-RS observations) |
| Report latency | 3msec (6 slots) |
| Report periodicity | 16msec |