3GPP TSG RAN WG1 #109-e 		R1-220XXXX

e-Meeting, May 9th – 20th, 2022

Title:	DRAFT LS on TCI indication in multicast DCI

Release:	Rel-17

Work Item:	NR_MBS

Source:	RAN1

To:	RAN2

Cc:

Contact person:	Tuo Yang

yangtuo@chinamobile.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

1	Overall description

RAN1 made the following agreement about the existence condition for the TCI field in DCI format 4_2 and would like to inform RAN2 that parameter tci-PresentInDCI is also used to indicate if TCI field is present or absent in DCI format 4_2.

Agreement Adopt the following text proposal to TS38.214 v17.1.0: - Reason for change - Clause 7.3.1.5.3 of 38.212 states that the existence condition for the TCI field in DCI format 4_2 is defined in clause 5.1.5 of 38.214. However, 5.1.5 of 38.214 does not yet have a corresponding description. - Summary of change - Add “If a UE is configured with the higher layer parameter tci-PresentInDCI that is set as ‘enabled’ for the CORESET scheduling the multicast PDSCH, the UE assumes that the TCI field is present in the DCI format 4_2 of the PDCCH transmitted on the CORESET.” - Consequences if not approved - The condition for the existence of TCI field in DCI format 4_2 is unclear. Send a LS to RAN2 to inform that parameter tci-PresentInDCI is also used to indicate if TCI field is present or absent in DCI format 4_2. ----------------- Start of TP to TS 38.214 v17.1.0 ---------------- <Unchanged text is omitted> If a UE is configured with the higher layer parameter tci-PresentInDCI that is set as ‘enabled’ for the CORESET scheduling the PDSCH, the UE assumes that the TCI field is present in the DCI format 1_1 of the PDCCH transmitted on the CORESET. If a UE is configured with the higher layer parameter tci-PresentDCI-1-2 for the CORESET scheduling the PDSCH, the UE assumes that the TCI field with a DCI field size indicated by tci-PresentDCI-1-2 is present in the DCI format 1_2 of the PDCCH transmitted on the CORESET. If a UE is configured with the higher layer parameter tci-PresentInDCI that is set as ‘enabled’ for the CORESET scheduling the multicast PDSCH, the UE assumes that the TCI field is present in the DCI format 4_2 of the PDCCH transmitted on the CORESET. If the PDSCH is scheduled by a DCI format not having the TCI field present, and the time offset between the reception of the DL DCI and the corresponding PDSCH of a serving cell is equal to or greater than a threshold timeDurationForQCL if applicable, where the threshold is based on reported UE capability [13, TS 38.306], for determining PDSCH antenna port quasi co-location, the UE assumes that the TCI state or the QCL assumption for the PDSCH is identical to the TCI state or QCL assumption whichever is applied for the CORESET used for the PDCCH transmission within the active BWP of the serving cell. <Unchanged text is omitted> ----------------- End of TP to TS 38.214 v17.1.0 ----------------

2	Actions

To: RAN2

ACTION: 	RAN1 respectfully asks RAN2 to take the above information into account of ASN.1 review.

3	Dates of next TSG RAN WG1 meetings

TSG RAN WG1 Meeting #110                       	22nd – 26th Aug, 2022			Toulouse, France

TSG RAN WG1 Meeting #110bis-e                       	10th – 19th  Oct, 2022			E-meeting