3GPP TSG RAN WG1 Meeting #109-e	R1-22xxxxx

e-Meeting, May 9 - 20, 2022

Agenda Item:	8.9

Source:	Moderator (Huawei)

Title:	Preparation phase discussion on 109-e-Prep-AI8.9 NB-IoT-eMTC

Document for:	Discussion and Decision

Introduction

This documents summarizes the preparation phase discussion of contributions submitted to AI 8.9 for Rel-17 WI NB-IoT and eMTC enhancements [2-5].

Issues

The issues are summarized in the following table.

| Issues | Summary of issues | Related contributions | Proposals |
| --- | --- | --- | --- |
| #1 | On whether and how to use the DwPTS in special subframes for NPDSCH with 16QAM. | [2] | Text proposal to TS 36.211: - On an NB-IoT carrier for which higher-layer parameter operationModeInfo indicates inband-SamePCI or inband-DifferentPCI, or higher-layer parameter inbandCarrierInfo is present, or on an NB-IoT carrier for SystemInformationBlockType1-NB for which sib1-carrierInfo indicates non-anchor and the value of the higher layer parameter sib-GuardbandInfo is set to sib-GuardbandInbandSamePCI or sib-GuardbandinbandDiffPCI, DwPTS in special subframe configuration 0 and 5 for normal cyclic prefix is not used for NPDCCH and NPDSCH transmission. DwPTS in special subframe configuration 9 for normal cyclic prefix is not used for NPDSCH transmission with 16QAM, when NPDSCH-16QAM-Config-NB is configured. |
|  |  | [4] | Text proposal to TS 36.211: On an NB-IoT carrier for which higher-layer parameter operationModeInfo indicates inband-SamePCI or inband-DifferentPCI, or higher-layer parameter inbandCarrierInfo is present, or on an NB-IoT carrier for SystemInformationBlockType1-NB for which sib1-carrierInfo indicates non-anchor and the value of the higher layer parameter sib-GuardbandInfo is set to sib-GuardbandInbandSamePCI or sib-GuardbandinbandDiffPCI, DwPTS in special subframe configuration 0 and 5 for normal cyclic prefix is not used for NPDCCH and NPDSCH transmission, and when npdsch-16QAM-Config-r17 is configured then DwPTS in special subframe configuration 9 for normal cyclic prefix is also not used for NPDSCH transmission. |
|  |  | [5] | Proposal 1: It is up to the eNB to ensure that NPDSCH transmission on DwPTS using 16-QAM is self-decodable (e.g. coding rate lower than 0.932) by the UE after rate matching. There is no need for specification change. |
| #2 | The power allocation for NPDSCH with 16QAM is missed for PDSCH in PUR procedure. | [3] | Text proposal to TS 36.213: If a UE is configured with higher layer parameters npdsch-16QAM-Config or pur-DL-16QAM-Config and nrs-PowerRatio, - the ratio of NPDSCH EPRE to NRS EPRE among NPDSCH REs in symbols with NRS is given by for a cell with one NRS antenna port and for a cell with two NRS antenna ports, where is given by the parameter nrs-PowerRatio. |

Discussion

It is proposed to have an email thread to address the two issues:

- One email thread to address the following issues for NB-IoT 16QAM

  - Issue #1: On whether and how to use the DwPTS in special subframes for NPDSCH with 16QAM.

    - Discussed in R1-2203223, R1-2204082 and R1-2204878

  - Issue #2: On the power allocation for NPDSCH with 16QAM in PUR procedure

    - Discussed in R1-2203631

Please input your comments for the proposed email discussion:

| Companies | Comments |
| --- | --- |

Summary

- References

- RP-211340, “WID revision: Additional enhancements for NB-IoT and LTE-MTC”, Huawei, HiSilicon, RAN#92e, E-meeting, June 2021.

- R1-2203223	On use of DwPTS for 16QAM NPDSCH in NB-IoT	Huawei, HiSilicon

- R1-2203631	Clarifications for DL power allocation for 16-QAM	ZTE, Sanechips

- R1-2204082	Support of 16-QAM for unicast in UL and DL in NB-IoT	Ericsson

- R1-2204878	Support of 16-QAM in NB-IoT TDD	Nokia, Nokia Shanghai Bell