3GPP TSG RAN WG1 #109-e			R1-220nnnn

e-Meeting, May 9th – 20th, 2022

Agenda item:	9.1.1.1

Source: 	Moderator (MediaTek)

Title: 	Moderator summary on extension of unified TCI framework for MTRP (Round 2)

Document for:	Discussion and Decision

- Introduction

In RAN#94e, the Rel-18 WID of MIMO evolution for downlink and uplink is approved [1]. In the approved WID, extension of unified TCI framework is a part of the RAN1 objectives, and the detailed scope of this agenda item (Item 1A) includes the following highlighted objectives:

RAN1: - … - Specify extension of Rel-17 Unified TCI framework for indication of multiple DL and UL TCI states focusing on multi-TRP use case, using Rel-17 unified TCI framework. - … - … - … - Study, and if needed, specify the following items to facilitate simultaneous multi-panel UL transmission for higher UL throughput/reliability, focusing on FR2 and multi-TRP, assuming up to 2 TRPs and up to 2 panels, targeting CPE/FWA/vehicle/industrial devices (if applicable) - UL precoding indication for PUSCH, where no new codebook is introduced for multi-panel simultaneous transmission - The total number of layers is up to four across all panels and total number of codewords is up to two across all panels, considering single DCI and multi-DCI based multi-TRP operation. - UL beam indication for PUCCH/PUSCH, where unified TCI framework extension in objective 2 is assumed, considering single DCI and multi-DCI based multi-TRP operation - For the case of multi-DCI based multi-TRP operation, only PUSCH+PUSCH, or PUCCH+PUCCH is transmitted across two panels in a same CC. - Study, and if justified, specify the following - Two TAs for UL multi-DCI for multi-TRP operation - Power control for UL single DCI for multi-TRP operation where unified TCI framework extension in objective 2 is assumed. For the case of simultaneous UL transmission from multiple panels, the operation will only be limited to the objective 6 scenarios.

Based on the contributions from companies [2]-[31], the followings are provided in this document:

- Summary of companies’ views on each of open issues raised by interested companies

- Observation and recommended proposal based on the summary of companies’ views

- Issue 1 – Extension of Unified TCI Framework

Open issues on unified TCI framework extension and company views are summarized below.

Table 1 Summary for Issue 1

| # | Issue | Companies’ views | FL note/observation |
| --- | --- | --- | --- |
| 1.3 | Max number of indicated joint TCI states (M1) for joint DL/UL TCI update Max number of indicated DL TCI states (M2) for separate DL/UL TCI update Max number of indicated UL TCI states (N2) for separate DL/UL TCI update | Atl1: M1 = 2, M2 = 2, N2 = 2 - Support: Samsung, Docomo, OPPO, Apple, Qualcomm, Intel, Nokia, ZTE, MTK, InterDigital, CATT, Spreadtrum, Sony, LGE, ITRI, TransHold, Fraunhofer, Fujitsu, Huawei, FGI, AT&T Atl2: M1 > 2, M2 > 2, N2 > 2 - Support: Ericsson (up to 4 indicated joint, DL, and/or UL TCI states) | From moderator’s observation, {M1, M2, N2} = {2, 2, 2} is sufficient to support MTRP operation, which is the use case that should be focused on according to the WID. Another potential use case (separate control and data beams) has been proposed in one contribution, however, {M1, M2, N2} = {2, 2, 2} doesn't prevent that use case. {M1, M2, N2} = {2, 2, 2} is incapable only when both use cases work at the same time (i.e., MTRP + separate control and data beams per TRP-link), but whether such direction is still within the scope defined in the WID is doubtful. Since these max numbers could impact the later designs a lot, moderator suggests concluding them as early as possible. Given the majority view, Proposal 1.B is recommended. How to configure/determine the exact number of indicated joint/DL/UL TCI states can be further discussed |
| 1.4 | The multiple indicated joint/DL/UL TCI states are updated by MAC-CE or DCI with the necessary MAC-CE based TCI state activation (analogous to Rel-17 procedure) | Support: Ericsson, Samsung, Docomo, OPPO, ZTE, vivo, Apple, Qualcomm, MTK, InterDigital, CATT, Futurewei, Spreadtrum, Sony, Xiaomi, LGE, Lenovo, CMCC, TransHold, Fraunhofer, Fujitsu, Nokia (s-DCI mode), FGI, AT&T, Intel Concern: | Given the majority view on this issue, Proposal 1.B is recommended accordingly. Details of TCI state update and activation are discussed in the following sub-issues |
| 1.5 | Individual TCI update mode (joint or separate DL/UL TCI update) for each TRP, i.e., one TRP with joint DL/UL TCI update and another with separate DL/UL TCI update | Support: Nokia (m-DCI mode), Qualcomm, , CATT, Sony, Xiaomi, ITRI, FGI, ZTE, Intel Concern: Apple (no use case), OPPO |  |
| 1.6 | TCI state update for S-DCI based MTRP | Use existing (single) TCI field in DCI to update all or subset of indicated TCI states: - Support: Ericsson, Samsung (DCI w/ DLA), Docomo, OPPO (DCI w/ DLA), Apple, Qualcomm, Intel, ZTE, vivo, InterDigital, CATT, TransHold, Futurewei, Spreadtrum, Sony, CEWiT, MTK, Nokia, Fujitsu, LG, AT&T - Concern: More than one TCI fields in DCI w/o DLA and each TCI field can update indicated TCIs respective to one of the TPRs: - Support: Samsung, OPPO, FGI, LG - Concern: Apple (DCI overhead), Intel | Given the majority view on this issue, Proposal 1.C is recommended accordingly. How to activate TCI states for indicated TCIs states can be discussed after the update scheme is sufficiently mature If single TCI field in DCI is agreed, whether to increase the max number of codepoints/bits can be further discussed If single TCI field in DCI is agreed, whether the switching between S-TRP and M-TRP is determined from the number of TCI states associated with the indicated codepoint can be further discussed |
| 1.7 | TCI state update for M-DCI based MTRP | Alt1: Use existing (single) TCI field in DCI associated with one of CORESETPoolIndex values to update the indicated TCI states respective to the CORESETPoolIndex value (i.e., no cross-TRP beam indication) - Support: Samsung, Nokia, Docomo, Qualcomm, Intel, ZTE, vivo, MTK, Xiaomi. LGE, Fraunhofer, FGI, OPPO, Fujitsu, TransHold - Concern: Apple, Ericsson Alt2: Use existing (single) TCI field DCI associated with one of CORESETPoolIndex values to update the indicated TCI states respective to both CORESETPoolIndex values (i.e., cross-TRP beam indication can be supported) - Support: Apple, Xiaomi - Concern: Docomo (not good in non-ideal backhaul), Ericsson, InterDigital, Intel | Two alternativities for potential down-selection are provided by Proposal 1.D How to activate TCI states for the indicated TCI states can be discussed after the update scheme is sufficiently mature |
| 1.8 | DCI format for updating the indicated TCI stares | Alt1: Reuse the same DCI formats as in Rel-17 (i.e., DCI formats 1_1/1_2 with or without DLA), and no additional DCI format is introduced - Support: ZTE, vivo, CATT, Apple, OPPO Docomo, Nokia, Ericsson, Fujitsu, LG - Concern: Atl2: In addition to the DCI formats used in Rel-17, introduce DCI formats 0_1/0_2 for updating at least the indicated UL TCI states: - Support: Xiaomi, Intel, FGI, LG - Concern: Docomo, Ericsson, Spreadtrum |  |
| 1.9 | RRC-configured TCI state lists | Alt1: Reuse Rel-17 design (i.e., one TCI state list for joint/DL TCI states and one TCI state list for UL TCI states) - Support: Ericsson, MTK, Docomo (if the max # of configured TCI states is not increased for MTRP), Nokia, Fraunhofer, Xiaomi, OPPO, Fujitsu, Intel - Concern: Apple (not good for TCI pool sharing for CCs with different sTRP/mTRP operation) Atl2: TRP-specific TCI state list(s) - Support: ZTE, Apple, vivo (if individual TCI update mode is allowed for each TRP), Docomo (if the max # of configured TCI states is increased for MTRP), FGI - Concern: Ericsson Increase the max number of configured joint/DL/UL TCI states for MTRP operation - Support: - Concern: |  |
| 1.10 | Introduction of TRP-ID associated with or included in each TCI state | Support: CMCC, Spreadtrum, ZTE (still case-by-case) Concern: Ericsson, MTK, Apple, Docomo, Nokia, CATT, OPPO, LG, Intel |  |
| 1.11 | Applying/mapping the indicated TCI states to channel/signals for S-DCI based MTRP | To inform to the UE at least which one or two indicated TCI states (or which one or two TRPs) is mapped to the corresponding channel(s)/signal(s), an indicator is introduced: - Per CORESET or per search space set: Ericsson, Xiaomi, ZTE, vivo, CATT, Nokia, MTK, Qualcomm, Samsung, Apple (CORESET), Docomo, FGI, OPPO (per CORESET), Fujitsu, LG, Intel (CORESET) - Per DCI with DL assignment for the scheduled/activated PDSCH: ZTE, vivo, MTK, Qualcomm, CATT, FGI, Fujitsu, LG - Per TDRA codepoint for scheduled/activated PDSCH/PUSCH: Apple - Per DCI with UL grant for the scheduled/activated PUSCH: vivo (reinterpret the SRS resource set indicator), Qualcomm, MTK, Xiaomi (reinterpret the SRS resource set indicator), Fujitsu, LG - Per dedicated PUCCH resource: Ericsson, ZTE, CATT (MAC-CE update), Nokia, MTK, Apple, Docomo, Xiaomi, LG - Per [P/SP] CSI-RS resource or resource set: Ericsson, ZTE, vivo, MTK, Apple (set), Docomo, Fraunhofer, Xiaomi, LG - Per [P/SP] SRS resource set: Ericsson, OPPO, Nokia, ZTE, vivo, MTK, Apple (set) , Docomo, Fraunhofer, Xiaomi, LG - Per DCI with CSI request for the triggered AP CSI-RS: vivo - Per DCI with SRS request for the triggered AP SRS: vivo - Per Type-1 CG configuration: Nokia, Fraunhofer, Xiaomi | Given the majority for PDCCH reception in this issue, Proposal 1.E is recommended accordingly. If two indicated TCI states are mapped to a channel, how to map the indicated TCI states to each of repetition occasions (or CDM groups) of the channel can be further discussed. |
| 1.12 | Applying/mapping the indicated TCI states to channels/signals for M-DCI based MTRP | Unified schemes for both S-DCI and M-DCI to apply/map the indicated TCI states to channel(s)/signal(s) - Support: Ericsson, Docomo - Concern: Nokia, Huawei The indicated TCI state(s) respective to one of CORESETPoolIndex values applies to: - PDCCH on the CORESET(s) configured/associated with the CORESETPoolIndex value (as in Rel-17): ZTE, Qualcomm, Nokia, vivo, Samsung, MTK, LGE, Xiaomi, Apple, Docomo, Fraunhofer, OPPO, Fujitsu, TransHold, Intel - PDSCH/PUSCH scheduled/activated by the DCI associated with the CORESETPoolIndex value: ZTE, Xiaomi, MTK, vivo, Qualcomm, Samsung, Apple, Fraunhofer, Fujitsu, TransHold, Intel - PUCCH with HARQ-ACK corresponding to the DCI associated with the CORESETPoolIndex value: Nokia, vivo, Qualcomm, Apple, Fraunhofer, TransHold - AP CSI-RS triggered by the DCI associated with the CORESETPoolIndex value: ZTE, Xiaomi, Nokia, ZTE, vivo, Qualcomm, Apple, OPPO - AP SRS triggered by the DCI associated with the CORESETPoolIndex value: ZTE, Xiaomi, Nokia, ZTE, vivo, Qualcomm, Apple For channels/signals that don't have explicit/implicit association with a CORESETPoolIndex value: - Introduce an indicator (reuse CORESETPoolIndex or a new one) to indicate which indicated TCI state(s) (or which TRP(s)) is associated with the corresponding channel/signal: Nokia, Apple, vivo, Fraunhofer, ZTE, MTK, Xiaomi, Docomo, FGI, LG | Whether an explicit association between indicated TCI state(s) and an CORESETPoolIndex value is needed may depend on the result of sub-issue 1.7, thus can be discussed later. |

Proposal 1.B: On unified TCI framework extension, support more than one indicated joint/DL/UL TCI states in a CC/BWP for MTRP operation

- The indicated TCI states are updated by MAC-CE or DCI with the necessary MAC-CE based TCI state activation

  - Up to 2 indicated joint TCI states can be provided in a CC/BWP for joint DL/UL TCI update

  - Up to 2 indicated DL TCI states can be provided in a CC/BWP for separate DL/UL TCI update

  - Up to 2 indicated UL TCI states can be provided in a CC/BWP for separate DL/UL TCI update

  - FFS: Whether indicated joint TCI state(s) can be provided together with indicated DL TCI state(s) and/or indicated UL TCI state(s) in a CC/BWP, and if applicable, the maximum number of the indicated joint/DL/UL TCI states in the CC/BWP

  - FFS: How to provide the exact number of indicated joint/DL/UL TCI states that need to be maintained in a CC/BWP, e.g., based on the indicated TCI codepoint, TCI state activation, or RRC configuration

  - FFS: Details of update and activation for the indicated TCI states for S-DCI based MTRP

  - FFS: Details of update and activation for the indicated TCI states for M-DCI based MTRP

  - FFS: How to map/apply one or two indicated TCI states to a target channel(s)/signal(s)

Proposal 1.C: On unified TCI framework extension, use the existing TCI field in DCI format 1_1/1_2 (with or without DL assignment) to update all or subset of indicated TCI states in a CC/BWP or a set of CCs/BWPs for single-DCI based MTRP

- FFS: Detail of mapping joint/DL/UL TCI states to a TCI field codepoint, e.g., possible combinations of joint, DL, and/or UL TCI states that can be mapped to a TCI field codepoint

- FFS: Whether to increase the max number of MAC CE activated TCI field codepoints, i.e., more than 8 codepoints

- FFS: Whether to increase the max number of TCI field bits, i.e., more than 3 bits

- Note: This doesn't imply that support of one additional TCI field or a field associating the TCI field to the TRP(s) is precluded

Proposal 1.D: On unified TCI framework extension, support at least one of the following alternatives for multi-DCI based MTRP:

- Alt1: Use the existing TCI field in the DCI format 1_1/1_2 (with or without DL assignment) associated with one of CORESETPoolIndex values to update the indicated TCI state(s) respective to the associated CORESETPoolIndex value

  - FFS: Association between indicated TCI state(s) and a CORESETPoolIndex value, if Alt1 is supported

- Alt2: Use the same TCI state update for single-DCI based MTRP, i.e., use the existing TCI field in any DCI format 1_1/1_2 (with or without DL assignment) to update all or subset of indicated TCI states

Proposal 1.E: When the UE is provided with more than one indicated DL/joint TCI states in a CC/BWP, support an indicator by RRC signaling to inform the UE which indicated DL/joint TCI state should be applied to PDCCH receptions on the CC/BWP

- FFS: Detail design of the indicator, e.g., how to indicate, the indicator is provided per CORESET or per search space set, whether to reuse the existing RRC parameter or introduce a new one, etc.

- FFS: Whether the same indicator is used to inform the UE that two indicated DL/joint TCI states are applied for PDCCH-SFN

- FFS: Whether the same indicator is used for both S-DCI and M-DCI based MTRP

Table 2 Additional inputs for Issue 1

| Company | Input |
| --- | --- |
| Mod V0 | - Please check the updated Proposal 1.B. To avoid those concerns raised during GTW discussion, the proposal is revised to agree on the maximum numbers first. - Please share your view on Proposals 1.C, no change from the 1st round discussion - Please share your view on new Proposal 1.D and 1.E |
| Ericsson | Proposal 1.B: Support Proposal 1.C: Propose to add “at least”: Proposal 1.C: On unified TCI framework extension, use the existing TCI field in DCI format 1_1/1_2 (with or without DL assignment) to update all or subset of indicated TCI states in a CC/BWP or a set of CCs/BWPs at least for single-DCI based MTRP Proposal 1.D: Do not support. We should avoid stating alternatives this early. Wait until further development of the sDCI solution has been performed – then we can compare solutions. Proposal 1.E: Support with a slight edit: Proposal 1.E: When the UE is indicated with more than one DL/joint TCI states in a CC/BWP, support an indicator by RRC signaling to inform the UE which indicated DL/joint TCI state should be applied to PDCCH receptions on the CC/BWP |
| Futurewei | Proposal 1.B: Support. Proposal 1.C: Support. We are also fine with Ericsson’s proposed modification. Proposal 1.D: Support. Proposal 1.E: We would like to have some clarifications on this proposal. First, to our understanding, this proposal is for S-DCI based MTRP as it is based on discussion on Issue 1.11. So we suggest adding “for single-DCI based MTRP” in the main bullet. Second, if existing RRC parameter(s) are reused as stated in the first FFS, depending on the scenario, it is possible that different parameter will be used for different scenario, instead of using just one single parameter. Therefore we would like to make the following modifications: Proposal 1.E: When the UE is provided with more than one indicated DL/joint TCI states in a CC/BWP for single-DCI based MTRP, support utilizing indicator(s) by RRC signaling to inform the UE which indicated DL/joint TCI state should be applied to PDCCH receptions on the CC/BWP - FFS: Detail design of the indicator(s), e.g., how to indicate, the indicator(s) are provided per CORESET or per search space set, whether to reuse the existing RRC parameter(s) or introduce a new one, etc. - FFS: Whether the same indicator(s) are used to inform the UE that two indicated DL/joint TCI states are applied for PDCCH-SFN |
| Xiaomi | Proposal 1.B: support Proposal 1.C: For the main bullet, we have two comments. First one, here only TCI update is mentioned, what about the first time indication? Second one, here mentioned all or subset, does it mean there is a reference set? Without the reference set, it is meaningless to talk about all or subset of indicated TCI states. According to the response from feature lead, the reference set will be decided by TCI mode. But from Proposal 1.B, we can see that TCI mode can be provided by the indicated TCI codepoint, TCI state activation, or RRC configuration. So if to keep “all or subset”, we suggest to define the reference set first. But from our point of view, the reference set can be defined if the TCI mode is provided by RRC configuration. While for the case of provided by the indicated TCI codepoint or TCI state activation, it is difficult to define it. So the simplest way is to remove “all or subset”. Proposal 1.D: Alt 2 is not clear. We suggest to update it as below: Proposal 1.D: On unified TCI framework extension, support at least one of the following alternatives for multi-DCI based MTRP: - Alt1: Use the existing TCI field in the DCI format 1_1/1_2 (with or without DL assignment) associated with one of CORESETPoolIndex values to update the indicated TCI state(s) respective to the associated CORESETPoolIndex value - FFS: Association between indicated TCI state(s) and a CORESETPoolIndex value, if Alt1 is supported - Alt2: Use the same TCI state update for single-DCI based MTRP, i.e., use the existing TCI field in any DCI format 1_1/1_2 (with or without DL assignment) associated with one of CORESETPoolIndex values to update all or subset of the indicated TCI state(s) for any one or two of CORESETPoolIndex values. Proposal 1.E: first we share same view as Samsung that ‘S-DCI based M-TRP’ should be added. In addition, we prefer to use ‘which indicated DL/joint TCI state(s)’ since PDCCH repetition and PDCCH-SFN should also be considered. Thirdly, we are not sure RRC signaling is sufficient or not, whether an association between TCI state(s) and TRP is necessary. So we suggest to add a FFS that “an association between TCI state(s) and TRP”. |
| DOCOMO | Proposal 1.B: Support. If other Rel.18 agenda (e.g. CJT) requires larger number of TRPs, we can discuss it later. The current description of “More than one indicated joint/DL/UL TCI states” is very general, hence, we support to list the supported combination as in FL proposal. Proposal 1.C: Support. Proposal 1.D: Support. We support Alt.1. We think Alt.2 is not suitable for non-ideal backhaul that one DCI from one TRP indicates two TCI states for both TRPs. Proposal 1.D: Support. We are ok with limiting the proposal to S-DCI. While we are also ok to cover both S-DCI and M-DCI, but in that case, “an indicator by RRC signaling” can be existing CORESETPoolIndex for M-DCI. For SFN-CORESET, no indicator is needed in case of 2 indicated TCI states, but indication would be needed if more than 2 indicated TCI states are indicated. Hence, we support to study for SFN-CORESET. |

- Issue 2 – UL Power Control for UL MTRP

Open issues on UL PC for UL MTRP and company views are summarized below.

Table 3 Summary for Issue 2

| # | Issue | Companies’ views | Moderator notes/observation |
| --- | --- | --- | --- |
| 2.1 | Discussion on Issue 2 should start after simultaneous UL transmission schemes are determined in AI 9.1.4.1 | Support: Samsung, ZTE, Ericsson, Huawei, HiSilicon, Intel Concern: Nokia, LG | From moderator perspective, sub-issue 2.2 still can be discussed first, at least for Rel-17 UL MTRP |
| 2.2 | Reuse Rel-17 TCI-specific UL PC parameter setting (including PLRS, and per-PUSCH/PUCCH/SRS P0, alpha, CL index) to support per panel/TRP power control | Support: Ericsson, Docomo, OPPO, vivo, Futurewei, Xiaomi, Lenovo, MTK, LGE, Fujitsu, CATT, Apple, Nokia, NEC, TransHold, Intel Concern: | Given the majority view on this issue, Proposal 2.A is recommended at least for Rel-17 UL MTRP. How to handle the case if the indicated joint or UL TCI states for S-DCI based UL MTRP are not associated with power control settings can be further discussed |
| 2.4 | Tx power limitation for simultaneous UL transmission | Study per-panel power limit - Support: Nokia, OPPO, Docomo, Huawei, ZTE, Qualcomm (per-TRP), vivo (LS to RAN4), CATT, Spreadtrum, LGE, Lenovo, CMCC, Apple, NEC, TransHold - Concern: Study total power limit shared by two panels - Support: Huawei, CATT, CMCC, Spreadtrum, Apple, Intel, NEC, OPPO, ZTE, LG, TransHold - Concern: |  |
| 2.5 | Send LS to RAN4 to check the feasibility for above two cases | Support: vivo, Apple, Lenovo, QC, CMCC Concern: |  |

Proposal 2.B: Send LS to RAN4 to check the feasibility of the followings:

- Power limitation per-panel for simultaneous UL transmission across multiple UE panes

- A total power limitation that is shared by multiple UE panels used for simultaneous UL transmission

Table 4 Additional inputs for Issue 2

| Company | Input |
| --- | --- |
| Mod V0 | Please share your inputs on sub-issue 2.5 and Proposal 2.B |
| Samsung | The reason to send LS is not clear, since many companies share view that both scenario could be studied in RAN1. Unless strong concerns are found in either scenario, we may not need to send LS to RAN4. But we would respect majority views. |
| Ericsson | OK. |
| Futurewei | Fine with FL proposal. |
| Apple | Support FL proposal. As explained in the 1st round, the per-panel power control option (i.e., the 1st sub-bullet) is feasible on condition that RAN4 plans to define a new panel-specific maximum transmission power, which is NOT available in current RAN4 FR2 spec. |
| Xiaomi | Support proposal 2.B. |
| DOCOMO | Fine with FL proposal. |

- Issue 3 – Beam reporting and beam failure recovery

Open issues on beam reporting and BFR enhancements and company views are summarized below.

Table 5 Summary for Issue 3

| # | Issue | Companies’ views | Moderator notes/observation |
| --- | --- | --- | --- |
| 3.1 | Enhance/extend group-based reporting to support simultaneous UL transmission | Support: Qualcomm, Samsung, vivo, MTK, Nokia, Xiaomi, ZTE, Huawei Concern: Apple, Ericsson, OPPO | This issue can be discussed once any Rel-18 MTRP scheme for simultaneous UL transmission is agreed |
| 3.2 | Enhance/extend Rel-17 UE capability index reporting to support simultaneous UL transmission | Support: Samsung, Nokia, CATT, LGE, MTK, AT&T, QC, Apple, Docomo, Lenovo, Xiaomi, OPPO, ZTE, Huawei Concern: Ericsson | This issue can be discussed once any Rel-18 MTRP scheme for simultaneous UL transmission is agreed |
| 3.3 | Enhancement to TRP-specific BFR under unified TCI framework | Support: InterDigital, vivo, Samsung, Apple, Qualcomm, Docomo, Nokia, NEC, Lenovo, Xiaomi, ZTE, Spreadtrum, TransHold, Huawei Concern: Ericsson |  |

Table 6 Additional inputs for Issue 3

| Company | Input |
| --- | --- |
| Mod V0 | Please check and update your views in Table 5 |
| QC | We are also fine for 3.2 with table updated |
| Apple | For beam report, in our view, current group based beam report cannot be reused, as it cannot provide enough information for simultaneous transmission. |
| Samsung | We don’t see strong association between supported STxMP tx schemes and beam management for STxMP. But O.K. to focus on other issues first before we tread this one. |
| Docomo | Added our views in the table. |
| Nokia | Added our views in the table. |
| NEC | We support to study issue#3.3. |
| Ericsson | We are OK to study 3.1 and 3.2 in AI 9.1.4.1. (In our view, 3.1 is needed, 3.2 is not needed). 3.3 is out of scope of the WI. |
| Lenovo | Added our views in the table. |
| vivo | Current description of 3.1 and 3.2 is not clear. Details should be provided for us to make decision. Additionally, we wonder whether 3.1 and 3.2 are exclusive? For example, can extending Rel-17 UE capability correspondence reporting work in group-based beam reporting? We suggest further study on this issue. |
| Futurewei | Our view is that these issues should be treated with lower priority than those listed under Issues 1 and 2. |
| CATT | We think how to facilitate gNB to know the association of panels and beams should be studied. 3.2 can be studied as a start point. |
| Xiaomi | Added our view in the table, same view with Vivo that 3.1 and 3.2 are two approaches to solve this issue. The use of the index of UE capability value set can be a starting point to facilitate the simultaneous multi-panel transmission. We think this issue is important for STxMP, and we are fine to either discuss here or in AI 9.1.4.1. |
| OPPO | Add our concern on Issue 3.1 that group-based reporting is based on DL operation in which UE may apply single Rx beam for receiving two DL beams. As for STxMP, such group-based reporting cannot be simply feasible for UL transmission. On Issue 3.2, we think it’s time to remove the artificial constraint on UE capability value set reporting. But we are fine to hold a while until there are progresses on STxMP in other AI. |
| ZTE | Please review our position in the above table. Generally speaking, we tend to agree with Samsung. We can first focus on the discussion of issue 1 and 2, and then consider issue 3 after they are stable. |
| QC | Among all 3 issues, we suggest to prioritize issue 3.1, which is needed for simultaneous UL beam Tx. - To Apple/OPPO, agree the current group-based report cannot be reused. Our understanding is that the issue 3.1 is to investigate how to make it work - To E///, we think the beam reporting issue should be treated in 9.1.1.1, since 9.1.4.1 is mostly for non-beam related issues, e.g. precoder as in the WID |
| Fujitsu | In our view, beam reporting should at least be able to distinguish STxMP scheme and panel selection/TDM-based scheme. In that sense, we are also open to the beam reporting 3.1 and 3.2. |
| Spreadtrum | We support 3.3 with table updated. Same view as vivo, we think the scheme in issue 3.1 and 3.2 could be described in detail before we have further discussion. |
| CMCC | At this stage, both 3.1 and 3.2 can be viewed as potential way to facilitate simultaneous UL transmission. For issue 3.3, we support to study it in this AI. |
| Transsion | Added our views in the table. |
| Samsung | We support 3.3 BFR enhancements. We think it is within the scope. |
| Huawei, HiSilicon | 3.1: Support. 3.2: Just to clarify, does the “UE capability correspondence reporting” refers to the capability value based reporting? If so, then we can support it. [Mod] Yes. Wording is revised to avoid confusion. 3.3: Ok to support. |
| Intel | 3.1 can be studied. Others are of lower priority and should be discussed after STxMP schemes are discussed in 9.1.4.1. Ideally 3.2 should be discussed in 9.1.4.1. |
| Mod V1 | Revised wording for sub-issues 3.1 and 3.2 to avoid confusion |
| Samsung | As response to Ericsson, for further clarification, TRP specific BFR enhancement in Rel-18 should focus on the possible issues raised by extending unified TCI framework to cover MTRP. So the procedure(s) of BFR not related to beam indication/update under MTRP unified TCI framework would be excluded in the discussion. In addition, please be noted that with unified TCI, most beam management or related operations should be associated with TCI state, and operation per TCI state should be the baseline (and potential BFR enhancements can be progressed along with the progress of issue 1 beam indication/update). While in current design, without modification, BFR would be the only exception. I hope this explanation can verity why TRP specific BFR can be discussed within the scope. |

- Other potential issues

Table 7 Inputs for other potential issues

| Company | Input |
| --- | --- |
| Mod V0 | Please share your view if there is any open issue that need to be addressed with high priority but is not captured above |

- Appendix A: Agreements in RAN1#109-e

Void

- References

- RP-213598	New WID: MIMO Evolution for Downlink and Uplink			Samsung

- R1-2203887	Views on unified TCI extension focusing on m-TRP			xiaomi

- R1-2203793	Unified TCI framework extension for multi-TRP				Sony

- R1-2203723	Consideration on Unified TCI framework for multi-TRP			OPPO

- R1-2203953	Unified TCI framework extension for multi-TRP				Sony

- R1-2204033	Unified TCI framework extension for multi-TRP				Ericsson

- R1-2204229	Views on unified TCI framework extension for multi-TRP			Apple

- R1-2204367	Discussion on unified TCI framework extension for multi-TRP		NTT DOCOMO, INC

- R1-2204141	Unified TCI framework extension for multi-TRP/panel			LG Electronics

- R1-2204162	Discussion of unified TCI framework for multi-TRP			Lenovo

- R1-2203681	Discussion on unified TCI framework extension for multi-TRP		NEC

- R1-2203541	Views on unified TCI framework extension for multi-TRP			vivo

- R1-2203378	On Extension of Unified TCI Framework				InterDigital, Inc.

- R1-2203441	On unified TCI framework extension for multi-TRP operation		CATT

- R1-2203149	Discussion on unified TCI framework extension for multi-TRP		Huawei, HiSilicon

- R1-2203061	Unified TCI framework extension for multi-TRP				FUTUREWEI

- R1-2203320	Discussion on Unified TCI framework extension for multi-TRP		Spreadtrum Communications

- R1-2203174	Discussion on Unified TCI framework extension for multi-TRP		CEWiT

- R1-2203263	Enhancements on unified TCI framework extension for multi-TRP		ZTE

- R1-2205071	Discussion on unified TCI framework extension for multi-TRP		Asia Pacific Telecom co. Ltd

- R1-2205074	Considerations on unified TCI for mTRP				Fujitsu Limited

- R1-2204785	On Unified TCI framework for mTRP					Intel Corporation

- R1-2204678	Multi-TRP enhancements for the unified TCI framework			Fraunhofer IIS, Fraunhofer HHI

- R1-2204857	Unified TCI framework extension for multi-TRP				AT&T

- R1-2205014	Extension of unified TCI framework for mTRP				Qualcomm Incorporated

- R1-2204584	Enhancement on unified TCI framework for multi-TRP			Transsion Holdings

- R1-2204538	Unified TCI framework extension for multi-TRP				Nokia, Nokia Shanghai Bell

- R1-2204684	Unified TCI framework extension for multi-TRP				MediaTek Inc.

- R1-2204287	Discussion on unified TCI framework extension for multi-TRP		CMCC

- R1-2204506	Unified TCI framework extension for multi-TRP				Sharp

- R1-2204440	Discussion on unified TCI framework extension for multi-TRP		ITRI