3GPP TSG RAN WG1 #109-e                                                                                  R1-220xxxx

e-Meeting, May 9th – 20th, 2022

Source:	Moderator (Intel Corporation)

Title:	Summary#x of AI: 9.9.1 NR PDCCH reception in symbols with LTE CRS REs

Agenda item:	9.9.1

Document for:	Discussion and Decision

# Introduction

In RAN#94-e meeting a work item on enhanced MIMO support was agreed for Rel-18 [1]. The objectives of the WID include enhancements to NR PDCCH reception as shown below:

The following objectives shall be included for improvement of NR spectrum efficiency for LTE-NR co-existence (RAN1): • Study and if needed specify NR PDCCH reception in symbols with LTE CRS REs. [RAN1]

This document contains summary of the companies’ and moderator’s proposals.

# Extended email discussion

## EVM clarifications and template for results collection

In order to agree on a template we need a set of common scenarios and KPIs. I have collected the following clarifications for the evaluation assumptions and KPIs/others. Please comment on (at least)

- whether the configurations for baseline and Rel-18 enhancements are agreeable (columns 2 and 3)

- column 4 is derived from what we agreed as example KPIs/Others in SLS assumptions. pls. comment if certain KPIs can be mandated or dropped.

Proposal#1:

| Scenario | Baseline | Rel-18 enhancement | Companies to report one or more of following (examples): |
| --- | --- | --- | --- |
| #1A | s0: reserved for LTE s1: reserved for LTE s2: NR PDCCH (any NR UE) s3-13: reserved for NR PDSCH | s0: reserved for LTE s1: NR PDCCH (only Rel18 UE) s2: NR PDCCH (any NR UE) s3-13: reserved for NR PDSCH | 1. NR PDCCH capacity improvement of Rel-18 enhancement compared to baseline 2. NR PDCCH coverage/outage due to Rel-18 enhancement 3. Potential degradation of LTE due to Rel-18 enhancement compared to baseline 4. Whether and how to achieve coexistence with legacy UEs 5. Assumptions on: Fraction of LTE UEs, Receiver |
| #2 | s0: reserved for LTE s1: reserved for LTE s2: NR PDCCH (any NR UE) s3-13: reserved for NR PDSCH | s0: reserved for LTE s1+s2: NR PDCCH (only Rel18 UE) s2: NR PDCCH (any NR UE) s3-13: reserved for NR PDSCH | 1. NR PDCCH capacity improvement of Rel-18 enhancement compared to baseline 2. NR PDCCH coverage/outage due to Rel-18 enhancement 3. Potential degradation of LTE due to Rel-18 enhancement compared to baseline 4. Whether and how to achieve coexistence with legacy UEs 5. Assumptions on: Fraction of LTE UEs, Receiver |
| #3 | s0: reserved for LTE s1: reserved for LTE s2-3: NR PDCCH (NR UE FG22-12) s4-13: reserved for NR PDSCH | s0: reserved for LTE s1+s2+s3: NR PDCCH (only Rel18 UE) s2+s3: NR PDCCH (NR UE FG22-12) s4-13: reserved for NR PDSCH | 1. NR PDCCH capacity improvement of Rel-18 enhancement compared to baseline 2. NR PDCCH coverage/outage due to Rel-18 enhancement 3. Potential degradation of LTE due to Rel-18 enhancement compared to baseline 4. Whether and how to achieve coexistence with legacy UEs 5. Assumptions on: Fraction of LTE UEs, Receiver |

| Company | Comment |
| --- | --- |

## Possible implementation options:

Possible implementation options that may be agreeable- I believe the focus is primarily on the UE side (e.g., impact to channel estimation, PDCCH rate-matching etc.) but pls. feel free on share your views.

| Company | Comment |
| --- | --- |

# References

[1] RP-213575, New WI: Enhancement of NR Dynamic spectrum sharing (DSS), Ericsson, 3GPP TSG RAN Meeting #94e, Electronic Meeting, Dec. 6 - 17, 2021.

[2] R1-2203137, Discussion on NR PDCCH reception in symbols with LTE CRS REs	Huawei, HiSilicon

[3] R1-2203210	Discussion on NR PDCCH reception for DSS	ZTE

[4] R1-2203344	Discussion on NR PDCCH reception in symbols with LTE CRS Res	Spreadtrum Communications

[5] R1-2203581	Discussion on PDCCH reception on CRS symbol	vivo

[6] R1-2203648	Evaluation of NR PDCCH overlapping with LTE CRS	InterDigital, Inc.

[7] R1-2203834	Discussion on NR PDCCH reception in symbols with LTE CRS REs	xiaomi

[8] R1-2203923	Considerations on PDCCH receptions in symbols with LTE CRS	Samsung

[9] R1-2204024	Discussion on NR PDCCH reception in symbols with LTE CRS REs	OPPO

[10] R1-2204260	Discussion on NR PDCCH reception in symbols with LTE CRS REs	Apple

[11] R1-2204323	Discussion on NR PDCCH reception in symbols with LTE CRS REs	CMCC

[12] R1-2204395	Discussion on NR PDCCH reception in symbols with LTE CRS REs	NTT DOCOMO, INC.

[13] R1-2204630	Discussion on NR PDCCH reception in symbols with LTE CRS REs	LG Electronics

[14] R1-2204709	Discussion on NR PDCCH reception in symbols with LTE CRS REs	MediaTek Inc.

[15] R1-2204815	Discussion on NR PDCCH reception in DSS	Intel Corporation

[16] R1-2204823	NR PDCCH overlapping with LTE CRS	Nokia, Nokia Shanghai Bell

[17] R1-2204885	NR PDCCH reception in symbols with LTE CRS REs	Ericsson

[18] R1-2205049	NR PDCCH reception in symbols with LTE CRS REs	Qualcomm Incorporated

# Appendix (Summary of the agreements)

The agreements made in RAN1#TBD meetings are provided below.