3GPP TSG RAN WG1 #110-bis-e	R1-2208379

Electronic Meeting, October 10 – 19, 2022

Agenda Item:	9.13.2

Source:	Futurewei

Title:	Low Power WUS Receiver Architectures Considerations and Modeling

Document for:	Discussion/Decision

# Introduction

A new study item on low-power wake-up signal and receiver for NR was approved in RAN#94e [1] and revised in RAN#97e [2]. The study item includes the following objectives:

- Identify evaluation methodology (including the use cases) & KPIs [RAN1]

  - Primarily target low-power WUS/WUR for power-sensitive, small form-factor devices including IoT use cases (such as industrial sensors, controllers) and wearables

    - Other use cases are not precluded

- Study and evaluate low-power wake-up receiver architectures [RAN1, RAN4]

- Study and evaluate wake-up signal designs to support wake-up receivers [RAN1, RAN4]

- Study and evaluate L1 procedures and higher layer protocol changes needed to support the wake-up signals [RAN2, RAN1]

- Study potential UE power saving gains compared to the existing Rel-15/16/17 UE power saving mechanisms, and the coverage availability, as well as latency impact of low-power WUR/WUS. System impact, such as network power consumption, coexistence with non-low-power-WUR UEs, network coverage/capacity/resource overhead should be included in the study [RAN1]

  - Note: The need for RAN2 evaluation will be triggered by RAN1 when necessary.

In this contribution, an examination of receiver architectures of power consumption ≤140 μW is performed. The trade-offs between power consumption, sensitivity, and selectivity/interference rejection capability are discussed. Further, receiver models to be considered for tradeoff characterization and performance evaluation of wake-up signal designs are proposed.

# Examination of Low-Power Receiver Architectures

In [3], a dedicated low-power receiver, a wake-up radio (WUR), is proposed as a supplement to a main radio of a UE to alleviate the power consumption associated with the current need of UEs to periodically wake up once per DRX cycle to monitor PDCCH. Figure 1 shows the trade-offs between receiver power consumption, sensitivity, and supported data rate for two carrier frequency ranges, fc≤1 GHz and 1 GHz <fc≤3 GHz, based on the receiver architectures surveyed in [4]. The figure suggests that receiver architectures consuming power of 40 μW<Pc≤140 μW can support sensitivity levels -97 dBm<Pmin≤-70 dBm at data rates 10 kbps≤R<200 kbps using non-coherent OOK modulation. In the following sections, a few of those receiver architectures are examined. In general, examined low-power receiver architectures can be categorized as mixer-first architectures, such as the uncertain-IF, the sub-sampling, and the dual uncertain-IF architectures; and envelope detection first architectures, such as the double-sampling and the 2-tone reception architectures.

Figure 1: Survey of Low-Power Receiver Architectures [4].

## Uncertain IF Architecture

The uncertain IF architecture is one of the architectures that attempts to reduce or eliminate the power consumption overhead associated with the front-end (FE) Phase Locked Loops (PLLs) and Low Noise Amplifiers (LNAs). This is achieved through the utilization of (1) a low-power and low-accuracy unlocked local oscillators (LOs) such as the ring oscillators, and (2) LNAs at IF instead of RF. The power consumption overhead associated with LNAs can further be eliminated by entirely discarding LNAs from the architecture at the expense of receiver sensitivity. An example uncertain IF architecture [5] is shown in Figure 2 where receiver selectivity, i.e., blockers elimination, is achieved through the utilization of passive high-Q front-end filters with additional filtering after the mixer, which is easier provided at lower frequencies. The architecture in [5] provides a -3dB bandwidth of 54 MHz through RF filtering while the IF bandwidth is limited by the utilized ring oscillator uncertainty.

Figure 2: An Example of Uncertain IF Receiver Architecture [5].

Therefore, in this architecture, sensitivity is limited, in general, by the integrated noise presented by the wide IF bandwidth required to deal with the LO uncertainty. In [5], a sensitivity of -88 dBm for 10-3 BER is achieved using a Manchester encoded (information bits are encoded as transitions from low-to-high or high-to-low signal levels) data rate of 250 kbps at a power consumption of ~50 μW in the 2.45 GHz band.

Observation 1: In-band selectivity of the uncertain IF receiver architecture is limited by wide IF bandwidth required to deal with the LO uncertainty.

## Double-Sampling (Synchronized Switching) Architecture

The, originally termed, double-sampling architecture is another architecture that attempts to reduce the power consumption overhead associated with the FE PLLs through the utilization of low-frequency oscillators that are 1 to 2 orders of magnitude below target RF frequency. The architecture also mitigates the impact of the 1/f (flicker) noise through the combination of the chopping/switching stage at RF, double-sampling/switching stage at IF, and utilization of a clock frequency above the flicker noise corner frequency. An example double-sampling architecture [6]-[7] is shown in Figure 3 where the IF BPF stage may be followed by an amplification stage. Since RF envelope detection is utilized in this architecture, receiver selectivity is mainly controlled by the RF FE filters.

In [6], FE selectivity is compromised, i.e., a -3dB bandwidth of 21MHz/59MHz in the 915MHz/2.4GHz band, for the low power consumption of ~51 μW and the receiver architecture achieves a sensitivity of -75dBm/-80dBm using a data rate of 100kbps/10kbps in the 915MHz band. In [7], the receiver architecture provides a FE -3dB bandwidth of 110 MHz, that is determined by the LNA and the input matching network, and achieves a sensitivity of -86.5dBm/-61dBm using a data rate of 10kbps for a power consumption of 146 μW/64 μW in the 780-950 MHz bands (a data rate of 100 kbps is supported at ~5dB degradation in sensitivity). However, the receiver selectivity may be improved to a -3dB bandwidth of only 13 MHz using a high-Q RF SAW filter at the expense of a ~2 dB degradation in sensitivity. Further power consumption reduction for the receiver architecture in [7] may be achieved by discarding LNAs at RF at the expense of further degradation in receiver sensitivity.

Figure 3: An Example of Synchronized Switching/Double-Sampling Receiver Architecture [6]-[7].

Observation 2: Interference rejection and In-band selectivity of the synchronized switching receiver architecture is limited by RF FE band-pass filtering capability due to envelope detection at RF.

## Sub-Sampling Architecture

Like the double-sampling architecture, the sub-sampling architecture attempts to reduce the power consumption overhead associated with the FE PLLs through the utilization of low-frequency oscillators that are 1 to 2 orders of magnitude below target RF frequency. However, instead of the utilized switching/chopping technique in the double-sampling architecture, i.e., multiplying the received RF signal with a square wave of low frequency, the sub-sampling architecture in [8] uses the low frequency clock to sub-sample the received RF signal and generate a signal at IF. Further, the receiver architecture in [8] still utilizes the uncertain IF topology, i.e., utilizes a low-power and low-accuracy reference clock, but improves receiver selectivity through the utilization of a period-based calibration circuit.

An example sub-sampling architecture [8] is shown in Figure 4 where the receiver selectivity is determined by a SAW filter and a two active-inductor based amplifier stages providing ~13 MHz of bandwidth. The architecture in [8] achieves a sensitivity of -75dBm using a Manchester encoded data rate of 200 kbps for a power consumption of ~22.9 μW (calibration circuit may on average consume 0.3μW for 1 ms per 100 ms calibration) in the 915 MHz band.

Figure 4: An Example of Sub-Sampling Receiver Architecture [8].

Observation 3: In-band selectivity of the sub-sampling & uncertain IF receiver architecture is limited by wide IF bandwidth required to deal with the LO uncertainty, which can be improved by a period-based oscillator calibration circuit.

## 2-Tone Reception Architecture

Like the double-sampling architecture, the architecture in [9] and shown in Figure 5 utilizes RF envelope detection and low-frequency oscillators for power consumption reduction. However, instead of the utilized switching/chopping technique in the double-sampling architecture, i.e., multiplying the received RF signal with a square wave of low frequency, the architecture in [9] uses a 2-tone transmission scheme. Further, the architecture treats the double-sampling/switching stage at IF, i.e., after envelope detection, as a mixing stage and utilizes a FE SAW filter to improve the receiver’s interference rejection capability.

The specific signal design where a 2-tone transmission scheme is considered allows the use of BPSK-IF as a modulation scheme for a non-coherent envelope detection-based receiver architecture. It also improves the receiver selectivity for better in-band interference rejection. The architecture in [9], therefore, manages out-of-band interference rejection through the SAW filter and in-band interference rejection through signal design and IF BPF after envelope detection. It achieves a sensitivity of -83dBm/-56dBm using a data rate of 10 kbps for a power consumption of ~121 μW/63.5 μW (+10 μW for IF clock generation) in the 915 MHz band. The sensitivity of this architecture is similar to the double-sampling architecture in [7] when accounting for the losses due to the SAW filter. However, it provides a much better interference rejection than the double-sampling architecture as it can tolerate between -19 dB to -10.5 dB of in-band carrier-to-interference ratio (CIR) at ± 1 MHz offset from each tone based on power consumption.

Figure 5: An Example of a 2-Tone Reception Envelope Detection Receiver Architecture [9].

Observation 4: Interference rejection and In-band selectivity of an RF envelope detection (double-sampling) receiver architecture can be improved using a 2-tone transmission scheme/signal design.

## Dual Uncertain IF Architecture

The dual uncertain-IF receiver architecture in [10], represented in Figure 6, reuses the uncertain-IF receiver architecture to reduce power consumption while improving the receiver’s selectivity by combining an unlocked low-Q resonator-referred LO (LC-DCO), where LC-DCO provides more accuracy than ring oscillators at the cost of a slight increase in power consumption, and distributed multi-stage high-Q N-path passive mixer (N-PPM) filtering technique.

The dual uncertain-IF architecture selectivity is then provided by two main narrow band-pass filtering stages, one at each of the two IF frequencies, enabling a tolerance of in-band carrier-to-interference ratio (CIR) between -25 dB to -22 dB at ± 3 MHz offset. The FE matching network and RF passive mixer provide an effective bandwidth of 20 MHz while the first IF passive mixer provides an effective bandwidth of 1 MHz. The architecture achieves a sensitivity of -97dBm/-92dBm using a data rate of 10kbps/50kbps for a power consumption of ~99 μW in the 2.4 GHz band.

Figure 6: A Representation of the Dual Uncertain-IF Receiver Architecture [10].

Envelope detection in the dual uncertain-IF architecture utilizes the high linearity response of the N-PPM to perform direct down-conversion of the signal from the second IF frequency to DC, ensuring bandwidth reduction and removal of the LO uncertainty effects.

Observation 5: Interference rejection and In-band selectivity of a mixer-first (uncertain-IF) receiver architecture can be improved using a combination of LC-DCO and distributed high-Q N-PPM filtering at the cost of increase in power consumption.

## Summary

The following table summarizes the key performance metrics of low-power receiver architectures as examined in the previous sections including power consumption, sensitivity (per certain data rate), and interference rejection/selectivity with corresponding effective bandwidth.

Table 1. Summary of Examined Low-Power Receiver Architectures.

|  | Mixer-First Architectures |  |  | Envelope Detection-First Architectures |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Uncertain IF[5] | Sub-Sampling & Uncertain IF[8] | Dual Uncertain IF[10] | Double-Sampling[6] |  | 2-Tone Reception[9] |
| LO generation | Unlocked 3-Stage Ring Oscillator | Unlocked 3-Stage Ring Oscillator † | Unlocked LC-DCO | Unlocked 3-Stage Ring Oscillator |  | Crystal Oscillator |
| Modulation | OOK | OOK | OOK | OOK |  | PSK-IF/OOK-IF |
| Frequency | 2.45 GHz | 915 MHz | 2.4 GHz | 915MHz | 2.4GHz | 915 MHz |
| Power Consumption | 50 uW | ~22.9 uW | 99 uW | 51 uW |  | 63.5 to 121 uW ** |
| Data Rate | 250 kbps (Man) | 200/10 kbps (Man) | 10/50 kbps | 100/10kbps |  | 10 kbps |
| Sensitivity (dBm) | -88 | -75/-78.5 | -97/-92 | -75/-80 | -64/-69 | -56 to -83 |
| Interferer Rejection/ Selectivity | RF N-PPM based filter | SAW filter | Multi-stage N-PPM based filters | FE Matching Network |  | SAW filter and 2-tone BPSK-IF |
| Effective Bandwidth | FE -3dB BW of 54 MHz | IF -3dB BW of 13 MHz | IF BW of 1 MHz | FE -3dB BW |  | ~<4 MHz |
|  |  |  |  | 21 MHz | 59 MHz |  |
| CIR* (dB) | N/A | N/A | -25/-22 @ +/-3 MHz (10 kbps) | N/A |  | -19 to -10.5 @ ±1MHz from each tone |
| Duty Cycling | No | No | Yes (18 nW Leakage) | No |  | No |

* CIR = Pcarrier, min sensitivity+3 dBm/Pinterferer [dB]

** excluding clock generation that consumes ~10μW

† 17MHz crystal is used for period-based calibration

Comparing these architectures, some further observations on the trade-offs amongst the key performance metrics can be drawn:

Observation 6: Low-power receiver architectures of power consumption around 100μW such as dual uncertain-IF and 2-tone reception show reasonable sensitivity and good selectivity with effective bandwidths <4 MHz.

Observation 7: Low-power receiver architectures of power consumption <60μW such as uncertain-IF, sub-sampling, and double-sampling may show reasonable sensitivity but at poor selectivity with effective bandwidths >10 MHz.

It is also observed that there are various trade-offs amongst the key performance metrics and that those trade-offs may depend on the receiver architecture as well as the associated transmission scheme/signal design. A proper trade-off may be defined/identified based on the KPI requirements as specified from system perspective.

With the assessment of the candidate architectures, we have the following proposal.

Proposal 1: the study of the modeling of the wake-up receiver should

- Consider the KPIs from system perspectives

- Decide whether a single or a small number of models should be adopted

- Reflect via a set of parameters the general design aspects of the receiver, including at least power consumption, sensitivity, and interference rejection / selectivity

- Study the valid value range of the receiver parameter set and select a small number of sets (with different tradeoffs) for performance evaluation and signal design

# Low-Power Receiver Modeling

A general low-power wake-up receiver (WUR) and signal design evaluation model is proposed in Figure 7. The WUR model consists of filtering, envelope detection, analog-to-digital conversion (ADC), and digital signal processing blocks. The proposed model can be adapted to capture the behavior of any of the architectures discussed in this contribution. The filtering block at the beginning of the receiver chain can be used to capture the selectivity, i.e., in-band interference rejection, of the modeled WUR architecture. The envelope detection block may have multiple alternatives, depending on the modeled WUR architecture, such as

- A square-law detector followed by a LPF to capture the behavior of WUR architectures such as the uncertain-IF and the sub-sampling architectures;

- A Mixer followed by a LPF to capture the behavior of direct down-conversion WUR architectures such as the dual uncertain-IF architecture; or

- A square-law detector, a BPF, a mixer, and finally followed by a LPF to capture the behavior of WUR architectures such as the double-sampling and the 2-tone reception architectures.

The ADC block can be a simple comparator or a multi-bit ADC. An oversampling ratio, i.e., number of samples to be evaluated per information bit duration, may be considered during the signal design evaluation. The DSP block can be used to perform any bit detection, correlation, or message parsing operations before sending a wake-up (WU) indication and/or forward any messages to the main radio. The mapping between the evaluated SNR (SNRmodel) and the actual expected SNR (SNRArch) is based on an architecture specific noise figure (NFArch).

Figure 7: Wake-Up Receiver and Signal Design Evaluation Model.

An interfering signal can be optionally introduced into the model to evaluate the LP-WUS co-existence with other signals in the same band. Different combinations of modeling block configurations may be considered to compare and differentiate between few of the most promising LP-WUR architectures.

Proposal 2: A general LP-WUR model can consist of a filtering block for selectivity, an envelope detection block for non-linearity characterization, a single/multi-bit ADC block, and a DSP block for sequence correlation or message parsing.

# Conclusion

The contribution examined few low-power receiver architectures, proposed a WUR and signal design evaluation model, and made the following observations.

Observation 1: In-band selectivity of the uncertain IF receiver architecture is limited by wide IF bandwidth required to deal with the LO uncertainty.

Observation 2: Interference rejection and In-band selectivity of the synchronized switching receiver architecture is limited by RF FE band-pass filtering capability due to envelope detection at RF.

Observation 3: In-band selectivity of the sub-sampling & uncertain IF receiver architecture is limited by wide IF bandwidth required to deal with the LO uncertainty, which can be improved by a period-based oscillator calibration circuit.

Observation 4: Interference rejection and In-band selectivity of an RF envelope detection (double-sampling) receiver architecture can be improved using a 2-tone transmission scheme/signal design.

Observation 5: Interference rejection and In-band selectivity of a mixer-first (uncertain-IF) receiver architecture can be improved using a combination of LC-DCO and distributed high-Q N-PPM filtering at the cost of increase in power consumption.

Observation 6: Low-power receiver architectures of power consumption around 100μW such as dual uncertain-IF and 2-tone reception show reasonable sensitivity and good selectivity with effective bandwidths <4 MHz.

Observation 7: Low-power receiver architectures of power consumption <60μW such as uncertain-IF, sub-sampling, and double-sampling may show reasonable sensitivity but at poor selectivity with effective bandwidths >10 MHz.

With the assessment of the candidate architectures, we have the following proposals.

Proposal 1: the study of the modeling of the wake-up receiver should

- Consider the KPIs from system perspectives

- Decide whether a single or a small number of models should be adopted

- Reflect via a set of parameters the general design aspects of the receiver, including at least power consumption, sensitivity, and interference rejection / selectivity

- Study the valid value range of the receiver parameter set and select a small number of sets (with different tradeoffs) for performance evaluation and signal design

Proposal 2: A general LP-WUR model can consist of a filtering block for selectivity, an envelope detection block for non-linearity characterization, a single/multi-bit ADC block, and a DSP block for sequence correlation or message parsing.

# References

RP-213645, “New SID: Study on low-power Wake-up Signal and Receiver for NR”, Vivo, RAN#94e, Dec. 6-17, 2021

RP-222644, “Revised SID: Study on low-power Wake-up Signal and Receiver for NR”, Vivo, RAN#97e, Sept. 12-16, 2022

RWS-210169, “New SID: Study on ultra-low power wake up signal in Rel-18”, Vivo, RAN Rel-18 workshop, June 28 – July 2, 2021.

David D. Wentzloff, “Low Power Radio Survey,” [Online]. www.eecs.umich.edu/wics/low_power_radio_survey.html.

C. Bryant, et.al, “A 2.45GHz 50uw wake-up receiver front-end with -88dBm sensitivity and 250kbps data rate”, ESSCIRC, 2014.

X. Huang, S. Rampu, X. Wang, G. Dolmans and H. de Groot, "A 2.4GHz/915MHz 51µW wake-up receiver with offset and noise suppression," 2010 IEEE International Solid-State Circuits Conference - (ISSCC), 2010, pp. 222-223, doi: 10.1109/ISSCC.2010.5433958.

X. Huang, P. Harpe, G. Dolmans, H. de Groot and J. R. Long, "A 780–950 MHz, 64–146 µW Power-Scalable Synchronized-Switching OOK Receiver for Wireless Event-Driven Applications," in IEEE Journal of Solid-State Circuits, vol. 49, no. 5, pp. 1135-1147, May 2014, doi: 10.1109/JSSC.2014.2307056.

S. Moazzeni, M. Sawan and G. E. R. Cowan, "An Ultra-Low-Power Energy-Efficient Dual-Mode Wake-Up Receiver," in IEEE Transactions on Circuits and Systems I: Regular Papers, vol. 62, no. 2, pp. 517-526, Feb. 2015, doi: 10.1109/TCSI.2014.2360336.

X. Huang, A. Ba, P. Harpe, G. Dolmans, H. de Groot and J. R. Long, "A 915 MHz, Ultra-Low Power 2-Tone Transceiver With Enhanced Interference Resilience," in IEEE Journal of Solid-State Circuits, vol. 47, no. 12, pp. 3197-3207, Dec. 2012, doi: 10.1109/JSSC.2012.2216706.

C. Salazar, A. Cathelin, A. Kaiser and J. Rabaey, "A 2.4 GHz Interferer-Resilient Wake-Up Receiver Using A Dual-IF Multi-Stage N-Path Architecture," in IEEE Journal of Solid-State Circuits, vol. 51, no. 9, pp. 2091-2105, Sept. 2016, doi: 10.1109/JSSC.2016.2582509.