3GPP TSG RAN WG1 #110bis-e	R1-2208630

e-Meeting, October 10th – 19th, 2022

Source: 	vivo

Title:	Discussion on SRS enhancement

Agenda Item:	9.1.3.2

Document for:	Discussion and Decision

Introduction

In the RAN#94 meeting, WID [1] on the further evolution of DL and UL MIMO was approved, which includes two objectives relevant to further SRS enhancement. One objective of SRS enhancement is related to TDD CJT for studying inter-TRP cross-SRS interference, and another objective is to enhance SRS to support 8Tx UL operation.

Objective 4: Study, and if justified, specify enhancements of CSI acquisition for Coherent-JT targeting FR1 and up to 4 TRPs, assuming ideal backhaul and synchronization as well as the same number of antenna ports across TRPs, as follows: - Rel-16/17 Type-II codebook refinement for CJT mTRP targeting FDD and its associated CSI reporting, taking into account throughput-overhead trade-off - SRS enhancement to manage inter-TRP cross-SRS interference targeting TDD CJT via SRS capacity enhancement and/or interference randomization, with the constraints that 1) without consuming additional resources for SRS; 2) reuse existing SRS comb structure; 3) without new SRS root sequences - Note: the maximum number of CSI-RS ports per resource remains the same as in Rel-17, i.e. 32 Objective 6: Study, and if justified, specify UL DMRS, SRS, SRI, and TPMI (including codebook) enhancements to enable 8 Tx UL operation to support 4 and more layers per UE in UL targeting CPE/FWA/vehicle/Industrial devices - Note: Potential restrictions on the scope of this objective (including coherence assumption, full/non-full power modes) will be identified as part of the study.

In this contribution, multiple potential schemes for 8 ports SRS for codebook/non-codebook based transmission and antenna switching are discussed. Several potential SRS enhancements on TDD CJT are also provided.

SRS enhancement for UL 8Tx

There are many usages of SRS, such as codebook, non-codebook, antenna switching, and so on. However, in the current NR system, one SRS resource can only support up to 4 ports for codebook based uplink transmission and antenna switching. Besides, there are up to 4 single-port SRS resources in one SRS resource set for non-codebook based uplink transmission. In Rel-18, it has been agreed to support more than 4 layers for CPE/FWA/vehicle/Industrial devices for 8 Tx uplink operation. Therefore, 8 ports SRS should be defined for codebook-based transmission and antenna switching, while 8 single-port SRS resources should be supported for non-codebook-based transmission. In the previous RAN1 meeting, the following agreements were achieved for UL 8Tx [2], and the potential solutions need to be further discussed.

Agreement For SRS enhancements to enable 8 Tx UL operation to support 4 and more layers per UE in UL targeting CPE/FWA/vehicle/Industrial devices, study aspects include, for SRS for CB/NCB/AS, - Design parameters, including the maximum number of SRS resource sets, number of SRS resource sets, number of SRS resources, number of ports per resource, number of OFDM symbols, the allowed configurations for comb / comb shifts / cyclic shifts, number of simultaneous ports / resources / resource sets per OFDM symbol - For the next decision point, study - Whether to support 8 ports in one or multiple resources - Whether to support 8 ports in one or multiple OFDM symbols - The maximum number of SRS resource sets. - Note: For SRS for NCB, number of ports per SRS resource is still 1 (same as R15)

Multiplexing principle of legacy SRS

In design for SRS, it can be classified as a cyclic shift and FDM based solution, since the multiplexing of multiple ports in one SRS resource is achieved by different cyclic shifts and/or frequency positions. The determination of the cyclic shift nSRScs,i for one SRS port can be shown as follows.

where nSRScs is the value of cyclic shift configured by RRC, nSRScs,max is the maximum number of cyclic shifts, pi is the index of one SRS port, NapSRS is the number of ports of the SRS resource. Moreover, the specific frequency position kTCpi of one SRS port can be presented by the following formula.

where KTC is the comb size of one SRS resource and kTC is the offset in the frequency domain configured by RRC. From the above equations, it can be observed that for different numbers of SRS ports and comb structures, the method used to multiplex SRS ports are different in some cases.

When one SRS resource is configured with 2 ports, no matter whether the comb is equal to 2, 4, or 8, SRS ports are only multiplexed by different cyclic shifts. However, in the case of one SRS resource with 4 ports, due to the different comb structures, there are different ways of multiplexing multiple ports. For instance, regarding comb 4, if SRS is configured with a value of the cyclic shift smaller than 6, 4 SRS ports are multiplexed by different cyclic shifts with the same interval. However, if SRS is configured with a value of the cyclic shift equal to or larger than 6, the SRS ports 1001 and 1003 are also  mapped in different frequency positions additionally, which can be configured for the case with larger transmission delay. Regarding comb 8 which is introduced for SRS in Rel-17, 6 cyclic shifts can’t be uniformly distributed with an interval of integer. Therefore, SRS ports 1000 and 1002 (or 1001 and 1003) are multiplexed with two cyclic shifts with the interval as 3 on the same frequency position, and SRS ports 1000 and 1001 (or 1002 and 1003) are mapped in different frequency positions.

Possible solutions for 8 ports SRS for codebook and antenna switching

To enhance SRS with 8 ports for codebook and antenna switching, there are mainly two directions, i.e specifying a SRS resource with 8 ports or 8 port SRS by aggregating multiple SRS resources. In the last meeting, the following agreements were achieved for 8 ports SRS for codebook and 8T8R antenna switching [3].

Agreement For 8 Tx SRS, at least support - 8 ports in 1 SRS resource for ‘antennaSwitching’; - FFS 8 ports in one or multiple SRS resources for ‘codebook’ Above does not imply support for 8 ports in one or multiple OFDM symbols Agreement For the maximum number of SRS resource sets for SRS with 8T8R with ‘antennaSwitching’, keep the existing value of the maximum number of SRS resource sets (as provided in Rel-17 antenna switching nTnR) Agreement For an 8-port SRS resource in an SRS resource set with usage antennaSwitching (i.e., for 8T8R antenna switching), the 8-port SRS resource is transmitted in at least one OFDM symbol. - FFS: the resource transmitted in multiple OFDM symbols where different ports are mapped to different symbols.

One resource based solution with cyclic shift and FDM

To keep the same multiplexing principle for one SRS resource with 8 ports, a natural way is to reuse a similar multiplexing principle of the legacy SRS resource, i.e., cyclic shift and FDM based solution. However, due to the increased number of SRS ports, the interval of cyclic shifts for each SRS port would be smaller than the legacy SRS resource. Therefore, some specific designs might be needed for different comb sizes.

- Comb 2

For comb 2, there are up to 8 cyclic shifts that can be used for SRS ports. When the number of ports in one SRS resource equals 8, one most direct way is to map 8 cyclic shifts to 8 ports, in this case, the interval between two adjacent cyclic shifts equals 1. The detail of the cyclic shift mapping is shown in the table below.

Cyclic shift mapping for SRS resource with 8 ports in comb 2

|  | Port 1000 | Port 1001 | Port 1002 | Port 1003 | Port 1004 | Port 1005 | Port 1006 | Port 1007 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CS 0 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| CS 1 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 0 |
| CS 2 | 2 | 3 | 4 | 5 | 6 | 7 | 0 | 1 |
| CS 3 | 3 | 4 | 5 | 6 | 7 | 0 | 1 | 2 |
| CS 4 | 4 | 5 | 6 | 7 | 0 | 1 | 2 | 3 |
| CS 5 | 5 | 6 | 7 | 0 | 1 | 2 | 3 | 4 |
| CS 6 | 6 | 7 | 0 | 1 | 2 | 3 | 4 | 5 |
| CS 7 | 7 | 0 | 1 | 2 | 3 | 4 | 5 | 6 |

Regarding the mapping on the frequency domain, three potential schemes can be considered as follows.

- Scheme 1: to map these 8 ports on the same frequency position. In this case, the orthogonality between different ports is only guaranteed by associated cyclic shifts.

- Scheme 2: to map port {1000, 1002, 1004, 1006} and port {1001, 1003, 1005,1007} in two different frequency positions.

- Scheme 3: to map two groups in different frequency positions only when SRS resource is associated with a certain value of cyclic shift by RRC. This scheme is the same as what for 4 ports SRS with comb 2.

- Comparing the above three schemes, the main drawback of scheme 1 is that performance degrades significantly when the channel delay is large, due to the small interval between different cyclic shifts. Besides, scheme 3 seems like a combination of scheme 1 and scheme2, which would be more flexible to handle the cases with different channel delays.

For comb 2, 8 different cyclic shifts can be exactly mapped on 8 SRS ports.

For comb 2, performance cannot be guaranteed with mapping 8 SRS ports in the same frequency position when the channel delay is large.

For comb 2, the same port mapping method as for 4 ports SRS can be considered for 8 ports SRS, considering the flexibility.

- Comb 4

For comb 4, there are 12 cyclic shifts available multiplexing 8 ports SRS. However, similar to the case of 4 ports SRS in comb 8, 12 cyclic shifts can’t be divided by 8, leading to unequal intervals among 8 ports. To handle this issue, mapping 2 or 4 different cyclic shifts for 8 ports SRS can be considered. For instance, when 4 different cyclic shifts are assigned for 8 ports SRS, the same solution as 4 ports SRS in comb 8 can be reused to map the cyclic shifts to 8 ports SRS. As shown in the table below, 4 configurable cyclic shifts with the interval of 3 are used for 8 ports.

Cyclic shift mapping for SRS resource with 8 ports in comb 4

|  | Port 1000 | Port 1001 | Port 1002 | Port 1003 | Port 1004 | Port 1005 | Port 1006 | Port 1007 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CS 0 | 0 | 0 | 3 | 3 | 6 | 6 | 9 | 9 |
| CS 1 | 1 | 1 | 4 | 4 | 7 | 7 | 10 | 10 |
| CS 2 | 2 | 2 | 5 | 5 | 8 | 8 | 11 | 11 |
| CS 3 | 3 | 3 | 6 | 6 | 9 | 9 | 0 | 0 |
| CS 4 | 4 | 4 | 7 | 7 | 10 | 10 | 1 | 1 |
| CS 5 | 5 | 5 | 8 | 8 | 11 | 11 | 2 | 2 |
| CS 6 | 6 | 6 | 9 | 9 | 0 | 0 | 3 | 3 |
| CS 7 | 7 | 7 | 10 | 10 | 1 | 1 | 4 | 4 |
| CS 8 | 8 | 8 | 11 | 11 | 2 | 2 | 5 | 5 |
| CS 9 | 9 | 9 | 0 | 0 | 3 | 3 | 6 | 6 |
| CS 10 | 10 | 10 | 1 | 1 | 4 | 4 | 7 | 7 |
| CS 11 | 11 | 11 | 2 | 2 | 5 | 5 | 8 | 8 |

Regarding the mapping on frequency-domain, the similar approach used for 4 ports SRS can be considered as a potential solution, i.e., to map the port {1000, 1002, 1004, 1006} and port {1001, 1003, 1005,1007} in two different frequency positions. Additionally, in the case of mapping 2 different cyclic shifts for 8 ports SRS with the interval of 6, mapping 8 SRS ports in four different frequency positions can also be considered as a potential solution.

For comb 4, a similar approach for 4 ports SRS for cyclic shifts and frequency position mapping can be considered for 8 ports SRS.

- Come 8

For comb 8, only up to 6 cyclic shifts are available for SRS ports multiplexing. However, it is impossible to assign 6 cyclic shifts to 8 ports with an integer interval. Therefore, using two configurable cyclic shifts for 8 ports SRS is a potential solution for this case. The detail of the cyclic shift mapping is shown in the table below.

Cyclic shift mapping for SRS resource with 8 ports in comb 8

|  | Port 1000 | Port 1001 | Port 1002 | Port 1003 | Port 1004 | Port 1005 | Port 1006 | Port 1007 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CS 0 | 0 | 0 | 0 | 0 | 3 | 3 | 3 | 3 |
| CS 1 | 1 | 1 | 1 | 1 | 4 | 4 | 4 | 4 |
| CS 2 | 2 | 2 | 2 | 2 | 5 | 5 | 5 | 5 |
| CS 3 | 3 | 3 | 3 | 3 | 0 | 0 | 0 | 0 |
| CS 4 | 4 | 4 | 4 | 4 | 1 | 1 | 1 | 1 |
| CS 5 | 5 | 5 | 5 | 5 | 2 | 2 | 2 | 2 |

Regarding the frequency-domain mapping, we can consider to map the ports {1000, 1004}, ports {1001, 1005}, ports {1002, 1006} and ports {1003,1007} in four different frequency positions.

For comb 8, 8 ports SRS can be mapped with two configurable cyclic shifts in four different frequency positions.

For codebook based transmission and antenna switching, support cyclic shift and FDM based enhancement on one SRS resource with 8 ports in one OFDM symbol.

One resource based solution with TD-OCC

Apart from cyclic shift and FDM based approach, 8 ports SRS resource can also be enhanced in time-domain, such as TD-OCC based solutions. In this case, more than one OFDM symbol would be occupied by one SRS resource with repetition, which is similar to the multiplexing mechanism of DMRS. However, several key issues should be discussed as follows, if mapping 8 ports based on TD-OCC.

- While increasing the capacity of SRS based on TD-OCC, the coverage performance of SRS transmission would be degraded.

- Collision resolution should be defined when all or partial symbols of SRS resource collide with other uplink signals.

- Additional restriction needs to be introduced to bundle the multiple OFDM symbols together when frequency hopping is enabled.

If 8 SRS ports are multiplexed based on TD-OCC, several key issues should be addressed, such as uplink coverage performance, effect on frequency hopping, and collision resolution.

The feasibility of one 8 ports SRS resource based on TD-OCC should be carefully discussed.

Multiple resources based solution

For codebook-based transmission in the current spec, up to 4 SRS ports per SRS resource can be configured for uplink sounding. To support more than 4 ports for uplink sounding and channel estimation, aggregating SRS ports from multiple SRS resources is also a possible way to achieve 8 SRS ports. For instance, two 4 port SRS resources can be used to aggregate a 8 port SRS. However, this solution requires some restrictions, such as the restriction of time-domain occupation of multiple SRS resources, the consistency of transmission power and phase of multiple SRS resources, and so on.

Additionally, whether multiple SRS resources are from the same SRS set or different SRS sets is another important issue. One feasible way is to reuse the design of Rel-17 MTRP PUSCH, i.e, two SRS resources are from two different SRS resource sets. In this way, the Rel-17 SRI indication can also be reused. Especially for 8Tx non-coherent and partial-coherent codebook which could be constructed by concatenating two current 4Tx codebook subsets, it is feasible to indicate two SRIs corresponding to two SRS resources from two different SRS resource sets.

8 SRS ports for codebook based transmission can be mapped on multiple SRS resources for different SRS resource sets.

For codebook based transmission, support to aggregate 8 ports from two SRS resources from two different SRS resource sets.

Enhancement on SRS for non-codebook

For non-codebook based uplink transmission, UE is configured with multiple single-port SRS resources in a SRS resource set. In the current NR spec, up to four SRS resources in one SRS resource set can be configured. Therefore, to support non-codebook based 8Tx transmission, up to 8 single-port SRS resources should be supported in Rel-18. Two potential solutions can be considered as follows.

- Alt 1: Up to 8 single-port SRS resources in one SRS resource set

- Alt 2: Up to 8 single-port SRS resources from two SRS resource sets

Alt 1 is a straightforward way to extend the maximum number of SRS resources to 8 in one SRS resource set. However, in this way, the SRI indication for uplink non-codebook transmission needs to be enhanced, especially the SRI indication tables in spec TS 38.212, which could be time-consuming. Regarding Alt 2, two SRS resource sets including 4 single-port SRS resources separately would be related for generating the precoder of uplink non-codebook transmission with two SRI indications. One easy way is to use 2 SRI fields introduced in Rel-17 MTRP PUSCH, where the first SRI field corresponds to the first SRS set and the second SRI field corresponds to the second SRS set. Based on Alt 2, the current SRI indication tables in spec TS 38.212 can be reused as much as possible.

For non-codebook based transmission, support up to 8 single-port SRS resources from two SRS resource sets based on two SRI fields.

SRS enhancement for TDD CJT

Coherent joint transmission (CJT) is a potential MTRP-based transmission scheme for massive MIMO. For TDD CJT scenario, SRS would be used to estimate the reciprocal channel information for non-PMI based transmission in downlink. However, as the number of UEs increases in the network, there would be a collision between SRS resources used by different UEs, leading to cross-SRS interference. To address the cross-SRS interference issue of SRS, there were a lot of potential schemes of SRS enhancement for TDD CJT proposed in the previous meeting.

Agreement Study the following for SRS enhancement to manage inter-TRP cross-SRS interference targeting TDD CJT via SRS interference randomization and/or capacity enhancement - Randomized frequency-domain resource mapping for SRS transmission - E.g., further enhancements to frequency hopping, comb hopping - Randomized code-domain resource mapping for SRS transmission - E.g., cyclic shift hopping/randomization, sequence hopping/randomization, per-hop sequence from a long SRS sequence - Randomized transmission of SRS - E.g., pseudo-random muting of SRS transmission for periodic and semi-persistent SRS - Per-TRP power control and/or power control of one SRS towards to multiple TRPs - SRS TD-OCC - Increasing the maximum number of cyclic shifts - E.g., multiplying mask sequence to the legacy SRS sequence to effectively increase the maximum cyclic shifts - Precoded SRS for DL CSI acquisition - Enhanced signaling for flexible SRS transmission - E.g., dynamic update of SRS parameters - Partial frequency sounding extensions - E.g., larger partial frequency sounding factor, starting RB location hopping enhancements, partial frequency hopping on other bandwidths corresponding to , besides the last bandwidth. - Enhanced configuration of SRS transmission to enable more efficient SRS parameter assignment - E.g., configuration of v (sequence index within a group) per SRS resource - E.g., configuration of cyclic shift per SRS port per SRS resource - Resource mapping for SRS transmission based on network-provided parameters or system parameters - E.g., SRS resource mapping based on network-provided parameters (e.g., configurable indexes) or system parameters (e.g., slot index) Note: PAPR performance and maintaining DFT waveform property should be considered when deciding the enhancement for Rel-18.

To further increase the SRS capacity or provide randomization for SRS, introducing muting mechanism for SRS transmission would be a feasible way to reduce the cross-SRS interference in Rel-18.

Cancellation indication based SRS transmission

In Rel-16, uplink cancellation indication (CI) carried by DCI format 2_4 scrambled with CI-RNTI was introduced for URLLC transmission, which is designed to cancel the eMBB PUSCH and SRS transmission early, and reserve available uplink resources for URLLC PUSCH. Therefore, one straightforward way is to use a similar mechanism of cancellation indication to achieve SRS muting in TDD CJT.

However, cancellation indication should be further enhanced for Rel-18 SRS capacity, since there are several main limitations of the current cancellation indication as follows, if used for SRS muting.

- For the current cancellation indication, the cancellation granularity in the frequency domain is RB level, which is too coarse to mute the SRS resource with comb structure. Especially, when there are multiple SRS resources multiplexing in different combs, all the SRS resources would be muted based on the current cancellation indication. If only one of the SRS resources is intended to be muted, it can’t be handled by the current cancellation indication.

- Cancellation indication is carried by DCI, and it can only dynamically mute the SRS occasion in one specific uplink resource region at a time. In other words, for periodic/semi-periodic SRS transmission, only one of the periodic SRS occasions would be canceled by one cancellation indication. Therefore, persistent muting can’t be achieved by the current cancellation indication.

Due to above mentioned limitations of cancellation indication in the current NR system, some necessary enhancements should be introduced, such as the association with SRS resource(s), and periodic/persistent cancellation by DCI to achieve a more feasible muting mechanism for SRS in Rel-18.

If cancellation indication is used to mute SRS transmission in Rel-18, some limitations of the current cancellation indication, such as cancellation granularity, and periodic/persistent cancellation by DCI should be addressed.

Support to enhance uplink cancellation indication to reduce cross-SRS interference in Rel-18.

Random muting based SRS transmission

Design principles for random muting

In addition to enhance the capacity of Rel-18 SRS, providing randomization in the frequency/time/code domain for SRS is also a significant way to reduce cross-SRS interference. Therefore, another feasible way to achieve SRS transmission muting is based on random muting in the time-domain, which could be achieved based on two potential methods below.

- Alt 1: Based on pseudo-random sequence

- Alt 2: Based on initial muting pattern and update rule for randomization

Regarding Alt 1, a pseudo-random sequence of bit 0/1 can be used to determine SRS transmission by bitmap, e.g., ‘0’ is corresponding to muting occasion of SRS, while ‘1’ is corresponding to transmission occasion of SRS Besides, the size of muting occasion depends on the muting granularity, e.g., one slot, one frequency hopping, or one symbol. An example is given in Figure 1, assume that pseudo-random sequence is {1 0 0 1 0 1 1 0 1 1 0 0…0 0 0 1}, and each bit corresponds to one symbol as the muting granularity. Based on Alt 1, if two SRS resources collide with each other, different pseudo-random sequences can be applied to them separately to transfer persistent interference between two colliding SRS resources to random interference.

Figure 1 SRS muting based on pseudo-random sequence

Regarding Alt 2, the initial muting pattern could be indicated by the network as a short sequence of bit 0/1. Similar to Alt 1, each bit is corresponding to a muting occasion or transmission occasion based on the muting granularity. Then based on the specific update rule such as bit flipping, and bit shifting, the initial muting pattern would convert to a new muting pattern in the next muting period, then the muting occasions and transmission occasions would also change synchronously to achieve randomization.

Figure 2 SRS muting based on initial muting pattern and update rule

As shown in Figure 2, assume that the initial muting pattern is configured as a short sequence of {1 0 0 0 1 1 0 1}, and each bit is corresponding to the muting granularity as one symbol. In slot #1, there are a total of 8 symbols occupied by SRS with 4 frequency hopping. Based on the initial muting pattern, only the SRS symbols corresponding to the transmission occasion would be transmitted. Then in slot #2, the initial muting pattern would be updated by bit shifting, and new muting occasions and transmission occasions would generate. Based on Alt 2, different initial patterns can be configured for different SRS resources to reduce mutual interference.

Moreover, one interesting point is that SRS transmission with longer periodicity can be achieved as a special case of random muting based on some specific muting patterns. For example, the muting pattern such as {1 0}, {1 0 0 0} without any update rule can be applied to achieve the same effect as SRS transmission with 2 times, 4 times the periodicity. Therefore, random muting based on muting pattern can provide great flexibility for the network to configure SRS transmission for different cases.

SRS transmission with longer periodicity can be regarded as a specific case of random muting based on a muting pattern, e.g., {1 0}.

Evaluation results

LLS evaluation results are given to show the benefit of random muting for SRS transmission. In the evaluation, 4 UEs transmit 4 SRS resources with different sequence group indexes u on the same frequency and time resources separately. Due to the different distances from 4 UEs to the target TRP for SRS receiving, different power of received SRS resources is modeled, where UE1 is set as the target UE for evaluation, and the cross-SRS interferences from UE 2/3/4 are assumed as -3dB, -6dB, -10dB.

To evaluate the benefit of random muting, some basic SRS transmission schemes are also simulated. The baseline is that 4 UEs transmit their SRS resources in the same SRS occasion with the same periodicity, leading to full interference on the UE1 from UE 2/3/4. Besides, SRS transmission with longer periodicity is evaluated, where the periodicity is doubled compared with the baseline. In this case, 4 UEs could be configured to transmit SRS on two different SRS occasions. However, cross-SRS interference on the target UE depends on how the network configures the SRS occasions for different UEs. If the network configures different SRS occasions for UE 2/3/4 and UE 1, then there would be no cross-SRS interference on UE1. If the network configures the same SRS occasions for UE 1 and a certain UE, then there would be different levels of cross-SRS interference on UE1. For instance, when UE 1 and UE 3 transmit SRS on the same SRS occasions, UE 1 would suffer the interference of -6dB from UE 3. When UE 1 and UE 4 transmit SRS on the same SRS occasions, UE 1 would suffer the interference of -10dB from UE 4.

Regarding random muting for SRS transmission, Alt 2 is used in the evaluation, where the initial muting patterns are {1 0 1 0} for UE 1, {0 1 0 1} for UE 2, {0 0 1 1} for UE 3, {1 1 0 0} for UE 4, and bit flipping is applied as the update rule. In this case, UE 1 and UE 2 would not interfere with each other, while UE 3 and UE 4 would also not interfere with each other, due to orthogonal SRS occasions. However, UE 1 would suffer different levels of interference from UE 3 and UE 4 on different SRS transmission occasions. Besides, to fairly compare with SRS transmission with longer periodicity, the muting granularity is set as one SRS occasion per SRS periodicity, which would guarantee the same number of SRS occasions for available SRS transmission.

| a) SRS occasion per 10ms | b) SRS occasion per 20ms |
| --- | --- |

Figure 3 SRS muting based on initial muting pattern and update rule for randomization

In Figure 3, NMSE results of the channel estimation by different SRS transmission schemes are given, where NMSE is calculated based on the ideal channel estimation. It is observed that when the target UE, i.e., UE 1 suffers the full interference, the channel estimation accuracy would be degraded severely. Regarding random muting based on muting pattern, the channel estimation result is much better than the baseline with full interference, since it can randomize the available SRS occasions for different UEs. Besides, compared with longer periodicity with persistent interference of -6dB from UE 3, the NMSE of random muting is smaller due to the randomization of different levels of interference from UE 3 and UE 4.  However, compared with longer periodicity with persistent interference of -10dB from UE 4, the NMSE of random muting is larger, since random muting introduces stronger interference from UE 3 on the same SRS occasions. In other words, random muting can achieve a statistical benefit based on randomizing the interference. Based on random muting, there is no need for the network to pre-judge the level of varying interferences from different UEs and configure the SRS occasion for each UE to control the overall cross-SRS interference.

Random muting could randomize the persistent cross-SRS interference into random interference, leading to a statistical benefit for SRS transmission.

Based on random muting, it can save the effort of the network to pre-judge the level of varying interferences from different UEs.

Support random muting based SRS transmission to reduce the cross-SRS interference, at least considering the following potential schemes in Rel-18

- Alt 1: Based on pseudo-random sequence

- Alt 2: Based on initial muting pattern and update rule for randomization

Conclusion

In summary, the following proposals are made for Rel-18 SRS enhancement.

- For comb 2, 8 different cyclic shifts can be exactly mapped on 8 SRS ports.

- For comb 2, performance cannot be guaranteed with mapping 8 SRS ports in the same frequency position when the channel delay is large。

- If 8 SRS ports are multiplexed based on TD-OCC, several key issues should be addressed, such as uplink coverage performance, effect on frequency hopping, and collision resolution.

- 8 SRS ports for codebook based transmission can be mapped on multiple SRS resources for different SRS resource sets.

- If cancellation indication is used to mute SRS transmission in Rel-18, some limitations of the current cancellation indication, such as cancellation granularity, and periodic/persistent cancellation by DCI should be addressed.

- SRS transmission with longer periodicity can be regarded as a specific case of random muting based on a muting pattern, e.g., {1 0}.

- Random muting could randomize the persistent cross-SRS interference into random interference, leading to a statistical benefit for SRS transmission.

- Based on random muting, it can save the effort of the network to pre-judge the level of varying interferences from different UEs.

- For comb 2, the same port mapping method as for 4 ports SRS can be considered for 8 ports SRS, considering the flexibility.

For comb 4, a similar approach for 4 ports SRS for cyclic shifts and frequency position mapping can be considered for 8 ports SRS.

For comb 8, 8 ports SRS can be mapped with two configurable cyclic shifts in four different frequency positions.

For codebook based transmission and antenna switching, support cyclic shift and FDM based enhancement on one SRS resource with 8 ports in one OFDM symbol.

The feasibility of one 8 ports SRS resource based on TD-OCC should be carefully discussed.

For codebook based transmission, support to aggregate 8 ports from two SRS resources from two different SRS resource sets.

For non-codebook based transmission, support up to 8 single-port SRS resources from two SRS resource sets based on two SRI fields.

Support to enhance uplink cancellation indication to reduce cross-SRS interference in Rel-18.

Support random muting based SRS transmission to reduce the cross-SRS interference, at least considering the following potential schemes in Rel-18

- Alt 1: Based on pseudo-random sequence

- Alt 2: Based on initial muting pattern and update rule for randomization

Reference

RP-213598, New WID: MIMO Evolution for Downlink and Uplink, Samsung

RAN WG1 #109e meeting Chairman’s Notes

RAN WG1 #110 meeting Chairman’s Notes