3GPP TSG RAN WG1 #110bis-e   	                              R1-2208900

e-Meeting, October 10th – 19th, 2022

Agenda item:		9.2.2.2

Source:		LG Electronics

Title:			Other aspects on AI/ML for CSI feedback enhancement

Document for:	Discussion and Decision

Introduction

In RAN1#110 meeting, following agreement and conclusion were made.

Agreement In CSI compression using two-sided model use case, the following AI/ML model training collaborations will be further studied: - Type 1: Joint training of the two-sided model at a single side/entity, e.g., UE-sided or Network-sided. - Type 2: Joint training of the two-sided model at network side and UE side, repectively. - Type 3: Separate training at network side and UE side, where the UE-side CSI generation part and the network-side CSI reconstruction part are trained by UE side and network side, respectively. - Note: Joint training means the generation model and reconstruction model should be trained in the same loop for forward propagation and backward propagation. Joint training could be done both at single node or across multiple nodes (e.g., through gradient exchange between nodes). - Note: Separate training includes sequential training starting with UE side training, or sequential training starting with NW side training [, or parallel training] at UE and NW - Other collaboration types are not excluded. Conclusion CSI-RS configuration and overhead reduction is NOT selected as one representative sub-use case for CSI feedback enhancement use case. Conclusion Resource allocation and scheduling is NOT selected as one representative sub-use case for CSI feedback enhancement use case. Agreement In CSI compression using two-sided model use case, further study potential specification impact on CSI report, including at least - CSI generation model output and/or CSI reconstruction model input, including configuration(size/format) and/or potential post/pre-processing of CSI generation model output/CSI reconstruction model input. - CQI determination - RI determination Agreement In CSI compression using two-sided model use case, further study potential specification impact on output CSI, including at least - Model output type/dimension/configuration and potential post processing Agreement In CSI compression using two-sided model use case, further discuss at least the following aspects, including their necessity/feasibility/potential specification impact, for data collection for AI/ML model training/inference/update/monitoring: - Assistance signaling for UE’s data collection - Assistance signaling for gNB’s data collection - Delivery of the datasets.

This contribution further discusses on several aspects on AI/ML for CSI feedback enhancement other than evaluation, and provides our view on sub use cases and potential specification impact.

Discussion

Sub use cases for CSI feedback enhancement

In RAN1#109e meeting, spatial-frequency domain CSI compression using two-sided AI model was chosen as one representative sub-use case. Also, in RAN1#110 meeting, there was intensive discussion on whether or not to introduce another representative sub use case. As an outcome, it was concluded that CSI-RS overhead reduction and resource allocation were not selected as one representative sub-use case for CSI feedback enhancement.

Other sub use cases including legacy codebook based enhancement and time-domain CSI prediction are still on-going discussion. Legacy codebooks such as Type I, Rel-15/Rel-16/Rel-17 Type II CSI employ multi stage codebook structure. For example, Type I CSI and Rel-15 Type II CSI have dual stage structure W=W1*W2 where W1 is base matrix representing spatial domain and W2 is selection or combining matrix. Rel-16/Rel-17 Type II CSI have W=W1*W2*WfH where the FD basis Wf, i.e. DFT vector, is utilized for frequency domain compression in order to reduce payload of Rel-15 Type II CSI. Also, in Rel-18 Type II CSI enhancement, it is ongoing discussion on how to reflect the future CSI for the high mobility scenarios, and introducing time domain or Doppler domain matrix can be considered as candidate options. In these codebooks, the accuracy is depending on the granularity of SD/FD/TD/DD basis and the quantization of combining coefficients. Thus, if AI/ML is used to optimize one or more component matrices, increased performance and/or reduced payload can be expected.

Another potential sub-use case is time-domain prediction based on the one-sided AI/ML model. As shown in Figure 1, the one-sided AI/ML at NW side or UE side employs historical CSI channel measurement as an input of AI/ML and then the output of AI/ML will be predicted future CSI.

Figure 1. Illustration of time-domain prediction based on the one-sided AI/ML model

In this sub-use case, there are some issues to be discussed. First, similar topic is under discussion in Rel-18 MIMO session for high mobility scenarios. Thus, some related sub issues can be overlapped, if this time-domain predication based on AI/ML is chosen as representative sub use case. Second, the baseline scheme should be Rel-18 Type II CSI to observe the benefit from AI/ML aided CSI prediction. If we set a baseline scheme with low performance such as sample-and-hold approach, the performance gain would be over-estimated. Therefore, baseline should be carefully determined by state-of-art scheme. Lastly, according to the WID, RAN1 needs to finalize sub use cases until RAN1#111 meeting, so there may not be enough time schedule to selected another sub use case. Therefore, it would be better to focus on the S-F compression CSI based two-sided AI/ML model.

Proposal 1: Focus on spatial-frequency domain CSI compression using two-sided AI model. CSI prediction is not selected as another representative sub-use case unless clear benefit is shown.

Potential specification impacts for CSI feedback enhancement

In order to support sub use cases for CSI feedback enhancement, the following potential specification impact can be considered.

- CSI reporting configurations

In order to support spatial-frequency domain CSI compression or legacy codebook based enhancement, it is required to define CSI reporting configurations. Especially for spatial-frequency domain CSI compression, it should be first determined that whether it is based on the explicit feedback or implicit feedback. Traditionally, LTE and NR employed implicit feedback where RI/CQI/PMI are reported to gNB instead of raw channel matrix or channel covariance matrix. The one motivation for employing implicit feedback is due to its low feedback overhead compared to that of explicit feedback. Meanwhile, explicit feedback reports the raw channel information such as eigenvector or channel covariance matrix, so their feedback payload is normally large. If AI/ML is utilized for CSI payload reduction, raw channel information can be compressed and feed-backed to gNB. Thus, it may lead to explicit feedback and its expected specification impact seems larger compared to implicit feedback based scheme. In order to support such AI/ML based CSI reporting, for example, new CSI reporting quantity representing CSI generated by AI/ML can be defined. Also, signal/channel carrying AI/ML based CSI and some rules such as CSI priority rule, CSI omission rule, and CSI dropping rule need to be discussed, if needed.

Proposal #2: Consider enhancement of CSI reporting configurations for AI/ML based CSI feedback.

- CSI processing unit (CPU) and CSI reference resource

In NR, CPU is defined to efficiently process CSI reports at UE. For CPU, the UE reports the number of supported simultaneous CSI calculation per CC and across CC. Based on the reported value and pre-defined CPU occupancy rule, UE determines whether to process triggered CSI reports or not. AI/ML based CSI calculation/reporting is quite different from the legacy CSI calculation/reporting in terms of computational complexity. In addition, we may consider employing multiple AI/ML models at the same time, and it may consider independent life cycle management for each of AI/ML model. In this situation, re-using legacy CSI processing criteria may not be sufficient. So, it needs to be discussed whether AI/ML based CSI calculation/reporting can be handled within legacy CSI processing criteria.

In case of CSI prediction based on the AI/ML model, the CSI reference resources also needs to be enhanced as multiple channel measurement and/or multiple future CSI reporting is considered. So, it is needed to re-define CSI reference resource. In the current specification, the CSI reference resource is defined as the time before CSI reporting. So, channel aging would be caused by the difference between the time for CSI reference resource and for PDSCH transmission. Based on AI/ML based CSI prediction, CSI reference resource can be modified to be based on the resource after CSI reporting, e.g., PDSCH transmission time. Fortunately, similar discussion is ongoing in Rel-18 MIMO. Thus, we can postpone the discussion until there are meaningful related outcome in Rel-18 MIMO CSI enhancement.

Proposal #3: Consider enhancement of UE CSI processing procedure including CPU and CSI reference resource for AI/ML based CSI reporting.

- Fallback operation

Since AI/ML is data-driven model, its performance depends on the trained dataset. In this sense, if the channel statistic is changed, the AI/ML based CSI reporting may not be valid. Regarding this issue, there is discussion on life cycle management in the frame work agenda. Normally, LCM is a process of model deployment, training, inference, monitoring and update. If the AI/ML model is not valid or outdated, the model needs to be re-trained or updated which may require a long processing time. In this case, UE needs to fallback to legacy CSI reporting. Therefore, it is worthwhile to discuss on the fallback operation when the AI/ML based CSI reporting is not valid.

Proposal #4: Consider fallback operation when AI/ML based CSI reporting is not valid.

- CQI / RI determination

In the last meeting, it was agreed to further study on CQI and/or RI determination for two-sided AI/ML based S-F compression CSI. In this case, UE usually deploys CSI encoder and gNB deploys CSI decoder. The problem is UE may not fully have information on the final decoder output, so there is ambiguity on the CQI and/or RI determination at UE side. To alleviate this problem, following options can be considered.

- Opt 1. Trigger another non-A/ML based CSI report

- Opt 2. Report new CSI contents instead of CQI and/or RI

- Opt 3. Allow UE to have AI/ML model information for decoder

In option 1, non-AI/ML based CSI report is triggered in addition to AI/ML based CSI report. Then, the CQI and RI can be determined based on the non-AI/ML based CSI report. In this case, determination of actual CQI and/or RI of AI/ML based CSI report can be up to gNB implementation.

Option 2 considers new CQI reporting quantity such as SINR for AI/ML based CSI report. Actually, CQI is the information of modulation order, code rate and spectral efficiency obtained from SINR measurement. Thus, gNB may have no problem on MCS configuration of PDSCH if UE reports actual SINR.

Lastly, option 3 allow UE to have AI/ML model of decoder. In this option, legacy CQI / RI determination can be reused. However, this option requires more UE capability and implementation complexity to enable both encoder and decoder at UE.

Proposal #5: For CQI and/or RI determination of two-sided AI/ML based Spatial-Frequency CSI compression, consider following options.

- Opt 1. Trigger another non-A/ML based CSI report

- Opt 2. Report new CSI contents instead of CQI and/or RI

- Opt 3. Allow UE to have AI/ML model information for decoder

Conclusion

In this contribution, we discussed on several aspects on AI/ML for CSI feedback enhancement other than evaluation. Based on the above discussion, we have the following proposals.

Proposal 1: Focus on spatial-frequency domain CSI compression using two-sided AI model. CSI prediction is not selected as another representative sub-use case unless clear benefit is shown.

Proposal #2: Consider enhancement of CSI-RS/CSI reporting configurations for AI/ML based CSI feedback.

Proposal #3: Consider enhancement of UE CSI processing procedure including CPU and CSI reference resource for AI/ML based CSI reporting.

Proposal #4: Consider fallback operation when AI/ML based CSI reporting is not valid.

Proposal #5: For CQI and/or RI determination of two-sided AI/ML based Spatial-Frequency CSI compression, consider following options.

- Opt 1. Trigger another non-A/ML based CSI report

- Opt 2. Report new CSI contents instead of CQI and/or RI

- Opt 3. Allow UE to have AI/ML model information for decoder