3GPP TSG RAN WG1 Meeting #110-bis-e	R1-2209063

e-Meeting, October 10th  – 19th, 2022

Source:	Intel Corporation

Title:	Discussion on Network energy saving performance evaluations

Agenda item:	9.7.1

Document for:	Discussion

# Introduction

The following was agreed corresponding to evaluation of different network energy saving techniques [1].

Agreement For evaluation purpose, the energy consumption modeling for a BS includes at least the following: - Reference configuration - FFS other details - Note FR1 and FR2 to be separately considered for detailed parameters - Multiple power state(s) including sleep/non-sleep mode(s) with relative power, and associated transition time/energy - Scaling method to be applied at least for non-sleep mode. - FFS other details including scaling for sleep mode Agreement For evaluation purpose, the BS energy consumption model should at least include the power consumption of BS on slot-level. - Note that symbol-level power consumption to reflect different BW (or RB utilization) / time-occupancy / tx-rx direction of different symbols in a slot is considered. - FFS details (e.g. explicit symbol-level power modelling, scaling slot-level power to symbol level power for various cases, etc.) Note: system simulation evaluations can be per slot regardless of detailed approach for calculating symbol-level power consumption Agreement - For evaluation, at least for non-sleep mode and TDD, the BS power consumption for DL and UL are separately modelled, allowing DL-only transmission or UL-only reception. - FFS: whether UL-only reception energy consumption model can be derived/simplified from DL-only transmission energy consumption model - FFS: the impact of UL reception and/or DL transmission on sleep modes and associated transition time/energy - FFS: whether/how to define an idle state, where BS is neither transmitting nor receiving but also doesn’t enter into any sleep mode or define it as sleep mode - FFS: whether the model for FDD can be based on the model for TDD Agreement For evaluation purpose, - Study how to define sleep modes and determine the characteristics for each mode from one or multiple of the below - Relative power - Transition time - Transition energy - Other approaches are not precluded - Note: BS components that can be turned off can be considered for discussion purpose when defining the specific values of the characteristics for sleep modes. - Study whether sleep mode is defined for DL(TX) and UL(RX) jointly or separately - Study the assumption of order for BS entering/resuming from a sleep mode to another mode (sleep or non-sleep) and the associated transition time and energy, i.e. state machine which may have impact on the transition energy. Agreement For evaluation, the scaling in a BS energy consumption model can be considered based on one or more of the following, - Number of used physical antenna elements, or TX/RX RUs - FFS: Mapping between used TX/RX RUs and used antenna ports - FFS: Mapping between physical antenna elements and TX/RX RUs - Occupied BW/RBs for DL and/or UL in a slot/symbol in one CC - number of CCs in CA - FFS dependency of RF sharing - number of TRPs - PSD or transmit power - FFS dependency on BW scaling - FFS: PA energy efficiency value - number of DL and/or UL symbols occupied within a slot - FFS other domain scaling - FFS scaling is linearly or else, for each domain Above does not necessarily imply that BS energy consumption model that takes into account all listed scaling factors will be developed Agreement For BS energy consumption evaluation, in addition to the energy saving gain, - At least UPT/UE power consumption/access delay/latency should be considered for performance impact evaluation Note: this doesn’t necessarily mean that all the above are considered for all evaluation results. However, multiple KPIs are expected to be evaluated for a given technique. And this does not preclude to consider other KPIs when found appropriate for certain techniques/scenarios. Agreement At least urban macro is prioritized for FR1. FFS the baseline deployment assumption for FR2. Agreement - FTP3 (0.5MB as packet size, 200ms as mean inter-arrival time), FTP3 IM (0.1MB as packet size, 2s as mean inter-arrival time) and VOIP can be considered in the evaluation - FFS: with possible further prioritization, different model between DL and UL, and/or other traffic models that can be optionally considered. FFS associated scenarios/configurations, e.g. C-DRX. Agreement For evaluation and BS energy consumption modeling purpose, for single CC case, at least the following in table should be considered for reference configuration - Note: other TX-RX RU number and corresponding BS antenna configuration can be considered in SLS assumptions Set 1 FR1; Set 2 FR1; Set 3 FR2. Duplex; TDD; FDD; TDD. System BW; 100 MHz; 20 MHz; 100 MHz. SCS; 30 kHz; 15 kHz; 120 kHz. Number of TRP; 1; 1; 1. Total number of DL TX RUs; 64; (working assumption) 32; 2. Total DL power level; 55dBm; [49dBm] – to be further discussed and finalized in future meetings; 43dBm – to be further discussed and finalized in future meetings EIRP limited to 78dBm – to be further discussed and finalized in future meetings. Total number of UL Rx RUs; 64; (working assumption) 32; 2. Agreement As a starting point, - macro cell BS for FR1 is assumed for energy consumption model. - FFS: micro cell BS for FR2 is assumed for energy consumption model. Agreement The evaluation baseline for energy saving study/evaluation for BS includes at least NR R15 mandatory without capability features. Optional features from R15 onwards (e.g. CA, MIMO) as well as implementation-based energy saving techniques should be explicitly reported and described if used in the evaluation baseline. - FFS: need of alignment for certain configurations/implementation-based schemes. Agreement - Similar to UE power saving study, percentage of energy consumption reduction from the baseline is used to express BS energy saving gain. - SLS is considered as baseline evaluation method. Other method, including numerical analysis and LLS can also be considered. At least one of the methods should be selected and used for evaluation of a specific technique (selection and criteria is up to proponent). Working assumption For evaluation, for energy consumption modelling for FDD and the case of simultaneous DL transmission and UL reception for non-sleep mode, study the following with potential down-selection in RAN1#110 - Option 1: the power consumption is the total of DL and UL power consumption - Option 2: the power consumption for UL is neglected - Other option is not precluded - Note the DL (or UL) power consumption can be obtained using a same approach as that obtained from the DL (or UL)-only in TDD model Agreement For non-sleep mode, the relative power value in power model table for UL reception and/or DL transmission is provided based on reference configuration. Agreement For set 2 FR1 FDD TxRx reference configuration, confirm the WA as 32 in reference configuration. Agreement The total DL power level is 49 dBm for set 2 FR1 FDD reference configuration. Agreement For the purpose of evaluation, adopt the following as BS power consumption model. These entries for this table is per reference configuration set. - FFS: One or multiple values for relative power and transition time. Power state; Characteristic; Relative Power; Additional transition energy3; Total transition time. Deep sleep1; There is neither DL transmission nor UL reception. Time interval for the sleep should be larger than the total transition time entering and leaving this state.; P1=1; E1; T1. Light sleep; There is neither DL transmission nor UL reception. Time interval for the sleep should be larger than the total transition time entering and leaving this state. (P2>P1); P2; E2; T2. Micro sleep; There is neither DL transmission nor UL reception. Immediate transition is assumed for network energy saving study purpose from or to a non-sleep state.; P3; 0; 0. Active DL; There is only DL transmission.; P4; NA; NA. Active UL; There is only UL reception.; P5; NA; NA. Note 1: Depending on implementations, there could be a state that the power is lower than deep sleep and requires larger total transition time, e.g. hibernating sleep or Quasi-off, which is not explicitly modeled in this study for evaluation purpose. Note 3: Unit in relative power times duration. FFS: Details on how transition energy is defined.. - For simultaneous DL and UL transmission for FDD, the power for UL reception is neglected in this study. - FFS: Optionally, a state machine where BS may transit between sleep modes without entering non-sleep mode can be considered. Companies are to report the involved sleep modes and the assumptions for inter-sleep mode transition time used in their evaluations. - FFS: Details on how to use the above table for low power uplink reception (e.g. for WUS). Working Assumption For reference configuration set 1, the values are provided as below. FFS set2 and set 3. Power state; Relative Power P; Total transition time T. Deep sleep; 1; 1; Cat 1: 50ms; Cat 2: 10s. Light sleep; Cat 1: 25; Cat 2: 2.1; Cat 1: 6 ms; Cat 2: 640 ms. Micro sleep; Cat1: 55; Cat 2: 5.5; 0; 0. Active DL; Cat 1: 280; Cat 2: 32; N.A.; N.A.. Active UL; Cat 1: 110; Cat 2: 6.5; N.A.; N.A.. Agreement - For FR1, urban micro can be optionally considered. - For FR2, urban micro is prioritized, with ISD=200 m is assumed. Agreement It is up to company report which traffic model is used among the agreed three traffic models in their evaluations. - Other models may be used as well. Parameter (e.g. packet size and arrival rate) adjustment can be optionally considered and reported. Agreement For set 3 FR2 reference configuration, the total DL power level and EIRP limit is set as 33 dBm and 63 dBm respectively. Note EIRP limit is also scaled with the number of TxRU. Agreement For evaluation purpose, network energy saving gain is computed based on the energy consumptions for a technique and the baseline over the same duration. Agreement For initial evaluations, there is always a non-sleep mode assumed between adjacent sleep modes. Agreement Companies to report the assumption details for the reception of a low-power UL channel/signal, if used, including power states, additional transition energy, and transition times, receiver details (e.g. architecture and receiver sensitivity), and other impact/change on the power consumption model. Agreement Update the RAN1 agreements with the following changes In the evaluation, - A load (L)% of a cell is a percentage of resources used for UE specific PDSCH/PUSCH. - The following load scenarios are considered. Load scenario; Characteristics. Idle/empty load; - Include cell-specific signals and channels, and - L = 0. low load; - Include cell-specific signals and channels, and - 0 < L≤15. Light load; - Include cell-specific signals and channels, and - 150 < L≤30. Medium load; - Include cell-specific signals and channels, and - 30 < L≤50. For CA, the companies report whether the load is defined per CC or across all CCs.. Agreement During the transition time period, relative power of sleep mode i is assumed to be consumed. Additional transition energy and total transition time also include energy and time for both ramping down and ramping up Agreement - The total transition time for set 2 and set 3 is the same as that for set 1. - Companies are encouraged to check the input and values provided in section 2.1.4.2 of R1-2208312 for further determination. Conclusion Companies are encouraged to check discussion in section 2.2.2 of R1-2208312 for scaling discussion in the next meeting. Agreement - FFS whether to set exact requirements/QoS target for UPT and/or latency impact - Other KPIs can be optionally reported, conditioned with clear definition/descriptions provided. - Note for potential new channel/signals, e.g. WUS from UE, the assumption for detection reliability at BS side is reported (performance and complexity impact would subject to results and further discussion). Agreement It is up to company report the use of UE C-DRX. - the baseline configuration (for alignment/calibration) for C-DRX, if reported, can be as below; - Other inactivity timer values can be optionally reported Traffic type; FTP; IM; VoIP. Model; FTP model 3; FTP model 3; As defined in R1-070674. Assume max two packets bundled.. Packet size; 0.5 Mbytes; 0.1 Mbytes. Mean inter-arrival time; 200 ms; 2 sec. DRX Period; 160 ms; 320 ms; 40 ms. DRX Inactivity timer; 100 ms; 80 ms; 10 ms. On duration; FR1: 8 ms FR2: 4 ms; FR1: 10 ms FR2: 5 ms; FR1: 4 ms FR2: 2 ms. Agreement - For FR1, adopt the Reference SLS configurations in Annex-A in R1-2208312 as baseline SLS assumptions. - Other carrier frequencies can be optionally considered. - FFS For FR2 adopt the Reference SLS configuration used in Dense Urban Config.B in Table2 of RP-180524 for IMT-2020 with the following clarification/update as initial SLS assumption. - BS antenna configurations - 2 TxRU (M, N, P, Mg, Ng; Mp, Np) = (4,8,2,2,2;1,1) - (dH, dV) = (0.5λ, 0.8λ) (dg,H, dg,V) = (4.0λ, 3.6λ) - Traffic model & UE density - Follow previous agreements with adjusted UE density - Total transmit power per TRxP - Value scaled from that in set 3 reference configuration considering BW - Further adjustment/clarification can be discussed in the next meeting.

Annex A of R1-2208312 is as follows:

Annex A of R1-2208312 Table A The evaluation assumption for BS power consumption model Parameters. Basic parameters; Channel model; 3D/HF-Uma based on TR 38.901 (low-loss O2I penetration model); 3D/HF-Uma based on TR 38.901 (low-loss O2I penetration model). Device deployment; 80% indoor, 20% outdoor; 80% indoor, 20% outdoor. Inter-site distance; 500m; 500m. Network Topology; 7*3 Sector; 7*3 Sector. Carrier Frequency; 2.1GHz; 4.0GHz or 2.6GHz. Multiple access; OFDMA; OFDMA. Duplexing; FDD (for set 2 ref. config); TDD (for set 1 ref. config.). Numerology; 15KHz, 14 OFDM symbol slot; 30kHz, 14 OFDM symbol slot. Guard band ratio on simulation bandwidth; FDD: 6.4% (104RB for 15kHz SCS and 20 MHz BW); TDD: 2.08% (272 RB for 30kHz SCS and 100 MHz bandwidth). Simulation bandwidth; Follow reference configuration, (equal split of 10 MHz for UL and DL); Follow reference configuration. Frame structure; DDDSU. UT attachment; Based on RSRP; Based on RSRP. Wrapping around method; Geographical distance based wrapping; Geographical distance based wrapping. Traffic model; Follow previous RAN1 agreements; Follow previous RAN1 agreements. BS parameters; BS antenna height; 25 m; 25 m. BS noise figure; 5 dB; 5 dB. BS antenna element gain; 8 dBi; 8 dBi. Antenna configuration at TRxP; For 32T: (M,N,P,Mg,Ng; Mp,Np) = (8,8,2,1,1;2,8) (dH, dV)=(0.5, 0.8)λ; For 64T: (M, N, P, Mg, Ng, MP, NP,) = (8, 8, 2, 1, 1, 4, 8). based on 38.802. UE parameters; UE power class; 23dBm; 23dBm. UE noise figure; 9 dB; 9 dB. UE antenna element gain; 0 dBi; 0 dBi. UE antenna height; Outdoor UEs: 1.5 m; Indoor Uts: 1.5m or consider floor height; Outdoor UEs: 1.5 m; Indoor Uts: 1.5m or consider floor height. Antenna configuration at UE; For 4R: (M,N,P,Mg,Ng; Mp,Np)= (1,2,2,1,1; 1,2) (dH, dV)=(0.5, N/A)λ; For 4R: (M,N,P,Mg,Ng; Mp,Np)= (1,2,2,1,1; 1,2) (dH, dV)=(0.5, N/A)λ. Transmission parameters; Modulation; Up to 256 QAM; Up to 256 QAM. Transmission scheme; SU-MIMO; SU-MIMO. SU dimension; For 4Rx: Up to 4 layers; For 4Rx: Up to 4 layers. DL CSI measurement; Non-precoded CSI-RS based; Precoded CSI-RS based. DL codebook; Type I/II codebook; non-PMI transmission. SRS transmission; N/A; For UE 4 Tx ports: Non-precoded SRS. CSI feedback; Company to report the assumptions; Company to report the assumptions. Interference measurement; SU-CQI; CSI-IM for inter-cell interference measurement; SU-CQI; CSI-IM for inter-cell interference measurement. Scheduling; PF; PF. Receiver; MMSE-IRC; MMSE-IRC. Channel estimation; Non-ideal; Non-ideal. Common RS; SSB period; 20ms; 20ms. (M, N, P, Mg, Ng; Mp, Np) - M: Number of vertical antenna elements within a panel, on one polarization - N: Number of horizontal antenna elements within a panel, on one polarization - P: Number of polarizations - Mg: Number of panels in a column; - Ng: Number of panels in a row; - Mp: Number of vertical TXRUs within a panel, on one polarization - Np: Number of horizontal TXRUs within a panel, on one polarization

Energy consumption is a major contributor to the network OPEX. Increasing capacity while keeping network energy consumption low is a big challenge for 5G NR based network. Energy consumption model and KPI development are important parts of Rel-18 Network Energy Saving study, which allow us to evaluate different network energy saving techniques. In RAN1- #110, good progress was made towards identifying a suitable evaluation framework for BS power consumption. In this contribution, we present our views on remaining aspects of evaluation methodology.

# Discussion on remaining aspects of evaluation methodology

## Discussion on sleep modes

In RAN1 #110, three sleep modes such as Deep sleep, Light sleep, and Micro-sleep were agreed with relative power values and transition times for two Categories. However, no consensus was reached regarding the additional transition energy values. Based on discussion from RAN1 #110, for set 1 FR1 TDD, our proposed values for the additional energy consumption (unit in relative power*(duration in ms)) is provided below:

Table 1:  Additional transition energy values for transition to/from SM for Set 1

| Power state | Additional transition energy Ei (unit in relative power*ms) |  |
| --- | --- | --- |
|  | Category 1 | Category 2 |
| Deep sleep | 1350 | 13000 |
| Light sleep | 90 | 1088 |

Since it was agreed that total transition times for sleep modes in Set 2 and Set 3 are the same as in Set 1, we propose to adopt same additional transition energy values for all the sets as well.

Proposal 1: Adopt the following additional transition energy values for Sets 1, 2, and 3.

| Power state | Additional transition energy Ei (unit in relative power*ms) |  |
| --- | --- | --- |
|  | Category 1 | Category 2 |
| Deep sleep | 1350 | 13000 |
| Light sleep | 90 | 1088 |

## Remaining aspects of reference configuration

Channel model applicable for FR1 FDD and TDD were assumed to be TR 38.901 UMa channel model. However, the text had been struck out in the evaluation parameter list. We suggest adding back the description for clarity.

Proposal 2: Adopt TR38.901 UMa as the channel model for the reference configuration for FR1 TDD and FDD.

|  | Parameters |  |  |
| --- | --- | --- | --- |
| Basic parameters | Channel model | 3D/HF-Uma based on TR 38.901 TR 38.901 UMa (low-loss O2I penetration model) | 3D/HF-Uma based on TR 38.901 TR 38.901 UMa (low-loss O2I penetration model) |

## Scaling of energy consumption model

Although relative power model for reference configuration was agreed in RAN1 #110, the scaling power model is still open. It is necessary to obtain a well-rounded scaling power model to consider different choices of antenna configurations, transmit power, occupied BW and symbols within a slot, considering both DL and UL.

At least for FR1 TDD, we think the BS power consumption for active DL state can be provided by

PDL= Pstatic      DL+ PdynamicDL 	(1)

where PstaticDL= Pstatic represents a static part for which the power does not scale based on the reference configurations. In our view,  Pstatic should corresponds to Micro-sleep state, P3, and the value should be 55 and 5.5 for Category 1 and Category 2 models, respectively.

On the other hand,  PdynamicDL refers to dynamic part of the power that is scaled based on the reference configuration and factors such as number of antennas, time/frequency resource occupation, transmit power, DL or UL symbols in a slot etc. Note that PdynamicDL ≤ P4-   Pstatic  holds, where P4 refers to the active DL relative power value for the reference configuration and is given by 280 and 32 for Category 1 and 2, respectively.

At least considering frequency, spatial and power domain, we think PdynamicDL  can be provided as

PdynamicDL= sa*Pdyn,ante +sf*spηsf,  sp*Pdyn,joint ,	(2)

where

- sa,sf, sp refer to the percentage of active TRxRUs, the ratio of RF bandwidth and maximum system BW and the ratio of PSD per TxRU between the DL transmission and reference configuration, respectively.

- ηsf,  sp is the PA efficiency

  - We suggest default value of 1 can be assumed for initial evaluations. Other values < 1 can be optionally evaluated

- Pdyn,ante refers to component of dynamic power that is scaled based on number of active set of antennas only

  - We suggest 110 value for this part for Category 1

- Pdyn,joint refers to component of dynamic power that is scaled based on both number of active set of antennas and amount of occupied BW

  - We suggest 115 value for this part for Category 1

- Note that choices of Pdyn,ante  and Pdyn,joint need to ensure that when all scaling factors are 1, PdynamicDL= P4-   Pstatic .

Now, the BS power consumption for active UL can be provided as follows:

PUL=PstaticUL+PdynamicUL                                                                                       (3)

Where PstaticUL=Pstatic  = P3, PdynamicUL=  sa*P5, and P5 refers to active UL state relative power for the reference configuration.

Considering a TDD slot comprising DL and/or UL symbols, the overall power consumption in the slot can be provided based on (1), (2), (3), as follows:

P=(1- αDL- αUL)Pstatic+αDL PDL +  αUL PUL,	   (4)

where   αDL and αUL represents the ratios of the number of active DL and UL symbols within a slot to  the number of symbols within a slot, respectively.

Proposal 3: Adopt the following power scaling model for obtaining relative power of a slot for different DL/UL symbol pattern in a slot, BW occupancy, antenna configuration and transmit power.

P=(1- αDL- αUL)Pstatic+αDL PDL +  αUL PUL,

where,

- Pstatic  = P3, a static part for which the power does not scale based on the reference configurations. P3 refers to Micro-sleep state power value.

  - Pstatic should be 55 and 5.5 for Category 1 and Category 2 models, respectively.

- αDL and αUL represents the ratios of the number of active DL and UL symbols within a slot to the number of symbols within a slot, respectively.

- PDL= Pstatic+ PdynamicDL , where  PdynamicDL= sa*Pdyn,ante +sf*spηsf,  sp*Pdyn,joint

  - sa, sf, sp refer to the percentage of active TRxRUs, the ratio of RF bandwidth and maximum system BW and the ratio of PSD per TxRU between the DL transmission and reference configuration, respectively.

  - ηsf,  sp is the PA efficiency, for which default value is 1. Other values < 1 can be optionally evaluated

  - Pdyn,ante refers to component of dynamic power that is scaled based on number of active set of antennas only

    - We suggest value of 110 for this part for Category 1

  - Pdyn,joint refers to component of dynamic power that is scaled based on both number of active set of antennas and amount of occupied BW

    - We suggest value of 115 value for this part for Category 1

- PdynamicUL=  sa*P5, where P5 refers to active UL state relative power for the reference configuration.

# Conclusions

In this contribution, we discussed issues related to evaluation methodology for network energy saving SI. The following is a summary of the proposals and observations:

Proposal 1: Adopt the following additional transition energy values for Sets 1, 2, and 3.

| Power state | Additional transition energy Ei (unit in relative power*ms) |  |
| --- | --- | --- |
|  | Category 1 | Category 2 |
| Deep sleep | 1350 | 13000 |
| Light sleep | 90 | 1088 |

Proposal 2: Adopt TR38.901 UMa as the channel model for the reference configuration for FR1 TDD and FDD.

|  | Parameters |  |  |
| --- | --- | --- | --- |
| Basic parameters | Channel model | 3D/HF-Uma based on TR 38.901 TR 38.901 UMa (low-loss O2I penetration model) | 3D/HF-Uma based on TR 38.901 TR 38.901 UMa (low-loss O2I penetration model) |

Proposal 3: Adopt the following power scaling model for obtaining relative power of a slot for different DL/UL symbol pattern in a slot, BW occupancy, antenna configuration and transmit power.

P=(1- αDL- αUL)Pstatic+αDL PDL +  αUL PUL,

where,

- Pstatic  = P3, a static part for which the power does not scale based on the reference configurations. P3 refers to Micro-sleep state power value.

  - Pstatic should be 55 and 5.5 for Category 1 and Category 2 models, respectively.

- αDL and αUL represents the ratios of the number of active DL and UL symbols within a slot to the number of symbols within a slot, respectively.

- PDL= Pstatic+ PdynamicDL , where  PdynamicDL= sa*Pdyn,ante +sf*spηsf,  sp*Pdyn,joint

  - sa, sf, sp refer to the percentage of active TRxRUs, the ratio of RF bandwidth and maximum system BW and the ratio of PSD per TxRU between the DL transmission and reference configuration, respectively.

  - ηsf,  sp is the PA efficiency, for which default value is 1. Other values < 1 can be optionally evaluated

  - Pdyn,ante refers to component of dynamic power that is scaled based on number of active set of antennas only

    - We suggest value of 110 for this part for Category 1

  - Pdyn,joint refers to component of dynamic power that is scaled based on both number of active set of antennas and amount of occupied BW

    - We suggest value of 115 value for this part for Category 1

- PdynamicUL=  sa*P5, where P5 refers to active UL state relative power for the reference configuration.

# References

- RAN1 #110 Chairman Notes