3GPP TSG RAN WG1 meeting #110bis-e                                 R1-2209195

E-meeting, Oct. 10th – 19th, 2022

Title: 	Discussion on NW energy saving performance evaluation

Source: 	ZTE, Sanechips

Agenda Item:	9.7.1

Document for:	Discussion and Decision

Introduction

The definition of base station energy consumption model, evaluation methodology and KPIs are important for the study on the network energy saving techniques. In RAN1 #109-e and RAN1 #110 meetings, some agreements were reached as follows [1][2]:

Agreement(RAN1 #109-e) For evaluation and BS energy consumption modeling purpose, for single CC case, at least the following in table should be considered for reference configuration - Note: other TX-RX RU number and corresponding BS antenna configuration can be considered in SLS assumptions Set 1 FR1; Set 2 FR1; Set 3 FR2. Duplex; TDD; FDD; TDD. System BW; 100 MHz; 20 MHz; 100 MHz. SCS; 30 kHz; 15 kHz; 120 kHz. Number of TRP; 1; 1; 1. Total number of DL TX RUs; 64; (working assumption) 32; 2. Total DL power level; 55dBm; [49dBm] – to be further discussed and finalized in future meetings; 43dBm – to be further discussed and finalized in future meetings EIRP limited to 78dBm – to be further discussed and finalized in future meetings. Total number of UL Rx RUs; 64; (working assumption) 32; 2. Agreement(RAN1 #110) For set 2 FR1 FDD TxRx reference configuration, confirm the WA as 32 in reference configuration. Agreement(RAN1 #110) The total DL power level is 49 dBm for set 2 FR1 FDD reference configuration. Agreement(RAN1 #110) For set 3 FR2 reference configuration, the total DL power level and EIRP limit is set as 33 dBm and 63 dBm respectively. Note EIRP limit is also scaled with the number of TxRU. Agreement(RAN1 #110) For non-sleep mode, the relative power value in power model table for UL reception and/or DL transmission is provided based on reference configuration. Working Assumption (RAN1 #110) For reference configuration set 1, the values are provided as below. FFS set2 and set 3. Power state; Relative Power P; Total transition time T. Deep sleep; 1; 1; Cat 1:50ms; Cat 2: 10s. Light sleep; Cat 1: 25; Cat 2: 2.1; Cat 1: 6 ms; Cat 2: 640 ms. Micro sleep; Cat1: 55; Cat 2: 5.5; 0; 0. Active DL; Cat 1: 280; Cat 2: 32; N.A.; N.A.. Active UL; Cat 1: 110; Cat 2: 6.5; N.A.; N.A.. Agreement(RAN1 #110) For initial evaluations, there is always a non-sleep mode assumed between adjacent sleep modes. Agreement(RAN1 #110) During the transition time period, relative power of sleep mode i is assumed to be consumed. Additional transition energy and total transition time also include energy and time for both ramping down and ramping up. Agreement(RAN1 #110) - The total transition time for set 2 and set 3 is the same as that for set 1. - Companies are encouraged to check the input and values provided in section 2.1.4.2 of R1-2208312 for further determination. Agreement(RAN1 #109-e) At least urban macro is prioritized for FR1. FFS the baseline deployment assumption for FR2. Agreement(RAN1 #110) - For FR1, urban micro can be optionally considered. - For FR2, urban micro is prioritized, with ISD=200 m is assumed. Agreement(RAN1 #109-e) - FTP3 (0.5MB as packet size, 200ms as mean inter-arrival time), FTP3 IM (0.1MB as packet size, 2s as mean inter-arrival time) and VOIP can be considered in the evaluation - FFS: with possible further prioritization, different model between DL and UL, and/or other traffic models that can be optionally considered. - FFS associated scenarios/configurations, e.g. C-DRX. Agreement(RAN1 #110) It is up to company report which traffic model is used among the agreed three traffic models in their evaluations. - Other models may be used as well. Parameter (e.g. packet size and arrival rate) adjustment can be optionally considered and reported. Agreement(RAN1 #109-e) For BS energy consumption evaluation, in addition to the energy saving gain, - At least UPT/UE power consumption/access delay/latency should be considered for performance impact evaluation - Note: this doesn’t necessarily mean that all the above are considered for all evaluation results. However, multiple KPIs are expected to be evaluated for a given technique. And this does not preclude to consider other KPIs when found appropriate for certain techniques/scenarios. Agreement(RAN1 #110) - FFS whether to set exact requirements/QoS target for UPT and/or latency impact - Other KPIs can be optionally reported, conditioned with clear definition/descriptions provided. - Note for potential new channel/signals, e.g. WUS from UE, the assumption for detection reliability at BS side is reported (performance and complexity impact would subject to results and further discussion). Agreement(RAN1 #110) Update the RAN1 agreements with the following changes In the evaluation, - A load (L)% of a cell is a percentage of resources used for UE specific PDSCH/PUSCH. - The following load scenarios are considered. Load scenario; Characteristics. Idle/empty load; - Include cell-specific signals and channels, and - L = 0. low load; - Include cell-specific signals and channels, and - 0 < L≤15. Light load; - Include cell-specific signals and channels, and - 150 < L≤30. Medium load; - Include cell-specific signals and channels, and - 30 < L≤50. For CA, the companies report whether the load is defined per CC or across all CCs.. Agreement(RAN1 #110) - For FR1, adopt the Reference SLS configurations in Annex-A in R1-2208312 as baseline SLS assumptions. - Other carrier frequencies can be optionally considered. - FFS For FR2 adopt the Reference SLS configuration used in Dense Urban Config.B in Table2 of RP-180524 for IMT-2020 with the following clarification/update as initial SLS assumption. - BS antenna configurations - 2 TxRU (M, N, P, Mg, Ng; Mp, Np) = (4,8,2,2,2;1,1) - (dH, dV) = (0.5λ, 0.8λ) (dg,H, dg,V) = (4.0λ, 3.6λ) - Traffic model & UE density - Follow previous agreements with adjusted UE density - Total transmit power per TRxP - Value scaled from that in set 3 reference configuration considering BW - Further adjustment/clarification can be discussed in the next meeting.

According to the agreements, the design principles and basic configurations have been approved. However, there are still many details need to be discussed. In this contribution, more analysis on the network energy consumption model and evaluation methodology are provided, and some basic simulation results are given.

Energy consumption model

The network power consumption model is beneficial for the provision of in-depth understanding of different network energy saving techniques. Till RAN1#110 meeting, the details about network power consumption model, such as, relative power values of the different power states, scaling factors for different configurations, additional transition energy, etc. have not been agreed yet. In this section, these remaining issues are further discussed.

Power state and relative power

- Relative power values for reference configurations

In RAN1 #109-e and RAN1 #110 meeting, the reference configurations for FR1 and FR2 were discussed. In RAN1#110 meeting, the working assumption about the relative power values for reference configuration set 1, i.e., FR1 TDD setting was agreed based on companies’ input. And the working assumption can be confirmed for the evaluation of power saving techniques.

Confirm the working assumption of relative power values of reference configuration set 1.

However, the relative power values for reference configuration set 2 and set 3 need to be further discussed.

As agreed in [1], the reference configurations for FR1 FDD and TDD BS, such as system bandwidth, the number of DL TxRUs and the total DL power level, are different. With all these different assumption for FDD and TDD, it is obvious that the absolute power consumption of FR1 TDD BS and FDD BS is different. However, in this study, a relative power consumption model instead of absolute power consumption model is used, wherein the power values of other states are defined in relative to the deep sleep mode. Even the absolute power values are different for FR1 TDD BS and FDD BS, the ratio between other state and deep sleep state is similar based on the current implementation. Therefore, for the evaluation of network energy saving techniques, the relative power consumption model of reference configuration set 1 can be reused for reference configuration set 2.

The relative power consumption model of reference configuration set 1 can be reused for reference configuration set 2.

In addition, the power values for reference configuration set 2 and set 3 were discussed in [3] and are extracted in Table 1.

Table 1 Relative power values for reference configuration set 2 and set 3

| Power state | Relative Power P for Category 1 |  | Relative Power P for Category 2 |  |
| --- | --- | --- | --- | --- |
|  | Set 2 | Set 3 | Set 2 | Set 3 |
| Deep sleep | 1 | 1 | 1 | 1 |
| Light sleep | 23 | 20 | 2.6 | 1.8 |
| Micro sleep | 50 | 38 | 5 | 3 |
| Active DL | 240 | 152 | 40 | 8.4 |
| Active UL | 90 | 80 | 5.8 | 4.2 |

With the same principle of the outcome of power values of set 1, the relative power values for reference configuration set 2 and set 3 are an average of the values of different companies’ input. From our perspective, the relative power values in Table 1 are also acceptable.

For reference configuration set 2, the power values can be determined by

- Alt 1: reusing the power values for reference configuration set 1;

- Alt 2: using the power values in Table 1.

For the power values of reference configuration set 3, the power values in Table 1 can be used.

- Additional transition energy

For network power consumption model, the additional transition energy can be determined by the following two principles, which are the same with UE power consumption model in TR38.840 [4].

First, the shadow area in Figure 1 can be used as the start point for calculating the additional transition energy. In addition, it should be noted that the switching between micro sleep state and active state does not require additional transition time or energy. Therefore, the power of the micro sleep can be used to replace the active state power when calculating the additional transition energy.

Second, when the conditions on transition to a deeper sleep state (for example, the deep sleep) are met, the power consumption for the base station entering into the deeper-level sleep state should be lower than that of the lighter sleep state.

Figure 1 Transition between active and sleep states

The additional transition energy can be determined with the following two principles,

- the shadow area in Figure 1 can be used as the start point;

- the power consumption for the base station entering into the deeper sleep state should be lower than that of the lighter sleep state.

As shown in [4], both the total transition time and additional transition energy remain the same for difference reference configuration sets. What’s more, according to the last meeting, it is agreed that the total transition time for set 2 and set 3 is the same as that for set 1. Therefore, we think the additional transition energy can also be the same for the three reference configuration sets similar to the UE power saving model to reduce the duplicated discussion. For the difference of power consumption values among different sets, it can be reflected by the power values of active states. Therefore, the above principles should be applied in the three reference configuration sets.

The additional transition energy can be set to the same for the three reference configuration sets.

Assuming that the additional transition energy for light sleep for category 1 and category 2 are Els-1 and Els-2, respectively, the additional transition energy for deep sleep for category 1 and category 2 are Eds-1 and Eds-2, respectively. Then the total energy of the sleep mode for TDD can be calculated as follows.

| The total energy of the sleep mode for TDD |  |
| --- | --- |
| category 1 | category 2 |
| Micro sleep for 6ms = 55*6*2 = 660 Light sleep for 6ms = 25*6 *2+ Els-1 = 300+Els-1 | Micro sleep for 640ms = 5.5*640*2= 7040 Light sleep for 640ms = 2.1*640*2 + Els-2 = 2688+Els-2 |
| Light sleep for 50ms = 25*50*2+Els-1 = 2500+Els-1 Deep sleep for 50ms = 1*50*2+Eds-1 =100+Eds-1 | Light sleep for 10s = 2.1*10000*2 + Els-2 = 42000+Els-2 Deep sleep for 10s = 1*10000*2+Eds-2 = 20000+Eds-2 |

According to the second principle in observation 2, the additional transition energy should meet the following requirements for TDD.

Formula (1):

Formula (2):

In addition, the additional transition energy (unit in relative power*(duration in ms)) for set 1 calculated according to the first principle in observation 1 is shown in Table 2.

Table 2 Additional transition energy calculated by the shadow area

| Power state | Additional transition energy Ei |  |
| --- | --- | --- |
|  | Category 1 | Category 2 |
| Deep sleep | 1350 | 22500 |
| Light sleep | 90 | 1088 |

It is clear that the values in Table 2 meet the requirements of Formula (1) and Formula (2), it is not problematic to use the values in Table 2 for as the additional transition energy for TDD.

The additional transition energy in Table 2 for TDD configuration meets the principles in Observation 2.

As we discussed before, the additional transition energy can be set to be the same for the three reference configuration sets. Therefore, the second principle in observation 2 should also be met for FDD scenarios. For FDD, the total energy of the sleep mode can be calculated as follows, where the power value is assumed to be the same with TDD.

| The total energy of the sleep mode for FDD |  |
| --- | --- |
| category 1 | category 2 |
| Micro sleep for 6ms = 55*6 = 330 Light sleep for 6ms = 25*6 + Els-1 = 150+Els-1 | Micro sleep for 640ms = 5.5*640= 3520 Light sleep for 640ms = 2.1*640 + Els-2 = 1344+Els-2 |
| Light sleep for 50ms = 25*50+Els-1 = 1250+Els-1 Deep sleep for 50ms = 1*50+Eds-1 =50+Eds-1 | Light sleep for 10s = 2.1*10000 + Els-2 = 21000+Els-2 Deep sleep for 10s = 1*10000+Eds-2 = 10000+Eds-2 |

So the additional transition energy should meet the following requirements.

Formula (3):

Formula (4):

According to formula (3) and (4), for FDD, the values in the Table 2 cannot guarantee that the deep sleep is more power efficient than the light sleep. Therefore, a smaller additional energy for deep sleep can be considered to ensure that the transition energy can be used in multiple reference configurations. When the value of Els-1 and Els-2 are 90, 1088, respectively, the additional transition energy for deep sleep for category 1 and category 2 should satisfy Eds-1 < 1290, and Eds-2 <12088.

In conclusion, the value in Table 3 can be used for additional transition energy both for set 1, set 2 and set 3. However, if most companies prefer to directly use the values in the Table 2, we can also accept it.

Table 3 Additional transition energy for different reference configuration sets

| Power state | Additional transition energy Ei |  |
| --- | --- | --- |
|  | Category 1 | Category 2 |
| Deep sleep | 1250 | 12000 |
| Light sleep | 90 | 1088 |

The additional transition energy in either Table 2 or Table 3 can be adopted for both TDD and FDD.

Energy consumption scaling for adaptation

In [3], the summary of power consumption scaling of network adaptation is provided.

Proposed conclusion: Using the below as a base for further discussion/determination of scaling details in the next meeting. Other alts may still be considered. Revised Alt 1-update: At least for FR1 TDD, - the BS power consumption for active DL is provided by - P= Pstatic+Pdynamic - Pstatic: a static part of which the power is not scaled based on reference configurations. Value is to be determined based on - Category 1: - [55, Huawei] - [140, CATT] - Category 2: - [5.5, Huawei] - [1.79, Nokia] - [16, CATT] - Pdynamic: a dynamic part of the power that is scaled based on reference configurations based on sa*Pdyn,ante+sf*spηsf, sp*Pdyn,joint, where - Pdyn,ante is - Category 1: - [0, Samsung] - [57, Huawei] - [110, Intel] - Category 2: - [0, Samsung] - [7.3, Huawei] - [1.62, Nokia] - [12, CATT] - Pdyn,joint is - Category 1: - [225, Samsung] - [84, Huawei] - [115, Intel] - [30, CATT] - Category 2: - [26.5, Samsung] - [9.6, Huawei] - [10.6, Nokia] - [4, CATT] - ηsf, sp is the PA efficiency - For initial evaluations, η= - [0.34, Samsung, Nokia, CATT] - [0.5, Huawei] - [1, Intel, CATT] - The Pdyn,ante and Pdyn,joint should be reported along with η, which may not be perfectly the candidate values in the current list - FFS whether/how to use a non-linear function to represent η. - sa, sf, sp is the percentage of active TRxRUs, the ratio of RF bandwidth and maximum system BW and the ratio of PSD per TxRU between the DL transmission and reference configuration, respectively. - FFS: the BS power consumption for active UL is provided by - P=PstaticUL+sa*PdynamicUL - PstaticUL=Pstatic Revised Alt 3 - The BS power consumption for active DL is provided by Psa,sf,sp=P3+sasfspηsf, spP4-P3 - P3 and P4 are relative power values of micro sleep and active DL transmission, respectively - sf is the resource usage ratio in frequency domain - sp is ratio of simulated DL power level per TxRU between the DL transmission and reference configuration - sa is percentage of active TxRUs with respect to the reference configuration - ηsf, sp is the ratio between a reference PA efficiency and actual PA efficiency depending on the actual transmit power and actual frequency domain usage. - Companies report how to model ηsf, sp. - Note: modelling ηsf, sp as a single value for all different power and frequency resource allocation values is not realistic. - FFS: BS power consumption for active UL - Notes, - In time domain, - when slot level model is provided, a time domain scaling factor is linearly applied on Pdynamic, if applicable, or on P, according to - (1-alpha)*P3 + alpha*P4 where alpha represents the ratio of the number of active DL symbols within a slot to the number of symbols within a slot - The symbol without active DL is to be treated as micro sleep. - Companies to describe how to scale for symbols with different frequency domain allocations. - If an explicit symbol level model is provided, scaling is not applied - (Already agreed) system simulation evaluations can be per slot regardless of detailed approach for calculating symbol-level power consumption - In frequency domain, for at least inter-band CA, the total power consumption of BS is calculated as the sum of the power consumption of each cell - For intra-band CA, a scaling factor of [0.75] is assumed on Pdynamic - In spatial domain, for M-TRP at least with separate RF chains, the total power consumption of BS is assumed as the sum of the power consumption of each TRP.

As discussed in last meeting, the framework of Revised Alt 1-update can reflect the impact of the frequency domain, spatial domain, power domain and the combination of the multi-domain adaption on the power consumption of active DL, which can be taken as starting point.

The framework of Revised Alt 1-update can be taken as a start point for the scaling of DL adaptation.

For the BS power consumption for active DL, Pstatic is the static part of which is not impacted by the DL adaptation in any domain. Regarding the value of Pstatic, the power consumption of micro sleep state is a good choice because the power consumption of micro sleep is the minimum value of the power consumption of active DL. Therefore, the Pstatic should be equal to 55 for category 1 and equal to 5.5 for category 2.

The Pstatic should be the power consumption of micro sleep.

To calculate Pdynamic, the value η, Pdyn,ante and Pdyn,joint should be coupled, the η value should be reported along with Pdyn,ante and Pdyn,joint. When there is no scaling adopted, the total power consumption of the Pdynamic should be equal to (Pactive - Pstatic).

In addition, both spatial domain adaptation and power domain adaptation have impact on the total DL transmission power. However, when the spatial domain techniques are adopted, e.g. adaptation of the active TxRUs, the components corresponding to the de-activated TxRUs will be muted, which is more energy efficient than changing PSD per TxRU with power domain adaptation. Therefore, when the percentage of active TxRUs(sa) and the ratio of PSD per TxRU( sp) is the same (sa= sp<1), the power consumption value of TxRU shut-down should be much smaller than that of changing the PSDs per TxRU.

The energy efficiency of spatial domain adaptation is higher than that of power domain adaptation since the components corresponding to the de-activated TxRUs can be muted.

In addition, even the values of Pstatic, η, Pdyn,ante and Pdyn,joint are different for Category 1 and Category 2 due to the difference in power values, the resultant scaling factors for DL power consumption calculated by Category 1 and Category 2 should be the same in time/frequency/spatial/power domain adaptation.

Scaling factors for DL consumption power calculated by Category 1 and Category 2 should be same in time/frequency/spatial/power domain adaptation.

Based on the above analysis, if the PA efficiency ηsf,  sp=0.5 for simplification, the Pdyn,ante and Pdyn,joint can be 103 and 61 for category 1, 12.5 and 7, for category 2, respectively.

The following values can be used to calculate the power consumption with scaling.

- Pstatic :

- Category 1: [55]

- Category 2: [5.5]

- Pdyn,ante :

- Category 1: [103]

- Category 2: [12.5]

- Pdyn,joint :

- Category 1: [61]

- Category 2: [7]

- ηsf,  sp as the PA efficiency:

- η=0.5

In time domain, when slot level model is provided, a time domain scaling factor is linearly applied on Pdynamic or P and the symbol without active DL is to be treated as micro sleep, i.e. Pstatic. Therefore, the time domain scaling model is according to (1-alpha)*P3 + alpha*P4, i.e. Pstatic+alpha*Pdynamic, where alpha represents the ratio of the number of active DL symbols within a slot to the number of symbols within a slot.

The time domain scaling is (1-alpha)*P3 + alpha*P4 where alpha represents the ratio of the number of active DL symbols within a slot to the number of symbols within a slot

In frequency domain, for the scaling factors of CA, a fixed scaling factor cannot represent all deployments and implementations. In the case of intra-band CA, some RF components are shared among difference cells. For intra-band CA with hardware coupling, the energy consumption of the multiple cells is less than the sum of the power consumption of each cell.

However, for a typical implementation of inter-band CA, the RF chains and other components of difference cells are independent. Therefore, for inter-band CA, the total energy consumption of multiple cells is basically equal to the sum of the power consumption of each cell.

In frequency domain, for at least inter-band CA, the total power consumption of BS is calculated as the sum of the power consumption of each cell.

Evaluation methodology

The evaluation methodology is important for the evaluation of performance gains and identification of efficient network energy saving techniques. In this section, the evaluation methodology, such as the traffic load, the evaluation assumptions for network energy saving are discussed.

- Traffic model

As agreed in RAN1#109-e and RAN1#110 meeting, FTP3 (0.5MB as packet size, 200ms as mean inter-arrival time), FTP3 IM (0.1MB as packet size, 2s as mean inter-arrival time) and VOIP can be considered in the evaluation. Other models with parameter (e.g. packet size and arrival rate) adjustment can be also considered and reported by companies.

According to the agreed traffic models, we have the following observations:

- The packet size of FTP3 or IM is too large for a typical network energy saving scenarios. For example, if we assume all the time/frequency resources are allocated for one UE for data transmission, and the maximum MCS index of 256QAM, 4 MIMO layers can be used in an extremely good channel condition, the number of bits can be transmitted within in one slot is 0.25M, which is far less than the packet size of 0.5M Bytes and 0.1 M Bytes.

- The packet size of VOIP (i.e. 38 bytes) is too small to support any higher load.

Therefore, we think the traffic model by adjusting the parameters of packet size and arrival rate to evaluate different load scenario flexibly and efficiently is allowed in the evaluations. For example, the traffic model of FTP3 with 20KByte packet size can be used for evaluations.

The packet sizes of agreed traffic models (i.e. FTP3, FTP3 IM and VOIP) are either too small or too large packet size for the SLS evaluation of network energy saving techniques.

Traffic models with parameter adjustment, e.g. FTP3 with the packet size of 20K Bytes can be used for evaluation on NW ES techniques.

For the UL transmissions, the agreed traffic models with adjustment of packet size and arrival rate can be considered in the evaluation.

The agreed traffic models with adjustment of packet size and arrival rate can be considered in the UL transmission evaluation.

- KPIs

For the KPIs, the following agreements were achieved. For the FFS bullet in the agreement of RAN1#110 meeting, since it is not easy to reach consensus about the exact target requirement, we think it is sufficient to report the impact on UPT as (UPTenhanced-UPTbaseline)/UPTbaseline, where average UPT is assumed. Similar rule can be applied to latency impact, if reported.

The impact on UPT is calculated as (UPTenhanced-UPTbaseline)/UPTbaseline, where average UPT is assumed.

Agreement(RAN1 #109-e) For BS energy consumption evaluation, in addition to the energy saving gain, - At least UPT/UE power consumption/access delay/latency should be considered for performance impact evaluation - Note: this doesn’t necessarily mean that all the above are considered for all evaluation results. However, multiple KPIs are expected to be evaluated for a given technique. And this does not preclude to consider other KPIs when found appropriate for certain techniques/scenarios. Agreement(RAN1 #110) - FFS whether to set exact requirements/QoS target for UPT and/or latency impact - Other KPIs can be optionally reported, conditioned with clear definition/descriptions provided. - Note for potential new channel/signals, e.g. WUS from UE, the assumption for detection reliability at BS side is reported (performance and complexity impact would subject to results and further discussion).

Preliminary system-level simulation results

Time domain

In this section, we provide some preliminary simulation results of the base station power consumption for techniques in time domain. The power consumption model is based on the analysis above and the additional assumption is listed as below.

- UE’s traffic: FTP3 for TDD and FTP3 IM with 200ms mean inter-arrival time for FDD.

- RU: 5%~40%.

Herein the cases in the simulation are shown in Table 4. Simulation results of network energy savings gains and average UPT gain for all the cases under different resource utilization are shown in Figure 2 based on the simulation assumption shown in Table A1 in Appendix. Wherein the UPT gain is calculated as (UPTenhanced-UPTbaseline)/UPTbaseline.

Table 4 SSB/SIB configurations

| Configuration | Simulation assumption |  |
| --- | --- | --- |
|  | TDD | FDD |
| SSB | 20ms, 8 beams 2 SSB per slot, 4 symbols for each SSB | 20ms, 4 beams 2 SSB per slot, 4 symbols for each SSB |
| SIB1 | 40ms, 8 beams, 1 SIB per slot | 20ms, 4 beams, 1 SIB per slot |

Figure 2 Evaluation results of SSB-less and SIB-less scheme

It is observed from the system-level simulation results that the SSB-less and SIB-less scheme can obtain 6.5% ~ 24.2% energy saving gain for TDD and 14.9%~45.5% energy saving gain for FDD in the cases of RU=5%~40%. It can be also observed that SSB-less/SIB-less scheme can improve the UPT performance.

It is observed that the SSB-less and SIB-less scheme can obtain 6.5% ~ 24.2% energy saving gain for TDD and 14.9%~45.5% energy saving gain for FDD in the cases of RU=5%~40% according to the system-level simulation results.

Frequency domain

In this section, we provide some preliminary simulation results of the base station energy saving gain and average UPT gain for techniques in frequency domain. The assumptions of traffic model and SSB configurations are the same as that in Table 4.

Figure 3 Evaluation results of SSB-less for inter-band CA

It is observed from the system-level simulation results shown in Figure 3 that the SSB-less SCell scheme can obtain 5%~14.8% energy saving gain in the cases of RU=5%~25% for TDD and 9.4%~26.4% energy saving gain in the case of RU=5%~15% for FDD. It can be also observed that SSB-less scheme can improve the UPT performance.

It is observed that the SSB-less SCell scheme can obtain 5%~14.8% energy saving gain in the cases of RU=5%~25% for TDD and 9.4%~26.4% energy saving gain in the case of RU=5%~15% for FDD according to the system-level simulation results.

Spatial domain

In this section, we provide some preliminary simulation results of the base station power consumption. The power model is based on the analysis above and the additional assumption is listed as below.

- UE’s traffic: FTP3 with 20KByte packet size.

- RU: 10%~35%.

Herein the cases in the simulation are shown in Table 5. Simulation results of network energy savings gains and average UPT for all the cases under different resource utilization are shown in Figure 4 based on the simulation assumption shown in Table A1 in Appendix.

Table 5 BS antenna configurations in SLS

| Case | Description |
| --- | --- |
| 64T | BS antenna configuration: (M, N, P, Mg, Ng) = (8, 8, 2, 1, 1); Max transmission power=55dBm |
| 32T | BS antenna configuration: (M, N, P, Mg, Ng) = (8, 4, 2, 1, 1); Max transmission power=52dBm |

Figure 4 Energy saving gain and UPT impact of antenna reduction

It is observed from the SLS results for TDD that the antenna reduction scheme can obtain 17.7%~26.4% energy saving gain at 3.7%~10.9% UPT loss in the cases of RU=10%~35% with some UPT loss.

It is observed that the antenna reduction scheme can obtain 17.7%~26.4% energy saving gain at 3.7%~10.9% UPT loss in the cases of RU=10%~35% according to the system-level simulation results for TDD.

Conclusion

In this contribution, we discuss the energy consumption model, the evaluation methodology. Some preliminary evaluation results are also provided. We have the following observations and proposals.

Observation 1: The relative power consumption model of reference configuration set 1 can be reused for reference configuration set 2.

Observation 2: The additional transition energy can be determined with the following two principles,

- the shadow area in Figure 1 can be used as the start point;

- the power consumption for the base station entering into the deeper sleep state should be lower than that of the lighter sleep state.

Observation 3: The additional transition energy in Table 2 for TDD configuration meets the principles in Observation 2.

Observation 4: The energy efficiency of spatial domain adaptation is higher than that of power domain adaptation since the components corresponding to the de-activated TxRUs can be muted.

Observation 5: The packet sizes of agreed traffic models (i.e. FTP3, FTP3 IM and VOIP) are either too small or too large packet size for the SLS evaluation of network energy saving techniques.

Observation 6: It is observed that the SSB-less and SIB-less scheme can obtain 6.5% ~ 24.2% energy saving gain for TDD and 14.9%~45.5% energy saving gain for FDD in the cases of RU=5%~40% according to the system-level simulation results.

Observation 7: It is observed that the SSB-less SCell scheme can obtain 5%~14.8% energy saving gain in the cases of RU=5%~25% for TDD and 9.4%~26.4% energy saving gain in the case of RU=5%~15% for FDD according to the system-level simulation results.

Observation 8: It is observed that the antenna reduction scheme can obtain 17.7%~26.4% energy saving gain at 3.7%~10.9% UPT loss in the cases of RU=10%~35% according to the system-level simulation results for TDD.

Proposal 1: Confirm the working assumption of relative power values of reference configuration set 1.

Proposal 2: For reference configuration set 2, the power values can be determined by

- Alt 1: reusing the power values for reference configuration set 1;

- Alt 2: using the power values in Table 1.

Proposal 3: For the power values of reference configuration set 3, the power values in Table 1 can be used.

Proposal 4: The additional transition energy can be set to the same for the three reference configuration sets.

Proposal 5: The additional transition energy in either Table 2 or Table 3 can be adopted for both TDD and FDD.

Proposal 6: The framework of Revised Alt 1-update can be taken as a start point for the scaling of DL adaptation.

Proposal 7: The Pstatic should be the power consumption of micro sleep.

Proposal 8: Scaling factors for DL consumption power calculated by Category 1 and Category 2 should be same in time/frequency/spatial/power domain adaptation.

Proposal 9: The following values can be used to calculate the power consumption with scaling.

- Pstatic :

- Category 1: [55]

- Category 2: [5.5]

- Pdyn,ante :

- Category 1: [103]

- Category 2: [12.5]

- Pdyn,joint :

- Category 1: [61]

- Category 2: [7]

- ηsf,  sp as the PA efficiency:

- η=0.5

Proposal 10: The time domain scaling is (1-alpha)*P3 + alpha*P4 where alpha represents the ratio of the number of active DL symbols within a slot to the number of symbols within a slot.

Proposal 11: In frequency domain, for at least inter-band CA, the total power consumption of BS is calculated as the sum of the power consumption of each cell.

Proposal 12: Traffic models with parameter adjustment, e.g. FTP3 with the packet size of 20K Bytes can be used for evaluation on NW ES techniques.

Proposal 13: The agreed traffic models with adjustment of packet size and arrival rate can be considered in the UL transmission evaluation.

Proposal 14: The impact on UPT is calculated as (UPTenhanced-UPTbaseline)/UPTbaseline, where average UPT is assumed.

References

Chair's notes RAN1#109-e, 2022-05.

Chair's notes RAN1#110-e, 2022-08.

R1-2208312. FL summary for Post-110-R18- NW_ES2, Moderator (Huawei), 2022-08.

3GPP TR 38.840, V16.0.0, 2019-06.

Appendix

Table A1 Evaluation assumptions on basic simulation parameters for UMa scenario

| Parameters | Value |  |
| --- | --- | --- |
| Duplex Mode / Simulation bandwidth | TDD/100 MHz | FDD/20 MHz (equal split of 10MHz for UL and DL) |
| Scenario | Urban Macro; Hexagonal grid, 7 macro sites, 3 sectors per site |  |
| Inter-BS distance | 500m |  |
| Min. BS - UE distance | 35m |  |
| Carrier frequency | 4GHz | 2.1GHz |
| SCS | 30KHz | 15KHz |
| TDD pattern | DDDSU | / |
| BS Antenna Configuration | 64Tx antenna ports, and (dH, dV) = (0.5λ, 0.5λ); 64Tx: (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1; 4,8) The antenna tilt is 12 degrees. | 32Tx antenna ports, and (dH, dV) = (0.5λ, 0.8λ); 32Tx: (M, N, P, Mg, Ng) = (8,8,2,1,1) The antenna tilt is 12 degrees. |
| UE Antenna Configuration | 4 Rx antenna ports, and dH = 0.5λ 4 Rx: (M, N, P, Mg, Ng; Mp, Np) = (1,2,2,1,1; 1,2); |  |
| Total DL power level | 55dBm | 49dBm |
| UE transmit power | 23 dBm |  |
| Antenna Height | 25 m for BS and 1.5 m for UE |  |
| Antenna Element Gain | 8dBi for BS and 0 dBi for UE |  |
| Receiver Noise Figure | 5 dB for BS and 9 dB for UE |  |
| UE distribution | Uniform in cell; 80% of users are indoor, 20% of users are outdoor; Use 3km/h for modeling fading channel |  |
| Number of UEs per cell | Depends on RU requirement |  |
| Scheduling Algorithm | SU-MIMO + PF |  |
| HARQ/repetition | HARQ retransmission |  |
| Channel estimation | Realist |  |
| PDCCH overhead | 1/7 (2symbols per 14symbol) |  |
| DMRS overhead | 1 symbol per 14symbol |  |
| BS receiver | MMSE-IRC |  |