3GPP TSG RAN WG1 #110bis-e                                              R1-2209215

e-meeting, Oct 10th – 19th, 2022

Source:         ZTE

Title:           Discussion on carrier phase measurement based positioning

Agenda item:    9.5.2.2

Document for:	Discussion and Decision

Introduction

In 3GPP RAN1#110 meeting, some agreements on NR carrier phase positioning were reached as follows [1].

Agreement Endorse the templates in section 17 under (H)(Round 1) Proposal 17-1 in R1-2207690 to collect carrier-phase based positioning simulation results, with the following notes: - The TR editor can adjust the sections/sub-sections arrangement - Adjust the titles of the tables to refer to NR carrier-phase based positioning - The detailed rows of the tables can be further discussed Agreement In the evaluation of NR carrier phase positioning, the following frequency errors can be considered, which are modeled independently for each UE and each TRP: - Initial Residual CFO (is the same for one measurement instances [or multiple phase measurement instances]): - Ideal: 0 (UE/TRP) - Practical: uniform distribution within - [-30, +30] Hz (FR1, UE), [-100, +100] Hz (FR1, UE), - [-120, +120] Hz (FR2, UE), [-400, +400] Hz (FR2, UE), - [-10, +10] Hz (for each TRP, FR1), - [-40, +40] Hz (for each TRP, FR2). - Oscillator-drift (is the same for one or multiple phase measurement instances for positioning fix): - Ideal: 0 (UE/TRP) - Practical: uniform distribution within [-0.1, 0.1] ppm (UE), [-0.02, +0.02] ppm (each TRP) within measurement duration - Note: The Doppler frequency can be determined based on the UE speed in the evaluation assumption. Agreement In the evaluation of NR carrier phase positioning, the offset between the initial phase of the transmitter and the initial phase of the receiver can be modeled as a random variable uniformly distributed within [0, X]. - Possible values of X: 2pi - Other values FFS Agreement In the evaluation of NR carrier phase positioning, the antenna reference point (ARP) location error of a TRP can be modeled as follows: - Ideal: no ARP error - Practical: a zero-mean, truncated Gaussian distribution with zero mean and standard deviation of T=[1, 5] cm truncated to 2T in each of (x, y, z) direction

In this contribution, we provide our views on carrier phase measurement based positioning techniques from RAN1 perspective.

On carrier phases positioning with single carrier

2.1 Theory of carrier phase based solution for RAT-dependent positioning

For easy description of the principle of carrier phase based positioning for wireless based positioning, the following Figure 1 is used as an example. The location of the target UE is to be measured. Two base stations will transmit PRS simultaneously (without any timing error for simplification).

Figure 1   Positioning network

For a UE, the carrier phase measured on signal from gNB i is

(Equ. 1)

Wherein, the  is the wave length of radio signal, the  is the fractional part of carrier phase (e.g., measurement from PLL), the N is the integer part of carrier phase, the  is the real distance between UE and gNB (LOS distance), the  is measurement noise or error.

The basic principle of this carrier phase positioning method is, one initial UE location can be reached via TOA/TDOA/AOA/AOD, etc. After that, similar to TDOA, a differential is applied to the distance equation. Then, an integer search is performed with measurements from multiple base stations, where optimal estimation with least error is performed. Finally, the location of UE and a set of integer part of carrier phase (N) can be reached (with least error).

As shown in Figure 1, the coordinate of the target UE is denoted as (x,y), the coordinate of gNB is (xi,yi), then the distance between UE and ith gNB is

(Equ. 2)

The square and expansion of Equ. 2 is

(Equ. 3)

Let  , then

(Equ. 4)

For the distance between UE and gNB, a differential between the 2nd gNB and the 1st gNB is

(Equ.5)

After moving some items based on Equ.5, the following can be reached.

(Equ. 6)

Similarly, a differential between the nth gNB and the 1st gNB can also be achieved as the following equation.

(Equ. 7)

For the equation above, it can be re-written as the following.

(Equ. 8)

wherein, ，， .

For each combination of {N1, …, Nn}, the position of UE (i.e., X=[x, y]’) can be achieved.

(Equ. 9)

Wherein, the AT is the transposition matrix of A.

An error function can be computed for each combination values of {N1, …, Nn} as the following. After searching all combination values of {N1, …, Nn}, the optimal combination values of {N1, …, Nn } with minimum error function value can be searched. Then, the coordinate (x, y) corresponding the optimal combination of {N1, …, Nn } is determined as the finally estimated UE location.

(Equ. 10)

Wherein,

(Equ. 11)

For the integer N searching, the following procedure is applied.

for(N1) for(N2) for(N3) for(NM) /// Where the M is the number of gNB that are involved in Positioning // To store N1 ~ NM // To calculate UE’s coordinates (x,y) // To calculate distance error under different combination of Ni. end end end end // To find a combination of Ni with least error of distance

It should be noted that, if the initial location of UE were estimated close to its actual location, it may be much easier for UE or LMF to search the optimal UE location.

In other words, large search range of integer values N will lead to high complexity at the receiver side. With the above method, we provide some simulation results as shown in the Figure 2 where we restrict Nn (integer multiples of wave between UE and nth gNB) values within a limited range for lower complexity, and the initial UE location is determined by TDOA method. If the real Nn value is beyond the limited range, large location errors may be caused.

In the simulation, multiple searching ranges of integer values N between UE and TRPs are used, no phase error at either gNB side or UE side is assumed, i.e. it is assumed that UE can achieve perfect phase between itself and each TRP. More details of the simulation assumptions can be found in Annex.

From the simulation results, it can be observed that, the wider range of the searching integer values, the better positioning performance is provided. Hence, we have the following observation.

Observation 1: In InF-SH, assuming receiver can get perfect phase between itself and the transmitter, very high positioning accuracy can be achieved, especially under a large searching range of integer values.

Figure 2   Different positioning accuracy under different integer range with perfect/ideal Φ

2.2 On PRU

For ideal synchronization (SYNC) of frequency, ideal timing align and perfect estimation of carrier phase, there is no need of PRU. However, in most real cases, it is hard to achieve these conditions.

As the following figure, for gNB k, (Fixed) UE 1 (PRU), UE 2 with coordinates of (x,y), the distance equations with carrier phase are as the following two equations.

(Φ1k+N1)λ=d1k                                   (Equ. 12)

(Φ2k+N2)λ=d2k                                   (Equ. 13)

Figure 3   Positioning with PRU (fixed UE)

After a differential between these two equations, the following equations can be reached.

(Φ2k−Φ1k)λ+(N2−N1)λ=d2k−d1k                      (Equ. 14)

(Φ2k−Φ1k)λ+(N2−N1)λ+d1k=d2k=(x−xk)2+(y−yk)2           (Equ. 15)

(x−xk)2+(y−yk)2=[(Φ2k−Φ1k)λ+(N2k−N1k)λ+d1k]2            (Equ. 16)

Similarly, for gNB j and UE 2, the following equation can be reached.

(x−xj)2+(y−yj)2=[(Φ2j−Φ1j)λ+(N2j−N1j)λ+d1j]2            (Equ. 17)

After a differential between these two gNB (i.e., Equ. 13 and 14), the following equations can be reached.

−2xj−xkx−2yj−yky+xj2+yj2−xk2+yk2=[(Φ2j−Φ1j)λ+(N2j−N1j)λ+d1j]2−[(Φ2k−Φ1k)λ+(N2k−N1k)λ+d1k]2    (Equ. 18)

−2xj−xkx−2yj−yky=[(Φ2j−Φ1j)λ+(N2j−N1j)λ+d1j]2−[(Φ2k−Φ1k)λ+(N2k−N1k)λ+d1k]2−xj2+yj2+xk2+yk2    (Equ. 19)

Similar to that in Section 2 in our previous TDoc R1-2203626, with integer search, the coordinates of UE (x, y) can be computed.

With PRU, the carrier phase error from gNBs can be alleviated. That is, the item Φ2j−Φ1j has no carrier phase error from gNB j, likewise for gNB k.

With this method, a simulation is performed as the following figures. From these figures, it can be observed that, the PRU can improve carrier phase error and positioning accuracy. Hence, we have the following observation.

Observation 2: PRU can alleviate the random initial phase error at gNB side from (0~2π) to about (0~0.02*2π) (i.e., 98% reduction).

Figure 4   Carrier phase error estimated (Left: at gNB, Right: at UE)

Figure 5   Positioning accuracy with / without PRU in case of  Φerror = (0~2π)

2.3 On dual differential with PRU

For easy description of the principle of carrier phase based positioning with dual differential, the following Figure 1 is used as an example. The location of the target UE is to be measured. Two base stations will transmit PRS (with carrier phase offset, for simplification).

Figure 6  Example for positioning with dual differential

In this figure, the location of UE 2 with coordinates (x,y) is to be determined. The coordinates of gNB j are (xj,yj). The coordinates of gNB k are (xk,yk). For UE 2, the distance between UE 2 and gNB j is

(Equ. 20)

Where the Φ2j is the carrier phase between UE 2 and gNB j, the ΦDriftj is the carrier phase error of gNB j, the N2j is the integer part of carrier phase between UE 2 and gNB j, the d2j is the distance between UE 2 and gNB j. It should be noted that, the wave length λ can be the virtual wave length in the section above.

Similarly, for gNB k, the following equation can be reached.

(Equ. 21)

If the Equ. 21 is subtracted by Equ. 20, then the following equation can be reached.

(Equ. 22)

Similarly, for UE 1 (PRU), the following equation can be reached.

(Equ. 23)

If the Equ. 22 is subtracted by Equ. 23, then the following equation can be reached.

(Equ. 24)

That is,

(Equ. 25)

From Equ. 25, it can be observed that, for UE 2, the item cancels the initial random phase error at UE2 side, likewise for UE1. Furthermore, the carrier phase error at TRPs can be removed by dual differential.

Now, by moving items in Equ. 25, the following equation can be reached.

(Equ. 26)

By linearization of Equ. 26 (e.g., via Taylor expansion), the following equation can be reached.

(Equ. 27)

where, , .

Now, by moving items in Equ. 27, the following equation can be reached.

(Equ. 28)

After that, the UE location (x0-∆x,y0-∆y) can be solved via  with multiple equations from multiple sites (e.g., in [2]), in where, the initial UE location (x0,y0) can be reached via TDOA (or other positioning method). The simulation results verified the previous observation.

Figure 7   Positioning accuracy with / without PRU in different phase error conditions

As shown in Figure 7, the phase error exerts great impact on the positioning accuracy when the PRU is not adopted, and larger phase error will decrease the positioning performance. On the country, PRU assisted positioning method can mitigate the phase error, and the variation of phase error has little impact on the performance. It is noted that the Taylor expansion introduces some error because the second order, third order, etc. are omitted as comparison between the left and the right figures in Figure 7.

Figure 8   Positioning accuracy comparison in different phase error conditions

Figure 8 demonstrates the superiority of dual differential with PRU especially when the system has large phase error. Through theoretical analysis and simulation results, we have the following observation.

Observation 3: The carrier phase error from initial random phases at both TRP and UE sides can be mitigated by dual differential operation with the help of PRU.

On carrier phases positioning with multiple frequency sections

For easy description of the principle of carrier phase based positioning with multiple carriers or frequency sections within one carrier, two frequency sections are taken as an example as the following figure and distance equations. It is assumed that these two frequency sections come from the same gNB (or TRP) with identical affect on propagation distance (dError).

Figure 9  Example for virtual wave length from two CP

(Equ. 29)

(Equ. 30)

Wherein, the λ is the wave length of radio signal, the Φ is the fractional part of carrier phase estimation, the ΦDrift is the carrier phase error, the N is the integer part of carrier phase, the d is the real distance between UE and gNB (LOS distance).

For Equ. 29 and Equ. 30, if they are divided by wave length λ at both sides, there will be

(Equ. 31)

(Equ. 32)

If the Equ. 32 is subtracted by Equ. 31, there will be

(Equ. 33)

If the Equ. 33 is multiplied by a virtual wave length λv=1/(1/λ2−1/λ1) , there will be

(Equ. 34)

Where, the ΔΦ=Φ2-Φ1, ΔN=N2-N1 .

From Equ. 34, it can be observed that, the carrier phase error (ΦDrift) from two frequency sections in one carrier within one site (or one gNB) is removed by single difference. For other gNB being involved in positioning, similar equations can be deduced. After that, equations in our previous TDoc (R1-2205903) [2] can be used to compute the location of UE. With this idea, a simulation is performed as the following figure. From this figure, it can be observed that, the multiple CP within one carrier is helpful for improvement of positioning accuracy.

Observation 4: The carrier phase error within one site can be removed by single differential operation for the case of carrier phase method from multiple frequency sections in one carrier.

Proposal 1: For multiple segments CP report within a carrier, center frequency or λ should be informed for each of each segment.

It should be noted that, if the virtual wave length λv=1/(1/λ2−1/λ1) were large enough (e.g., 300m, for Indoor Factory), then the integer part (ΔN) is very small (e.g., zero). Hence, the integer search will be easy. However, it should also be noted that, the positioning accuracy will heavily depend on the accuracy of carrier phase differential (ΔΦ). Hence, we have the following observation.

Observation 5: An appropriate scope of integer should be given for integer search.

Figure 10  Simulation results for virtual wave length from two CP

Simulation with practical error(s)

This section deals with three error models agreed in the last RAN1#110 meeting. That is, frequency error (e.g., CFO, Oscillator-drift, Doppler shift, etc), initial phase error, ARP error.

4.1 Frequency error

4.1.1  Under perfect phase

If there were frequency error on one carrier, it will cause carrier phase error as the following.

ΔΦ=2π∗CFO∗[0~(FFT∗CombSize−1)]∗Ts                   (Equ. 35)

Where, FFT=4096 for BW=100MHz@SCS=30kHz, CombSize=2, Ts=1/(FFT*SCS).

From this equation, it can be observed that, the maximum ΔΦ is 0.0067*2π=0.0419 Rad for the case of CFO=100Hz.

Observation 6: The maximum ΔΦ is 0.0067*2π=0.0419 Rad for CFO=100Hz, BW=100MHz, SCS=30kHz, CombSize=2.

With the frequency error model agreed in the last RAN1#110 meeting, simulations are performed as the following figure. From this figure, it can be observed that, frequency error has minor impact on carrier phases positioning. Hence, we have the following observation.

Observation 7: A small frequency error has minor impact on CPP even under perfect Φ.

Figure 11  Positioning accuracy under different frequency error (for ideal CP)

4.1.2  Under practical phase estimation

In practice, the carrier phase measurement may have error as shown in the following figure (left). From this figure, the carrier phase error is about 0.037*2π=0.2325 Rad@CDF=90% which is much larger than the phase error from CFO ( 0.0067*2π=0.0419 Rad in section 4.1.1). Hence, we have the following observation.

Observation 8: The phase error ΔΦ=0.0067*2π=0.0419 Rad from CFO is much less than the phase estimation error@CDF=90% which is about 0.037*2π=0.2325 Rad.

With the frequency error model agreed in the last RAN1#110 meeting, simulations are performed as the following figure (right). From this figure, it can be observed that, frequency error has little impact on carrier phases positioning. Hence, we have the following observation.

Observation 9: A small frequency error has negligible impact on CPP under practical Φ.

Figure 12  CP estimation error and positioning accuracy under different frequency error (for practical CP)

4.2 Initial phase offset between Tx/Rx

In practice, there may exist initial phase offset between transmitter and receiver, e.g., a random value from [0, 2π] as agreed in the last RAN1#110 meeting. With this phase error model, simulations are performed as the following figure. From this figure, it can be observed that, a larger phase error will bring a worse positioning accuracy. Hence, we have the following observation.

Observation 10: More phase estimation error cause larger location error in the case without help of PRU or differential operation.

Figure 13  Positioning accuracy with random carrier phase error at UE (No phase calibration)

4.3 ARP error

4.3.1 Under perfect CP

For ARP error, the RAN1#110 agreement indicates that “a zero-mean, truncated Gaussian distribution with zero mean and standard deviation of T=[1, 5] cm truncated to 2T” is applied for ARP error model. From this error model, the maximum 2D and 3D distance (between UE and TRP) error Δdmax are

∆dmax=2∗T∗2                                    (Equ. 36)

∆dmax=3∗T∗2                                    (Equ. 37)

Where the T is agreed in RAN1#110.

Figure 14   Description of Δdmax

From the equations above, it can be observed that, the maximum Δd is 0.1414m (2D) or 0.1732m (3D) for ARP error=5 cm. Hence, we have the following observations.

Observation 11: The maximum Δd is 0.1414m (2D) or 0.1732m (3D) for ARP error=5 cm.

Observation 12: The corresponding ΔΦ is Δdmax /λ=((0.1414/0.0857) mod 1.0)*2 π=0.6511*2 π=4.091 Rad (for 2D) for single Φ@3.5GHz, or 0.0472*2 π=0.2966 Rad (for 2D) for dual Φ@100MHz bandwidth.

With the frequency error model agreed in the last RAN1#110 meeting, simulations are performed as the following figure. From this figure, it can be observed that, ARP error has large impact on carrier phases positioning. Hence, we have the following observations.

Observation 13: A small ARP error has large impact on CPP under perfect Φ, especially, for a short wave length.

Observation 14: A long virtual wave length is helpful for alleviating the impact from ARP error.

Figure 15  Positioning accuracy under different ARP error (for ideal CP)

4.3.2 Under practical CP

In practice, the carrier phase may have estimation error. In addition, because the distance error Δd (<0.1414m for 2D) from ARP error is less than that achievable positioning accuracy, it is submerged under achievable positioning accuracy. Hence, the ARP error has significant impact on positioning accuracy. Hence, we have the following observation.

Observation 15: A small ARP error has significant impact on CPP under practical Φ, especially, for a short wave length.

Figure 16  Positioning accuracy under different ARP error (for practical CP)

Specification effort on carrier phase based positioning

5.1 Specific measurement results that should be reported

For the specification effort on carrier phase based positioning, the following questions should be taken into consideration.

- How the carrier phase (Φ) is measured? As described in Section 3.2 of [2], it can have three potential methods. One method of them (or other method) should be specified in the future.

- What measurement results should be reported? The integer search is time consuming. It is appropriate that the UE (or the network) suggests an initial location to determine the scope of integer N. By the way, the UE (or the network) can also suggest the scope of integer N. Hence, we have the following proposal.

Proposal 2: The scope of integer N should be configured / reported for carrier phase based positioning.

As discussed in previous meetings (RAN1#109-e, RAN1#110), the carrier phase based positioning may be an independent or dependent positioning technology. If it were a dependent positioning technology, it can be combined with other positioning technologies (such as the timing based positioning technologies, e.g., TDOA). The benefit is, an initial integer N0, which is used for integer search, can be deduced from the timing based positioning technology. By the way, initial coordinates (x0,y0) , which are used in Taylor expansion, can also be deduced from the timing based positioning technology. Hence, we have the following proposal.

Proposal 3: Phase or phase difference report should be supported at least for TDOA.

A gNB (or TRP) or UE may have several antennas. Different antenna may have different phase error in measurement when performing carrier phase measurement. As the following figure, it is appropriate to measure carrier phase with the antenna(s) within a phase error group (PEG). Hence, we have the following proposal.

Proposal 4: The same Phase Error Group(PEG) should be used for CP measurement.

Figure 17  Conception for PEG

In general, there might be multi-PRS transmitted between the same TRP and the UE pair with different (sub-)carriers in a positioning process, and the measured carrier phase of these different (sub-)carriers might be different. At the same time, the number of PRS carrier phase, physical channel environment and PRS propagation multi-path could affect the accuracy of carrier phase evaluation. Therefore, a UE can report the uncertainty of phase measurement based on the above factors, which provide the corresponding positioning range, further evaluate the rationality of other positioning techniques. Hence, we have the following observation.

Observation 16: The UE can report the carrier phase with corresponding phase uncertainty, to evaluate the rationality of other positioning techniques.

5.2 Mapping between the scope of integer and measurement uncertainty

5.2.1 Measurement uncertainty in TDOA

In current OTDOA positioning process, the UE will send a ProvideLocationInformation message to the server to transfer location information in LPP. And the corresponding message in OTDOA Location Information Elements includes a field that specifies the OTDOA measurement quality, specifies the UE’s best estimate of the uncertainty in OTDOA (or TOA) measurement (unit: m). The uncertainty of this location estimation can be used to determine the scope of integer N of carrier phase assisted positioning. Hence, we have the following proposal.

Proposal 5: The scope of integer N in OTDOA could be configured based on the measurement uncertainty.

5.2.2 Measurement uncertainty in AOA/AOD

In a general positioning process, the NR-DL-PRS-AssistanceData includes a field named as nr-DL-PRS-ExpectedAoD-or-AoA, which specifies the expected AoD or AoA towards UE together with uncertainty. The uncertainty of this angle estimation can be used to determine the scope of integer N of carrier phase assisted positioning. Hence, we have the following proposal.

Proposal 6: The scope of integer N in AOA/AOD could be configured based on the angle measurement uncertainty.

5.3 Multi-carrier phase measurement

5.3.1 The scope of integer for different carriers

The distance range between a TPR and UE indicates by the same scope of integer for different carrier may be different. Take Carrier center frequency Fc=3.5GHz, frequency band B=100MHz as an example, the distance range specified using scope of integer [90, 110] for carrier Fc is [771.3cm, 942.7cm], for carrier Fc +B is [749.7cm, 916.3cm]. Therefore, the scope of integer for different carrier could be selected based on the frequency feature. Hence, we have the following proposal.

Proposal 7: The scope of integer N for different carrier could be selected based on the frequency feature.

5.3.2 Number of reported carrier phases

In general, the carrier phase measurement noise, CFO, PCO, etc., will affect the carrier phase positioning result. Single CP measurement may result in some accidental errors/positioning error, further deteriorate the positioning performance, but on the other hand, multiple CP measurement could increase the overhead. A trade-off between positioning accuracy and measurement overhead could be determined by the number of carrier phase to be reported. Hence, we have the following proposal.

Proposal 8: The number of carrier phases to be reported could be determined based on the positioning accuracy and measurement overhead requirements.

Conclusion

In this contribution, we provide our views on carrier phase measurement based positioning techniques, observations and proposals are listed as follows:

Observation 1: In InF-SH, assuming receiver can get perfect phase between itself and the transmitter, very high positioning accuracy can be achieved, especially under a large searching range of integer values.

Observation 2: PRU can alleviate the random initial phase error at gNB side from (0~2π) to about (0~0.02*2π) (i.e., 98% reduction).

Observation 3: The carrier phase error from initial random phases at both TRP and UE sides can be mitigated by dual differential operation with the help of PRU.

Observation 4: The carrier phase error within one site can be removed by single differential operation for the case of carrier phase method from multiple frequency sections in one carrier.

Observation 5: An appropriate scope of integer should be given for integer search.

Observation 6: The maximum ΔΦ is 0.0067*2π=0.0419 Rad for CFO=100Hz, BW=100MHz, SCS=30kHz, CombSize=2.

Observation 7: A small frequency error has minor impact on CPP even under perfect Φ.

Observation 8: The phase error ΔΦ=0.0067*2π=0.0419 Rad from CFO is much less than the phase estimation error@CDF=90% which is about 0.037*2π=0.2325 Rad.

Observation 9: A small frequency error has negligible impact on CPP under practical Φ.

Observation 10: More phase estimation error cause larger location error in the case without help of PRU or differential operation.

Observation 11: The maximum Δd is 0.1414m (2D) or 0.1732m (3D) for ARP error=5 cm.

Observation 12: The corresponding ΔΦ is Δdmax /λ=((0.1414/0.0857) mod 1.0)*2 π=0.6511*2 π=4.091 Rad (for 2D) for single Φ@3.5GHz, or 0.0472*2 π=0.2966 Rad (for 2D) for dual Φ@100MHz bandwidth.

Observation 13: A small ARP error has large impact on CPP under perfect Φ, especially, for a short wave length.

Observation 14: A long virtual wave length is helpful for alleviating the impact from ARP error.

Observation 15: A small ARP error has significant impact on CPP under practical Φ, especially, for a short wave length.

Observation 16: The UE can report the carrier phase with corresponding phase uncertainty, to evaluate the rationality of other positioning techniques.

Proposal 1: For multiple segments CP report within a carrier, center frequency or λ should be informed for each of each segment.

Proposal 2: The scope of integer N should be configured / reported for carrier phase based positioning.

Proposal 3: Phase or phase difference report should be supported at least for TDOA.

Proposal 4: The same Phase Error Group(PEG) should be used for CP measurement.

Proposal 5: The scope of integer N in OTDOA could be configured based on the measurement uncertainty.

Proposal 6: The scope of integer N in AOA/AOD could be configured based on the angle measurement uncertainty.

Proposal 7: The scope of integer N for different carrier could be selected based on the frequency feature.

Proposal 8: The number of carrier phases to be reported could be determined based on the positioning accuracy and measurement overhead requirements.

Reference

- Draft_Minutes_report_RAN1#110_v020, 3GPP RAN1#110.

- R1-2205903, Discussion on carrier phase measurement based positioning, ZTE, 3GPP RAN1#110.

- TR 38.859, Study on Expanded and Improved NR Positioning.

Simulation results for TR38.859 section B.4.X

B.4.X.1	Description of evaluation scenarios

The evaluated scenario is InF-SH.

Table B.4.X.1-1: NR carrier phase positioning enhancements - evaluation scenarios and parameters from [X]

| Parameter | [Case ID=1], [InF-SH] With sub-ID in “UE/TRP Initial phase offset” | [Case ID=2], [InF-SH] With sub-ID in “CFO/Doppler” | [Case ID=3], [InF-SH] With sub-ID in “ARP errors” |
| --- | --- | --- | --- |
| Scenario [TS 38.855, TS 38.857] | InF-SH | InF-SH | InF-SH |
| Single carrier frequency, or multiple carrier frequencies, GHz | Single carrier | Single carrier | Single carrier, with two carrier phases separated at half bandwidth. See cases in “ARP errors”. Single carrier with single CP in some cases. |
| Bandwidth, MHz | 100 | 100 | 100 |
| Subcarrier spacing, kHz | 30 | 30 | 30 |
| RS signal descriptions (PRS or posSRS, Number of OFDM symbols, Comb size) | PRS (2 symbols, Comb=2) | PRS (2 symbols, Comb=2) | PRS (2 symbols, Comb=2) |
| NR Carrier phase positioning method (DL, UL, or DL+UL(RTT)) | DL | DL | DL |
| R16/R17 positioning method (if it is used together with CPP) | TDOA | TDOA | TDOA |
| Carrier phase estimation techniques (time-domain, freq-domain, references) | Freq-domain, with averaging | Freq-domain, with averaging | Freq-domain, with averaging |
| Differential positioning techniques if used (e.g., single differential, double differential, etc.) | Single differential. | Single differential | Dual differential, the first differential is for two carrier phases within a gNB while the second differential is for inter-gNB. See cases in “ARP errors”. Single differential in some cases. |
| Integer ambiguity resolution techniques (e.g., virtual Integer ambiguity, LAMBDA, cost functions, Least squares, …) | Cost function, with least distance error | Cost function, with least distance error | Cost function, with least distance error |
| Multipath mitigation techniques (e.g., first path detection, ...) | First path detection | First path detection | First path detection |
| Single-measurement instance CPP, or multiple measurement instances CPP | Single-measurement instance CPP | Single-measurement instance CPP | Single-measurement instance CPP |
| UE position calculation algorithm (e.g. Least squares, Taylor series, …) | Least squares | Least squares | Least squares |
| Network synchronization assumption (e.g., 0ns, 10ns, ..) | 0ns | 0ns | 0ns |
| UE/TRP Initial phase offset | Case ID=1-1 for [0,0.001]* 2π Case ID=1-2 for [0,0.005]* 2π Case ID=1-3 for [0,0.01]* 2π Case ID=1-4 for [0,0.02]* 2π Case ID=1-5 for [0,0.04]* 2π Case ID=1-6 for [0,0.06]* 2π Case ID=1-7 for [0,0.08]* 2π Case ID=1-8 for [0,0.1]* 2π Case ID=1-9 for [0,0.2]* 2π Case ID=1-10 for [0,1]* 2π withou PRU Case ID=1-11 for [0,1]* 2π with PRU Case ID=1-12 for [0,0.2]* 2π without PRU Case ID=1-13 for [0,0.1]* 2π without PRU Case ID=1-14 for [0,0.02]* 2π without PRU Case ID=1-15 for [0,0.005]* 2π without PRU Case ID=1-16 for [0,0.2]* 2π with PRU Case ID=1-17 for [0,0.1]* 2π with PRU Case ID=1-18 for [0,0.02]* 2π with PRU Case ID=1-19 for [0,0.005]* 2π with PRU | 0 | 0 |
| CFO/Doppler | 0 | Case ID=2-1-1 for [-100, +100] Hz with perfect CP measurement Case ID=2-1-2 for [-1000, +1000] Hz with perfect CP measurement Case ID=2-1-3 for [-4000, +4000] Hz with perfect CP measurement Case ID=2-1-4 for [-8000, +8000] Hz with perfect CP measurement Case ID=2-2-1 for [-50, +50] Hz with practical CP measurement Case ID=2-2-2 for [-100, +100] Hz with practical CP measurement Case ID=2-2-3 for [-1000, +1000] Hz with practical CP measurement | 0 |
| Oscillator-drifts | 0 | 0 | 0 |
| ARP errors | 0 | 0 | a zero-mean, truncated Gaussian distribution with zero mean and standard deviation of T=[1, 5] cm truncated to 2T Case ID=3-1-1 for T=1 with perfect single CP Case ID=3-1-2 for T=2 with perfect single CP Case ID=3-1-3 for T=5 with perfect single CP Case ID=3-1-4 for T=1 with perfect dual CP Case ID=3-1-5 for T=2 with perfect dual CP Case ID=3-1-6 for T=5 with perfect dual CP Case ID=3-2-1 for T=1 with practical single CP Case ID=3-2-2 for T=2 with practical single CP Case ID=3-2-3 for T=5 with practical single CP |
| Phase Center Offsets | 0 | 0 | 0 |
| Phase noise (FR2) | 0 | 0 | 0 |
| Additional notes, if any |  |  |  |

B.4.X.2	Positioning accuracy evaluation results for NR Carrier Phase Positioning

The simulation results are listed in the following table. It should be noted that the requirement for CPP is not defined yet.

Table B.4.X.2-1: NR carrier phase positioning - horizontal accuracy from [X]

| [Case ID], [Scenario] [additional descriptions] | 50% | 67% | 80% | 90% | Meet target requirements? (Y/N) | Additional comments |
| --- | --- | --- | --- | --- | --- | --- |
| Case ID=1-1, [InF-SH], [for Phase offsetΦ=[0,0.001]* 2π] | 0 | 0 | 0.105 | 0.205 | N/A |  |
| Case ID=1-2 for [0,0.005]* 2π | 0 | 0 | 0.105 | 0.211 |  |  |
| Case ID=1-3 for [0,0.01]* 2π | 0 | 0.002 | 0.158 | 0.243 |  |  |
| Case ID=1-4 for [0,0.02]* 2π | 0.002 | 0.048 | 0.201 | 0.338 |  |  |
| Case ID=1-5 for [0,0.04]* 2π | 0.092 | 0.241 | 0.556 | 0.782 |  |  |
| Case ID=1-6 for [0,0.06]* 2π | 0.224 | 0.392 | 0.685 | 0.837 |  |  |
| Case ID=1-7 for [0,0.08]* 2π | 0.317 | 0.598 | 0.767 | 0.895 |  |  |
| Case ID=1-8 for [0,0.1]* 2π | 0.424 | 0.609 | 0.778 | 0.944 |  |  |
| Case ID=1-9 for [0,0.2]* 2π | 0.530 | 0.704 | 0.843 | 0.931 |  |  |
| Case ID=1-10 for [0,1]* 2π withou PRU Phase error is at gNB. | 0.306 | 0.379 | 0.444 | 0.520 |  |  |
| Case ID=1-11 for [0,1]* 2π with PRU Phase error is at gNB. | 0.065 | 0.174 | 0.275 | 0.471 |  |  |
| Case ID=1-12 for [0,0.2]* 2π without PRU | 0.258 | 0.349 | 0.436 | 0.557 |  |  |
| Case ID=1-13 for [0,0.1]* 2π without PRU | 0.171 | 0.264 | 0.348 | 0.434 |  |  |
| Case ID=1-14 for [0,0.02]* 2π without PRU | 0.002 | 0.103 | 0.206 | 0.296 |  |  |
| Case ID=1-15 for [0,0.005]* 2π without PRU | 0 | 0.001 | 0.175 | 0.265 |  |  |
| Case ID=1-16 for [0,0.2]* 2π with PRU | 0.097 | 0.115 | 0.148 | 0.233 |  |  |
| Case ID=1-17 for [0,0.1]* 2π with PRU | 0.084 | 0.114 | 0.142 | 0.224 |  |  |

| Case ID=1-18 for [0,0.02]* 2π with PRU | 0.101 | 0.130 | 0.169 | 0.243 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Case ID=1-19 for [0,0.005]* 2π with PRU | 0.096 | 0.117 | 0.140 | 0.214 |  |  |
| Case ID=2-1-1, [InF-SH], [for CFO/Doppler [-100, +100] Hz with perfect CP measurement] | 0 | 0 | 0.116 | 0.205 |  |  |
| Case ID=2-1-2 for [-1000, +1000] Hz with perfect CP measurement | 0 | 0 | 0.123 | 0.210 |  |  |
| Case ID=2-1-3 for [-4000, +4000] Hz with perfect CP measurement | 0 | 0 | 0.049 | 0.192 |  |  |
| Case ID=2-1-4 for [-8000, +8000] Hz with perfect CP measurement | 0 | 0 | 0 | 0.144 |  |  |
| Case ID=2-2-1 for [-50, +50] Hz with practical CP measurement | 0.003 | 0.148 | 0.244 | 0.376 |  |  |
| Case ID=2-2-2 for [-100, +100] Hz with practical CP measurement | 0.003 | 0.148 | 0.244 | 0.376 |  |  |
| Case ID=2-2-3 for [-1000, +1000] Hz with practical CP measurement | 0.003 | 0.148 | 0.244 | 0.376 |  |  |
| Case ID=3-1-1, [InF-SH], [for ARP error T=1 with perfect single CP] | 0.512 | 0.694 | 0.854 | 0.922 |  |  |
| Case ID=3-1-2 for T=2 with perfect single CP | 0.532 | 0.726 | 0.867 | 0.991 |  |  |
| Case ID=3-1-3 for T=5 with perfect single CP | 0.669 | 0.838 | 0.959 | 1.071 |  |  |
| Case ID=3-1-4 for T=1 with perfect dual CP | 0.015 | 0.026 | 0.133 | 0.214 |  |  |
| Case ID=3-1-5 for T=2 with perfect dual CP | 0.030 | 0.051 | 0.140 | 0.216 |  |  |
| Case ID=3-1-6 for T=5 with perfect dual CP | 0.076 | 0.122 | 0.158 | 0.235 |  |  |
| Case ID=3-2-1 for T=1 with practical single CP | 0.243 | 0.354 | 0.440 | 0.526 |  |  |
| Case ID=3-2-2 for T=2 with practical single CP | 0.303 | 0.376 | 0.477 | 0.598 |  |  |
| Case ID=3-2-3 for T=5 with practical single CP | 0.280 | 0.378 | 0.443 | 0.515 |  |  |

Summary of observations for TR 38.859

Some high level observations are selected from the above section 6 for TR 38.859 as follows:

Observation a: PRU can alleviate the random initial phase error at gNB side from (0~2π) to about (0~0.02*2π) (i.e., 98% reduction).

Observation b: The carrier phase error from initial random phases at both TRP and UE sides can be mitigated by dual differential operation with the help of PRU.

Observation d: The carrier phase error within one site can be removed by single differential operation for the case of carrier phase method from multiple frequency sections in one carrier.

Observation d: A small frequency offset has negligible impact on CPP under practical Φ.

Observation e: More phase estimation error cause larger location error in the case without help of PRU or differential operation.

Observation f: A small ARP error has large impact on CPP