3GPP TSG RAN WG1 Meeting #110bis-e	R1-2209220

e-Meeting, October 10 – 19, 2022

Agenda item:	9.3.3

Source: 	Lenovo

Title: 	Potential enhancements on dynamic/flexible TDD

Document for:	Discussion and Decision

# Introduction

In RAN1#110, the following was agreed for CLI handling in dynamic/flexible TDD and/or SBFD,

Agreement Study the feasibility and potential benefits of gNB-to-gNB co-channel CLI measurement for gNB-to-gNB CLI handling which can be specific for dynamic/flexible TDD and/or common for both SBFD and dynamic/flexible TDD, at least includes: - Measurement resource configuration - Measurement details - Relevant information exchange - Usage of measurement Agreement Study the feasibility and potential benefit of UE-to-UE co-channel CLI measurement and reporting, which can be specific for dynamic/flexible TDD and/or common for both SBFD and dynamic/flexible TDD, at least includes: - Measurement resource/reporting configuration - Measurement/reporting details (including UE processing delay) - Relevant information exchange (between gNBs) if needed - Usage of measurement at gNB Agreement Study the feasibility and potential benefits of coordinated scheduling for time/frequency resources between gNBs for gNB-to-gNB co-channel CLI handling which can be specific for dynamic/flexible TDD and/or common for both SBFD and dynamic/flexible TDD, the study at least includes: - Details of coordinated scheduling for time/frequency resources - Relevant information exchange Agreement Study the feasibility and potential benefits of spatial domain coordination method for gNB-to-gNB co-channel CLI handling which can be specific for dynamic/flexible TDD and/or common for both SBFD and dynamic/flexible TDD, the study at least includes: - Details for spatial domain coordination - Relevant information exchange Note1: Study can include method for FR1 and FR2 Agreement Study the feasibility and potential benefits of coordinated scheduling for time/frequency resources between gNBs (if needed) for UE-to-UE co-channel CLI handling which can be specific for dynamic/flexible TDD and/or common for both SBFD and dynamic/flexible TDD, at least includes: - Details of coordinated scheduling for time/frequency resources - Relevant information exchange (if needed) Agreement Study the feasibility and potential benefit of UE-to-UE co-channel CLI handling based on spatial domain coordination method which can be specific for dynamic/flexible TDD and/or common for both SBFD and dynamic /flexible TDD, at least includes: - Details for spatial domain coordination by gNB - Relevant information exchange (if needed) Note1: Study can include method for FR1 and FR2

In this contribution, we discuss details on potential enhancements for inter-gNB CLI handing and inter-UE CLI handling for dynamic/flexible TDD.

# Discussion

In dynamic/flexible TDD, the inter-gNB and inter-UE CLI might happen in symbols in which different (colliding) transmission directions (DL/UL) are configured in different cells. The inter-gNB CLI is a DL-to-UL interference coming from DL signals from one gNB interfering UL signals of a neighbor gNB, while inter-UE CLI is a UL-to-DL interference coming from UL signals from a UE in a cell interfering DL signal for a UE in a neighbor cell. Figure 1 illustrates an example of inter-gNB and inter-UE CLI.

The colliding symbols could be determined based on gNB coordination. Rel.16 specified inter-gNB coordination of the intended DL/UL configuration through backhaul signaling. From the coordination, one gNB could know the resources for fixed DL or UL transmission directions in the coordinating neighbor gNB and also the flexible resources which the neighbor gNB might use for either DL or UL transmission. The gNB could then locate the colliding symbols where different transmission directions are used in the neighbor gNBs. Accordingly, the gNB might determine the strategy for the data scheduling in the colliding and non-colliding symbols.

Figure 1 Inter-UE CLI and inter-gNB CLI in dynamic/flexible TDD

## Inter-gNB CLI handling

It is noted that in Rel.16, inter-gNB CLI was considered, but no solutions were specified. Instead, it was assumed that the measurement and mitigation of inter-gNB CLI are based on gNB implementation.  Rel.18 aims to further study CLI handling between gNBs of the same or different operators to enable dynamic/flexible TDD in commercial networks.

To mitigate inter-gNB CLI, the gNB needs to measure the CLI from neighbor gNBs. The existing DL RS, i.e., CSI-RS and SSB, can be used for inter-gNB CLI measurement due to flexibility in terms of configuration and transmission. Rel.16 specified RIM-RS is another option, but the high overhead might be an issue. Typically, the CLI measurement RS is configured to be transmitted/received in the colliding symbols, i.e., it is transmitted by the aggressor gNBs in the DL symbols and received by the victim gNB in the UL symbols.

Proposal 1: Support of use existing DL RS (CSI-RS, SSB) for inter-gNB CLI measurement.

In order to realize gNB-to-gNB CLI measurement, the aggressor gNB should coordinate with potential victim gNBs on the use of reference signals. The aggressor gNB can configure CSI-RS (and/or SSB) and send the configuration information to potential victim gNBs over the backhaul (e.g., Xn). The victim gNBs can then perform interference measurement on the CSI-RS and report high-interference CSI-RS resources back to the aggressor gNB.

Proposed 2: Support exchange of CSI-RS (and/or SSB) configuration information over the backhaul among the aggressor gNB and potential victim gNBs for the purpose of inter-gNB CLI measurement. Support reporting of high-interference CSI-RS resources from a victim gNB back to the aggressor gNB.

The victim gNB may experience different levels of inter-gNB CLI from different beams of the aggressor gNB. This motivates per-beam CSI-RS transmission by the aggressor gNB and correspondingly per-beam interference measurement at the victim gNB. The victim gNB might report the beam(s) that incur high interference level to the aggressor gNB such that beam level coordinated scheduling among gNBs can be performed based on the CLI reporting.

Proposal 3: Support per-beam inter-gNB CLI measurement and reporting to enable coordinate scheduling/beamforming.

Details of measurement and reporting mechanisms in Proposals 1-3 are presented below.

With application of analog beamforming at FR2, interference from the aggressor gNB is potentially mitigated by Tx beamforming of signals towards the target UE, which is likely to be spatially (angularly) distant from the victim gNB. Similarly, the victim gNB applies Rx beamforming towards its own target UE, which is potentially distant from the aggressor gNB. Therefore, it is expected that analog beamforming reduces the probability of inter-gNB CLI.

However, due to imperfect analog beam patterns (e.g., side lobes and faulty phased elements), and especially in heterogenous deployments, the adverse effect of potential inter-gNB CLI, once it occurs, is exacerbated in the presence of beamforming gains on Tx and/or Rx sides, which can impose significant constraints on the SINR and the resulting data rates.

The following figure illustrates an example scenario. In this example, DL Tx beam 1 of gNB1 does not cause excessive interference on gNB2. However, DL Tx beam 2 does have the potential to cause a large inter-gNB CLI depending on Rx beamforming at gNB2. If gNB2 applies UL Rx beam 1, the resulting CLI can be excessive, but not otherwise. Therefore, on the one hand, gNB2 should expect a large CLI in the case that it happens to apply UL Rx beam 2 right when gNB1 applies DL Tx beam 1. But on the other hand, assuming the worst-case CLI every time gNB2 applies UL Rx beam 1 can lead to significant underutilization of the bandwidth.

Figure 2 Inter-gNB CLI in the presence of analog beamforming

This issue motivates beam coordination among gNBs for the purpose of CLI measurement and handling.

Coordination signaling can occur on the Xn interface directly between gNBs and/or on the NG interface indirectly through the core network. Current XnAP specification supports exchanging Intended TDD DL-UL Configuration NR IEs that allow gNBs to inform neighbor gNBs of potential TDD interference. Remote interference management, on the other hand, utilizes NGAP communication through the core network for interference measurement and management. Neither of these mechanisms, however, supports beam coordination.

In order to address the issue of variable inter-gNB CLI in the presence of beamforming, especially at FR2, we propose the following.

First, gNBs should be enabled to inform other gNBs in their vicinity of beam-specific interference each of its downlink beam causes. Absent such information, victim gNBs are not able to acquire the worst-case interference, as interference measurements on random downlink symbols are not guaranteed to capture the interference from the worst beams. For this purpose, the aggressor gNB should configure reference signals for the purpose of inter-gNB CLI.

Proposal 4: Support reference signal configuration and inter-gNB signaling for aggressor gNB to inform other gNBs in the vicinity of beam-specific interference.

Next, if all receiving gNBs measuring beam-specific CLI assume the worst-case scenario, which is interference from the beam that causes the highest interference, that can lead to significant underutilization of resources as the corresponding data rates can be quite low. Instead, we propose the aggressor gNB indicate to the other gNBs of any restrictions it may apply for using high-interference beams.

Proposal 5: Support aggressor gNB indicating to victim gNBs of any restrictions on using high-interference beams.

Since the aggressor gNB does not have prior knowledge of which beams are harmful to neighbor gNBs, we further propose to allow victim gNBs to indicate high-interference Tx beams of the aggressor gNB. Similar to RIM, this feedback mechanism can be specified over backhaul or over the air (OTA).

Proposal 6: Support victim gNB sending feedback to aggressor gNB about its high-interference beams.

When the victim gNB learns about potential inter-gNB CLI, it can take interference handling measures by implementation, e.g., link adaptation in the uplink. Similarly, when the aggressor gNB is informed of high-interference beams, it can attempt to reduce the interference on those beams by implementation, for example, avoiding those beams in the downlink or reducing the DL Tx power on those beams.

However, it is possible for gNBs to coordinate TDD configurations on high-interference beams, especially at FR2. For example, if the aggressor gNB needs to use certain Tx beams for scheduling DL to UEs, and a victim gNB reports that it experiences a large interference from those Tx beams, the aggressor gNB can coordinate and match TDD configurations on certain slots and then use the high-interference Tx beams only when scheduling DL on those slots. This way, the aggressor gNB can make sure that a DL/UL symbol collision does not happen when it uses the high-interference Tx beams.

Similarly, if the victim gNB needs to use certain Rx beams for scheduling UL for UEs, but it receives a large interference on those Rx beams, it can use those Rx beams only when scheduling UL on the coordinated slots. This way, the victim gNB can make sure that a DL/UL symbol collision does not happen when it uses the coordinated Rx beams.

Since different Tx beams can be harmful to different victim gNBs, and similarly, different Rx beams can receive different interference from different aggressor gNBs, it is beneficial to enable multiple such coordinated/matched TDD DL/UL among gNBs in a vicinity. This motivates grouping of gNBs and coordinating/matching TDD DL/UL within each group.

An example is illustrated in Figure 3. In this example, the aggressor gNB causes a large interference when applying Tx beams 1 and 2 on victim gNBs 1 and 2, but not the other way around. Therefore, the aggressor gNB can coordinate/match TDD DL/UL on certain slots for scheduling UE1 and coordinate TDD DL/UL on other slots for scheduling UE2.

Figure 3 Inter-gNB CLI in the presence of analog beamforming

Proposal 7: To enable coordinated scheduling/beamforming, support coordination/matching of TDD DL/UL on certain slots/symbols for use of high-interference beams.

Moreover, several companies have proposed to take the existing interference handling mechanisms, such as Rel-16 inter-UE CLI and RIM as the starting point for interference handling with duplexing enhancements. It is possible to pursue a unified interference handling approach through which gNBs can measure interference from neighbor gNBs and UEs (gNB-to-gNB and UE-to-gNB interference), and furthermore, configure their own UEs to measure gNB-to-UE and UE-to-UE interference on the same reference signals. Since SRS-RSRP reporting is specified for inter-UE CLI management, the same specification can be used for gNB-to-UE interference if the gNB transmits the SRS (which can then be used by implementation for gNB-to-gNB CLI as well).

Allowing gNB to transmit SRS for this purpose has at least two advantages: 1) SRS resources can be shared for downlink and uplink interference measurements, hence saving resource and signaling overhead. 2) Specification of the victim UE (measuring and reporting SRS-RSRP) can be unified, which reduces specification efforts.

Proposal 8: Study unified inter-cell CLI handling through transmitting SRS by aggressor gNB/UE and measuring interference by victim gNB/UE.

The victim gNB needs to receive the CLI measurement RS from the aggressor gNB in the UL symbols. If RS is transmitted in the resource overlapping with a PUSCH transmission, the victim gNB has to pause the PUSCH reception and starts to receive the RS. Then PUSCH detection performance might be impacted. It could be considered to solve this issue by gNB implementation. For example, the victim gNB could schedule PUSCH such that PUSCH resources do not collide with those for CLI measurement RS. Alternatively, the PUSCH resource can be scheduled to include the overlapping resource, and PUSCH is more robustly transmitted to mitigate the impact from the CLI measurement RS.

Proposal 9: The impact on the PUSCH reception when receiving CLI measurement RS can be solved by gNB implementation.

## Inter-UE CLI handling

To handle inter-UE CLI, Rel-16 introduced UE side interference measurement and reporting. A UE from a cell could be configured to measure the signals from UEs in a neighboring cell. The measurement could be SRS-RSRP or CLI-RSSI based on configuration. The UE will report the measurement to gNB so that the gNB could take it into consideration when perform scheduling.

To measure SRS-RSRP, the victim UE is provided with configurations for SRS transmission, which are expected to be the same with those configured for the SRS transmission from the potential aggressor UEs. This also means that the gNB should know the SRS configurations of the potential aggressor UEs from a neighbor gNB, otherwise it cannot configure SRS to the victim UEs for interference measurement. It is noted that Rel.16 did not specify the coordination of SRS configurations due to the overhead issue. In Rel.18, the coordination of SRS configurations among gNBs might be a potential area to be enhanced.

Proposal 10: Study to introduce coordination of SRS configurations for SRS-RSRP measurement.

Besides the UE-to-UE inter-cell co-channel CLI measurement, the SRS configuration (especially SRS configuration in time domain) may also be used for UE-to-UE inter-cell inter-subband CLI measurement, that is, the victim UE may acquire the SRS-RSRP on the configured SRS resources and may further measure CLI-RSSI on resources overlapping in time with the configured SRS resources within the adjacent subbands. Therefore, for the UE-to-UE inter-cell co-channel and inter-subband CLI measurement, common schemes on coordination of SRS configurations and scheduling information should be studied.

Proposal 11: For the UE-to-UE inter-cell co-channel and inter-subband CLI measurement, common schemes on coordination of SRS configurations and intended TDD DL-UL configurations should be studied.

Furthermore, Rel-16 CLI measurement and reporting mechanisms do not consider potentially different interference levels measured by a UE when different spatial filters are used for interference measurements. Considering that received signal power may vary significantly depending on Rx beams and Rx antenna panels as shown in Figure 4, for Rel-18 CLI measurement and reporting, it may be beneficial to enable a UE to report impact of spatial filters to be used by the UE for intended communications on observed interference levels. Accordingly, gNB can use the reported information for interference handling with proper scheduling of UEs and corresponding serving beams.

Figure 4 Exemplary received signal power for different Rx beams of different Rx antenna panels

Observation 1: Observed interference level may vary significantly depending on Rx beams and Rx antenna panels.

Proposal 12: Support spatially differentiated CLI measurement and reporting.

Another potential area is more dynamic interference measurement and reporting. This is different from Rel-16 CLI measurement, which is based on linear averaging of a long-term measurement. When a cell in unpaired spectrum operates with semi-statically configured flexible symbols, existence of cross-link interference may depend on dynamic usage of the configured flexible symbols. That is, if communication directions of flexible symbols change dynamically, existence of dominant CLI changes dynamically. Therefore, UE may not be able to measure a CLI level accurately based on semi-statically configured CLI measurement occasions. If UE performs CLI measurement only when CLI exists based on dynamic indication from a network, UE measurement can accurately reflect the CLI level even with dynamic TDD operation. Further, the dynamic indication may include spatial information associated with active CLI measurement occasions so that UE can perform CLI measurement based on the indicated spatial information. In commercial network with UE mobility, more dynamic interference measurement and reporting might be useful.

Proposal 13: Study more dynamic interference measurement and reporting for inter-UE CLI mitigation.

# Conclusion

In summary, we provide have the following observation and proposals for CLI mitigation in dynamic/flexible TDD,

Proposal 1: Support of use existing DL RS (CSI-RS, SSB) for inter-gNB CLI measurement.

Proposed 2: Support exchange of CSI-RS (and/or SSB) configuration information over the backhaul among the aggressor gNB and potential victim gNBs for the purpose of inter-gNB CLI measurement. Support reporting of high-interference CSI-RS resources from a victim gNB back to the aggressor gNB.

Proposal 3: Support per-beam inter-gNB CLI measurement and reporting to enable coordinate scheduling/beamforming.

Proposal 4: Support reference signal configuration and inter-gNB signaling for aggressor gNB to inform other gNBs in the vicinity of beam-specific interference.

Proposal 5: Support aggressor gNB indicating to victim gNBs of any restrictions on using high-interference beams.

Proposal 6: Support victim gNB sending feedback to aggressor gNB about its high-interference beams.

Proposal 7: To enable coordinated scheduling/beamforming, support coordination/matching of TDD DL/UL on certain slots/symbols for use of high-interference beams.

Proposal 8: Study unified inter-cell CLI handling through transmitting SRS by aggressor gNB/UE and measuring interference by victim gNB/UE.

Proposal 9: The impact on the PUSCH reception when receiving CLI measurement RS can be solved by gNB implementation.

Proposal 10: Study to introduce coordination of SRS configurations for SRS-RSRP measurement.

Proposal 11: For the UE-to-UE inter-cell co-channel and inter-subband CLI measurement, common schemes on coordination of SRS configurations and intended TDD DL-UL configurations should be studied.

Observation 1: Observed interference level may vary significantly depending on Rx beams and Rx antenna panels.

Proposal 12: Support spatially differentiated CLI measurement and reporting.

Proposal 13: Study more dynamic interference measurement and reporting for inter-UE CLI mitigation.

# References

- RAN1#110 Chairman notes.