3GPP TSG RAN WG1 #110bis-e			R1-2209552

e-Meeting, October 10th – 19th, 2022

Agenda Item:	5

Source:	Apple

Title:	Discussion on RAN2 LS on IUC with Non-preferred Resource Set

Document for:	Discussion/Decision

# Introduction

RAN2 sent an LS on inter-UE coordination (IUC) with non-preferred resource set [1]. The LS talks about the scenario in which UE-B receives IUC scheme 1 with non-preferred resource set from UE-A, but UE-B does not perform sensing in the resource pool associated with the non-preferred resource set. The following questions were raised:

Q1:	Is the scenario described above a valid scenario or not?

Q2:	If the answer to Q1 is yes, does resource exclusion based on non-preferred resource set needs to be performed by UE B or not?

Q3:	If the answer to Q2 is yes, then, in RAN1’s view, which specification (PHY or MAC) should capture the resource exclusion behavior?

In this contribution, we discuss the IUC with non-preferred resource set for the LS from RAN2.

# Discussion

The following RAN1 agreements and working assumption were made:

Working Assumption For Scheme 1, following cast type(s) are supported for inter-UE coordination information transmission triggered by a condition other than explicit request reception - Groupcast/Broadcast for non-preferred resource set, FFS for preferred resource set - FFS: Under which conditions groupcast/broadcast can be supported - Unicast - FFS: Under which conditions unicast can be supported Agreements: - In R17, a SL Mode 2 Tx resource pool can be (pre-)configured to enable full sensing only, partial sensing only, random resource selection only, or any combination(s) thereof - FFS details, including usage, potential restrictions, whether/how any enhancement or condition is needed for the coexistence of full sensing and power saving RA scheme(s) in a same resource pool, etc.

It is clear that in case where IUC is triggered by a condition other than explicit requestion reception, the IUC with non-preferred resource set can be sent via unicast, groupcast and broadcast. If UE-A sends IUC with non-preferred resource set in a resource pool which supports multiple resource allocation mechanisms (e.g., full sensing, partial sensing, and random resource selection), then it is possible that a random resource selection UE-B receives the IUC. Hence, our answer to the first question in RAN2 LS is that the scenario is a possible scenario.

Observation 1: The scenario where UE-B receives IUC scheme 1 with non-preferred resource set from UE-A, but UE-B does not perform sensing in the same resource pool is a possible scenario.

Given that the scenario where UE-B receives IUC scheme 1 with non-preferred resource set from UE-A, but UE-B does not perform sensing in the same resource pool is a possible scenario, we think UE-B’s behavior has not been defined in the specification for this case.

The following procedure has been mentioned in TS38.321 [2]:

3> if configured by RRC, interUECoordinationScheme1Explicit or interUECoordinationScheme1Condition enabling reception of preferred resource set and non-preferred resource set and when the UE does not have own sensing result as specified in clause 8.1.4 of TS 38.214 [7] and if a preferred resource set is received from a UE: 4> randomly select the time and frequency resources for one transmission opportunity from the resources belonging to the received preferred resource set for a MAC PDU to be transmitted to the UE providing the preferred resource set, according to the amount of selected frequency resources and the remaining PDB of SL data available in the logical channel(s) allowed on the carrier. 3> if configured by RRC, interUECoordinationScheme1Explicit or interUECoordinationScheme1Condition enabling reception of preferred resource set and non-preferred resource set and when the UE has own sensing result as specified in clause 8.1.4 of TS 38.214 [7] and if a preferred resource set is received from a UE: 4> randomly select the time and frequency resources for one transmission opportunity within the intersection of the received preferred resource set and the resources indicated by the physical layer as specified in clause 8.1.4 of TS 38.214 [7] for a MAC PDU to be transmitted to the UE providing the preferred resource set, according to the amount of selected frequency resources and the remaining PDB of SL data available in the logical channel(s) allowed on the carrier.

Basically, the UE’s MAC layer procedure does not include the case when UE does not have own sensing result but receives a non-preferred resource set. Hence, from UE’s MAC layer procedure point of view, UE-B’s behavior has not been defined.

Observation 2: UE-B’s behavior has not been defined in the current specification when receiving IUC scheme 1 with non-preferred resource set from UE-A while not performing sensing in the resource pool associated with the non-preferred resource set.

The second question from RAN2 LS [1] was extensively discussed in RAN1 #110 meeting [3]. Companies’ views were almost equally splitted, which depend on opposite understandings of the following agreement made on UE-B’s behavior when receiving the IUC information from UE-A.

Agreement: In scheme 1, at least following UE-B’s behavior in its resource (re-)selection is supported when it receives inter-UE coordination information from UE-A: - For preferred resource set, the following two options are supported: - Option A): UE-B’s resource(s) to be used for its transmission resource (re-)selection is based on both UE-B’s sensing result (if available) and the received coordination information - UE-B uses in its resource (re-)selection, resource(s) belonging to the preferred resource set in combination with its own sensing result - UE-B uses in its resource (re-)selection, resource(s) not belonging to the preferred resource set when condition(s) are met - FFS: Details of condition(s) - This option is supported when UE-B performs sensing/resource exclusion - FFS: Other details (if any) - Option B): UE-B’s resource(s) to be used for its transmission resource (re-)selection is based only on the received coordination information - UE-B uses in its resource (re-)selection, resource(s) belonging to the preferred resource set - This option is supported at least when UE-B does not support sensing/resource exclusion - FFS: Whether the support is conditional or UE capability - FFS: Other details (if any) - FFS: Other option(s), and other details (if any) - For non-preferred resource set, - UE-B’s resource(s) to be used for its transmission resource (re-)selection is based on both UE-B’s sensing result (if available) and the received coordination information - UE-B excludes in its resource (re-)selection, resource(s) overlapping with the non-preferred resource set - FFS: Details including - Whether/how UE-B can use in its resource (re-)selection, resource(s) overlapping with the non-preferred resource set, definition of the overlap, and other details (if any) - When UE-B excludes in its resource (re-)selection, resource(s) overlapping with the non-preferred resource set - FFS: UE-B reselects in its resource (re-)selection, resource(s) to be used for its transmission when the resource(s) are fully/partially overlapping with the non-preferred resource set - FFS: Other option(s), and other details (if any)

On one hand, from the blue highlighted text in the agreement, one camp of companies think if UE-B’s sensing result is unavailable, then only the received non-preferred resource set information is used in UE-B’s resource selection procedure. Additionally, the usage of the received non-preferred resource set information could enhance the performance of UE-B’s resource selection. Hence, the specification needs to be updated to reflect this RAN1 agreement.

On the other hand, from the yellow highlighted text in the agreement, the other camp of companies think for preferred resource set, two options are defined where Option A has the same descriptions as that for non-preferred resource set. However, only a single option is defined for the case of non-preferred resource set. This implies the similar case as Option B for preferred resource set is not defined for the non-preferred resource set case in this agreement. In the maintenance phase of Rel-17 NR sidelink, it is unnecessary to define UE-B’s behavior to handle this scenario. The straightforward approach to address this issue is that UE-B does not perform resource exclusion based on non-preferred resource set. UE-B can continue using random resource selection and simply ignore the received IUC information.

After 3 rounds of email discussions in RAN1 #110 meeting, no consensus can be achieved on whether UE-B performs resource exclusion based on non-preferred resource set. Hence, we have the following proposal.

Proposal 1: Regarding the second question in R1-2205728, there is no consensus in RAN1 that UE-B performs resource exclusion based on non-preferred resource set.

The corresponding draft reply LS on IUC with non-preferred resource set is in [4].

# Conclusion

In this contribution, we provided our views on IUC with non-preferred resource set. Our observations and proposal are as follows:

Observation 1: The scenario where UE-B receives IUC scheme 1 with non-preferred resource set from UE-A, but UE-B does not perform sensing in the same resource pool is a possible scenario.

Observation 2: UE-B’s behavior has not been defined in the current specification when receiving IUC scheme 1 with non-preferred resource set from UE-A while not performing sensing in the resource pool associated with the non-preferred resource set.

Proposal 1: Regarding the second question in R1-2205728, there is no consensus in RAN1 that UE-B performs resource exclusion based on non-preferred resource set.

# References

- R1-2205728, “LS on IUC with non-preferred resource set,” May 2022.

- 3GPP TS 38.321, NR; Medium Access Control (MAC) protocol specification, v17.1.0, Jul. 2022.

- R1-2208080, “Moderator summary of email discussion on reply LS in R1-2205728,” Aug. 2022.

- R1-2209553, “Draft reply LS on IUC with non-preferred resource set,” Oct. 2022.