3GPP TSG-RAN WG1 #110bis	R1-2209689

e-Meeting,October 10th – 19th, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | DRAFT | rev |  | Current version: | 16.11.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for Intra-UE prioritization related to SP-CSI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_L1enh_URLLC-Core |  |  |  |  |  | Date: |  |  | 2022-9-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The following agreement made in RAN1#110 is not captured in the specification yet. Agreement For cases 3, 4, 5, UE follows CG-CG handling, the UE is expected to cancel a repetition of the PUSCH transmissions of smaller priority index from the starting symbol of the repetition if the repetition of the PUSCH transmissions of smaller priority index overlaps in time with the PUSCH transmissions of larger priority index. - Case 3: HP PUSCH with SP-CSI without DCI and LP CG PUSCH - Case 4: HP PUSCH with SP-CSI without DCI and LP PUSCH with SP-CSI without DCI - Case 5: LP PUSCH with SP-CSI without DCI and HP CG PUSCH |  |  |  |  |  |  |  |  |
| Summary of change: |  | Capture the following agreement in the specification Agreement For cases 3, 4, 5, UE follows CG-CG handling, the UE is expected to cancel a repetition of the PUSCH transmissions of smaller priority index from the starting symbol of the repetition if the repetition of the PUSCH transmissions of smaller priority index overlaps in time with the PUSCH transmissions of larger priority index. - Case 3: HP PUSCH with SP-CSI without DCI and LP CG PUSCH - Case 4: HP PUSCH with SP-CSI without DCI and LP PUSCH with SP-CSI without DCI - Case 5: LP PUSCH with SP-CSI without DCI and HP CG PUSCH |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear UE behaviour when resolving the overlapping for the following cases - Case 3: HP PUSCH with SP-CSI without DCI and LP CG PUSCH - Case 4: HP PUSCH with SP-CSI without DCI and LP PUSCH with SP-CSI without DCI - Case 5: LP PUSCH with SP-CSI without DCI and HP CG PUSCH |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

9 UE procedure for reporting control information

*** Unchanged text is omitted ***

If a UE would transmit the following channels, including repetitions if any, that would overlap in time

-	a first PUCCH of larger priority index with SR and a second PUCCH or PUSCH of smaller priority index, or

-	a configured grant PUSCH of larger priority index and a PUCCH of smaller priority index, or

-	a first PUCCH of larger priority index with HARQ-ACK information only in response to PDSCH(s) reception without corresponding PDCCH(s) and a second PUCCH of smaller priority index with HARQ-ACK information only in response to PDSCH(s) reception without corresponding PDCCH(s), or a second PUCCH of smaller priority index with SR and/or CSI, or a configured grant PUSCH with smaller priority index, or a PUSCH of smaller priority index with SP-CSI reports without a corresponding PDCCH, or

-	a PUSCH of larger priority index with SP-CSI reports without a corresponding PDCCH and a PUCCH of smaller priority index with SR, or CSI, or HARQ-ACK information only in response to PDSCH(s) reception without corresponding PDCCH(s), or

-	a configured grant PUSCH of larger priority index and a configured grant PUSCH of smaller priority index on a same serving cell, or

-     a first PUSCH of larger priority index with SP-CSI reports without a corresponding PDCCH and a second configured grant PUSCH of smaller priority index or a second PUSCH of smaller priority index with SP-CSI reports without a corresponding PDCCH on a same serving cell, or

-     a first configured grant PUSCH of larger priority index and a second PUSCH of smaller priority index with SP-CSI reports without a corresponding PDCCH on a same serving cell

the UE is expected to cancel a repetition of the PUCCH/PUSCH transmissions of smaller priority index before the first symbol overlapping with the PUCCH/PUSCH transmission of larger priority index if the repetition of the PUCCH/PUSCH transmissions of smaller priority index overlaps in time with the PUCCH/PUSCH transmissions of larger priority index.

*** Unchanged text is omitted ***