3GPP TSG RAN WG1 #110bis-e			     R1-2209879

e-Meeting, October 10th – 19th, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR |  | rev |  | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Editorial corrections for SL IUC (38.213) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO, INC. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh-Core |  |  |  |  |  | Date: |  |  | 2022-09-30 |
| Category: | D |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Names of higher layer parameters should be aligned with those in 38.331. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correction of the parameters. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE might refer to different parameters. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.3.0, 16.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 16.3.0	UE procedure for transmitting PSFCH with control information

<Unchanged parts omitted>

A UE can be enabled, by sl-iInter-UECoordinationScheme2, to transmit a PSFCH with conflict information in a resource pool. The UE can determine, based on an indication by a SCI format 1-A, a set of resources that includes one or more slots and resource blocks that are reserved for PSSCH transmission. If the UE determines a conflict for a reserved resource for PSSCH transmission, the UE provides conflict information in a PSFCH.

<Unchanged parts omitted>

A first UE determines a UE for providing the conflict information to in a PSFCH as follows

-	if, for a resource pool, typeAUEScheme2 sl-TypeUE-A is disabled, the first UE has been indicated a first reserved resource and a second reserved resource as resources for PSSCH reception or, if for a resource pool typeAUEScheme2 sl-TypeUE-A is enabled, has been indicated at least the first reserved resource or the second reserved resource for PSSCH reception,

-	detects a first SCI format 1-A that includes a first priority value, p1, and the first reserved resource for PSSCH transmission from a second UE,

-	detects a second SCI format 1-A that includes a second priority value, p2<p1, and the second reserved resource for PSSCH transmission from a third UE, and

-	determines that the first and second resources overlap in time and frequency

-	the PSFCH occasions for resource conflict information of the second UE and the third UE are valid

-	the indicationUEB flag in SCI Format 1-A from the second UE and the third UE is set to 1, if indicationUEBScheme2 sl-IndicationUE-B = 'enabled'

-	determines the first SCI format 1-A and the second SCI format 1-A are not received later than sl-MinTimeGapPSFCH before the PSFCH occasion for conflict information

-	determines to transmit to the second UE the PSFCH with the conflict information

-	determines to transmit to either the second UE or the third UE the PSFCH with the conflict information, if p2=p1

The first UE can be provided conditions by optionForCondition2A1Scheme2 sl-OptionForCondition2-A-1 to determine conflict of reserved resources in a resource pool

-	if optionForCondition2A1Scheme2 sl-OptionForCondition2-A-1 = 'RSRP-ThresPerPriorities' 0, the first UE can be provided by, ThresPSSCH-RSRP-List Thpi,pj, a list of RSRP thresholds for each priority combination pi,pj [6, TS 38.214]

-	if the first UE is an intended receiver for PSSCH in a reserved resource of the second UE, the first UE determines a resource conflict if the RSRP [6, TS 38.214] of the third UE is above a threshold Thp2,p1

-	if the first UE is an intended receiver for PSSCH in a reserved resource of the third UE, the first UE determines a resource conflict if the RSRP of the second UE is above a threshold Thp1,p2

-	if optionForCondition2A1Scheme2 sl-OptionForCondition2-A-1 = 'RSRP-ThresWithRsrpMeasurement' 1, the first UE can be provided a value Delta_Th by deltaRSRPThresh

-	if the first UE is an intended receiver for PSSCH in a reserved resource of the second UE, the first UE determines a resource conflict if RSRP2>RSRP1+Delta_Th, where RSRP1 and RSRP2 are the RSRP measurements from the first UE for the second UE and the third UE, respectively

-	if the first UE is an intended receiver for PSSCH in a reserved resource of the third UE, the first UE determines a resource conflict if RSRP1>RSRP2+Delta_Th

If a UE transmits a PSFCH with conflict information corresponding to a reserved resource indicated in an SCI format 1-A, the UE transmits the PSFCH in the resource pool in a slot determined based on PSFCHOccasionScheme2 sl-PSFCH-Occasion

-	If PSFCHOccasionScheme2 sl-PSFCH-Occasion = 'followSCI' 0, the UE transmits the PSFCH in a first slot that includes PSFCH resources and is at least a number of slots, provided by sl-MinTimeGapPSFCH, of the resource pool after a slot of a PSCCH reception that provides the SCI format 1-A. The PSFCH resource is in a slot that is at least T3 slots [6, TS 38.214] before the resource associated with the conflict information; otherwise, the UE does not transmit the PSFCH with conflict information.

-	If PSFCHOccasionScheme2 sl-PSFCH-Occasion = 'followReservedResource' 1, the UE transmits the PSFCH in a latest slot that includes PSFCH resources and is at least T3 slots before a slot of the resource associated with conflict information. The PSFCH resource is in a slot that is at least sl-MinTimeGapPSFCH slots after a slot of a PSCCH reception that provides the SCI format 1-A; otherwise, the UE does not transmit the PSFCH with conflict information.

<Unchanged parts omitted>

### 16.3.1	UE procedure for receiving PSFCH with control information

<Unchanged parts omitted>

A UE that transmitted SCI format 1-A, indicating one or more reserved resources in a resource pool enabled by sl-iInter-UECoordinationScheme2, attempts to receive associated PSFCH with conflict information in a resource pool with PSFCH resources that the UE determines as described in clause 16.3.0. If the UE determines presence of a resource conflict based on conflict information in a PSFCH reception, the UE reports the resource conflict to higher layers

-	if slotLevelResourceExclusionScheme2 sl-SlotLevelResourceExclusion is not provided, the UE reports resources overlapping with a next in time reserved resource indicated by the SCI format 1-A

-	if slotLevelResourceExclusionScheme2 sl-SlotLevelResourceExclusion is provided, the UE reports resources in a slot of a next in time reserved resource indicated by the SCI format 1-A

If a UE receives a PSFCH with conflict information corresponding to a reserved resource indicated in an SCI format 1-A, the UE receives the PSFCH in the resource pool in a slot determined based on PSFCHOccasionScheme2 sl-PSFCH-Occasion

-	If PSFCHOccasionScheme2 sl-PSFCH-Occasion = 'followSCI' 0, the UE receives the PSFCH in a first slot that includes PSFCH resources and is at least a number of slots, provided by sl-MinTimeGapPSFCH, of the resource pool after a slot of a PSCCH transmission that provides the SCI format 1-A. The PSFCH resource is in a slot that is at least T3 slots [6, TS 38.214] before the resource associated with the conflict information; otherwise, the UE does not receive the PSFCH with conflict information

-	If PSFCHOccasionScheme2 sl-PSFCH-Occasion = 'followReservedResource' 1, the UE receives the PSFCH in a latest slot that includes PSFCH resources and is at least T3 slots before a slot of the resource associated with conflict information. The PSFCH resource is in a slot that is at least sl-MinTimeGapPSFCH slots after a slot of a PSCCH transmission that provides the SCI format 1-A; otherwise, the UE does not receive the PSFCH with conflict information

<Unchanged parts omitted>