3GPP TSG RAN WG1 #110bis-e																R1- 2209989

e-Meeting, October 10th – 19th, 2022

Agenda item:	9.5.1.1

Source: 	Qualcomm Incorporated

Title: 	Sidelink Positioning Evaluation Assumptions and Results

Document for:		Discussion and Decision

# Introduction

The study on expanded and improved NR positioning introduces sidelink positioning as an objective [1]:

- Study solutions for sidelink positioning considering the following: [RAN1, RAN2] - Scenario/requirements - Coverage scenarios to cover: in-coverage, partial-coverage and out-of-coverage - Requirements: Based on requirements identified in TR38.845 and TS22.261 and TS22.104 - Use cases: V2X (TR38.845), public safety (TR38.845), commercial (TS22.261), IIOT (TS22.104) - Spectrum: ITS, licensed - Identify specific target performance requirements to be considered for the evaluation based on existing 3GPP work and inputs from industry forums [RAN1] - Define evaluation methodology with which to evaluate SL positioning for the uses cases and coverage scenarios, reusing existing methodologies from sidelink communication and from positioning as much as possible [RAN1]. - Study and evaluate performance and feasibility of potential solutions for SL positioning, considering relative positioning, ranging and absolute positioning: [RAN1, RAN2] - Evaluate bandwidth requirement needed to meet the identified accuracy requirements [RAN1] - Study of positioning methods (e.g. TDOA, RTT, AOA/D, etc) including combination of SL positioning measurements with other RAT dependent positioning measurements (e.g. Uu based measurements) [RAN1] - Study of sidelink reference signals for positioning purposes from physical layer perspective, including signal design, resource allocation, measurements, associated procedures, etc, reusing existing reference signals, procedures, etc from sidelink communication and from positioning as much as possible [RAN1] - Study of positioning architecture and signalling procedures (e.g. configuration, measurement reporting, etc) to enable sidelink positioning covering both UE based and network based positioning [RAN2, including coordination and alignment with RAN3 and SA2 as required] Note: When the bandwidth requirements have been determined and the study of sidelink communication in unlicensed spectrum has progressed, it can be reviewed whether unlicensed spectrum can be considered in further work. Checkpoint at RAN#97 to see if sufficient information is available for this review.

In this contribution, we provide evaluation results for the different use cases and scenarios.

# Evaluation Results

## 2.1 V2X Scenarios

This section captures the evaluation assumptions used in the V2X evaluations and the results:

| Parameter | V2X Simulation Assumptions |
| --- | --- |
| Carrier frequency | 5.9 MHz |
| Subcarrier spacing | 30 KHz SCS |
| Reference Signal Transmission Bandwidth | 40, 100 MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) | Comb-4/4-symbols without interference across the devices |
| Reference signal including PRS, SRS and SL-PRS | QPSK sequence, 1 port |
| Number of symbols used per occasion | 4 |
| number of occasions used per positioning estimate | 1 |
| Power-boosting level | 0 dB |
| Uplink power control (applied/not applied) | N/A |
| interference modelling (ideal muting, or other) | Ideal muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | Threshold-based IFFT TOA Estimation |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | Least square |
| Synchronization assumptions | Ideal |
| UE/gNB RX and TX timing error assumption | Ideal |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | 2x2 |

| Parameters | Case 1 | Case 2 |
| --- | --- | --- |
| BS/RSU deployment for relative positioning/ranging | - N/A | - Parallel |
| Positioning method | - RTT | - Multi-RTT |
| Scenario | - Highway | - Highway |

| Case ID and brief description | 50% | 67% | 80% | 90% | Whether meet the requirement of set A | Whether meet the requirement of set B |
| --- | --- | --- | --- | --- | --- | --- |
| Case #1, 100MHz, RTT | 0.6 | 0.85 | 1.09 | 1.3 | No | Yes |
| Case #1, 40MHz, RTT | 1.55 | 2.1 | 2.6 | 3.3 | No | No |
| Case #2, 100MHz, Multi-RTT | 2.6 | 3.78 | 5.0 | 6.6 | No | No |
| Case #2, 40MHz, Multi-RTT | 3.2 | 6.3 | 8.0 | 10.0 | No | No |

Figure 1 CDF of range estimation error in a highway V2X scenario.

Observation 1: Set A and Set B requirements for ranging in V2X applications cannot be met by the bandwidth available for ITS bands, i.e. 40 MHz.

Observation 2: Set A requirements for ranging in V2X is achieved when the SL-PRS bandwidth is 100 MHz.

Observation 3: Set B requirements for ranging in V2X is not met when the SL-PRS bandwidth is 100 MHz or less.

Observation 4: 0.5m ranging accuracy in V2X is achieved for 42% of the UE when the SL-PRS bandwidth is 100 MHz.

Figure 3 Absolute positioning error in the highway scenario.

Observation 5: Neither Set A nor Set B absolute-positioning requirements can be met in the highway scenario

## 2.2 Public Safety Scenarios

The following simulation assumptions are used for evaluation of public safety scenarios:

| Parameter | Public Safety Simulation Assumptions |
| --- | --- |
| Carrier frequency | 700 MHz (FDD) |
| Subcarrier spacing | 15 KHz SCS |
| Reference Signal Transmission Bandwidth | 10, 20, 40 MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) | Comb-4/4-symbols without interference across the devices |
| Reference signal including PRS, SRS and SL-PRS | QPSK sequence, 1 port |
| Number of symbols used per occasion | 4 |
| number of occasions used per positioning estimate | 1 |
| Power-boosting level | 0 dB |
| Uplink power control (applied/not applied) | Not applied |
| interference modelling (ideal muting, or other) | Ideal muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | Threshold-based IFFT TOA Estimation |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | RANSAC with a cost function minimization |
| Synchronization assumptions | Ideal |
| UE/gNB RX and TX timing error assumption | Ideal |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Single Antenna used for transmission at a UE |
| Additional notes, if any | Channel model from TS 36.843 A.2.1.2 |

Table p-1 Assumptions for public safety if they are different from or not specified in the agreements

| Parameters | Case 1 | Case 2 | Case 3 | Case 4 | Case 5 | Case 6 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidth | 10MHz | 20 MHz | 40 MHz | 10MHz | 20 MHz | 40 MHz |  |  |
| UE Antenna model | (1,1,2) |  |  |  |  |  |  |  |
| TRP antenna model | (2,2,2) |  |  |  |  |  |  |  |
| BS/RSU deployment for absolute positioning | - UMI 200m 19-site Hexagonal grid |  |  |  |  |  |  |  |
| BS/RSU deployment for relative positioning/ranging | - UMI 200m 19-site Hexagonal grid |  |  |  |  |  |  |  |
| Selected values of X (relative positioning or ranging is performed between two UEs within X m) | - 20 outdoor UEs per sector uniformly dropped in the xy plane. Each device is trying to do ranging with the closest 10 devices. |  |  |  |  |  |  |  |
| Positioning method | - RTT | - RTT | - RTT | - AoA | - AoA | - AoA |  |  |

Table p-2 Simulation results for public safety for absolute positioning - horizontal accuracy

| Case ID and brief description | 50% | 67% | 80% | 90% | Whether meet the requirement |
| --- | --- | --- | --- | --- | --- |
| Case 1, 10 MHz, RTT+AoA | 2 m | 2.6 m | 3.1 m | 4.6 m | No (20% meets < 1m) |
| Case 2, 20 MHz, RTT+AoA | 1.6 m | 2 m | 2.3 m | 2.8 m | No (25% meets < 1m) |
| Case 3, 40 MHz, RTT+AoA | 0.3 m | 0.4 m | 0.7m | 1.1m | No (87.5% meets < 1m) |
| Case 1, 10 MHz, RTT | 2.4 m | 3 m | 4.1 m | 5.8 m | No (12% meets < 1m) |
| Case 2, 20 MHz, RTT | 1.9 m | 2.5 m | 2.8 m | 3.7 m | No (15% meets < 1m) |
| Case 3, 40 MHz, RTT | 0.4 m | 0.5 m | 0.9 m | 1.8m | No (80% meets < 1m) |

Table p-6 Simulation results for public safety for ranging - distance accuracy

| Case ID and brief description | 50% | 67% | 80% | 90% | Whether meet the requirement |
| --- | --- | --- | --- | --- | --- |
| Case 1, 10 MHz, RTT | 3.3 m | 4.6 m | 8.1 | >50 m | No (5% meets <1m) |
| Case 2, 20 MHz, RTT | 1.4 m | 2.1 m | 3.3 m | 9.5 m | No (15% meets <1m) |
| Case 3, 40 MHz, RTT | 0.7 m | 1.1 m | 4.7 m | 13.5 m | No (25% meets <1m) |

Table p-7 Simulation results for public safety for ranging positioning - angle accuracy

| Case ID and brief description | 50% | 67% | 80% | 90% | Whether meet the requirement |
| --- | --- | --- | --- | --- | --- |
| Case 4, 10 MHz, AoA | 4.7 deg | 8.0 deg | 11.6 deg | 16.1 deg | Set A: No (86.5% meets < 15 deg) Set B: No (67.5% meets < 8 deg) |
| Case 5, 20 MHz, AoA | 2.7 deg | 5.1 deg | 7.3 deg | 11.0 deg | Set A: Yes Set B: No (83% meets < 8 deg) |
| Case 6, 40 MHz, AoA | 2.7 deg | 4.4 deg | 6.4 deg | 10.5 deg | Set A: Yes Set B: No (86% meets < 8 deg) |

Observation 6: AoA measurements provide incemental accuracy gains over RTT-only positioning mainly for scenarios with smaller bandwidths.

Observation 7: For the simulated public safety scenario, achieving 1m accuracy at 90% requires more than 20 MHz of SL bandwidth.

## 2.3 Commercial Scenarios

The following simulation assumptions are used for evaluation of commercial scenarios:

| Parameter | Commercial Simulation Assumptions |
| --- | --- |
| Carrier frequency | 3.5 GHz (TDD) |
| Subcarrier spacing | 30 KHz SCS |
| Reference Signal Transmission Bandwidth | 100 MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) | Comb-4/4-symbols without interference across the devices |
| Reference signal including PRS, SRS and SL-PRS | QPSK sequence, 1 port |
| Number of symbols used per occasion | 4 |
| number of occasions used per positioning estimate | 1 |
| Power-boosting level | 0 dB |
| Uplink power control (applied/not applied) | Not applied |
| interference modelling (ideal muting, or other) | Ideal muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | Threshold-based IFFT TOA Estimation |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | RANSAC with a cost function minimization |
| Synchronization assumptions | Ideal |
| UE/gNB RX and TX timing error assumption | Ideal |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Single Antenna used for transmission at a UE |
| Additional notes, if any | Channel model from TS 36.843 A.2.1.2 |

| Parameter | Value |
| --- | --- |
| Cellular Deployment layout Assumption and Channel model | UMI FR1, 200m ISD, 3.5 GHz, 30 KHz SCS, according to the simulation assumptions of TS 38.855 |
| D2D Channel model | Channel model from TS 36.843 A.2.1.2 |
| UE drop | 10 outdoor UEs per sector uniformly dropped in the xy plane. All UEs are outdoors, and no buildings are dropped. |
| Anchor UE selection for absolute positioning | Closest 10 devices are being simulated for ranging and Angle of Arrival (AoA) Estimation |
| SL PRS configuration | Comb-2/2-symbols without interference across the devices |

Table c-1 Assumptions for commercial use case if they are different from or not specified in the agreements

| Parameters | Case 1 | Case 2 | Case 3 |
| --- | --- | --- | --- |
| Scenario | 100 MHz, 3 SL UEs, SL/Uu-Positioning | 100 MHz, 3 SL UEs, SL-Only | 100 MHz, 3 SL UEs, SL-Only |
| UE Antenna model | (1,2,2) | (1,2,2) | (1,2,2) |
| TRP antenna model | (2,16,2) | (2,16,2) | (2,16,2) |
| BS/RSU deployment for absolute positioning | - UMI 200m 19-site Hexagonal grid |  |  |
| BS/RSU deployment for relative positioning/ranging | - UMI 200m 19-site Hexagonal grid |  |  |
| Selected values of X (relative positioning or ranging is performed between two UEs within X m) | - 10 outdoor UEs per sector uniformly dropped in the xy plane. Each device is trying to do ranging with the closest 10 devices. |  |  |
| Positioning method | - RTT+AoA | - RTT+AoA | - RTT |

Table c-2 Simulation results for commercial for absolute positioning - horizontal accuracy

| Case ID and brief description | 50% | 67% | 80% | 90% | Whether meet the requirement |
| --- | --- | --- | --- | --- | --- |
| Case 1, 100 MHz, FR1, SL/Uu-Positioning, 3 SL Anchors, Uu RTT+AoA & SL RTT+AoA | 0.3 m | 0.4 m | 0.5 m | 0.6 m | Yes |
| Case 2, 100 MHz, FR1, SL-Only, 3 SL Anchors, SL RTT+AoA | 0.3 m | 0.4 m | 0.5 m | 0.7 m | Yes |
| Case 3, 100 MHz, FR1, SL-Only, 3 SL Anchors, SL RTT | 0.35 m | 0.38 m | 0.75 m | 1.8 m | No (85% meets <1m) |

Observation 8: For the simulated commercial scenario, significant gains are observed even when a small number of SL anchors are being employed  in the positioning engine.

Observation 9: For absolute positioning in the commercial scenario, having 3 or more SL anchors achieves sub-meter performance even without Uu measurements.

Observation 10: For absolute positioning in the commercial scenario, RTT+AoA shows significant gains over RTT-only Positioning, especially at the tail (e.g. 90% 95% CDF point).

## 2.4 IIoT Scenarios

In the next table we summarize the main parameters for the evaluation of IIoT scenarios:

| Parameter | IIoT Simulation Assumptions |
| --- | --- |
| Carrier frequency | 3.5 GHz (TDD) |
| Subcarrier spacing | 30 KHz SCS |
| Reference Signal Transmission Bandwidth | 100 MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) | Comb-4/4-symbols without interference across the devices |
| Reference signal including PRS, SRS and SL-PRS | QPSK sequence, 1 port |
| Number of symbols used per occasion | 4 |
| number of occasions used per positioning estimate | 1 |
| Power-boosting level | 0 dB |
| Uplink power control (applied/not applied) | Not applied |
| interference modelling (ideal muting, or other) | Ideal muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | Threshold-based IFFT TOA Estimation |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | RANSAC with a cost function minimization |
| Synchronization assumptions | Ideal |
| UE/gNB RX and TX timing error assumption | Ideal |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Single Antenna used for transmission at a UE |
| Additional notes, if any | Channel model from TS 36.843 A.2.1.2 |

Table i-1 Assumptions for IIOT if they are different from or not specified in the agreements

| Parameters | Case 1 | Case 2 |
| --- | --- | --- |
| UE Antenna model | (1,2,2) | (1,2,2) |
| TRP antenna model | (4,4,2) | (4,4,2) |
| BS/RSU deployment for absolute positioning | - InF-SH, 18 TRPs | - InF-SH, 18 TRPs |
| BS/RSU deployment for relative positioning/ranging | - InF-SH, 18 TRPs | - InF-SH, 18 TRPs |
| Selected values of X (relative positioning or ranging is performed between two UEs within X m) | - Closest 10 devices are being simulated for ranging and Angle of Arrival (AoA) Estimation |  |
| Positioning method | - RTT+AoA | - RTT+AoA |

Table i-2 Simulation results for IIOT for absolute positioning - horizontal accuracy

| Case ID and brief description | 50% | 67% | 80% | 90% | Whether meet the requirement of set A | Whether meet the requirement of set B |
| --- | --- | --- | --- | --- | --- | --- |
| Case 1, 100 MHz, InF-SH, 3 SL Anchors, RTT+AoA | 0.1 m | 0.18 m | 0.25 m | 0.45 m | Yes | No (70% meet <0.2m) |
| Case 2, 100 MHz, InF-SH, 5 SL Anchors, RTT+AoA | 0.1 m | 0.18 m | 0.3 m | 0.7 m | Yes | No (70% meet <0.2m) |

Observation 11: For absolute positioning in the InF-DH scenario, having 3 or more SL anchors achieves sub-meter performance even without Uu measurements (e.g. 90% 95% CDF point).

# Conclusions

Observation 1: Set A and Set B requirements for ranging in V2X applications cannot be met by the bandwidth available for ITS bands, i.e. 40 MHz.

Observation 2: Set A requirements for ranging in V2X is achieved when the SL-PRS bandwidth is 100 MHz.

Observation 3: Set B requirements for ranging in V2X is not met when the SL-PRS bandwidth is 100 MHz or less.

Observation 4: 0.5m ranging accuracy in V2X is achieved for 42% of the UE when the SL-PRS bandwidth is 100 MHz.

Observation 5: Set A and Set B absolute horizontal accuracy requirements for the longitudinal position are met for the highway scenario with 100 MHz SL PRS bandwidth using the parallel RSU distribution.

Observation 6: Only 11% and 55% of UEs meet the 0.5m and 1.5m absolute horizontal accuracy requirements, respectively, for the longitudinal position for the highway scenario with 40 MHz SL PRS bandwidth using the parallel RSU distribution.

Observation 7: Set A and Set B absolute horizontal accuracy requirements for the longitudinal position is met for the highway scenario with 100 MHz SL PRS bandwidth using the staggered RSU distribution.

Observation 8: Only 17% and 72% of UEs meet the 0.5m and 1.5m absolute horizontal accuracy requirements, respectively, for the longitudinal position for the highway scenario with 40 MHz SL PRS bandwidth using the staggered RSU distribution.

Observation 9: Set A and Set B absolute accuracy requirements for the lateral position are not met for the highway scenario using sidelink-only positioning.

Observation 10: The RSU drop model for highway scenarios leads to very large diluation of precision in the lateral position.

Observation 11: AoA measurements provide incemental accuracy gains over RTT-only positioning mainly for scenarios with smaller bandwidths.

Observation 12: For the simulated public safety scenario, achieving 1m accuracy at 90% requires more than 20 MHz of SL bandwidth.

Observation 13: For the simulated commercial scenario, significant gains are observed even when a small number of SL anchors are being employed  in the positioning engine.

Observation 14: For absolute positioning in the commercial scenario, having 3 or more SL anchors achieves sub-meter performance even without Uu measurements.

Observation 15: For absolute positioning in the commercial scenario, RTT+AoA shows significant gains over RTT-only Positioning, especially at the tail (e.g. 90% 95% CDF point).

Observation 16: For absolute positioning in the InF-DH scenario, having 3 or more SL anchors achieves sub-meter performance even without Uu measurements (e.g. 90% 95% CDF point).

Proposal 1: For evaluation of sidelink positioning, sidelink PRS and other sidelink communications are not FDMed with each other, i.e. they can only be TDMed.

# References

- RP-213588 “Revised SID on Study on expanded and improved NR positioning,” Intel (Email discussion moderator), RAN #94-e.

- 3GPP TS 38.885, “Study on NR Vehicle-to-Everything (V2X) (Release 16)”

- 3GPP TS 37.885, “Study on evaluation methodology of new Vehicle-to-Everything (V2X) use cases for LTE and NR (Release 15)”

- 3GPP TS 38.101-1 “User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone.”

- RAN1 Chair Notes, RAN1 #103 Eelectronic Meeting.

- 3GPP TS 36.843, “Study on LTE device to device proximity services (Release 12)”

- R1-2205036, “Sidelink Positioning Scenarios and Requirements,” Qualcomm, Inc. RAN1 #109.

- R1-2205038, “Potential Solutions for Sidelink Positioning,” Qualcomm, Inc. RAN1 #109.

# Appendix

## 5.1 Simulation Assumptions

This section includes the simulation assumptions used for the V2X evaluations in Section 3.

| UE drop | Highway |
| --- | --- |
| Lanes | 6, 3 in each direction. |
| Number of UEs | 42 |
| UE speed | 140 km/h |
| Ranging method | SL-RTT, single-shot, genie calibration. |
| Antenna configuration | 2x2 |

## 5.2 Bounds on Accuracy

This sections presents the theoretical analysis based on the Cramer-Rao bound. Assuming a peak-finding algorithm for estimating the time of arrival (ToA), the variance of such and estimator is lower-bounded by:

σToA2≥18π2⋅M⋅SNRT⋅Brms2

Where

- M is the number of antennas, assumed to be 2 in this ranging evaluation.

- SNRT is the signal energy of the direct path over the noise spectral density.

- Assumed uniform PSD for the SL-PRS signal, then Brms2=B2/3 with B= 20, 40, 100 MHz in the evaluations.

For SL-RTT, the time uncertainty is twice as much as ToA.

Using the above bound, we obtain the lower bound on range uncertainty in an AWGN setting shown in Figure 2. We note that the simulations in Figure 1 used a fading channel model and using that a more realistic channel model instead of AWGN for the lower bound analysis would further increase the lower bound.

Figure 2 Theoretical lower bound SL-RTT range uncertainty in AWGN with for SL-PRS bandwidths 20, 40, and 100 MHz