3GPP TSG-RAN WG1 #110-bis-e	R1-2210178

e-meeting, October 10th – 19th, 2022

Agenda Item:	9.5.2.3

Source:	Ericsson

Title:	Evaluations for Low Power High Accuracy Positioning

Document for:	Discussion, Decision

# Introduction

The SID for the for Rel-18[1] includes the following objective for low power high accuracy positioning (LPHAP)

- Study the requirements on LPHAP as developed by SA1 and evaluate whether existing RAN functionality can support these power consumption and positioning requirements. Based on the evaluation, and, if found beneficial, study potential enhancements to help address any limitations [RAN2, RAN1] - Study is limited to a single representative use case (use case 6 as defined TS 22.104). The choice of selected use case can be reviewed at the start of the study. - Study is limited to enhancements to RRC_INACTIVE and/or RRC_IDLE state

In this contribution, we evaluate the baseline performance of UL and DL positioning according to the previous meetings agreements for the power consumption.  The power consumption for extended DRX durations and ultra deep sleep power profiles, agreed during RAN1#110,  are also evaluated.  Finally, additional issue related to timing and synchronization and SRS mobility are raised for further discussion.

# Discussion

## Reference device average power consumption and target LPHAP power consumption.

### List of agreements

During RAN1#109e, a reference device was agreed to be defined, with parameters to be studied further for definition of battery life and power consumption. In RAN1#110, the following was agreed for the definition of the reference device used for evaluations:

Agreement In the LPHAP evaluation, adopt the following model to convert the relative power unit to the battery life: - Alt. 1: battery life is used as the metric to identify the gap - K is an implementation factor, K = 1 (baseline); K = 0.5, 2, 4 (optional) - Note: The definition of the notations will be captured in the updates of TR. - Note: The voltage is assumed to be the same for the reference device and the LPHAP device. Note: the variables were defined during RAN1#109e as: - C1 is the battery capacity of the reference device; - T1 is the battery life of the reference device; - P1 is the relative power unit obtained based on the reference traffic type; - X is the percentage of the power consumed by the reference traffic type; - C2 is the battery capacity of the LPHAP device; - P2 is the evaluated relative power unit of the LPHAP device; - P2_req is the target relative power unit of the LPHAP device; - T2_req is the target battery life of the LPHAP device Agreement - In the LPHAP evaluation, adopt the following example parameter values in the conversion model to evaluate the battery life: - For the reference device in the conversion model: C1 (mAh); T1 (hour); X; reference traffic type. 4500; 12; 20%; FTP (model 3). - For the LPHAP device, consider 2 types in the conversion model: LPHAP device; C2 (mAh); T2req (month). Type A (baseline); 800; 6~12. Type B (optional); 4500; 6~12. - Note: As the reference device and LPHAP device characteristics, and therefore the parameter values of the model for determining battery life, is dependent on implementation factors, manufacturer, design options and cost options, it is up to individual company to evaluate the optional K values, and report the corresponding parameter values. Agreement In the LPHAP evaluation, adopt the example value of relative power unit of the reference device P1 = 50 to further align the battery life among companies.

### Power consumption target

Considering reference battery life of T1 of 12 hours, reference power consumption P1=50 PUs, the agreed reference battery size C1=4500mAh and a LPHAP target battery life of 1 year (maximum case) the target power consumption P2 for the LPHAP device can be summarized as follow for different values of K and C2.

| LPHAP device | K=0.5 | K=1 (baseline) | K=2 | K=4 |
| --- | --- | --- | --- | --- |
| Type A (baseline), C2=800mAh | 0.0304 | 0.0608 | 0.1217 | 0.2434 |
| Type B (optional), C2=4500mAh | 0.1711 | 0.3422 | 0.6845 | 1.3689 |

For the baseline of K=1 and Type A LPHAP UE with C2=800mAh, the average PU consumption target is 0.0608

## Power consumption evaluation

### DL TDOA power consumption evaluation

The following was agreed for the evaluation of DL positioning:

Agreement - For DL positioning, at least the following power components and parameter values are considered for the baseline evaluation of Rel-17 RRC_INACTIVE positioning: - For the UE-assisted DL positioning, - SSB proc. with 2 ms duration and the periodicity of I-DRX cycle; - Paging with 2 ms duration, the periodicity of I-DRX cycle, and group paging rate of 10%; - DL PRS measurement with 0.5 ms duration; - CG-SDT with 1ms duration and the periodicity of positioning interval; - RRCRelsease after the CG-SDT can be optionally included with [1] ms duration; - (Optional) BWP switching with [1] ms duration; - (Optional) Intra-/inter-frequency RRM measurement in low SINR condition with [1] ms duration; - (Optional) RA-SDT (e.g., including CORSET0 + SIB1, PRACH, RAR, Msg 3/4/5) in case of CG-SDT is unavailable; - For the UE-based DL positioning, - SSB proc. with 2 ms duration and the periodicity of I-DRX cycle; - Paging with 2 ms duration, the periodicity of I-DRX cycle, and group paging rate of 10%; - DL PRS measurement with 0.5 ms duration; - (Optional) BWP switching with [1] ms duration; - (Optional) Intra-/inter-frequency RRM measurement in low SINR condition with [1] ms duration; - Note: The power component and parameter values for UE-assisted DL positioning is also applicable to the DL part of UE-assisted DL+UL positioning method. - Note: Individual company may consider additional power components and different parameter values in bracket in the evaluation. - Note: Companies are encouraged to provide the assumption on the timeline between different power consumption events in the evaluation of potential enhancements to reduce the transition times between different power states and to extend the sleeping time as much as possible.

We already know that a single slot of PRS measurement are enough to produce a good TOA/RSTD measurement at the receiver from the rel17 evaluation. Additionally, we assume a single PRS per TRP in FR1, aligned with the TRP’s SSB. Therefore, we assume that it is not necessary for the UE to accumulate measurement from the same TRP over multiple DRX cycles.

With a single PRS to measure per TRP, 18 PRSs in total should be measured.  According to the agreed evaluation options, we can have 4 or 8 TRP measured. With  4 TRP measured per DRX cycle, 5 DRX cycles per measurement update (30s) are required to span the whole 18 TRPs, which can be achieved with 1.28 DRX period. With 8 TRP measured per DRX, it is possible to scan the whole 18TRPs within 3 DRX periods, which allows to use 10.24sec DRX and still be in the 30 sec update rate targeted by case 6 requirements. If 1.28s DRX period is configured, it is enough to configure PRS every N=8DRX if 8 TRPs can be measured per DRX.

With 8 TRP measured per DRX, it is possible to completely sound the indoor factory deployment in 3 10.24s DRX cycles, and still be within the reporting periodicity budget of use case 6 (30 secs).

Knowledge of the DRX pattern configured to the UE by the LMF is beneficial in order to optimize the assistance data.

The following combinations are considered:

Case 1 and 2 : 8 TRP receptions, DRX period of 1.28s, N=8, 23dBm PUSCH for CG based reporting.

Three  DRX with PRS measurement per 30sec period

Single DRX with CG SDT reporting

Case 3 and 4: 8 TRP receptions, DRX period of 10.24s, N=1, 23dBm PUSCH for CG based reporting.

Three  DRX with PRS measurement per 30sec period

Single DRX with CG SDT reporting

For each combination, we also evaluated different Deep Sleep strategy

    - Only deep sleep for both the sleep between paging and receiving PRS and for the remaining time after receiving PRS or sending the measurement report

    - Only deep sleep for the sleep between paging and receiving PRS and ultra deep sleep as agreed in RAN1#110 for the remaining time after receiving PRS or sending the measurement report

    - Note: only ultra deep sleep option 1 (0.015PU slot consumption, 400ms and 2000PU transition)

Figure 1 power consumption profile for reception of DL PRS

The detailed results are given in appendix for inclusion in TR 38.859.  the  average power consumption assement is given below. We note that, as observe in RAN1#110, sleep accounts for more than 99 percent of the UE power consumption.:

Table 1 Results summary for DL methods power consumption

| Evaluation case description | Slot-averaged relative power unit (P2) | Battery life (in month) | Target requirement are met – Yes/No; If no, provide gaps |  |
| --- | --- | --- | --- | --- |
|  |  |  | 6 months | 12 months |
| Case 1, : 8 TRP receptions, DRX period of 1.28s, N=8, 23dBm PUSCH for CG based reporting, Deep sleep | 1,7978 | 0.4 | no | no |
| Case 2, : 8 TRP receptions, DRX period of 1.28s, N=8, 23dBm PUSCH for CG based reporting, Deep sleep+ultra deep sleep | 0.8273 | 0.9 | No | no |
| Case 3, : 8 TRP receptions, DRX period of 10.24s, N=1, 23dBm PUSCH for CG based reporting Deep sleep | 1.1440 | 0.6 | No | no |
| Case 4, : 8 TRP receptions, DRX period of 10.24s, N=1, 23dBm PUSCH for CG based reporting Deep sleep+ultra deep sleep | 0.1609 | 4.5 | no | no |

### UL TDOA power consumption evaluation

For UL TDOA, we assume an SRS configured to coincide with the DRX cycle so that there is 1 UL SRS per positioning occasion and per N DRX cycle. As agreed in RAN1#109e, N depends on the DRX cycle duration:

Agreement Adopt the following periodicity of DL PRS / UL SRS for positioning in the baseline evaluation of Rel-17 RRC_INACTIVE positioning: - 1 DL PRS / UL SRS for positioning occasion per N I-DRX cycle(s); - Candidate values of N to evaluate is 1 and 8 for I-DRX cycle of 1.28s; - Note: Individual company may consider either one or both in the evaluation. - Candidate value of N to evaluate is 1 for I-DRX cycle of 10.24s. Agreement For the purpose of LPHAP evaluation, the following assumptions on eDRX configuration and/or paging reception can be optionally considered: - The eDRX cycle to evaluate: 20.48s; 30.72s; - For paging reception: - 1 paging occasion is included in one eDRX cycle - 10% paging rate - No paging reception can be optionally evaluated; - 1 DL PRS and/or UL SRS for positioning occasion per 1 eDRX cycle - Minimizing the gap between PRS measurement, SRS transmission and/or measurement reporting with paging monitoring in time domain can be evaluated.

We note that for the considered use case with an update rate of 30 seconds, it could be enough to transmit SRS as little as once per measurement update, as it was shown in rel17 that UL SRS in indoor factory provided good coverage without the need for repeated transmission. Therefore, further power saving may be possible by using a the agreed eDRX of 30.72s . We also note that short SRS (2 symbols, comb2) is enough to provide coverage in the indoor factory case. Therefore, we limit our evaluation to the case of 210 PU consumption, i.e. the baseline case.

Short SRS is sufficient to provide UL coverage in InF cases

We consider the following cases for power consumption in UL TDOA:

Case 5 and 6:  DRX period of 10.24s, N=1,   short SRS transmssion

Three  DRX with SRS measurement per 30sec period

Case 7 and 8: 4 TRP receptions, DRX period of 30.72s, N=1,   short SRS transmission

24  DRX with SRS measurement per 30sec period

The power consumption profile for transmission of UL SRS is shown in figure 2. The power consumption for each signals in the UL TDOA procedure is summarized in table 2 for each cases.

Figure 2 power consumption profile for transmission of UL SRS

The power consumption for each case is summarized as follow:

Table 2 Results summary for UL methods power consumption

| Evaluation case description | Slot-averaged relative power unit (P2) | Battery life (in month) | Target requirement are met – Yes/No; If no, provide gaps |  |
| --- | --- | --- | --- | --- |
|  |  |  | 6 months | 12 months |
| Case 5, DRX period of 10.24s, N=1, short SRS transmission, 3 DRX with SRS per 30s period Deep Sleep | 1.0958 | 0.66 | no | no |
| Case 6, DRX period of 10.24s, N=1, short SRS transmission, 3 DRX with SRS per 30s period Ultra Deep Sleep | 0.1099 | 6.63 | yes | no |
| Case 7, DRX period of 30.72s, N=1, short SRS transmission, 1 DRX with SRS per 30s period Deep Sleep | 1. 0319 | 0.7 | No | no |
| Case 8, DRX period of 30.72s, N=1, short SRS transmission, 1 DRX with SRS per 30s period Ultra Deep Sleep | 0.0466 | 15.7 | yes | yes |

## Summary of Evaluations

In the following evaluations, we will only consider the baseline case of K=1, type A LPHAP UE.

We evaluated UL  TDOA and DL TDOA for the two DRX cycle durations of 1.28s and 10.24s.  The evaluations shows ultra deep sleep is required to even consider meeting the required power consumption for 6/12 months battery life. With Ultra deep sleep, the lowest power consumption occurs for UL positioning, with the required power consumption for 6 months battery life being met  with SRS with 10.24s DRX, and 12 month battery life can be achieved with extending the DRX to 30.720s. . This can be explained by the long DRX length which reduces the number of paging measurements and SRS transmissions.

The evaluation summary tables formatted for the SI TR are available in appendix.  It can be seen that the requirements may be met with ultra deep sleep with UL SRS with DRX periods of 10 (6months battery life) or 30 seconds (1 year battery life).

The proposed ultra deep sleep option 1 profile, together with 10.24s DRX cycle with a single SRS transmission per DRX, fulfill the power requirements for 6 months battery life

The proposed ultra deep sleep option 1 profile, together with 30.72s DRX cycle with a single SRS transmission per DRX, fulfill the power requirements for 12 months battery life

## Considerations for the impact of timing / UE-network synchronization

Since UL methods are evaluated to be the most power effective, the issue of SRS timing synch must be considered in order to make sure interference levels are kept low and UEs can be multiplexed without interfering with each other.

Synch mechanisms, such as RA-based synch using the RA procedure to send a TA adjustment to the UE can be used, but are costly in power  as they required multiple signals/channels transmission and reception, and cannot (as of now) be triggered by the network if synch loss is detected.  For the network to trigger a TA adjustment, MT-SDT may be needed (to be specified in Rel18), or the UE would have to go back to connected mode but it is not reasonable to bring the UE back to RRC connected just for receiving a MAC CE TA adjustment, considering the PU cost, unless these adjustment are needed very infrequently, i.e. with a periodicity in minutes or more. This is the case when the UE being considered is mostly static such as in warehouse asset tracking.

The margin for the UL cases is as follow:

    - For the case of UL SRS transmitted with 10.24s DRX, the slot average power unit (P2) is 0.1099. the target requirement was 2*0.0608PU for 6months battery life, giving a margin of 0.117. the margin translate to a margin of 2340 PU per DRX that could be used.

    - For the case of UL SRS transmitted with 30.70s DRX, the slot average power unit (P2) is 0.0466. the target requirement was 0.0608PU for 1 year battery life, giving a margin of 0.0499. the margin translate to a margin of 8520 PU per DRX that could be used.

SDT procedure for synching of the UL SRS may introduce addition power consumption that need to be taken into account if needed frequently.

## Consideration for SRS mobility

SRS mobility enhancements was proposed in  [NOK, HW, ZTE, CATT, vivo] during previous RAN1 meetings. In order for the UE to save power by avoiding reconfiguration of the SRS when moving from cell to cell, it was proposed for the UE to be able to preserve its SRS configuration when changing cell while in RRC_INACTIVE.

In order to maintain an acceptable level of interference, cells where the UE in RRC inactive is roaming need to be aware of the SRS configuration, and since the UE is not in connected state, the different cells must all assume the UE is utilizing the SRS resource. This means that the time/frequency allocation of the SRS configuration must be allocated at all time in all cells where there UE is expected to roam, which is ineficient from the network perspective.

The configuration of SRS in multiple cell to allow SRS mobility for RRC_INACTIVE UE is not efficient from the resource utilization perspective.

On the other hand, from the power efficiency perspective, it would be beneficial to be able to reconfigure the SRS without bringing the UE back to connected mode. Procedure for reconfiguration of the SRS as part of the RA procedure could be investigated, as proposed during RAN1#110. However, we believe this could be done in RAN2. RAN1 could provide further power consumption evaluations of the proposed procedures if needed.

# Conclusion

In the previous sections we made the following observations:

Observation 1	For the baseline of K=1 and Type A LPHAP UE with C2=800mAh, the average PU consumption target is 0.0608

Observation 2	With 8 TRP measured per DRX, it is possible to completely sound the indoor factory deployment in 3 10.24s DRX cycles, and still be within the reporting periodicity budget of use case 6 (30 secs).

Observation 3	Knowledge of the DRX pattern configured to the UE by the LMF is beneficial in order to optimize the assistance data.

Observation 4	Short SRS is sufficient to provide UL coverage in InF cases

Observation 5	The proposed ultra deep sleep option 1 profile, together with 10.24s DRX cycle with a single SRS transmission per DRX, fulfill the power requirements for 6 months battery life

Observation 6	The proposed ultra deep sleep option 1 profile, together with 30.72s DRX cycle with a single SRS transmission per DRX, fulfill the power requirements for 12 months battery life

Observation 7	SDT procedure for synching of the UL SRS may introduce addition power consumption that need to be taken into account if needed frequently.

Observation 8	The configuration of SRS in multiple cell to allow SRS mobility for RRC_INACTIVE UE is not efficient from the resource utilization perspective.

# References

RP-213588, “Study on expanded and improved NR positioning”, Intel, December 2021.

# Appendix – Summary of Evaluations

Table 3.3-1: Evaluation cases and assumptions

| Evaluation assumption | Case 1,FR1, UE-A DL pos, Type A UE | Case 2,FR1, UE-A DL pos, Type A UE | Case 3,FR1, UE-A DL pos, Type A UE | Case 4,FR1, UE-A DL pos, Type A UE | Case 5,FR1, UL pos, Type A UE | Case 6,FR1, UL pos, Type A UE | Case 7,FR1, UL pos, Type A UE | Case 8,FR1, UL pos, Type A UE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sleep state | deep sleep | Ultra deep sleep opt 1 | deep sleep | Ultra deep sleep opt 1 | deep sleep | Ultra deep sleep opt 1 | deep sleep | Ultra deep sleep opt 1 |
| DRX cycle | 1.28s | 1.28s | 10.24s | 10.24s | 10.24s | 10.24s | 30.72s | 30.72s |
| paging reception | yes | yes | yes | yes | yes | yes | yes | yes |
| RS periodicity | 1 PRS occ per DRX | 1 PRS occ per DRX | 1 PRS occ per DRX | 1 PRS occ per DRX | 1 SRS occ per DRX | 1 SRS occ per DRX | 1 SRS occ per DRX | 1 SRS occ per DRX |
| M-sample | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| RRM measurement | no | No | no | No | no | no | no | no |
| BWP switching | yes | Yes | yes | Yes | yes | yes | yes | yes |
| Measurement reporting (e.g., RA/CG-SDT, reporting interval) | 1per 30s | 1per 30s | 1per 30s | 1per 30s | No | No | No | No |
| implementation factor K | baseline | Baseline | baseline | Baseline | baseline | baseline | baseline | baseline |
| Note |  |  |  |  |  |  |  |  |

- Note: Companies are recommended to provide the following information for each evaluation case:

- Case ID

- Positioning method: e.g., UE-assisted DL positioning, UL positioning, UE-assisted DL+UL positioning, etc.

- Frequency range: e.g., FR1

- LPHAP device type: e.g., Type A, Type B

Table 3.3-2-1: UE power consumption result for PRS based positioning (DL-TDOA)

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case 1 | e.g., Deep/light/micro sleep, SSB, paging, PRS measurement, UL, SRS, etc |  |  |  |  |  |  |
|  | SSB |  | 4 | 24 | 96 | 0.0781 | 4% |
|  | Light sleep | 20+100 transition | 20 | 24 | 480 | 0.1758+0.0879 | 10%+5% |
|  | Paging | 57 | 4 | 24 | 96 | 0.0445 | 2% |
|  | Deep sleep | 1+ 450 transition | 60 | 3 | 180 | See below | See below |
|  | BWP switching for PRS |  | 2*2 | 3 | 4 | 0.0098 | 1% |
|  | PRS meas. |  | 1 | 3 | 3 | 0.0073 | <1% |
|  | Light sleep | 20+100 transition | 20 | 1 | 20 | See above | See above |
|  | CG SDT (PUSCH+RRCrelease) | 700+120 | 4 | 1 | 4 | 0.0267 | 1% |
|  | Deep Sleep | 1+450 transition | Remaining slots | 1 | Remaining slots | 0.9800 | 55%+22% transition |
|  | Total (every power cycle) |  |  |  | 61440 | 110395 |  |
|  | Slot-averaged power unit |  |  |  |  | 1.7968 |  |
|  | Battery life (in month) |  |  |  |  |  |  |
| Case 2 | e.g., Deep/light/micro sleep, SSB, paging, PRS measurement, UL, SRS, etc |  |  |  |  |  |  |
|  | SSB |  | 4 | 24 | 96 | 0.0781 | 9% |
|  | Light sleep | 20+100 transition | 20 | 24 | 480 | 0.1758+0.0879 | 21%+11% |
|  | Paging | 57 | 4 | 24 | 96 | 0.0445 | 5% |
|  | Deep sleep | 1+ 450 transition | 60 | 3 | 180 | 0.0234+0.3516 | 3+42% |
|  | BWP switching for PRS |  | 2*2 | 3 | 4 | 0.0098 | 1% |
|  | PRS meas. |  | 1 | 3 | 3 | 0.0073 | <1% |
|  | Light sleep | 20+100 transition | 20 | 1 | 20 | See above | See above |
|  | CG SDT (PUSCH+RRCrelease) | 700+120 | 4 | 1 | 4 | 0.0267 | 3% |
|  | Ultra deep Sleep | 0.015 + 2000 transition | Remaining slots | 1 | Remaining slots | 0.0143+0.0078 | 2%+1% transition |
|  | Total (every power cycle) |  |  |  | 61440 | 50829 |  |
|  | Slot-averaged power unit |  |  |  |  | 0.8273 |  |
|  | Battery life (in month) |  |  |  |  |  |  |
| Case 3 | e.g., Deep/light/micro sleep, SSB, paging, PRS measurement, UL, SRS, etc |  |  |  |  |  |  |
|  | SSB |  | 4 | 3 | 12 | 0.0098 | 4% |
|  | Light sleep | 20+100 transition | 20 | 3 | 60 | 0.0260+0.0130 | 2%+1% |
|  | Paging | 57 | 4 | 3 | 12 | 0.0056 | <1% |
|  | Deep sleep | 1+ 450 transition | 60 | 3 | 180 | See below | See below |
|  | BWP switching for PRS |  | 2*2 | 3 | 4 | 0.0098 | 1% |
|  | PRS meas. |  | 1 | 3 | 3 | 0.0073 | <1% |
|  | Light sleep | 20+100 transition | 20 | 1 | 20 | See above | See above |
|  | CG SDT (PUSCH+RRCrelease) | 700+120 | 4 | 1 | 4 | 0.0267 | 2% |
|  | Deep Sleep | 1+450 transition | Remaining slots | 1 | Remaining slots | 0.9974+0.0483 | 87%+4% transition |
|  | Total (every power cycle) |  |  |  | 61440 | 70287 |  |
|  | Slot-averaged power unit |  |  |  |  | 1.1440 |  |
|  | Battery life (in month) |  |  |  |  |  |  |
| Case 4 | e.g., Deep/light/micro sleep, SSB, paging, PRS measurement, UL, SRS, etc |  |  |  |  |  |  |
|  | SSB |  | 4 | 3 | 12 | 0.0098 | 6% |
|  | Light sleep | 20+100 transition | 20 | 3 | 60 | 0.0260+0.0130 | 16%+8% |
|  | Paging | 57 | 4 | 3 | 12 | 0.0056 | <3% |
|  | Deep sleep | 1+ 450 transition | 60 | 3 | 180 | 0.0029+0.0439 | 2%+27% |
|  | BWP switching for PRS |  | 2*2 | 3 | 4 | 0.0098 | 6% |
|  | PRS meas. |  | 1 | 3 | 3 | 0.0073 | <5% |
|  | Light sleep | 20+100 transition | 20 | 1 | 20 | See above | See above |
|  | CG SDT (PUSCH+RRCrelease) | 700+120 | 4 | 1 | 4 | 0.0267 | 17% |
|  | Ultra Deep Sleep opt 1 | 0.015+2000 transition | Remaining slots | 1 | Remaining slots | 0.0149+0.00097656 | 87%+4% transition |
|  | Total (every power cycle) |  |  |  | 61440 | 9885 |  |
|  | Slot-averaged power unit |  |  |  | 0.1609 |  |  |
|  | Battery life (in month) |  |  |  |  |  |  |

Table 3.3-2-2: UE power consumption result for SRS based positioning (UL-TDOA)

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case 5 | e.g., Deep/light/micro sleep, SSB, paging, PRS measurement, UL, SRS, etc |  |  |  |  |  |  |
|  | SSB |  | 4 | 3 | 12 | 0.0098 | <1% |
|  | Light sleep | 20+100 transition | 20 | 3 | 60 | 0.0391+0. 0195 | 4%+2% |
|  | Paging | 57 | 4 | 3 | 12 | 0.0056 | <1% |
|  | Light sleep | 20+100 transition | 20 | 3 | 60 | See above | See above |
|  | BWP switching for PRS |  | 2*2 | 3 | 12 | 0.0098 | <1% |
|  | SRS Transmission |  | 1 | 3 | 3 | 0.0103 | <1% |
|  | Deep Sleep | 1+450 transition | Remaining slots | 1 | Remaining slots | 0.9975+ 0.0044 | 91%+1% transition |
|  | Total (every power cycle) |  |  |  | 61440 | 67326 |  |
|  | Slot-averaged power unit |  |  |  | 1.0958 |  |  |
|  | Battery life (in month) |  |  |  |  |  |  |
| Case 6 | e.g., Deep/light/micro sleep, SSB, paging, PRS measurement, UL, SRS, etc |  |  |  |  |  |  |
|  | SSB |  | 4 | 3 | 12 | 0.0098 | 9% |
|  | Light sleep | 20+100 transition | 20 | 3 | 60 | 0.0391+0. 0195 | 36%+18% |
|  | Paging | 57 | 4 | 3 | 12 | 0.0056 | 5% |
|  | Light sleep | 20+100 transition | 20 | 3 | 60 | See above | See above |
|  | BWP switching for PRS |  | 2*2 | 3 | 12 | 0.0098 | 9% |
|  | SRS Transmission |  | 1 | 3 | 3 | 0.0103 | 9% |
|  | Ultra Deep Sleep opt 1 | 0.015+2000 transition | Remaining slots | 1 | Remaining slots | 0.0150+ 9.7656e-04 | 14%+1% transition |
|  | Total (every power cycle) |  |  |  | 61440 | 6752 |  |
|  | Slot-averaged power unit |  |  |  | 0.1099 |  |  |
|  | Battery life (in month) |  |  |  |  |  |  |
| Case 7 | e.g., Deep/light/micro sleep, SSB, paging, PRS measurement, UL, SRS, etc |  |  |  |  |  |  |
|  | SSB |  | 4 | 1 | 4 | 0.0033 | <1% |
|  | Light sleep | 20+100 transition | 20 | 1 | 20 | 0.0130+0. 065 | 1%+<1% |
|  | Paging | 57 | 4 | 1 | 4 | 0.0019 | <1% |
|  | Light sleep | 20+100 transition | 20 | 1 | 20 | See above | See above |
|  | BWP switching for PRS |  | 2*2 | 1 | 4 | 0.0033 | <1% |
|  | SRS Transmission |  | 1 | 1 | 1 | 0.0034 | <1% |
|  | Deep Sleep | 1+450 transition | Remaining slots | 1 | Remaining slots | 0.9992+ 0.0015 | 97%+1% transition |
|  | Total (every power cycle) |  |  |  | 61440 | 63400 |  |
|  | Slot-averaged power unit |  |  |  | 1. 0319 |  |  |
|  | Battery life (in month) |  |  |  |  |  |  |
| Case 8 | e.g., Deep/light/micro sleep, SSB, paging, PRS measurement, UL, SRS, etc |  |  |  |  |  |  |
|  | SSB |  | 4 | 1 | 4 | 0.0033 | 7% |
|  | Light sleep | 20+100 transition | 20 | 1 | 20 | 0.0130+0. 065 | 28%+14% |
|  | Paging | 57 | 4 | 1 | 4 | 0.0019 | 4% |
|  | Light sleep | 20+100 transition | 20 | 1 | 20 | See above | See above |
|  | BWP switching for PRS |  | 2*2 | 1 | 4 | 0.0033 | 7% |
|  | SRS Transmission |  | 1 | 1 | 1 | 0.0034 | 7% |
|  | Ultra Deep Sleep opt 1 | 0.015+2000 transition | Remaining slots | 1 | Remaining slots | 0.0150+ 3.2552e-4 | 32%+1% transition |
|  | Total (every power cycle) |  |  |  | 61440 | 2863.1 |  |
|  | Slot-averaged power unit |  |  |  | 0. 0466 |  |  |
|  | Battery life (in month) |  |  |  |  |  |  |

Table 3.3-3: Summary for UE power consumption results

| Evaluation case description | Slot-averaged relative power unit (P2) | Battery life (in month) | Target requirement are met – Yes/No; If no, provide gaps |  |
| --- | --- | --- | --- | --- |
|  |  |  | 6 months | 12 months |
| Case 1, : 8 TRP receptions, DRX period of 1.28s, N=8, 23dBm PUSCH for CG based reporting, Deep sleep | 1,7978 | 0.4 | No (5.6) | No (11.6) |
| Case 2, : 8 TRP receptions, DRX period of 1.28s, N=8, 23dBm PUSCH for CG based reporting, Deep sleep+ultra deep sleep | 0.8273 | 0.9 | No (5.1) | No (11.1) |
| Case 3, : 8 TRP receptions, DRX period of 10.24s, N=1, 23dBm PUSCH for CG based reporting Deep sleep | 1.1440 | 0.6 | No (5.4) | No (11.4) |
| Case 4, : 8 TRP receptions, DRX period of 10.24s, N=1, 23dBm PUSCH for CG based reporting Deep sleep+ultra deep sleep | 0.1609 | 4.5 | No (5.5) | No (11.5) |
| Case 5, DRX period of 10.24s, N=1, short SRS transmission, 3 DRX with SRS per 30s period Deep Sleep | 1.0958 | 0.66 | No (5.3) | No (11.3) |
| Case 6, DRX period of 10.24s, N=1, short SRS transmission, 3 DRX with SRS per 30s period Ultra Deep Sleep | 0.1099 | 6.63 | yes | No (5.4) |
| Case 7, DRX period of 30.72s, N=1, short SRS transmission, 1 DRX with SRS per 30s period Deep Sleep | 1. 0319 | 0.7 | No (5.3) | No (11.3) |
| Case 8, DRX period of 30.72s, N=1, short SRS transmission, 1 DRX with SRS per 30s period Ultra Deep Sleep | 0.0466 | 15.7 | yes | Yes |

# Additional Evaluated DRX patterns and sleep power consumption

During RAN1#109e and RAN1#110, the following was agreed regarding DRX and sleep power consumption:

Agreement For the purpose of LPHAP evaluation, an ultra-deep sleep state is considered. The following options of the power consumption model of the ultra-deep sleep state can be further discussed: - Option 1: - The relative power unit: 0.015 - Additional transition energy: 2000 - Total transition time: 400ms - Option 2: - The relative power unit: 0.01 - Additional transition energy: 450; - Total transition time: 25ms - FFS: restrictions in processing associated with option 2 after the UE comes out of ultra-deep sleep state - Notes: the values above can be further discussed Agreement For option 1 in the agreement above, the value of additional transition energy is changed to “a value between 2000 and 20000”. FFS which value. Agreement Adopt the following periodicity of DL PRS / UL SRS for positioning in the baseline evaluation of Rel-17 RRC_INACTIVE positioning: - 1 DL PRS / UL SRS for positioning occasion per N I-DRX cycle(s); - Candidate values of N to evaluate is 1 and 8 for I-DRX cycle of 1.28s; - Note: Individual company may consider either one or both in the evaluation. - Candidate value of N to evaluate is 1 for I-DRX cycle of 10.24s. Agreement - The I-DRX configuration is included in the baseline evaluation of Rel-17 RRC_INACTVIE positioning. - Note: This does not preclude the case where no I-DRX cycle nor paging is considered in the evaluation of potential solutions to maximize the battery life. - Adopt the following I-DRX cycle to evaluate: - 1.28s (baseline); 10.24s (optional). Agreement For the purpose of LPHAP evaluation, the following assumptions on eDRX configuration and/or paging reception can be optionally considered: - The eDRX cycle to evaluate: 20.48s; 30.72s; - For paging reception: - 1 paging occasion is included in one eDRX cycle - 10% paging rate - No paging reception can be optionally evaluated; - 1 DL PRS and/or UL SRS for positioning occasion per 1 eDRX cycle - Minimizing the gap between PRS measurement, SRS transmission and/or measurement reporting with paging monitoring in time domain can be evaluated.