3GPP TSG-RAN WG1 Meeting #110bis-e 	R1-2210215

eMeeting, October 10 – 19, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [DRAFT] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR |  | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifying ambiguous usage of TCI-State |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_feMIMO-Core |  |  |  |  |  | Date: |  |  | 2022-09-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | It is mentioned in Clause 5.1.5 of 38.214 that - The UE is not expected to be configured with TCI-State in a CC in a band if the UE is configured dl-OrJoint-TCIStateList in any CC in the same band; and - The UE can assume that when the UE is configured with TCI-State in any CC in the CC list configured by simultaneousTCI-UpdateList1-r16, simultaneousTCI-UpdateList2-r16, simultaneousSpatial-UpdatedList1-r16, or simultaneousSpatial-UpdatedList2-r16, the UE is not configured with dl-OrJoint-TCIStateList in any CC within the same band in the CC list. Above is meant to specify that Rel-17 unified TCI state and the legacy TCI state cannot be simultanuously configured for a UE in the same band. However, according to the latest 38.331, both tci-StatesToAddModList and dl-OrJoint-TCIStateList configure TCI-State which means that TCI-State is used for both Rel-17 unified TCI state and the legacy TCI state. Therefore, the current text in Clause 5.1.5 of 38.214 should be corrected to clarify that dl-OrJoint-TCIStateList and tci-StatesToAddModList cannot be configured for a UE in the same band. |  |  |  |  |  |  |  |  |
| Summary of change: |  | TCI-State is changed to tci-StatesToAddModList in two occasions in Clause 5.1.5 of 38.214 to clarify the actual intention of the spec that Rel-17 unified TCI state and the legacy TCI state cannot be simulatanuously configured in the same band. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The current specification is incorrect as it states that UE is not expected to be configured with TCI-State and dl-OrJoint-TCIStateList in the same band. However, dl-OrJoint-TCIStateList is a container that includes a set of TCI-State. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

5.1.5	Antenna ports quasi co-location

<Unchanged parts are omitted>

If the TCIState or UL-TCIState configurations are absent in a BWP of the CC, the UE can apply the TCIState or UL-TCIState configurations from a reference BWP of a reference CC. The UE is not expected to be configured with tci-StatesToAddModList, SpatialRelationInfo or PUCCH-SpatialRelationInfo, except SpatialRelationInfoPos in a CC in a band, if the UE is configured with dl-OrJoint-TCIStateList or UL-TCIState in any CC in the same band. The UE can assume that when the UE is configured with tci-StatesToAddModList in any CC in the CC list configured by simultaneousTCI-UpdateList1-r16, simultaneousTCI-UpdateList2-r16, simultaneousSpatial-UpdatedList1-r16, or simultaneousSpatial-UpdatedList2-r16, the UE is not configured with dl-OrJoint-TCIStateList or UL-TCIState in any CC within the same band in the CC list.

<Unchanged parts are omitted>