3GPP TSG-RAN WG1 Meeting #110bis-e										R1-220xxxx

e-Meeting, Oct 10th - 19th, 2022

Agenda Item:	8.17

Source:			Moderator (ZTE)

Title:			Summary of preparation phase email discussion for Rel-17 SDT

Document for:	Discussion

Introduction

This document contains the summary of issues related to the maintenance of Rel-17 Small data transmission WI in RAN1#110bis-e meeting. The following email discussion is initiated to collect companies’ views:

[110bis-e-R17-Others-01] Email discussion to determine maintenance issues to be handled in RAN1#110bis-e by October 12 – Ziyang (ZTE)

- Additional email discussions will be set up once the maintenance issues for RAN1#110bis-e are determined

Preparation phase discussion

The following issues are collected from companies’ contributions and summarized for the maintenance of Rel-17 SDT in RAN1#110bis-e.

| Issue # | Description | TDoc # |
| --- | --- | --- |
| 1 | CD-SSBs for RedCap UE SSB to PUSCH mapping - For RedCap UE performing SDT, vivo proposes that RAN1 should conclude that only CD-SSBs in initial DL BWP is used for SSB to CG PUSCH mapping and further discuss whether any specification change is needed to explicitly restrict the SSBs to be only among the SSBs configured by ssb-PositionsInBurst in SIB1 or by ServingCellConfigCommon. | R1-2208623 vivo |
| 2 | Redundancy versions of repetitions for CG-SDT In last meeting, RAN1 captures RAN2 agreement in TS 38.213 as “For initial transmission or autonomous retransmission of an initial transport block provided for the PUSCH transmission as described in clause 18.0 in [19, TS 38.300], the UE encodes the transport block using redundancy version number 0”. In the draft CR, vivo proposes that the redundancy version determination should be based on clause 6.1.2.3 of 38.214 when repK-RV is provided. | R1-2208624 vivo |
| 3 | Repetitions for CG-SDT In RAN1#110, in LS R1-2205736, RAN2 has the following reply on repetitions: “With regards to the above issue about repetition for CG-SDT, the signalling in TS 38.331 reuses the existing ConfiguredGrantConfig and hence the signalling allows configuration of parameters related to repetition (i.e. repK, repK-RV (including repK-r17), pusch-RepTypeIndicator-r16 and frequencyHoppingPUSCH-RepTypeB-r16) within this IE.” Therefore, ZTE, Xiaomi and Samsung propose to discuss how to capture repetitions in RAN1 spec. | R1-2208711 ZTE R1-2209710 Samsung R1-2209711 Samsung R1-2209255 Xiaomi |
| 4 | Editorial correction on parameter name Parameter name sdt-SSB-perCG-PUSCH in TS 38.213 in section 19.1 is not aligned with TS 38.331. | R1-2208712 ZTE |
| 5 | Power offset of feature combination In the feature combination, power offset between Msg3 or MsgA-PUSCH and RACH preamble transmission namely deltaPreamble is introduced. If configured, this parameter overrides msg3-DeltaPreamble or msgA-DeltaPreamble. But in the current spec, this parameter is missing in TS 38.213, then the power determination of Msg3 and MsgA PUSCH in feature combination is incorrect. So ZTE proposes that parameter deltaPreamble should be captured in section 7.1.1. | R1-2208713 ZTE |
| 6 | QCL assumption for DL reception during subsequent SDT procedure Xiaomi proposes that the DL beam for PDCCH and PDSCH reception can be changed along with the selected SSB for legacy RACH and the text in clause 19.1 and 19.2 for TS 38.213 can be changed. | R1-2209255 Xiaomi |
| 7 | Collision handling between valid PO and DL resources for HD-FDD UEs For RedCap UE supporting SDT, if valid CG-SDT resource overlaps with DL resource, Xiaomi proposes to discuss the collision handling rule. | R1-2209255 Xiaomi |
| 8 | Paging monitoring for RedCap UE in separate BWP Huawei proposes that if separate initial BWP does not include CD-SSB and CORESET#0, but is configured for SDT, a UE does not expect to be scheduled on the BWP during paging/SI update indication monitoring procedure and during CG-SDT resource verifying procedure. - The BWP paging/SI update indication monitoring procedure may include every paging monitoring occasions, SSB acquisition time, the possible retuning time, the time between the paging and the updated SI. - The CG-SDT resource verifying procedure may include SSB transmission time before every CG resources and the possible retuning time. | R1-2209831 Huawei |

To identify the essential issues, please provide your assessment “H/N/E” on these maintenance issues with the following classification:

- High priority (H): high-priority item (essential, pending issues, broken spec components) and proposed editorial changes that either enhance the clarity of the specs or correct mistakes

- Non-essential (N): all other purposes such as spec optimization and low priority issues

- Editorial (E): editorial issues that will be handled as editorial CRs (to be communicated to the editors/chairs)

| Company | Issue#1 | Issue#2 | Issue#3 | Issue#4 | Issue#5 | Issue#6 | Issue#7 | Issue#8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

Any other comments?

| Company | Comment |
| --- | --- |

Summary

The outcome of email discussion will be updated later.

References

- R1-2208623	Remaining issues for NR small data transmissions in RRC INACTIVE state	vivo

- R1-2208624	Corrections of reduncancy version of repeated CG PUSCH for SDT	vivo

- R1-2208711	Correction on repetitions for CG-SDT	ZTE, Sanechips

- R1-2208712	Correction on SSB subset for CG-SDT	ZTE, Sanechips

- R1-2208713	Correction on deltapreamble of feature combination	ZTE, Sanechips

- R1-2209710	Discussion on the repetition aspect for SDT in active state	Samsung

- R1-2209711	Draft CR for the repetition aspect for SDT in active state	Samsung

- R1-2209255	Discussion on physical layer aspects of small data transmission	xiaomi

- R1-2209831	Remaining issues on paging monitoring and measurement during SDT procedure	Huawei, HiSilicon