3GPP TSG RAN WG1 #110bis-e                                      R1-220xxxx

e-Meeting, October 10th – 19th, 2022

Agenda item:	9.17

Title: 	FL summary #1 of remaining issues for two overlapping CRS patterns

Source: 	Moderator (ZTE Corporation)

Document for:	Discussion and Decision

- Discussion

In [R1-2208859, OPPO], it proposes that the LTE CRS pattern lists in the following 38.211 text should also be extended to include at least lte-CRS-PatternList3. It may not be necessary in certain cases to include lte-CRS-PatternList4, because the adding of lte-CRS-PatternList4 needs the adding of lte-CRS-PatternList3 as a pre-requisite.

##### 7.4.1.1.2 Mapping to physical resources # …… For PDSCH mapping type A # …… - single-symbol DM-RS, l1=11 except if all of the following conditions are fulfilled in which case l1=12: - the higher-layer parameter lte-CRS-ToMatchAround, lte-CRS-PatternList1, or lte-CRS-PatternList2, or lte-CRS-PatternList3 is configured; and - the higher-layer parameter dmrs-AdditionalPosition is equal to 'pos1' and l0=3; and - the UE has indicated it is capable of additionalDMRS-DL-Alt # For PDSCH mapping type B # …… - if the higher-layer parameter lte-CRS-ToMatchAround, lte-CRS-PatternList1, or lte-CRS-PatternList2, or lte-CRS-PatternList3 is configured, the PDSCH duration ld=10 symbols for normal cyclic prefix, the subcarrier spacing configuration μ=0, single-symbol DM-RS is configured, and at least one PDSCH DM-RS symbol in the PDSCH allocation collides with a symbol containing resource elements as indicated by the higher-layer parameter lte-CRS-ToMatchAround, lte-CRS-PatternList1, or lte-CRS-PatternList2, lte-CRS-PatternList3, or lte-CRS-PatternList4, then l shall be incremented by one in all slots. # ……

First round

Per FL’s understanding, the issue is valid and the proposed TP seems correct. Therefore, FL proposes to adopt above TP. Companies are encouraged to provide your views below.

| Company | Views |
| --- | --- |

Reference

R1-2208859, 	Discussion on remaining issues in Rel-18 eDSS	, OPPO.