3GPP TSG RAN WG1 #111 	     		  R1-2211136

Toulouse, France, November 14th – 18th, 2022

Title:	[Draft] Reply LS on RAN dependency for Ranging/Sidelink Positioning

Response to:	R1-2210821 / S2-2209961

Release:	Rel-18

Work Items:	FS_Ranging_SL

Source:	CATT [to be RAN WG1]

To:	SA WG2

CC:	RAN WG2, RAN WG3

Contact Person:

Name:	Xiaotao REN

E-mail Address:	renxiaotao@catt.cn

Attachment:

1. Overall Description:

RAN1 thanks SA2 for the LS on RAN dependency for Ranging/Sidelink Positioning, which lists several issues and expects RAN WGs to evaluate them and give SA2 the responses. The following issues in the LS have been identified to be related to RAN1:

3) To support Ranging/SL Positioning using Assistant UE, how the determination of using assistant UE and the assistant UE selection/reselection is performed from RAN perspective? 6) For out-of-coverage SA2 would like to understand how resource coordination and scheduling will be done to enable SL Positioning/Ranging.

RAN1 has discussed above issues in RAN1#111, and would provide the following responses to SA2:

Response to SA2:

- Regarding the issue 3) in the SA2 LS, RAN1 would like to provide the following response:

  - From RAN1 perspective, there is no difference between Anchor UE and SL reference UE, and RAN1 assumes that any distinction between Assistant UE and SL reference UE is transparent to RAN1. Hence, the determination of using assistant UE and the assistant UE selection/reselection are the same as these of Anchor UE from RAN1 perspective.

- Regarding the issue 6) in the SA2 LS, RAN1 would like to provide the following response:

- For resource coordination and scheduling under out-of-coverage, RAN1 has identified Scheme 2 (UE autonomous SL-PRS resource allocation) should be introduced to enable SL Positioning/Ranging. RAN1 has made the following agreement on SL-PRS resource allocation in RAN1#110:

Agreement Regarding SL-PRS resource allocation, both Scheme 1 and Scheme 2 should be introduced for supporting SL positioning/ranging: - Scheme 1: Network-centric operation SL-PRS resource allocation (e.g. similar to a legacy Mode 1 solution) - The network (e.g. gNB, LMF, gNB & LMF) allocates resources for SL-PRS. - Scheme 2: UE autonomous SL-PRS resource allocation (e.g. similar to legacy Mode 2 solution) - At least one of the UE(s) participating in the sidelink positioning operation allocates resources for SL-PRS

.

2. Actions:

To SA WG2

ACTION: RAN1 respectfully requests SA2 to take the above responses into account in the future work.

3. Date of Next RAN WG1 Meetings:

TSG-RAN WG1 Meeting #112				Feb. 27 to Mar. 3, 2023		Athens, GR

TSG-RAN WG1 Meeting #112bis-e			Apr. 17 to Apr. 26, 2023		e-meeting