3GPP TSG RAN WG1 #111	R1-2211292

Toulouse, Nov. 14th – 18th, 2022

Source:	Lenovo

Title:	CSI enhancements for high mobility and coherent JT

Agenda Item:	9.1.2

Document for:	Discussion and Decision

# Introduction

In RAN#94-e [1], it was agreed to study, and, if applicable, specify CSI enhancements for CJT under FR1, CSI reporting enhancements for high mobility, in addition to time-domain channel property reporting enhancements. In this contribution we provide our views on different aspects of the three aforementioned scenarios.

# CSI enhancement for coherent joint transmission

In RAN1#110bis-e [2], the following was agreed for CSI enhancements for CJT under MIMO

Agreement On the Type-II codebook refinement for CJT mTRP, following legacy (Rel-16 regular eType-II and Rel-17 PS FeType-II), for a given CSI-RS resource: - SD basis selection is layer-common and polarization-common, with N1, N2, O1, O2 defined per Rel-16 specification for refinement based on Rel-16 regular eType-II, and per Rel-17 specification for refinement based on Rel-17 PS FeType-II - FD basis selection is - For refinement based on Rel-16 regular eType-II: per-layer with Mv, pv, N3, and R defined per Rel-16 specification - For refinement based on Rel-17 PS FeType-II: layer-common with M, N3, and R defined per Rel-17 specification - FFS: Details on FD basis selection window Note: The supported value(s) for each of the defined parameters are to be discussed separately (e.g. possibilities of adding new or removing existing value(s) in addition to those supported by legacy specification). Agreement For the Rel-18 Type-II codebook refinement for CJT mTRP, the constraint on the maximum number of non-zero coefficients (NZCs) per-layer (K0) is defined jointly across all N CSI-RS resources - TBD: the constraint on the total number of NZCs across all layers Agreement For the Rel-18 Type-II codebook refinement for CJT mTRP, - Only CSI reporting over PUSCH is supported - FFS: Whether AP only, or both AP and SP (following legacy), is supported - An associated Resource Setting includes a CMR comprising K≥1 NZP CSI-RS resources from one CSI-RS resource set - Periodic, semi-persistent, and aperiodic NZP CSI-RS are supported - The supported CSI-RS resource parameter settings follow the legacy specification (without additional enhancement) - FFS: Whether or not the K NZP CSI-RS resources are constrained to be in the same slot Agreement For the Rel-18 Type-II codebook for CJT mTRP, the switching between mode-1 and mode-2 is gNB-initiated via RRC signalling Agreement On the SD basis selection for Type-II codebook refinement for CJT mTRP, support the following on the L parameter: - Per-CSI-RS-resource Ln parameter - TBD: Whether {Ln, n=1, ..., N} are higher-layer configured by gNB, or the total n=1NLn is higher-layer configured by gNB while {Ln, n=1, ..., N} are reported by the UE, one L configured and {Ln} determined from configured L - FFS: The value of Ln is taken from a pre-defined set Conclusion On the Type-II codebook refinement for CJT mTRP, regarding W2 quantization group and Strongest Coefficient Indicator (SCI) design, there is no consensus on supporting “strongest” CSI-RS resource indicator in addition to the agreed SCI. - Note: This doesn’t preclude any (future) proposal on reference CSI-RS resource(s) for other purpose(s) Agreement For the Rel-18 Type-II codebook refinement for CJT mTRP, following legacy, support both aperiodic and semi-persistent CSI reporting on PUSCH. Agreement On the Type-II codebook refinement for CJT mTRP, the selection of N CSI-RS resources is performed by UE and reported as a part of CSI report where N∈{1,..., NTRP} - N is the number of cooperating CSI-RS resources, while NTRP is the maximum number of cooperating CSI-RS resources configured by gNB via higher-layer signaling - The selection of N out of NTRP CSI-RS resources is reported via NTRP-bit bitmap in CSI part 1 - Note: The value of N is inferred from the selection - A restricted configuration (gNB-configured via higher-layer signaling) where N=NTRP is supported - NTRP-bit bitmap is not reported when the restriction is configured - FFS: Whether other RRC-configured TRP selection restriction including configuring the value of N is supported - This feature is UE optional Note: This agreement does not impact the decision on Ln being configured by gNB or selected by UE Note: per WID and previous agreement, the candidate values for NTRP of are 1, 2, 3, and 4. Note: only one transmission hypothesis is reported. UE is not mandated to calculate CSI for multiple transmission hypotheses. Agreement On the Type-II codebook refinement for CJT mTRP, regarding W2 quantization group, for each layer: - Support the following: (Alt1) One group comprises one polarization across all N CSI-RS resources (Cgroup,phase=1, Cgroup,amp=2) - FFS: Amplitude quantization table enhancement - For the amplitude group other than the group associated with the SCI, the reference amplitude is reported - Working assumption: Alt3 is supported in addition to Alt1 (to be confirmed in RAN1#111) - (Alt3). One group comprises one polarization for one CSI-RS resource with a common phase reference across N CSI-RS resources (Cgroup,phase=1, Cgroup,amp=2N) - For each of the (2N–1) amplitude groups (other than the group associated with the SCI), the reference amplitude is reported - If the support Alt3 in addition to Alt1 is confirmed, only one of the two schemes will be a basic feature for UEs supporting Rel-18 Type-II CJT codebook Agreement For the Rel-18 Type-II codebook refinement for CJT mTRP, also support a constraint on the total number of non-zero coefficients (NZCs) summed across all layers: - Following the legacy specification, the maximum total number is 2K0 Agreement On the SD basis selection for Type-II codebook refinement for CJT mTRP, on the L parameter, down select from the following alternatives (by RAN1#111): - Alt1. Each of the {Ln, n=1, ..., N} is gNB-configured via higher-layer (RRC) signaling - FFS: The candidate values for Ln, e.g. follow the legacy specification - Alt2. Ltot=n=1NLn where Ltot is gNB-configured via higher-layer (RRC) signaling and the relative value(s) of {Ln, n=1, ..., N} are reported by the UE - TBD: Whether for a given configured value of Ltot, the possible combinations of {Ln, n=1, ..., N} are fixed/pre-determined or gNB-configured via higher-layer (RRC) signaling - TBD: Whether the value(s) of {Ln, n=1, ..., N} are reported implicitly or explicitly, and whether some value(s) don’t need to be reported - FFS: The candidate values for Ln - Alt3. An L parameter is gNB-configured via higher-layer (RRC) signaling and {Ln, n=1, ..., N} are determined from the value of L - TBD: How to determine {Ln, n=1, ..., N} from L, e.g. L1=L and other Ln = L/2 - FFS: The candidate values for L - Alt4. Lmax is gNB-configured via higher-layer (RRC) signaling and the relative value(s) of {Ln, n=1, ..., N} are reported by the UE - The relative value(s) of {Ln, n=1, ..., N} are reported by the UE, such that n=1NLn≤Lmax - TBD: Whether the value(s) of {Ln, n=1, ..., N} are reported implicitly or explicitly, and whether some value(s) don’t need to be reported - FFS: The candidate values for Ln Agreement On the Type-II codebook refinement for CJT mTRP, regarding the codebook parameters, for a given CSI-RS resource, the supported value(s) of the following parameters follow the legacy (Rel-16 regular eType-II and Rel-17 PS FeType-II) specification: - N1, N2, N3, O1, O2 - M (only for design based on Rel-17 PS FeType-II) For the following parameters, decide in RAN1#111 whether the supported value(s) follow the legacy (Rel-16 regular eType-II and Rel-17 PS FeType-II) specification or further refinement is needed: - R: including, e.g. supporting only R=1, or supporting larger R values - Mv/pv (Rel-16 regular eType-II): including, e.g. supporting smaller pv values such as {1/8, 1/4, 1/2} for v=1,2 and/or removing larger legacy value(s) - : including, e.g. supporting smaller values such as {1/16, 1/8, 3/8} Note: The outcome of Parameter Combination discussion will further restrict the supported combinations of parameter value(s) FFS: For N>1, whether the maximum 2N1N2 (identical to the number of CSI-RS ports used for CMR) is limited to 32 just as in legacy specification Agreement On the Type-II codebook refinement for CJT mTRP, regarding the bitmap(s) for indicating the locations of NZCs, down-select from the following alternatives for the size of the bitmap for CSI-RS resource n (Bn) (by RAN1#111): - Alt1. Analogous to legacy, Bn=2LnMv,n (Mv,n=Mv for mode 2) - Alt2. Non-rectangular bitmap, i.e., NZC bitmap allowing different lengths for different SD/FD basis vectors. - TBD: How to determine the lengths for different SD/FD basis vectors Agreement For the Rel-18 Type-II codebook for CJT mTRP, for mode-1, the number of FD basis vectors (Mv related to pv for Rel-16, M for Rel-17) is common across all N CSI-RS resources

In the remainder of this section, we provide our views on codebook CSI enhancement for CJT, in light of the agreements and discussions in the previous two RAN1 meetings.

## Maximum number of CSI-RS ports

In RAN1#110 [3], it was agreed to support N={1,2,3,4} TRPs for CJT with equal priority. Moreover, it was also agreed that each TRP is associated with a distinct NZP CSI-RS resource. In light of both agreements, CSI framework for CJT may require channel measurement and reporting corresponding to up to 128 CSI-RS ports, resulting in significant CSI measurement complexity and CSI reporting overhead. In light of that, setting an upper bound on the maximum number of CSI-RS ports across the N CSI-RS resources to X CSI-RS ports can help reduce this complexity. In our opinion, X=48, 64 are reasonable candidate values to be considered. We therefore have the following proposal

For the Rel-18 Type-II codebook for CJT mTRP, the maximum number of CSI-RS ports across the N NZP CSI-RS resources is set to X CSI-RS ports

- FFS: whether X= 48, 64

## Codebook design

One aspect of potential Rel-18 CJT codebook design is reporting the total number of non-zero coefficients. In our opinion, two alternatives exist:

Alt1. Reporting the total number of non-zero coefficients across all TRPs in Part 1 of the CSI report, whereas the number of non-zero coefficients per TRP can be derived from the per TRP bitmap.

Alt2. Reporting the number of non-zero coefficients per TRP separately in the CSI report. FFS: whether/how they are reported across the two parts of the CSI report

While Alt1 may result in less overall CSI feedback overhead, Alt2 has the advantage of having straightforward mapping between the reported number of non-zero coefficients and the respective PMI. Further study and down selection of both alternatives is needed.

Further study whether/how the number of non-zero coefficients is reported in the CSI report, e.g., separately for each TRP or jointly for all TRPs

In RAN1 #110, it was agreed that the CMR comprises N>1 NZP CSI-RS resources, where one resource corresponds to one TRP/TRP-group, such that each of the CSI-RS resources has a same number of CSI-RS ports. To facilitate CJT, the same N1, N2 value pair, corresponding to the number of antenna ports along two dimensions, can be configured for each CSI-RS resource in CMR, with the same oversampling factor value pair O1, O2 corresponding to N1, N2, respectively. Note that the supported values of N1, N2 may be restricted by a bound on the total number of ports across TRPs for CJT and/or UE capability.

The same N1, N2 and O1, O2 values are configured for each TRP

In RAN1 #110bis-e, four alternatives were agreed for down selection on parameter L for SD basis selection. For Alt1 and Alt3, the number of beams is higher-layer configured by the network. Both alternatives are simple and have less standardization impact since they follow legacy codebook design. For Alt2 and Alt4, the number of beams is determined by the UE and reported to the network, with a configured constraint on the total number of beams across TRPs or a constraint on the maximum number of beams per TRP, respectively. This approach may improve the beam selection efficiency, at the expense of higher design complexity and larger variation in CSI format fed back over the UCI, as well as a larger variation in the CSI feedback overhead. Therefore, we prefer Alt1 or Alt3. For Alt1, each TRP is associated with a distinct value Ln, whereas under Alt3 the number of beams per TRP Ln is determined based on a rule from a network-configured value L, which, in our understanding, includes L1=…=Ln=L. Our preference is Alt3 with a simplified rule of L1=…=Ln=L. In our opinion, supporting unequal number of beams per TRP is unreasonable, whether via configuration or via a rule, since the UE can always omit weaker beams via coefficient selection indicated by the bitmap. Supporting unequal number of beams would lead to asymmetries and irregularities that will complicate the UCI ordering later on. Hence, our preference is Alt3 with L1=…=Ln=L.

Support Alt3 for parameter L of SD basis selection with L1=…=Ln=L

In RAN1 #110, it was also agreed that two modes are supported for Rel-18 Type-II codebook for CJT. Thus, codebook configuration should be designed to support two codebook modes. For legacy Rel-16 eType-II codebook, the values of L, β and pυ are determined by the higher layer parameter paramCombination-r16, which is used to achieve some balance between CSI reporting overhead and system performance. Assuming the value L is fixed for all TRPs, for Mode 1 codebook structure, different values for pv and β can be supported, i.e., pv,n, βn, respectively, and combined configuration parameters can be used, where one L, βn, pυ,n can be configured for each TRP/TRP group n. On the other hand, for Mode 2 codebook structure, one combined configuration parameter L, β, pυ can be configured across all TRPs. Moreover, smaller β value, e.g., 1/8, can be considered to reduce CSI overhead, especially in case of a larger number of TRPs, e.g., N=3,4 for both Mode 1 and Mode 2 codebook structures.

Support separate combined parameter configuration L, βn, pυ,n per TRP for Mode 1 codebook structure and one combined parameter configuration L, β, pυ across TRPs for Mode 2 codebook structure

Support smaller β value, e.g., 1/8 for CSI overhead reduction at N=3,4

## Quantization scheme of the non-zero coefficients

In RAN1 #110b, Alt1 was agreed for W2 quantization group, whereas Alt3 was set as a working assumption that is to be confirmed in RAN1#111. For Alt3, one group comprises one polarization for one CSI-RS resource with a common phase reference across N CSI-RS resources (Cgroup,phase=1, Cgroup,amp=2N). The precision for amplitude quantization is improved by quantization per TRP. It can be used in inter-site CJT scenario where large RSRP difference may exist between TRPs. Moreover, supporting 2N amplitude coefficient groups helps ensure that at least one non-zero coefficient is reported per TRP per polarization, which avoids power amplifier inefficiencies that may be caused if no non-zero coefficient is selected for a given polarization per TRP. Therefore, we support to confirm the work assumption Alt3.

Confirm the work assumption Alt3 for W2 quantization group and strongest coefficient indicator design

For Alt3, for each of the 2N–1 amplitude groups (other than the group associated with the SCI), the reference amplitude is reported. Here, one amplitude group may correspond to one polarization of a TRP/TRP group. For the reference amplitude reporting, differential quantization can be used between the reference amplitude for one amplitude group and the largest reference amplitude across all amplitude groups, where the amplitude coefficient of the largest reference amplitude across all amplitude groups is set to ‘1’. The Rel-16 eType-II reference amplitude quantization for parameter i2,3,l provided in Table 5.2.2.2.5-2, Clause 5.2.2.5 of TS 38.214 [4], can be reused for differential quantization of reference amplitude of one amplitude group since amplitude groups are defined corresponding to both TRP and polarization. Due to the larger number of reference amplitude values, a reduction in the quantization resolution of the 2N–1 reference amplitude values should be further studied

Reuse Rel-16 eType-II reference amplitude quantization for differential quantization of the strongest coefficients for each polarization per TRP

- FFS: whether the number of reference amplitude quantization bits can be reduced

Additionally, in order to limit the increased CSI feedback overhead for N >2 TRPs, one approach that can be adopted is to reduce the amplitude/phase coefficient reporting overhead via supporting two coefficient quantization schemes for CJT with two different resolutions, such that a subset of the non-zero coefficients are quantized with the higher resolution quantization scheme, whereas the remainder of the non-zero coefficients are quantized with the higher resolution quantization scheme. A few alternatives exist for classifying the non-zero coefficients based on the quantization resolution, as follows:

Alt1. TRP-common quantization resolution: If N = 2, both TRPs are associated with a high-resolution quantization scheme. Otherwise, if N > 2, all N TRPs are associated with a lower-resolution quantization scheme.

Alt2. TRP specific quantization: For N > 2, the strongest 2 TRPs are associated with the higher resolution quantization scheme, whereas the remainder N–2 TRPs are associated with the lower resolution quantization scheme.

- Note that a third alternative of the quantization scheme can be based on the strongest N polarizations of a layer, as illustrated in the following alternative.

- Alt3. Polarization-specific quantization: The stronger polarization of each of the N TRPs per layer is associated with the higher resolution quantization scheme, whereas the remainder of coefficients are associated with the lower resolution quantization scheme.

In our opinion, the three alternatives should be studied. Further details are FFS.

For CJT with N TRPs, support two coefficient quantization schemes with different resolution levels, with down selection from the following alternatives:

- Alt1. TRP-common quantization resolution: If N =2, all TRPs are associated with the higher resolution quantization scheme, otherwise if N >2, all TRPs are associated with the lower resolution quantization scheme

- Alt2. TRP-specific quantization resolution: The strongest 2 TRPs are associated with the higher resolution quantization scheme, whereas if N >2, the remainder N–2 TRPs are associated with the lower resolution quantization scheme

- Alt3. Polarization-specific quantization resolution: The stronger polarization of each of the N TRPs per layer is associated with the higher resolution quantization scheme, whereas the remainder of coefficients are associated with the lower resolution quantization scheme

## NZC bitmap design

For CJT codebook design, the CSI feedback overhead corresponding to reporting the bitmap for indicating the locations of the non-zero coefficients is expected to be proportional with the number of TRPs N. In RAN1#110bis-e, it was agreed to consider the following two alternatives for the size of the bitmap for CSI-RS resource n (Bn):

- Alt1. Analogous to legacy, Bn=2LnMv,n (Mv,n=Mv for mode 2)

- Alt2. Non-rectangular bitmap, i.e., NZC bitmap allowing different lengths for different SD/FD basis vectors.

One advantage of Alt1 is its straightforward design based on the legacy eType-II and FeType-II codebooks, and hence the specification effort is reduced. On the other hand, Alt2 is beneficial for scenarios with a sparse number of ones in the bitmaps, e.g., at smaller β values. In our understanding, one interpretation of Alt2 is to report only a subset of the 2LnMv,n bitmap corresponding to each TRP. For instance, assume CJT with Mode 2, with N=2, L1=L2=4, and M=4. Under Alt1, the bitmap overhead is 64 bits. For ease of exposition, we consider the following example:

B1=10000010101100100000001010000000 ,  B2=00000101100011010010000000001000

Based on the example above, only two and three columns of B1, B2, respectively, comprise one-valued entries. If the UE can indicate the column indices with at least one non-zero entry for each bitmap, e.g., via two length-4 bitmaps for both CSI-RS resources, e.g., b1=[1 0 1 0], and b2=[1 1 0 1], the bitmaps B1, B2 overhead can be reduced by reporting B1', B2', as follows

B1'=1001111001010100 ,  B2'=001001100110010010001000

Note that the total UCI overhead corresponding to the bitmaps b1, b2, B1' and B2' is 48 bits, which is a 25% reduction of the CSI overhead corresponding to reporting legacy B1, B2 bitmaps. Note that the same idea can be applied to bitmap rows, i.e., beams. We therefore have the following proposal

For CJT codebook design, support Alt2 for size of bitmap based on non-rectangular bitmaps

For Alt2 bitmap design, a first stage bitmap for each CSI-RS resource indicates the indices of rows and/or columns with at least one non-zero coefficient, and a second stage bitmap whose size is based on the indicated rows and/or columns in the first stage bitmap per CSI-RS resource

One other aspect that needs consideration in bitmap design is that at least one non-zero coefficient needs to be reported per TRP per polarization, i.e., at least two entries with value one per bitmap corresponding to both polarizations. This avoids power amplifier inefficiencies that may be caused if no non-zero coefficient is selected for a given polarization per TRP.

At least two non-zero coefficients corresponding to different polarizations are identified in each bitmap Bn

## Mapping order of CSI fields

For CJT-based CSI report, the CSI report fields may include CSI for multiple TRPs including a maximum of N = 4 cooperative TRPs. The potential CSI report may include a distinct CSI Part 1 and CSI Part 2 for each TRP or a joint CSI Part 1 and CSI Part 2 for the cooperative TRPs, with each jointly designed CSI part comprising jointly encoded parameters across TRPs, e.g., joint basis selection and/or number of non-zero coefficients reporting. For CSI Part 1, it may possibly include fields corresponding to RI, CQI, total number of non-zero coefficients across layers. For CSI Part 2, it may include PMI for multiple cooperative TRPs, where some PMI parameters may be separately or jointly encoded for multiple cooperative TRPs. The mapping order of the bits corresponding to the CSI fields within the CJT CSI report should be carefully designed, given that some fields may exist for multiple cooperative TRPs. For instance, the mapping order of CSI fields may be based on concatenating N groups of bit sequences, each group comprising the CSI fields corresponding to a given TRP. Alternatively, the CSI fields may be grouped by concatenating different groups corresponding to different CSI report quantities, with each CSI report quantity comprising parameters that are ordered with respect to the TRP index. Furthermore, in case additional co-phase/co-amplitude scaling parameters are introduced for per TRP/TRP group SD/FD basis selection or per TRP/TRP group joint SD/FD basis selection, the mapping order of these TRP/TRP group common parameters needs to de decided, which may impact the conventional CSI report partitioning, e.g., modifying the existing group 0/1/2 partitioning of Part 2 CSI report.

Study the mapping order of CSI fields corresponding to CJT-based CSI reporting

Note that in the case of uplink resource scarcity, or in case the uplink channel quality is low, the resources allocated for UCI may not suffice to carry all CSI fields corresponding to the CJT-based CSI report, and hence the UE may need to omit a portion of the CSI report to adjust to the available uplink resources, e.g., via omitting a subset of CSI Part 2. The conventional CSI omission is defined based on priority level for CSI reports for single TRP transmission, which is defined as Table 5.2.3-1 in TS 38.214 [4], in which CSI Part 2 is omitted level by level, beginning with the lowest priority level until the target data requirement is met, i.e., the maximum number of levels of Part 2 CSI are transmitted given the constraints on the available UL resources and the target rate. When CSI Part 2 fields corresponding to a particular priority level is omitted, the UE shall omit all of the fields corresponding to that priority level. For CJT-based CSI report, the CSI report fields may include CSI for multiple TRPs including a maximum of N=4 cooperative TRPs. The CSI omission scheme may need specific design corresponding to CJT CSI reporting, based on the reported CSI quantities as well as the number of cooperating TRPs. In detail, the CSI omission granularity, such as per TRP omission or joint TRP omission can be further studied for CJT-based CSI reporting. Similarly, whether supporting partial reporting of a CSI quantity in one priority reporting level can be discussed to achieve finer omission granularity. When joint TRP omission is made based on defined bit groups, bit priority for determining CSI bits in group 1 or group 2 of a CJT CSI report would need to be further studied.

Study CSI omission for CJT-based CSI reporting

# CSI enhancement for high/medium UE velocities

In RAN1#110bis-e [2], the following, was agreed for CSI enhancements for high/medium velocities:

Agreement For the Rel-18 Type-II codebook refinement for high/medium velocities, support RI={1,2,3,4}. Agreement On the CSI reporting and measurement for the Rel-18 Type-II codebook refinement for high/medium velocities, when UE-side prediction is assumed, support UE “predicting” channel/CSI after slot l where the location of slot l is configured (from multiple candidate values) by gNB via higher-layer signalling - Candidates of slot l location include the legacy CSI reference resource location (n – nCSI,ref ) and slot (n+δ) where δ ≥ 0 - FFS: Possible value(s) of δ and possible value(s) of WCSI Note: Per legacy behavior, the legacy CSI reference resource, i.e., (n – nCSI,ref ), is reused for locating the last CSI-RS occasion used for a CSI report For a UE that supports UE-side prediction, the support of l = (n – nCSI,ref ) is UE optional. Agreement For the Type-II codebook refinement for high/medium velocities, down-select from the following alternatives: - Alt1. Q different 2-dimensional bitmaps are introduced for indicating the location of the NZCs, where the qth (q=1,…., Q) 2-dimensional bitmap corresponds to qth selected DD basis vector - The number of selected DD basis vectors is denoted as Q - This implies that for each layer, the location of NZCs in SD-FD can be different for different selected DD basis vectors. - Alt2. A DD-basis-common per-layer 2-dimensional bitmap for indicating the location of NZCs used in Rel-16/17 Type-II is used - This implies that for each layer, the location of NZCs in SD-FD is common across all the Q selected DD basis vectors FFS: Further overhead reduction on bitmap(s) FFS: Whether the number of NZCs is upper bounded across all DD basis vectors or per DD basis vector Agreement For the Rel-18 Type-II codebook refinement for high/medium velocities, support the following codebook structure where N4 is gNB-configured via higher-layer signaling: - For N4=1, Doppler-domain basis is the identity (no Doppler-domain compression) reusing the legacy W1, W2, and Wf, e.g. W1W2(Wf)H - For N4>1, Doppler-domain orthogonal DFT basis commonly selected for all SD/FD bases reusing the legacy W1 and Wf, e.g. W1W2Wf⨂WdH - Only Q (denoting the number of selected DD basis vectors) >1 is allowed - TBD (by RAN1#110bis): whether rotation is used or not - FFS: identical or different rotation factors for different SD components - FFS: Whether Q is RRC-configured or reported by the UE Note: Detailed designs for SD/FD bases including the associated UCI parameters follow the legacy specification FFS: Whether one CSI reporting instance includes multiple W2 and a single W1 and Wf report. Conclusion On the usage of CSI reporting and measurement for the Rel-18 Type-II codebook refinement for high/medium velocities, there is no consensus in supporting any specification enhancement for the following assumptions: - Legacy UE procedure for CSI measurement/calculation (equivalent to the combination of l = (n – nCSI,ref ) and WCSI=1) - gNB-side prediction - Note: This doesn’t preclude any gNB implementation Agreement For the Type-II codebook refinement for high/medium velocities, only CSI reporting over PUSCH is supported - Following legacy, support both aperiodic and semi-persistent CSI reporting on PUSCH. Agreement For the Type-II codebook refinement for high/medium velocities, the selection of DD basis vectors is layer-specific - The number of selected DD basis vector (denoted as Q) is layer-common Agreement On the CSI reporting and measurement for the Rel-18 Type-II codebook refinement for high/medium velocities, support the following CSI-RS resource types/structures for CMR: - Time-domain behaviour for NZP CSI-RS resource: periodic (P), semi-persistent (SP), aperiodic (AP) - FFS: Whether to introduce constraints on allowed configuration - Down select from the following: - Alt1. Support K>1 NZP CSI-RS resources, received via a single triggering instance, for aperiodic (AP) -CSI-RS-based channel measurement in a same CSI-RS resource set where the separation between 2 consecutive AP-CSI-RS resources is m slot(s): - Alt2. Support one NZP CSI-RS resource in a CSI-RS resource set, where K>1 occasions are received via a single triggering instance, for aperiodic (AP)-CSI-RS-based channel measurement where the separation between 2 consecutive AP-CSI-RS resources is m slot(s). - For any of the alternatives: - No CRI is reported - FFS: Details, e.g., supported value(s) of K, m, other use cases for the AP-CSI-RS resources (e.g., for training filter coefficients, prediction or performance monitoring) - Support only one NZP CSI-RS resource for P or SP-CSI-RS-based channel measurement Agreement For the Rel-18 Type-II codebook refinement for high/medium velocities, when N4>1, if multiple candidates of Q value are supported, the value of Q is gNB-configured via higher-layer (RRC) signalling Agreement On the CSI reporting and measurement for the Rel-18 Type-II codebook refinement for high/medium velocities, support the following CSI-RS resource types/structures for CMR, support the following: - (Alt1) Support K>1 NZP CSI-RS resources, received via a single triggering instance, for aperiodic (AP) CSI-RS-based channel measurement in a same CSI-RS resource set where the separation between 2 consecutive AP-CSI-RS resources is m slot(s) Agreement For the Rel-18 Type-II codebook refinement for high/medium velocities, when N4>1, down-select from the following alternatives (by RAN1#111) for the orthogonal DFT DD basis: - Alt1. No rotation factor - Alt2. A rotation factor is selected for each SD basis vector - FFS: Supported values of rotation factor Note: At least two companies opine that Alt2 is not aligned either with the agreement in RAN1#110bis-e or WID objective #1 Agreement For the Type-II codebook refinement for high/medium velocities, for N4>1, study the supported values for Q from (but not limited to) the following candidates, in conjunction with the supported values of N4 and DD units: - Alt1. Q is determined as a function of N4, e.g., Q=2 for N4=2, and Q=ceil(N4/2) for N4>2 - Alt2. Q is selected from multiple candidate values, e.g., {2, 3, 4, …,} (or a subset thereof, e.g. {2, 3}), the maximum value is FFS - Alt3. Only single value is supported, e.g. Q=2 only or Q=4 only Agreement On the CSI reporting and measurement for the Rel-18 Type-II codebook refinement for high/medium velocities, when UE-side prediction is assumed, study the supported value(s) for δ and WCSI from (but not limited to) the following candidates, in conjunction with the supported values of N4 and DD units: - δ (slots): {0, 1, 2, 3, 4, 6, 8}, or a subset thereof with at least two values including 0, or a single fixed value (e.g. 0 or 1) - WCSI (slots): 1, N4, following periodicity of P/SP-CSI-RS or SP-CSI (e.g., 4, 5, 8, 10, 16, 20, 40), dN4 (d=DD unit size in slots, N4 is unit-less) FFS: Dependence on sub-carrier spacing should also be studied

In this section, we discuss the outline of CSI reporting enhancements for high-speed UEs and provide our views on the different aspects of the codebook design that are still open for discussion.

## Codebook design

In RAN1#109-e [5], it was agreed to use Rel-16 eType-II regular codebook and Rel-17 FeType-II PS codebook as baseline for Rel-18 potential codebook design, with the possibility to prioritize/down-select from the two codebooks. Since Rel-17 FeType-II PS codebook is primarily designed for scenarios in which partial UL-DL channel reciprocity is assumed, it is preferred to prioritize Rel-16 eType-II codebook as a baseline codebook for potential Rel-18 codebook, since it is not restricted by any UL-DL channel reciprocity condition. Therefore, we support prioritizing Rel-16 eType-II codebook as a baseline for potential Rel-18 codebook design

Only Rel-16 eType-II codebook is selected as a baseline for Rel-18 codebook

Moreover, it was agreed in RAN1#110bis-e to support the following codebook structure for Rel-18 codebook:

- Alt-2A. Doppler-domain basis commonly selected for all SD/FD bases, e.g. W1W2(Wf⨂Wd)H, where Wd is an N4xQ matrix whose columns are selected from a size N4 DFT matrix.

For ease of exposition, our preference is to restructure the codebook to the following format:

W(l)=W1W2(l)Wz(l)H

- Where Wz(l)=Wf(l)⨂Wd(l). Here, the 1D DFT-based transformation matrix Wf(l) is replaced with a 2D DFT-based transformation matrix Wz(l) of size N3N4xMQ corresponding to a joint time/frequency domain transformation, where N4 corresponds to the number of time samples. This approach would simplify the description of the precoder, since the structure of Wz(l) would resemble that of the spatial-domain transformation matrix W1, which is also based on a sub-selection of columns of a 2D DFT matrix.

Delay and Doppler domain basis matrices are reported in the form of a 2D DFT matrix Wz(l), where Wz(l)=Wf(l)⨂Wd(l)

Also, from the codebook structure defined under Alt-2A, the size of W2(l) is 2LxMQ. One alternative is to report W2(l) in the form of Q concatenated matrices W2,1(l), …, W2,Q(l),  as follows:

W2(l)=W2,1(l)…W2,Q(l).

Under this format, the legacy definition and format of the non-zero coefficient matrices and the corresponding bitmaps can be reused. We therefore have the following proposal:

The codebook coefficients are reported in the form of Q non-zero coefficient matrices and Q bitmap matrices W2,q(l),  Bq(l), respectively, where q=1,…,Q

- Furthermore, one important aspect of the Rel-18 codebook design for high speed is the corresponding set of values for N4, Q, WCSI and d, where N4, Q, are described above, and WCSI, d represent the CSI prediction window (in slots) and the duration (in slots) in which each of the N4 codebooks is valid, respectively. Therefore, the three parameters N4, WCSI and d can be tied with the following equation: WCSI=d.N4.

The CSI prediction window is set to be WCSI=d.N4, where d is the duration (in slots) in which each of the N4 codebooks is valid

- Moreover, our preference is that the four parameters N4, Q, WCSI and d are higher-layer configured by the network, since variations in these parameters would significantly impact the CSI feedback overhead, and hence UE selection of these parameters would result in higher variation in CSI feedback overhead, making it harder to match the UCI resources allocated by the network prior to CSI reporting.

The four parameters N4, Q, WCSI and d are network configured

| - UE speed (km/h) | Channel coherence time (slots) |
| --- | --- |
| 3 | 180 |
| 10 | 54 |
| 30 | 18 |
| 60 | 9 |
| 90 | 6 |
| 180 | 3 |

Table 1. Channel coherence time (in slots) at Fc= 4GHz, SCS= 30kHz

In our opinion, the value pairs of N4, Q need to be configured jointly, corresponding to different assumptions of the Doppler behavior of the channel. For instance, Table 1 illustrates a few examples of the channel coherence time as a function of the UE speed for a scenario with 4 GHz carrier frequency and 30 kHz sub-carrier spacing, which showcase the reduction in channel coherence time as the UE speed increases. Given that, a UE moving with higher speed is expected to incur faster degradation of reported CSI resolution, due to the weaker correlation between the channel measured at time slot t with the channel reported at time slot t + t0, when the codebook-based precoder is applied to the PDSCH transmission. As a result, the parameter pair N4, Q needs to be jointly configured with multiple configuration values. An example of a codebook for the values of the parameter pair N4, Q is provided in Table 2.

The values of the parameters N4, Q are jointly configured with a codebook of a value pairs that correspond to different Doppler behavior channel assumptions

| Index | N4 | Q |
| --- | --- | --- |
| 0 | 1 | 1 |
| 1 | 2 | 2 |
| 2 | 4 | 2 |
| 3 | 8 | 2 |
| 4 | 16 | 2 |
| 5 | 16 | 4 |
| 6 | 32 | 2 |
| 7 | 32 | 4 |

Table 2. Codebook of values for parameter pair N4, Q

One other parameter that needs to be designed is the parameter d, corresponding to the duration (in slots) in which each of the N4 precoders is valid. In our opinion, the CSI prediction must be associated with a CSI-RS transmission occasion, and hence the parameter value d should take on a value that is at least equal to the CSI-RS transmission periodicity for periodic and semi-persistent CSI-RS resources, or at least equal to the value m corresponding to the spacing between two consecutive transmissions of aperiodic CSI-RS resources.

The parameter value d is configured to be at least equal to the periodicity of a periodic or semi-persistent CSI-RS resource, or at least equal to the value m corresponding to the spacing (in slots) between two consecutive transmissions of aperiodic CSI-RS resources

## CSI report timing

For CSI reporting under Rel-18 Type-II codebook, it was agreed in RAN1#110bis-e that two candidates of slot location for CSI reporting are to be considered, as follows:

- Alt1. The legacy CSI reference resource location n – nCSI,ref

- Alt2. Slot n+δ, where δ ≥ 0.

Since it was agreed in RAN1#110 [3] that the CSI prediction is UE-based, the prediction window WCSI must be set such that nCSI,ref<WCSI, so that at least one of the N4 precoders corresponds to a slot that succeeds the CSI reporting slot, and hence the UE prediction condition is satisfied.

For Alt1 with CSI reporting based on the legacy CSI reference resource location n – nCSI,ref , the prediction window WCSI is set such that nCSI,ref<WCSI, such that at least one of the N4 precoders corresponds to a slot that succeeds the CSI reporting slot, so as to satisfy the UE prediction condition

For Alt2 with CSI reporting at slot n+δ, the range of values of the parameter δ should be small, since the CSI prediction accuracy would decrease with δ. Our preference is to support the set of values {0,2,4,8} as candidate values for the parameter δ

For Alt2 with CSI reporting at slot n+δ, the set {0,2,4,8} are supported as candidate values for the parameter δ

It was also agreed in RAN1#110bis-e that the support of Alt1 is UE optional. In our opinion, Alt1 incurs significantly less CSI prediction complexity compared with Alt2, and hence should be supported in general as a basic UE feature. Moreover, with Alt1, the UE is able to compute the CSI based on a combination of CMR, IMR and CSI-IM measured at the CSI reference resource slot. On the other hand, Alt2 corresponding to CSI that succeeds the CSI report slot, cannot be based on IMR or CSI-IM, unless the network shares interference information with the UE corresponding to future slots. Hence, Alt1 should be supported as a basic feature

Support Alt1 with CSI reporting based on the legacy CSI reference resource as a basic UE feature

## CSI-RS configuration

In RAN1#110bis-e, it was agreed to support configuring the UE with a CMR based on a single periodic CSI-RS resource, a single semi-persistent CSI-RS resource, or multiple aperiodic CSI-RS resources with a spacing of m slots between two consecutive CSI-RS resource transmissions. Given that the minimum CSI-RS resource periodicity for periodic and semi-persistent NZP CSI-RS is 4 slots, configuring multiple aperiodic CSI-RS resources with a spacing of m=1,2 slots can be beneficial at high UE speeds corresponding to a shorter channel coherence time, as illustrated in Table 1. In our opinion, supporting m values beyond the value 2 is not well motivated. Moreover, the value of m, should be higher-layer configured by the network, and not UE selected. One alternative for the network is to select the value m based on TDCP reporting.

For Rel-18 codebook at high speed, if the UE is configured with a CMR based on multiple aperiodic CSI-RS resources with a spacing of m slots between two consecutive CSI-RS resource transmissions, the values {1,2} are supported for parameter m

## CQI reporting

- For UE-based CSI prediction, both UE and network need to have a common understanding of the correspondence between the CSI quantities and their corresponding time intervals. Given the codebook design agreed in RAN1#110bis-e with the dimensions of matrix Wd being N4xQ, the UE reports Q sets of W2 coefficients, which expand to generate N4 precoders, where N4≥Q. In our opinion, for high-speed CSI reporting, a degradation in channel quality is to some extent inevitable due to the inverse proportionality between the channel coherence time and the UE speed. Given that, conventional CQI reported for the measured channel may be mismatched with the quality of the channel at the time of PDSCH transmission using the codebook-based precoder, i.e., CQI value(s) corresponding to estimated precoder at UE at time t may not match the CQI value(s) corresponding to the estimated precoder during PDSCH transmission at time t + t0. Given that, a few alternatives should be considered for CQI reporting, as follows:

- Alt1: A single CQI value corresponding to the entire CSI report

- Alt2: Two CQI values corresponding to the first and last time slots of the CSI report interval WCSI.

- Alt3: z CQI values corresponding to z selected slots, where z<N4. FFS: whether the z slot indices are network configured or UE selected

- Alt4: N4 CQI values corresponding to the N4 CSI occasions

- Alt1 can be considered as a baseline corresponding to legacy CSI reporting, whereas in Alt2 two CQI values are reported corresponding to the first and last slots of the interval associated with the CSI report, and hence the network can use interpolation to estimate the CQI value corresponding to slots in between. In Alt3, z CQI values are reported corresponding to z time slots that are sub-selected from the N4 units corresponding to CSI reporting occasions. Whether the indices of the z time slot indices are UE selected or network configured can be further discussed. Under Alt4, N4 CQI values are reported corresponding to the N4 CSI occasions. Clearly, the aforementioned alternatives vary in terms of CQI accuracy and reporting overhead. Additionally, more discussion is needed on whether reporting multiple subband CQI values is supported if a subband CQI format is configured. Further details are FFS

On the CSI reporting and measurement for the Rel-18 Type-II codebook refinement for high/medium velocities, the following alternatives are studied for CQI reporting

- Alt1: A single CQI value corresponding to the entire CSI report

- Alt2: Two CQI values corresponding to the first and last time slots of the CSI report interval

- Alt3: z CQI values corresponding to z selected slots, where z<N4.

  - FFS: whether the z slot indices are network configured or UE selected

- Alt4: N4 CQI values corresponding to the N4 CSI occasions

- FFS: Whether/How subband CQI reporting is supported for each alternative

## NZC bitmap design

One important design aspect that needs discussion is the NZC bitmap design for Rel-18 codebook for high speed. Given that Q sets of W2 coefficients are fed back by the UE, two alternatives exist for the codebook design, as follows:

- Alt1. Q bitmaps of size 2LxM each are reported corresponding to the Q sets of W2 coefficients matrices

- Alt2. One bitmap of size 2LxM is reported that is common for all Q sets of W2 coefficients matrices

While the bitmap reporting overhead under Alt2 bitmap design is smaller than that of Alt1 bitmap design, the overall CSI feedback overhead corresponding to Alt1 may be smaller than that of Alt2, since the per W2 matrix bitmap reduces the CSI feedback overhead corresponding to reporting the amplitude and phase values of the non-zero coefficients, whose CSI overhead saving can dominate the additional bitmap overhead.

For bitmap reporting under Rel-18 Type-II codebook for high speed, study the following two alternatives

- Alt1: Q bitmaps corresponding to the Q sets of W2 coefficients matrices

- Alt2: One bitmap that is common for the Q sets of W2 coefficients matrices

# TDCP reporting

- In RAN1#110bis-e [2], the following agreements were made for TDCP reporting

Agreement For the Rel-18 TRS-based TDCP reporting, down select one of the following alternatives by RAN1#110bis-e: - AltA. Based on Doppler profile - E.g., Doppler spread derived from the 2nd moment of Doppler power spectrum, average Doppler shifts, Doppler shift per resource, maximum Doppler shift, relative Doppler shift, etc - AltB. Based on quantized amplitude of time-domain correlation profile - E.g. Correlation within one TRS resource, correlation across multiple TRS resources - Note: The correlation over one or more lags of TRS resource may be considered. The lags may be within one TRS burst or different TRS bursts Note: Different alternatives may or may not apply to different use cases FFS: The need for a measure of confidence level in the TDCP report, and/or UE behaviour when the quality of TDCP measurement is not sufficiently high FFS: TDCP parameter(s) signaled with respect to each alternative Conclusion For the Rel-18 TRS-based TDCP reporting, there is no consensus in supporting periodic, semi-persistent, and event-triggered/UE-initiated TDCP reporting. Agreement For the Rel-18 TRS-based TDCP reporting, the description in the 2nd and 3rd columns of Table 1 in R1-2210523 (“what to report” and “how to calculate”, respectively) will be used as a reference for further evaluation and down selection in RAN1#111, with the following edit (underlined and in red text): - Scheme B column 2: “Amplitude At,τ vs. delay value τ, e.g. Non-zero quantized version of amplitude At,τ for a number of delay values τ (quantized amplitude vs delay) ….”

## PUSCH reporting time-domain behavior

In RAN1#110 [3], it was agreed to support standalone TDCP reporting based on TRS signaling with aperiodic reporting, and furthermore, it was concluded in RAN1#110bis-e that no consensus is available to support periodic, semi-persistent, and event-triggered/UE-initiated TDCP reporting. In our opinion, aperiodic TDCP reporting should follow the same behavior as conventional aperiodic CSI reporting, in terms of being reported only over PUSCH. Aperiodic TDCP reporting over PUCCH should not be supported.

Aperiodic TDCP reporting is supported over PUSCH

## TDCP report fields

In RAN1#110bis-e, it was agreed to down-select from the following options corresponding to the fields reported within a TDCP report:

| TDCP report | What to report | How to calculate: examples, possible UE implementation for evaluation |
| --- | --- | --- |
| A1. Doppler spread | One Doppler spread value, i.e. fd,max-fd,min. (see column 3 of A1) | The normalized channel correlation for each delay τ can be calculated as follows ri,jτ=hiτhj*τhiτhjτ where hi(τ) indicates the time domain estimated channel at delay τ in symbol i within a TRS burst. Further averaging for multiple TRS busts is necessary. The UE can optionally perform interference/noise reduction to get ri,jτ. The Doppler for each delay can be calculated as ri,jτ=J0(2πfd(τ)(j-i)γ) Where γ is the time duration for an OFDM symbol. To calculate fd(τ), the UE can search candidate fd to minimize the error of e(k)(τ)=(i,j)ϵSri,jτ-J0(2πfd(k)(j-i)γ)2 fdτ=argmin(e(k)(τ)) UE does not need to calculate fd(τ) for the each tap, but it only needs to calculate it for top N tops. From multiple fd(τ), the UE can calculate the following: fd,max=max{fdτ1,…,fd(τN)} fd,min=min{fdτ1,…,fd(τN)} |
| A2. Relative Doppler shift per resource | With N>=1 TRS resources: Doppler shift per resource (e.g. differential or absolute) | - Doppler shift fd is derived based on the following equation: fd = angle(r)/(2*pi*t) - where r is the channel correlation measured from different TRS symbols and t is the time domain interval for the channel correlation. - For differential manner, the differential value (e.g., relative Doppler shift) is: fd - fd_reference |
| A3. Single Doppler shift | One Doppler shift value | - The average Doppler shift across multiple delay-paths/peaks in measured CIR - UE calculates and selects the first M peaks/delay-paths according to CIR (Channel Impulse Response)/ PDP (Power Delay profile) - UE calculates Doppler shifts fd,0…. fd,M-1according to M peaks/delay-paths respectively - UE calculates and reports average Doppler shift by power weighted, i.e. fd,avg=r=0M-1fd,r*Prr=0M-1Pr - Maximum Doppler shift across multiple delay-paths/peaks in measured CIR - UE calculates and selects the first M peaks/delay-paths according to CIRn/ PDPn - UE calculates Doppler shifts fd,0…. fd,M-1according to M peaks/delay-paths respectively - UE reports Maximum Doppler shifts fd,max among the M peaks/delay-paths |
| A4. Relative Doppler shift per CIR peak | With M identified peaks in measured CIR: (1) N Doppler shifts; (2) M values of delay shift in CIR | - UE-side: - UE calculate and select the first M peaks/delay-paths according to CIRn/ PDP - UE calculate im Doppler shifts fd,0, …, fd, im-1 according to the m-th peak/delay-path respectively - UE reports N=i0+…+iM Doppler shift FFS: M/N is pre-defined by the specification or configured by gNB - gNB-side: - gNB matches fd,0 to the strongest path measured by SRS - gNB matches N Doppler shifts to the M paths measured by SRS - gNB matches M-1 paths measured by SRS according to (M-1) differential Delay shifts to the strongest path or (M-1) Delay shifts reported by UE |
| A5. Doppler spread estimated from peak Doppler frequency | DFT index corresponding to the peak Doppler frequency: argmaxC(f) where Cf=DFT(c(τ)) and c(τ) is the TRS time-correlation function cτ=1Nn=1Nhnthnt+τ*, where hn(t) is the TRS measurement on subcarrier n at time t |  |
| B. Time-domain correlation profile | Amplitude At,τ vs. delay value , e.g., non-zero quantized version of amplitude At,τ for a number of delay values τ (quantized amplitude vs delay) Example equation At,τ=ct,τct,0 where ct,τ=n=0N-1hnt+τ∙hn*t and hn is the channel for subcarrier n. | - Normalized auto-correlation of a time series measured from a TRS resource. - Multiple auto-correlation values can be calculated from different lags of the same resource or different resources - The autocorrelation can be estimated by replacing the channel hn for subcarrier n in the defining formula in column 2, with the matched filter subcarrier components Xn=Rn∙Sn* of the received signal Rn where Sn* is the complex conjugate of the known transmitted TRS signal. For ct,0 one can use the arithmetic average over the two TRS symbols separated by the time τ , i.e. At,τ≈n=0N-1Xnt+τ∙Xn*t12∙n=0N-1Xnt∙Xn*t+Xnt+τ∙Xn*t+τ Or, alternatively, one may use the geometric average for ct,0, i.e. At,τ≈n=0N-1Xnt+τ∙Xn*tnXnt+τ2nXnt2 Further methods to remove noise bias and to suppress noise can be used. |

In the following, we provide our views on each of the alternatives above, as follows.

- Alt-A: In general, one issue with all Alt-A approaches based on Doppler shift reporting is whether/how the Doppler shift can be differentiated from CFO, since they both cause a frequency shift with the same order of values. Even if the process of inferring the Doppler shift from the aggregate frequency shift can be handled in a spec-transparent manner based on UE implementation, it is important that the proponents of Alt-A explain the process so we can assess the feasibility and efficiency of Alt-A before. In the sequel, we provide further views on each of the five variants of Alt-A, as follows

- Alt-A1: In our opinion, Alt-A1 is a variant of Alt-B, since the channel autocorrelation is computed as an intermediate step before computing the Doppler spread. Moreover, Alt-A1 would require significant specification impact, and hence is not preferred.

- Alt-A2: Reporting the Doppler shift per resource is unclear, since the Doppler shift is associated with the channel path, i.e., each channel path is associated with a distinct Doppler shift value.

- Alt-A3: Reporting an average Doppler shift is also unclear. For instance, for a multi-path channel with rich scattering, the Doppler shift values corresponding to the strongest M taps may have different signs, and hence can be averaged out, e.g., a UE with speed v and M=2 paths with equal power and angles of incidence that are 180° would have Doppler shift valued of -fd,1, fd,2, such that fd,avg = 0. Similarly, Alt-A3 corresponding to reporting the maximum Doppler shift is also questionable, since the power of the path associated with the maximum Doppler shift may be less than power of the remainder of the paths, especially at richly scattered environments.

- Alt-A4: This alternative resembles Alt-A1, with the difference stemming from the relative Doppler shift in Alt-A4 being matched with the SRS differential delay shifts, rather than matching with Bessel’s function of the first type. One issue with Alt-A4 is that it presumes the power-delay profile (PDP) computed based on the SRS symbols is similar to the PDP corresponding to the TRS symbols. If this assumption holds in reality, the TDCP can be inferred from SRS signaling without the need of TDCP reporting.

- Alt-A5: This alternative is not clear, for instance, no details on how the mapping between DFT indices and the Doppler shift is provided. Moreover, Alt-A5 is also based on the channel autocorrelation which needs to be computed as an intermediate step, similar to Alt-A1.

- Alt-B: One advantage of Alt-B is that it is the most straightforward approach, since the channel autocorrelation needs to be computed as an intermediate step for most other Alt-A variants. Two autocorrelation functions are proposed for Alt-B, based on either arithmetic or geometric scaling of the channel correlation. One example of TDCP reporting under Alt-B is via introducing a codebook of quantized amplitude values corresponding to the autocorrelation function At,τ, however, for a meaningful mapping between the quantized amplitude values and  the autocorrelation value, the autocorrelation function needs to be specified. For a small codebook of values, e.g., with two values corresponding to either a large or small autocorrelation value, specifying the channel autocorrelation function is not needed. Note that instead of reporting the quantized correlation amplitude for a fixed lag, alternatively the lag is reported (in terms of a symbol index or TRS occasion index) with respect to a fixed correlation amplitude. The fixed correlation amplitudes can be configured from a small set of values, e.g., two values corresponding to strong, weak correlation, so that the process is less dependent on the underlying autocorrelation function as much as possible. In light of that, we support Alt-B, with the TDCP report indicating a correlation lag value corresponding to a large channel autocorrelation value, i.e., channel coherence time, in terms of  an index of a TRS symbol at high speed, or an index of a TRS transmission occasion over periodic TRS transmissions at low/moderate speed. Hence, TDCP reporting can include indices of TRS symbols/TRS occasions with respect to a reference TRS/TRS symbol corresponding to a period of strong correlation, rather than reporting exact time-domain correlation values and/or Doppler-profile parameters, e.g., Doppler spread. Given that reporting TRS/TRS symbol indices corresponds to time-domain reporting, we prefer Alt-B over Alt-A

Alt-B is preferred over Alt-A, since the channel correlation in time can be measured in terms of TRS/TRS symbol indices corresponding to a period of strong channel correlation with respect to a reference TRS/TRS symbol, compared with reporting absolute Doppler-domain parameters for all TRSs/TRS symbols

- In light of that, our  preference is to support Alt-B for TDCP reporting of channel correlation in terms of indices of TRS/TRS symbols with respect to a reference TRS/TRS symbol

For TDCP reporting format, support Alt-B with channel correlation reported in terms of TRS/TRS symbol indices with respect to a reference TRS/TRS symbol corresponding to a channel coherence interval

# Conclusion

This contribution addressed CSI enhancements for NR Rel. 18, including enhancements for high mobility scenarios, TDCP reporting enhancements, as well as CSI enhancements for CJT.

- For CSI enhancements for CJT, we have the following proposals:

- For the Rel-18 Type-II codebook for CJT mTRP, the maximum number of CSI-RS ports across the N NZP CSI-RS resources is set to X CSI-RS ports

- FFS: whether X=32, 48, 64

Further study whether/how the number of non-zero coefficients is reported in the CSI report, e.g., separately for each TRP or jointly for all TRPs

The same N1, N2 and O1, O2 values are configured for each TRP

Support Alt3 for parameter L of SD basis selection with L1=…=Ln=L

Support separate combined parameter configuration L, βn, pυ,n per TRP for Mode 1 codebook structure and one combined parameter configuration L, β, pυ across TRPs for Mode 2 codebook structure

Support smaller β value, e.g., 1/8 for CSI overhead reduction at N=3,4

Confirm the work assumption Alt3 for W2 quantization group and strongest coefficient indicator design

Reuse Rel-16 eType-II reference amplitude quantization for differential quantization of the strongest coefficients for each polarization per TRP

- FFS: whether the number of reference amplitude quantization bits can be reduced

For CJT with N TRPs, support two coefficient quantization schemes with different resolution levels, with down selection from the following alternatives:

- Alt1. TRP-common quantization resolution: If N =2, all TRPs are associated with the higher resolution quantization scheme, otherwise if N >2, all TRPs are associated with the lower resolution quantization scheme

- Alt2. TRP-specific quantization resolution: The strongest 2 TRPs are associated with the higher resolution quantization scheme, whereas if N >2, the remainder N–2 TRPs are associated with the lower resolution quantization scheme

- Alt3. Polarization-specific quantization resolution: The stronger polarization of each of the N TRPs per layer is associated with the higher resolution quantization scheme, whereas the remainder of coefficients are associated with the lower resolution quantization scheme

- For CJT codebook design, support Alt2 for size of bitmap based on non-rectangular bitmaps

- For Alt2 bitmap design, a first stage bitmap for each CSI-RS resource indicates the indices of rows and/or columns with at least one non-zero coefficient, and a second stage bitmap whose size is based on the indicated rows and/or columns in the first stage bitmap per CSI-RS resource

- At least two non-zero coefficients corresponding to different polarizations are identified in each bitmap Bn

- Study the mapping order of CSI fields corresponding to CJT-based CSI reporting

- Study CSI omission for CJT-based CSI reporting

For CSI enhancements for high mobility, we have the following proposals:

- Only Rel-16 eType-II codebook is selected as a baseline for Rel-18 codebook

- Delay and Doppler domain basis matrices are reported in the form of a 2D DFT matrix Wz(l), where Wz(l)=Wf(l)⨂Wd(l)

- The codebook coefficients are reported in the form of Q non-zero coefficient matrices and Q bitmap matrices W2,q(l),  Bq(l), respectively, where q=1,…,Q

- The CSI prediction window is set to be WCSI=d.N4, where d is the duration (in slots) in which each of the N4 codebooks is valid

- The four parameters N4, Q, WCSI and d are network configured

- The values of the parameters N4, Q are jointly configured with a codebook of a value pairs that correspond to different Doppler behavior channel assumptions

- The parameter value d is configured to be at least equal to the periodicity of a periodic or semi-persistent CSI-RS resource, or at least equal to the value m corresponding to the spacing (in slots) between two consecutive transmissions of aperiodic CSI-RS resources

- For Alt1 with CSI reporting based on the legacy CSI reference resource location n – nCSI,ref , the prediction window WCSI is set such that nCSI,ref<WCSI, such that at least one of the N4 precoders corresponds to a slot that succeeds the CSI reporting slot, so as to satisfy the UE prediction condition

- For Alt2 with CSI reporting at slot n+δ, the set {0,2,4,8} are supported as candidate values for the parameter δ

- Support Alt1 with CSI reporting based on the legacy CSI reference resource as a basic UE feature

- For Rel-18 codebook at high speed, if the UE is configured with a CMR based on multiple aperiodic CSI-RS resources with a spacing of m slots between two consecutive CSI-RS resource transmissions, the values {1,2} are supported for parameter m

- On the CSI reporting and measurement for the Rel-18 Type-II codebook refinement for high/medium velocities, the following alternatives are studied for CQI reporting

- Alt1: A single CQI value corresponding to the entire CSI report

- Alt2: Two CQI values corresponding to the first and last time slots of the CSI report interval

- Alt3: z CQI values corresponding to z selected slots, where z<N4.

  - FFS: whether the z slot indices are network configured or UE selected

- Alt4: N4 CQI values corresponding to the N4 CSI occasions

- FFS: Whether/How subband CQI reporting is supported for each alternative

- For bitmap reporting under Rel-18 Type-II codebook for high speed, study the following two alternatives

- Alt1: Q bitmaps corresponding to the Q sets of W2 coefficients matrices

- Alt2: One bitmap that is common for the Q sets of W2 coefficients matrices

- For TDCP reporting enhancements, we have the following proposals:

- Aperiodic TDCP reporting is supported over PUSCH

- Alt-B is preferred over Alt-A, since the channel correlation in time can be measured in terms of TRS/TRS symbol indices corresponding to a period of strong channel correlation with respect to a reference TRS/TRS symbol, compared with reporting absolute Doppler-domain parameters for all TRSs/TRS symbols

- For TDCP reporting format, support Alt-B with channel correlation reported in terms of TRS/TRS symbol indices with respect to a reference TRS/TRS symbol corresponding to a channel coherence interval

# References

| [1] | RP-213598, Samsung, "New WID: MIMO Evlution for Downlink and Uplink," Online, Dec. 6-17, 2021. |
| --- | --- |
| [2] | 3GPP TSG RAN WG1 #110bis-e, "Draft Meeting Report," Online, Oct. 10-19, 2022. |
| [3] | 3GPP TSG RAN WG1 #110, "Draft Meeting Report," Toulouse, France, Aug. 22-26, 2022. |
| [4] | 3GPP TS 38.214, "Physical layer procedures for data," Mar. 2020. |
| [5] | 3GPP TSG RAN WG1 #109-e, "Draft Meeting Report," Online, May 9-20, 2022. |