3GPP TSG RAN WG1 #111                                                       	R1-2211404

Toulouse, France, November 14th – 18th, 2022

Agenda Item:	9.5.1.1

Source: 	Intel Corporation

Title:	Evaluation of SL positioning

Document for:        Discussion/Decision

- Introduction

Agreement

For evaluations of relative positioning, the horizontal plane is assumed parallel to the ground.

Woking assumption

For evaluation of V2X use-cases for SL positioning, the following accuracy requirements are considered:

- Set A (similar to “Set 2” defined in TR 38.845)

  - Horizontal accuracy of 1.5 m (absolute and relative); Vertical accuracy of 3 m (absolute and relative) for 90% of UEs

- Set B (similar to “Set 3” defined in TR 38.845)

  - Horizontal accuracy of 0.5 m (absolute and relative); Vertical accuracy of 2 m (absolute and relative) for 90% of UEs

- Note 1: For evaluated SL positioning methods, companies are expected to report:

  - (1) whether each of the two requirements are satisfied, and

  - (2) %-ile of UEs satisfying the target positioning accuracy for a requirement that may not be satisfied with 90%.

- Note 2: target positioning requirements may not necessarily be reached for all scenarios and deployments

- Note 3: all positioning techniques may not achieve all positioning requirements in all scenarios

Agreement

- The following performance metrics for SL positioning accuracy evaluation is defined:

  - For relative and absolute positioning

    - horizontal accuracy

    - vertical accuracy

  - For ranging

    - Ranging for distance, i.e. accuracy of distance

    - Ranging for angle, i.e. accuracy of angle

- Companies are required to output

  - The percentiles of positioning accuracy error including 50%, 67%, 80%, 90% of UEs,

    - FFS others

  - And the CDF of positioning accuracy error

- Performance metrics other than positioning accuracy, such as PHY/end-to-end latency, are up to companies

Agreement

- For absolute positioning evaluation, anchor UEs’ locations are known

  - In the evaluation of SL only positioning

    - Anchor UEs are used to locate target UEs

  - In the evaluation of Joint Uu/SL positioning

    - Both BS and anchor UEs are used to locate target UEs

- In the evaluation, relative positioning or ranging is performed between two UEs within X m

  - FFS X which can be different for different scenarios, e.g. highway, urban grid, etc.

  - Companies can consider to provide simulation results based on multiple X values

Agreement

For evaluations of relative positioning, the horizontal plane is assumed parallel to the ground.

Woking assumption

For evaluation of V2X use-cases for SL positioning, the following accuracy requirements are considered:

- Set A (similar to “Set 2” defined in TR 38.845)

  - Horizontal accuracy of 1.5 m (absolute and relative); Vertical accuracy of 3 m (absolute and relative) for 90% of UEs

- Set B (similar to “Set 3” defined in TR 38.845)

  - Horizontal accuracy of 0.5 m (absolute and relative); Vertical accuracy of 2 m (absolute and relative) for 90% of UEs

- Note 1: For evaluated SL positioning methods, companies are expected to report:

  - (1) whether each of the two requirements are satisfied, and

  - (2) %-ile of UEs satisfying the target positioning accuracy for a requirement that may not be satisfied with 90%.

- Note 2: target positioning requirements may not necessarily be reached for all scenarios and deployments

- Note 3: all positioning techniques may not achieve all positioning requirements in all scenarios

Agreement

- The following performance metrics for SL positioning accuracy evaluation is defined:

  - For relative and absolute positioning

    - horizontal accuracy

    - vertical accuracy

  - For ranging

    - Ranging for distance, i.e. accuracy of distance

    - Ranging for angle, i.e. accuracy of angle

- Companies are required to output

  - The percentiles of positioning accuracy error including 50%, 67%, 80%, 90% of UEs,

    - FFS others

  - And the CDF of positioning accuracy error

- Performance metrics other than positioning accuracy, such as PHY/end-to-end latency, are up to companies

Agreement

- For absolute positioning evaluation, anchor UEs’ locations are known

  - In the evaluation of SL only positioning

    - Anchor UEs are used to locate target UEs

  - In the evaluation of Joint Uu/SL positioning

    - Both BS and anchor UEs are used to locate target UEs

- In the evaluation, relative positioning or ranging is performed between two UEs within X m

  - FFS X which can be different for different scenarios, e.g. highway, urban grid, etc.

  - Companies can consider to provide simulation results based on multiple X values

In RAN1 #109-e, the following agreements were made on scenarios/requirements and SL positioning methodologies [1].

In this paper, we provide evaluation results for SL positioning based on the evaluation methodologies agreed in RAN1 #109-e. This is an updated version of [3] with additional results for IIoT and commercial scenarios.

- Evaluation results for V2X use cases

  - Highway evaluation

Highway simulations in this section reuse the road configuration, antenna configuration and channel model from TR 37.885[2] as agreed in RAN1 #110.  Road length is 2000m and has 3 lanes for each direction. UE-type RSUs are located with 200m spacing on both side of the road. General simulation assumptions are provided in Appendix.

    - Ranging

Ranging accuracy is expressed as the difference (error) between the calculated distance and the actual distance in relation to another node. In this section, we provide evaluation results for ranging accuracy in the V2X highway scenario based on RTT. Table 1 and Figure 1 show the results for different bandwidth and X (minimum distance between two UEs engaged in ranging) values.

Figure 1. CDF of Ranging error for V2X highway scenario

Table 1. Ranging error (distance in meters) for V2X highway scenario

| Case | 50% | 67% | 80% | 90% | Whether meet the requirement of Set A | Whether meet the requirement of Set B |
| --- | --- | --- | --- | --- | --- | --- |
| Freeway, No RSU BW=40MHz, X=50m | 0.5674 | 0.8595 | 1.2340 | 1.6772 | No (85.3%) | No (45.9%) |
| Freeway, No RSU BW=40MHz, X=100m | 0.5627 | 0.8754 | 1.2657 | 1.7267 | No (84.3%) | No (45.1%) |
| Freeway, No RSU BW=100MHz, X=50m | 0.3268 | 0.5167 | 0.6955 | 0.9988 | Yes | No (66.8%) |
| Freeway, No RSU BW=100MHz, X=100m | 0.3246 | 0.5073 | 0.7125 | 1.0147 | Yes | No (67.5%) |

Observation 1: In V2X Highway scenario, as X decreases from 100m to 50m, ranging accuracy for distance improves by 3% and 1.6% for 40MHz and 100MHz bandwidth respectively.

Observation 2: For ranging accuracy for SL positioning for V2X highway scenario, for the evaluated positioning methods,

- For 100MHz bandwidth, Set A requirements can be met for both X=50m and 100m while 66.8%-tile and 67.5%-tile UEs satisfies Set B requirement for X=50m and X=100m, respectively.

- For 40MHz bandwidth, 85.3%-ile of UE satisfies Set A target positioning accuracy and 45.9%-ile of UE satisfies Set B target positioning accuracy with X=50m.

    - Absolute positioning

Absolute positioning accuracy is expressed as the difference (error) between the calculated horizontal position and the actual horizontal position. In this section, we provide evaluation results for absolute accuracy in the V2X highway scenario based on DL TDOA. Figure 2 and Table 2 show the horizontal absolute error with different bandwidth and RSU deployment.

Figure 2. CDF of Absolute error for V2X highway scenario

Table 2. Absolute positioning error (meters) for V2X highway scenario

| Case |  | 50% | 67% | 80% | 90% | Whether meet the requirement of Set A | Whether meet the requirement of Set B |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Freeway, RSU (symmetric) BW=40MHz | Horizontal | 1.7843 | 2.5839 | 3.7326 | 5.0870 | No (42.2%) | No (9.2%) |
|  | Direction along the road (X) | 0.3208 | 0.4736 | 0.6003 | 0.8393 | Yes |  |
|  | Direction perp. to the road (Y) | 1.7593 | 2.5483 | 3.6461 | 5.0858 |  |  |
| Freeway, RSU (staggered) BW=40MHz | Horizontal | 1.7848 | 2.5153 | 3.6933 | 4.9242 | No (43.0%) | No (9.2%) |
|  | Direction along the road (X) | 0.3245 | 0.4751 | 0.6018 | 0.8219 |  |  |
|  | Direction perp. to the road (Y) | 1.7629 | 2.5035 | 3.5550 | 4.9230 |  |  |
| Freeway, RSU (symmetric) BW=100MHz | Horizontal | 0.8877 | 1.3001 | 1.7296 | 2.3902 | No (71.3%) | No (30.4%) |
|  | Direction along the road (X) | 0.1632 | 0.2310 | 0.3062 | 0.4348 |  |  |
|  | Direction perp. to the road (Y) | 0.8575 | 1.2905 | 1.7110 | 2.3579 |  |  |
| Freeway, RSU (Staggered) BW=100MHz | Horizontal | 0.8877 | 1.2954 | 1.6748 | 2.2716 | No (72.2%) | No (31.3%) |
|  | Direction along the road (X) | 0.1633 | 0.2281 | 0.3053 | 0.4348 |  |  |
|  | Direction perp. to the road (Y) | 0.8575 | 1.2847 | 1.6426 | 2.2670 |  |  |

Observation 3: In V2X Highway scenario, absolute positioning accuracy can be improved by using staggered UE-type RSU deployment.

Observation 4: In V2X Highway scenario, due to the nature of the deployment, the error along the y-axis contributes significantly more to the horizontal error than the error along the x-axis.

Observation 5: For absolute positioning accuracy for horizontal errors for SL positioning for V2X highway scenario, for the evaluated positioning methods,

- Set A and Set B requirements for V2X highway scenario are not satisfied for horizontal error. However,

  - Error along the direction of the road can satisfy the 1m bound as in Set A for both 40 MHz and 100 MHz bandwidth choices.

  - Error along the direction of the road can satisfy the 0.5 m bound as in Set B for 100 MHz bandwidth.

  - Urban grid evaluation

Urban grid simulation in this section reuses the road configuration, antenna configuration and channel model specified for urban grid scenario in TR 37.885 [2] as agreed in RAN1 #110 meeting.  UE-type RSUs are located at the intersections. General simulation assumptions are provided in Appendix.

    - Ranging

In this section, we provide evaluation results for ranging accuracy (distance error) for the V2X urban grid scenario based on RTT.  Figure 3 and Table 3 show the accuracy results for different X values and bandwidth.

Figure 3. CDF of ranging error for V2X Urban grid scenario

Table 3. Ranging error (distance in meters) for V2X urban grid scenario

| Case | 50% | 67% | 80% | 90% | Whether meet the requirement of Set A | Whether meet the requirement of Set B |
| --- | --- | --- | --- | --- | --- | --- |
| Urban Grid, No RSU BW=40MHz, X=50m | 0.8530 | 1.3433 | 1.9306 | 3.5304 | No (72.4%) | No (34.2%) |
| Urban Grid , No RSU BW=40MHz, X=100m | 0.9537 | 1.5218 | 2.3618 | 4.1848 | No (66.1%) | No (30.4%) |
| Urban Grid , No RSU BW=100MHz, X=50m | 0.4730 | 0.7526 | 1.1483 | 2.0263 | No (83.8%) | No (52.5%) |
| Urban Grid , No RSU BW=100MHz, X=100m | 0.5205 | 0.8853 | 1.5914 | 3.0596 | No (78.6%) | No (49.0%) |

Observation 6: In V2X urban grid scenario, as X decreases from 100m to 50m, ranging accuracy for distance improves by 5.8% and 9% for 40MHz and 100MHz bandwidth respectively.

Observation 7: For ranging distance in V2X urban grid scenario, for the evaluated positioning methods,

- Set A and Set B requirements for ranging accuracy cannot be met for both bandwidth 40MHz and 100MHz.

  - For 40MHz, 72.4%-ile of UE satisfies Set A target and 34.2%-ile of UE satisfies Set B target with X=50m.

  - For 100MHz, 83.8%-ile of UE satisfies Set A target and 52.5%-ile of UE satisfies Set B target with X=50m.

    - Absolute positioning

In this section, we provide evaluation results for absolute positioning in the V2X urban grid scenario. Table 4 and Figure 4 show the results with different bandwidths.

Figure 4. CDF of absolute error for V2X urban grid scenario

Table 4. Absolute error (meters) for V2X urban grid scenario

| Case |  | 50% | 67% | 80% | 90% | Whether meet the requirement of Set A | Whether meet the requirement of Set B |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Urban Grid, RSU BW=40MHz | Horizontal | 2.8165 | 4.0782 | 5.9472 | 8.9635 | No (22.2%) | No (2.7%) |
| Urban Grid, RSU BW=100MHz | Horizontal | 2.0612 | 3.5916 | 5.1706 | 7.9541 | No (37.2%) | No (10.2%) |

Observation 8: For absolute positioning considering horizontal errors in V2X urban grid scenario, for the evaluated positioning methods,

- Set A and Set B requirements for absolute accuracy cannot be met for both 40MHz and 100MHz bandwidth choices.

  - For 40MHz, 22.2%-ile of UE satisfies Set A target and 2.7%-ile of UE satisfies Set B target.

  - For 100MHz, 37.2%-ile of UE satisfies Set A and 10.2%-ile of UE satisfies Set B target.

- Evaluation results for IIOT use case

IIoT simulation in this section reused deployment configuration, antenna configuration and channel model specified for InF-SH scenario in TR 38.857 as agreed in RAN1 #109 meeting. General simulation assumptions are provided in Appendix.

    - Ranging

In this section, we provide evaluation results for ranging accuracy (distance error) for the InF-SH scenario based on RTT.  Table 5 shows the accuracy results for different X values for 100MHz bandwidth.

Table 5. Ranging error (distance in meters) for IIoT InF-SH scenario

| Case | 50% | 67% | 80% | 90% | Whether meet the requirement of Set A | Whether meet the requirement of Set B |
| --- | --- | --- | --- | --- | --- | --- |
| InF-SH BW=100MHz, X=10m | 0.4288 | 0.7624 | 1.0009 | 1.5956 | No (79.81%) | No (30.29%) |
| InF-SH BW=100MHz, X=25m | 0.4353 | 0.7544 | 1.0054 | 1.7455 | No (79.54%) | No (28.12%) |
| InF-SH BW=100MHz, X=50m | 0.4436 | 0.7380 | 1.0534 | 2.0541 | No (78.35%) | No (27.72%) |

Observation 9: For ranging distance in IIoT (InF-SH) scenario, for the evaluated positioning methods,

- Set A and Set B requirements for ranging accuracy cannot be met with bandwidth of 100MHz for the evaluated methods.

  - For 100MHz, 79.8%-ile of UE satisfies Set A target and 30.3%-ile of UE satisfies Set B target with X=10m.

- Evaluation results for Commercial use case

Commercial simulation in this section used deployment configuration, antenna configuration and channel model specified for UMi scenario (200m ISD, 19-site hexagonal grid), according to the simulation assumptions of TS 38.855 and TS 36.843 A.2.1.2 as agreed in RAN1 #109 meeting. General simulation assumptions are provided in Appendix.

    - Ranging

In this section, we provide evaluation results for ranging accuracy (distance error) for the commercial scenario based on RTT.  Table 6 shows the accuracy results for different X values for 100MHz bandwidth.

Table 6. Ranging error (distance in meters) for Commercial scenario

| Case | 50% | 67% | 80% | 90% | Whether meet the requirement of Set A | Whether meet the requirement of Set B |
| --- | --- | --- | --- | --- | --- | --- |
| BW=100MHz, X=25m | 0.3267 | 0.4025 | 0.4605 | 0.4754 | Yes | No (35.71%) |
| BW=100MHz, X=50m | 0.3267 | 0.4840 | 0.5902 | 1.0752 | No (85.71%) | No (29.80%) |
| BW=100MHz, X=100m | 0.3185 | 0.5394 | 0.9733 | 1.1976 | No (80.65%) | No (22.18%) |

Observation 10: For ranging distance in commercial scenario, for the evaluated positioning methods,

- Set A requirements for ranging accuracy can be met for bandwidth 100MHz and X=25m.

- Both Set A and Set B requirements for ranging accuracy cannot be met for bandwidth 100MHz for X=50m and X=100m.

  - For 100MHz, 85.7%-ile of UE satisfies Set A target and 29.8%-ile of UE satisfies Set B target with X=50m.

# 3. Conclusions

In this contribution, we provide evaluation results for SL positioning. Based on the results, the following observations are made.

Observation 1: In V2X Highway scenario, as X decreases from 100m to 50m, ranging accuracy for distance improves by 3% and 1.6% for 40MHz and 100MHz bandwidth respectively.

Observation 2: For ranging accuracy for SL positioning for V2X highway scenario, for the evaluated positioning methods,

- For 100MHz bandwidth, Set A requirements can be met for both X=50m and 100m while 66.8%-tile and 67.5%-tile UEs satisfies Set B requirement for X=50m and X=100m, respectively.

- For 40MHz bandwidth, 85.3%-ile of UE satisfies Set A target positioning accuracy and 45.9%-ile of UE satisfies Set B target positioning accuracy with X=50m.

Observation 3: In V2X Highway scenario, absolute positioning accuracy can be improved by using staggered UE-type RSU deployment.

Observation 4: In V2X Highway scenario, due to the nature of the deployment, the error along the y-axis contributes significantly more to the horizontal error than the error along the x-axis.

Observation 5: For absolute positioning accuracy for horizontal errors for SL positioning for V2X highway scenario, for the evaluated positioning methods,

- Set A and Set B requirements for V2X highway scenario are not satisfied for horizontal error. However,

  - Error along the direction of the road can satisfy the 1m bound as in Set A for both 40 MHz and 100 MHz bandwidth choices.

  - Error along the direction of the road can satisfy the 0.5 m bound as in Set B for 100 MHz bandwidth.

Observation 6: In V2X urban grid scenario, as X decreases from 100m to 50m, ranging accuracy for distance improves by 5.8% and 9% for 40MHz and 100MHz bandwidth respectively.

Observation 7: For ranging distance in V2X urban grid scenario, for the evaluated positioning methods,

- Set A and Set B requirements for ranging accuracy cannot be met for both bandwidth 40MHz and 100MHz.

  - For 40MHz, 72.4%-ile of UE satisfies Set A target and 34.2%-ile of UE satisfies Set B target with X=50m.

  - For 100MHz, 83.8%-ile of UE satisfies Set A target and 52.5%-ile of UE satisfies Set B target with X=50m.

Observation 8: For absolute positioning considering horizontal errors in V2X urban grid scenario, for the evaluated positioning methods,

- Set A and Set B requirements for absolute accuracy cannot be met for both 40MHz and 100MHz bandwidth choices.

  - For 40MHz, 22.2%-ile of UE satisfies Set A target and 2.7%-ile of UE satisfies Set B target.

  - For 100MHz, 37.2%-ile of UE satisfies Set A and 10.2%-ile of UE satisfies Set B target.

Observation 9: For ranging distance in IIoT (InF-SH) scenario, for the evaluated positioning methods,

- Set A and Set B requirements for ranging accuracy cannot be met for both bandwidth 100MHz.

  - For 100MHz, 79.8%-ile of UE satisfies Set A target and 30.3%-ile of UE satisfies Set B target with X=10m.

Observation 10: For ranging distance in commercial scenario, for the evaluated positioning methods,

- Set A requirements for ranging accuracy can be met for bandwidth 100MHz and X=25m.

- Both Set A and Set B requirements for ranging accuracy cannot be met for bandwidth 100MHz for X=50m and X=100m.

  - For 100MHz, 85.7%-ile of UE satisfies Set A target and 29.8%-ile of UE satisfies Set B target with X=50m.

# References

- Chairman’s Notes, 3GPP RAN1 #109-e.

- 3GPP TR 37.885. “Study on evaluation methodology of new Vehicle-to-Everything (V2X) use cases for LTE and NR”

- R1- 2210038, “Evaluation of SL positioning,” Intel Corporation, RAN1 #110bis-e.

# Appendix

# Simulation assumptions

Table 5. Evaluation scenarios and parameters template

| Parameter | Case XYZ (channel model, FRx) |
| --- | --- |
| Scenario (baseline, otherwise state any modifications) | Baseline |
| Carrier frequency | 6GHz |
| Subcarrier spacing | 30KHz |
| Reference Signal Transmission Bandwidth | 100 MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | DL PRS: Comb-2 |
| Reference signal (type of sequence, number of ports, …) | DL PRS: single port |
| Number of sites | 1 |
| Number of symbols used per occasion | DL PRS: 2 symbols |
| number of occasions used per positioning estimate | 1 |
| Power-boosting level | DL PRS: 3 dB |
| Uplink power control (applied/not applied) | N/A |
| interference modelling (ideal muting, or other) | Ideal muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | MUSIC |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | Taylor series |
| Network synchronization assumptions | Ideal synchronization |
| UE/gNB RX and TX timing error | No timing calibration error |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | No Tx/Rx beam sweeping |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Single Tx port |
| UE antenna configuration | (1, 2, 2, 1, 1) |
| Number of UE branches | 4 |
| Description of enhancement solutions, if any | N/A |
| gNB antenna configuration | N/A |
| UE noise figure | 9 dB |
| UE antenna height | VUE: 1.6m, RSU: 5m |
| gNB antenna height | N/A |