3GPP TSG RAN WG1 #111                                                                      R1-2211802

Toulouse, France, November 14th – 18th, 2022

Agenda Item:	9.1.4.1

Source:	Apple Inc.

Title:	Views on UL precoding indication for multi-panel simultaneous transmissions

Document for:	Discussion/Decision

# Introduction

In Rel-16, multi-TRP based simultaneous PDSCH transmissions using a single DCI was specified for both FDM and SDM PDSCH transmissions. While Rel-17 supports multi-TRP based PUSCH, where different repetitions in time may be associated with different beams, simultaneous PUSCH transmission across different panels is one of the objectives to be studied in R18. In previous RAN1 meetings, some progress toward simultaneous uplink transmission over multi-panel (STxMP) was made. Most importantly, in RAN1#110 it was agreed under a working assumption that in Rel-18, SDM scheme is supported for single-DCI based STxMP PUSCH transmission in mTRP based system. In RAN1#110b-e, more details on SDM based STxMP was agreed like SRS resource set indication, layer indication and antenna port indication. In this contribution, we provide our views on remaining issues, for both sDCI based and mDCI based, for multi-panel simultaneous PUSCH transmissions.

# Simultaneous multi-beam PUSCH transmissions

## Indication of PUSCH Resources by a s-DCI

Multi-TRP based simultaneous PDSCH transmissions using a single DCI is achieved in Rel-16 by indicating the PDSCH resources associated to each TCI state (i.e., each beam). More precisely,

- For FDM simultaneous PDSCH transmissions through multi-TRP and by a single DCI indication, a UE is configured by higher layer parameter repetitionScheme set to one of 'fdmSchemeA', or 'fdmSchemeB', and the UE is indicated a TCI codepoint associated with two TCI states, and the indicated DM-RS port(s) are within one CDM group. FDRA bit-field indicates frequency domain resources that will be eventually split between the two TCI states, as part of PRB bundling and depending on the precoding granularity size, such that PDSCH occasions associated to each TCI state has non-overlapping frequency domain resource allocation with respect to the other PDSCH transmission occasion, as described in TS 38.214, Clause 5.1.2.3

- For SDM simultaneous PDSCH transmissions through multi-TRP and by a single DCI indication, the UE is indicated a TCI codepoint associated with two TCI states, but the indicated DM-RS ports are within two CDM groups, where the first TCI state corresponds to the CDM group of the first antenna port, and the second TCI state corresponds to the other CDM group (TS 38.214, Clause 5.1.6.2). Here, FDRA bitfield indicates a PDSCH transmission occasion that is used by both TRPs, although different layers of that single TB are transmitted by different TRPs.

In RAN1#110, SDM based STxMP was agreed. For STxMP PUSCH transmissions, a similar design as in Rel-16 PDSCH resource indication can be adopted. On single CW vs 2 CWs for SDM, in our view support of two CWs is not well justified. If the two PUSCHs toward two TRPs are so different for which different MCS is needed, switching back to single TRP transmission seems a better and more robust solution.

Proposal 1: For SDM STxMP, PUSCH with two codewords is not supported.

Proposal 2: For SDM STxMP, FDRA bit-field indicates a PUSCH transmission occasion that is used by both panels, although different layers of that single TB are transmitted by different panels.

## PUSCH Mapping to REs

If UE is indicated to transmit a single TB through simultaneous PUSCH transmissions over multi-panel, current specification can be adopted to determine the TB size. For mapping of the coded symbols to the REs, different options can be discussed, For example

- Option 1: start with the first allocated symbol, the coded symbols are mapped in frequency first to all resource elements (REs) associated to the 1st PUSCH and next to REs associated to the 2nd PUSCH. Next move to the next symbol.

- Option 2: start with 1st PUSCH, coded bits are mapped in all frequency and time REs associated to 1st PUSCH (frequency next time). Next, move to 2nd PUSCH.

Proposal 3: RAN1 should study different PUSCH mapping schemes in terms of UE complexity and/or PUSCH reliability.

## UL Precoding Indication

In Rel-17, when UE is indicated to perform PUSCH repetition in time with different UL beams associated to each repetition, SRI bit-field, and TPMI for CB based, is extended to indicate beam association to each repetition. The procedure assumes same number of layers, which is indeed the case for R17 TDM based repetitions. For SDM STxMP, it was agreed in RAN1#110b-e that

Agreement

For SDM scheme of single-DCI based STxMP PUSCH

- Configure two SRS resource sets for CB or NCB.

  - FFS: These two SRS resource sets can have different number of SRS resources for codebook -based or non-codebook based.

- For codebook-based PUSCH, DCI indicates two TPMI fields, and each TPMI field separately indicates the precoding information and the number of layers conveyed over the SRS ports of the indicated SRS resource in each SRS resource set.

- For non-codebook based PUSCH and codebook-based PUSCH, DCI indicates two SRI fields and each field indicates SRS resource(s) for each SRS resource set separately.

  - FFS: For codebook-based PUSCH, the two SRS resources indicated by the two SRI fields can have different number of SRS ports

Based on the above agreement, independent SRI/TPMI bit-fields is supported to indicate UL precoding and layer indication per PUSCH. On different number of SRS resources for the two configured SRS resource sets, the issue can be discussed separately for CB based and NCB based PUSCH. For CB based PUSCH, in our opinion there is no strong justification to have different SRS resources for the two configured sets, given that UE’s configuration for ul-FullPowerTransmission is applicable to both PUSCHs. For NCB based PUSCH though, different number of SRS resources for the two configured SRS resource sets is directly associated to the number of layers per PUSCH. Based on this discussion, we propose the following:

Proposal 4: If UE is configured with two SRS-ResourceSet with usage set to ‘codebook' and is indicated for STxMP with SDM scheme, the UE is not expected to be configured with different number of SRS resources in the two SRS resource sets

On different number of SRS ports for the two indicated SRS resources by the two SRI fields for CB based PUSCH, similar to the number of SRS resources across the two SRS resource sets for NCB based PUSCH, the issue is directly associated to the number of layers per PUSCH. In theory, for CB based PUSCH, a Rel-18 UE can be configured with two SRS resource sets and the indicated resources by SRI may have different number of antenna ports, nrofSRS-Ports. Although to achieve this configuration/indication, some combinations of maximum number of layers per SRS resource set for mTRP based SDM transmission, i.e. (L1,Mmax, L2,Mmax), and the maximum number of layers for sTRP based PUSCH LSmax, shall be excluded based on some number of indicated nrofSRS-Ports across the two SRS resources. For instance, if UE is configured with codebookSubset = nonCoherent, transform precoder is disabled, and ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and (LSmax, L1,Mmax, L2,Mmax) = (2/3/4,1,1), and (N1,ant, N2,ant) = (4,2) or (2,4), then in this case the total number of bits for TPMI fields associated to the two SRS resource sets for mTRP based STxMP is 3 bits, while if UE is dynamically indicated to switch to sTRP based operation, size of TPMI bit field is 4 bits. Generally speaking, when UE is configured with two SRS resource sets and STxMP operation, UE will assume two separate/independent SRI/TPMI bit-fields for which the total size of TPMI (or SRI) fields for mTRP based operation shall not be less than that of sTRP based operation. Based on the above discussion, we have the following proposals:

Proposal 5: For m-TRP based simultaneous PUSCH transmission with SDM scheme, UE is indicated with a triple (LSmax, L1,Mmax, L2,Mmax) where

- LSmax: represents maximum number of layers when UE is indicated to switch back to single TRP

- L1,Mmax, L2,Mmax: represents maximum number of layers respectively associated to the 1st and 2nd  SRS resource sets, when UE is indicated to perform simultaneous mTRP based transmission

Proposal 6: If UE is configured with two SRS-ResourceSet with usage set to ‘codebook' and it is indicated for STxMP with SDM scheme, UE can be configured and indicated with different higher layer parameters nrofSRS-Ports in SRS-Resource in different SRS-ResourceSet, subject to excluding any configuration/indication for which total number of bits for TMPIs for mTRP operation become less than TPMI bit-filed size for sTRP.

## UL Antenna Port Indication

For single-DCI based mTRP system and DMRS port indication in SDM based scheme of STxMP PUSCH transmission, it was agreed in RAN1#110b-e that

Agreement (110b-e)

- Reuse the DCI field ‘Antenna Ports’ in DCI format 0_1 and 0_2 to indicate DMRS ports for SDM scheme of single-DCI based STxMP PUSCH:

  - The total numbers of layers, L, indicated by two TPMI fields of CB PUSCH or two SRI fields of NCB PUSCH is used to determine the DMRS port indication table.

  - L1 of the indicated DMRS ports are associated with the L1 PUSCH layers which are indicated by the first TPMI field for CB PUSCH or the first SRI field for NCB PUSCH and the rest L- L1 of the indicated DMRS ports are associated with the L-L1 PUSCH layers which are indicated by the second TPMI field for CB PUSCH or the second SRI field for the NCB PUSCH.

    - FFS: how to partition the indicated DMRS ports among the PUSCH layers.

- Down-select one from the following two Alts for SDM scheme in RAN1#111:

  - Alt1: the DMRS ports associated with two TPMI/SRI fields must be in different CDM groups.

  - Alt2: the DMRS ports associated with two TPMI/SRI fields can be in same or different CDM groups.

Regarding the FFS on how to partition the indicated DMRS ports, in our view, the issue is already addressed within the agreement. Basically, SRS resource set indicator will indicate the association of each SRS resource set to the first and second SRI and TMPI (for CB PUSCH). Once that is indicated, based on the above agreement, UE is indicated on the antenna port indication of each PUSCH based on L1+L2 or L2+L1. Regarding whether or not the DMRS ports associated with two TPMI/SRI fields can be in the same CDM groups, in our view this can be handled by the scheduler and no additional specification is needed to keep different CDM groups. Based on this discussion, we support Alt2.

Proposal 7: For SDM scheme, the DMRS ports associated with two TPMI/SRI fields can be in same or different CDM groups.

## UCI multiplexing procedures

If a m-TRP based PUSCH indicated for STxMP with FDM-A or SDM scheme overlaps in time with a PUCCH that is associated to single TRP (s-TRP), UCI multiplying procedure may follow the existing specification per PUCCH+PUSCH resources that are associated to the same TRP. In other words, UE may resolve per panel PUCCH+PUSCH collision (multiplex UCI, drop PUCCH, etc) based existing procedures.

Proposal 8: If a mTRP based PUSCH+PUSCH overlaps in time with a PUCCH that is associated to a sTRP, UCI is multiplexed only on the PUSCH resource that is associated to the same TRP/beam/panel as of PUCCH

- FFS: How the code-rate for UCI multiplexing is determined, i.e., based on REs allocated to both PUSCHs, or based on only those REs associated to the PUSCH toward the same TRP as PUCCH

In current specification, TDAI bit-field in uplink DCI indicates number of HARQ-ACK bits that shall be multiplexed on PUSCH. More precisely, when there is overlapping PUSCHs and PUCCH, up to one PUSCH will be selected for HARQ-ACK multiplexing, where no overlapping PUSCHs within the same carrier with the same priority index, is expected. Now in Rel-18, if STxMP for PUSCH+PUSCH is supported for m-DCI based m-TRP, when UE is scheduled with fully or partially overlapping PUSCHs in the time and frequency domain by multiple PDCCHs with two different values of coresetPoolIndex, TDAI indication should be per panel, as shown in the following example. In this case, UE will multiplex HARQ-ACKs to each PUSCH independently, unlike current spec which only one PUSCH with UL-TDAI not equal to 4 carries HARQ-ACK

Figure 3: TDAI indication for dynamic HARQ-ACK codebook and m-DCI based STxMP

Proposal 9: If UE is scheduled with fully or partially overlapping PUSCHs in the time and frequency domain by multiple PDCCHs that are associated to different ControlResourceSets having different values of coresetPoolIndex, TDAI indication is per panel.

# Simultaneous multi-beam PUCCH transmissions

In NR R17, UE can transmit multiple repetitions of a PUCCH resource across different UL beams, where repetitions are TDM:

- Same PUCCH resource with the same UCI is repeated in intra-slot (scheme 3) or inter-slot (scheme 1)

  - two spatial relation info’s (PUCCH-SpatialRelationInfo) can be activated via MAC CE (38.213, Sec. 6.1.3.25), and/or

  - two PUCCH power control parameter sets is activated via MAC-CE

- If number of repetitions is more than 2, PUCCH mapping to beam can be cyclic or sequential

## PUCCH+PUCCH indication

In RAN1#109-e, it has been discussed to study and evaluate the following schemes for STxMP PUCCH transmission in single-DCI based mTRP:

- PUCCH FDM-A scheme: different frequency domain parts of one PUCCH resource are transmitted by two different UE panels.

- PUCCH FDM-B scheme: two FDM-ed PUCCH transmission occasion of the same UCI with the same PUCCH format are transmitted from two different UE panels

- PUCCH SFN scheme: The same PUCCH/PUCCH-DMRS is transmitted from two different UE panels with simultaneously

For simultaneous transmission of PUCCH over multi-panel using a single DCI, the STxMP scheme indication can be as part of RRC PUCCH resource configuration. For example, a PUCCH resource configuration is tagged with SFN in RRC configuration. Once that PUCCH resource in selected by DCI (or MAC-CE), UE is indicated to perform STxMP for PUCCH transmission, subject to UE capability, with the corresponding scheme.

Proposal 10: For simultaneous transmission of PUCCH over multi-panel (STxMP), UE will be indicated about the STxMP scheme in a single-DCI, as part of RRC PUCCH resource configuration

- a PUCCH resource configuration is tagged with SFN in RRC configuration

- Once that PUCCH resource in selected, UE is indicated to perform STxMP for PUCCH transmission with the corresponding scheme

# Conclusion

In this contribution, we provided some views on multi-panel simultaneous uplink transmissions. Based on what we discussed, the following proposals are made:

Proposal 1: For SDM STxMP, PUSCH with two codewords is not supported.

Proposal 2: For SDM STxMP, FDRA bit-field indicates a PUSCH transmission occasion that is used by both panels, although different layers of that single TB are transmitted by different panels.

Proposal 3: RAN1 should study different PUSCH mapping schemes in terms of UE complexity and/or PUSCH reliability.

Proposal 4: If UE is configured with two SRS-ResourceSet with usage set to ‘codebook' and is indicated for STxMP with SDM scheme, the UE is not expected to be configured with different number of SRS resources in the two SRS resource sets

Proposal 5: For m-TRP based simultaneous PUSCH transmission with SDM scheme, UE is indicated with a triple (LSmax, L1,Mmax, L2,Mmax) where

- LSmax: represents maximum number of layers when UE is indicated to switch back to single TRP

- L1,Mmax, L2,Mmax: represents maximum number of layers respectively associated to the 1st and 2nd  SRS resource sets, when UE is indicated to perform simultaneous mTRP based transmission

Proposal 6: If UE is configured with two SRS-ResourceSet with usage set to ‘codebook' and it is indicated for STxMP with SDM scheme, UE can be configured and indicated with different higher layer parameters nrofSRS-Ports in SRS-Resource in different SRS-ResourceSet, subject to excluding any configuration/indication for which total number of bits for TMPIs for mTRP operation become less than TPMI bit-filed size for sTRP.

Proposal 7: For SDM scheme, the DMRS ports associated with two TPMI/SRI fields can be in same or different CDM groups.

Proposal 8: If a mTRP based PUSCH+PUSCH overlaps in time with a PUCCH that is associated to a sTRP, UCI is multiplexed only on the PUSCH resource that is associated to the same TRP/beam/panel as of PUCCH

- FFS: How the code-rate for UCI multiplexing is determined, i.e., based on REs allocated to both PUSCHs, or based on only those REs associated to the PUSCH toward the same TRP as PUCCH

Proposal 9: If UE is scheduled with fully or partially overlapping PUSCHs in the time and frequency domain by multiple PDCCHs that are associated to different ControlResourceSets having different values of coresetPoolIndex, TDAI indication is per panel.

Proposal 10: For simultaneous transmission of PUCCH over multi-panel (STxMP), UE will be indicated about the STxMP scheme in a single-DCI, as part of RRC PUCCH resource configuration

- a PUCCH resource configuration is tagged with SFN in RRC configuration

- Once that PUCCH resource in selected, UE is indicated to perform STxMP for PUCCH transmission with the corresponding scheme

# References

- RP-213517, “New WID: MIMO Evolution for Downlink and Uplink”, Samsung (Moderator), Dec. 2021