3GPP TSG RAN WG1 #111	Tdoc R1-2212151

Toulouse, France, Nov 14-18, 2022

Agenda Item:	8.7

Source:	Ericsson

Title:	Miscellaneous corrections for alignment CR for UE power savings

Document for:	Discussion

# 1		Introduction

In this paper, some editorial corrections related to Rel-17 UE power savings are listed for consideration by the editors for RAN1 specifications 38.213 and 38.212.

For 38.213

- Subclause 10.1: Correct the reference IE for pei search space as highlighted below.

-	a Type2A-PDCCH CSS set configured by pei-SearchSpace in pei-ConfigBWP for a DCI format 2_7 with CRC scrambled by a PEI-RNTI on the primary cell of the MCG

- Subclause 10.1: Correct the RRC parameter name as highlighted below.

If a UE is provided

-	one or more search space sets by corresponding one or more of searchSpaceZero, searchSpaceSIB1, searchSpaceOtherSystemInformation, pagingSearchSpace, pei-SearchSpace, ra-SearchSpace, or a CSS set by PDCCH-Config, and

-	a SI-RNTI, a P-RNTI, a PEI-RNTI, a RA-RNTI, a MsgB-RNTI, a SFI-RNTI, an INT-RNTI, a TPC-PUSCH-RNTI, a TPC-PUCCH-RNTI, or a TPC-SRS-RNTI

then, for a RNTI from any of these RNTIs, the UE does not expect to process information from more than one DCI format with CRC scrambled with the RNTI per slot.

- Subclause 10.4B: Correct the RRC parameter name as shown below.

A UE in RRC_IDLE state or RRC_INACTIVE state can be provided by trs-ResourceSetConfiga set of TRS occasions [6, TS 38.214]. If trs-ResourceSetConfigis provided, a DCI format 2_7, if pei-SearchSpace is provided, and a DCI format 1_0 with CRC scrambled by P-RNTI includes a TRS availability indication field [4, TS 38.212] that provides a bitmap to groups of TRS resource sets where the configuration of each TRS resource set includes an association to a bit of the bitmap. The UE can be additionally provided a multiple, by validityDuration, for a number of frames provided by defaultPagingCycle for TRS resource sets with indicated presence; if validityDuration is not provided, the multiple is equal to 2.

For 38.212

- Subclause 7.3.1.2.1: Correct the RRC parameter name as shown below.

TRS availability indication – 1, 2, 3, 4, 5, or 6 bits, where the number of bits is equal to one plus the highest value of all the indBitID(s) provided by the trs-ResourceSetConfig if configured; 0 bits otherwise.

- Subclause 7.3.1.3.8: Correct the RRC parameter name as shown below.

The size of DCI format 2_7 is indicated by the higher layer parameter payloadSizeDCI-2-7, according to Clause 10.4A of [5, TS 38.213]. The number of information bits in format 2_7 shall be equal to or less than the payload size of format 2_7. If the number of information bits in format 2_7 is less than the size of format 2_7, the remaining bits are reserved.

# References

[1] 38.213-h30

[2]38.212-h30