3GPP TSG RAN WG1 Meeting#111		R1-2212290

Toulouse, France, November 14th – 18th, 2022

Agenda item:	9.3.2

Source:	KT Corp.

Title:	Discussion on subband non-overlapping full duplex

Document for:	Discussion

# Introduction

In the last RAN1 #110bis e-meeting, RAN1 could not reach consensus on below Proposed Agreements [1]:

Proposed Agreement:

For a SBFD aware UE semi-statically configured with UL subband in a symbol configured as DL in TDD-UL-DL-ConfigCommon, the following is agreed as baseline in the RAN1 study:

- UL transmissions within UL subband are allowed in the symbol

- UL transmissions outside UL subband are not allowed in the symbol

- A set of contiguous RBs in a carrier excluding RBs of UL subband and guardband(s) are defined as a DL subband for discussion purpose

  - FFS whether frequency location of DL subband(s) and guardband(s) are explicitly indicated or implicitly determined

  - FFS whether frequency location of UL subband/DL subband(s) and guardband(s), if explicitly indicated, is with reference to carrier or BWP

- DL receptions within DL subband(s) are allowed in the symbol

- FFS whether DL receptions within UL subband are allowed or not in the symbol

- FFS whether/how the symbols can be converted to DL-only symbols

Proposed Agreement:

For a SBFD aware UE semi-statically configured with UL subband in a symbol configured as flexible in TDD-UL-DL-ConfigCommon,

- UL transmissions within UL subband are allowed in the symbol

- FFS whether DL receptions within UL subband are allowed or not in the symbol

- FFS whether UL transmissions outside UL subband are allowed or not in the symbol

- FFS whether DL receptions outside UL subband are allowed or not in the symbol

- FFS: interaction of transmissions and receptions with TDD-UL-DL-ConfigDedicated and dynamic SFI

- FFS whether/how the symbols can be converted to non-SBFD symbols

- FFS: whether/how guardband(s) are explicitly indicated

In this contribution, above Proposed Agreements from the last RAN1 meeting are further discussed.

# Discussion

For a SBFD aware UE semi-statically configured with UL subband in a symbol configured as DL in TDD-UL-DL-ConfigCommon, it seems that we have common understanding allowing on UL subband in the symbol. Also, for DL reception within UL subband, as this can be achieved by dynamic indication of SBFD time domain location (i.e. indicating the symbol as non-SBFD symbol) this does not need to be stated here. Hence, KT proposes to delete the sub-bullet.

Proposal 1: Delete the sub-bullet on DL receptions within UL subbands are allowed or not in the symbol

For a SBFD aware UE semi-statically configured with UL subband in a symbol configured as DL in TDD-UL-DL-ConfigCommon, the following is agreed as baseline in the RAN1 study:

- UL transmissions within UL subband are allowed in the symbol

- UL transmissions outside UL subband are not allowed in the symbol

- A set of contiguous RBs in a carrier excluding RBs of UL subband and guardband(s) are defined as a DL subband for discussion purpose

  - FFS whether frequency location of DL subband(s) and guardband(s) are explicitly indicated or implicitly determined

- DL receptions within DL subband(s) are allowed in the symbol

- FFS whether DL receptions within UL subband are allowed or not in the symbol

- FFS whether/how the symbols can be converted to DL-only symbols

For a SBFD aware UE semi-statically configured with UL subband in a symbol configured as flexible in TDD-UL-DL-ConfigCommon, current proposal does not indicate the flexibility usage of TDD symbol, which allows conversions from SBFD symbol to DL-only or UL-only. And this should also enable DL reception outside UL subband since this is configured as flexible. Hence, KT proposes to include this in the proposal as below:

Proposal 2: Add and modify sub-bullets as below (in Red):

For a SBFD aware UE semi-statically configured with UL subband in a symbol configured as flexible in TDD-UL-DL-ConfigCommon,

- UL transmissions within UL subband are allowed in the symbol

- Allow conversion from SBFD symbol to DL-only, or UL-only symbol

- FFS whether DL receptions within UL subband are allowed or not in the symbol

- UL transmissions outside UL subband are not allowed in the symbol

- DL receptions outside UL subband are allowed in the symbol

- FFS: interaction of transmissions and receptions with TDD-UL-DL-ConfigDedicated and dynamic SFI

- FFS whether/how the symbols can be converted to non-SBFD symbols

- FFS: whether/how guardband(s) are explicitly indicated

# Conclusion

Based on the discussion in the above section, the following proposal is suggested.

Proposal 1: Confirm below proposal:

For a SBFD aware UE semi-statically configured with UL subband in a symbol configured as DL in TDD-UL-DL-ConfigCommon, the following is agreed as baseline in the RAN1 study:

- UL transmissions within UL subband are allowed in the symbol

- UL transmissions outside UL subband are not allowed in the symbol

- A set of contiguous RBs in a carrier excluding RBs of UL subband and guardband(s) are defined as a DL subband for discussion purpose

  - FFS whether frequency location of DL subband(s) and guardband(s) are explicitly indicated or implicitly determined

- DL receptions within DL subband(s) are allowed in the symbol

- FFS whether/how the symbols can be converted to DL-only symbols

Proposal 2: Confirm below proposal:

For a SBFD aware UE semi-statically configured with UL subband in a symbol configured as flexible in TDD-UL-DL-ConfigCommon,

- UL transmissions within UL subband are allowed in the symbol

- Allow conversion from SBFD symbol to DL-only, or UL-only symbol

- FFS whether DL receptions within UL subband are allowed or not in the symbol

- UL transmissions outside UL subband are not allowed in the symbol

- DL receptions outside UL subband are allowed in the symbol

- FFS: interaction of transmissions and receptions with TDD-UL-DL-ConfigDedicated and dynamic SFI

- FFS whether/how the symbols can be converted to non-SBFD symbols

- FFS: whether/how guardband(s) are explicitly indicated

# Reference

- R1-2210317, Summary#4 of subband non-overlapping full duplex. RAN1 #110bis-e