3GPP TSG RAN WG1 Meeting #111				 		R1-2212950

Toulouse, France, November 14th – 18th, 2022

Source:	Moderator (Intel Corporation)

Title:	Summary of offline discussion on bandwidth requirements on SL positioning

Agenda item:	9.5.1

Document for:	Discussion and Decision

# Introduction

This document presents a summary of offline discussion on bandwidth requirements for SL positioning.

# Discussion

Based on the submitted evaluation results for SL positioning and the agreed observations, the following proposal is made:

## Proposal 1

- Capture the following as part of the Conclusions section of TR 38.859:

  - Evaluation results reported as part of the study indicate that, depending on use-cases, scenarios, and positioning methods used, the identified target requirements can be satisfied with different values of SL-PRS bandwidth choices.

    - For FR1 spectrum:

      - For certain combinations of use-cases, scenarios, assumptions, and positioning methods, some target requirements can be satisfied with SL-PRS bandwidths of 20 MHz or 40 MHz.

      - For certain other combinations of use-cases, scenarios, assumptions, and positioning methods, some target requirements require SL-PRS bandwidth of 100 MHz or may not be satisfied even with SL-PRS bandwidth of 100 MHz.

    - For FR2 spectrum, based on submitted results from up to two sources:

      - For certain combinations of use-cases, scenarios, assumptions, and positioning methods, some target requirements can be satisfied with SL-PRS bandwidth of 200 MHz.

      - For certain combinations of use-cases, scenarios, assumptions,  and positioning methods, some of the Set B target requirements may not be satisfied even with SL-PRS bandwidth of 400 MHz.

  - From RAN1’s perspective, it is recommended that SL-PRS bandwidths of up to 100 MHz are supported by the specifications in FR1 spectrum.

  - [From RAN1’s perspective, it is recommended that SL-PRS bandwidths of up to 400 MHz are supported by the specifications in FR2 spectrum, if SL positioning in FR2 spectrum is considered for support.]

    - Note: The above recommendations are based on the evaluations in licensed and ITS spectra.

# Proposal for online session

## Proposal 1

- Capture the following as part of the Conclusions section of TR 38.859:

  - Evaluation results reported as part of the study indicate that, depending on use-cases, scenarios, and positioning methods used, the identified target requirements can be satisfied with different values of SL-PRS bandwidth choices.

    - For FR1 spectrum:

      - For certain combinations of use-cases, scenarios, assumptions, and positioning methods, some target requirements can be satisfied with SL-PRS bandwidths of 20 MHz or 40 MHz.

      - For certain other combinations of use-cases, scenarios, assumptions, and positioning methods, some target requirements require SL-PRS bandwidth of 100 MHz or may not be satisfied even with SL-PRS bandwidth of 100 MHz.

    - For FR2 spectrum, based on submitted results from up to two sources:

      - For certain combinations of use-cases, scenarios, assumptions, and positioning methods, some target requirements can be satisfied with SL-PRS bandwidth of 200 MHz.

      - For certain combinations of use-cases, scenarios, assumptions,  and positioning methods, some of the Set B target requirements may not be satisfied even with SL-PRS bandwidth of 400 MHz.

  - From RAN1’s perspective, it is recommended that SL-PRS bandwidths of up to 100 MHz are supported by the specifications in FR1 spectrum.

  - [From RAN1’s perspective, it is recommended that SL-PRS bandwidths of up to 400 MHz are supported by the specifications in FR2 spectrum, if SL positioning in FR2 spectrum is considered for support.]

    - Note: The above recommendations are based on the evaluations in licensed and ITS spectra.

# References

- RP-213588, Revised SID on Study on expanded and improved NR positioning, Intel (Email discussion moderator), RAN #94-e.