3GPP TSG RAN WG1#111			R1-2212965

Toulouse, France, Nov 14th – 18th, 2022

Title:	LS to RAN2 on msg1/msgA transmission channel access control in SIB1

Release:	Rel-17

Work Item:	NR_ext_to_71GHz-Core

Source:	RAN1

To:	TSG RAN WG2

Cc:

Contact person:	Jing Sun

jingsun@qti.qualcomm.com

# 1	Overall description

In RAN1 #111, RAN1 reaches the following agreement

Agreement

Introduce ra-ChannelAccess-r17 in SIB1 and other relevant RRC indicator fields (if needed) to control msg1/msgA transmission with and without LBT

- The TP CA-1-4 below is provided as an example of the expected RAN1 specification change.

- Send LS to RAN2 to request introducing ra-ChannelAccess-r17 in SIB1 and other relevant RRC indicator fields (if needed). If configured, indicating UE should perform LBT before msg1/msgA transmission. Ask RAN2 to confirm whether RAN2 is able to introduce this RRC change and confirm to RAN1.

- Note: gNB is responsible to configure the ra-ChannelAccess-r17 properly to comply with local regulation

====TP CA-1-4 for 37.213===============

4.4.5       Exempted transmissions from sensing

In regions where channel sensing is required to access a channel for transmission and short control signalling exemption is allowed by regulation, a gNB/UE may transmit the following transmission(s) on a channel without sensing the channel:

-     Transmission(s) of the discovery burst by the gNB

-     If the higher layer parameter ra-ChannelAccess-r17 is not configured, transmission(s) of the first message in a random access procedure by the UE

===End of TP CA-1-4======================

RAN1 would like to request RAN2 to introduce ra-ChannelAccess-r17 in SIB1 and other relevant RRC indicator fields (if needed). If ra-ChannelAccess-r17 is configured, UE should perform LBT before msg1/msgA transmission if channelAccessMode2-r17 is provided. RAN1 would like RAN2 to confirm whether RAN2 is able to introduce this RRC change.

# 2	Actions

To TSG RAN2

ACTION: 	RAN1 respectfully asks RAN2 to consider the above agreement and capture it in RAN2 spec or inform RAN1 if this RRC change cannot be introduced.

# 3	Dates of next TSG RAN WG1 meetings

TSG RAN WG1 Meeting #112	Feb 27 – March 03, 2023 		Athens, GR

TSG RAN WG1 Meeting #112-bis-e	April 17 – April 26, 2023	 	Online