3GPP TSG RAN WG1 #112	   	 	  	                	      R1-2300524

Athens, Greece, February 27th – March 3rd, 2023

Agenda item:	9.1.2

Source:	LG Electronics

Title:	Potential CSI enhancement for high/medium UE velocities and coherent JT

Document for:	Discussion and Decision

Introduction

In this contribution, we discuss potential CSI enhancement for high/medium UE velocities and for coherent JT. Since specification impact and functionality are different for high speed UE and for CJT, we first discuss Type II codebook enhancement for high/medium UE velocities in Section 2.1, TDCP reporting in Section 2.2, and potential CSI enhancement for CJT in Section 2.3

Discussion

Type II codebook enhancement for high speed UE

In this section, we focus the type II codebook enhancement for high speed UE and list up potential issues in the following order: channel measurement, CSI reporting window, TD (time domain) basis.

In order to support burst channel measurement for aperiodic CSI-RS, the following agreement was made in the last meeting.

Agreement

On the CSI reporting and measurement for the Rel-18 Type-II codebook refinement for high/medium velocities, support the following CSI-RS resource types/structures for CMR, support the following:

- (Alt1) Support K>1 NZP CSI-RS resources, received via a single triggering instance, for aperiodic (AP) CSI-RS-based channel measurement in a same CSI-RS resource set where the separation between 2 consecutive AP-CSI-RS resources is m slot(s)

Since the K AP NZP CSI-RS resources are burst CMRs for the same channel, UE should assume the same antenna port for the same antenna port index across the CSI-RS resources like TRS. Since it was agreed that the K CSI-RS resources are configured in NZP-CSI-RS-ResourceSet without CRI reporting, UE can assume the same antenna port for the same antenna port index across the CSI-RS resources if CRI reporting is not configured with CMR of multiple CSI-RS resources.

Proposal 1: UE assumes the same antenna port for the same antenna port index across the K AP CSI-RS resources if CRI reporting is not configured with CMR of multiple CSI-RS resources in a same CSI-RS resource set.

Also, since the K AP NZP CSI-RS resources are burst CMRs for the same channel, CPU relaxation is needed. According to current specification, L CPUs are occupied when L AP CSI-RS are configured for AP CSI reporting but given that the K CSI-RSs show the same antenna port, one CPU may be enough. In addition, CSI processing time relaxation is needed because UE should predict channel for N4 future time instances and calculate corresponding future PMIs. Legacy Z and Z’ is not sufficiently large enough considering the additional complexity.

Proposal 2: one CPU is occupied for an AP CSI report configured with K AP NZP CSI-RS resources for burst CMR.

Proposal 3: Study CSI processing time relaxation with consideration of additional complexity to predict future channel and calculate multiple future PMIs.

For CQI enhancement, three alternatives were agreed in the last RAN1 111 meeting and the following offline proposal was discussed during latest email discussion.

Agreement

For the Rel-18 Type-II codebook refinement for high/medium velocities, regarding the time instance and/or PMI(s) in which a CQI is associated with, given the CSI reporting window WCSI (in slots), assuming 1 CQI in one sub-band and one CSI reporting instance, down-select (by RAN1#112) one from the following alternatives:

- Alt1. The CQI is associated with the entire duration of the CSI reporting window and all the N4 W2 matrices

- Alt2A. The CQI is associated with the first/earliest slot of the CSI reporting window and the first/earliest of the N4 W2 matrices

- Alt2B.  The CQI is associated with the first/earliest d slots of the CSI reporting window and the first/earliest one of the N4 W2 matrices

Note: The N4 W2 matrices represent the combining coefficients before DD compression at the UE, or after DD de-compression at the gNB

Offline proposal 2.B.1: For the Rel-18 Type-II codebook refinement for high/medium velocities, regarding the time instance and/or PMI(s) in which a CQI is associated with, given the CSI reporting window WCSI (in slots), assuming 1 CQI in one sub-band and one CSI reporting instance, as well as the number of CQIs (=X) in one sub-band and one CSI reporting instance, support only the following:

- Basic feature: X=1 and the CQI is associated with the first/earliest slot of the CSI reporting window and the first/earliest of the N4 W2 matrices

- Optional features:

  - X=1 and the CQI is associated with:

    - the first/earliest slot of the CSI reporting window (slot l) and the first/earliest of the N4 W2 matrices, and

    - the last slot of the CSI reporting window (slot l+WCSI–1) and the N4-thW2 matrices

  - X=2 and

    - The 1st CQI is associated with the first/earliest slot of the CSI reporting window (slot l) and the first/earliest of the N4 W2 matrices, and

    - The 2nd CQI is associated with the middle slot of the CSI reporting window (slot l+WCSI/2) and the (N4 /2)-thW2 matrices

We support Alt 2A, which is aligned with legacy CQI calculation based on single reference resource slot.

Regarding Alt 1, it cannot achieve potential gain of PMI prediction even though CQI window and PMI window is matched. This is because, CQI should satisfy target BLER in every single slot in PMI window. As a result, conservative CQI (more conservative as N4 increases) will be reported, so that low MCS will be applied resulting in low DL throughput. If companies want to achieve potential gain of PMI prediction by matching CQI and PMI window, Alt 1 enhancement based on X=1 is not reasonable and X=N4 should be supported.

Alt 2B may have no difference with Alt 2A in terms of CQI outcome. This is because it is hard to expect channel quality varies during the first d slots and even though there is small variation there may be no impact on CQI outcome due to CQI quantization error.

Regarding potential compromised approach which is captured in Offline proposal 2.B.1, it is hard to predict interference especially in the last slot of CSI window. It means that CQI corresponding to the last slot of CSI window is inaccurate so that DL throughput performance is degraded. In our view, for X=1, all alternatives do not provide sufficient gain so that we prefer the simplest approach (Alt 2A) and in order to achieve sufficient gain we support X>1.

Proposal 4: Support Alt2A for single (i.e., X=1) CQI report

Proposal 5: Support X=N4 CQIs report to fully achieve potential gain of channel prediction

Regarding the upper bound of NNZC for DD compression codebook, legacy approach can be reused as much as possible. One simple approach is to apply K0 as upper bound per layer across all DD basis vectors and 2*K0 as upper bound across all layers across all DD basis vectors, where K0=⌈β×2LMQ⌉. If Q >2 is supported, then in order to avoid increasing upper bound we can consider K0=⌈β×2LM*min(2,Q)⌉.

Proposal 6: For upper bound of NNZC for DD compression codebook, apply K0 as upper bound per layer across all DD basis vectors and 2*K0 as upper bound across all layers across all DD basis vectors, where K0=⌈β×2LM*min(2,Q)⌉.

TDCP reporting for high speed UE

In the last meeting, several candidates of TDCP parameters were discussed and three alternatives were agreed. In our view, alternative A1 corresponding to Doppler spread is sufficient and simple.

Proposal 7: Support alternative A1 as TDCP parameter.

In the last meeting, it was agreed to support AP stand-alone report for TDCP, and one of remaining issue is how to handle collision with other UCI parameters such as CSI/ACK/SR on PUCCH or PUSCH. Like legacy UCI, multiplexing TDCP with other UCI should be supported instead of always dropping. Also, in case multiplexing is not available, how to select UCI to be reported/dropped, e.g., based on priority, should be studied.

Proposal 8: Support multiplexing TDCP with legacy UCI in case of collision and study how to drop TDCP/UCI if multiplexing is not available.

Potential CSI enhancement for coherent JT

In the last meeting, the following CSI-RS configuration for CJT was agreed.

Agreement

For the Rel-18 Type-II codebook refinement for CJT mTRP with NTRP>1 TRP/TRP-groups, the following is supported:

- The CMR comprises K>1 NZP CSI-RS resources, where one resource corresponds to one TRP/TRP-group (i.e. K=NTRP)

  - Each of the CSI-RS resources has a same number of CSI-RS ports

- Note: The terms TRP and TRP-group are used for discussion purposes only (no spec impact is implied).

Regarding multiple CSI-RS configuration for CJT, one remaining issue is how to aggregate CSI-RS ports of K NZP CSI-RS. One simple approach is to index ports of K NZP CSI-RS resources in the order of CSI-RS resource ID.

Proposal 9: For aggregated K NZP CSI-RS resources, index ports of the CSI-RS resources in the order of CSI-RS resource ID.

Another remaining issue is time domain restriction for K NZP CSI-RS resources. If those CSI-RS are distributed in different slot, there is possibility that UE measures each CSI-RS with random phase rotation and it is not able to report correct phase coherence information among K CSI-RS resources, which is essential part of coherent JT. Also, there is UE side buffering issue when K CSI-RS resources are far distributed in time. A similar issue was discussed in Rel-17 NCJT CSI-RS configuration and the restriction of the same slot or 1 slot difference between two CSI-RS for NCJT was agreed.

Proposal 10: Support to configure K NZP CSI-RS resources for CJT CMR in the same slot only.

In the last meeting, the following was discussed and agreed.

Agreement

On the Type-II codebook refinement for CJT mTRP, the selection of N CSI-RS resources is performed by UE and reported as a part of CSI report where N∈{1,..., NTRP}

- N is the number of cooperating CSI-RS resources, while NTRP is the maximum number of cooperating CSI-RS resources configured by gNB via higher-layer signaling

- The selection of N out of NTRP CSI-RS resources is reported via NTRP-bit bitmap in CSI part 1

  - Note: The value of N is inferred from the selection

- A restricted configuration (gNB-configured via higher-layer signaling) where N=NTRP is supported

  - NTRP-bit bitmap is not reported when the restriction is configured

  - FFS: Whether other RRC-configured TRP selection restriction including configuring the value of N is supported

- This feature is UE optional

Note: This agreement does not impact the decision on Ln being configured by gNB or selected by UE

Note: per WID and previous agreement, the candidate values for NTRP of are 1, 2, 3, and 4.

Note: only one transmission hypothesis is reported. UE is not mandated to calculate CSI for multiple transmission hypotheses.

Since selecting N TRPs among NTRP increases UE complexity, UE needs more CPUs. Even though UE is not mandated to calculate CSI for multiple transmission hypotheses, it does not preclude full search UE implementation. For given N and NTRP, there are NTRPCN hypotheses and if one CPU is used for each hypothesis NTRPCN CPUs are needed. Alternatively, instead of increasing CPU, we can consider to increase CSI processing time Z/Z’. In addition, to reduce complexity for TRP selection, some restriction for candidate combination for N TRPs can be considered. For example, gNB can configure N value and UE selects N out of NTRP for a given N.

Proposal 11: For UE side TRP selection, study CPU/CSI processing time relaxation with consideration of additional complexity to select CJT transmission hypothesis.

Proposal 12: For UE side TRP selection, introduce additional restriction such as configuring N value by gNB.

Additionally, we see codebook normalization issue for CJT codebook. In legacy codebook, selected precoding matrix is normalized to make sum of layer power to be 1. If the same normalization method is applied for agreed CJT codebook, UE selects lower CQI than what UE can actually achieve because transmission power gain from multiple TRPs is not reflected in codebook due to the conventional normalization.

Observation 1: UE selects lower CQI than what UE can actually achieve, if CJT codebook is normalized in conventional way.

Proposal 13: Study how to normalize CJT codebook considering transmission power gain from multiple TRP transmission.

Regarding W2 quantization group and Strongest Coefficient Indicator (SCI) design, the following was agreed in the last meeting.

Agreement

On the Type-II codebook refinement for CJT mTRP, regarding W2 quantization group, for each layer:

- Support the following: (Alt1) One group comprises one polarization across all N CSI-RS resources (Cgroup,phase=1, Cgroup,amp=2)

  - FFS: Amplitude quantization table enhancement

  - For the amplitude group other than the group associated with the SCI, the reference amplitude is reported

- Working assumption: Alt3 is supported in addition to Alt1 (to be confirmed in RAN1#111)

  - (Alt3). One group comprises one polarization for one CSI-RS resource with a common phase reference across N CSI-RS resources (Cgroup,phase=1, Cgroup,amp=2N)

    - For each of the (2N–1) amplitude groups (other than the group associated with the SCI), the reference amplitude is reported

- If the support Alt3 in addition to Alt1 is confirmed, only one of the two schemes will be a basic feature for UEs supporting Rel-18 Type-II CJT codebook

In our view, Alt 3 should be supported. Alt 1 is not sufficient to consider RSRP difference among TRPs. As a result, performance may not be guaranteed in case of distributed TRPs especially for larger ISD scenario.

Proposal 14: Confirm working assumption with supporting Alt 3 for W2 quantization group.

Regarding parameter configuration Ln, p, beta, this issue is not only related to UE side Ln combination selection but also related to TRP selection of UE, which was agreed as a new bitmap reporting with CSI. In other words, UE side TRP selection has an impact on {pv, β}. For example, if UE selects only k-th TRP among 4 TRPs (i.e., STRP transmission) it makes sense to determine {pv, β} only based on Lk. To this end, we prefer to define the parameter combinations for {Ltot, pv, β} and Ltot=n=1NLn where Ln=0 for non-selected TRP.

Proposal 15: Define the parameter combinations for {Ltot, pv, β}, where Ltot=n=1NLn and Ln=0 for non-selected TRP.

Regarding codebook subset restriction, whether to support separate configuration for each TRP or common configuration for all TRPs needs to be discussed. In our view, separate configuration should be supported because it is natural that each TRP can be implemented with different antenna configuration n1, n2. Even though applying common configuration is simple in terms of specification, it is too restrictive.

Proposal 16: Support separate codebook subset restriction for each cooperative TRPs.

Conclusion

In this contribution, we discuss potential CSI enhancement for high/medium UE velocities and for coherent JT and propose the followings.

Type II codebook enhancement for high speed UE:

Proposal 1: UE assumes the same antenna port for the same antenna port index across the K AP CSI-RS resources if CRI reporting is not configured with CMR of multiple CSI-RS resources in a same CSI-RS resource set.

Proposal 2: one CPU is occupied for an AP CSI report configured with K AP NZP CSI-RS resources for burst CMR.

Proposal 3: Study CSI processing time relaxation with consideration of additional complexity to predict future channel and calculate multiple future PMIs.

Proposal 4: Support Alt2A for single (i.e., X=1) CQI report.

Proposal 5: Support X=N4 CQIs report to fully achieve potential gain of channel prediction.

Proposal 6: For upper bound of NNZC for DD compression codebook, apply K0 as upper bound per layer across all DD basis vectors and 2*K0 as upper bound across all layers across all DD basis vectors, where K0=⌈β×2LM*min(2,Q)⌉.

TDCP reporting for high speed UE:

Proposal 7: Support alternative A1 as TDCP parameter.

Proposal 8: Support multiplexing TDCP with legacy UCI in case of collision and study how to drop TDCP/UCI if multiplexing is not available.

Potential CSI enhancement for coherent JT:

Proposal 9: For aggregated K NZP CSI-RS resources, index ports of the CSI-RS resources in the order of CSI-RS resource ID.

Proposal 10: Support to configure K NZP CSI-RS resources for CJT CMR in the same slot only.

Proposal 11: For UE side TRP selection, study CPU/CSI processing time relaxation with consideration of additional complexity to select CJT transmission hypothesis.

Proposal 12: For UE side TRP selection, introduce additional restriction such as configuring N value by gNB.

Observation 1: UE selects lower CQI than what UE can actually achieve, if CJT codebook is normalized in conventional way.

Proposal 13: Study how to normalize CJT codebook considering transmission power gain from multiple TRP transmission.

Proposal 14: Confirm working assumption with supporting Alt 3 for W2 quantization group.

Proposal 15: Define the parameter combinations for {Ltot, pv, β}, where Ltot=n=1NLn and Ln=0 for non-selected TRP.

Proposal 16: Support separate codebook subset restriction for each cooperative TRPs.

- References

[1] RP-213598	New WID: MIMO Evolution for Downlink and Uplink		Samsung

[2] R1-2210523	Summary of TDCP Alternatives for Comparison			Samsung