3GPP TSG RAN WG1 #112                                          	          R1- 2300548

Athens, Greece, February 27th – March 3rd, 2023

Agenda Item:	9.1.3.1

Source:	Xiaomi

Title:                     Discussion on DMRS enhancement

Document for:	Discussion and Decision

Introduction

In RAN 94e meeting, the work item of further enhancements on MIMO for NR with respect to DL MIMO was proposed as follows [1]:

- Study, and if justified, specify CSI reporting enhancement for high/medium UE velocities by exploiting time-domain correlation/Doppler-domain information to assist DL precoding, targeting FR1, as follows:

  - Rel-16/17 Type-II codebook refinement, without modification to the spatial and frequency domain basis

  - UE reporting of time-domain channel properties measured via CSI-RS for tracking

- Specify extension of Rel-17 Unified TCI framework for indication of multiple DL and UL TCI states focusing on multi-TRP use case, using Rel-17 unified TCI framework.

- Study, and if justified, specify larger number of orthogonal DMRS ports for downlink and uplink MU-MIMO (without increasing the DM-RS overhead), only for CP-OFDM,

  - Striving for a common design between DL and UL DMRS

  - Up to 24 orthogonal DM-RS ports, where for each applicable DMRS type, the maximum number of orthogonal ports is doubled for both single- and double-symbol DMRS

- Study, and if justified, specify enhancements of CSI acquisition for Coherent-JT targeting FR1 and up to 4 TRPs, assuming ideal backhaul and synchronization as well as the same number of antenna ports across TRPs, as follows:

  - Rel-16/17 Type-II codebook refinement for CJT mTRP targeting FDD and its associated CSI reporting, taking into account throughput-overhead trade-off

  - SRS enhancement to manage inter-TRP cross-SRS interference targeting TDD CJT via SRS capacity enhancement and/or interference randomization, with the constraints that 1) without consuming additional resources for SRS; 2) reuse existing SRS comb structure; 3) without new SRS root sequences

  - Note: the maximum number of CSI-RS ports per resource remains the same as in Rel-17, i.e. 32

In this contribution, we will further discuss some possible DMRS enhancements to support larger number of orthogonal DMRS ports and DMRS enhancements for 8 Tx UL operation in Rel18.

Discussion on DMRS enhancement to support lager number of DMRS ports

Potential Performance issue of R18 DMRS

In RAN1 110-bis meeting, we have made down selection among the possible DMRS enhancements and opt.1 should be support as shown in agreement below [5].

Agreement

Confirm the working assumption in RAN1#110 with the following update:

To increase the number of DMRS ports for PDSCH/PUSCH, support at least Opt.1 (introduce larger FD-OCC length than Rel.15 (e.g. 4 or 6)).

- FFS: FD-OCC length for Rel.18 DMRS type 1 and type 2.

- FFS: Whether it is needed to handle potential performance issues of Opt 1. For example, study if there is performance loss in case of large delay spread scenario. If needed, how (e.g. additionally support other options).

Based on the simulation in [2], there is channel estimation performance loss for Opt. 1, in which length 4 OCC is applied in frequency to support more DMRS ports, in case of large delay spread.

Observation 1: There is channel estimation performance loss for Opt.1 in case of large delay spread.

When there is large channel delay spread, the coherent bandwidth becomes small. For R18 DMRS design, more RE should be used to jointly channel estimation. Then, the small coherent bandwidth has more impact on R18 DMRS than that on legacy DMRS. The simplest way to handle this issue is that the DMRS can fall back to legacy DMRS when there is much channel estimation performance loss.

Proposal 1: R18 DMRS can fall back to legacy DMRS based on Network indication if there is much performance loss because of large delay spread.

Signalling design

DMRS port(s) indication

About DMRS port indication, we have following agreements in RAN1 111 meeting [2].

Agreement

- For the antenna ports indication in Rel.18 eType1/eType2 DMRS ports with maxLength = 1/2 for PDSCH, all of the following port combinations can be indicated:

  - Cat. 1) Legacy port indexes (eType 1: p=0~7, eType 2: p=0~11)

  - Cat. 2) New port indexes (eType 1: p=8~15, eType 2: p=12~23)

  - Cat. 3) Legacy port indexes and New port indexes at least within a CDM group at least for maxLength=1 (eType 1: up to 4 ports from {0, 1, 8, 9} and/or up to 4 ports from {2, 3, 10, 11}, eType 2: up to 4 ports from {0, 1, 12, 13} and/or up to 4 ports from {2, 3, 14, 15} and/or up to 4 ports from {4, 5, 16, 17}) at least for S-TRP case,

    - For up to 4 ranks, only one CDM group is used per UE. For larger than 4 ranks, more than one CDM groups can be used per UE.

- FFS: Whether to increase the size of antenna ports field in DCI format 1_1/1_2, or introduce new DCI field for antenna ports indication, or not.

- FFS: Whether the new antenna port(s) table is specified or not.

- FFS: MU restrictions for certain entries. e.g., DMRS ports = {0,2}, or {8,10}, etc.

- FFS: Cat.3 for M-TRP case.

- Note: DMRS port index for PDSCH is determined by p +1000

Agreement

For the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH, at least for S-TRP case, support the following rows of DMRS port combinations and Number of DMRS CDM group(s) without data.

- FFS: Antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH for M-TRP case.

Table 7.3.1.2.2-1-X: Antenna port(s) (1000 + DMRS port), dmrs-Type=eType1, maxLength=1

| One Codeword: Codeword 0 enabled, Codeword 1 disabled |  |  |  | Two Codewords: Codeword 0 enabled, Codeword 1 enabled |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Notes | Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Notes |
| 0 | [1] | [0] | Cat. 1 | [0] | [2] | [0,1,2,3,8] | [Rank 5-8 with one DMRS symbol] |
| 1 | [1] | [1] |  | [1] | [2] | [0,1,2,3,8,10] |  |
| 2 | [1] | [0,1] |  | [2] | [2] | [0,1,2,3,8,9,10] |  |
| 3 | 2 | 0 |  | [3] | [2] | [0,1,2,3,8,9,10,11] |  |
| 4 | 2 | 1 |  |  |  |  |  |
| 5 | 2 | 2 |  |  |  |  |  |
| 6 | 2 | 3 |  |  |  |  |  |
| 7 | 2 | 0,1 |  |  |  |  |  |
| 8 | 2 | 2,3 |  |  |  |  |  |
| 9 | [2] | [0-2] |  |  |  |  |  |
| 10 | [2] | [0-3] |  |  |  |  |  |
| 11 | [2] | [0,2] |  |  |  |  |  |
| 12 | [1] | [8] | Cat.2 |  |  |  |  |
| 13 | [1] | [9] |  |  |  |  |  |
| 14 | [1] | [8,9] |  |  |  |  |  |
| 15 | 2 | 8 |  |  |  |  |  |
| 16 | 2 | 9 |  |  |  |  |  |
| 17 | 2 | 10 |  |  |  |  |  |
| 18 | 2 | 11 |  |  |  |  |  |
| 19 | 2 | 8,9 |  |  |  |  |  |
| 20 | 2 | 10,11 |  |  |  |  |  |
| 21 | [2] | [8-10] |  |  |  |  |  |
| 22 | [2] | [8-11] |  |  |  |  |  |
| 23 | [2] | [8, 10], [9, 11] |  |  |  |  |  |
| 24 | [1] | [0,1,8] | Cat.3 |  |  |  |  |
| 25 | [1] | [0,1,8,9] |  |  |  |  |  |
| 26 | 2 | 0,1,8 |  |  |  |  |  |
| 27 | 2 | 0,1,8,9 |  |  |  |  |  |
| 28 | 2 | 2,3,10 |  |  |  |  |  |
| 29 | 2 | 2,3,10,11 |  |  |  |  |  |

Based on the agreement, legacy port indexes should be included in R18 antenna port(s) table. Then, there are two possible methods to design the R18 antenna port(s) table. First, we can specify a new antenna port(s) table and legacy port indexes are copied to the new table. Secondly, we can partially reuse the current table as shown in Table 1, in which legacy port indexes is reused and the Cat.2 and Cat.3 port indexes will be added subsequently. Both methods seem feasible to us. To reduce the storage requirement of terminal, we slight prefer reusing the existing Tables

Proposal 2: We slightly prefer reusing the existing Tables

In addition, because the supported DMRS port is doubled in R18 DMRS, how to indicate UE the DMRS port(s) should also be specified. A straight forward way to do this is to extend the current antenna field in DCI. While, there is another possible way where a “DMRS port(s) offset indicator” field can be introduced to indicate R18 DMRS port(s). For example, if this field is set “1”, DMRS port(s) are incremented with X from the indicated DMRS port(s) by antenna port(s) field.

However, this “DMRS port(s) offset indicator” method has less flexibility of network scheduling. For example, the table for dmrs-Type=1and single symbol, are selected among DMRS port 0~3, then the DMRS port of R18 DMRS indicated to UE can only be selected among port 0~3 or port 4~7 in this method. What’s more, Cat. 3 DMRS port indexes can not be supported in this method.

Observation 2: Introducing a “DMRS port(s) offset indicator” field to indicate R18 DMRS port(s) may lead to less flexibility of network scheduling and Cat. 3 DMRS port indexes cannot be supported.

Tab. 1 Reusing the current antenna table

To increase the flexibility of DMRS port(s) indication, M>1 bits “DMRS port(s) offset indicator” can be introduced to indicate R18 DMRS port. For example, as shown in Figure1, assuming the DMRS port(s) indicated by antenna ports field in DCI is {0, 1}, if the offset is 2, then the actual DMRS ports are {3, 4}. If the offset is 3, then the actual DMRS ports are {4, 5}.

Fig.1 M>1 bits “DMRS port(s) offset indicator”

Proposal 3: M>1 bits “DMRS port(s) offset indicator” can be introduced to increase the flexibility of DMRS port indication.

While, even introducing M>1 bits “DMRS port(s) offset indicator” can slightly increase the flexibility of DMRS port indication, but, still, some DMRS ports combination cannot be indicated in this method. For example, no matter what the value the DMRS port(s) offset indicator is, DMRS port {0,1,8} can not be indicated. The simplest and effective method is to increase the size of antenna ports field.

Proposal 4: Increase the size of antenna ports field DCI is a more appropriate way to indicate R18 DMRS port(s) compared with introducing a “DMRS port(s) offset indicator” field.

DMRS type indication

In current specification, DMRS type is indicated via RRC parameter dmrs-type which can be set to type1 or type2. In R18, enhanced DMRS will be introduced to support larger number of DMRS ports and there are two types of DMRS which are type1 and type2, denoted as Type1E and Type2E. Then, how to indicate UE one of these four DMRS types should be specified.

We can still use RRC parameter dmrs-type to indicate UE one of these four DMRS types. To do that, a new indicator can be introduced to inform UE whether the RRC parameter dmrs-type indicates legacy DMRS type or R18 DMRS type. For example, when the indicator is set to “A” and dmrs-type is “type2”, then, the DMRS type is Type2E. And if the indicator is set to “B” and dmrs-type is “type2”, the DMRS type is legacy DMRS Type2. The new indicator can be RRC signalling or DCI.

Proposal 5: Introduce a new RRC or DCI indicator to indicate the legacy DMRS or enhanced DMRS to UE.

Coexistence of legacy UE and R18 UE

Most companies preferred to support coexistence of legacy UE and R18 UE in MU-MIMO and we have reached an agreement about the coexistence as shown below [3]:

Agreement

Support MU-MIMO between Rel.15 DMRS ports and Rel.18 DMRS ports.

- For MU-MIMO by different CDM groups, no MU-MIMO scheduling restriction of PUSCH/PDSCH (i.e. MU-MIMO between Rel.15 UE and Rel.18 UE is allowed).

- For MU-MIMO within a CDM group, study whether and how to support MU-MIMO between Rel.15 DMRS ports and Rel.18 DMRS ports for PDSCH.

  - Note: the study includes MU-MIMO between Rel.15 UE and Rel.18 UE, and between Rel.18 UEs.

- Note: PUSCH above is CP-OFDM waveform.

For the case that R15 DMRS port and R18 port are in different CDM group, there is no MU-MIMO scheduling restriction of PUSCH/PDSCH. While, whether and how to support multiplexing of legacy DMRS port and R18 DMRS port within the same CDM group in MU-MIMO needs more discussion. In last RAN1 meeting, it is confirmed that length 4 OCC will be used to support larger number of DMRS ports in R18. Then, let’s assume that one of the length 4 OCC is [A,B,C,D]T, like the Hadamard code or Cyclic shift code as discussed in section 2.1. In order to support the multiplexing of R18 DMRS and legacy DMRS, [A,B]T is supposed to be orthogonal to [+1,+1]T and [+1,-1]T. Then we have the following equation:

Apparently, there is no non-zero solution for this equation. Hence, there is no such a length 4 OCC which is used in frequency to support larger number of DMRS ports can be orthogonal to length 2 OCC used in legacy DMRS.

Observation 3: There is no such a length 4 OCC can be simultaneously orthogonal to both [+1,+1]T and [+1,-1]T, used in legacy DMRS

Then, in order to multiplex legacy UE and R18 UE within one CDM group, there is restriction on DMRS port indication. For example, assuming Hadamard code shown in table 1 is used, if legacy UE is indicated with DMRS port 0 which applies OCC [+1,+1]T, then for R18 UE, only DMRS port(s) using FD-OCC with index 1 and 2 can be indicated.

Tab.1 Hadamard code

| FD-OCC index | wf(0) | wf(1) | wf(2) | wf(3) |
| --- | --- | --- | --- | --- |
| 0 | +1 | +1 | +1 | +1 |
| 1 | +1 | -1 | +1 | -1 |
| 2 | +1 | +1 | -1 | -1 |
| 3 | +1 | -1 | -1 | +1 |

Observation 4: Restriction on DMRS port indication is needed if the multiplexing of legacy UE and R18 UE within one CDM group needs to be supported.

Enhancements on DMRS ports for 8Tx UL SU-MIMO

The Rel-18 WID for MIMO Evolution for Downlink and Uplink is approved [1], which includes the following objective:

Study, and if justified, specify UL DMRS, SRS, SRI, and TPMI (including codebook) enhancements to enable 8 Tx UL operation to support 4 and more layers per UE in UL targeting CPE/FWA/vehicle/Industrial devices

- Note: Potential restrictions on the scope of this objective (including coherence assumption, full/non-full power modes) will be identified as part of the study.

In RAN1#109-e meeting, the following agreement has been made on DMRS configuration [3]:

Agreement

Study the following potential DMRS enhancement for potential support of more than 4 layers SU-MIMO PUSCH.

  - Extend DMRS port allocation table for rank 5~8

    - Note: DL DMRS table can be a reference

  - Enhancement for DMRS to PTRS mapping

- Study whether to utilize Rel.18 DMRS ports for more than 4 layers SU-MIMO PUSCH.

- Note: the above study does not imply more than 4 layers SU-MIMO PUSCH is supported.

- Note: other study for potential DMRS enhancement for potential support of more than 4 layers SU-MIMO PUSCH is not precluded.

In RAN1#110 meeting, the following agreement has been made on 8Tx transmission and DMRS configuration [4]:

Agreement

For > 4 layers PUSCH, support rank = 5,6,7,8 for both DMRS type 1/2, and for both single-symbol/double-symbol DMRS.

In RAN1#110bis meeting, the following agreement has been made on 8Tx transmission and DMRS configuration [5]:

Agreement

For more than 4 layers SU-MIMO PUSCH, support

- Both Rel.15 Type 1/Type 2 DMRS ports and Rel.18 eType 1/eType 2 DMRS ports.

  - For UE supporting Rel.18 eType 1/eType 2 DMRS ports, UE can be indicated with either of Rel.15 Type 1/Type 2 DMRS ports or Rel.18 eType 1/eType 2 DMRS ports.

    - RRC based indication is supported as the baseline. FFS whether DCI based indication is further needed.

  - For UE not supporting Rel.18 eType 1/eType 2 DMRS ports, UE can be indicated with Rel.15 Type 1/Type 2 DMRS ports only.

In RAN1#111 meeting, the following agreement has been made on 8Tx transmission and DMRS configuration [6]:

Agreement

- For > 4 layers PUSCH, support new antenna ports tables for rank = 5,6,7,8 for both single-symbol/double-symbol DMRS.

  - For Type 1/Type 2 Rel.15 DMRS ports, new antenna ports tables are the following:

    - The same DMRS port combination(s) as that for rank = 5,6,7,8 for PDSCH is reused at least for full or non-coherent UL codebook.

  - For Rel.18 eType1/eType2 DMRS ports,

    - New antenna ports tables with new DMRS port combinations are used for rank = 5,6,7,8 (FFS: details).

    - Note: Whether the DMRS port combination allows to use single symbol DMRS for rank = 5,6,7,8 should be checked.

  - FFS: For partial coherent UL codebook, support layers to DMRS port mapping that layers associated to the same antenna port group are multiplexed into the same DMRS CDM group.

  - FFS: One or more than one DMRS port combination(s) for each rank and TPMI

  - Note: New DMRS port combinations above does not preclude the new antenna ports tables including the current DMRS port combination(s) for PDSCH for rank = 5,6,7,8 in Rel.15-17.

  - FFS: Whether the antenna ports combinations for rank = 5,6,7,8 can be indicated by the reserved entries of existing antenna ports tables for rank =1,2,3,4, if the rank is indicated together with DMRS antenna ports.

For the uplink transmission with 8Tx, it needs further enhancements on DMRS design if it is agreed to support the maximum number of transmission layers to 8. In current spec, only 4 transmission layers for the PUSCH is supported. It has been decided that whether the supported maximum number of uplink layers is more than 4 or not is up to the UE capability. And for UEs supporting more than 4 layers, DMRS ports indication should be extended.

DMRS enhancements

Since the scenario mainly targeting SU-MIMO for 8Tx enhancement, DMRS port allocation design would be quite aligned with the DL, the DMRS port allocation for RANK5-8 should be introduced to each category of DMRS table. Currently the DMRS port allocation table is categorized according to DMRS type, max number of front-load DMRS symbols, and under each category DMRS tables for Rank 1-4 are specified separately. For DMRS type 1 with single-symbol configuration, the maximum number of DMRS ports is 4 which can’t support the RAN5 indication for 8Tx transmission. And also for DMRS type 2 with single –symbol configuration, the maximum number of DMRS ports is 6 which can support up to RAN6 of DMRS port indication. For DMRS Type 1/2 with double-symbol configuration cases, the supported RANK can be from 5 to 8.

For partial coherent UL codebook, we support layers to DMRS port mapping that layers associated with the same antenna port group can be multiplexed into the same or different DMRS CDM groups, which is aligned with the current principle of UL DMRS ports allocation. No scheduling limitation is required which is not the same principle as the DL.

It is necessary to support more than one DMRS port combinations for each Rank and TPMI that can provide more flexibility to the network especially for higher Rank cases since more DMRS ports can be allocated when new Rel-18 DMRS type is applied.

Proposal 6: For partial coherent UL codebook, support layers to DMRS port mapping that layers associated with the same antenna port group are multiplexed into the same or different DMRS CDM group.

Proposal 7: Support more than one DMRS port combination(s) for each rank and TPMI at least for Rel.18 eType1/eType2 DMRS design.

PT-RS enhancements

Also if DMRS ports are extended to 8 for more than 4 layers PUSCH transmission, PT-RS port(s) associated with DMRS port(s) should be redefined too. The enhancements is to indicate the association between PTRS port(s) and DMRS port(s) for the transmission of one PT-RS port and two PT-RS ports respectively

The following agreement was made in RAN1#110bis meeting on 8Tx and PT-RS enhancements [4]:

Agreement

For 8TX PUSCH, at least support

- Ng=1, 2, 4

Note: The above does not restrict the Ng for the non-coherent case

Agreement

For support of more than 4 layers SU-MIMO PUSCH, study the following potential enhancements for PTRS-DMRS association.

- Whether to support more than 2-port UL PTRS.

- Whether to increase the DCI size of PTRS-DMRS association field in DCI format 0_1/0_2.

To support more than 4 layers PUSCH with current 2-port PT-RS, the PTRS and DMRS association for 8Tx needs to be enhanced:

- 3bits are needed for the indication of PT-RS and DMRS ports association for UL PTRS port 0;

- 4bits are needed for the indication of PTRS and DMRS association when 2 PTRS ports are used, 2bits MSB  are for the indication of PTRS port 0, and 2 bits LSB are for the indication of PTRS port 1;

Proposal 8: Redefine the association between PT-RS port(s) and DMRS port(s), support the extension of the current DCI indication of PTRS and DMRS association for RANK>4.

Proposal 9: For Rank>4, support the following definition of the PTRS and DMRS association of 2-port PTRS for 8Tx.

- 3bits are needed for the indication of PT-RS and DMRS ports association for UL PTRS port 0;

- 4bits are needed for the indication of PTRS and DMRS association when 2 PTRS ports are used, 2bits MSB  are for the indication of PTRS port 0, and 2 bits LSB are for the indication of PTRS port 1;

If antenna ports can be divided into 4 antenna groups, eg. 4 separate panels, 2- port PT-RS may not be enough for the all CPE resources. 4-Port PTRS can be considered from performance point of view. The spec efforts should also be considered.

Proposal 10: Support 4-port PTRS for 8Tx transmission.

Conclusion

In this contribution, we discuss about the DMRS enhancements to support CJT and 8Tx UL transmission. Based on above discusses, we provide the following proposals related to DMRS enhancement for CJT:

Proposals wrt. DMRS enhancement to support lager number of DMRS ports:

Observation 1: there is channel estimation performance loss for Opt.1 in case of large delay spread.

Proposal 1: R18 DMRS can be fall back to legacy DMRS based on Network indication if there is much performance loss because of large delay spread.

Proposal 2: We slightly prefer reusing the existing Tables

Observation 2: Introducing a “DMRS port(s) offset indicator” field to indicate R18 DMRS port(s) may lead to less flexibility of network scheduling and Cat. 3 DMRS port indexes cannot be supported.

Proposal 3: M>1 bits “DMRS port(s) offset indicator” can be introduced to increase the flexibility of DMRS port indication.

Proposal 4: Increase the size of antenna ports field DCI is a more appropriate way to indicate R18 DMRS port(s) compared with introducing a “DMRS port(s) offset indicator” field.

Proposal 5: Introduce a new RRC or DCI indicator to indicate the legacy DMRS or enhanced DMRS to UE.

Observation 3: There is no such a length 4 OCC can be simultaneously orthogonal to both [+1,+1]T and [+1,-1]T, used in legacy DMRS

Observation 4: Restriction on DMRS port indication is needed if the multiplexing of legacy UE and R18 UE within one CDM group needs to be supported.

Proposals wrt. DMRS ports for 8Tx UL SU-MIMO:

Proposal 6: For partial coherent UL codebook, support layers to DMRS port mapping that layers associated with the same antenna port group are multiplexed into the same or different DMRS CDM group.

Proposal 7: Support more than one DMRS port combination(s) for each rank and TPMI at least for Rel.18 eType1/eType2 DMRS design.

Proposal 8: Redefine the association between PT-RS port(s) and DMRS port(s), support the extension of the current DCI indication of PTRS and DMRS association for RANK>4.

Proposal 9: For Rank>4, support the following definition of the PTRS and DMRS association of 2-port PTRS for 8Tx.

- 3bits are needed for the indication of PT-RS and DMRS ports association for UL PTRS port 0;

- 4bits are needed for the indication of PTRS and DMRS association when 2 PTRS ports are used, 2bits MSB  are for the indication of PTRS port 0, and 2 bits LSB are for the indication of PTRS port 1;

Proposal 10: Support 4-port PTRS for 8Tx transmission.

References

- RP-213517, “New WID: MIMO Evolution for Downlink and Uplink”, TSG RAN Meeting #94e Electronic Meeting, Dec. 6 - 17, 2021

- R1-2206623, “Discussion on DMRS enhancement”, TSG RAN WG1 #110 Meeting, Aug.22-26, 2022, Xiaomi

- RAN1 #109-e meeting Chairman notes, May 2022

- RAN1 #110 meeting Chairman notes, August 2022

- RAN1 #110-bis meeting Chairman notes, October 2022

- RAN1 #111 meeting Chairman notes, November 2022