3GPP TSG RAN WG1 #112		                                                          R1-2300643

Athens, Greece, February 27th – March 3rd, 2023

Source:	CATT

Title:	Discussion on the remaining issues on channel access mechanism

Agenda Item:	8.2

Document for:	Discussion and Decision

- Introduction

In RAN1#110b-e meeting [1], majority issues were discussed and reached consensus. In this contribution, we continue to discuss the channel access type upgrade within gNB COT and using Type 2 to resume UE COT for frequency range 2-2.

- Discussion

In NR system, the channel access update procedure can reduce the unnecessary sensing duration within the gNB COT and improve the channel access opportunity of UE. For shared spectrum operation on FR1, the UE may switch from Type 1 channel access procedure to type 2A if the scheduled PUSCH duration is within COT duration indicated by format 2_0.  Similar procedure shall be introduced for shared spectrum operation on FR2_2..

For channel access type update, if the UE determines the COT indicated by gNB from DCI format 2_0, the UE may switch from type 1 channel access to type 2 or type 3 channel access within the gNB COT. Considering the LBT mandatory region, if there is still no consensus to introduce a higher layer parameter to indicate UE whether switching to type 2 channel access procedure or type 3 channel access procedure, one simple solution is that the UE always switch from type 1 channel access to type 2 channel access within the gNB COT. Moreover, since the type 2 channel access depends on UE capability, the channel access type update within gNB COT procedure can be only applied to the UE indicating type 2 channel access capability.

Proposal 1: The channel access type upgrade within gNB COT upon detection of a DCI  2_0  should be supported in Rel-17. If a UE determines the COT indicated by gNB from DCI format 2_0, the following procedure can be applied only to the UE indicating type 2 channel access capability:

- The UE may switch from Type 1 channel access procedures to Type 2 channel access procedures for UL transmissions within the COT.

Draft CRs for 37.213 related to Proposal 1 are in [2].

Resuming a gNB initiated COT and COT sharing between gNB and UE are supported in FR 2-2. To achieve similar benefit, UE resuming a UE initiated COT should be supported in FR 2-2 as well. In RAN1, there is no consensus to introduce a higher layer parameter to indicate type 2 or type 3 LBT for UE resuming a UE initiated COT procedure. To comply with the regulations in all the regions, for the UE with type 2 LBT capability, type 2 LBT can be applied to UE if the gap between the DL transmissions and any previous transmission is at least 8 us. Otherwise, for the UE without type 2 LBT capability, type 1 LBT should be applied to UE if the gap between the DL transmissions and any previous transmission is at least 8 us.

Proposal 2: UE resuming a UE initiated COT procedure should be supported in Rel-17 as following:

- When type 2 channel access capability is indicated by UE, if the gap between the UL transmission(s) and any previous transmission(s) corresponding to the channel occupancy initiated by the UE is at least 8μs, the UL transmission(s) occurs following type 2 channel access procedure. Otherwise, the UL transmission(s) occurs following type 3 channel access procedure.

- When Type 2 channel access is not indicated by UE, if the gap between the UL transmission(s) and any previous transmission(s) corresponding to the channel occupancy initiated by the UE is at least 8μs, the UL transmission(s) occurs following type 1 channel access. Otherwise, the UL transmission(s) occurs following type 3 channel access procedure.

Draft CRs for 37.213 related to Proposal 2 are in [3].

- Conclusion

In this document, the channel access type upgrade within gNB COT, using Type 2 to resume UE COT is discussed. We provide several proposals as followings.

Proposal 1: The channel access type upgrade within gNB COT upon detection of a DCI  2_0  should be supported in Rel-17. If a UE determines the COT indicated by gNB from DCI format 2_0, the following procedure can be applied only to the UE indicating type 2 channel access capability:

- The UE may switch from Type 1 channel access procedures to Type 2 channel access procedures for UL transmissions within the COT.

Draft CR for 37.213 related to Proposal 1 are in [2].

Proposal 2: UE resuming a UE initiated COT procedure should be supported in Rel-17 as following:

- When Type 2 channel access capability is indicated by UE, if the gap between the UL transmission(s) and any previous transmission(s) corresponding to the channel occupancy initiated by the UE is at least 8μs, the UL transmission(s) occurs following Type 2 channel access procedure. Otherwise, the UL transmission(s) occurs following Type 3 channel access procedure.

- When Type 2 channel access is not indicated by UE, if the gap between the UL transmission(s) and any previous transmission(s) corresponding to the channel occupancy initiated by the UE is at least 8μs, the UL transmission(s) occurs following Type 1 channel access. Otherwise, the UL transmission(s) occurs following Type 3 channel access procedure.

Draft CR for 37.213 related to Proposal 2 are in [3].

- References

R1-2212587 FL summary for channel access mechanism of FR2-2 maintenance, ver 02 Moderator (Qualcomm)

R1-2300645, Draft CR on channel access type update within gNB COT, CATT, RAN1#112, February 27th – March 3rd, 2023

R1-2300644, Draft CR on UE resuming a UE initiated COT, CATT, RAN1#112, February 27th – March 3rd, 2023