3GPP TSG RAN WG1 #112	R1-2300903

Athens, Greece, February 27th - March 3rd, 2023

Agenda item:		9.14.3

Source:		Mavenir

Title:	Discussion on solutions for NR dynamic switching between DFT-S-OFDM and CP-OFDM

Document for:		Discussion and Decision

# 1	Introduction

3GPP Work Item Description [1] provides a potential target in R18. The objective of this work item is to specify further uplink coverage enhancements for PRACH, power domain and DFT-S-OFDM. Specify enhancements to support dynamic switching between DFT-S-OFDM and CP-OFDM (RAN1).

In the previous meeting (RAN WG1 #111), there was broad agreement that supported dynamically scheduled PUSCH dynamic waveform switching indication from UL scheduling DCI and assumed that supporting new 1-bit field for dynamic waveform indication from UL scheduling DCI.

Agreement

For DCI based solution,

- For supported dynamically scheduled PUSCH, support dynamic waveform switching indication from UL scheduling DCI

Note: “Supported dynamically scheduled PUSCH” is to be confirmed in further discussion

Note: It does not imply that the waveform switching indication applies to other transmission or not

- Indication from non-UL scheduling DCI is not supported.

Note: the working assumption made in RAN1#110b-e for “Support at least one of the following options for the dynamic waveform indication in R18” does not need to be confirmed

Working Assumption

Support new 1-bit field for dynamic waveform indication from UL scheduling DCI

- Note: no change of the current size alignment procedure between UL DCI and DL DCI

Agreement

Study the necessity of the following potential enhancements to assist the scheduler in determining waveform switching:

- Reporting power headroom related information based on PCMAX,f,c applicable to a target waveform

  - Target waveform can be same or different from waveform of an actual PUSCH transmission

  - FFS target RB allocation and/or target modulation order can be same or different from respective properties of an actual PUSCH transmission

  - FFS determination of target waveform, target RB allocation, target modulation order

  - FFS details, e.g. report PCMAX,f,c or Type 1 power headroom for a waveform, or difference thereof between waveforms

- PHR triggering enhancements, e.g.

  - Network-triggered PHR

  - PH becomes lower (higher) than a threshold

  - PHR triggered by waveform switching

- Reporting of recommended waveform or request to switch waveform

- Other solutions not precluded

We provide a waveform indication definition of the 1-bit for both DCI 0_0 and 0_1, besides, we recommend MAC CE as a dynamic switching method being used for CG PUSCH.

# 2	Dynamic switching between DFT-S-OFDM and CP-OFDM

we focus our attention on how to achieve dynamic switching between DFT-S-OFDM and CP-OFDM. This contribution proposes that not only DCI 0_1 but also DCI 0_0 could be used for dynamic switching, for DCI 0_0 could cover more scheduling scenarios than DCI_0_1 and DCI 0_2, for example, Msg3 and coverage limitation cases.

Propose1: Not only DCI 0_1 but also DCI 0_0 could be used for dynamic switching.

This contribution provides a UL waveform indicator option when 1-bit adding for dynamic switching to DCI format 0_0 and 0_1, as follows:

Porpose2: UL waveform indicator: 1 bit according to Table 1.

Table 1: UL waveform indicator

| Value of UL waveform indicator | UL waveform |
| --- | --- |
| 0 | CP-OFDM |
| 1 | DFT-S-OFDM |

MAC-CE can be applied to CG-type1 and CG-type 2 and DG PUSCH, while DCI only applied to CG-type2 and DG PUSCH. We propose to re-consider MAC-CE for CG PUSCH, as the waveform switching could help the moving CG-type1 UEs achieving a better coverage and performance. Moreover, this method can save DCI overhead to indicate waveform to the subsequent transmissions.

UL Waveform Activation/Deactivation MAC CE are new defined MAC CEs, whose details should be discussed with RAN2.

Propose3: UL waveform dynamic switching for CG PUSCH could be indicated by MAC CE

# 3	Conclusions

Propose1: Not only DCI 0_1 but also DCI 0_0 could be used for dynamic switching.

Porpose2: UL waveform indicator: 1 bit according to Table 1.

Table 1: UL waveform indicator

| Value of UL waveform indicator | UL waveform |
| --- | --- |
| 0 | CP-OFDM |
| 1 | DFT-S-OFDM |

Propose3: UL waveform dynamic switching for CG PUSCH could be indicated by MAC CE

# 4. References

[1] RP-220937  Revised WID on Further NR coverage enhancements