3GPP TSG RAN WG1 #112			     R1-2200949

Athens, Greece, February 27th – March 3rd, 2023

Source:	Intel Corporation

Title:	Physical Layer Enhancements for SL Operating in Unlicensed Spectrum

Agenda item:	9.4.1.2

Document for:	Discussion and Decision

# Introduction

In 3GPP TSG RAN meeting #94, a new work item related to NR Sidelink (SL) evolution was approved, whose description has been recently updated during the latest RAN meeting [1]. As part of the objectives of this working item (WI), that following aspects were included:

Study and specify support of sidelink on unlicensed spectrum for both mode 1 and mode 2 where Uu operation for mode 1 is limited to licensed spectrum only [RAN1, RAN2, RAN4] - Channel access mechanisms from NR-U shall be reused for sidelink unlicensed operation - Assess the applicability of sidelink resource reservation from Rel-16/Rel-17 to sidelink unlicensed operation within the boundaries of unlicensed channel access mechanism and operation - No specific enhancements for Rel-17 resource allocation mechanisms - If the existing NR-U channel access framework does not support the required SL-U functionality, WGs will make appropriate recommendations for RAN approval. - Physical channel design framework: Required changes to NR sidelink physical channel structures and procedures to operate on unlicensed spectrum - The existing NR sidelink and NR-U channel structure shall be reused as the baseline. - No specific enhancements for existing NR SL feature - The study should focus on FR1 unlicensed bands (n46 and n96/n102) and is to be completed by RAN#98.

In the context of enhancements to enable NR SL to operate in the FR-1 unlicensed band, which we will refer to as SL-U in the remaining of this document, during the past RAN1 meetings [2-5] the agreements captured at the end of this document within the Appendix were made.

In this contribution, a few topics related to enhancements to the physical channel design will be discussed, while view on other aspects are provided in our companion contribution [6]:

- Considerations on RB set and resource pool design

- Considerations on PSSCH/PSCCH design

- PSFCH Design and HARQ consideration

- Considerations on S-SSB design

- Considerations on SL slot design

# Considerations on RB Set and Resource Pool Design

During prior RAN1 meeting [2], it was agreed that the SL resource pool in Rel. 16/17 NR SL and the RB set in Rel.16 NR-U would be used as a baseline for SL-U design:

Agreement SL BWP, SL resource pool in R16/R17 NR SL and RB set in R16 NR-U are reused for SL-U as baseline - Only one SL BWP is (pre-)configured within a carrier - The SL BWP is (pre-)configured to include one or multiple SL resource pools - At least support that one SL resource pool can be (pre-)configured to include integer number of RB sets - FFS: whether/how to support one SL resource pool can include sub-set of PRBs of one RB set - FFS: the applicable resource pool - FFS: the impact on sub-channel size and number of sub-channels in a resource pool if sub-channel is supported - PRBs within intra-cell guard band of two adjacent RB sets belong to a resource pool if the resource pool includes the two adjacent RB sets - FFS details, e.g., how such PRBs are used, the applicable resource pool, etc. - FFS: whether R16/R17 NR SL S-SSB slots and/or new S-SSB slots (if supported) are excluded from resource pool - FFS: which slots belong to resource pool, e.g., how to set the value of bitmap, whether to consider SL-U/NR-U operating in the same carrier and whether TDD configuration are considered, etc. - FFS: the impact of PSCCH/PSSCH mapping to frequency resources on resource pool configuration, on sub-channel definition if sub-channel is supported, etc.

However, many aspects have been left for further study. Among them, it was left as an FFS (1st FFS of above agreement) whether to support one SL resource pool including a sub-set of PRBs of one RB set. In this matter, given that in regions where the ETSI BRAN [6] requirements must be fulfilled an RB set should have the form of an RB-based interlace, and the location of the RBs and their number is defined so that 80% of the channel bandwidth is occupied, it is quite unclear what benefit it may bring to support in an RB set only a portion of those RBs since the OCB requirements mandated by the ETSI BRAN [6] may not be met.

Observation 1:

- When 80% OCB must be met, supporting a sub-set of PRBs of one RB set would not meet regulatory requirements, and therefore the benefit in supporting a sub-set of PRBs in one RB set is unclear.

Another aspect that was also left for further study (2nd FFS of above agreement) is whether to support both contiguous RB and RB-based interlaced allocation in the same resource pool. Given that either one or the other type of allocation can be used at a given time based on the regional regulatory requirements, there is no technical reason to support both in the same resource pool.

Observation 2:

- There is no technical reason to support both contiguous RB and RB-based interlaced allocation in the same resource pool, since either one or the other can be used at a given time based on the regional regulatory requirements.

Another aspect to be finalized (3rd FFS of above agreement) is regarding on whether any restrictions should be applied to a sub-channel in terms of size and how many sub-channels should be supported in a resource pool. In our view, given that an LBT BW is equivalent to 20 MHz, which is the granularity over which an LBT is performed, a sub-channel should not be exceeding this BW, and a resource pool should be configured so that it is also smaller or equal to this BW.

Proposal 1:

- The sub-channel size should never exceed the LBT BW, i.e., 20 MHz and a resource pool should be configured so that the resource pool is smaller or equal to it.

In prior RAN1 meeting [2], in the context of the RB set design it was agreed that similarly as in Rel.16 NR-U no filtering is required to ensure that the transmission from one RB set won’t cause significant interference to the adjacent RB sets by having intra-cells guard bands between them. Furthermore, it was agreed that PRBs within an intra-cell guard band of two adjacent RB sets belong to a resource pool, if the resource pool includes the two adjacent RB sets, but it was left for further study (4th FFS in above agreement) how such PRBs should be used. In this matter, during last RAN1 meeting [4] the following agreement was reached:

Agreement Regarding usage of PRBs within intra-cell guard band of two adjacent RB sets: - Such PRBs can be used for PSSCH transmission if and only if a UE can transmit on the respective LBT channels after performing channel access procedure in multi-channel case and the UE uses both of these two RB sets for PSSCH transmission - FFS details, e.g., handling of potential unequal sub-channel size, for interlaced RB based transmission, whether the PRB(s) in the intra-cell guard band have the same interlace index(s) as the PRBs for PSSCH transmission in these two RB sets - Such PRBs are not used for PSCCH transmission - FFS: whether or not such PRBs are used for PSFCH/S-SSB transmission

Among other aspects, it was left for further study on whether or not different interlace index(s) should be supported between the PRBs in the intra-cell guard band and the other PRBs for PSSCH transmission. However, there is no clear technical reason to do so, while this may deteriorate PAPR, increase signalling overhead and significantly complicate the design.

Observation 3:

- There is no clear technical reason to support different interlace index(s) for PRBs in the intra-cell guard band compared to PRBs for PSSCH transmission.

Furthermore, while during last meeting it was concluded that PRBs in the intra-cell guard band of two adjacent RB sets cannot be used for PSCCH transmission, it was left for further study on whether these can be used for PSFCH/S-SSB transmission especially in light of meeting the OCB requirements. As discussed later along this document, for S-SSB transmissions a temporary OCB exemption could be applied to meet the OCB requirements mandated by the ETSI BRAN, and therefore there is no motivation in needing additional PRBs for such type of transmission. Similarly, as for PSFCH, even if this is mapped to an RB-based interlace, a PSFCH transmission could be mapped over a 10 RBs interlace without needing any additional PRBs for such type of transmission.

Proposal 2:

- The use of the PRBs within an intra-cell guard band of two adjacent RB sets is precluded for both S-SSB and PSFCH transmissions.

In prior agreement listed at the top of this section another FFS to be discussed (5th FFS in above agreement) is related on whether the S-SSB slots should or should not be excluded from the resource pool configuration. During prior RAN1 meetings [3-4], it was agreed that in addition to the S-SSB occasions defined in Rel.16/17, additional candidate S-SSB occasions will be defined, and also that at least the legacy S-SSB slots would be excluded from the SL resource pool:

Agreement If RAN1 decides that LBT is performed for S-SSB transmission, in addition to the S-SSB occasions in R16/R17 NR SL design, support additional candidate S-SSB occasions - FFS the number and locations of additional candidate S-SSB occasions - FFS when a UE transmits S-SSB on such additional candidate S-SSB occasions, and the related Rx UE’s behavior Agreement At least R16/R17 NR SL S-SSB slots are excluded from SL resource pool. - Note: whether or not additional candidate S-SSB occasions are excluded from resource pool will be discussed after the details of additional candidate S-SSB occasions are clearer

Given that in Rel.16 SL, the S-SSB slots are excluded from the resource pool to enhance the detection performance of the S-SSB blocks by ensuring that no other transmissions take place in those slots, we believe same principle and rationale can be applied for SL-U for either Rel.16/17 S-SSB occasions and additional occasions.

Proposal 3:

- Any additional S-SSB slots used for additional occasions are excluded from the resource pool configuration.

In agreement at the top of this section, another aspect that was left for FFS (6th FFS in above agreement) is on how to determine which slots belong to a resource pool, and whether to consider SL-U/NR-U operating in the same carrier. In this matter, the legacy SL design principles could be reused, where a bitmap is used to indicate the time domain resource of a resource pool through which the S-SSB slots, slots without enough UL symbols, and slots indicated as 0 by the bitmap are excluded from the resource pool. Furthermore, while a bitmap indication can be reused, for SL-U the case of bitmap value of all “1”s should be also allowed. As for simultaneous SL-U and NR-U operation in same unlicensed carrier, we do not see any motivation for this, since we believe in practice they will be operated once at the time.

Proposal 4:

- Similarly, as in Rel.16 SL a bitmap is used to indicate the available time domain resources of a resource pool, where bitmap value of all “1”s is allowed.

Finally, in above agreement last aspect that was left as FFS (7th FFS in above agreement) is on the impact of PSCCH/PSSCH mapping to frequency resources on resource pool configuration. In this matter, in order to reduce amount for blind decoding at the UE side to determine where a PSSCH/PSSCH transmission has been mapped, it may be preferable to simplify the design by having a 1:1 mapping between one sub-channel and one RB set.

Proposal 5:

- A sub-channel is contained within a single RB set.

# Considerations on PSSCH/PSCCH Design

In prior RAN1 meeting [5], the mapping between a sub-channel and the PRBs has been discussed, and the following agreement was reached:

Agreement For contiguous RB-based PSCCH/PSSCH transmission in SL-U: - Regarding mapping between sub-channel and PRBs, further study the following options: - Option 1 (sub-channel aligns with resource pool boundary): Same as in legacy NR SL, i.e., the mapping of sub-channel starts from the first PRB of the resource pool and mapped sequentially within the resource pool according to the sub-channel size - FFS: whether/how to use sub-channel(s) which include intra-cell guardband PRBs - FFS: whether/how to handle the case when the number of PRBs of the resource pool cannot be divided by sub-channel size - Option 2 (sub-channel aligns with RB set boundary): In each RB set, the mapping of sub-channel starts from the first PRB of the RB set and mapped sequentially within the RB set according to the sub-channel size - FFS: whether/how to use intra-cell guardband PRBs - FFS: whether/how to handle the case when the number of PRBs of one RB set cannot be divided by sub-channel size - Option 3 (sub-channel aligns with RB set boundary): In each RB set, the mapping of sub-channel starts from the first PRB of the RB set and mapped sequentially within the RB set and/or guardband PRB according to the sub-channel size - FFS: how to use intra-cell guardband PRBs - FFS: how to use the subchannel including PRBs in guardband

Option 1 is in line with the legacy SL design, and there is no strong technical reason to deviate from it when the OCB requirements do not need to be met. Under this option, the case that number of PRBs of the resource pool cannot be divided by sub-channel size should never occur since the allocated number of resources are still an integer number of PRBs in one sub-channels.

Proposal 6:

- For contiguous RB-based PSCCH/PSSCH transmission, regarding mapping between sub-channel and PRBs, similarly as Rel. 16 NR SL, the mapping of sub-channel starts from the first PRB of the resource pool and mapped sequentially within the resource pool according to the sub-channel size (option 1).

In prior RAN1 meetings [2-4], several agreements have been made regarding PSSCH/PSCCH design. In particular, while it has been agreed that an RB-based interlaced structure is used to meet the 80% OCB requirements mandate by ETSI BRAN [6], it has been still left as FFS on whether different resource allocation mapping should be additionally supported. In this matter, we believe that additional resource allocation mapping with different granularity is not needed since we do not expect any significant IBE advantage from it, while this may negatively impact multiplexing capability and require a significant specification impact.

Observation 4:

- Other candidate frequency domain resource allocation granularities for the interlaced resource block structure may lead to insignificant IBE advantages, while lowering possible multiplexing capabilities and introducing very large specification impact.

To strive to have a unified design for the cases with and without OCB requirements, a logical mapping between a SL sub-channel and an interlace is defined, and the relationship between the two could be dictated based upon the frequency span of a SL sub-channel. In particular, in prior RAN1 meetings [2-3] it has been agreed that one sub-channel may be equal to K interlaces, where it was left as FFS on whether K is fixed to 1 or (pre-) configured. However, during last meeting [4], it has been concluded that K could be pre-configured and K= {1,2} and K={1} has been supported for 15 kHz and 30 kHz subcarrier spacing (SCS):

Agreement For interlace RB-based PSCCH/PSSCH transmission in SL-U: - Regarding 1 sub-channel equals K interlace(s) - At least K=1 and K=2 is supported for 15 kHz SCS - At least K=1 is supported for 30 kHz SCS - FFS: details related to multiple RB sets

However, considering that in Rel.16 a sub-channel could be configured to have 20, 50 and 100 PRBs, K= {5,10} for 15 kHz SCS and K={2, 5} for 30 kHz SCS could be additionally supported to achieve higher flexibility and support similar configurability as Rel.16.

Proposal 7:

- K= {5,10} for 15 kHz SCS and K= {2,5} for 30 kHz SCS are supported, where K is the number of interlaces within a single sub-channel.

During prior RAN1 meeting [4] it has been discussed on whether the same or different interlace indexes could be assigned to different RB sets when more than one RB set is used for transmissions:

Agreement Regarding frequency domain resource indication for interlace RB-based PSSCH transmission: - When more than one RB set is used for transmissions, down-select one of the followings - Option A: Support that the used interlace index(s) in different RB sets are always the same - Option B: Support that the used interlace index(s) in different RB sets can be different - FFS details

In this matter, there is no strong technical reason to support different interlace index(s) in different RB sets when more than one RB set is used for transmissions, while this may deviate from Rel.16 NR-U principles, but more importantly have severe negative impact on PAPR and signaling overhead.

Proposal 8:

- The interlace index is the same across different RB sets when a UE uses more than one RB set for transmission (Option A).

In the prior RAN1 meeting [5], the mapping between a sub-channel and an RB-based interlace has been discussed, and the following agreement was reached:

Agreement For interlace RB-based PSCCH/PSSCH transmission in SL-U: - Regarding mapping between sub-channel and interlace, RAN1 further study the followings: - Option 1: 1 sub-channel is defined and indexed within 1 RB set, and is periodically indexed across different RB sets within the resource pool - Option 2: 1 sub-channel is defined within 1 RB set, and is incrementally indexed firstly within an RB set, then across different RB sets within the resource pool - Option 3: 1 sub-channel is defined across all RB sets within the resource pool, i.e., 1 sub-channel includes K interlace(s) across all RB sets within the resource pool - Option 4: 1 sub-channel is defined within 1 RB set or 2 adjacent RB sets, and is incrementally indexed firstly within an RB set, then across different RB sets within the resource pool - Option 5: 1 sub-channel is defined within 1 RB set, and is incrementally indexed firstly across different RB sets within the resource pool, then across different interlaces in the RB set - FFS: whether/how to use intra-cell guardband PRBs

It is our understanding that both option 3 and 4 introduce a new mechanism that is neither used in legacy NR-U or SL, and that may require a larger specification impact compared to other options. Furthermore, option 4 is more an optimization of a special case rather than a general solution since it is designed having in mind the case of two adjacent interlaces (mainly for 30 kHz SCS). With that said, for both option 1, 2 and 5 the sub-channel is defined within an RB set. However, option 1 follows more strictly the design principles from Rel. 16 NR-U, and may require limited specification impact. Also this option precludes, compared to option 2 and 5, the use of different interlace across different RB sets, which as discussed above is highly preferred.

Proposal 9:

- For interlace RB-based PSCCH/PSSCH transmission, regarding mapping between sub-channel and interlace, one sub-channel is defined and indexed within 1 RB set and is periodically indexed across different RB sets within the resource pool (option 1).

# Considerations on PSFCH and HARQ Design

In prior RAN1 meeting [3], it has been agreed that to meet the 80% OCB requirements mandated by the ETSI BRAN [6] an RB-based interlaced structure would be supported for at least 15 and 30 kHz SCS when this requirement must be met. Furthermore, different alternatives have been identified on how to map a PSFCH when an RB-based interlaced structure is used and some of them have been defined with the intention to enhance capacity, given that this may reduce when more RBs are associated for single PSFCH transmission. In particular, in prior RAN1 meeting, the following agreement was made:

Agreement Regarding PSFCH transmission under 15 kHz and 30 kHz SCS, RAN1 continues studying the following updated alternatives: - Alt 1-1a: each PSFCH transmission occupies 1 common interlace and K3 dedicated PRB(s) - FFS: value of K3 - Alt 2-1a: each PSFCH transmission occupies 1 interlace, and further apply frequency-domain OCC - FFS: details of FD-OCC, e.g., OCC length, RB-level, RE-level, etc. - Alt 2-2a: each PSFCH transmission occupies 1 interlace, and further apply PRB-level cyclic shift - A UE transmits dedicated cyclic shift on K1 dedicated PRB(s) within this interlace, and transmits common cyclic shift on other PRBs of this interlace - FFS: value of K1 - Alt 2-3a: each PSFCH transmission occupies 1 interlace - Alt 2-4a: each PSFCH transmission occupies 1 interlace, and further apply PRB-level cyclic shift - A UE uses different cyclic shifts on different PRBs in the interlace - Alt 3-1a: each PSFCH transmission occupies 1 dedicated PRB and K2 common PRBs, where K2 common PRBs locate at the two edges of a RB set - The above dedicated PRB and common PRBs are within 1 interlace - FFS: value of K2 - Alt 3-2a: each PSFCH transmission occupies 1 dedicated PRB and 2 common PRBs, where 2 common PRBs locate at the two edges of a RB set - FFS: the impact of PSD limit, e.g., whether/how to handle the case when common PRB and dedicated PRB locate within the same 1 MHz bandwidth - FFS: whether IBE issue exists and whether/how to address it - Note: in the above descriptions - The dedicated PRB/cyclic shift conveys ACK/NACK information - Note: as previously agreed: to meet OCB and PSD requirement for PSFCH transmission, at least RB-based interlace is supported at least for 15 kHz and 30 kHz SCS.

While option 1-1a, 2-1a, 2-2a, and 2-3a clearly meet the OCB requirements by design, for both 3-1a and 3-2a it is debatable and unclear how these may be met, and how the previous agreement regarding utilizing an RB-based interlace structure for 15 kHz and 30 kHz SCS still folds.

Proposal 10:

- For 15 and 30 kHz subcarrier spacing, RAN1 should down-select both Alt-3-1a and Alt-3-2a.

As for the case of 60 kHz SCS, the following two alternatives were identified by RAN1:

Agreement Regarding PSFCH transmission under 60 kHz SCS, further study the following alternatives: - Alt 1: Each PSFCH transmission occupies K dedicated PRB(s) and some common PRBs - FFS details - Alt 2: Each PSFCH transmission occupies some dedicated PRBs - FFS details

However, both options could be applicable to either an RB-based interlace or a contiguous RB allocation, and therefore before down-selecting one of them RAN1 should first discuss and conclude on this aspect. Given that in Rel.16 NR-U no RB-based interlace structure was introduced for 60 kHz SCS and given that in this case the gain in terms of capacity would be very minimal, while this may require a large effort from RAN1 with also large specification impact, it is highly preferrable to not support any RB-based interlace structure in SL-U as well.

Proposal 11:

- Regarding PSFCH transmission for 60 kHz subcarrier spacing, no RB-based interlace is introduced, and the OCB requirements are met through contiguous RB allocation.

As discussed in our companion contribution [6], it is beneficial to transmit a PSFCH transmission within a shared COT and may be possible in some cases to aim for an LBT free design if the UE appends a properly long cyclic prefix extension before a PSFCH transmission so that a gap between any prior burst within the COT and this transmission may be less than 16 us. Despite on the case when a gap smaller than 16 us could be supported, a PSFCH transmission may be still subject to LBT if the gap is larger than 16 us, and in this case a PSFCH may not be transmitted due to LBT failure. In order to mitigate this issue, enhancements to the HARQ-feedback procedure defined in Rel.16 SL may be needed, and in this case a PSSCH/PSSCH slot could be associated to multiple PSFCH occasions through a one-to-many mapping, as illustrated in Figure 1, in order to allow a RX SL UE multiple opportunities to perform LBT and transmit the related PSFCH.

Figure 1 - Illustration of one-to-many mapping from PSCCH/PSSCH slot to a PSFCH occasion.

Proposal 12:

- In SL-U, multiple PSFCH occasions for a PSSCH/PSCCH slot are supported.

In prior RAN1 meeting [3], it was further discussed how the PSFCH resources should be indicated, and whether a (pre-)configured or dynamic indication would be needed. During the meeting, it was argued that in order to cope with changes in MCOT length and likelihood that PSFCH occasions may fall outside of a COT, and then be subject to higher level of congestion, a more dynamic indication of the PSFCH occasions may be preferred. In this matter, while as indicated in our companion document [5] we agree that PSFCH outside a COT should be avoided by design, given that to cope with possible LBT failures multi PSFCH occasions could be defined, there is no necessity to dynamically configure the resources and these could be still (pre-)configured as in Rel.16 SL design, while cross-COT PSFCH occasions should not be allowed.

Proposal 13:

- The PSFCH resources are (pre-)configured.

Proposal 14:

- Cross-COT PSFCH occasions are not allowed.

In Rel.16 NR SL design, due to half-duplexing issues of which SL UEs may suffer, it has been established a procedure according with a UE in RA mode 1 may generate a NACK when the UE does not receive PSFCH in any PSFCH reception occasions associated with a PSSCH transmission in a resource provided by either scheduling DCI or configured grant. Furthermore, a UE in RA mode 1 may additionally generate a NACK when the UE does not transmit PSSCH in any resource provided by either scheduling DCI or configured grant. Since in SL-U a UE may additionally not receive a PSFCH and may not transmit a PSSCH due to an LBT failure, this aspect should be also reflected in this Rel.16 NR SL procedure.

Proposal 15:

- A UE in RA mode 1 may report a NACK if

- it does not receive PSFCH due to an LBT failure in any PSFCH reception occasions associated with a PSSSCH transmission in a resource provided by DCI or for a configured grant.

- it is unable to transmit PSSCH due to LBT failure in any of the resource provided by DCI or for a configured grant.

# Considerations on S-SSB Design

In prior RAN1 meetings, in order to overcome the latency effect that an LBT failure may have on an S-SSB transmission, RAN1 has conveyed to introduce additional S-SSB occasions. Furthermore, in order to meet the regulatory requirements in terms of OCB, which are mandated by the ETSI BRAN [6], several design options have been identified:

Agreement For S-SSB and synchronization in SL-U: - FFS the time domain locations of S-SSB resources, e.g., whether/how to introduce more candidate occasions compared with R16/R17 NR SL design, etc. - Down-selection at least one of the following solutions to meet OCB and PSD requirement for S-SSB transmission - Option 1: Using interlaced RB transmission - Option 2: S-SSB multiplexing with other SL transmissions in the same slot - Option 3: Repetition of S-PSS/S-SSS/PSBCH in frequency domain - Option 4: S-PSS/S-SSS/PSBCH with wider bandwidth - FFS: whether to support 4 symbols S-SSB - Note: 4 symbols S-SSB can be considered with options 1/2/3/4 above - FFS whether the temporary exemption of OCB requirement is applicable for S-SSB transmission - FFS whether any changes to R16/R17 NR SL synchronization procedure Agreement If RAN1 decides that LBT is performed for S-SSB transmission, in addition to the S-SSB occasions in R16/R17 NR SL design, support additional candidate S-SSB occasions - FFS the number and locations of additional candidate S-SSB occasions - FFS when a UE transmits S-SSB on such additional candidate S-SSB occasions, and the related Rx UE’s behavior Agreement For S-SSB and synchronization in SL-U: - No changes on R16 NR SL S-PSS/S-SSS sequence generation Continue studying the 4 options from the previous agreement and whether/how temporary exemption of OCB requirement is applicable for S-SSB transmission, e.g., how to meet the minimum of 2 MHz requirement under 15 kHz SCS Agreement For S-SSB transmission, down-select one or more of the following for 15 kHz and 30 kHz SCS: - Option 1-1: Using interlaced RB transmission for all of S-PSS/S-SSS/PSBCH - Option 1-2: Using interlaced RB transmission for PSBCH only, and apply OCB exemption to S-PSS and S-SSS - Option 3-1: Repeat S-PSS/S-SSS/PSBCH N times in frequency domain, and there is a gap between the repetition(s) to meet OCB requirement - FFS details, e.g., the length of gap is (pre-)configured or pre-defined, value of N (e.g., N=2) - FFS gap of 0 - Option 3-2: Repeat only S-PSS/S-SSS K times in frequency domain, and PSBCH is rate matched. There is a gap between the repetition(s) to meet OCB requirement - FFS details, e.g., the length of gap is (pre-)configured or pre-defined, value of K - FFS gap of 0 - FFS PSBCH resource - Option 3-3: keep the legacy S-PSS/S-SSS/PSBCH while repeating PSBCH N times in frequency domain and rate-matching PSBCH to S-PSS/S-SSS symbols, and there is a gap between the PSBCH repetition(s) to meet OCB requirements - FFS details, e.g. the length of gap is (pre-)configured or pre-defined, value of N - Option A: Apply OCB exemption to all of S-PSS/S-SSS/PSBCH - For Option 1-1 and 1-2 above - FFS: whether/how to handle the case when each interlace has only 10 PRBs in a RB set - FFS: whether transient period issue exists and whether/how to address it

In this matter, one aspect that may require discussion is related on whether a 4 symbols S-SSB may need to be introduced or not. On this aspect, while we agree that this is a desirable from channel perspective since a larger number of S-SSB opportunities could be defined within the same set of slots, same coverage and reliability as legacy design is not possible with none of the options identified by RAN1 especially for 60 kHz SCS. With that said, 4 symbols S-SSB is not a preferred solution.

Proposal 16:

- 4 symbol S-SSB is not supported for SL-U

Furthermore, given that ETSI BRAN [6] allows for an OCB exemption for transmissions which may occur in a temporal manner as long as a minimum of 2 MHz BW occupancy is guaranteed, and given that as per third agreement listed above no spec impact is needed when this exemption is applied to S-SSB for 30 kHz and 60 kHz SCS (in this case S-SSB does occupy more than 2 MHz), and considering that legacy S-SSB transmissions may be infrequent and may in fact qualify as a short control signaling, it seems proper to apply such exemption to this channel:

Proposal 17:

- The 2 MHz exemption defined by ETSI BRAN is applied to the legacy S-SSB transmission.

While as mentioned above, if the OCB exemption is applied to legacy S-SSB, its physical structure can be reused as is for 30 and 60 kHz, for 15 kHz SCS the legacy S-SSB spans over 11 PRBS and for 15 kHz this corresponds to 1.92 MHz. If RAN1 conveys that 15 kHz SCS is needed, in order to limit specification, impact the PSBCH could be mapped and wrapped around S-PSS/S-SSS so that to occupy 12 RBs as illustrated in Figure 2.

Figure 2 – Illustration of PSBCH mapping for 15 kHz SCS in order to meet the 2 MHz minimum BW occupancy

Proposal 18:

- In order to support 15 kHz SCS, PSBCH is mapped so that to span over 12 RBs, and is wrapped around S-PSS and S-SSS.

During prior RAN1 meetings, it was agreed to introduce additional S-SSB occasions (conditional on whether LBT would be performed for S-SSB transmissions), and some options have been identified on how to employ them:

Agreement Regarding the number and location(s) of additional candidate S-SSB occasions, RAN1 further study the followings: - Option 1: Reuse legacy NR SL design, and increase the available values in sl-NumSSB-WithinPeriod for each SCS - Option 2: Each R16/R17 NR SL S-SSB slot has K corresponding additional candidate S-SSB occasion, and the gap between them is (pre-)configured - FFS details, e.g., value of K, details on gap length, etc. - Option 3: The number and location(s) of additional candidate S-SSB occasions are separately (pre-)configured - Option 4: Introduce M contiguous candidate S-SSB occasions in one S-SSB period - Option 5: the number of candidate S-SSB occasions is (pre-)configured, and locations are determined based on the (pre-)configured number

In this matter, it is important to note that

- Option 1 reuses the current legacy SL framework, and very limited specification impact may be required, while it may still be needed to specify in which occasions to transmit conditionally on the LBT procedure.

- Option 2 refers to the case where 4 symbol S-SSB is used, and more than 1 S-SSB transmission could occur in a slot, which as discussed along this document is not preferred.

- Option 3 may require slightly more specification impact than option 1 since an additional (pre-)configured parameter may need to be introduced but may allow to indicate explicitly and more flexibly the additional candidate S-SSB.

- Option 4 and 5 define a new framework where for the first the S-SSB occasions are back-to-back. On top of higher specification impact required for both, Option 4 procedure may lead to very large synchronization delays for 60 kHz SCS, where there would not be sufficient gap between occasions to perform LBT, and therefore procedure would imply that either all occasions are transmitted or all are dropped due to LBT failure before the first one.

With that said, either option 1 or option 3 would be preferable.

Proposal 19:

- Regarding the number and location(s) of additional candidate S-SSB occasions, either Option 1 or Option 3 are preferred.

In prior RAN1 meeting [5], the group identified different options on when a UE would attempt a transmission on an S-SSB (candidate) occasions conditionally to the LBT procedure of the other occasions, and the following agreement was reached:

Agreement Regarding additional candidate S-SSB occasions: - In the same S-SSB period, RAN1 further study the followings: - Alt 1: UE attempts to transmit on all or some of additional candidate S-SSB occasion(s) only when it fails to transmit on R16/R17 S-SSB occasion(s) - Alt 2: UE attempts to transmit on all additional candidate S-SSB occasion(s) regardless of whether or not it transmitted on R16/R17 S-SSB occasion(s) - Alt 3: UE can attempt to transmit on all or some of additional candidate S-SSB occasion(s) regardless of whether or not it transmitted on R16/R17 S-SSB occasion(s) - Alt 4: upon LBT failure on a (candidate) S-SSB occasion, a UE attempts to transmit on the subsequent additional candidate S-SSB occasion if within a period S-SSB transmission has not been transmitted in any prior occasions - FFS details

It is important to note that Alt. 4 mimics the principles of Rel.16 NR-U, and it seems unnecessary to actually transmit on multiple occasions even if LBT has succeeded in a prior one, and transmission was successfully done. In our view, this may be highly detrimental from co-existence perspective and may also lead SL-U design in case the additional S-SSB occasions are not excluded from the resource pool.

Proposal 20:

- Upon LBT failure on a (candidate) S-SSB occasion, a UE attempts to transmit on the subsequent additional candidate S-SSB occasion if within a period S-SSB transmission has not been transmitted in any prior occasions (Alt 4).

# Considerations on SL Slot Design

During prior RAN1 meetings [4-5], the following working assumption and agreement have been made related to the SL slot design:

Working assumption: Support maximum 2 candidate starting symbols within a slot for a PSCCH/PSSCH transmission. - RAN1 strives to have unified design for PSCCH/PSSCH transmission from 1st or 2nd starting symbol - The candidate starting symbol(s) are intended for AGC purpose - FFS: other potential uses of the candidate starting symbol(s) - FFS other details, e.g., applicable scenarios (including SCS), position of 2nd starting symbol, TBS determination, PSCCH blind decoding complexity, processing time constraints, etc. - FFS whether 2 candidate starting symbols is also supported for slots with PSFCH Agreement For slots with 2 candidate starting symbols for a PSCCH/PSSCH transmission: - Regarding the location of 1st starting symbol, down-select one of the followings: - Option 1: it is fixed as symbol#0 - Option 2: it is indicated by sl-StartSymbol as in R16 NR SL - Regarding the location of 2nd starting symbol, down-select one of the followings: - Option A: it is a fixed location - FFS the location, e.g., symbol#4, #7, etc. - Option B: it is a (pre-)configured location per resource pool - FFS the details of candidate locations - Note: assume symbol index in a slot starts from #0

In this matter, RAN1 converged in supporting a maximum of 2 candidate starting symbols within a slot for PSCCH/PSSCH. However, RAN1 has not yet converged on the exact location of these starting positions, together with how the location of a starting symbol should be defined in SL-U in case of a single candidate starting symbol. In this regard, we do not see any need to deviate from the legacy SL design principles and therefore:

- For the case of either 1 or 2 candidate starting symbols for a PSCCH/PSSCH transmission, for location of the 1st starting symbol could be indicated by sl-StartSymbol as in Rel.16 NR SL.

- For the case with 2 candidate starting symbols for a PSCCH/PSSCH transmission, the 2nd starting symbol could be (pre-configured) per resource pool.

Proposal 21:

- For slots with either only 1 or 2 candidate starting symbols for a PSCCH/PSSCH transmission, the location of the 1st starting symbol is indicated by sl-StartSymbol as in Rel. 16 NR SL.

Proposal 22:

- For slots with 2 candidate starting symbols for a PSCCH/PSSCH transmission, the location of the 2nd starting symbol is (pre-)configured per resource pool.

As for the conditions on when 2 candidate starting symbols, different considerations should be drawn based on the deployment and whether an incumbent technology is absent. If the network can guarantee the absence of an incumbent technology, there would not be any impact on the spectrum utilization if all SL transmissions have a single predefined starting position as in Rel.16 NR SL, and this would even more beneficial to mitigate possible mutual blocking across devices. However, when the absence of the incumbent technology cannot be guaranteed, 2 candidate starting symbols would be beneficial.

Proposal 23:

- For semi-static channel access mode, a resource pool only includes slots with one candidate starting symbol.

Proposal 24:

- For dynamic channel access mode, a resource pool always includes slots with two candidate starting symbols, and this applies on whether a single slot or a multi-consecutive slot transmission is performed.

In prior RAN1 meeting [5], the group discussed on how to determine the transport block size (TBS) in case a resource pool includes slots with 2 candidate starting symbols for a PSCCH/PSSCH transmission, and in this case which symbol should be used as a reference, and the following agreement was reached:

Agreement If a resource pool includes slots with 2 candidate starting symbols for a PSCCH/PSSCH transmission: - TBS is determined based on a reference symbol length, down-select one of the followings: - Option 1: The reference symbol length is dynamically indicated by Tx UE - Option 2: The reference symbol length is determined based on 1st starting symbol - Option 3: The reference symbol length is determined based on 2nd starting symbol - Option 4: The reference symbol length is (pre-)configured

In this regard, it is important to note that:

- Option 1 is a viable solution, but this would assume a specific implementation and that UE may need MAC layer to prepare two TBs each of specific symbol lengths or PHY to rate match to additional number of symbols.

- Option 2 may lead to higher code rate than first candidate transmission if the UE will end up transmitting in the second opportunity. However, option 2 does not have the drawback of option 1. In this case, if LBT fails on the first symbol, the UE will simply drop the symbols from first to second starting symbol and transmit in the remaining symbols if LBT succeeds and those symbols could be still decodable.

- Option 3 is highly undesirable since the code rate of first transmission is calculated based on a shorter SL length.

- Option 4 provides an unnecessary flexibility and may inherits the same issues as Option 3.

With that said, option 2 is preferred.

Proposal 25:

- If a resource pool includes slots with 2 candidate starting symbols for a PSCCH/PSSCH transmission, TBS is determined using the 1st starting symbol as a reference symbol length.

Finally, in prior meeting the following agreement was reached regarding the Tx and Rx UE’s behavior. For a slot with 2 candidate starting symbols for a PSCCH/PSSCH transmission

Agreement For a slot with 2 candidate starting symbols for a PSCCH/PSSCH transmission: - Regarding Tx UE behaviour: - If PSCCH/PSSCH transmission starts from 1st starting symbol, down-select one of the followings - Option 1: The PSCCH/PSSCH transmission has 2 symbols for AGC purpose - Option 2: The PSCCH/PSSCH transmission has only 1 symbol for AGC purpose - Option 3: The PSCCH/PSSCH transmission has 1 or 2 symbol(s) for AGC purpose depending on conditions, FFS details - If PSCCH/PSSCH transmission starts from 2nd starting symbol, the PSCCH/PSSCH transmission has only 1 symbol for AGC purpose - Regarding Rx UE behaviour, down-select one of the followings: - Option A: The Rx UE always monitors two AGC symbols in such slot - Option B: The Rx UE monitors two AGC symbols in such slot by default, but could drop monitoring the 2nd AGC symbol at least if it detects a PSCCH/PSSCH transmission starting from the 1st starting symbol - FFS details - Option C: The Rx UE monitors two AGC symbols in such slot by default, but it is up to UE implementation whether to drop monitoring the 2nd AGC symbol - Option D: It is up to UE implementation to monitor 1 or 2 AGC symbol(s) in such slot

For what concerns the Tx UE’s behavior, it seems unnecessary for a UE to transmit two AGC symbols within a PSSCH/PSSCH transmission given that this would additionally lead to a loss of spectral efficiency. Therefore, as in legacy Rel.16 SL design, an AGC symbol could be only the first symbol of a SL transmission regardless of its starting position. However, an Rx UE should monitor the AGC symbol for both the 1st or 2nd starting position and could potentially skip the AGC symbol monitoring of the 2nd starting position in case this may assess that the PSSCH/PSCCH has started from the 1st starting symbol. However, this aspect could be left up to implementation.

Proposal 26:

- For a slot with 2 candidate starting symbols, from a Tx UE point of view regardless of whether the PSCCH/PSSCH transmission starts from 1st or 2nd starting symbol, the PSCCH/PSSCH transmission has only 1 symbol for AGC purpose.

Proposal 27:

- For a slot with 2 candidate starting symbols, from an Rx UE point of view, it is left up to implementation to monitor 1 or 2 AGC symbols in such a slot.

Conclusions

In this contribution, we have provided our views on the physical structure for SL operation in the unlicensed band. In summary, we have the following list of proposals and observations:

Observation 1:

- When 80% OCB must be met, supporting a sub-set of PRBs of one RB set would not meet regulatory requirements, and therefore the benefit in supporting a sub-set of PRBs in one RB set is unclear.

Observation 2:

- There is no technical reason to support both contiguous RB and RB-based interlaced allocation in same resource pool, since either one or the other can be supported due to regional requirements

Proposal 1:

- The sub-channel size should never exceed the LBT BW, i.e., 20 MHz and a resource pool should be configured so that the resource pool is smaller or equal to it.

Observation 3:

- There is no clear technical reason to support different interlace index(s) for PRBs in the intra-cell guard band compared to PRBs for PSSCH transmission.

Proposal 2:

- The use of the PRBs within an intra-cell guard band of two adjacent RB sets is precluded for both S-SSB and PSFCH transmissions.

Proposal 3:

- Any additional S-SSB slots used for additional occasions are excluded from the resource pool configuration.

Proposal 4:

- Similarly, as in Rel.16 SL a bitmap is used to indicate the available time domain resources of a resource pool, where bitmap value of all “1”s is allowed.

Proposal 5:

- A sub-channel is contained within a single RB set.

Proposal 6:

- For contiguous RB-based PSCCH/PSSCH transmission, regarding mapping between sub-channel and PRBs, similarly as Rel. 16 NR SL, the mapping of sub-channel starts from the first PRB of the resource pool and mapped sequentially within the resource pool according to the sub-channel size (option 1).

Observation 4:

- Other candidate frequency domain resource allocation granularities for the interlaced resource block structure may lead to insignificant IBE advantages, while lowering possible multiplexing capabilities and introducing very large specification impact.

Proposal 7:

- K= {5,10} for 15 kHz SCS and K= {2,5} for 30 kHz SCS are supported, where K is the number of interlaces within a single sub-channel.

Proposal 8:

- The interlace index is the same across different RB sets when a UE uses more than one RB set for transmission (Option A).

Proposal 9:

- For interlace RB-based PSCCH/PSSCH transmission, regarding mapping between sub-channel and interlace, one sub-channel is defined and indexed within 1 RB set and is periodically indexed across different RB sets within the resource pool (option 1).

Observation 5:

- Both Alt-3-1a and Alt-3-2a do not meet the OCB requirements by design.

Proposal 10:

- For 15 and 30 kHz subcarrier spacing, RAN1 should down-select both Alt-3-1a and Alt-3-2a.

Proposal 11:

- Regarding PSFCH transmission for 60 kHz subcarrier spacing, no RB-based interlace is introduced, and the OCB requirements are met through contiguous RB allocation.

Proposal 12:

- In SL-U, multiple PSFCH occasions for a PSSCH/PSCCH slot are supported.

Proposal 13:

- The PSFCH resources are (pre-)configured.

Proposal 14:

- Cross-COT PSFCH occasions are not allowed.

Proposal 15:

- A UE in RA mode 1 may report a NACK if

- it does not receive PSFCH due to an LBT failure in any PSFCH reception occasions associated with a PSSSCH transmission in a resource provided by DCI or for a configured grant.

- it is unable to transmit PSSCH due to LBT failure in any of the resource provided by DCI or for a configured grant.

Proposal 16:

- 4 symbol S-SSB is not supported for SL-U

Proposal 17:

- The 2 MHz exemption defined by ETSI BRAN is applied to the legacy S-SSB transmission.

Proposal 18:

- In order to support 15 kHz SCS, PSBCH is mapped so that to span over 12 RBs, and is wrapped around S-PSS and S-SSS.

Proposal 19:

- Regarding the number and location(s) of additional candidate S-SSB occasions, either Option 1 or Option 3 are preferred.

Proposal 20:

- Upon LBT failure on a (candidate) S-SSB occasion, a UE attempts to transmit on the subsequent additional candidate S-SSB occasion if within a period S-SSB transmission has not been transmitted in any prior occasions (Alt 4).

Proposal 21:

- For slots with either only 1 or 2 candidate starting symbols for a PSCCH/PSSCH transmission, the location of the 1st starting symbol is indicated by sl-StartSymbol as in Rel. 16 NR SL.

Proposal 22:

- For slots with 2 candidate starting symbols for a PSCCH/PSSCH transmission, the location of the 2nd starting symbol is (pre-)configured per resource pool.

Proposal 23:

- For semi-static channel access mode, a resource pool only includes slots with one candidate starting symbol.

Proposal 24:

- For dynamic channel access mode, a resource pool always includes slots with two candidate starting symbols, and this applies on whether a single slot or a multi-consecutive slot transmission is performed.

Proposal 25:

- If a resource pool includes slots with 2 candidate starting symbols for a PSCCH/PSSCH transmission, TBS is determined using the 1st starting symbol as a reference symbol length.

Proposal 26:

- For a slot with 2 candidate starting symbols, from a Tx UE point of view regardless of whether the PSCCH/PSSCH transmission starts from 1st or 2nd starting symbol, the PSCCH/PSSCH transmission has only 1 symbol for AGC purpose.

Proposal 27:

- For a slot with 2 candidate starting symbols, from an Rx UE point of view, it is left up to implementation to monitor 1 or 2 AGC symbols in such a slot.

References

- RP-221938, “WID revision: NR sidelink evolution”, OPPO, September 2022.

- RAN1 Chairman’s note, 3GPP TSG RAN WG1 e-Meeting #109-e, May 2022.

- RAN1 Chairman’s note, 3GPP TSG RAN WG1 #110, Toulouse France, August 2022.

- RAN1 Chairman’s note, 3GPP TSG RAN WG1 e-Meeting #110b-e, October 2022.

- RAN1 Chairman’s note, 3GPP TSG RAN WG1 #111, Toulouse France, November 2022.

- R1-2200948, “On the Channel Access Mechanisms for SL Operating in Unlicensed Spectrum”, Intel Corporation, February/March 2023.

- ETSI EN 301 893, “5 GHz RLAN; Harmonised Standard covering the essential requirements of article 3.2 of Directive 2014/53/EU”, ETSI, V2.1.1, May 2017.

- R4-1816666, “Reply LS on NR V2X RF parameters for RAN1 physical layer design aspects”, Spokane, USA, November 2018.

Appendix – RAN1 Agreements

This section captures all the agreements made during the past RAN1 meetings [2-5]:

RAN1 #109-e: Agreement SL BWP, SL resource pool in R16/R17 NR SL and RB set in R16 NR-U are reused for SL-U as baseline - Only one SL BWP is (pre-)configured within a carrier - The SL BWP is (pre-)configured to include one or multiple SL resource pools - At least support that one SL resource pool can be (pre-)configured to include integer number of RB sets - FFS: whether/how to support one SL resource pool can include sub-set of PRBs of one RB set - FFS: the applicable resource pool - FFS: the impact on sub-channel size and number of sub-channels in a resource pool if sub-channel is supported - PRBs within intra-cell guard band of two adjacent RB sets belong to a resource pool if the resource pool includes the two adjacent RB sets - FFS details, e.g., how such PRBs are used, the applicable resource pool, etc. - FFS: whether R16/R17 NR SL S-SSB slots and/or new S-SSB slots (if supported) are excluded from resource pool - FFS: which slots belong to resource pool, e.g., how to set the value of bitmap, whether to consider SL-U/NR-U operating in the same carrier and whether TDD configuration are considered, etc. - FFS: the impact of PSCCH/PSSCH mapping to frequency resources on resource pool configuration, on sub-channel definition if sub-channel is supported, etc. Agreement For PSCCH and PSSCH in SL-U: - Both R16/R17 NR SL contiguous RB-based and R16 NR-U interlace RB-based transmissions are considered as starting point - RAN1 strives to have unified design for both contiguous RB-based and interlace RB-based transmissions - FFS: whether/how to address IBE (In Band Emission) impact Agreement For PSCCH and PSSCH in SL-U: - For interlace RB-based transmission (if supported), at least the following candidates can be discussed: - Frequency domain resource allocation granularity is one sub-channel for PSSCH transmission - FFS: Other resource allocation granularity, e.g., RB-level - 1 sub-channel equals K interlaces if sub-channel is supported - FFS details - Other candidates are not precluded - FFS: mapping of PSCCH to frequency resources - FFS: resource indication in time/frequency domain, e.g., how to handle using one RB set or multiple RB sets, etc. Agreement For slot structure in SL-U: - At least R16/R17 NR SL slot-based PSCCH/PSSCH transmission is supported - FFS: whether/how to support additional starting symbol(s) within a slot for the PSCCH/PSSCH transmission Agreement For PSFCH and SL-HARQ in SL-U: - At least R16 NR SL PSFCH format 0 is supported - FFS whether to introduce new PSFCH format - FFS: how to meet OCB and PSD requirement for PSFCH transmission, e.g., using interlaced RB transmission, whether/how to avoid too small PSFCH capacity, etc. - FFS: the locations of PSFCH resources, e.g., (pre-)configured, dynamically indicated, etc. - FFS: whether/how to address PSFCH transmission dropping due to LBT failure, e.g., whether to have multiple PSFCH occasions for a PSSCH and the related PSSCH-PSFCH mapping relationship, impact on SL HARQ-ACK reporting to the gNB for Mode 1, etc. - FFS: whether/how to address PSFCH and related PSSCH in different COTs Agreement For S-SSB and synchronization in SL-U: - FFS the time domain locations of S-SSB resources, e.g., whether/how to introduce more candidate occasions compared with R16/R17 NR SL design, etc. - Down-selection at least one of the following solutions to meet OCB and PSD requirement for S-SSB transmission - Option 1: Using interlaced RB transmission - Option 2: S-SSB multiplexing with other SL transmissions in the same slot - Option 3: Repetition of S-PSS/S-SSS/PSBCH in frequency domain - Option 4: S-PSS/S-SSS/PSBCH with wider bandwidth - FFS: whether to support 4 symbols S-SSB - Note: 4 symbols S-SSB can be considered with options 1/2/3/4 above - FFS whether the temporary exemption of OCB requirement is applicable for S-SSB transmission - FFS whether any changes to R16/R17 NR SL synchronization procedure RAN1 #110: Agreement For PSCCH and PSSCH in SL-U: - Both R16/R17 NR SL contiguous RB-based and interlace RB-based transmissions similar to R16 NR-U are supported Agreement For PSCCH and PSSCH in SL-U: - For interlace RB-based transmission - Frequency domain resource allocation granularity is one sub-channel for PSSCH transmission - 1 sub-channel equals K interlace - FFS: whether K is fixed as 1 or (pre-)configured - Discuss whether one or both of the following alternatives are supported - Alt 1: 1 sub-channel is confined within 1 RB set - Alt 2: 1 sub-channel spans 1 or multiple RB set(s) belonging to a resource pool Agreement For PSCCH and PSSCH resource indication in time/frequency domain: - For time domain: R16 NR SL TRIV is reused as baseline - For frequency domain: - further study sub-channel indexing and resource indication - FFS: whether any enhancement needed on R16 NR SL TRIV/FRIV if new feature is introduced in SL-U, e.g., multi-slot consecutive transmission Agreement To meet OCB and PSD requirement for PSFCH transmission, at least RB-based interlace is supported at least for 15 kHz and 30 kHz SCS, FFS details. Agreement Regarding PSFCH transmission, at least the followings alternatives can be further studied - Alt 1: each PSFCH transmission occupies a common interlace and zero or one or more dedicated PRB(s) - Alt 2: each PSFCH transmission occupies an interlace, and may or may not further apply code domain enhancement (e.g., OCC, PRB-level cyclic shifts) - Alt 3: each PSFCH transmission occupies some dedicated PRBs and some common PRBs - FFS details of above alternatives Agreement If RAN1 decides that LBT is performed for PSFCH transmission, for the time and frequency domain locations of PSFCH resources, at least the followings alternatives can be further studied - Alt 1: PSFCH resources are (pre-)configured - Alt 2: PSFCH resources are dynamically indicated - Combination of above alternatives are not precluded - FFS details of above alternatives Agreement If RAN1 decides that LBT is performed for S-SSB transmission, in addition to the S-SSB occasions in R16/R17 NR SL design, support additional candidate S-SSB occasions - FFS the number and locations of additional candidate S-SSB occasions - FFS when a UE transmits S-SSB on such additional candidate S-SSB occasions, and the related Rx UE’s behavior Agreement For S-SSB and synchronization in SL-U: - No changes on R16 NR SL S-PSS/S-SSS sequence generation - Continue studying the 4 options from the previous agreement and whether/how temporary exemption of OCB requirement is applicable for S-SSB transmission, e.g., how to meet the minimum of 2 MHz requirement under 15 kHz SCS RAN1 #110b-e: Agreement For interlace RB-based PSCCH/PSSCH transmission in SL-U: - Regarding 1 sub-channel equals K interlace(s) - At least K=1 and K=2 is supported for 15 kHz SCS - At least K=1 is supported for 30 kHz SCS - FFS: details related to multiple RB sets Working assumption: Support maximum 2 candidate starting symbols within a slot for a PSCCH/PSSCH transmission. - RAN1 strives to have unified design for PSCCH/PSSCH transmission from 1st or 2nd starting symbol - The candidate starting symbol(s) are intended for AGC purpose - FFS: other potential uses of the candidate starting symbol(s) - FFS other details, e.g., applicable scenarios (including SCS), position of 2nd starting symbol, TBS determination, PSCCH blind decoding complexity, processing time constraints, etc. - FFS whether 2 candidate starting symbols is also supported for slots with PSFCH Agreement To meet OCB and PSD requirement for S-SSB transmission, down-select between the followings for 15 kHz and 30 kHz SCS: - Option 1: Using interlaced RB transmission for S-PSS/S-SSS/PSBCH - Option 3: Repetition of S-PSS/S-SSS/PSBCH in frequency domain - FFS: whether/how the above options apply to all or subset of channel type of S-PSS/S-SSS/PSBCH - Note: RAN1 further study the relationship between above options and temporary OCB exemption, and the discussion on temporary OCB exemption can continue even if option 1 or option 3 is supported FFS: how to handle 60 kHz SCS (if needed, not limited to option 1 or option 3) Agreement Regarding frequency domain resource indication for interlace RB-based PSSCH transmission: - When more than one RB set is used for transmissions, down-select one of the followings - Option A: Support that the used interlace index(s) in different RB sets are always the same - Option B: Support that the used interlace index(s) in different RB sets can be different - FFS details Agreement Regarding frequency domain resource indication for interlace RB-based PSSCH transmission: - Down-select one of the followings - Option 1: Support explicitly indicating the used sub-channel index(s) and RB set index(s) - Option 2: Support explicitly indicating at least the used sub-channel index(s) - At least RB set index(s) is not explicitly indicated - FFS details Agreement For PSCCH and PSSCH in SL-U: - PSCCH is transmitted within 1 sub-channel - At least support Option 1 below - Option 1: PSCCH locates in the lowest sub-channel of lowest RB set of corresponding PSSCH - Note: the lowest sub-channel may not be entirely contained in the lowest RB set - FFS whether/how to handle the case where UEs supporting different bandwidths can use the same resource pool to communicate with each other, e.g., whether/how to additionally support Option 2 below - Option 2: PSCCH locates in every RB set of corresponding PSSCH - Note: the above options do not imply any restriction on the mapping of sub-channels to PRBs. - FFS other details Agreement Regarding usage of PRBs within intra-cell guard band of two adjacent RB sets: - Such PRBs can be used for PSSCH transmission if and only if a UE can transmit on the respective LBT channels after performing channel access procedure in multi-channel case and the UE uses both of these two RB sets for PSSCH transmission - FFS details, e.g., handling of potential unequal sub-channel size, for interlaced RB based transmission, whether the PRB(s) in the intra-cell guard band have the same interlace index(s) as the PRBs for PSSCH transmission in these two RB sets - Such PRBs are not used for PSCCH transmission - FFS: whether or not such PRBs are used for PSFCH/S-SSB transmission Agreement At least R16/R17 NR SL S-SSB slots are excluded from SL resource pool. - Note: whether or not additional candidate S-SSB occasions are excluded from resource pool will be discussed after the details of additional candidate S-SSB occasions are clearer Agreement At least there is 1 PSFCH occasion per PSCCH/PSSCH transmission, FFS details Agreement To address PSFCH transmission dropping due to LBT failure, the followings are to be studied: - Alt 1: Support more than 1 PSFCH occasion per PSCCH/PSSCH transmission - Alt 2: PSFCH resources are dynamically indicated - Alt 3: Convey SL-HARQ feedback information in PSCCH/PSSCH, e.g., new SCI or new MAC-CE - Alt 4: drop PSFCH transmission - Alt 5: Support trigger based HARQ feedback reporting for non-numerical HARQ FB and one shot HARQ FB - Combination of above alternatives are not precluded - FFS details of above alternatives Agreement Regarding additional candidate S-SSB occasions - Their number and time domain locations are (pre-)configured or pre-defined RAN1 #111: Agreement For slots with 2 candidate starting symbols for a PSCCH/PSSCH transmission: - Regarding the location of 1st starting symbol, down-select one of the followings: - Option 1: it is fixed as symbol#0 - Option 2: it is indicated by sl-StartSymbol as in R16 NR SL - Regarding the location of 2nd starting symbol, down-select one of the followings: - Option A: it is a fixed location - FFS the location, e.g., symbol#4, #7, etc. - Option B: it is a (pre-)configured location per resource pool - FFS the details of candidate locations - Note: assume symbol index in a slot starts from #0 Agreement If a resource pool includes slots with 2 candidate starting symbols for a PSCCH/PSSCH transmission: - TBS is determined based on a reference symbol length, down-select one of the followings: - Option 1: The reference symbol length is dynamically indicated by Tx UE - Option 2: The reference symbol length is determined based on 1st starting symbol - Option 3: The reference symbol length is determined based on 2nd starting symbol - Option 4: The reference symbol length is (pre-)configured Agreement Regarding PSFCH transmission under 15 kHz and 30 kHz SCS, RAN1 continues studying the following updated alternatives: - Alt 1-1a: each PSFCH transmission occupies 1 common interlace and K3 dedicated PRB(s) - FFS: value of K3 - Alt 2-1a: each PSFCH transmission occupies 1 interlace, and further apply frequency-domain OCC - FFS: details of FD-OCC, e.g., OCC length, RB-level, RE-level, etc. - Alt 2-2a: each PSFCH transmission occupies 1 interlace, and further apply PRB-level cyclic shift - A UE transmits dedicated cyclic shift on K1 dedicated PRB(s) within this interlace, and transmits common cyclic shift on other PRBs of this interlace - FFS: value of K1 - Alt 2-3a: each PSFCH transmission occupies 1 interlace - Alt 2-4a: each PSFCH transmission occupies 1 interlace, and further apply PRB-level cyclic shift - A UE uses different cyclic shifts on different PRBs in the interlace - Alt 3-1a: each PSFCH transmission occupies 1 dedicated PRB and K2 common PRBs, where K2 common PRBs locate at the two edges of a RB set - The above dedicated PRB and common PRBs are within 1 interlace - FFS: value of K2 - Alt 3-2a: each PSFCH transmission occupies 1 dedicated PRB and 2 common PRBs, where 2 common PRBs locate at the two edges of a RB set - FFS: the impact of PSD limit, e.g., whether/how to handle the case when common PRB and dedicated PRB locate within the same 1 MHz bandwidth - FFS: whether IBE issue exists and whether/how to address it - Note: in the above descriptions - The dedicated PRB/cyclic shift conveys ACK/NACK information - Note: as previously agreed: to meet OCB and PSD requirement for PSFCH transmission, at least RB-based interlace is supported at least for 15 kHz and 30 kHz SCS. Agreement Slots with PSFCH symbols only have 1 candidate starting symbol for PSCCH/PSSCH. Agreement For interlace RB-based PSCCH/PSSCH transmission in SL-U: - Regarding mapping between sub-channel and interlace, RAN1 further study the followings: - Option 1: 1 sub-channel is defined and indexed within 1 RB set, and is periodically indexed across different RB sets within the resource pool - Option 2: 1 sub-channel is defined within 1 RB set, and is incrementally indexed firstly within an RB set, then across different RB sets within the resource pool - Option 3: 1 sub-channel is defined across all RB sets within the resource pool, i.e., 1 sub-channel includes K interlace(s) across all RB sets within the resource pool - Option 4: 1 sub-channel is defined within 1 RB set or 2 adjacent RB sets, and is incrementally indexed firstly within an RB set, then across different RB sets within the resource pool - Option 5: 1 sub-channel is defined within 1 RB set, and is incrementally indexed firstly across different RB sets within the resource pool, then across different interlaces in the RB set - FFS: whether/how to use intra-cell guardband PRBs Agreement For S-SSB transmission, down-select one or more of the following for 15 kHz and 30 kHz SCS: - Option 1-1: Using interlaced RB transmission for all of S-PSS/S-SSS/PSBCH - Option 1-2: Using interlaced RB transmission for PSBCH only, and apply OCB exemption to S-PSS and S-SSS - Option 3-1: Repeat S-PSS/S-SSS/PSBCH N times in frequency domain, and there is a gap between the repetition(s) to meet OCB requirement - FFS details, e.g., the length of gap is (pre-)configured or pre-defined, value of N (e.g., N=2) - FFS gap of 0 - Option 3-2: Repeat only S-PSS/S-SSS K times in frequency domain, and PSBCH is rate matched. There is a gap between the repetition(s) to meet OCB requirement - FFS details, e.g., the length of gap is (pre-)configured or pre-defined, value of K - FFS gap of 0 - FFS PSBCH resource - Option 3-3: keep the legacy S-PSS/S-SSS/PSBCH while repeating PSBCH N times in frequency domain and rate-matching PSBCH to S-PSS/S-SSS symbols, and there is a gap between the PSBCH repetition(s) to meet OCB requirements - FFS details, e.g. the length of gap is (pre-)configured or pre-defined, value of N - Option A: Apply OCB exemption to all of S-PSS/S-SSS/PSBCH - For Option 1-1 and 1-2 above - FFS: whether/how to handle the case when each interlace has only 10 PRBs in a RB set - FFS: whether transient period issue exists and whether/how to address it Agreement For a slot with 2 candidate starting symbols for a PSCCH/PSSCH transmission: - Regarding Tx UE behaviour: - If PSCCH/PSSCH transmission starts from 1st starting symbol, down-select one of the followings - Option 1: The PSCCH/PSSCH transmission has 2 symbols for AGC purpose - Option 2: The PSCCH/PSSCH transmission has only 1 symbol for AGC purpose - Option 3: The PSCCH/PSSCH transmission has 1 or 2 symbol(s) for AGC purpose depending on conditions, FFS details - If PSCCH/PSSCH transmission starts from 2nd starting symbol, the PSCCH/PSSCH transmission has only 1 symbol for AGC purpose - Regarding Rx UE behaviour, down-select one of the followings: - Option A: The Rx UE always monitors two AGC symbols in such slot - Option B: The Rx UE monitors two AGC symbols in such slot by default, but could drop monitoring the 2nd AGC symbol at least if it detects a PSCCH/PSSCH transmission starting from the 1st starting symbol - FFS details - Option C: The Rx UE monitors two AGC symbols in such slot by default, but it is up to UE implementation whether to drop monitoring the 2nd AGC symbol - Option D: It is up to UE implementation to monitor 1 or 2 AGC symbol(s) in such slot Agreement To address PSFCH transmission dropping due to LBT failure, RAN1 down-select one of followings, or support the combination of followings: - Alt 1: Support more than 1 PSFCH occasion per PSCCH/PSSCH transmission - FFS other details, e.g., HARQ-ACK timeline - Alt 2: PSFCH occasions are dynamically indicated - FFS: Whether/how to handle the case where some TB’s corresponding PSFCH cannot be transmitted within the same or different COT - FFS other details, e.g., dynamically indicate one or more PSFCH transmission(s), container of the indication, etc. - FFS: Whether such PSFCH occasions are within the same or different COT of corresponding PSSCH - FFS: Whether/how to address PSFCH collision if any - FFS: Whether/how to handle the linearly decreased PSFCH capacity Agreement Regarding the number and location(s) of additional candidate S-SSB occasions, RAN1 further study the followings: - Option 1: Reuse legacy NR SL design, and increase the available values in sl-NumSSB-WithinPeriod for each SCS - Option 2: Each R16/R17 NR SL S-SSB slot has K corresponding additional candidate S-SSB occasion, and the gap between them is (pre-)configured - FFS details, e.g., value of K, details on gap length, etc. - Option 3: The number and location(s) of additional candidate S-SSB occasions are separately (pre-)configured - Option 4: Introduce M contiguous candidate S-SSB occasions in one S-SSB period - Option 5: the number of candidate S-SSB occasions is (pre-)configured, and locations are determined based on the (pre-)configured number Agreement Regarding additional candidate S-SSB occasions: - In the same S-SSB period, RAN1 further study the followings: - Alt 1: UE attempts to transmit on all or some of additional candidate S-SSB occasion(s) only when it fails to transmit on R16/R17 S-SSB occasion(s) - Alt 2: UE attempts to transmit on all additional candidate S-SSB occasion(s) regardless of whether or not it transmitted on R16/R17 S-SSB occasion(s) - Alt 3: UE can attempt to transmit on all or some of additional candidate S-SSB occasion(s) regardless of whether or not it transmitted on R16/R17 S-SSB occasion(s) - Alt 4: upon LBT failure on a (candidate) S-SSB occasion, a UE attempts to transmit on the subsequent additional candidate S-SSB occasion if within a period S-SSB transmission has not been transmitted in any prior occasions - FFS details Agreement For contiguous RB-based PSCCH/PSSCH transmission in SL-U: - Regarding mapping between sub-channel and PRBs, further study the following options: - Option 1 (sub-channel aligns with resource pool boundary): Same as in legacy NR SL, i.e., the mapping of sub-channel starts from the first PRB of the resource pool and mapped sequentially within the resource pool according to the sub-channel size - FFS: whether/how to use sub-channel(s) which include intra-cell guardband PRBs - FFS: whether/how to handle the case when the number of PRBs of the resource pool cannot be divided by sub-channel size - Option 2 (sub-channel aligns with RB set boundary): In each RB set, the mapping of sub-channel starts from the first PRB of the RB set and mapped sequentially within the RB set according to the sub-channel size - FFS: whether/how to use intra-cell guardband PRBs - FFS: whether/how to handle the case when the number of PRBs of one RB set cannot be divided by sub-channel size - Option 3 (sub-channel aligns with RB set boundary): In each RB set, the mapping of sub-channel starts from the first PRB of the RB set and mapped sequentially within the RB set and/or guardband PRB according to the sub-channel size - FFS: how to use intra-cell guardband PRBs - FFS: how to use the subchannel including PRBs in guardband Agreement Regarding PSFCH transmission under 60 kHz SCS, further study the following alternatives: - Alt 1: Each PSFCH transmission occupies K dedicated PRB(s) and some common PRBs - FFS details - Alt 2: Each PSFCH transmission occupies some dedicated PRBs - FFS details Agreement Regarding S-SSB, RAN1 further study the following: - How to transmit S-SSB when a SL BWP contains multiple RB sets