3GPP TSG RAN WG1 #112			     R1-2300950

Athens, Greece, February 27th – March 3rd, 2023

Source:	Intel Corporation

Title:	On the Co-channel Coexistence between LTE and NR Sidelink

Agenda item:	9.4.2

Document for:	Discussion and Decision

Introduction

In 3GPP TSG RAN meeting #94, a new work item related to NR Sidelink (SL) evolution was approved. The work item description was updated during RAN#97 [1]. As part of the objectives of this working item (WI), that following aspects were included:

- Study and specify, if necessary, mechanism(s) for co-channel coexistence for LTE sidelink and NR sidelink including performance, necessity, feasibility, and potential specification impact if any [RAN1, RAN2, RAN4] - Reuse the in-device coexistence framework defined in Rel-16 as much as possible - Note, RAN1 continues the work on dynamic resource pool sharing based on existing agreements and WID with high priority for Type A devices and operating combination A - Rel-18 sidelink should be able to coexist with Rel-16/17 sidelink in the same resource pool. This does not preclude the possibility of operating Rel-18 sidelink in a dedicated resource pool.

In the context of co-existence between NR SL and LTE SL, during the past RAN1 meetings [2-5] the agreements made are captured in the Annex A section. Furthermore, during RAN#97, it was agreed to prioritize discussions on dynamic resource pool sharing assuming Type A devices and operating combination A as per agreements.

With that said, in this contribution, the aspects related to potential mechanisms for coexistence of LTE V2X and NR V2X in co-channel scenarios are discussed, while our view related to other Rel.18 SL aspects are provided in our companion contributions [5-7].

# Discussion

While V2X market penetration may increase over time, both LTE V2X and NR V2X devices may need to well co-exist in co-channel scenarios. However, given that the available spectrum for V2X is quite sparse (especially in some regions), the impact that co-existence may have on both technologies may be quite detrimental and may worsen as market penetration may increase. In this sense, while 3GPP is working on introducing features such as carrier aggregation (CA), sidelink (SL) over unlicensed band and support of SL in FR2 band, which will enable higher data rate and support of larger bandwidth for non-ITS bands, for ITS band, where LTE V2X will be likely prioritized to enable basic safety V2X use cases in a relatively short term, enablement of co-channel co-existence mechanisms are necessary to minimize the mutual impact that these technologies may have on each other.

## PSFCH Issue

One of the important issues to be resolved by RAN1 for LTE-NR co-existence is to decide how to avoid impact of PSFCH transmissions in NR on PSCCH/PSSCH transmissions in LTE, especially given that PSFCH transmission may not be easily detected with the RSRP and RSSI mechanisms available in LTE. Furthermore, the transmission of the PSFCH only utilizes the last symbols of a subframe/slot, thus introducing AGC mismatch and channel estimation mismatch issues.

The following mechanisms, at high level, are being discussed in RAN1 to avoid PSFCH impact on LTE:

- Solution direction 1: Transmitting UE of PSCCH/PSSCH selects resources which do not lead to overlap of corresponding PSFCH with LTE transmissions;

- Solution direction 2: Receiving UE of PSCCH/PSSCH decides whether to transmit PSFCH if it would overlap with LTE transmission;

- Solution direction 3: Transmitting LTE UEs may account for PSFCH transmissions using RSSI mechanism, with help of PSFCH on NR being transmitted with periodicity which is an integer divider of 100 ms (RSSI averaging period).

In our view, a combination of directions 1 and 2 are naturally possible within current NR framework, and the trade-off between NR and LTE performance can be controlled by soft metrics such as RSRP and associated priority.

For both cases, when TX or RX UEs assess overlap of PSFCH with LTE reserved resources, RSRP threshold criterion depending on priority levels need to be used, i.e., a PSFCH resource from TX and/or RX UE perspective may be allowed to be used if LTE sidelink RSRP (with necessary conversion/adjustment as discussed in the next section) is smaller than a (pre-)configured RSRP threshold, which is a function of both priority of the PSFCH and priority of the reserved LTE resource. For TX and RX different RSRP thresholds may be used.

For the RX side solution, it should be stressed than when NACK-only option is used for PSFCH transmission, the UE should not drop the PSFCH when overlap criteria with LTE is met, so that DTX-to-ACK error is minimized for NACK-only cases, which it is crucial for system performance.

In order to minimize LTE to PSFCH collisions semi-statically, periodicity of PSFCH resources may be further increased, e.g., up to 8.

Proposal 1:

- When a UE is selecting resources for PSSCH transmission with enabled HARQ feedback, a lower priority can be given to PSSCH resources which lead to overlap of the corresponding PSFCH for HARQ feedback with reserved LTE transmission meeting RSRP overlapping criteria,

  - Separate RSRP threshold per pair of priorities is (pre-)configured for this case.

- When a UE is preparing PSFCH transmission in response to PSSCH, additional prioritization can be applied whether to transmit the PSFCH in the slot if it would overlap with reserved LTE transmission meeting overlapping criteria,

  - If the PSFCH is carrying a NACK-only HARQ feedback, it is not subject to this additional prioritization rule,

  - Separate RSRP threshold per pair of priorities is (pre-)configured for this case.

- The following PSFCH periodicities are additionally introduced to existing 1,2,4 logical slots: 8 and [10].

## Use of Information Shared by LTE SL Module

In the last meeting, further decision was made to utilize LTE sensing information at NR PHY layer, assuming it is integrated into resource exclusion procedures described in TS 38.214, Section 8.1.4.

Agreement For dynamic resource pool sharing, the NR SL module uses the candidate information shared by the LTE SL module to the NR SL module, where - The NR SL module excludes resources based on the shared information from its own candidate resource set when performing the resource (re)selection procedure in the PHY layer. - FFS how to exclude resources at least based on the time and frequency locations of LTE SL transmissions that have been indicated in the shared candidate information. - FFS how the exclusion is performed according to clause 8.1.4 of TS 38.214. - FFS: whether/how NR SL module excludes resources not belonging to the generated LTE SL’s candidate resource set SB from its own candidate resource set.

There are several types of information that can be included into the NR procedures:

- LTE sensing information

- LTE non-monitored subframes

- Own LTE transmission subframes and parameters

In our understanding, the first two types of information directly impact LTE system performance in a co-existence scenario. The last information may be useful to optimize in-device NR and LTE co-existence, however in our understanding it is already being handled by in-device co-existence framework introduced in Rel-16 and may be out of scope of Rel-18 co-channel co-existence between LTE and NR.

Proposal 2:

- Two types of information are shared from LTE module to NR module and used in NR sensing and resource selection:

  - Sensing measurements and SCIs received from other LTE devices,

  - Slots not monitored by the LTE module within the sensing window.

For the first type of sensing information, it needs to be discussed how to utilize the LTE SCIs and measurements in the NR framework. For that purpose, it is proposed to introduce an intermediate conversion step between LTE sensing information and NR sensing information.

Figure 1 - Illustration of an intermediate step of LTE info into NR info conversion

There are multiple aspects that need to be considered for such conversion:

- Frequency allocation conversion

  - To adapt LTE frequency allocation to NR frequency allocation we are also considering that resource pool configuration, timing, bandwidth, SCS and other parameters are not necessarily aligned between the two RATs. The NR module needs to search for all NR single-slot sub-channel resources which overlap with a given LTE reserved resource.

  - LTE PSCCH and LTE PSSCH are multiplexed in frequency domain, have different PSD and may potentially be adjacent or non-adjacent. Considering this, a singe PSCCH+PSSCH transmission may be split onto two transmissions before performing the search of the overlapping NR sub-channels.

  - Multiple NR reservations may in this case describe one or multiple LTE reservations.

- RSRP conversion

  - Conversion of LTE RSRP need to consider the following aspects:

    - LTE sidelink RSRP is measured on DMRS of PSCCH, and PSCCH has a 3 dB PSD boost comparing to PSSCH. It needs to be considered when converted to NR-like measurement.

    - RSRP is an average over REs, thus it would need to consider actual overlapping portion of NR sub-channel and LTE reserved resource.

RSRPVirtual,NR=RSRPLTEBOverlap,LTEBNR

      - BOverlap,LTE is the overlapping bandwidth of LTE and NR within the considered bandwidth of NR BNR in Hz

- Time reservation conversion

  - Converted time domain resources need to consider whether one or multiple NR SCIs can describe a given resource LTE resource within a period or for the next period as well

  - LTE supports ‘backward’ indication, i.e., indication in past. This can be handled by the converted NR reservation as well.

- RSSI measurement conversion

  - One specific feature which is not present in NR but is present in LTE, is the usage of RSSI measurement to rank resources which do not have associated RSRP measurements, i.e., reserving SCIs received.

  - To support it in NR, a given single-subframe LTE resource which has an associated sidelink RSSI measurement may be converted to a NR SCI with a default priority, default periodicity and an associated RSRP measurement, which is a function of RSSI measurement.

    - RSSI to RSRP conversion need to consider that RSSI captures also interference and noise, and is defined per symbol, not per RE. If it can be allowed for a UE to convert RSSI to RSRP considering appropriate scaling, it may capture the effect of RSSI to a large extent.

From the above discussion, the following is proposed:

Proposal 3:

- The sensing measurements and SCIs received from other LTE devices are converted to NR SCIs and associated measurements first, and then considered in NR resource selection same way as NR SCIs and measurements. The conversion of LTE sensing information to NR sensing information follows the following principles:

  - PSCCH and PSSCH portions of LTE reservation are split into two different LTE SCIs, with respective RSRP recalculation given that PSCCH PSD is boosted by 3 dB, for both adjacent and non-adjacent resource pool cases.

  - A given set of sub-channels LLTE={l0LTE,l1LTE,…lS-1LTE} in one subframe of LTE reserved by a LTE SCI may be converted by a NR module to a set of NR sub-channels LNR={l0NR,l1NR,…lZ-1NR} by searching all NR sub-channels in a given logical slot in a given resource pool which overlap in time and frequency both fully and partially with any of the LTE sub-channels LLTE.

  - The set of overlapping NR sub-channels is associated with one or multiple NR SCIs so that each overlapping sub-channel is contained in only one NR SCI, and NR SCI signaling constraints are maintained.

  - The RSRP is calculated from LTE RSRP by taking into account the overlapping bandwidth between NR sub-channel and actual LTE reserved resource.

  - LTE sub-channels with RSSI measurement are converted to NR SCIs with a default priority, default periodicity, and with RSSI measurement converted to approximate RSRP measurement considering sub-channel bandwidth.

Also, it can be considered to also exclude non-monitored LTE subframes in the LTE sensing window:

Proposal 4:

- The subframes not monitored on LTE sensing window are converted to NR slots not monitored in the sensing window and are considered same way as the NR slots not monitored in the sensing window.

## Timeline of Information Shared by LTE SL Module

The following was agreed with respect to timeline for sharing LTE information with NR SL module:

Agreement For dynamic resource pool sharing, the NR SL module is expected to use the information shared by the LTE SL module to the NR SL module which is known by NR SL module at the latest T ms prior to slot n (as defined in clause 8.1.4 of TS 38.214), to determine a set of resources for its own (re)transmission. - T is defined using - T≤Tmax ms, and is based on UE implementation, according to the Rel-16 NR SL timeline for in-device coexistence. - FFS: Value of Tmax - FFS: any discussion on the earliest information, if needed Agreement Based on the agreement in RAN1#110bis-e, the value of Tmax = 4 ms.

By carefully reading the agreement, it appears that Tmax is an equivalent of a processing delay of LTE shared information. However it does not capture whether the LTE shared information is as recent as T or Tmax. Leaving it up to UE implementation, similar to Rel.16 IDC short-term solutions, does not work since it is now not only internal to a UE, but affects the system performance and fairness of the distributed sidelink communication.

To fix this issue, additional deadline is needed. In our understanding, it may be described in the following manner using additional ‘Tshare’ timing:

- The information shared by the LTE SL module corresponds to the sensing information that would be used by the LTE SL module for resource selection in moment n – Tshare. The value of Tshare is implementation specific, but the smaller the value, the better the system performance. It may be discussed as part of capability or fixed in the specification. However, at least Tshare equal to Tmax (=4ms) may be considered as one candidate.

Observation 1:

- The agreements on T and Tmax introduce an equivalent of a processing delay of LTE shared information to consider in the NR resource selection once this information is available in NR module, however, do not capture any age or relevance of the LTE shared information in moment n-T.

Proposal 5:

- The information shared by the LTE SL module corresponds to the sensing information that would be used by the LTE SL module for resource selection not earlier than in moment n - Tshare, where Tshare is FFS and may include Tshare = Tmax.

## Higher SCS

When LTE and NR employ different sub-carrier spacing in bandwidth part, the duration and granularity of the transmission can be shorter for NR. If NR transmissions can occur at any slot, this may create so-called AGC issue, where the gain calculated at the beginning of an LTE subframe may not be valid in the other parts of the subframe.

For the case of LTE operating at 15 kHz and NR 30 kHz SCS, the following cases are considered (w/o PSFCH):

- Case 1: For a given LTE subframe from a given LTE receiving UE perspective, the total energy of LTE and NR transmission sources in the first half of the subframe is larger than in the second half of the subframe by Δ dB

  - this case is operating, since the AGC setting does not lead to clipping / saturation at the second half of the subframe.

- Case 2: For a given LTE subframe from a given LTE receiving UE perspective, the total energy of LTE and NR transmission sources in the first half of the subframe is smaller than in the second half of the subframe by Δ dB

  - this case is not operating, since the AGC setting may lead to clipping / saturation at the second half of the subframe.

- Case 3: For a given LTE subframe from a given LTE receiving UE perspective, the difference in the total energy of LTE and NR transmission sources between the first half of the subframe and the second half of the subframe is smaller than Δ dB

  - this case is operating, since the energy change is within ADC backoff / Δ dB

Figure 2 - Illustration of an intermediate step of LTE info into NR info conversion

Another issue is the channel estimation at LTE receiver. An existing UE implementation, which does not know about potential presence of NR transmissions, may employ 2D estimation over multiple DMRS symbols, which are in different parts of the LTE subframe, and which may experience different channel, interference, and noise because of the time-selective interference.

To avoid these issues, the most important solution is to minimize situations where NR transmission from a given source appears in some symbols overlapping with LTE subframe reception, but does not appear in the first symbol of the LTE subframe.

Observation 2:

- Usage of NR SCS higher than LTE introduces two issues at LTE receivers:

  - AGC issue, which is pronounced in case the total energy at the initial LTE symbol of the subframe is substantially smaller than the total energy at the later parts of LTE subframe that lead to ADC clipping,

  - Channel estimation issue, where a legacy LTE UE may not assume changes in sources and levels of interference throughout the LTE subframe during the demodulation.

- The solutions of NR and LTE co-existence with different SCS should target minimizing cases when strong NR transmission source is not present in the first symbol of LTE subframe used for LTE sidelink and is present in any other part of the LTE subframe.

Considering the above, it is preferred to focus on NR resource selection procedures which allow a UE to select resources so that within an LTE subframe it also occupies the resource overlapping with the first symbol. At the same time, forcing a UE in this case to artificially prolong NR transmission throughout the whole LTE subframe is detrimental for both LTE and NR performance due to increased half-duplex and interference. It may be enough to introduce additional resource selection rules / criteria which instruct UE to pick at leas the first resource in the group of resources overlapping with an LTE subframe.

Proposal 6:

- In case of higher SCS of NR than LTE, when a NR UE is selecting resources for PSSCH transmission that would overlap with LTE subframes used for LTE sidelink transmission, the following selection rules are applied:

  - The selected resources shall include the resource which overlaps with the AGC symbol of LTE subframe.

  - A UE is not required to select more resources for transmission than it is indicated by higher layers (i.e., there is no repetition of NR transmission within LTE subframe only for the purpose of keeping the energy within the LTE subframe).

Conclusions

In this contribution, we discussed several aspects related to coexistence between NR SL and LTE SL, and made the following proposals and observations:

Proposal 1:

- When a UE is selecting resources for PSSCH transmission with enabled HARQ feedback, a lower priority can be given to PSSCH resources which lead to overlap of the corresponding PSFCH for HARQ feedback with reserved LTE transmission meeting RSRP overlapping criteria,

  - Separate RSRP threshold per pair of priorities is (pre-)configured for this case.

- When a UE is preparing PSFCH transmission in response to PSSCH, additional prioritization can be applied whether to transmit the PSFCH in the slot if it would overlap with reserved LTE transmission meeting overlapping criteria,

  - If the PSFCH is carrying a NACK-only HARQ feedback, it is not subject to this additional prioritization rule,

  - Separate RSRP threshold per pair of priorities is (pre-)configured for this case.

- The following PSFCH periodicities are additionally introduced to existing 1,2,4 logical slots: 8 and [10].

Proposal 2:

- Two types of information are shared from LTE module to NR module and used in NR sensing and resource selection:

  - Sensing measurements and SCIs received from other LTE devices,

  - Slots not monitored by the LTE module within the sensing window.

Proposal 3:

- The sensing measurements and SCIs received from other LTE devices are converted to NR SCIs and associated measurements first, and then considered in NR resource selection same way as NR SCIs and measurements. The conversion of LTE sensing information to NR sensing information follows the following principles:

  - PSCCH and PSSCH portions of LTE reservation are split into two different LTE SCIs, with respective RSRP recalculation given that PSCCH PSD is boosted by 3 dB, for both adjacent and non-adjacent resource pool cases.

  - A given set of sub-channels LLTE={l0LTE,l1LTE,…lS-1LTE} in one subframe of LTE reserved by a LTE SCI may be converted by a NR module to a set of NR sub-channels LNR={l0NR,l1NR,…lZ-1NR} by searching all NR sub-channels in a given logical slot in a given resource pool which overlap in time and frequency both fully and partially with any of the LTE sub-channels LLTE.

  - The set of overlapping NR sub-channels is associated with one or multiple NR SCIs so that each overlapping sub-channel is contained in only one NR SCI, and NR SCI signaling constraints are maintained.

  - The RSRP is calculated from LTE RSRP by taking into account the overlapping bandwidth between NR sub-channel and actual LTE reserved resource.

  - LTE sub-channels with RSSI measurement are converted to NR SCIs with a default priority, default periodicity, and with RSSI measurement converted to approximate RSRP measurement considering sub-channel bandwidth.

Proposal 4:

- The subframes not monitored on LTE sensing window are converted to NR slots not monitored in the sensing window and are considered same way as the NR slots not monitored in the sensing window.

Observation 1:

- The agreements on T and Tmax introduce an equivalent of a processing delay of LTE shared information to consider in the NR resource selection once this information is available in NR module, however, do not capture any age or relevance of the LTE shared information in moment n-T.

Proposal 5:

- The information shared by the LTE SL module corresponds to the sensing information that would be used by the LTE SL module for resource selection not earlier than in moment n - Tshare, where Tshare is FFS and may include Tshare = Tmax.

Observation 2:

- Usage of NR SCS higher than LTE introduces two issues at LTE receivers:

  - AGC issue, which is pronounced in case the total energy at the initial LTE symbol of the subframe is substantially smaller than the total energy at the later parts of LTE subframe that lead to ADC clipping,

  - Channel estimation issue, where a legacy LTE UE may not assume changes in sources and levels of interference throughout the LTE subframe during the demodulation.

- The solutions of NR and LTE co-existence with different SCS should target minimizing cases when strong NR transmission source is not present in the first symbol of LTE subframe used for LTE sidelink and is present in any other part of the LTE subframe.

Proposal 6:

- In case of higher SCS of NR than LTE, when a NR UE is selecting resources for PSSCH transmission that would overlap with LTE subframes used for LTE sidelink transmission, the following selection rules are applied:

  - The selected resources shall include the resource which overlaps with the AGC symbol of LTE subframe.

  - A UE is not required to select more resources for transmission than it is indicated by higher layers (i.e., there is no repetition of NR transmission within LTE subframe only for the purpose of keeping the energy within the LTE subframe).

References

- RP-221798, “WID revision: NR sidelink evolution”, OPPO, June 2022.

- RAN1 Chairman’s note, 3GPP TSG RAN WG1 e-Meeting #109-e, May 2022.

- RAN1 Chairman’s note, 3GPP TSG RAN WG1 #110, Toulouse France, Aug 2022.

- RAN1 Chairman’s note, 3GPP TSG RAN WG1 e-Meeting #110b-e, October 2022.

- RAN1 Chairman’s note, 3GPP TSG RAN WG1 e-Meeting #111, Toulouse France, November 2022.

- R1-2211400, “On the Channel Access Mechanisms for SL Operating in Unlicensed Spectrum”, Intel Cooperation, Toulouse, France, February/March 2023.

- R1-2300949, “On the Physical Layer Enhancements for SL Operating in Unlicensed Spectrum “, Intel Corporation, February/March 2023.

- R1-2300951, “On Sidelink Operation in FR2 Licensed Spectrum”, Intel Cooperation, February/March 2023.

- Annex A – List of Agreements

RAN1 #109e:

Agreement

For co-channel coexistence in Rel-18, no changes in the LTE SL specifications are allowed.

Agreement

For co-channel coexistence in Rel-18, Rel-16/17 simulation assumptions are reused for evaluation of solutions, except for the UE dropping model.

- FFS: UE dropping model

Agreement

For the study of co-channel coexistence solutions in Rel-18, the combination of operational modes Mode 2 NR SL with Mode 4 LTE SL (Combination A) is considered with high priority.

- FFS: Whether/how to support Mode 1 NR SL + Mode 4 LTE SL (Combination B) and/or Mode 2 NR SL + Mode 3 LTE SL (Combination C).

Agreement

For evaluation of co-channel coexistence solutions in Rel-18, support the inclusion of dual module devices with NR+LTE modules using the following UE dropping models:

- UE Dropping Model A: The distance between 1 LTE SL module and 1 NR SL module are maintained as zero to model a co-located dual module device. The inter-device distance between any two adjacent devices in the same lane, which may be either a single module or a dual module device, is modified by doubling the time in the upper limit, resulting in max{2 meter, an exponential random variable with the average of the speed * 4sec}.

- UE Dropping Model B: The distance between 1 LTE SL module and 1 NR SL module are maintained as zero to model a co-located dual module device. The inter-device distance between any two adjacent devices in the same lane, which may be either a single module or a dual module device, is maintained the same as current assumptions, i.e., max{2 meter, an exponential random variable with the average of the speed * 2sec}.

Companies should mention the UE dropping model and the distribution of each device type (single/dual module) used in their simulation assumptions.

Agreement

Feasibility of semi-static resource pool partitioning and dynamic resource sharing as possible solutions for co-channel coexistence are to be studied.

Agreement

For studying the feasibility of dynamic resource sharing as a possible solution for co-channel coexistence,

- For device type A, the NR SL module uses the sensing and resource reservation information shared by the LTE SL module.

  - FFS details on how the NR SL module uses this information.

  - FFS details on how the LTE SL module shares the information to the NR SL module, exact information shared, timeline etc.

- FFS: Whether/how to define other method(s) for device type A to be aware of resources being occupied by LTE SL.

- FFS: Whether/how device type B should be supported.

RAN1 #110:

Working assumption

Co-channel coexistence between LTE SL and NR SL is supported for device type A. Device type A contains both LTE SL and NR SL modules. For device type A, the NR SL module may use the sensing and resource reservation information shared by the LTE SL module.

Conclusion

For co-channel coexistence in Rel-18, RAN1 concludes that the TDM-based semi-static resource pool partitioning based on Rel-16/17 specifications is one possible solution to ensure co-channel coexistence between LTE-V UEs and NR-V UEs.

  - Note: The LTE and NR resource pools do not overlap in time with each other in the TDM-based semi-static resource pool partitioning.

  - Note 2: Rel-16 in-device coexistence framework can ensure alignment between the slot boundary of the NR SL time slot and the subframe boundary of the LTE SL subframe

  - FFS: potential enhancements for synchronization can be further investigated

Agreement

For co-channel coexistence in Rel-18, dynamic resource pool sharing is studied, with the following constraints:

  - NR SL resource pool is configured with 15 kHz SCS.

    - FFS support of NR SL resource pool configured with higher SCS, including other solutions to overcome the AGC issue caused by the differing SCSs between the NR SL and LTE SL resource pools

  - For NR PSFCH (if configured), at least the following alternatives are studied:

    - Alt 1: Avoid PSFCH transmission in time slots that overlap with subframes used for LTE SL transmissions.

      - FFS: Avoiding PSFCH transmissions can be performed by the UE transmitting PSFCH and/or the UE transmitting PSSCH.

    - Alt 2: NR SL UEs use a periodically repeating set of PSFCH slots.

      - FFS: periodicities of the set.

RAN1 #110b-e:

Agreement

For dynamic resource pool sharing, the candidate information shared by the LTE SL module to the NR SL module may include one or more of the following parameters, to be down-selected:

- Time and frequency locations of reserved resources by other LTE UEs, determined based on decoded SCIs

- SL RSRP measurement results

- Resource reservation periods based on decoded SCI and for own LTE SL transmissions

- Priority based on decoded SCI and for own LTE SL transmissions

- Time and frequency location of resources used for own LTE SL transmissions

- Candidate resource set SA or SB

- SL RSSI measurements

- LTE logical subframe related information

- Resources corresponding to half-duplex subframes which are not monitored by the LTE SL UE

Agreement

For dynamic resource pool sharing, the NR SL module uses the information shared by the LTE SL module to the NR SL module to determine the set of resources for its own transmission.

- FFS: which layer carries out the resource determination: PHY layer or MAC layer.

Agreement

For dynamic resource pool sharing, where the NR SL module uses the candidate information shared by the LTE SL module to the NR SL module, continue studying the following alternatives:

- Alt 1: The LTE SL module provides the NR SL module with the candidate information (excluding at least the candidate resource sets SA or SB)

  - The NR SL module identifies a set of resources based on information shared by the LTE SL module.

    - FFS: how to identify the set of resources

  - The NR SL module excludes these identified resources from its own candidate resource set when performing the resource (re)selection procedure.

  - The exclusion process is performed in the PHY layer.

  - Note: implementation of Alt 1 should not have specification impact to LTE

- Alt 2: The LTE SL module provides the NR SL module with the candidate resource sets SA or SB shared by the LTE SL module

  - The LTE PHY SL module is provided information from the higher layer to generate a candidate resource set SA or SB. The resource set SA or SB is then shared to NR SL module.

  - The NR SL module performs an intersection operation with the candidate resource set received from the LTE SL module and the candidate resource set generated by the NR SL module.

    - FFS: how to handle the case where this results in an insufficient set of resources

  - The intersection operation is performed in the MAC layer.

  - FFS: How to handle NR V2X parameter settings that are not supported by LTE V2X, e.g., periodicities, sub-channel sizes, etc

  - Note: implementation of Alt 2 should not have specification impact to LTE

- In the next meeting strive to decide between the two alternatives

Agreement

For dynamic resource pool sharing, the NR SL module is expected to use the information shared by the LTE SL module to the NR SL module which is known by NR SL module at the latest T ms prior to slot n (as defined in clause 8.1.4 of TS 38.214), to determine a set of resources for its own (re)transmission.

- T is defined using

  - T≤Tmax ms, and is based on UE implementation, according to the Rel-16 NR SL timeline for in-device coexistence.

    - FFS: Value of Tmax

- FFS: any discussion on the earliest information, if needed

RAN1 #111:

Agreement

Based on the agreement in RAN1#110bis-e, the value of Tmax = 4 ms.

Agreement

For dynamic resource pool sharing, the NR SL module uses the candidate information shared by the LTE SL module to the NR SL module, where

- The NR SL module excludes resources based on the shared information from its own candidate resource set when performing the resource (re)selection procedure in the PHY layer.

  - FFS how to exclude resources at least based on the time and frequency locations of LTE SL transmissions that have been indicated in the shared candidate information.

  - FFS how the exclusion is performed according to clause 8.1.4 of TS 38.214.

  - FFS: whether/how NR SL module excludes resources not belonging to the generated LTE SL’s candidate resource set SB from its own candidate resource set.