3GPP TSG RAN WG1 #112	                                                   	           R1-2300968

Athens, Greece, February 27th – March 3rd, 2023

Source:	Intel Corporation

Title:	On Timing Advance Management

Agenda item:	9.12.2

Document for:	Discussion and Decision

# Introduction

In this paper, we discuss timing advance (TA) management for Rel-18 mobility enhancement WI [1]

- To specify mechanism and procedures of L1/L2 based inter-cell mobility for mobility latency reduction: - Timing Advance management [RAN1, RAN2] Note 2: FR2 specific enhancements are not precluded, if any. Note 3: The procedure of L1/L2 based inter-cell mobility are applicable to the following scenarios: - Standalone, CA and NR-DC case with serving cell change within one CG - Intra-DU case and intra-CU inter-DU case (applicable for Standalone and CA: no new RAN interfaces are expected) - Both intra-frequency and inter-frequency - Both FR1 and FR2 - Source and target cells may be synchronized or non-synchronized

# TA acquisition after cell-switch

The following was agreed in RAN1#110bis-e

Agreement Support TA acquisition of candidate cell(s) before cell switch command is received in L1/L2 based mobility. - FFS: whether this can be applied to candidate cell when it is deactivated SCell (if defined in RAN2)

In addition to acquiring TA for candidate cells before a cell switch command we expect that TA can be acquired for a target cell after the cell switch command is received. This allows downlink synchronization and RACH to be performed late into the cell switch procedure for better coverage. A high-level signal flow describing this is shown below:

Figure 1: High level signal flow for L1/L2 based mobility

Proposal-1: Support PRACH transmission by UE and TA acquisition of a target cell after cell switch command is received in L1/L2 based mobility

# PDCCH ordered RACH

The following was agreed in RAN1#111

Agreement On mechanism to acquire TA of the candidate cell(s) in Rel-18 LTM, at least support PDCCH ordered RACH. - The PDCCH order is only triggered by source cell - FFS: the details including content of DCI, RACH resource configuration, RAR transmission mechanism, etc. - Note: any other RACH-based solutions are for discussion separately Agreement For PDCCH ordered RACH in LTM, at least the following enhancements are supported - Introduce indication of candidate cell and/or RO of candidate cell in DCI - configuration of RACH resource for candidate cell(s) is provided prior to the PDCCH order - FFS: whether/how to transmit RAR Agreement On whether RAR is needed for PDCCH ordered RACH for a candidate cell in LTM, the following alternatives are considered for further study - Alt 1: RAR is needed - Alt 2: RAR is not needed - Note: If Alt 2 is supported, TA value of candidate cell is indicated in cell switch command - Alt 3: whether RAR is needed can be configured

In order to support PDCCH ordered RACH the following is proposed:

RACH resource pre-configuration for one or more candidate cells:

- In order to enable a low-overhead L1/L2 based mobility, a significant portion of the candidate cell information (SpCellConfig) can be pre-configured to a UE via RRC. This can be done for one or more candidate cells. This includes RACH configuration supporting CBRA/CFRA and a type of RACH procedure (including full-RACH, 2-step RACH or RACH-less) associated with a candidate cell. The choice to use CBRA without a contention resolution step (due to no RAR) can be left up to the NW for use cases of lightly loaded cells for example.

DCI content for PDCCH ordered RACH:

- As in legacy DCI format 1_0 – additionally explicit indication of candidate cell (as agreed)

Proposal-2: Consider RRC pre-configuration of RACH supporting CBRA/CFRA and RACH procedure types (full-RACH, 2-step RACH, RACH-less) for one or more candidate cells

In terms of the number of TA for candidate cells that needs to be acquired at least a minimum of 1 is required but after that it can be a UE capability.

RAR transmission mechanism:

We believe there is no need for RAR corresponding to candidate cells as there is no expectation that UE will transmit uplink reference signals for TA maintenance.

Proposal-3: Regarding RAR issue, support Alt-2 (RAR not needed)

# RACH-less procedure

For synchronized TRPs, a RACH-less access solution can be supported in a manner similar to LTE where a rach-Skip configuration allows a UE to perform PUSCH transmission to a target PCell with a TA value of 0 or the latest TA value used for the source cell. In addition to this, we think that a UE based procedure may be considered where a UE autonomously determines and applies a TA for an indicated candidate cell.

Proposal-4: Support TA=0 or latest TA value of the source cell for a RACH-less solution as in LTE. In addition to this a UE based procedure may be considered where a UE autonomously determines and applies a TA for an indicated candidate cell.

# Cell switch command

A cell switch MAC-CE is expected to provide at least the following information to a UE

  - Index to a configured candidate cell (the target cell)

  - RACH preamble for CBRA or CFRA indication, PRACH Mask (if TA is not acquired before cell-switch)

  - RO of the target cell

  - TA of the target cell (if TA is acquired before cell-switch)

Proposal-5: Consider defining a cell switch MAC-CE that includes an index to a configured candidate target cell, RO or TA information for the target cell

# Conclusions

Proposal-1: Support PRACH transmission by UE and TA acquisition of a target cell after cell switch command is received in L1/L2 based mobility

Proposal-2: Consider RRC pre-configuration of RACH supporting CBRA/CFRA and RACH procedure types (full-RACH, 2-step RACH, RACH-less) for one or more candidate cells

Proposal-3: Regarding RAR issue, support Alt-2 (RAR not needed)

Proposal-4: Support TA=0 or latest TA value of the source cell for a RACH-less solution as in LTE. In addition to this a UE based procedure may be considered where a UE autonomously determines and applies a TA for an indicated candidate cell.

Proposal-5: Consider defining a cell switch MAC-CE that includes an index to a configured candidate target cell, RO or TA information for the target cell

# References

- RP-213565, New WID on Further NR mobility enhancements, MediaTek, 3GPP TSG RAN #94e, Electronic Meeting, Dec. 6 - 17, 2021