3GPP TSG RAN WG1 #112			                     R1- 2301074

Athens, Greece, February 27th – March 3rd, 2023

Agenda item:	9.14.1

Source:	LG Electronics

Title:	Discussion on PRACH repeated transmission for NR coverage enhancement

Document for:	   Discussion and Decision

- Introduction

In the RAN1 #111 meeting, the following agreements were made for PRACH coverage enhancements of Rel-18 NR coverage enhancement WI as in [1]:

Agreement For multiple PRACH transmissions with same Tx beam, support to differentiate at least between multiple PRACH transmissions and single PRACH transmissions. Agreement For multiple PRACH transmissions with same Tx beam, to differentiate the multiple PRACH transmissions with single PRACH transmission, consider one or multiple of the following options. - Option 1: Multiple PRACH are transmitted with separate preamble on shared ROs. - Option 2: Multiple PRACH are transmitted on separate ROs. - Option 3: Partial of multiple PRACHs are transmitted with separate preamble on shared ROs, while the other multiple PRACHs are transmitted on separate ROs. - Other options are not precluded. - Note: Shared or separate RO/preamble means that the RO/preamble is shared or separated with single PRACH transmission. Agreement - Study at least the following case for multiple PRACH transmissions with different Tx beams. - UE uses different TX beams to transmit the multiple PRACH over ROs associated with the same SSB/CSI-RS - FFS: UE uses different TX beams to transmit the multiple PRACH over ROs associated with different SSBs /CSI-RSs, where the different SSBs/CSI-RSs are not associated with the same RO. - Note: not related to decision on CFRA Note: UE uses different TX beams to transmit the multiple PRACH over ROs associated with different SSBs/CSI-RSs, where the different SSBs/CSI-RSs are associated with the same RO is not considered. Working Assumption Simulation results for multiple PRACH transmissions with different beam(s) and same beam(s) (baseline) to be discussed in the next meeting. - Simulation assumptions in TR 38.830 are used as the starting point for the simulation. - Focus on FR2. - UE antenna configuration 2-2-2(baseline), 1-4-1(optional) - Performance metric: 0.1% false alarm, 1% miss-detection - Companies report the number of beams, the beam widths, beam correspondence assumption, and the boresights. - Channel model for link-level simulation: CDL-A defined in table 7.7.1-1 in TR 38.901. - Both that UE fulfills beamCorrespondence requirements Without UL-BeamSweeping and UE fulfils beamCorrespondence requirements With UL-BeamSweeping can be considered in the simulation are used as starting point for simulation. Agreement For multiple PRACH transmissions with same Tx beam, down-select one option from the following options. - Option 1: gNB can only configure one value for the number of multiple PRACH transmissions. - Option 2: gNB can configure one or multiple values for the number of multiple PRACH transmissions. - FFS: details Agreement - For multiple PRACH transmissions with same Tx beam, at least SSB-RSRP threshold(s) are used to determine the number of PRACH transmissions at least for the first RACH attempt. - Note: whether to support multiple numbers of PRACH transmissions is separately discussed.

In this contribution, we discuss on the PRACH coverage enhancements for Rel-18 NR coverage enhancement.

- Discussion

It was concluded that RAN1 would discuss and specify the PRACH coverage enhancements in Rel-18 NR coverage enhancement. In general, the PRACH repeated transmission can be considered to enhance PRACH coverage. Here, in terms of the beam direction of the PRACH preamble, two different techniques may be considered: repeated transmission with the same beam and repeated transmission with different beams. Regarding repeated transmission with different beams, it may not be suitable for PRACH coverage enhancement, because it does not seem easy for gNB to combine multiple PRACH preambles with different beams. Also, additional specification works would be necessary, such as the best beam selection procedure. So, in order to enhance the PRACH coverage, we think that the PRACH repeated transmission with the same beam is more appropriate. Therefore, we prefer to prioritize the PRACH repeated transmissions with the same beam for 4-step RACH procedure in Rel-18 NR coverage enhancement.

Proposal 1. Prioritize the PRACH repeated transmissions with the same beam for 4-step RACH procedure in Rel-18 NR coverage enhancement.

Moreover, in order to determine the repetition number of PUCCH for Msg4 HARQ-ACK at UE side, both repetition number(s) and RSRP threshold(s) for PRACH repeated transmission can be considered to be provided via higher layer signaling (e.g., SIB). For example, if a single repetition number is provided, a single RSRP threshold may be necessary, and if multiple repetition numbers are provided, multiple RSRP thresholds may be necessary. Moreover, regarding on RSRP threshold, we can discuss further whether to support a new RSRP threshold for PRACH repeated transmission or reuse the legacy RSRP threshold for Msg. 3 PUSCH repeated transmission.

Proposal 2. One or multiple repetition number(s) and RSRP threshold(s) can be provided via higher layer signaling (e.g., SIB).

Meanwhile, in case when the PRACH repeated transmission is configured, the UE can transmit multiple PRACH preambles over multiple ROs. At this moment, if a specific RO is not defined for RA-RNTI calculation, gNB and UE may use/expect different RA-RNTIs respectively, and as a result, the RACH procedure may be abnormally terminated. Therefore, it is necessary to define a reference RO for gNB and UE to use/expect the same RA-RNTI value. For example, the last RO among multiple ROs configured for PRACH repetition can be defined as a reference RO, and then gNB and UE can use/expect the same RA-RNTI value based on the predefined reference RO.

Proposal 3. Define the last RO among multiple ROs configured for multiple PRACH transmissions for one RACH attempt as a reference RO, and the RA-RNTI value is calculated based on the reference RO.

Moreover, in case when the UE transmits the multiple PRACH preambles via multiple ROs, it is necessary to define additional UE behavior for determining the starting point of the RAR window. Similar to the previous example, starting point of the RAR window may be determined by UE based on the last RO among multiple ROs configured for PRACH repetition. Also, we prefer to support only one RAR window for multiple PRACH transmissions for one RACH attempt.

Proposal 4. The RAR window starts after the last RO among multiple ROs configured for multiple PRACH transmissions for one RACH attempt.

Proposal 5. Support only one RAR window for multiple PRACH transmissions for one RACH attempt.

In the RAN1 #111 meeting, the issue of RACH resource allocation for multiple PRACH transmissions was discussed and related agreements were made. First of all, it was agreed that RACH resource differentiates between multiple PRACH transmissions and single PRACH transmission. Here, we think that this agreement can be operated based on RACH partitioning which is RAN2’s Rel-17 WI. This is because, RACH partitioning is a technique in which a gNB informs which RACH resource (e.g., preamble indexes) a UE should select when the UE has a specific feature or when the UE tries to request the feature. Thus, in order to allocate the RACH resources for multiple PRACH transmissions through RACH partitioning, it is necessary to introduce a PRACH repetition feature first.

For the first alternative, it can be considered to introduce a new parameter (e.g., msg1-Repetitions-r18) for the PRACH repetition feature into the SIB (e.g., in FeatureCombination-r17). To be specific, one of the spare parameters in FeatureCombination-r17 can be replaced with a new parameter for the PRACH repetition feature as shown in Figure 1. In addition, since PRACH repetition and Msg. 3 PUSCH repetition would generally be required at the same time, it may be desirable to combine the PRACH repetition feature and Msg. 3 PUSCH repetition feature simultaneously.

FeatureCombination-r17 ::= SEQUENCE {

redCap-r17                 ENUMERATED {true}                          OPTIONAL,  -- Need R

smallData-r17              ENUMERATED {true}                          OPTIONAL,  -- Need R

nsag-r17                   NSAG-List-r17                              OPTIONAL,  -- Need R

msg3-Repetitions-r17       ENUMERATED {true}                          OPTIONAL,  -- Need R

msg1-Repetitions-r18       ENUMERATED {true}                          OPTIONAL,  -- Need R

spare3                     ENUMERATED {true}                          OPTIONAL,  -- Need R

spare2                     ENUMERATED {true}                          OPTIONAL,  -- Need R

spare1                     ENUMERATED {true}                          OPTIONAL   -- Need R

}

Figure 1. Example of new parameter for PRACH repetition in FeatureCombination-r17.

For another alternative, it can be considered to reuse one of the legacy parameters in FeatureCombination-r17 as a PRACH repetition feature. For example, as mentioned above, since PRACH repetition and Msg. 3 PUSCH repetition would generally be required at the same time, “msg3-Repetitions-r17” can be used to indicate the PRACH repetition feature in addition to the existing Msg.3 PUSCH repetition feature.

Proposal 6. Support the PRACH repetition feature in SIB for RACH resource partitioning to request PRACH repeated transmission. One of the following alternatives can be supported.

- Alt 1: Introduce a new parameter (e.g., msg1-Repetitions-r18) for the PRACH repetition feature into the SIB (e.g., in FeatureCombination-r17).

- Alt 2: “msg3-Repetitions-r17” in FeatureCombination-r17 is used as a PRACH repetition feature in addition to the existing Msg.3 PUSCH repetition feature.

Moreover, three options for RACH resource allocation for multiple PRACH transmissions were discussed and a related agreement was also made. In this agreement, option 1 and option 2 can be configured by using the Rel-17 RACH partitioning technique. Here, an example of the higher layer parameters that gNB can utilize for RACH partitioning is shown in Figure 2.

Figure 2. Example of higher layer parameters for RACH resource partitioning

For one example, if gNB wants to configure that multiple PRACH are transmitted with separate preamble on shared ROs, as in Option 1, it is possible to indicate the PRACH repetition feature (e.g., msg1-Repetitions-r18) as true, which is provided in Feature Combination-r17 of existing rach-ConfigCommon (or AdditionalRACH-Config-r17) while gNB should provide a preamble index group so that it does not overlap with other preamble index groups.

For another example, if gNB wants to configure that multiple PRACH are transmitted on separate ROs, as in Option 2, it is possible to provide an RO dedicated to PRACH repetition through AdditionalRACH-Config-r17. To be specific, gNB can indicate the PRACH repetition feature (e.g., msg1-Repetitions-r18) as true, which is provided in Feature Combination-r17 of AdditionalRACH-Config-r17, and gNB can indicate all preamble indexes are used for PRACH repetition feature.

Proposal 7. For multiple PRACH transmissions with same beam, consider one or multiple of the following options

- Option 1: Multiple PRACH are transmitted with a separate preamble on shared ROs using FeatureCombination-r17 in either legacy configuration or AdditionalRACH-Config-r17.

- Option 2: Multiple PRACH are transmitted on separate ROs using FeatureCombination-r17 in AdditionalRACH-Config-r17.

Meanwhile, it is natural UE behavior that a UE can transmit multiple PRACH transmissions using the RACH occasion allocated for PRACH repetition. However, if a UE can randomly select the RO allocated for PRACH repetition, and if the UE can initiate multiple PRACH transmissions in that RO, the gNB should receive and detect multiple PRACH preambles, assuming that multiple PRACH transmissions can be initiated in all potential ROs allocated for PRACH repetition. As a result, it will increase detection complexity on the gNB side. Therefore, in order to handle this issue, it can be considered to support the starting RACH slot (and/or starting RO) and period for multiple PRACH transmissions. If the starting RACH slot (and/or starting RO) is supported, a UE should initiate the multiple PRACH transmissions at starting RO in starting RACH slot.

Two alternatives can be considered to determine the starting RACH slot (and/or starting RO) and period. For the first alternative, the starting RACH slot (and/or starting RO) and period can be predefined based on PRACH configuration, SSB-to-RO mapping, and repetition number. For another alternative, the starting RACH slot (and/or starting RO) and period can be provided by gNB via SIB.

Proposal 8. Support starting RACH slot (and/or starting RO) and period for PRACH repeated transmission. One of the following alternatives can be supported.

- Alt 1) Starting RACH slot (and/or starting RO) and period can be predefined based on PRACH configuration, SSB-to-RO mapping, and repetition number.

- Alt 2) Starting RACH slot (and/or starting RO) and period can be provided by gNB via SIB.

In the RAN1 #110b-e meeting, the ROs pattern for multiple PRACH transmission was discussed and related agreement was made.

Agreement For multiple PRACH transmissions with same beam, at least ROs located at different time instances can be utilized for the transmissions. - FFS: whether/how the starting RB of ROs can be different at different time instances for multiple PRACH transmissions. - FFS: whether/how multiple PRACH transmissions located in the same time instance, e.g., for UEs with multiple Tx chains.

Regarding the first FFS point of this agreement, RAN1 needs to discuss whether to support the RO hopping in the frequency domain at different time instances for multiple PRACH transmissions. Basically, the RO hopping in the frequency domain has an advantage in terms of frequency diversity. In addition, it is possible to prevent the concentration of UEs for PRACH repetition in a specific RO among FDMed ROs. Therefore, it is preferred to support the RO hopping in the frequency domain at different time instances for multiple PRACH transmissions

Furthermore, it is necessary to determine which RO the UE should select among the multiple FDMed ROs. For example, the first RO may be randomly selected by UE, and then the predefined RO hopping pattern provided by gNB can be used for other ROs.

Proposal 9. Support the RO hopping in the frequency domain at different time instances for multiple PRACH transmissions.

- FFS on how to determine which RO among the FDMed ROs is used for PRACH repeated transmission.

By the way, regarding the second FFS point, we are not sure it is beneficial that FDMed ROs is utilized for the PRACH repetition, even for the UE with multiple TX chains. First of all, gNB reception complexity would increase since gNB should accumulate the multiple PRACH preambles transmitted in different ROs at the same time instance. Also, if it is supported, additional specification works would be necessary for the RACH procedure. For example, RAN1 needs to determine which RAR UL grant should be selected by UE for Msg3 PUSCH transmission among multiple RAR UL grants according to multiple FDMed ROs. After all, we think it is desirable to focus on multiple PRACH transmissions in TDMed ROs because there is not enough time to discuss these issues further.

Proposal 10. Do not support the multiple PRACH transmissions located in the same time instance.

Meanwhile, in the previous meetings, the transmission power for multiple PRACH transmissions was discussed. For multiple PRACH transmissions with the same beam in one RACH attempt, it is preferable that transmission power ramping is not applied during the multiple PRACH transmissions. The reason is that if transmission power is increased for each PRACH transmission, it may cause unintended interference to neighbor cells. In addition, we think it is beneficial that the same measurement of the same reference signal to calculate the path loss is applied for each PRACH transmissions in one RACH attempt. And then, if re-transmission may be required, the transmission power ramping can be applied for another RACH attempt, similar to the legacy behaviour.

Proposal 11. For multiple PRACH transmissions with same beam, transmission power ramping is not applied during the multiple PRACH transmissions in one RACH attempt.

- The same measurement of the same reference signal to calculate the path loss is applied for each PRACH transmissions

Furthermore, in the previous meetings, the SSB-to-RO mapping methods for multiple PRACH transmissions were discussed. Regarding this issue, we prefer to support SSB-to-RO mapping by RO group, while maintaining the existing SSB-to-RO mapping scheme (i.e., frequency first – time second manner). For example, a RO group can consist of multiple (N) consecutive ROs in time, and N can be a repetition number for multiple PRACH transmissions.

Figure 2 shows an example of RO group-wise SSB-to-RO mapping by comparing it with an example of legacy SSB-to-RO mapping. In this example, we assume that the number of ROs in RO group (N) is 2, the repetition number for multiple PRACH transmission is 2, and 4 ROs per SSB are associated. In this way, gNB and UE can understand that SSB-to-RO mapping is performed in a frequency first–time second manner based on a RO group consisting of N consecutive ROs in time. Thereafter, the UE can transmit multiple PRACH preambles using multiple ROs with the same UL TX beam.

Figure 2. Example of legacy and RO group-wise SSB-to-RO mapping (N = 2, Repetition number = 2, and 4 ROs per SSB)

Proposal 12. In case when the separate RO is allocated for multiple PRACH repetitions, consider the following for SSB-to-RO mapping for multiple PRACH transmissions.

- RO group-wise SSB-to-RO mapping by frequency first-time second manner

- One RO group consists of multiple (N) consecutive ROs in time

- Conclusion

In this contribution, we discussed the PRACH coverage enhancements for Rel-18 NR coverage enhancement, we have the following proposals.

Proposal 1. Prioritize the PRACH repeated transmissions with the same beam for 4-step RACH procedure in Rel-18 NR coverage enhancement.

Proposal 2. One or multiple repetition number(s) and RSRP threshold(s) can be provided via higher layer signaling (e.g., SIB).

Proposal 3. Define the last RO among multiple ROs configured for multiple PRACH transmissions for one RACH attempt as a reference RO, and the RA-RNTI value is calculated based on the reference RO.

Proposal 4. The RAR window starts after the last RO among multiple ROs configured for multiple PRACH transmissions for one RACH attempt.

Proposal 5. Support only one RAR window for multiple PRACH transmissions for one RACH attempt.

Proposal 6. Support the PRACH repetition feature in SIB for RACH resource partitioning to request PRACH repeated transmission. One of the following alternatives can be supported.

- Alt 1: Introduce a new parameter (e.g., msg1-Repetitions-r18) for the PRACH repetition feature into the SIB (e.g., in FeatureCombination-r17).

- Alt 2: “msg3-Repetitions-r17” in FeatureCombination-r17 is used as a PRACH repetition feature in addition to the existing Msg.3 PUSCH repetition feature.

Proposal 7. For multiple PRACH transmissions with same beam, consider one or multiple of the following options

- Option 1: Multiple PRACH are transmitted with a separate preamble on shared ROs using FeatureCombination-r17 in either legacy configuration or AdditionalRACH-Config-r17.

- Option 2: Multiple PRACH are transmitted on separate ROs using FeatureCombination-r17 in AdditionalRACH-Config-r17.

Proposal 8. Support starting RACH slot (and/or starting RO) and period for PRACH repeated transmission. One of the following alternatives can be supported.

- Alt 1) Starting RACH slot (and/or starting RO) and period can be predefined based on PRACH configuration, SSB-to-RO mapping, and repetition number.

- Alt 2) Starting RACH slot (and/or starting RO) and period can be provided by gNB via SIB.

Proposal 9. Support the RO hopping in the frequency domain at different time instances for multiple PRACH transmissions.

- FFS on how to determine which RO among the FDMed ROs is used for PRACH repeated transmission.

Proposal 10. Do not support the multiple PRACH transmissions located in the same time instance.

Proposal 11. For multiple PRACH transmissions with same beam, transmission power ramping is not applied during the multiple PRACH transmissions in one RACH attempt.

- The same measurement of the same reference signal to calculate the path loss is applied for each PRACH transmissions

Proposal 12. In case when the separate RO is allocated for multiple PRACH repetitions, consider the following for SSB-to-RO mapping for multiple PRACH transmissions.

- RO group-wise SSB-to-RO mapping by frequency first-time second manner

- One RO group consists of multiple (N) consecutive ROs in time

- Reference

[1] RAN1 Chair’s Notes in the 3GPP TSG RAN WG1 #111