3GPP TSG-RAN WG1 Meeting #112	R1-2301154

Athens, Greece, February 27th – March 3rd, 2023

Agenda Item:	9.16.1

Source:	Ericsson

Title:	NR support of spectrum less than 5MHz for FR1

Document for:	Discussion

# 1	Introduction

The Rel-18 “NR support for dedicated spectrum less than 5MHz for FR1” [1] includes the following objectives touching upon RAN1:

The following objectives shall be included for dedicated FDD spectrum in FR1: - Identify and specify necessary changes to NR physical layer with minimum specification impact to operate in spectrum allocations from approximately 3 MHz up to below 5 MHz [RAN1]: - Restrict to subcarrier spacing of 15kHz and the use of normal cyclic prefix. - For SSB: - Reuse PSS/SSS specification without puncturing. - PBCH based on current design - Identify and specify necessary minimum changes to PDCCH, CSI-RS/TRS, PUCCH, and PRACH for functional support based on existing design, without optimization.

In this contribution we provide a follow-up on the RAN1 agreements and conclusions reached in RAN1#111 [2], as well as initial views on other topics that were not yet addressed.

# 2	Overall assumptions & feedback awaiting from RAN4

In RAN1# 111, the following agreements were reached in connection with two questions included in a Liaison Statement (LS) sent to RAN4 [3].

Agreement In an LS to RAN4, in addition to reuse 5 MHz channel bandwidth, RAN1 suppose only 3 MHz channel bandwidth is supported, and would like to get RAN4 responses on the maximum transmission bandwidth (the number of PRBs) for this channel BW. Agreement RAN1 would like to ask RAN4 if finer sync. raster for the 3MHz and/or 5MHz channel bandwidth is feasible, as well as any input from RAN1 for RAN4’s answer to this question.

RAN4 will have their first Time Units (TUs) for this Work Item (WI) in RAN4# 106, meanwhile RAN1 will continue working based on the following overall assumption [2]:

Agreement Before getting RAN4 responses, RAN1 assume maximum transmission bandwidth, 15RBs or 16RBs for 3 MHz channel BW for evaluation and analysis. Note: include agreement into the LS

# 3	Follow-up on the physical channels and signals

During RAN1# 111 several agreements and conclusions were reached on the physical channels (SSB, PDCCH, PUCCH, and PRACH) mentioned in the WID, in line with it in the subsections below we provide a follow-up on them accounting for the corresponding agreements and conclusions [2]. Moreover, the WID also mentions legacy signals (CSI-RS/TS) for which no agreements have been reached yet, hence we provide our initial views on them.

The performance evaluations shown in subsequent sections made use of the following general assumptions (Assumptions on specific physical channels are provided in their corresponding subsections).

Table 1: General assumptions for NR less than 5 MHz channel BW

| Parameters | Values |
| --- | --- |
| # UE Tx chains | 1 |
| # UE Rx chains | 2 |
| # gNB Tx chains | 2 |
| # gNB Rx chains | 2 |
| Channel bandwidth | 3 MHz (15 PRBs, 15 kHz SCS) |
| Carrier frequency | 900 MHz (representative for n100, n26, n28) |

## 3.1	SSB in NR spectrum “less than 5 MHZ for FR1”

Today prior to Rel-18, SSB for NR overall has the following characteristics:

- In the time-domain SSB spans over 4 OFDM symbols.

- In the frequency-domain SSB spans over 240 subcarriers (See Figure 1).

  - PSS/SSS encompasses 127 subcarriers

  - PBCH encompasses 240 subcarriers

- One or more SSBs can be transmitted per SS burst according with Table 2.

| Table 2: Maximum number of SSBs transmitted per SS burst Maximum number of SSBs per SS burst. SCS (kHz); fc < 3GHz; 3GHz ≤ fc ≤ 6GHz; 6 GHz < fc. Case A; 15; 4; 8. Case B; 30; 4; 8. Case C; 30; 4; 8. Case D; 120; 64. Case E; 240; 64. | Figure 1: SSB structure |
| --- | --- |

In RAN1# 111, the following agreements were reached touching upon SSB:

Agreement Before getting RAN4 responses, RAN1 assume that the UE could know which RBs are used for SSB transmission after PSS/SSS is detected for evaluation and analysis. Note: it does not mean indication signaling is needed. Note: include this agreement into the LS Agreement For transmission bandwidths of <5MHz for 3MHz and 5MHz channel bandwidth, a subset of PRBs of 20-PRB PBCH are used for PBCH transmission if the transmission BW of a channel is less than 20PRBs. - FFS which PRBs are used and how to use the PRBs - Note: PRBs for PSS/SSS are not punctured. Agreement Study whether and how to recover PBCH detection performance for transmission bandwidths of <5MHz for 3MHz and 5MHz channel bandwidth. The following options are considered, - Opt.1: Power boosting - Opt.2: Multiple PBCH receptions - Opt.3: PBCH remapping - Opt.4: PBCH payload reduction - Opt.5: PBCH rate matching around the punctured PRBs - Opt.6: no enhancement specified

One of the agreements above states “a subset of PRBs of 20-PRB PBCH are used for PBCH transmission if the transmission BW of a channel is less than 20PRBs”, and in addition there is a note stating “PRBs for PSS/SSS are not punctured”. Based on those agreed premises we have applied a puncturing technique on SSB in a channel bandwidth of 3MHz assuming a transmission bandwidth of 15 RBs.

On the FFS about “which PRBs are used and how to use the PRBs”, examples can be found in Table 3 along with other general assumptions.

Table 3: Puncturing assumptions and general assumptions on SSB

| Puncturing assumptions |  |  |
| --- | --- | --- |
| Transmission bandwidth of 15 RBs |  |  |
| 15 RBs unpunctured | 14 RBs unpunctured | 12 RBs unpunctured |
| 1 RB punctured from top, 4 RBs punctured from bottom | 2 RBs punctured from top, 4 RBs punctured from bottom | 4 RBs punctured from top, 4 RBs punctured from bottom |
| - Parameter |  | Value |
| - UE speed (km/h) |  | 40 |
| - Channel model |  | TDL-C 300 ns |
| - (Residual) frequency offset (UE) |  | 0.1 ppm |
| - SS burst set periodicity |  | 20 ms |
| - Precoder |  | Precoder cycling |
| - Number of transmissions (shots) |  | 4 |
| - BLER target |  | 1% |

For SSB, RAN1 agreed that “a subset of PRBs of 20-PRB PBCH are used for PBCH transmission if the transmission BW of a channel is less than 20PRBs,” keeping as “FFS which PRBs are used and how to use the PRBs”. Resolving this FFS should be prioritized, since likely it will be needed as complementary input for RAN4 towards completing the sync-raster design.

Prioritize resolving the FFS about the “subset of PRBs of 20-PRB PBCH are used for PBCH transmission if the transmission BW of a channel is less than 20PRBs” (i.e., “FFS which PRBs are used and how to use the PRBs”).

Using the simulation set-up in Table 3, and accounting for the agreement stating “RAN1 assume that the UE could know which RBs are used for SSB transmission after PSS/SSS is detected for evaluation and analysis”, the obtained results are shown in Figure 2.

Figure 2: BLER performance of punctured and non-punctured PBCH transmissions.

The puncturing applied on SSB preserving 15RBs, 14RBs, or 12RBs results respectively in a performance difference of ~1.75dB, ~2.5dB, and ~3.6dB with respect to a non-punctured curve in terms of SNR required to achieve 1% BLER.

The performance difference was observed assuming the same power per sub-carrier for both the non-punctured and punctured PBCH. The performance difference was small (i.e., less than 2dB when preserving 15RBs unpunctured) even without using yet any compensation technique (e.g., power boosting).

- Preserving unpunctured less than 15 SSB RBs may result in not fulfilling RRM requirements in RAN4 RRM (e.g. UE shall detect the cell with the assumption SNR=-6dB and 90% of success rate, including RF impairment margin), thus performance-wise preserving unpunctured 15RBs is recommended.

One important aspect to considered is that “subcarrier 0 of the first resource block of the SS/PBCH block” is used in several parts of the specification as a reference point. On this matter, is important to be aware that if the transmission bandwidth were defined as 15RBs, then it won’t be possible to preserve “subcarrier 0 of the first resource block of the SS/PBCH block” since from subcarrier 0 to the last subcarrier used by PSS/SSS (i.e., subcarrier k = 182) there are 183 subcarriers which exceed the 15RBs. Hence, towards preserving 15RBs unpunctured, the 5RBs to be punctured with respect to the 20RBs legacy SSB structure can in principle be obtained from any of the combinations shown in Table 4, but in the end only one of them should be chosen as to have a fully deterministic puncturing on the 20-PRBs of SSB which will facilitate the sync-raster design, RAN4 requirements, and that won’t require the UE to hypothesize or receiving signalling indicating the punctured region.

Table 4: Alternatives to preserve 15 PRBs unpunctured with respect to the 20RBs legacy SSB structure

| a) 4 RBs punctured from top, 1 RB punctured from bottom | b) 1 RB punctured from top, 4 RBs punctured from bottom |
| --- | --- |
| c) 3 RBs punctured from top, 2 RBs punctured from bottom | d) 2 RBs punctured from top, 3 RBs punctured from bottom |

Towards preserving 15RBs unpunctured, the 5RBs to be punctured with respect to the 20RBs of the legacy SSB structure can in principle be obtained from either:

- a) 4 RBs punctured from top, 1 RB punctured from bottom.

- b) 1 RB punctured from top, 4 RBs punctured from bottom.

- c) 3 RBs punctured from top, 2 RBs punctured from bottom.

- d) 2 RBs punctured from top, 3 RBs punctured from bottom.

- In the end, only one of the above alternatives should be chosen as to have a fully deterministic/invariant puncturing on the 20-PRBs of SSB, which will facilitate the sync-raster design, RAN4 requirements, and won’t require the UE to hypothesize or receiving signalling (e.g., early indication) to indicate the punctured region.

The number of unpunctured/preserved PRBs overall determine the BLER performance of SSB regardless of the location of the punctured subcarriers/PRBs within the legacy 20-PRBs structure, hence alternative “a)” where 4 RBs are punctured from top and 1 RB is punctured from bottom is preferred because of the following reason:

- If the 5 PRBs to be punctured from the 20 PRBs SSB structure, they were mostly taken from to the top (e.g., 4 RBs) and minimally from the bottom (e.g., 1 RB), then existing offsets (i.e., “kSSB” and “Offset (RBs)”) and their legacy reference points could be kept unmodified for functional support (see Figure 3).

Figure 3: Relationship between the location in frequency of CORESET 0 and a 15 PRB punctured SSB.

If the puncturing is minimally applied (i.e., only 1 PRB) at the bottom of the 20-PRB legacy SSB structure (See right-hand side of Figure 3), the existing offsets (e.g., “kSSB” and “Offset (RBs)”) and their legacy reference points can be kept unmodified for functional support. In this case:

- If both “kSSB” and “Offset (RBs)” are equal to 0, CORESET#0 will be 12 subcarriers (1 PRB) apart from the bottom of the 15 PRBs unpunctured of the SSB structure. That is, if both “kSSB” and “Offset (RBs)” are 0, the lowest RB of legacy CORESET#0 is virtually/hypothetically aligned with the 20-PRB legacy SSB structure.

- Otherwise, any other combination of “kSSB” and “Offset (RBs)” where at least one of them uses a value different than 0 results in shifting further downwards (i.e., beyond the already 1 PRB apart) CORESET#0 with respect to the 15 PRBs unpunctured of the SSB structure.

Overall, when existing offsets (i.e., “kSSB” and “Offset (RBs)”) and their legacy reference points are kept unmodified, the degree of flexibility seems to be reasonable taking into account that CORESET#0’s structure (e.g., minimum configuration is 24 RBs) is larger than the SSB structure (20 RBs that become reduced to 15 RBs after puncturing) hence the possibility of shifting CORESET#0 downwards with respect to the 15PRBs unpunctured of the SSB structure (which allows moving around to some extent the CORSET#0’s region to be punctured) should be feasible/enough as to do not require optimizing existing offsets (i.e., “kSSB” and “Offset (RBs)”).

Towards minimizing the specification impact, the puncturing applied on SSB could be reflected through a “Note” relying on Table 7.4.3.1-1 in TS 38.211 which describes the legacy SSB structure. Below we provide an example of a “Note” describing a puncturing applied on the 20-PRB SSB legacy structure where 4-PRBs were punctured at the top and 1-PRB was punctured at the bottom. That is, with respect to the legacy 20-PRB SSB structure, only subcarrier number= 12, 13, … 191 are expected to be received by the UE. Moreover, since Table 7.4.3.1-1 reflects the legacy 20-PRB SSB structure and the puncturing is fully deterministic, then subcarrier 0 of the 20-PRB SSB structure can be kept as a virtual/hypothetical reference for applying kSSB.

Table 7.4.3.1-1: Resources within an SS/PBCH block for PSS, SSS, PBCH, and DM-RS for PBCH.

| Channel or signal | OFDM symbol number relative to the start of an SS/PBCH block | Subcarrier number relative to the start of an SS/PBCH block |
| --- | --- | --- |
| PSS | 0 | 56, 57, …, 182 |
| SSS | 2 | 56, 57, …, 182 |
| Set to 0 | 0 | 0, 1, …, 55, 183, 184, …, 239 |
|  | 2 | 48, 49, …, 55, 183, 184, …, 191 |
| PBCH | 1, 3 | 0, 1, …, 239 |
|  | 2 | 0, 1, …, 47, 192, 193, …, 239 |
| DM-RS for PBCH | 1, 3 |  |
|  | 2 |  |

Note: When the UE is configured with LessThan5MHzFR1, the UE is only expected to receive subcarrier number = 12, 13, … 191 as described in Table 7.4.3.1-1, but the UE may continue assuming subcarrier 0 of the first resource block of the SS/PBCH block as a hypothetical reference for applying kSSB.

Preserve the legacy SSB NR structure unmodified, applying puncturing on SSB when required for transmission bandwidths <5 MHz for 3MHz and 5MHz channel bandwidths.

- For transmission bandwidths <5MHz for 3MHz channel bandwidth, where puncturing is required to be applied on SSB:

  - “a subset of 15PRBs of 20-PRB PBCH are used for PBCH transmission”.

Adopt a deterministic puncturing, where with respect to the 20-PRB SSB legacy structure, 4 PRBs are punctured from the top and 1 PRB is punctured from bottom.

      - This allows keeping a minimum distance (i.e., 1RB) between the bottom of the 15-PRB SSB punctured structure and the 20-PRB SSB legacy structure, which aims at minimizing the impact on legacy offsets (i.e., “kSSB” and “Offset (RBs)”).

To minimize the specification impacts from puncturing, the puncturing applied on SSB is reflected through adding a “Note” that relies on Table 7.4.3.1-1 in TS 38.211 which describes the legacy SSB structure:

      - Note: When the UE is configured with LessThan5MHzFR1, the UE is only expected to receive subcarrier number k = 12, 13, … 191 as described in Table 7.4.3.1-1.

Keep “subcarrier 0 of the first resource block of the SS/PBCH block” as a hypothetical/virtual reference for applying kSSB. That is, since Table 7.4.3.1-1 reflects the legacy 20-PRB SSB structure and the puncturing is fully deterministic, then subcarrier 0 can be kept as a virtual/hypothetical reference. The following statement can be appended to the “Note” describing the puncturing:

- The UE may continue assuming subcarrier 0 of the first resource block of the SS/PBCH block as a hypothetical reference for applying kSSB

Existing offsets (i.e., “kSSB” and “Offset (RBs)”) are kept unmodified for functional support.

      - If the UE continues assuming subcarrier 0 of the first resource block of the SS/PBCH block as a hypothetical reference applying “kSSB”, and if on the 20-PRB SSB, 4 PRBs were punctured from the top and 1 PRB were punctured from bottom as to keep a minimum distance (i.e., 1RB) between the bottom of the 15-PRB SSB punctured structure and the 20-PRB SSB legacy structure, then both “kSSB” and “Offset (RBs)” can be kept used as in legacy for functional support (i.e., there is no strict need for optimizing, since offsets are functional/usable).

### 3.1.1	View on recovery techniques for SSB

As mentioned earlier, RAN1 reached the following agreement touching upon the potential usage of recovery techniques when the NR SSB legacy structure does not fit entirely into the transmission bandwidth: “Study whether and how to recover PBCH detection performance for transmission bandwidths of <5MHz for 3MHz and 5MHz channel bandwidth. The following options are considered Opt.1: Power boosting Opt.2: Multiple PBCH receptions Opt.3: PBCH remapping Opt.4: PBCH payload reduction Opt.5: PBCH rate matching around the punctured PRBs Opt.6: no enhancement specified”. Below we present an overall one-on-one comparison among the considered options:

Table 5: Overall comparison among the options considered as recovery techniques for SSB

|  | Candidate Recovery Techniques for SSB |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Opt.1: Power boosting | Opt.2: Multiple PBCH receptions | Opt.3: PBCH remapping | Opt.4: PBCH payload reduction | Opt.5: PBCH rate matching around the punctured | Opt.6: no enhancement specified |
| Highlevel description | If the total power that otherwise is used on the legacy NR structure were now concentrated into the unpunctured region, then the punctured transmission would have a higher PSD, creating a power boosting effect that would improve the performance. | In [4], it has been said “since PBCH is periodically transmitted with mostly the same payload, multi-time PBCH reception could be performed to improve PBCH detection performance”. Moreover, PBCH combining is not a standardized procedure since the combining or not of PBCH receptions is up to the UE implementation. TS 38-101-4 accounts for the possibility of PBCH combining in case the UE had that ability [5]. | In our understanding, “PBCH remapping” refers to re-locating in the time-domain (e.g., by adding a new OFDM symbol) the part of PBCH not fitting into the transmission bandwidth. | This option is self-explanatory, and is about restricting the “PBCH payload ” to a smaller size. | In our understanding, “rate matching” can be used to avoid/skip the RBs that are to be punctured, as to map the payload only on the unimpacted PBCH RBs. | Since the performance difference between a punctured and a non-puncture evaluation is not that large (e.g., ~ 2dB), and SSB is not foreseen to be a bottle-neck channel, then the option of not introducing any performance recovery technique seems to be also a feasible choice. |
| Pros | - It is foreseen that the power boosting will end-up fully or almost-fully compensating for the performance difference observed between the punctured and non-punctured evaluation. - No extra cost (i.e., the power boosting comes from a total power already used today). - No specification impact foreseen other than perhaps having to set a max power limit (or any band-related exception) in the RAN4 specs. | - This technique takes advantage of the existing periodic PBCH transmissions, and will not require any specification impact if the combining is left up to the UE implementation. RAN4 specs can continue asuming the possibility of using PBCH combining. | - Ideally upon re-structuring SSB through re-mapping, all PBCH PRBs would be fully transmitted within the reduced transmission bandwidth. | - None on its own (See comment on “Cons“) | - The PBCH payload sizes fitting into the usabled PBCH RBs according with the “rate matching” rules will be fully transmitted into the transmission bandwidth. | - No specification impacts at all. - Opt.6 (i.e., no recovery technique) will anyway hold in case any of the other options (if introduced) were not applicable in certain scenarios. |
| Cons | - It may not be possible to apply a power boosting in all scenarios (e.g., in a co-located NR-R&GSM-R scenario), but yet again it will depend on RAN4 requirements. | - As mentioned in [4], “The cost here is higher latency for PBCH detection“. - Interference and/or radio conditions impacting differently the PBCH transmissions may make sometimes the combining challenging or even unfeasible. | - Re-structuring SSB by re-locating in the time-domain what does not fit into the frequency-domain is foreseen to result in not nefligible specification impacts, which does not seem to be in line with the guidance of the WID. - If part of the PBCH is re-located in the time-domain, likely the UE will have to decode one additional OFDM symbol per transmitted SSB, and on top of the SSB re-structuring per-se, it might need to be re-discussed/re-designed the number of SSBs transmitted per SS burst. | - A reduced PBCH payload on its own is foreseen not to be an improver of the performance, since the structure of SSB will remain the same with some part of PBCH not being transmitted regardless of the payload size. | - One or more rate matching patterns may need to be introduced, which can be “RB level rate matching” and/or “subcarrier level rate matching”. - No “rate matching” existing in the NR specs are foreseen to be re-usable, especially if a sync-raster with finer granularity is to be introduced. - Compared to other solutions, “rate matching” will likely result in a non-negligible specification impact. | - There will be a performance difference between a punctured and a non-punctured transmission. Such a performance difference won’t be compensated (at least not through a technique impacting the technical specifications) |

The performance difference between a punctured and a non-punctured SSB was found to be small (less than 2dB @ 1% BLER when 15-PRBs are kept unpunctured). Moreover, SSB is not foreseen to be a bottle-neck channel link budget-wise. Thus, accounting for the above a recovery technique with minor specification impact (Opt1 which may only require setting a limit in RAN4 specs) or no specification impact at all (Opt6) is preferred.

## 3.2	PDCCH in NR spectrum “less than 5MHZ for FR1”

PDCCH has a strong connection with CORESET 0, hence below we elaborate on both in sections 3.2.1 and 3.2.2 respectively.

### 3.2.1	PDCCH

Today prior to Rel-18, PDCCH for NR overall has the following characteristics:

- PDCCH is mapped to a specific CORESET.

- PDCCH occupies 1, 2, 4, 8, or 16 Control Channel Elements (CCEs).

  - 1 CCE corresponds to 6 Resource Element Groups (REGs).

    - 1 REG equals to 1 Resource Block (RB) in 1 OFDM symbol.

- The CCE-to-REG mapping depends on a specific time-frequency resource configuration of a CORESET that spans up to 3 symbols in the time-domain.

- The CCE-to-REG mapping can utilize one of the two following approaches:

  - Interleaved mapping (Always applied in CORESET 0)

  - Non-Interleaved mapping

PDCCH is mapped to a specific CORESET. Thus, the highest “Aggregation level” that can be used depends on the resource blocks in the frequency domain according with “NRBCORESET” and symbols “NsymbCORESET” in the time domain configured for a CORESET.

### 3.2.2	CORESET 0

As mentioned in the previous section, PDCCHs are transmitted in control resource sets (CORESETs) which span over one, two, or three contiguous OFDM symbols over multiple resource blocks (RBs). In frequency domain, a CORESET can span over one or multiple chunks of 6 RBs. For CORESETs other than CORESET 0, multiple chunks of 6 RBs can be either contiguous or non-contiguous. On the other hand, CORESET 0 (which is configured and used during the initial access) can only have 24, 48, or 96 RBs.

For CORESET 0 configured by the ControlResourceSetZero IE:

-	NRBCORESET and NsymbCORESET are defined by clause 13 of [6, TS 38.213];

-	the UE may assume interleaved mapping

-	L=6; This variable refers to the REG bundle size.

-	R=2; This variable refers to the Interleaver size.

-	nshift=NIDcell;

-	the UE may assume normal cyclic prefix when CORESET 0 is configured by MIB or SIB1;

-	the UE may assume the same precoding being used within a REG bundle.

Moreover, CORESET 0 must be contiguous in frequency domain and only supports interleaved CCE-to-REG mapping [6].

- REG bundle  is defined as REGs  where L is the REG bundle size, i=0,1,…,NREGCORESETL-1, and NREGCORESET=NRBCORESETNsymbCORESET is the number of REGs in the CORESET.

- For interleaved CCE-to-REG mapping, L∈2,6for NsymbCORESET=1 and L∈NsymbCORESET,6 for NsymbCORESET∈2,3. The interleaver is defined by

fx=rC+c+nshift mod NREGCORESETL
x=cR+r
r=0,1,…,R-1
c=0,1,…,C-1
C=NREGCORESET(LR)

Another important aspect to consider is that there is a close connection between the location in frequency of CORESET 0 and SSB, since CORESET 0 has an offset (i.e., “Offset (RBs)”) defined with respect to “kssb offset” that in turn uses SSB as reference as previously depicted in Figure 3.

The puncturing on PDCCH in CORESET 0 will be determined by the puncturing applied on SSB, and the offset parameters “kssb” and “Offset (RBs)”.

In RAN1# 111, the following agreements were reached touching upon PDCCH and CORESET 0:

Agreement For CORESET#0 configuration for transmission bandwidths <5 MHz for 3MHz and 5MHz channel bandwidth, following options are for study, - Opt.1: Existing configuration table for 15kHz SCS, 5MHz minimum channel BW (i.e., table 13-1 in TS38.213) is reused for configuration - Opt.2: A new CORESET#0 configuration table is to be introduced for the configuration. Agreement Study whether and how to recover PDCCH detection performance of CORESET#0 for transmission bandwidths of <5MHz for 3MHz and 5MHz channel bandwidth. The following options are considered, - Opt.1: Power boosting - Opt.2: Non-interleaved CCE-to-REG mapping - Opt.3: A new interleaver to ensure PDCCH is fully mapped in the spectrum - Opt.4: New aggregation level(s) for fit in the spectrum - Opt.5: PDCCH rate matching - Opt.6.: no enhancement specified

According with the agreements from RAN1# 111, for the purpose of studying the CORESET 0 configuration for transmission bandwidths <5 MHz for 3MHz and 5MHz channel bandwidth, two options have been considered: Opt1 re-using existing configuration for the minimum channel bandwidth available in legacy (i.e., 5MHz), or Opt2 consider a new CORESET#0 configuration.

For the CORESET 0 configuration for transmission bandwidths <5 MHz for 3MHz and 5MHz channel bandwidth, Opt1 is preferred (i.e., Existing configuration from table 13-1 in TS 38.213) as it is in line with the WID’s guidelines on adding “functional support based on existing design, without optimization”.

In line with “Opt1”, we have applied puncturing on CORESET 0 according with Table 6 which shows both the puncturing and general assumptions used for 2-OFDM symbol CORESET 0 and 3-OFDM symbol CORESET 0 evaluations.

Table 6: Puncturing assumptions and general assumptions on 2-OFDM & 3-OFDM symbol CORESET 0

| Puncturing assumptions |  | Parameter | Value |
| --- | --- | --- | --- |
| 3 RBs from bottom, 6 RBs from top | 2 RBs from bottom, 7 RBs from top | UE speed (km/h) | 40 |
| 2-OFDM symbol CORESET 0: AL8 PDCCH | 3-OFDM symbol CORESET 0: AL8 PDCCH | Channel model | TDL-C 300 ns |
|  |  | DCI payload | 40 bits + CRC |
|  |  | Aggregation level (AL) | 8 |
|  |  | CORESET | 2 or 3 symbols x 24 PRBs |
|  |  | CCE-to-REG bundle mapping | Interleaved, size 2 |
|  |  | Precoding | Precoder cycling at CCE level (REG bundle=6) |
|  |  | BLER target | 1% |

Using the simulation set-up in Table 6, and assuming the UE knows about the punctured region, the obtained results are shown in Figure 4.

| A) | B) |
| --- | --- |

Figure 4: BLER performance of a punctured and non-punctured PDCCH transmission mapped on CORESET 0: A) 2-OFDM symbol CORESET #0, and B) 3-OFDM symbol CORESET #0.

The puncturing applied on CORESET 0 results in a performance difference of:

- ~ 2.7dB with respect to a non-punctured curve in terms of SNR required to achieve 1% BLER. The percentage of punctured CCEs of a PDCCH candidate in this 2-OFDM symbol CORESET 0 example is 37.5%.

- ~ 2.1dB with respect to a non-punctured curve in terms of SNR required to achieve 1% BLER. The percentage of punctured CCEs of a PDCCH candidate in this 3-OFDM symbol CORESET 0 example is 31.25%.

The performance difference (i.e., ~ 2.7dB for 2-OFDM symbol and ~ 2.1dB for 3-OFDM symbol CORESET 0 respectively) was observed assuming the same power per sub-carrier for both non-punctured and punctured CORESET 0. That is, the performance difference was small even without using yet any compensation technique (e.g., power boosting).

Preserve the legacy CORESET 0 NR structure unmodified, applying puncturing on CORESET 0 when needed for transmission bandwidths <5 MHz for 3MHz and 5MHz channel bandwidths.

### 3.2.3	View on recovery techniques for PDCCH on CORESET 0

As mentioned earlier, RAN1 reached the following agreement touching upon the potential usage of recovery techniques when NR PDCCH on CORESET 0 legacy structure does not fit entirely into the transmission bandwidth: “Study whether and how to recover PDCCH detection performance of CORESET#0 for transmission bandwidths of <5MHz for 3MHz and 5MHz channel bandwidth. The following options are considered Opt.1: Power boosting Opt.2: Non-interleaved CCE-to-REG mapping Opt.3: A new interleaver to ensure PDCCH is fully mapped in the spectrum Opt.4: New aggregation level(s) for fit in the spectrum Opt.5: PDCCH rate matching Opt.6.: no enhancement specified”. Below we present an overall one-on-one comparison among the considered options:

Table 6: Overall comparison among the options considered as recovery techniques for PDCCH on CORESET 0

|  | Candidate Recovery Techniques for PDCCH on CORESET 0 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Opt.1: Power boosting | Opt.2: Non-interleaved CCE-to-REG mapping | Opt.3: A new interleaver to ensure PDCCH is fully mapped in the spectrum | Opt.4: New aggregation level(s) for fit in the spectrum | Opt.5: PDCCH rate matching | Opt.6: no enhancement specified |
| Highlevel description | If the total power that otherwise is used on the legacy NR structure were now concentrated into the unpunctured region, then the punctured transmission would have a higher PSD, creating a power boosting effect that would improve the performance. | This option is about introducing “Non-interleaved CCE-to-REG mapping” for CORESET 0. | This option is about introducing a “new interleaver to ensure PDCCH is fully mapped in the spectrum”. | This option is about introducing AL(s) other than 1, 2, 4, 8, 16 which are the ones available today in the technical specifications. | In our understanding, “rate matching” can be used to avoid/skip the RBs that are to be punctured, as to map PDCCH only on the unimpacted CORESET 0 RBs. This technique is somehow equivalent to changing the CORESET 0 size. | Since the performance difference between a punctured and a non-punctured evaluation is not that large (e.g., ~ 2.7dB with 2-OFDM-symbols & ~ 2.1dB with 3-OFDM-symbols), and PDCCH on CORESET 0 is not foreseen to be a bottle-neck channel, then the option of not introducing any performance recovery technique seems to be also a feasible choice. |
| Pros | - It is foreseen that the power boosting will end-up fully or almost-fully compensating for the performance difference observed between the punctured and non-punctured evaluation. - No extra cost (i.e., the power boosting comes from a total power already used today). - No specification impact foreseen other than perhaps having to set a max power limit (or any band-related exception) in the RAN4 specs. | - A non-interleaved CCE-to-REG mapping helps to maximize the number candidate PDCCH that will be located into the transmission bandwidth. Nonetheless, there are other solutions that could achieve the same goal with less specification impacts, for example using positive and negative values in “Offset (RBs) ” as to shift dowards and updwards CORESET 0 with respec to SSB. | - As the description says, the main benefit would be “to ensure PDCCH is fully mapped in the spectrum”. | - The new AL(s) can be tailored-made to the reduced transmission bandwidths as to avoid a performance lose. | - Upon identifying the maximum aggregation level that can be supported into the rate-matched CORESET 0, then the PDCCH candidates can be fully transmitted. | - No specification impacts at all. - Opt.6 (i.e., no recovery technique) will anyway hold in case any of the other options (if introduced) were not applicable in certain scenarios. |
| Cons | - It may not be possible to apply a power boosting in all scenarios (e.g., in a co-located NR-R&GSM-R scenario), but yet again it will depend on RAN4 requirements. | - The technical specification states for CORESET 0 that “the UE may assume interleaved mapping”, such a fundamental principle for CORESET 0 will be changed if Opt.2 were adopted. - Changing the CCE-to-REG mapping from interleaved to non-interleave for CORESET 0 does not seem to be in line with the WID’s guidance of adding “functional support based on existing design, without optimization”. | - In legacy the interleaver is defined by a function that depends on several variables (i.e.,r,C,c,nshift,NREGCORESET, L) that in turn depend on other definitions. For introducing a new interleaver, similar dependencies are expected to be kept, thus the specification impact is not foreseen to be minor. - Does not seem to be in line with the WID’s guidance of adding “functional support based on existing design, without optimization”. | - Introducing new AL(s) is foreseen to result in not nefligible specification impacts, which does not seem to be in line with the guidance of the WID. - New AL(s) will likely be something in between the existing ones at the lower-end, hence the gain may not be that significant with respect to some of the existing choices. | - Only the legacy smallest aggregation levels could be supported. - No “rate matching” existing in the NR specs are foreseen to be re-usable, especially if a sync-raster with finer granularity is to be introduced since CORESET 0 is related to it through “OffsetSSB” and “Offset(RBs)”. - Compared to other solutions, “rate matching” will likely result in a non-negligible specification impact. | - There will be a performance difference between a punctured and a non-punctured transmission. Such a performance difference won’t be compensated (at least not through a technique impacting the technical specifications) |

The performance difference between a punctured and a non-punctured PDCCH on CORESET 0 was found not to be large (~ 2.7dB for 2-OFDM symbol and ~ 2.1dB for 3-OFDM symbol and @ 1% BLER). Moreover, CORESET0 is not foreseen to be a bottle-neck channel link budget-wise. Thus, accounting for the above a recovery technique with minor specification impact (Opt1 which may only require setting a limit in RAN4 specs) or no specification impact at all (Opt6) is preferred.

iff the concern on mapping PDCCH on CORESET 0 in a reduced bandwidth were about maximizing the number of PDCCH candidate that will fall into the usable spectrum, then RAN1 can consider a simpler solution such as adding more values (e.g., negative ones) to the existing “kSSB” and/or “Offset(RBs)” as to add more freedom for locating CORESET 0 with respect to SSB.

## 3.3	CSI-RS/TRS in NR spectrum “less than 5 MHZ for FR1”

The Information Element (IE) CSI-FrequencyOccupation is used to configure the frequency domain occupation of a channel state information measurement resource (e.g. NZP-CSI-RS-Resource, CSI-IM-Resource) [7]. The following descriptions can be highlighted:

- nrofRBs: Number of PRBs across which this CSI resource spans. Only multiples of 4 are allowed. The smallest configurable number is the minimum of 24 and the width of the associated BWP. If the configured value is larger than the width of the corresponding BWP, the UE shall assume that the actual CSI-RS bandwidth is equal to the width of the BWP.

- startingRB: PRB where this CSI resource starts in relation to common resource block #0 (CRB#0) on the common resource block grid. Only multiples of 4 are allowed (0, 4, ...).

For CSI-RS, the smallest configurable number of RBs is 24. Moreover, “If the configured value is larger than the width of the corresponding BWP, the UE shall assume that the actual CSI-RS bandwidth is equal to the width of the BWP”.

In our understanding, the “BWP” mentioned in the previous observation is not the “Initial BWP” since “CSI-RS/TRS” is transmitted in connected-mode, hence the “BWP” should be flexible enough as to fit “CSI-RS/TRS” in a reduced bandwidth using the “MIN function” between the “number of RBs” and the “BWP”.

Conclusion: No puncturing is needed for “CSI-RS/TRS,” hence “CSI-RS/TRS” can be used “for functional support based on existing design, without optimization” on transmission bandwidths of <5MHz for 3MHz and 5MHz channel bandwidth.

## 3.4	PUCCH in NR spectrum “less than 5 MHZ for FR1”

In relation with PUCCH, the following conclusion was reached in RAN1# 111:

Conclusion No enhancements are needed for PUCCH to support transmission bandwidths of <5MHz for 3MHz and 5MHz channel bandwidth, - FFS: the necessity for PUCCH FH disabling.

Based on the above conclusion, what remains to be addressed is the FFS on “the necessity for PUCCH FH disabling”. For illustration purposes below we depict PUCCH using frequency hopping (FH) and PUSCH being simultaneously transmitted on a 15 RBs transmission bandwidth.

Figure 5: PUCCH using frequency hopping (FH) and PUSCH being simultaneously transmitted on a 15 RBs transmission bandwidth.

Towards taking a decision on “the necessity for PUCCH FH disabling”, we should take the following aspects into account:

- In our understanding, PUCCH frequency hopping is enabled by default for PUCCH of Msg4.

- For the support of a reduced transmission bandwidth consisting of e.g., 15 RBs, if PUCCH and PUSCH happen to be simultaneously transmitted (which only occurs during the random-access procedure), PUSCH can use up to 14 RBs if PUCCH FH is disabled, and up to 13 RBs is PUCCH FH is enabled. The difference is +/- 1RB for PUSCH depending on whether PUCCH FH is disabled/enabled.

- It has been mentioned that for the support of a reduced transmission bandwidth consisting of e.g., 15 RBs, PUCCH FH should be disabled just as it was done in Rel-17 RedCap, however RedCap decided to disable FH to avoid PUSCH resource fragmentation because the carrier bandwidth is large (e.g., 100 MHz) and there are restrictions on the placement of the RedCap UE channel bandwidth, which are conditions that do not apply here and therefore “PUSCH resource fragmentation” which motivated the FH disabling is a non-existing problem for this Rel-18 Work Item.

Account for the above, our view is as follows:

About the FFS on “the necessity for PUCCH FH disabling”, no “necessity” has been identified since different than RedCap there is no “PUSCH resource fragmentation issue”. Moreover, PUCCH frequency hopping is enabled by default for PUCCH of Msg4, and the difference between enabling/disabling PUCCH FH is +/- 1 RB for PUSCH.

Based on “Observation 13,” and in line with the WID about “Identify and specify necessary minimum changes to … PUCCH … for functional support based on existing design, without optimization.” There is no “necessity for PUCCH FH disabling”.

## 3.5	PRACH in NR spectrum “less than 5 MHZ for FR1”

For PRACH, the following conclusion and agreement were reached in RAN1# 111:

Conclusion No enhancements are required for PRACH to operate NR on transmission bandwidths of <5MHz for 3MHz and 5MHz channel bandwidth. - Note: PRACH formats and configurations not fitting into the transmission BW are not applicable Agreement Short PRACH formats with 15kHz SCS, and long PRACH formats with 1.25kHz SCS are supported for transmission bandwidths <5 MHz for 3MHz and 5MHz channel bandwidth.

Given the conclusion and agreement on PRACH to operate on transmission bandwidths of <5MHz for 3MHz and 5MHz channel bandwidth, for the moment no other clarification or follow-up touching upon PRACH is foreseen to be needed.

# 4	Other considerations

## Early indication (UE knowing which RBs are used for transmission)

Towards performing evaluations, it has been agreed “RAN1 assume that the UE could know which RBs are used for SSB transmission after PSS/SSS is detected for evaluation and analysis.”, but it has also been noted that “Note: it does not mean indication signaling is needed.” In relation with it, we have the following observations:

It is important to mention that if both the location and number of PRBs to be punctured from the 20-PRBs SSB structure were invariant, then such a fully deterministic scheme won’t require an “early indication”.

On the other hand, if either the location and/or the number of PRBs to be punctured from the 20-PRBs SSB structure were to vary, then (unless the UE were to run some hypothesis) there will be a need for introducing an early indication.

      - The complication with it is that for the UE to know which RBs are used for transmission (i.e., the unpunctured RBs), the indication cannot be carried on PBCH (or at least not only in it) since PBCH per-se is to be punctured, hence an earlier indication will be needed (e.g., on PSS/SSS). Thus, as mentioned in earlier sections, a fully deterministic puncturing on the 20-PRBs of SSB is preferred which facilitates the sync-raster design, RAN4 requirements, and won’t require the UE to hypothesize or receiving signalling to indicate the punctured region.

## Link-budget

The Rel-18 WID has listed some physical channels and signals (i.e., SSB, PDCCH, CSI-RS/TRS, PUCCH, and PRACH) to “Identify and specify necessary changes to NR physical layer with minimum specification impact to operate in spectrum allocations from approximately 3 MHz up to below 5 MHz”. On this matter, other NR physical channels and signals not requiring e.g., puncturing may need to be investigated as well (e.g., link-budget wise) as to discard any potential “bottle neck” at the moment of operating in “spectrum allocations from approximately 3 MHz up to below 5 MHz”.

In addition to SSB, PDCCH, CSI-RS/TRS, PUCCH, and PRACH, other NR physical channels and signals not requiring puncturing may need to be investigated as well (e.g., link-budget wise) as to discard any potential “bottle neck” at the moment of operating in “spectrum allocations from approximately 3 MHz up to below 5 MHz”.

# 5	Conclusion

Based on the discussion in the previous section we made the following observations:

Observation 1	For SSB, RAN1 agreed that “a subset of PRBs of 20-PRB PBCH are used for PBCH transmission if the transmission BW of a channel is less than 20PRBs,” keeping as “FFS which PRBs are used and how to use the PRBs”. Resolving this FFS should be prioritized, since likely it will be needed as complementary input for RAN4 towards completing the sync-raster design.

Observation 2	The puncturing applied on SSB preserving 15RBs, 14RBs, or 12RBs results respectively in a performance difference of ~1.75dB, ~2.5dB, and ~3.6dB with respect to a non-punctured curve in terms of SNR required to achieve 1% BLER.

Observation 3	The performance difference was observed assuming the same power per sub-carrier for both the non-punctured and punctured PBCH. The performance difference was small (i.e., less than 2dB when preserving 15RBs unpunctured) even without using yet any compensation technique (e.g., power boosting).

-	Preserving unpunctured less than 15 SSB RBs may result in not fulfilling RRM requirements in RAN4 RRM (e.g. UE shall detect the cell with the assumption SNR=-6dB and 90% of success rate, including RF impairment margin), thus performance-wise preserving unpunctured 15RBs is recommended.

Observation 4	Towards preserving 15RBs unpunctured, the 5RBs to be punctured with respect to the 20RBs of the legacy SSB structure can in principle be obtained from either:

a) 4 RBs punctured from top, 1 RB punctured from bottom.

b) 1 RB punctured from top, 4 RBs punctured from bottom.

c) 3 RBs punctured from top, 2 RBs punctured from bottom.

d) 2 RBs punctured from top, 3 RBs punctured from bottom.

In the end, only one of the above alternatives should be chosen as to have a fully deterministic/invariant puncturing on the 20-PRBs of SSB, which will facilitate the sync-raster design, RAN4 requirements, and won’t require the UE to hypothesize or receiving signalling (e.g., early indication) to indicate the punctured region.

Observation 5	The number of unpunctured/preserved PRBs overall determine the BLER performance of SSB regardless of the location of the punctured subcarriers/PRBs within the legacy 20-PRBs structure, hence alternative “a)” where 4 RBs are punctured from top and 1 RB is punctured from bottom is preferred because of the following reason:

-	If the 5 PRBs to be punctured from the 20 PRBs SSB structure, they were mostly taken from to the top (e.g., 4 RBs) and minimally from the bottom (e.g., 1 RB), then existing offsets (i.e., “kSSB” and “Offset (RBs)”) and their legacy reference points could be kept unmodified for functional support (see Figure 3).

Observation 6	PDCCH is mapped to a specific CORESET. Thus, the highest “Aggregation level” that can be used depends on the resource blocks in the frequency domain according with “NRBCORESET” and symbols “NsymbCORESET” in the time domain configured for a CORESET.

Observation 7	The puncturing on PDCCH in CORESET 0 will be determined by the puncturing applied on SSB, and the offset parameters “kssb” and “Offset (RBs)”.

Observation 8	For the CORESET 0 configuration for transmission bandwidths <5 MHz for 3MHz and 5MHz channel bandwidth, Opt1 is preferred (i.e., Existing configuration from table 13-1 in TS 38.213) as it is in line with the WID’s guidelines on adding “functional support based on existing design, without optimization”.

Observation 9	The puncturing applied on CORESET 0 results in a performance difference of:

	~ 2.7dB with respect to a non-punctured curve in terms of SNR required to achieve 1% BLER. The percentage of punctured CCEs of a PDCCH candidate in this 2-OFDM symbol CORESET 0 example is 37.5%.

	~ 2.1dB with respect to a non-punctured curve in terms of SNR required to achieve 1% BLER. The percentage of punctured CCEs of a PDCCH candidate in this 3-OFDM symbol CORESET 0 example is 31.25%.

Observation 10	The performance difference (i.e., ~ 2.7dB for 2-OFDM symbol and ~ 2.1dB for 3-OFDM symbol CORESET 0 respectively) was observed assuming the same power per sub-carrier for both non-punctured and punctured CORESET 0. That is, the performance difference was small even without using yet any compensation technique (e.g., power boosting).

Observation 11	For CSI-RS, the smallest configurable number of RBs is 24. Moreover, “If the configured value is larger than the width of the corresponding BWP, the UE shall assume that the actual CSI-RS bandwidth is equal to the width of the BWP”.

Observation 12	In our understanding, the “BWP” mentioned in the previous observation is not the “Initial BWP” since “CSI-RS/TRS” is transmitted in connected-mode, hence the “BWP” should be flexible enough as to fit “CSI-RS/TRS” in a reduced bandwidth using the “MIN function” between the “number of RBs” and the “BWP”.

Observation 13	About the FFS on “the necessity for PUCCH FH disabling”, no “necessity” has been identified since different than RedCap there is no “PUSCH resource fragmentation issue”. Moreover, PUCCH frequency hopping is enabled by default for PUCCH of Msg4, and the difference between enabling/disabling PUCCH FH is +/- 1 RB for PUSCH.

Observation 14	Given the conclusion and agreement on PRACH to operate on transmission bandwidths of <5MHz for 3MHz and 5MHz channel bandwidth, for the moment no other clarification or follow-up touching upon PRACH is foreseen to be needed.

Observation 15	It is important to mention that if both the location and number of PRBs to be punctured from the 20-PRBs SSB structure were invariant, then such a fully deterministic scheme won’t require an “early indication”.

Observation 16	On the other hand, if either the location and/or the number of PRBs to be punctured from the 20-PRBs SSB structure were to vary, then (unless the UE were to run some hypothesis) there will be a need for introducing an early indication.

	The complication with it is that for the UE to know which RBs are used for transmission (i.e., the unpunctured RBs), the indication cannot be carried on PBCH (or at least not only in it) since PBCH per-se is to be punctured, hence an earlier indication will be needed (e.g., on PSS/SSS). Thus, as mentioned in earlier sections, a fully deterministic puncturing on the 20-PRBs of SSB is preferred which facilitates the sync-raster design, RAN4 requirements, and won’t require the UE to hypothesize or receiving signalling to indicate the punctured region.

Observation 17	In addition to SSB, PDCCH, CSI-RS/TRS, PUCCH, and PRACH, other NR physical channels and signals not requiring puncturing may need to be investigated as well (e.g., link-budget wise) as to discard any potential “bottle neck” at the moment of operating in “spectrum allocations from approximately 3 MHz up to below 5 MHz”.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Prioritize resolving the FFS about the “subset of PRBs of 20-PRB PBCH are used for PBCH transmission if the transmission BW of a channel is less than 20PRBs” (i.e., “FFS which PRBs are used and how to use the PRBs”).

Proposal 2	Preserve the legacy SSB NR structure unmodified, applying puncturing on SSB when required for transmission bandwidths <5 MHz for 3MHz and 5MHz channel bandwidths.

-	For transmission bandwidths <5MHz for 3MHz channel bandwidth, where puncturing is required to be applied on SSB:

o	“a subset of 15PRBs of 20-PRB PBCH are used for PBCH transmission”.

Proposal 3	Adopt a deterministic puncturing, where with respect to the 20-PRB SSB legacy structure, 4 PRBs are punctured from the top and 1 PRB is punctured from bottom.

-	This allows keeping a minimum distance (i.e., 1RB) between the bottom of the 15-PRB SSB punctured structure and the 20-PRB SSB legacy structure, which aims at minimizing the impact on legacy offsets (i.e., “kSSB” and “Offset (RBs)”).

Proposal 4	To minimize the specification impacts from puncturing, the puncturing applied on SSB is reflected through adding a “Note” that relies on Table 7.4.3.1-1 in TS 38.211 which describes the legacy SSB structure:

-	Note: When the UE is configured with LessThan5MHzFR1, the UE is only expected to receive subcarrier number k = 12, 13, … 191 as described in Table 7.4.3.1-1.

Proposal 5	Keep “subcarrier 0 of the first resource block of the SS/PBCH block” as a hypothetical/virtual reference for applying kSSB. That is, since Table 7.4.3.1-1 reflects the legacy 20-PRB SSB structure and the puncturing is fully deterministic, then subcarrier 0 can be kept as a virtual/hypothetical reference. The following statement can be appended to the “Note” describing the puncturing:

-	The UE may continue assuming subcarrier 0 of the first resource block of the SS/PBCH block as a hypothetical reference for applying kSSB

Proposal 6	Existing offsets (i.e., “kSSB” and “Offset (RBs)”) are kept unmodified for functional support.

-	If the UE continues assuming subcarrier 0 of the first resource block of the SS/PBCH block as a hypothetical reference applying “kSSB”, and if on the 20-PRB SSB, 4 PRBs were punctured from the top and 1 PRB were punctured from bottom as to keep a minimum distance (i.e., 1RB) between the bottom of the 15-PRB SSB punctured structure and the 20-PRB SSB legacy structure, then both “kSSB” and “Offset (RBs)” can be kept used as in legacy for functional support (i.e., there is no strict need for optimizing, since offsets are functional/usable).

Proposal 7	The performance difference between a punctured and a non-punctured SSB was found to be small (less than 2dB @ 1% BLER when 15-PRBs are kept unpunctured). Moreover, SSB is not foreseen to be a bottle-neck channel link budget-wise. Thus, accounting for the above a recovery technique with minor specification impact (Opt1 which may only require setting a limit in RAN4 specs) or no specification impact at all (Opt6) is preferred.

Proposal 8	Preserve the legacy CORESET 0 NR structure unmodified, applying puncturing on CORESET 0 when needed for transmission bandwidths <5 MHz for 3MHz and 5MHz channel bandwidths.

Proposal 9	The performance difference between a punctured and a non-punctured PDCCH on CORESET 0 was found not to be large (~ 2.7dB for 2-OFDM symbol and ~ 2.1dB for 3-OFDM symbol and @ 1% BLER). Moreover, CORESET0 is not foreseen to be a bottle-neck channel link budget-wise. Thus, accounting for the above a recovery technique with minor specification impact (Opt1 which may only require setting a limit in RAN4 specs) or no specification impact at all (Opt6) is preferred.

Proposal 10	iff the concern on mapping PDCCH on CORESET 0 in a reduced bandwidth were about maximizing the number of PDCCH candidate that will fall into the usable spectrum, then RAN1 can consider a simpler solution such as adding more values (e.g., negative ones) to the existing “kSSB” and/or “Offset(RBs)” as to add more freedom for locating CORESET 0 with respect to SSB.

Proposal 11	Conclusion: No puncturing is needed for “CSI-RS/TRS,” hence “CSI-RS/TRS” can be used “for functional support based on existing design, without optimization” on transmission bandwidths of <5MHz for 3MHz and 5MHz channel bandwidth.

Proposal 12	Based on “Observation 13,” and in line with the WID about “Identify and specify necessary minimum changes to … PUCCH … for functional support based on existing design, without optimization.” There is no “necessity for PUCCH FH disabling”.

# References

RP-213603, “WID on IoT NTN enhancements,” RAN #95e, Electronic Meeting, March 17th – 23rd, 2022.

Chairman Notes RAN1#111, Agenda Item 9.16.1, Toulouse, France, August 14th – 18th, 2022.

R1- 2212919, Liaison Statement: “LS on NR support for dedicated spectrum less than 5MHz for FR1,” Toulouse, France, August 14th – 18th, 2022.

R1-2211572, “Enhancements to operate NR on dedicated spectrum less than 5 MHz,” RAN1” 111, Toulouse, France, August 14th – 18th, 2022.

TS 38.101-4, “NR; User Equipment (UE) radio transmission and reception; Part 4: Performance requirements,” 3GPP Technical Specification, version 17.7.0.

TS 38.211, “NR; Physical channels and modulation”, 3GPP Technical Specification, version 17.2.0.

TS 38.331, “NR; Radio Resource Control (RRC); Protocol specification”, 3GPP Technical Specification, version 17.1.0.