3GPP TSG RAN WG1 #112	                                                                                R1-2301290

Athens, Greece, February 27th – March 3rd, 2023

Agenda Item:		9.13.2

Source:				Samsung

Title:					Receiver architecture for LP-WUS

Document for:	    Discussion

Introduction

- Identify evaluation methodology (including the use cases) & KPIs [RAN1]

  - Primarily target low-power WUS/WUR for power-sensitive, small form-factor devices including IoT use cases (such as industrial sensors, controllers) and wearables

    - Other use cases are not precluded

- Study and evaluate low-power wake-up receiver architectures [RAN1, RAN4]

- Study and evaluate wake-up signal designs to support wake-up receivers [RAN1, RAN4]

- Study and evaluate L1 procedures and higher layer protocol changes needed to support the wake-up signals  [RAN2, RAN1]

- Study potential UE power saving gains compared to the existing Rel-15/16/17 UE power saving mechanisms, the coverage availability, as well as latency impact of low-power WUR/WUS. System impact, such as network power consumption, coexistence with non-low-power-WUR UEs, network coverage/capacity/resource overhead should be included in the study [RAN1]

    - Note: The need for RAN2 evaluation will be triggered by RAN1 when necessary.

In the approved revised SI for low-power Wake-up Signal and Receiver for NR [1], one important direction is to study and evaluate low-power wake-up receiver architectures.

This contribution discusses aspects related to low-power wake-up receiver architectures.

The design criteria for receiver architecture

According to the SID shown as above, new signals will be introduced to be used as LP-WUS to further reduce the power consumption compared to the existing Rel-15/16/17 UE power saving mechanisms. To further prolong the battery life, a separate receiver will be designed to monitor the new wake-up signal with ultra-low power consumption. When the WUR detects the wake-up signal, it will trigger the main radio on in order for the main radio to receive signals and/or channels. Otherwise, the main radio will maintain off state (from Rel-16) or off as a new state which can be further discussed during SI (e.g., ultra deep sleep).

When a new receiver architecture is taken into account in NR for LP-WUS, the coverage should be evaluated. For increasing the application of LP-WUS, the coverage of LP-WUS should be consistent with the legacy signal of the main receiver. That is to say, the UE in the edge of the cell can also receive the LP-WUS to reduce their power consumption. Therefore, amplifier (like low noise amplifier) can be adopted when designing the WUR.

Proposal 1: The coverage of LP-WUS should be consistent with the legacy signal of the main receiver.

To match the coverage of LP-WUS with one of legacy signal of the main receiver (e.g. PDCCH), the WUR architecture maybe not achieve maximum energy efficiency. However, considering that the WUR is assumed to be on to monitor LP-WUS when the main receiver is off, the power consumption should still be much lower than the main radio.

Proposal 2: The power consumption of the separate WUR should be reduced dramatically compared with main radio.

Since out of synchronization has a great impact on OOK signal detection, the synchronization issue should be further considered when designing the WUR. Besides, when LP-WUS is transmitted based on OFDM symbol by OOK modulation, the interference between LP-WUS and other signals cannot be ignored. In this case, Phase-locked loop (PLL)/ Frequency Locked Loop (FLL) and/or Low pass filter (LPF) and/or band-pass filter (BPF) can be essential at the WUR.

Proposal 3: Study synchronization and interference issue in LP-WUS reception.

According to the survey [2]-[16], we found that the data rate of LP-WUS can range from 0.3 to 200 kbps. In addition, the sensitivity can reach -80 dBm in most papers, and the power consumption ranges from 4.5 uW to 578 mW. It should be noticed that there is a tradeoff among sensitivity, data rate and power consumption in the process of WUR designing. If we pursue higher sensitivity and data rate, it will lead to higher power consumption then.

Proposal 4: Study the impact of the tradeoff among sensitivity, data rate and power consumption in the process of WUR designing.

The basic structure for WUR

For hardware module of the wake-up receiver used for signal detecting and processing, a separate receiver for reception of LP-WUS in addition to main NR radio transceiver can be a basic UE architecture for the study. During the RAN1 110b-e meeting, the study on three types of receiver architectures for LP-WUR, potentially combining with OOK or FSK modulations, was agreed as following:

Agreement Study the architecture with RF envelope detection based on at least the following diagram for LP-WUR. - The RF signal is converted into baseband signal directly via an RF envelope detector. - There is no Local Oscillator (LO) and no Phase-Locked Loop (PLL). - 1-bit or multi-bit ADC is applied. - Some component(s), e.g., RF LNA and/or BB AMP, can be optionally applied. - High-Q matching network and/or mi] can be used to suppress adjacent channel interference or interference from legacy NR signals and/or other LP WUS on adjacent subcarriers. - FFS the support of band and/or carrier tuning Agreement Study the heterodyne architecture with IF envelope detection based on at least the following diagram for LP-WUR. - The RF signal is down converted into IF signal via an RF mixer with a LO. The IF signal is converted into baseband signal via an IF envelope detection. - There may be one or multiple IF stages depending on design. - The choice of the LO is one of the major factors that determines the power consumption. - Lower power consumption can be achieved by relaxing the accuracy and stability requirements of the LO. However, such increased frequency offset and phase noise should be taken into account in the design and evaluation. - FLL (frequency locked loop) may replace PLL for non-coherent detection. - 1-bit or multi-bit ADC is applied. - High-Q matching network and/or RF BPF and/or IF BPF [and/or BB LPF] can be used to suppress adjacent channel interference or interference from legacy NR signals and/or other LP WUS on adjacent subcarriers. - Some component(s), e.g., RF LNA and/or IF AMP and/or BB AMP, can be optionally applied. - Image rejection filter or an image rejection mixer is required. - FFS the support of band and/or carrier tuning - FFS the choice of IF frequency range Agreement Study the homodyne/zero-IF architecture with baseband envelope detection based on at least the following diagram for LP-WUR. - The RF signal is directly down converted into baseband signal via an RF mixer with a LO. - Baseband envelope detection can be done either in analog domain or in digital domain depending on design, which is not explicitly shown in the diagram. - The choice of the LO is one of the major factors that determines the power consumption. - Lower power consumption can be achieved by relaxing the accuracy and stability requirements of the LO. However, such increased frequency offset and phase noise should be taken into account in the design and evaluation. - FLL (frequency locked loop) may replace PLL for non-coherent detection. - 1-bit or multi-bit ADC is applied. - High-Q matching network and/or RF BPF and/or BB BPF [and/or BB LPF] can be used to suppress adjacent channel interference or interference from legacy NR signals and/or other LP WUS on adjacent subcarriers. - No image rejection filter is required. - Some component(s), e.g., RF LNA and/or BB AMP, can be optionally applied. - FFS the support of band and/or carrier tuning Agreement Further study the receiver architectures for FSK, with two examples shown below: - Example 1: parallel OOK receivers and a comparator circuit, e.g., - Each path can be implemented using either of [the architecture with RF envelope detection,] heterodyne architecture with IF envelope detection, or homodyne/zero-IF architecture with baseband envelope detection. - Example 2: using an FM-to-AM detector [or an FM detector] - Alt 1: Use an analog FM-to-AM detector with a similar architecture as for OOK (e.g. heterodyne or zero-IF architecture), except that the envelope detector is replaced by a FM-to-AM detector. - Analog FM-to-AM detector can be implemented at least in BB or low-IF. - Alt 2: Use a FM-to-AM detector [or an FM detector] implemented in digital domain after ADC, with a heterodyne or zero-IF architecture. - Digital FM-to-AM detector implementation can be considered as part of digital baseband processing. - Here is an example of using zero-IF architecture: - The FM-AM detector can be implemented using a frequency discriminator, which converts frequency variations into amplitude changes. It can be implemented in either analog domain (as in Alt 1) or digital domain (as in Alt 2). - One example, as shown in the figure below, is a conventional quadrature FM discriminator. It multiplies received frequency modulated signal with a phase shifted version, followed by a low pass filter. The amplitude of the output signal is proportional to the frequency of the input signal. - Note: Other architectures are not precluded.

  - The receiver architecture with FSK modulation

For Example 1 of the receiver architecture with FSK modulation, since there are two paths for signaling reception, and each path can be implemented by one of the architectures with RF envelope detection, heterodyne architecture with IF envelope detection, or homodyne/zero-IF architecture with baseband envelope detection, the power consumption is much higher than the corresponding architecture with OOK modulation.  For Example 2 of the receiver architecture with FSK modulation, extra component, i.e., FM-to-AM detector or an FM detector is needed to demodulate the LP-WUS at the UE side, which also brings additional power consumption. Since FSK modulation is easily affected by frequency offset, the performance gain is uncertain with more power consumption. Therefore, the receiver architecture with FSK modulation can be deprioritized in our opinion.

Proposal 5: Considering the uncertain performance gain with more power consumption, the receiver architecture with FSK modulation can be deprioritized.

  - OFDMA-based receiver architecture

OFDMA-based receiver architecture was also discussed to be one of the possible architectures for LP-WUS reception in the previous meeting. It is undeniable that the OFDMA-based receiver architecture has higher power consumption, but the noise figure and the sensitivity/coverage is much better than the agreed architectures with OOK/FSK modulation. Considering that the main power consumption comes from the LNA and the LO/PLL, low accuracy hardware modules can be used to reduce the power consumption of the OFDMA-based receiver architecture. In addition, if sequence based WUS is adopted, non-coherent detection in time domain can also be applied to further reduce the power consumption. Therefore, we suggest that the tradeoff between the performance gain and the power consumption should be carefully evaluated for the OFDMA-based receiver architecture.

Proposal 6: The tradeoff between the performance gain and the power consumption should be carefully evaluated for the OFDMA-based receiver architecture.

  - The receiver architecture with OOK modulation

In RAN1#111 meeting, the following agreements on WUR architecture were achieved:

Agreement The following observation to be captured in TR38.869: For the architecture with RF envelope detection, - It can achieve relatively low power consumption due to the removal of LO/PLL. - Interference suppression for adjacent channel interference requires very high-Q matching network and/or RF BPF, which is challenging due to the high Q values and may require off-chip components. - Interference suppression for interference from legacy NR signals and/or other LP WUS on adjacent subcarriers, if performed in RF, requires very high-Q matching network and/or RF BPF, which is challenging due to the high Q values and may require off-chip components. - The support of multiple bands and/or carriers may require multiple high-Q matching networks and/or RF BPFs or multiple off-chip components. - RF LNA can be applied to improve sensitivity, with the cost of additional power consumption. - The noise figure can be relatively high. Agreement The following observation to be captured in TR38.869: For homodyne/zero-IF architecture with baseband envelope detection, - For the support of band and/or carrier tuning, the band and/or carrier tuning can be achieved via tuning the LO frequency. - The matching network and RF BPF for LP WUR may or may not reuse those of the main radio. - It is more effective and less complex to use BB BPF/LPF instead of high-Q matching network and/or RF BPF to suppress adjacent channel interference or interference from legacy NR signals and/or other LP WUS on adjacent subcarriers. - Using FLL instead of PLL consumes less power, but it may result in larger frequency error. - It can suffer from LO leakage (DC offset) and flicker (1/f) noise. The impact may be alleviated by using BB BPF in some cases. - RF LNA can be applied to improve sensitivity, with the cost of additional power consumption. - The baseband envelope detection can be done in either analog domain (before ADC) or digital domain (after ADC). Agreement The following observation to be captured in TR38.869: For heterodyne architecture with IF envelope detection, - For the support of band and/or carrier tuning, the band and/or carrier tuning can be achieved via tuning the LO frequency. - The matching network and RF BPF for LP WUR may or may not reuse those of the main radio. - It is more effective and less complex to use IF BPF instead of high-Q matching network and/or RF BPF to suppress adjacent channel interference or interference from legacy NR signals and/or other LP WUS on adjacent subcarriers. - Using FLL instead of PLL consumes less power, but it may result in larger frequency error. - The IF frequency can be properly selected to avoid LO leakage (DC offset) and flicker (1/f) noise. - Image rejection can be done via either image rejection filter or image rejection mixer. - Image rejection filter can be done in either RF or IF, which may require high-Q filter. - Image rejection mixer requires two-branch (I/Q) mixing with good matching in gain and phase, which consumes additional power. - RF LNA and/or IF AMP can be applied to improve sensitivity, with the cost of additional power consumption.

For the architecture with RF envelope detection, the LP-WUS is first received by the RF end via the antenna and then passes through the matching network that filters and boosts the incoming LP-WUS. After input matching, the signal passes through the RF BPF and low noise amplifier (LNA) for increasing the sensitivity and the coverage of the receiver by amplifying weak signals while meeting noise requirements. The LNA dominates in terms of power consumption, and the value can be 27~35 μW. Then, an RF envelope detector (ED) is adopted for signal detection and conversion to baseband signal to make the circuit simpler and energy efficient. The power consumption of RF ED can be 1.5~6 μW. To further reduce the interference between other signals and/or channels and the LP-WUS in frequency domain, and to improve the detection accuracy of LP-WUS, BB end should be included in the separate WUR in our view. Therefore, BB AMP and BB LPF can be used to amplify the signal and reduce interference separately. In summary, although the architecture with RF envelope detection can achieve the lowest power consumption compared with the other two architectures, the receiver sensitivity or coverage of this architecture cannot match with other NR signals and channels, like the coverage of PDCCH. In addition, the most critical problem is since LO is discarded in this architecture, the LP-WUS should be bounded with specific frequency, e.g. ARFCN. Multiple high-Q matching network and/or RF BPFs can be adopted to support of multiple bands and/or carriers transmission, but the higher Q values are challenge for implementation and it may need extra off-chip components. As a consequence, the architecture with RF envelope detection can be deprioritized.

Proposal 7: Considering the feasibility and the performance requirements of WUR architecture, the receiver architecture with RF envelope detection can be deprioritized.

Compared with the architecture with RF envelope detection, the heterodyne architecture with IF envelope detection uses mixer, local oscillator, IF AMP, IF BPF and IF ED to replace RF ED to further reduce the interference. With the highest power consumption, the heterodyne architecture with IF envelope detection can achieve best interference resiliency and best sensitivity in the three agreed architectures with OOK modulation. To solve the image interference issue in the heterodyne architecture with IF envelope detection, image rejection filter or image rejection mixer can be used with additional power consumption.

For the homodyne/zero-IF architecture with baseband envelope detection, the RF envelope detector (ED) in the architecture with RF envelope detection is replaced by mixer and LO directly. Since it is not switched to IF for signal processing, the power consumption is much lower than the heterodyne architecture with IF envelope detection. But it should be noticed that the interference resiliency and sensitivity of this architecture will be reduced accordingly. Besides, DC offset and flicker noise will affect the detection performance, which can be solved by using BB BPF with additional power consumption. In summary, the final WUR architecture should be evaluated based on a tradeoff between power consumption and the performance.

Proposal 8: Considering the interference resiliency and sensitivity of LP-WUS, the heterodyne architecture with IF envelope detection architecture and the homodyne/zero-IF architecture with baseband envelope should be evaluated based on a tradeoff between power consumption and the performance.

According to the agreement in the previous meeting, the matching network and RF BPF for LP WUR can reuse those hardware components of the main radio. To avoid repeated hardware deployment, the possibility to reuse some other components of the existing MR receiver should be studied. Considering the density of the device panel, this operation can reduce the size of the device. Moreover, the hardware overhead will be reduced accordingly.

Proposal 9:  Study the possibility to reuse part of components of the main radio for the heterodyne architecture with IF envelope detection architecture and the homodyne/zero-IF architecture with baseband envelope.

Conclusion

This contribution discusses the architectures for low-power wake-up receiver. Observations and proposals are summarized as follows:

Proposal 1: The coverage of LP-WUS should be consistent with the legacy signal of the main receiver.

Proposal 2: The power consumption of the separate WUR should be reduced dramatically compared with main radio.

Proposal 3: Study synchronization and interference issue in LP-WUS reception.

Proposal 4: Study the impact of the tradeoff between sensitivity, data rate and power consumption in the process of WUR designing.

Proposal 5: Considering the uncertain performance gain with more power consumption, the receiver architecture with FSK modulation can be deprioritized.

Proposal 6: The tradeoff between the performance gain and the power consumption should be carefully evaluated for the OFDMA-based receiver architecture.

Proposal 7: Considering the feasibility and the performance requirements of WUR architecture, the receiver architecture with RF envelope detection can be deprioritized.

Proposal 8: Considering the interference resiliency and sensitivity of LP-WUS, the heterodyne architecture with IF envelope detection architecture and the homodyne/zero-IF architecture with baseband envelope should be evaluated based on a tradeoff between power consumption and the performance.

Proposal 9:  Study the possibility to reuse part of components of the main radio for the heterodyne architecture with IF envelope detection architecture and the homodyne/zero-IF architecture with baseband envelope.

- References

- RP-222644, Revised SID: Study on low-power Wake-up Signal and Receiver for NR, Sep, 2022.

- J. Im, J. Breiholz, S. Li, B. Calhoun and D. D. Wenzloff, "A Fully Integrated 0.2V 802.11ba Wake-Up Receiver with -91.5dBm Sensitivity," 2020 IEEE Radio Frequency Integrated Circuits Symposium (RFIC), 2020, pp. 339-342, doi: 10.1109/RFIC49505.2020.9218383.

- R. Liu et al., "An 802.11ba-Based Wake-Up Radio Receiver With Wi-Fi Transceiver Integration," in IEEE Journal of Solid-State Circuits, vol. 55, no. 5, pp. 1151-1164, May 2020, doi: 10.1109/JSSC.2019.2957651.

- R. Dorrance et al., "An Ultra-Low Power, Fully Integrated Wake-Up Receiver and Digital Baseband with All-Digital Impairment Correction and -92.4dBm Sensitivity for 802.11ba," 2019 Symposium on VLSI Circuits, 2019, pp. C80-C81, doi: 10.23919/VLSIC.2019.8778013.

- J. Im, H. Kim and D. D. Wentzloff, "A 220-uW −83-dBm 5.8-GHz Third-Harmonic Passive Mixer-First LP-WUR for IEEE 802.11ba," in IEEE Transactions on Microwave Theory and Techniques, vol. 67, no. 7, pp. 2537-2545, July 2019, doi: 10.1109/TMTT.2019.2913654.

- X. Huang, P. Harpe, G. Dolmans, H. de Groot and J. R. Long, "A 780–950 MHz, 64–146 µW Power-Scalable Synchronized-Switching OOK Receiver for Wireless Event-Driven Applications," in IEEE Journal of Solid-State Circuits, vol. 49, no. 5, pp. 1135-1147, May 2014, doi: 10.1109/JSSC.2014.2307056.

- K. -W. Cheng and S. -E. Chen, "An Ultralow-Power OOK/BFSK/DBPSK Wake-Up Receiver Based on Injection-Locked Oscillator," in IEEE Transactions on Very Large Scale Integration (VLSI) Systems, vol. 29, no. 7, pp. 1379-1391, July 2021, doi: 10.1109/TVLSI.2021.3073166.

- X. Huang, S. Rampu, X. Wang, G. Dolmans and H. de Groot, "A 2.4GHz/915MHz 51µW wake-up receiver with offset and noise suppression," 2010 IEEE International Solid-State Circuits Conference - (ISSCC), 2010, pp. 222-223, doi: 10.1109/ISSCC.2010.5433958.

- A. Dissanayake, H. L. Bishop, S. M. Bowers and B. H. Calhoun, "A 2.4 GHz-91.5 dBm Sensitivity Within-Packet Duty-Cycled Wake-Up Receiver," in IEEE Journal of Solid-State Circuits, vol. 57, no. 3, pp. 917-931, March 2022, doi: 10.1109/JSSC.2021.3104540.

- Oller, J. , et al. "IEEE 802.11-enabled wake-up radio system: Design and performance evaluation." Electronics Letters, 50.20(2014):1484-1486, doi: 10.1049/EL.2014.2468.

- D. Yoon et al., "A New Approach to Low-Power and Low-Latency Wake-Up Receiver System for Wireless Sensor Nodes," in IEEE Journal of Solid-State Circuits, vol. 47, no. 10, pp. 2405-2419, Oct. 2012, doi: 10.1109/JSSC.2012.2209778.

- E. Nilsson and C. Svensson, "Ultra Low Power Wake-Up Radio Using Envelope Detector and Transmission Line Voltage Transformer," in IEEE Journal on Emerging and Selected Topics in Circuits and Systems, vol. 3, no. 1, pp. 5-12, March 2013, doi: 10.1109/JETCAS.2013.2242777.

- M. Magno and L. Benini, "An ultra low power high sensitivity wake-up radio receiver with addressing capability," 2014 IEEE 10th International Conference on Wireless and Mobile Computing, Networking and Communications (WiMob), 2014, pp. 92-99, doi: 10.1109/WiMOB.2014.6962155.

- S. J. Marinkovic and E. M. Popovici, "Nano-Power Wireless Wake-Up Receiver With Serial Peripheral Interface," in IEEE Journal on Selected Areas in Communications, vol. 29, no. 8, pp. 1641-1647, September 2011, doi: 10.1109/JSAC.2011.110913.

- P. -H. P. Wang et al., "A 6.1-nW Wake-Up Receiver Achieving −80.5-dBm Sensitivity Via a Passive Pseudo-Balun Envelope Detector," in IEEE Solid-State Circuits Letters, vol. 1, no. 5, pp. 134-137, May 2018, doi: 10.1109/LSSC.2018.2875826.

- H. Jiang et al., "24.5 A 4.5nW wake-up radio with −69dBm sensitivity," 2017 IEEE International Solid-State Circuits Conference (ISSCC), 2017, pp. 416-417, doi: 10.1109/ISSCC.2017.7870438.