3GPP TSG RAN WG1 #112		     R1-2301384

Athens, Greece, February 27th – March 3rd, 2023

Agenda item:	7.2

Source: 	Qualcomm Incorporated

Title: 	Clarification on Rel-16 UL Tx switching

Document for:	Discussion/Decision

# Introduction

In Rel-16 UL Tx switching for two bands were specified. The Tx would switch from one carrier to the other carrier following the Network scheduling or configuration. However, the configured resource may not always be used if UE doesn’t transmit. With this paper, we want to clarify whether the skipped UL Tx would triger the switch or not.

# Discussion

In current specification, UL Tx switching is triggered by the UL transmission scheduled by DCI or higher layer configuration.

Section 6.1.6.2 of TS 38.214 For a UE indicating a capability for uplink switching with BandCombination-UplinkTxSwitch for a band combination, and if it is for that band combination configured with uplink carrier aggregation: - If the UE is configured with uplink switching with parameter uplinkTxSwitching, when the UE is to transmit in the uplink based on DCI(s) received before T0-Toffset or based on a higher layer configuration(s): - When the UE is to transmit a 2-port transmission on one uplink carrier and if the preceding uplink transmission is a 1-port transmission on another uplink carrier, then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the two carriers. - …

The higher layer configuration is from RRC or other higher layer which could configure UL transmission resources for potential transmission. The configured resource provides transmission opportunities for UE and could be skipped if UE doesn’t have corresponding signals to transmit. Figure 1 illustrates the case. In Tx Occasion A, Network configures the resource, but UE doesn’t transmit. On couple slots later UE transmits in Tx Occasion B. Before Tx Occasion B, Network and UE need to align the UE RF state and expectation on whether the switching period is needed. If UE doesn’t switch the Tx chain from Band B to Band A for the skipped UL on Tx Occasion A, UE would need to switch the Tx chain for Tx before Occasion B and network needs to budget certain switching time period when scheduling UL transmission. Otherwise, if the UE switches the Tx chain on Tx Occasion A from Band B to Band A even no actual transmission, no more switch is needed for transmission on Occasion B.

Band A

Band B

2Tx

UL Tx

Configured UL Tx but skipped by UE

Tx Occasion A

Tx Occasion B

Figure 1 Illustrative examples of the skipped UL

Based on above analysis there are two possible interpretations of UE Tx chain state:

- interpretation 1: UE would not switch the Tx chain for the skipped UL

- interpretation 2: UE would switch the Tx chain for the skipped UL even no actual transmission

It would be important to clarify which of the above two cases is the RAN1 interpretation of current specification. We propose RAN1 to discuss and conclude the proper interpretation on whether skipped UL would trigger Tx chain switching or not.

Proposal: RAN1 to discuss and conclude the proper interpretation on whether skipped UL would trigger Tx chain switching or not.

Based on current specification cited above, the switching triggering mechanism is “when the UE is to transmit in the uplink based on DCI(s) received before T0-Toffset or based on a higher layer configuration(s):”. Therefore, we slightly prefer the first interpretation. For the skipped UL which UE doesn’t transmit, the Tx state may not be changed. However, we would be open to discuss the second Interpretation if this is majority view.

# Conclusions

In this paper, we discussed the ambiguity on whether the skipped UL would trigger the Tx switching or not and have following proposal.

Proposal: RAN1 to discuss and conclude the proper interpretation on whether skipped UL would trigger Tx chain switching or not.