3GPP TSG RAN WG1 #108-e			           R1-2301618

Athens, Greece, February 27th – March 3rd, 2023

Agenda Item: 9.7.2

Source: MediaTek Inc.

Title: On NW Energy Saving Enhancements for Cell DTX/DRX Mechanism

Document for: Discussion and Decision

# Introduction

In RAN#98-e meeting, there approved the work item scope for Rel-18 network energy saving [1]. One item of the work scope is related to cell DRX/DRX mechanism:

Specify enhancement on cell DTX/DRX mechanism including the alignment of cell DTX/DRX and UE DRX in RRC CONNECTED mode, and inter-node information exchange on cell DTX/DRX [RAN2, RAN1, RAN3] - Note: No change for SSB transmission due to cell DTX/DRX. - Note: The impact to IDLE/INACTIVE UEs due to the above enhancement should be avoided.

In this contribution, potential work split between RAN1 and RAN2 is discussed, and potential RAN1 work scope is identified for further discussion and decision.

# Discussion and Proposals

To avoid impact to legacy UEs, the work scope specifies two constraints:

- No change for SSB transmission due to cell DTX/DRX.

- The impact to IDLE/INACTIVE UEs due to the above enhancement should be avoided.

To comply with the two constraints, Cell DTX/DRX should not apply to signal or occasions related to idle/inactive UE operations, including SSB, PO, PEI and RACH. Since such restriction can be realized by flexible configuration or exemption rule of Cell DTX/DRX, it is proposed to be handled by RAN2.

Proposal 1: Discussion on the configuration or exemption rule of Cell DTX/DRX to avoid impact to signal or occasions related idle/inactive UE operations, including SSB, PO, PEI and RACH, is up to RAN2.

In TR38.864 [2], it is concluded adaptation of DTX/DRX, including the alignment of Cell DTX/DRX with UE DRX, is beneficial for network energy saving. The energy saving gain is by allowing BS to enter sleep with (much) lower power consumption than active DL/UL, as shown in Table 1:

Table 1: Relative power values P for reference configuration Set 1, Set 2 and Set 3

| Power state | BS Category 1 |  |  | BS Category 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Set 1 | Set 2 | Set 3 | Set 1 | Set 2 | Set 3 |
| Deep sleep | 1 |  |  | 1 |  |  |
| Light sleep | 25 |  |  | 2.1 |  |  |
| Micro sleep | 55 | 50 | 38 | 5.5 | 5 | 3 |
| Active DL | 280 | 200 | 152 | 32 | 26 | 17.6 |
| Active UL | 110 | 90 | 80 | 6.5 | 5.8 | 4.2 |

Since sleep is defined as “There is neither DL transmission nor UL reception”, the active times of ALL UEs in the cell should not overlap with Cell DTX/DRX. It is therefore suggested to support cell-wise alignment of cell DTX/DRX and UE DRX in RRC CONNECTED mode.

Observation 1: Cell-wise alignment of cell DTX/DRX and UE DRX in RRC CONNECTED mode should be supported to avoid interruption to BS sleep and ineffective network energy saving gain.

Alignment of UE DRX should also consider the following two aspects:

- Per-UE reconfiguration of DRX parameters can induce significant signaling overhead and BS energy overhead

- Average packet delay is governed by the values of DRX cycle and timers which are optimized w.r.t. UE’s service type and the corresponding QoS requirement. For example, the DRX settings for a VoIP UE and a Video streaming UE will be different because of the different packet latency requirements.

Observation 2: Per-UE reconfiguration of DRX parameter(s) should be avoided due to the signaling overhead and the corresponding BS energy overhead.

Observation 3: The values of each UE’s DRX cycle and timers should be kept for complying with the QoS requirement for each UE’s service.

Proposal 2: Cell-wise alignment of cell DTX/DRX and UEs’ DRX offset values should be supported.

In Rel-17 UE power saving enhancement, the configuration and signaling design for providing cell-wise TRS information is an efficient example. Given there is very limited time for the work item, RAN1 can first discuss the feasibility of an analogous design to achieve efficient indication for the cell-wise alignment. If feasible and agreed, RAN1 can further send LS to RAN2 for deciding whether/how to include the L1 indication design.

Observation 4: Efficient design to trigger cell-wise alignment of cell DTX/DRX and UEs’ DRX offset values can be based on SI configuration and L1 indication by paging DCI and PEI, as adopted in Rel-17 feature of TRS for idle/inactive UEs.

Proposal 3: RAN1 to discuss the feasibility of L1 indication by paging DCI and PEI and the related SI configuration to trigger cell-wise alignment of cell DTX/DRX and UEs’ DRX offset values. If feasible and agreed, RAN1 to further send LS to RAN2 for deciding whether/how to include the L1 indication design.

# Summary

In this contribution the following observations and proposals are provided:

Proposal 1: Discussion on the configuration or exemption rule of Cell DTX/DRX to avoid impact to signal or occasions related idle/inactive UE operations, including SSB, PO, PEI and RACH, is up to RAN2.

Observation 1: Cell-wise alignment of cell DTX/DRX and UE DRX in RRC CONNECTED mode should be supported to avoid interruption to BS sleep and ineffective network energy saving gain.

Observation 2: Per-UE reconfiguration of DRX parameter(s) should be avoided due to the signaling overhead and the corresponding BS energy overhead.

Observation 3: The values of each UE’s DRX cycle and timers should be kept for complying with the QoS requirement for each UE’s service.

Proposal 2: Cell-wise alignment of cell DTX/DRX and UEs’ DRX offset values should be supported.

Observation 4: Efficient design to trigger cell-wise alignment of cell DTX/DRX and UEs’ DRX offset values can be based on SI configuration and L1 indication by paging DCI and PEI, as adopted in Rel-17 feature of TRS for idle/inactive UEs

Proposal 3: RAN1 to discuss the feasibility of L1 indication by paging DCI and PEI and the related SI configuration to trigger cell-wise alignment of cell DTX/DRX and UEs’ DRX offset values. If feasible and agreed, RAN1 to further send LS to RAN2 for deciding whether/how to include the L1 indication design.

# Reference

RP-223540, “New WID: Network energy savings for NR”, RAN#98-e

3GPP TR 38.864, “Study on network energy savings for NR (Release 18)”, Dec., 2022; online available: https://portal.3gpp.org/desktopmodules/Specifications/SpecificationDetails.aspx?specificationId=3987