3GPP TSG RAN WG1 #112	R1-2301642

Athens, Greece, February 27th – March 3rd, 2023

Agenda item:		9.1.1.1

Source:	Nokia, Nokia Shanghai Bell

Title:	Unified TCI framework extension for multi-TRP

Document for:		Discussion and Decision

# Introduction

New Work Item for Rel18 MIMO evolution was approved in RAN#94e [1]. The unified TCI framework was introduced in Rel-17 which facilitates a streamlined multi-beam operation targeting FR2. As Rel-17 focuses on single-TRP use cases, extension of unified TCI framework that focuses on multi-TRP use cases is seen beneficial and is targeted in Rel-18. Further, with the introduction of features for fast UL panel and beam selection in Rel-17, advanced UEs (e.g. CPE, FWA, vehicle, industrial devices) can benefit from higher UL coverage and average throughput with simultaneous UL multi-panel transmission. In addition, further enhancements to facilitate UL multi-TRP deployments via enhanced UL power control can offer additional UL performance improvement. These aspects are being discussed in this contribution following the related WI description as follows [1]:

… 2. Specify extension of Rel-17 Unified TCI framework for indication of multiple DL and UL TCI states focusing on multi-TRP use case, using Rel-17 unified TCI framework. … 6. Study, and if needed, specify the following items to facilitate simultaneous multi-panel UL transmission for higher UL throughput/reliability, focusing on FR2 and multi-TRP, assuming up to 2 TRPs and up to 2 panels, targeting CPE/FWA/vehicle/industrial devices (if applicable) - UL precoding indication for PUSCH, where no new codebook is introduced for multi-panel simultaneous transmission - The total number of layers is up to four across all panels and total number of codewords is up to two across all panels, considering single DCI and multi-DCI based multi-TRP operation. - UL beam indication for PUCCH/PUSCH, where unified TCI framework extension in objective 2 is assumed, considering single DCI and multi-DCI based multi-TRP operation - For the case of multi-DCI based multi-TRP operation, only PUSCH+PUSCH, or PUCCH+PUCCH is transmitted across two panels in a same CC. … 7. Study, and if justified, specify the following - Two TAs for UL multi-DCI for multi-TRP operation - Power control for UL single DCI for multi-TRP operation where unified TCI framework extension in objective 2 is assumed. For the case of simultaneous UL transmission from multiple panels, the operation will only be limited to the objective 6 scenarios.

# Discussion

## 2.1 General framework for unified TCI framework

### 2.1.1 Combinations of joint/DL/ UL TCI states that can be indicated

Supported combinations of joint/DL/UL TCI states that can be indicated should be selected based on the considered use cases. The related agreements regarding the use cases and combinations have been made in the previous meetings:

Agreement On unified TCI framework extension, consider all the intra and inter-cell MTRP schemes specified in Rel-16 and Rel-17 - Consider, if STxMP is supported, Rel-18 MTRP scheme(s) with STxMP Agreement On unified TCI framework extension, at least for the target use cases agreed in RAN1#109-e in AI 9.1.1.1, up to 4 TCI states can be indicated in a CC/BWP or a set of CCs/BWPs in a CC list to DL receptions and/or UL transmissions, where these TCI states are indicated/updated by MAC-CE/DCI with the necessary MAC-CE based TCI state activation - FFS: The possible combination(s) of joint/DL/UL TCI states that can be indicated to DL receptions and/or UL transmissions in a BWP/CC/TRP - Note: This agreement does not imply that there will be more than 2 DL or UL or joint TCI states indicated in a CC/BWP for the target use cases agreed in RAN1#109-e in AI 9.1.1.1 - Note: The maximum number of TCI states that can be indicated to each of the target use cases agreed in RAN1#109-e in AI 9.1.1.1 is remained the same as in Rel-16/17 Note: The maximum number of TCI states that can be indicated simultaneously to CJT-based PDSCH reception and the required type(s) of TCI states (i.e., DL /UL/joint) are independently discussed in this AI

The intra and inter-cell MTRP schemes specified in Rel-16 and Rel-17 are as follows:

- Single/multi-DCI based M-TRP PDSCH NCJT in Rel.16

- Single-DCI based M-TRP PDSCH repetition with SDM/FDM/TDM in Rel.16

- M-TRP PDCCH/PUSCH/PUCCH repetition in Rel.17

- M-TRP inter cell in Rel.17

- SFN-PDCCH/PDSCH in Rel.17

- M-TRP BFR in Rel.17

Furthermore, it has been discussed whether CJT CoMP with up to four transmitting TRPs would also be supported via unified TCI framework. We think that unified TCI framework shall support all use cases in general and thus we also consider that unified TCI framework should support CJT CoMP for PDSCH.

In the following table, we go through the different use cases and what kind of combinations would be needed.

Table 1 Combinations of joint/DL/UL TCI states needed for different use cases

| Scheme | Combinations |
| --- | --- |
| Multi-DCI M-TRP PDSCH NJCT in Rel16 | - 1 joint TCI state indicated for the first TRP and 1 joint TCI state indicated for the second TRP - 1 pair of DL and UL TCI states indicated for a first TRP and 1 pair of DL and UL TCI states indicated for a second TRP - 1 pair of DL and UL TCI states indicated for a first TRP and 1 DL TCI state indicated for a second TRP. |
| Single-DCI M-TRP PDSCH NJCT in Rel16 | - 1 joint TCI state indicated for the first TRP and 1 joint TCI state indicated for the second TRP - 1 pair of DL and UL TCI states indicated for a first TRP and 1 DL TCI state indicated for a second TRP |
| Single-DCI based M-TRP PDSCH repetition with SDM/FDM/TDM in Rel.16 | - 1 joint TCI state indicated for the first TRP and 1 joint TCI state indicated for the second TRP - 1 pair of DL and UL TCI states indicated for a first TRP and 1 DL TCI state indicated for a second TRP |
| M-TRP inter cell in Rel.17 | - 1 joint TCI state indicated for the first TRP and 1 joint TCI state indicated for the second TRP (different PCI) - 1 pair of DL and UL TCI states indicated for a first TRP and 1 pair of DL and UL TCI states indicated for a second TRP (different PCI) - 1 pair of DL and UL TCI states indicated for a first TRP and 1 DL TCI state indicated for a second TRP (different PCI) |
| SFN-PDCCH/PDSCH in Rel.17 | - 1 joint TCI state indicated for the first TRP and 1 joint TCI state indicated for the second TRP - 1 pair of DL and UL TCI states indicated for a first TRP and 1 pair of DL and UL TCI states indicated for a second TRP - 1 pair of DL and UL TCI states indicated for a first TRP and 1 DL TCI state indicated for a second TRP |
| CJT COMP | - Up to 2 joint or DL TCI states |

Observation 1: The following combinations of joint/DL/UL TCI states would be needed:

- 1 joint TCI state indicated for the first TRP and 1 joint TCI state indicated for the second TRP (same or different PCI)

- 1 pair of DL and UL TCI states indicated for a first TRP and 1 pair of DL and UL TCI states indicated for a second TRP (same or different PCI)

- 1 pair of DL and UL TCI states indicated for a first TRP and 1 DL TCI state indicated for a second TRP (same or different PCI)

- Up to 2 joint or DL TCI states (same PCI)

Proposal 1: Support the following combinations of joint/DL/ UL TCI states that can be indicated:

- 1 joint TCI state indicated for the first TRP and 1 joint TCI state indicated for the second TRP (same or different PCI)

- 1 pair of DL and UL TCI states indicated for a first TRP and 1 pair of DL and UL TCI states indicated for a second TRP (same or different PCI)

- 1 pair of DL and UL TCI states indicated for a first TRP and 1 DL TCI state indicated for a second TRP (same or different PCI)

- Up to 2 joint or DL TCI states (same PCI)

### 2.1.2	On supporting CJT PDSCH with the unified TCI framework

As discussed above, it has been considered whether the unified TCI framework should support also CJT CoMP with up to four transmitting TRPs in FR1. RAN1#110bis-e made the following agreement:

Agreement On unified TCI framework extension, up to 2 joint TCI states can be indicated by MAC-CE/DCI and applied to CJT-based PDSCH reception (PDSCH-CJT) in a BWP/CC configured with joint DL/UL TCI mode - Support of 1 or 2 indicated joint TCI states for PDSCH-CJT is up to UE capability - FFS: QCL type(s)/assumption(s) of the indicated joint TCI state(s) applied to PDSCH-CJT - Note: On how to inform UE to apply which indicated joint TCI state(s) to target channel(s)/signal(s) in the BWP/CC, it is discussed individually in AI 9.1.1.1 Agreement On unified TCI framework extension, down-select at least one of the following alternatives for PDSCH-CJT applying both indicated joint TCI states (if the UE supports two indicated joint/DL states for PDSCH-CJT): - Alt1: PDSCH DMRS port(s) is QCLed with the DL RSs of both indicated joint TCI states with respect to QCL-TypeA - Alt2: PDSCH DMRS port(s) is QCLed with the DL RSs of both indicated joint TCI states with respect to QCL-TypeA except for QCL parameters {Doppler shift, Doppler spread} of the second indicated joint TCI state - Alt3: PDSCH DMRS port(s) is QCLed with the DL RS of the first indicated joint TCI state with respect to QCL-TypeA and QCLed with the DL RS of the second indicated joint TCI state with respect to QCL-TypeB

Regarding the alternatives provided by RAN1#111 for the QCL type(s)/assumption(s) of the indicated joint TCI state(s) applied to PDSCH-CJT we consider the Alt1: PDSCH DMRS port(s) is QCLed with the DL RSs of both indicated joint TCI states with respect to QCL-TypeA as the preferred option due to the following reasons:

- Time domain parameters are not expected to be that different among TRPs

- Doppler characteristics were more of an issue e.g. in Rel17 HST-SFN

Proposal 2: Support Alt1: PDSCH DMRS port(s) is QCLed with the DL RSs of both indicated joint TCI states with respect to QCL-TypeA.

## 2.2	TCI state update and activation

### 2.2.1	TCI state update for M-DCI based M-TRP

For TCI state update in m-DCI based M-TRP, the following agreements was made in RAN1#111:

Agreement On unified TCI framework extension for M-DCI based MTRP, the same configuration/rule used in Rel-17 unified TCI framework (for determining whether the UE shall apply the indicated joint/DL TCI state to PDCCH on a CORESET and respective PDSCH) is reused to determine whether the UE shall apply the indicated joint/DL TCI state specific to a coresetPoolIndex value to PDCCH on a CORESET associated with the same coresetPoolIndex value and PDSCH scheduled/activated by the PDCCH. Agreement On unified TCI framework extension for M-DCI based MTRP, the UE shall apply the indicated joint/UL TCI state specific to a coresetPoolIndex value to PUSCH transmission scheduled/activated by PDCCH (including DG-PUSCH and Type2 CG-PUSCH) on a CORESET that is associated with the same coresetPoolIndex value.

As shown above, the agreements in RAN1#111 cover the PDCCH, PDSCH and PUSCH. In the following, we discuss how unified TCI framework can be extended to other signals and channels in M-DCI m-TRP framework. For instance, AP-CSI-RS can follow Indicated TCI state of the CORESET of the triggering PDCCH. Determining of which of the Indicated TCI states is updated would be based on the coresetPoolIndex of the CORESET of the PDCCH transmitting the DCI carrying the AP-CSI-RS trigger in m-DCI scenario.

Observation 2: In m-DCI, determining of which of the Indicated TCI states is applied would be based on the coresetPoolIndex of the CORESET of the PDCCH transmitting the DCI AP-CSI-RS trigger.

Regarding dedicated PUCCH resources, it would be needed to group the resources so that one set of resources would be associated to the first Indicated TCI state (may refer as the indicated TCI state corresponding to the CORESETPoolIndex 0) and other set of the resources would be associated to the second Indicated TCI state (may refer as the indicated TCI state corresponding to the CORESETPoolIndex 1). In general, there may be different alternatives to facilitate the association between PUCCH resources and indicated TCI states, e.g. one alternative could be to define an association between a coresetPoolIndex and a set of PUCCH resources while the other alternative could be to define a separate grouping for the PUCCH resources.

Observation 3: In m-DCI, configuration needs to be provided for dedicated periodic/semi-persistent PUCCH resource(s) that whether the resource(s) follow Indicated TCI state 0 or Indicated TCI state 1.

As currently in Rel-17, for SRS resource (set) it’s provided in the configuration whether the SRS resource (set) follows Indicated TCI state or TCI state, it would need to be extended that the configuration should provide whether the SRS resource (set) follows the first Indicated TCI state, the second Indicated TCI state or TCI state.

On the other hand, as in current Rel-17 version of 38.331, the CORESET configuration has a flag whether or not the CORESET follow Indicated TCI state as well as SRS resource set configuration. Thus, the extension would be needed at least for:

- configuration needs to be provided for dedicated PUCCH resource(s) that whether the resource(s) follow Indicated TCI state 1 or Indicated TCI state 2

- SRS resource (set) that whether the resource(s) follow Indicated TCI state 1, Indicated TCI state 2 or TCI state

Observation 4: In m-DCI, configuration needs to be provided for SRS resource (set) that whether the resource(s) follow Indicated TCI state 0, Indicated TCI state 1 or TCI state.

Proposal 3: In M-DCI, for the AP CSI-RS resource (set) for BM and CSI, TCI state assumption is configured in RRC whether it is the Indicated joint or DL TCI state associated to coresetPoolIndex of the CORESET of the triggering PDCCH or joint or DL TCI state.

Proposal 4: In M-DCI, for the P-CSI-RS, TCI state assumption configured in RRC.

Proposal 5: In M-DCI, for the SP-CSI-RS, TCI state assumption configured in RRC and can be updated in MAC.

Proposal 6: In M-DCI, for the P-/SP-Dedicated PUCCH resource, TCI state assumption is configured in RRC whether Indicated joint or UL TCI state 0 or Indicated joint or UL TCI state 1.

Proposal 7: In M-DCI, for the DCI triggered dedicated PUCCH resource, TCI state assumption is the Indicated joint or UL TCI state associated to a coresetPoolIndex of the CORESET of the triggering PDCCH.

Proposal 8: In M-DCI, for the SRS resource (set), TCI state assumption is configured in RRC whether Indicated joint or UL TCI state 0 or Indicated joint or UL TCI state 1 or joint or UL TCI state.

Proposal 9: In M-DCI, for the DCI triggered SRS resource (set), TCI state assumption is the Indicated joint or UL TCI state associated to a coresetPoolIndex of the CORESET of the triggering PDCCH.

Proposal 10: In M-DCI, for the CG PUSCH type 1, TCI assumption is configured in RRC whether Indicated joint or UL TCI state 0 or Indicated joint or UL TCI state 1.

### 2.2.2	TCI state update for S-DCI based M-TRP

The following related agreement was made in RAN1#111:

Agreement On unified TCI framework extension for S-DCI based MTRP, in one beam indication instance, the existing TCI field in DCI format 1_1/1_2 (with or without DL assignment) can indicate joint/DL/UL TCI state(s) for one or both of the two TRPs in a CC/BWP or a set of CCs/BWPs in a CC list - FFS: Increase on the size of the TCI field - Note: The term TRP is used only for discussion purpose in RAN1 and whether/how to capture this is FFS

In the case of S-DCI based M-TRP and in the case of separate DL and separate UL TCI states, one TCI codepoint may have up to 4 TCI states: 2 DL TCI and 2 UL TCI states. We consider that for flexibility it should be possible to activate two DL and two UL TCI states per TRP. That would require 16 different codepoints to be supported. Thus, we think that it would make sense to increase the max number of MAC CE activated TCI codepoints e.g., up to 16.

Proposal 11: Support the max number of MAC CE activated TCI codepoints being 16.

Proposal 12: Support up to four-bit TCI field in DCI.

### 2.2.3	Potential impact and enhancement to common TCI state ID update/activation for CA when S-DCI/M-DCI MTRP is configured

In Rel-17 multi-CC scenario, RRC-configured TCI state pool(s) can be absent in the PDSCH configuration (PDSCH-Config) for each BWP/CC, and replaced with a reference to RRC-configured TCI state pool(s) in a reference BWP/CC as depicted in Figure 1:

Figure 1 Illustration of the reference BWP/CC.

Then, the next question is that how common TCI state update operation should be defined in case for multi-TRP in general and for a mix of CCs/cells configured with single-TRP and multi-TRP modes.

S-DCI scenario

In general, in S-DCI MTRP scenario the CC/BWPs may be configured with either multi-TRP and single-TRP configurations. Then, each CC/BWP should be provided that which indicated TCI state(s) of the reference CC/BWP the CC/BWP would follow as depicted in Figure 2.

Figure 2 Indication of which of the indicated TCI states of the reference CC/BWP to apply in other CCs/BWP when the other CCs/BWPs may be single-TRP or multi-TRP.

Proposal 13: Support signaling to UE which indicated TCI state(s) of the reference CC/BWP to follow in other CCs/BWPs when the other CCs/BWPs may be configured either with single- or multi-TRP configuration.

M-DCI scenario

In m-DCI MTRP scenario the CC/BWPs may be configured with coreset pool indices and also one CC/BWP may have either one of two coreset pool indices configured. In that case in general, the assumption of which indicated TCI state to follow could be based on the coreset pool indices as illustrated in Figure 3.

Figure 3 Indication of which of the indicated TCI states of the reference CC/BWP to apply in other CCs/BWP when the other CCs/BWPs may be single-TRP or multi-TRP.

Proposal 14: Support signalling to UE which indicated TCI state(s) of the reference CC/BWP to follow in other CCs/BWPs when the other CCs/BWPs may be configured with one or two coreset pool indices.

## How to associate the indicated TCI state(s) with each target channel/signal in S-DCI based MTRP

For TCI state indication in s-DCI based MTRP, the following agreements were made in RAN1#110bis-e and in RAN1#111:

RAN1#110bis-e: Agreement: On unified TCI framework extension for S-DCI based MTRP, to inform the association with the joint/DL TCI state(s) indicated by DCI/MAC-CE for PDCCH repetition, PDCCH-SFN, and PDCCH w/o repetition/SFN, support the following: - Use RRC configuration to inform that the UE shall apply the first one, the second one, both, or none of the joint/DL TCI states indicated by DCI/MAC-CE to a CORESET or a group of CORESETs (if CORESET group configuration is supported) RAN1#111: Agreement On unified TCI framework extension for S-DCI based MTRP, a DCI field in DCI format 1_1/1_2 that schedules/activates PDSCH reception is used to determine which one or both of the indicated joint/DL TCI states shall be applied to the scheduled/activated PDSCH reception - The presence of the DCI field is configurable by RRC; when the DCI field is not present in DCI format 1_1/1_2, the UE shall apply the default indicated joint/DL TCI state(s) to PDSCH reception - FFS: Details on the default indicated joint/DL TCI state(s) to PDSCH reception - FFS: The DCI field is a new indicator field or an existing field (e.g., the existing TCI field) - FFS: Regardless the DCI field is present or not present, how to apply the indicated joint/DL TCI state(s) to PDSCH reception if the offset between the reception of the DCI format 1_1/1_2 and the corresponding PDSCH reception is less than a threshold FFS: How to apply the indicated joint/DL TCI state(s) to PDSCH reception scheduled/activated by DCI format 1_0. Above applies for the case where PDSCHs scheduled by the same DCI. Agreement On unified TCI framework extension for S-DCI based MTRP, a new indicator field is supported as the DCI field in DCI format 1_1/1_2 that schedules/activates PDSCH reception to determine which one or both of the indicated joint/DL TCI states shall be applied to the scheduled/activated PDSCH reception - FFS: Detail design of the new indicator field Agreement On unified TCI framework extension for S-DCI based MTRP, use RRC configuration to inform that the UE shall apply the first one, the second one, or both of the indicated joint/UL TCI states to a PUCCH resource/group - Note: Detail of the RRC configuration is left to RAN2 design

Regarding s-DCI PDSCH scheme we consider that the main principle is to have to both the existing TCI field and a new 2-bit indicator field with the following purposes:

TCI field is used to update/switch the current Indicated TCI states (procedure having the beam application time)

New 2-bit indicator field is used to select which one of or both of the Indicated TCI states are to be applied for the scheduled PDSCHs

Regarding the following FFS point

- FFS: Regardless the DCI field is present or not present, how to apply the indicated joint/DL TCI state(s) to PDSCH reception if the offset between the reception of the DCI format 1_1/1_2 and the corresponding PDSCH reception is less than a threshold

we consider that e.g. applying Rel16 rules (lowest codepoint having two TCI states) would complicate the system operation. We think that the most simple and efficient way would be to assume the current Indicated TCI states as default QCL assumptions.

Proposal 15: If the offset between the reception of the DCI format 1_1/1_2 and the corresponding PDSCH reception is less than a threshold assume the current Indicated TCI states as default QCL assumptions for the PDSCH(s).

Regarding the FFS point:

FFS: How to apply the indicated joint/DL TCI state(s) to PDSCH reception scheduled/activated by DCI format 1_0.

we think the most simple and straightforward solution is to apply the first Indicated TCI state to the scheduled PDSCH.

Proposal 16: Apply the first Indicated TCI state to the PDSCH reception scheduled/activated by DCI format 1_0.

In s-DCI PDSCH schemes AP CSI-RSs would need two Indicated TCI states to facilitate CSI acquisition from the intended PDSCH beams.

Observation 5: In s-DCI PDSCH schemes, AP CSI-RS resource (set) should be able to be associated to one of the (two) Indicated TCI state where one the of Indicated TCI states is the same as for the CORESET used for the PDCCH triggering the AP CSI-RS.

Proposal 17: In S-DCI, for the AP CSI-RS resource (set) for BM and CSI, TCI state assumption is configured in RRC whether Indicated TCI state 0, Indicated TCI state 1 or TCI state.

Proposal 18: In S-DCI, for the P-CSI-RS, TCI state assumption configured in RRC.

Proposal 19: In S-DCI, for the SP-CSI-RS, TCI state assumption configured in RRC and can be updated in MAC.

Associations between TCI state(s) and UL signals or channels could follow the same principles as above in general. What comes to PUSCH transmission triggered by the DCI format 0_1/0_2 we think that legacy functionality should be remained, i.e. that SRI is provided in the DCI while Indicated TCI state can be associated to the corresponding SRS resource set. This is to enable port indication via SRI as in legacy.

In addition, one further aspect should be taken into consideration. It’s that a UE may have more than two antenna panels equipped typically while two SRS resource sets are assumed to be configured to the UE currently. In order to enable dynamic panel switching it should be possible to configure higher number of SRS resource sets, dynamically activate SRS resource set(s) and associate the certain active set(s) to the certain Indicated TCI state(s). It’s also to be noted that each panel (more than two panels) may have different number of antenna ports and thus each panel would need own SRS resource set configuration.

Observation 6: In order to enable dynamic panel switching in case of UE is equipped with more than two panels it should be possible to configure higher number of SRS resource sets than each set representing characteristics of a panel (number of SRS ports), dynamically activate SRS resource set(s) and dynamically associate the certain active set(s) to the certain Indicated TCI state(s).

Proposal 20: In S-DCI, for the SRS resource set, support dynamic association between the SRS resource set and Indicated TCI state.

Proposal 21: In S-DCI, for the CG PUSCH type 1, TCI assumption is configured in RRC whether Indicated joint or UL TCI state 0 or Indicated joint or UL TCI state 1 or both (repetition).

### 2.3.2	On PUSCH transmission

Regarding PUSCH transmission under unified TCI framework the following agreement was made in the previous meeting:

Agreement On unified TCI framework extension for S-DCI based MTRP, down-select one alternative from the followings in RAN1#111 for PUSCH transmission scheduled/activated by a DCI format 0_1/0_2: - Alt1: Use an indicator field (could be reusing an existing DCI field or introducing a new DCI field) in the DCI format 0_1/0_2 to inform which joint/UL TCI state(s) indicated by MAC-CE/DCI the UE shall apply to PUSCH transmission scheduled/activated by the DCI format 0_1/0_2 - Alt2: PUSCH transmission scheduled/activated by the DCI format 0_1/0_2 follows the spatial domain transmission filter(s) used for the SRS resource(s) indicated by the DCI format 0_1/0_2 - FFS : PL-RS(s), and UL PC parameter setting(s) (including P0, alpha, and closed loop index) for the PUSCH

Figure 4 illustrates the Alt1 and Alt2. Basically, both approaches have similar functionality but Alt2 (legacy) would allow higher number of candidate UL beams in the UL sounding phase while Alt1 would have up to two UL candidate beams. Regarding power control in Alt2, since SRS resource indicated in DCI by SRI would be having a TCI state to provide spatial source as well as associated power control parameters, PC control setting should be similar as in Alt1:

TCI-UL-State-r17 ::= SEQUENCE { tci-UL-State-Id-r17 TCI-UL-State-Id-r17, servingCellId-r17 ServCellIndex OPTIONAL, -- Need R bwp-Id-r17 BWP-Id OPTIONAL, -- Cond CSI-RSorSRS-Indicated referenceSignal-r17 CHOICE { ssb-Index-r17 SSB-Index, csi-RS-Index-r17 NZP-CSI-RS-ResourceId, srs-r17 SRS-ResourceId }, additionalPCI-r17 AdditionalPCIIndex-r17 OPTIONAL, -- Need R ul-powerControl-r17 Uplink-powerControlId-r17 OPTIONAL, -- Need R pathlossReferenceRS-Id-r17 PUSCH-PathlossReferenceRS-Id-r17 OPTIONAL, -- Need R ... } Uplink-powerControl-r17 ::= SEQUENCE { ul-powercontrolId-r17 Uplink-powerControlId-r17, p0AlphaSetforPUSCH-r17 P0AlphaSet-r17 OPTIONAL, -- Need R p0AlphaSetforPUCCH-r17 P0AlphaSet-r17 OPTIONAL, -- Need R p0AlphaSetforSRS-r17 P0AlphaSet-r17 OPTIONAL -- Need R }

It’s also to be noted that Alt2 can provide the functionality of Alt1 by configuring SRS resource set(s) to follow Indicated TCI state(s). Thus, we consider that it would be preferable to support Alt2. Further, it’s to be noted that functionality remains in case no SRIs are in the DCI, i.e. in the case there is a single SRS resource in each SRS resource set.

Figure 4 Illustration of Alt1 and Alt2.

Proposal 22: Support Alt2: PUSCH transmission scheduled/activated by a DCI format 0_1/0_2 follows the spatial domain transmission filter(s) used for the SRS resource(s) indicated by the DCI format 0_1/0_2.

## UL PC for UL MTRP

In this section, we discuss potential UL power control enhancements for UL single-DCI for multi-TRP operation considering unified TCI framework, as captured in the Rel-18 objective below:

- Study, and if justified, specify the following - Two TAs for UL multi-DCI for multi-TRP operation - Power control for UL single DCI for multi-TRP operation where unified TCI framework extension in objective 2 is assumed. For the case of simultaneous UL transmission from multiple panels, the operation will only be limited to the objective 6 scenarios.

### 2.4.1	General power control enhancements

In RAN1#109-e, the support of the extension of the Rel-17 unified TCI framework in terms of power control for PUCCH/PUSCH was agreed as follows:

Agreement On unified TCI framework extension, if an indicated joint or UL TCI state applies to a PUSCH /PUCCH transmission occasion at least for S-DCI based PUSCH/PUCCH repetition with TDM and the indicated joint or UL TCI state is associated with an UL PC parameter setting for PUSCH /PUCCH (including P0, alpha for PUSCH , and closed loop index) and a PL-RS, the UE should apply the UL PC parameter setting and the PL-RS for the PUSCH /PUCCH transmission occasion. - FFS: How to extend to other Rel-18 MTRP scheme(s) with STxMP, if supported - FFS: UL PC enhancement for CB and non-CB SRS in above case FFS: The applied UL PC parameter setting if one or both indicated joint or UL TCI state(s) is not associated with an UL PC parameter setting (including P0, alpha for PUSCH, and closed loop index) for PUCCH/PUSCH

On the first FFS point in the above agreement, we think that the agreed operation would also apply to Rel-18 M-TRP with STxMP. Whether additional enhancements are needed can be further discussed.

On the other hand, it’s worth recalling that Rel-17 introduced some power control enhancements for multi-TRP PUCCH/PUSCH operations, such as:

- the possibility for the gNB to configure a second TPC command field in DCI (formats 1_1/1_2 and 0_1/0_2) for both multi-TRP PUCCH and PUSCH operations,

- the possibility to indicate two open-loop power control parameter P0 values for M-TRP PUSCH operation, in case there is no SRI field in DCI (format 0_1 and 0_2).

Considering the extension of the Rel-17 unified TCI framework to multi-TRP use cases in Rel-18, the following can be observed on the Rel-17 M-TRP power control enhancements (for single-DCI mode):

- The Rel-17 enhancements on introducing a second TPC command field in DCI for both multi-TRP PUCCH and PUSCH operations could be used in Rel-18. Specifically, a second TPC command field can be configured in DCI, and for a multi-TRP PUSCH/PUCCH operation considering two indicated TCI states (each associated with a different closed-loop index), first and second indicated TPC commands would apply to UL transmissions/repetitions mapped to a first and second TCI states, respectively; this assumes that these two TCI states are associated to different closed-loop indexes.

  - If the second TPC field is not configured, indicated TPC command applies to UL transmissions/repetitions mapped to both first and second TCI states.

- Note that the Rel-17 enhancement to indicate two open-loop power control parameter P0 values for M-TRP PUSCH operation is inherited in the Rel-17 unified TCI framework design, since P0 is associated with an UL or joint TCI state. And the extension of this feature to M-TRP has been agreed in RAN1#109-e, as shown earlier.

Proposal 23: For Rel-18 multi-TRP PUSCH/PUCCH operations under unified TCI framework, reuse the Rel-17 multi-TRP power control enhancement as follows:

- a second TPC field can be configured in DCI formats 1_1/1_2 and 0_1/0_2 to support per-TRP closed-loop power control. Specifically, first and second indicated TPC commands would apply to UL transmissions/repetitions mapped to first and second indicated TCI states, respectively;

- if the second TPC field is not configured, indicated TPC command applies to UL transmissions/repetitions mapped to both first and second indicated TCI states.

Further, for the cases where at least one of the indicated joint/UL TCI states would not be associated to UL PC parameter setting, the following proposal has been proposed in RAN1#110bis-e (based on the related RAN1#110 proposal listing three alternatives):

Proposal 4.A: On unified TCI framework extension, if one or both of indicated joint/UL TCI states applying to PUSCH/PUCCH transmission occasions in an UL BWP at least for S-DCI based PUSCH/PUCCH repetition with TDM is/are not associated with UL PC parameter setting (including P0, alpha for PUSCH, and closed loop index) for PUCCH/PUSCH, down-selection one alternative from the followings: - Alt1: Support two default UL PC parameter settings configured in BWP-UplinkDedicated, and the UE should apply the one or two default UL PC parameter settings configured in the corresponding UL BWP - FFS: 1-to-1 association between an indicated joint/UL TCI state and a default UL PC parameter setting - Alt2: No change from Rel-17 unified TCI framework

Overall, we support this proposal and are in principle fine with Alt.1 and Alt.2, with a slight preference towards Alt.1. It’s worth noting that, under Rel-17 M-TRP UL enhancements, two default power control parameter sets are considered. And this is more in line with Alt.1.

Proposal 24: On unified TCI framework extension, if one or both of indicated joint/UL TCI states applying to PUSCH/PUCCH transmission occasions in an UL BWP at least for S-DCI based PUSCH/PUCCH repetition with TDM is/are not associated with UL PC parameter setting (including P0, alpha for PUSCH, and closed loop index) for PUCCH/PUSCH, select any of Alt1 and Alt2:

- Alt1: Support two default UL PC parameter settings configured in BWP-UplinkDedicated, and the UE should apply the one or two default UL PC parameter settings configured in the corresponding UL BWP

  - FFS: 1-to-1 association between an indicated joint/UL TCI state and a default UL PC parameter setting

- Alt2: No change from Rel-17 unified TCI framework, i.e., the UE should apply the one single default UL PC parameter setting configured in the corresponding UL BWP regardless the UL PC parameter setting is absent from one or both of indicated joint/UL TCI states.

### 2.4.2	Power control related enhancements considering multi-panel UE and simultaneous UL transmissions

In legacy operations, PCMAX,f,c, which basically represents the maximum allowed power for a UE for its UL transmissions, is defined per carrier for a serving cell. For simultaneous UL transmissions, where two PUSCHs or two PUCCHs that overlap in time would be (simultaneously) transmitted on the same carrier, and more generally for multi-panel UE, one important point to discuss and decide is whether to keep PCMAX per carrier or whether to define panel specific PCMAX. Panel specific PCMAX may be needed, when, for example, panel-specific UE power class (e.g., due to different PA characteristics for each panel) is introduced, or panel specific MPR related considerations are assumed, etc.

A related discussion happened in RAN1#109-e, where it was agreed to send an LS to RAN4 in order to check some related aspects, such as whether it’s feasible to assume power limitation per panel or over all UE panels for STxMP, as captured in the below agreement:

Agreement On UE power limitation for STxMP for FR2, send LS to RAN4 to check the followings: - Whether it is feasible to assume power limitation per panel for STxMP (Assumption 1) - Whether it is feasible to assume a total power limitation per UE over all UE panels used for STxMP (Assumption 2) - In either of Assumption1 or Assumption 2, whether the total power limitation per UE over all UE panels used for STxMP or the sum of per-panel power limitation for STxMP can be different from (greater than) the existing power limitation for a given power class? - If both Assumption 1 and Assumption 2 are feasible, whether both assumptions can be applied to a same UE, and what is the relationship between the per-panel power limitation and total power limitation if both are applied (e.g., the sum of per-panel power limitation can be larger than the total power limitation per UE, or should be always the same)? FFS: Detail of exact LS if agreed Note: Scenarios of above include at least single carrier scenario for FR2 Note: Above power limitation includes both total radiated power and EIRP LS to RAN4 is endorsed in R1-2205639.

In FR2, typically UE devices are having a distributed PA architecture where the PA is part of the mmW module instead of a PA that can be switched to different antennas which is the typical architecture assumption in FR1. The distributed PA architecture means that there is a low power PA per antenna element or per small set of antenna elements within a panel. That means that antenna panels are having their own PAs, in contrast to FR1 where multiple antennas may be switched to PA(s).

Observation 7: In FR2, typically devices are equipped with a distributed PA architecture where each antenna panel is having its own PA(s).

That makes the Assumption1 (Power limitation per panel for STxMP) more suitable for UEs to be adopted in FR2, especially for the high-power class UE devices as targeted in Rel18: FWA, CPE, Vehicular, etc. In addition, as uplink coverage is typically the bottleneck, the higher total EIRP would be beneficial to alleviate uplink coverage issues.

Proposal 25: Support at least Assumption 1: Power limitation per panel for STxMP.

Once the above aspects are clarified, PH (power headroom) aspect would need to be discussed. It’s worth recalling that Rel-17 introduced the reporting of two PHs for multi-TRP PUSCH operation. This could be used as a starting point for the Rel-18 discussions on PH aspects for simultaneous UL transmissions, and more generally for multi-panel transmissions. However, given that the network will be switching between single-TRP and multi-TRP UL schemes (such as SDM and TDM), e.g., based on some UE reporting such as capability value set index reporting, having two PHRs calculated and reported for a cell may not be needed when M-TRP UL schemes are not feasible/needed/applicable, i.e., when only single-TRP scheme is feasible/needed, be it from the UE and/or network perspectives. For instance, if there is a single (UL/joint) TCI state indicated/applicable in the cell, then calculating and reporting two PHRs would not be necessary and would thus unnecessarily increase the UL control overhead.

Another aspect that would need to be discussed is when two-PHR mode is not configured for a cell, i.e., the UE would potentially be required to calculate and report one PH. In this case, for two simultaneous PUSCH transmissions, it should be discussed which PUSCH transmission to consider for the PH calculation and reporting.

Observation 8: Considering the feasibility/need of single-TRP UL scheme versus multi-TRP UL scheme (including simultaneous UL transmissions, such as SDM) from UE and/or network perspectives, two-PHR reporting may not be needed when M-TRP UL schemes are not feasible/applicable. In addition, the UE would be required to potentially calculate and report one PH in case the two-PHR mode is not configured.

Proposal 26: RAN1 to discuss PH (power headroom) aspects considering the Rel-18 schemes, including simultaneous UL transmissions, such as (i) number of PHRs to report at a given time and which UL scheme to use for the calculation of the PHR(s), and (ii) the cases where the UE is not configured with two-PHR mode.

Further, given that Rel-18 MIMO objective 6 is targeting “CPE/FWA/vehicle/industrial devices”, it should be discussed whether MPE issues would need to be considered in the study. In other words, it should be discussed whether, for instance, some industrial devices can be handheld devices or not. Based on this, RAN1 could decide whether, considering Rel-17 MPE reporting as a basis, some MPE enhancements for simultaneous UL transmission operation are needed.

Proposal 27: Decide whether MPE is an issue considering the targeted devices in the Rel-18 objective on simultaneous UL transmissions.

Finally, the existing specifications define prioritization operation for transmission power reductions for parallel UL transmission on different (serving) cells for the CA case when the UE total required transmit power would exceed the maximum allowed power. Given that Rel-18 introduce the support of STxMP in a same cell, it should be discussed whether/how the transmission power reduction operation needs to be adapted to also account for the presence of Rel-18 STxMP operation.

Proposal 28: Discuss prioritization operation for transmission power reductions, for the CA case, considering the support of STxMP.

## Beam reporting and indication for simultaneous multi-panel UL

In this section we discuss about beam indication for simultaneous multi-panel transmission. First, we discuss about capability value index reporting developed in Rel-17 and whether an extension is needed to facilitate simultaneous multi-panel UL transmissions. Then, we discuss about beam indication aspects.

### 2.5.1 Enhanced capability reporting

Rel-17 has introduced a panel capability aware beam reporting to support fast UL beam and panel selection for single panel UL transmission as described in the following RAN1#108-e agreements and conclusion:

Agreement On Rel.17 enhancements to facilitate UE-initiated panel activation and selection, support the UE reporting a list of UE capability value [sets] - Each UE capability value [set] comprises the max supported number of SRS ports - Any two capability values [sets] are different - Whether the UE capability value [set] can be common across all BWPs/CCs in same band or BC can be discussed in UE feature session Agreement On Rel.17 enhancements to facilitate UE-initiated panel activation and selection, UE can report one index of UE capability value [set] for each reported CRI/SSBRI in one beam reporting. Conclusion On Rel.17 enhancements to facilitate UE-initiated panel activation and selection, there is no consensus in supporting indication of DL-only panel using one value [set] of the max supported number of SRS ports Agreement On Rel.17 enhancements to facilitate UE-initiated panel activation and selection, all types of time-domain behavior, i.e., periodic, semi-persistent, and aperiodic reporting, are supported for the enhanced beam report with index(es) of UE capability value [set]. - Support of aperiodic and semi-persistent reporting is optional

Rel-17 new reporting was designed with a single-panel PUSCH (codebook based PUSCH) in mind. Thus, for instance two panels at the UE side having the same number of SRS antenna ports are seen at the gNB side as one panel due to the following agreement:

Agreement On Rel.17 enhancements to facilitate UE-initiated panel activation and selection, support the UE reporting a list of UE capability value [sets] - … - Any two capability values [sets] are different - …

In other words, the UE may report two different SSBRIs/CRIs with the same capability value index but the gNB cannot determine whether the UE has measured the reported SSBRIs/CRIs with the same or different panel. Thus, the gNB cannot get aware based on Rel-17 reporting whether the UE would be able to transmit to different TRPs with one panel (e.g., in TDM manner) or with two different panels (e.g., in FDM or SDM manner) at a time. Thus, we think that it would be beneficial to extend the Rel-17 capability value index reporting to support also simultaneous multi-panel uplink transmission. In the enhanced reporting, the report should carry information about UE’s simultaneous transmission possibility of two panels.

Observation 9: The UE may report two different SSBRIs/CRIs with the same capability value index but the gNB cannot determine whether the UE has measured the reported SSBRIs/CRIs with the same or different panel. Thus, the gNB cannot get aware based on Rel-17 reporting whether the UE would be able to transmit to different TRPs with one panel (e.g., in TDM manner) or with two different panels (e.g., in FDM or SDM manner) at a time.

Proposal 29: Enhance capability value index reporting to carry information to gNB on whether or not the UE is able to transmit simultaneously from two panels, where the panels may have same or different maximum number of SRS ports.

### 2.5.2 Enhanced reporting to alleviate inter-beam interference in simultaneous multi-panel uplink transmissions

In multi-TRP scenario, the distance and radio conditions between the transmitting panels and receiving TRPs may differ quite much. For the uplink transmission, an open loop power control is applied that aims at compensating the higher path loss by the higher transmission power. Thus, the panels transmitting simultaneously may be having quite different EIRP per resource element/subcarrier. Even though the transmit beams may be spatially separated (main lobes), the higher EIRP panel may have sidelobes in relatively high level compared to the main lobe of the transmit beam of the other panel. Depending on the transmit beams the higher EIRP panel may cause high interference with its side lobes to the receiving TRP of the other panel with lower EIRP. In that case, FDM transmission scheme may be preferred over the SDM scheme while in other cases SDM may be preferred.

Then the question is that how the gNB could get aware of the situation so that it could make a proper transmission scheme selection for the simultaneous multi-panel uplink transmission. One way to determine feasible transmission scheme is when the SRS transmissions are received and potential precoder matrix/matrices are estimated at the gNB side. Another way could be to consider how the UE would choose RSs for reporting as potential source RSs to determine uplink transmit spatial filters. Let’s denote RS1 as the candidate spatial source RS for the first panel towards the first TRP and RS2 as the candidate spatial source RS for the second panel towards the second TRP. Both panels measure using their intended TX beam (TX beam of first panel according to RS1 and TX beam of second panel according to RS2) as RX beam RSRP of both spatial source RSs. Then if the RSRP of RS1 measured by the second panel is in the same level as the RSRP of RS1 measured by the first panel, or if the RSRP of RS2 measured by the first panel is in the level as the RSRP of RS2 measured by the second panel, there may be an inter-beam interference issue and e.g. SDM may not be feasible but rather FDM would be preferable in this case. In the beam reporting the UE could inform gNB about feasible pairs of DL RSs to determine uplink spatial transmit filters and feasible multiplexing method(s) (SDM, FDM, SFN or TDM). DL RSs could also reflect the QCL-Type D RSs of the indicated TCI states.

Proposal 30: Study schemes to alleviate the inter-beam interference issue for simultaneous multi-panel uplink transmissions.

Proposal 31: Consider UE to report information regarding feasible multiplexing schemes of simultaneous two-panel UL PUSCH based on DL RS measurements where DL RSs represent the candidate UL TX beams.

### 2.5.3 Beam indication for single-DCI and multi-DCI modes

Once the reporting capability is defined/supported to enable the simultaneous UL transmission operation, both UE and gNB would have same understanding on which panels and UL/joint TCI states could be used for simultaneous UL transmissions. Then, for two UL/joint TCI states, both the UE and gNB would know whether the UE can perform simultaneous UL transmissions or not using these two TCI states, where each TCI state would be mapped to a different UL transmission(s) of the simultaneous UL transmission operation.

Hence, if the gNB decides to schedule the UE with simultaneous UL transmissions, the gNB would need to have two indicated UL/joint TCI states corresponding to capability value set indices / panels for which the UE has already indicated the capability of simultaneous transmissions using these panels / TCI states – through for example the enhanced reporting suggested in the previous subsections.

Note that the above reasoning could be applicable for simultaneous UL transmissions scheduled via single-DCI mode or multi-DCI mode.

Observation 10: Enhanced capability value set reporting to enable beam indication for simultaneous UL transmissions could be used for both single-DCI and multi-DCI modes.

In general, for single-DCI and multi-DCI modes, it’s expected that the beam indication for simultaneous UL transmissions would follow the respective operations that will be defined to enable Rel-18 multi-TRP UL operations considering unified TCI framework; we discuss such operations in an earlier section.

Observation 11: For single-DCI and multi-DCI modes, it’s expected that the beam indication for simultaneous UL transmissions would follow the respective operations that will be defined to generally enable Rel-18 multi-TRP UL operations considering unified TCI framework.

## 2.6 BFR Aspects

RAN1#110bis-e made the following agreements to study enhancements for TRP-specific BFR.

Agreement On unified TCI framework extension, study the following enhancements for TRP-specific BFR: - Implicit BFD-RS determination based on the indicated joint/DL TCI states for S-DCI based MTRP - Enhancement to beam update after NW response to TRP-specific BFR request Agreement On unified TCI framework extension for S-DCI based MTRP, to inform the association with the joint/DL TCI state(s) indicated by DCI/MAC-CE for PDCCH repetition, PDCCH-SFN, and PDCCH w/o repetition/SFN, support the following: - Use RRC configuration to inform that the UE shall apply the first one, the second one, both, or none of the joint/DL TCI states indicated by DCI/MAC-CE to a CORESET or a group of CORESETs (if CORESET group configuration is supported)

### 2.6.1 S-DCI BFR extension

As the S-DCI mTRP does not use CORESETpoolIndex, the determination of BFD-RS in BFD-RS sets would need to be based on the indicated TCI states (first and/or second). The S-DCI framework allows one or two indicated TCI states for PDCCH (e.g. for repetition) based on the configuration associated with one or more CORESETs.

If the CORESET(s) of a BWP are associated with a configuration for first indicated TCI state only UE determines that it monitors the beam failure on using one BFD-RS set. If UE receives indication for second TCI State it is applied for at least PDSCH reception but not for PDCCH monitoring and thus it does not have effect on implicit BFD-RS configuration

Proposal 32: If CORESET(s) of a BWP are associated with configuration of first indicated TCI state, UE monitors beam failure on one set of q0, and the set includes RS included in the TCI state.

If a CORESET (or set) are associated with a configuration for first indicated TCI state and other CORESET (or sets) with a configuration for second indicated TCI state, the UE determines to monitor beam failure on two BFD-RS sets. The RS indicated by the first TCI state is included in the first BFD-RS set and the RS indicated by the second TCI state in the second BFD-RS set.

Proposal 33: If a CORESET (or set) are associated with a configuration for first indicated TCI state and other CORESET (or sets) with a configuration for second indicated TCI state, the UE determines to monitor beam failure on two BFD-RS sets (set q0,0 includes RS included in the first TCI state, set q0,1 includes RS included in the second TCI state).

If a CORESET on BWP is associated with configuration of both for first indicated TCI state and second indicated TCI state i.e. the CORESET is assumed as SFN CORESET, the RSs of both TCI States could be monitored on one BFD-RS set. However, if on a BWP there is one CORESET with SFN configuration and e.g additional CORESETs set without SFN configuration (associated with one of the indicated TCI state configurations) the UE could monitor the beam failure on two sets: set q0,0 includes RS included in the first TCI state, set q0,1 includes RS included in the second TCI state.

Proposal 34: If BWP has configuration of only an SFN CORESET, the beam failure is monitored on one BFD-RS including RS indicated by the first and second sets.

Proposal 35: If BWP has configuration of an SFN CORESET and a non-SFN CORESET, the beam failure is monitored on two BFD-RS sets: set q0,0 includes RS included in the first TCI state, set q0,1 includes RS included in the second TCI state.

### 2.6.2 M-DCI BFR extension

For M-DCI, the extension should be rather straightforward if the CORESETpoolIndex approach is used. Each CORESETPoolIndex is associated with should be rather straightforward to extend the R17 mTRP BFR to unified TCI for M-DCI. The BFD-RS sets are associated with the CORESETpoolIndex, and the RS in respective sets are based on the indicated joint or DL unified TCI State for the CORESETPoolIndex.

Proposal 36: On unified TCI framework extension for M-DCI, the CORESETpoolindices are associated with respective BFD-RS sets.

Proposal 37: For a BFD-RS set, the RS to be included in the set is determined based on the indicated joint or (separate) DL unified TCI state for the CORESETpoolindex.

Proposal 38: The beam update after BFR is applied for associated with the CORESETpoolIndex of the respective failed BFD-RS set.

# 3.	Conclusions

Based on the discussion above we made the following observations and proposals:

General framework for unified TCI framework:

Observation 1: The following combinations of joint/DL/UL TCI states would be needed:

- 1 joint TCI state indicated for the first TRP and 1 joint TCI state indicated for the second TRP (same or different PCI)

- 1 pair of DL and UL TCI states indicated for a first TRP and 1 pair of DL and UL TCI states indicated for a second TRP (same or different PCI)

- 1 pair of DL and UL TCI states indicated for a first TRP and 1 DL TCI state indicated for a second TRP (same or different PCI)

- Up to 2 joint or DL TCI states (same PCI)

Proposal 1: Support the following combinations of joint/DL/ UL TCI states that can be indicated:

- 1 joint TCI state indicated for the first TRP and 1 joint TCI state indicated for the second TRP (same or different PCI)

- 1 pair of DL and UL TCI states indicated for a first TRP and 1 pair of DL and UL TCI states indicated for a second TRP (same or different PCI)

- 1 pair of DL and UL TCI states indicated for a first TRP and 1 DL TCI state indicated for a second TRP (same or different PCI)

- Up to 2 joint or DL TCI states (same PCI)

Proposal 2: Support Alt1: PDSCH DMRS port(s) is QCLed with the DL RSs of both indicated joint TCI states with respect to QCL-TypeA.

TCI state update and activation:

Observation 2: In m-DCI, determining of which of the Indicated TCI states is applied would be based on the coresetPoolIndex of the CORESET of the PDCCH transmitting the DCI AP-CSI-RS trigger.

Observation 3: In m-DCI, configuration needs to be provided for dedicated periodic/semi-persistent PUCCH resource(s) that whether the resource(s) follow Indicated TCI state 0 or Indicated TCI state 1.

Observation 4: In m-DCI, configuration needs to be provided for SRS resource (set) that whether the resource(s) follow Indicated TCI state 0, Indicated TCI state 1 or TCI state.

Proposal 3: In M-DCI, for the AP CSI-RS resource (set) for BM and CSI, TCI state assumption is configured in RRC whether it is the Indicated joint or DL TCI state associated to coresetPoolIndex of the CORESET of the triggering PDCCH or joint or DL TCI state.

Proposal 4: In M-DCI, for the P-CSI-RS, TCI state assumption configured in RRC.

Proposal 5: In M-DCI, for the SP-CSI-RS, TCI state assumption configured in RRC and can be updated in MAC.

Proposal 6: In M-DCI, for the P-/SP-Dedicated PUCCH resource, TCI state assumption is configured in RRC whether Indicated joint or UL TCI state 0 or Indicated joint or UL TCI state 1.

Proposal 7: In M-DCI, for the DCI triggered dedicated PUCCH resource, TCI state assumption is the Indicated joint or UL TCI state associated to a coresetPoolIndex of the CORESET of the triggering PDCCH.

Proposal 8: In M-DCI, for the SRS resource (set), TCI state assumption is configured in RRC whether Indicated joint or UL TCI state 0 or Indicated joint or UL TCI state 1 or joint or UL TCI state.

Proposal 9: In M-DCI, for the DCI triggered SRS resource (set), TCI state assumption is the Indicated joint or UL TCI state associated to a coresetPoolIndex of the CORESET of the triggering PDCCH.

Proposal 10: In M-DCI, for the CG PUSCH type 1, TCI assumption is configured in RRC whether Indicated joint or UL TCI state 0 or Indicated joint or UL TCI state 1.

Proposal 11: Support the max number of MAC CE activated TCI codepoints being 16.

Proposal 12: Support up to four-bit TCI field in DCI.

Proposal 13: Support signaling to UE which indicated TCI state(s) of the reference CC/BWP to follow in other CCs/BWPs when the other CCs/BWPs may be configured either with single- or multi-TRP configuration.

Proposal 14: Support signalling to UE which indicated TCI state(s) of the reference CC/BWP to follow in other CCs/BWPs when the other CCs/BWPs may be configured with one or two coreset pool indices.

How to associate the indicated TCI state(s) with each target channel/signal in S-DCI based MTRP:

Proposal 15: If the offset between the reception of the DCI format 1_1/1_2 and the corresponding PDSCH reception is less than a threshold assume the current Indicated TCI states as default QCL assumptions for the PDSCH(s).

Proposal 16: Apply the first Indicated TCI state to the PDSCH reception scheduled/activated by DCI format 1_0.

Observation 5: In s-DCI PDSCH schemes, AP CSI-RS resource (set) should be able to be associated to one of the (two) Indicated TCI state where one the of Indicated TCI states is the same as for the CORESET used for the PDCCH triggering the AP CSI-RS.

Proposal 17: In S-DCI, for the AP CSI-RS resource (set) for BM and CSI, TCI state assumption is configured in RRC whether Indicated TCI state 0, Indicated TCI state 1 or TCI state.

Proposal 18: In S-DCI, for the P-CSI-RS, TCI state assumption configured in RRC.

Proposal 19: In S-DCI, for the SP-CSI-RS, TCI state assumption configured in RRC and can be updated in MAC.

Observation 6: In order to enable dynamic panel switching in case of UE is equipped with more than two panels it should be possible to configure higher number of SRS resource sets than each set representing characteristics of a panel (number of SRS ports), dynamically activate SRS resource set(s) and dynamically associate the certain active set(s) to the certain Indicated TCI state(s).

Proposal 20: In S-DCI, for the SRS resource set, support dynamic association between the SRS resource set and Indicated TCI state.

Proposal 21: In S-DCI, for the CG PUSCH type 1, TCI assumption is configured in RRC whether Indicated joint or UL TCI state 0 or Indicated joint or UL TCI state 1 or both (repetition).

Proposal 22: Support Alt2: PUSCH transmission scheduled/activated by a DCI format 0_1/0_2 follows the spatial domain transmission filter(s) used for the SRS resource(s) indicated by the DCI format 0_1/0_2.

UL PC for UL MTRP:

Proposal 23: For Rel-18 multi-TRP PUSCH/PUCCH operations under unified TCI framework, reuse the Rel-17 multi-TRP power control enhancement as follows:

- a second TPC field can be configured in DCI formats 1_1/1_2 and 0_1/0_2 to support per-TRP closed-loop power control. Specifically, first and second indicated TPC commands would apply to UL transmissions/repetitions mapped to first and second indicated TCI states, respectively;

- if the second TPC field is not configured, indicated TPC command applies to UL transmissions/repetitions mapped to both first and second indicated TCI states.

Proposal 24: On unified TCI framework extension, if one or both of indicated joint/UL TCI states applying to PUSCH/PUCCH transmission occasions in an UL BWP at least for S-DCI based PUSCH/PUCCH repetition with TDM is/are not associated with UL PC parameter setting (including P0, alpha for PUSCH, and closed loop index) for PUCCH/PUSCH, select any of Alt1 and Alt2:

- Alt1: Support two default UL PC parameter settings configured in BWP-UplinkDedicated, and the UE should apply the one or two default UL PC parameter settings configured in the corresponding UL BWP

  - FFS: 1-to-1 association between an indicated joint/UL TCI state and a default UL PC parameter setting

- Alt2: No change from Rel-17 unified TCI framework, i.e., the UE should apply the one single default UL PC parameter setting configured in the corresponding UL BWP regardless the UL PC parameter setting is absent from one or both of indicated joint/UL TCI states.

Observation 7: In FR2, typically devices are equipped with a distributed PA architecture where each antenna panel is having its own PA(s).

Proposal 25: Support at least Assumption 1: Power limitation per panel for STxMP.

Observation 8: Considering the feasibility/need of single-TRP UL scheme versus multi-TRP UL scheme (including simultaneous UL transmissions, such as SDM) from UE and/or network perspectives, two-PHR reporting may not be needed when M-TRP UL schemes are not feasible/applicable. In addition, the UE would be required to potentially calculate and report one PH in case the two-PHR mode is not configured.

Proposal 26: RAN1 to discuss PH (power headroom) aspects considering the Rel-18 schemes, including simultaneous UL transmissions, such as (i) number of PHRs to report at a given time and which UL scheme to use for the calculation of the PHR(s), and (ii) the cases where the UE is not configured with two-PHR mode.

Proposal 27: Decide whether MPE is an issue considering the targeted devices in the Rel-18 objective on simultaneous UL transmissions.

Proposal 28: Discuss prioritization operation for transmission power reductions, for the CA case, considering the support of STxMP.

Beam reporting and indication for simultaneous multi-panel UL:

Observation 9: The UE may report two different SSBRIs/CRIs with the same capability value index but the gNB cannot determine whether the UE has measured the reported SSBRIs/CRIs with the same or different panel. Thus, the gNB cannot get aware based on Rel-17 reporting whether the UE would be able to transmit to different TRPs with one panel (e.g., in TDM manner) or with two different panels (e.g., in FDM or SDM manner) at a time.

Proposal 29: Enhance capability value index reporting to carry information to gNB on whether or not the UE is able to transmit simultaneously from two panels, where the panels may have same or different maximum number of SRS ports.

Proposal 30: Study schemes to alleviate the inter-beam interference issue for simultaneous multi-panel uplink transmissions.

Proposal 31: Consider UE to report information regarding feasible multiplexing schemes of simultaneous two-panel UL PUSCH based on DL RS measurements where DL RSs represent the candidate UL TX beams.

Observation 10: Enhanced capability value set reporting to enable beam indication for simultaneous UL transmissions could be used for both single-DCI and multi-DCI modes.

Observation 11: For single-DCI and multi-DCI modes, it’s expected that the beam indication for simultaneous UL transmissions would follow the respective operations that will be defined to generally enable Rel-18 multi-TRP UL operations considering unified TCI framework.

BFR Aspects:

Proposal 32: If CORESET(s) of a BWP are associated with configuration of first indicated TCI state, UE monitors beam failure on one set of q0, and the set includes RS included in the TCI state.

Proposal 33: If a CORESET (or set) are associated with a configuration for first indicated TCI state and other CORESET (or sets) with a configuration for second indicated TCI state, the UE determines to monitor beam failure on two BFD-RS sets (set q0,0 includes RS included in the first TCI state, set q0,1 includes RS included in the second TCI state).

Proposal 34: If BWP has configuration of only an SFN CORESET, the beam failure is monitored on one BFD-RS including RS indicated by the first and second sets.

Proposal 35: If BWP has configuration of an SFN CORESET and a non-SFN CORESET, the beam failure is monitored on two BFD-RS sets: set q0,0 includes RS included in the first TCI state, set q0,1 includes RS included in the second TCI state.

Proposal 36: On unified TCI framework extension for M-DCI, the CORESETpoolindices are associated with respective BFD-RS sets.

Proposal 37: For a BFD-RS set, the RS to be included in the set is determined based on the indicated joint or (separate) DL unified TCI state for the CORESETpoolindex.

Proposal 38: The beam update after BFR is applied for associated with the CORESETpoolIndex of the respective failed BFD-RS set.

# References

- RP-213598, ”New WID: MIMO Evolution for Downlink and Uplink”, Samsung (Moderator)