3GPP TSG RAN WG1 #112			R1- 2302000

Athens, Greece, February 27th – March 3rd, 2023

Agenda item:	9.1.1.1

Source: 	Moderator (MediaTek)

Title: 	Moderator summary on extension of unified TCI framework (Round 1)

Document for:	Discussion and Decision

- Introduction

In RAN#94e, the Rel-18 WID of MIMO evolution for downlink and uplink is approved. In the approved WID, extension of unified TCI framework is a part of the RAN1 objectives, and the detailed scope of this agenda item (AI 9.1.1.1) includes the following highlighted objectives:

RAN1: - Specify extension of Rel-17 Unified TCI framework for indication of multiple DL and UL TCI states focusing on multi-TRP use case, using Rel-17 unified TCI framework. - Study, and if needed, specify the following items to facilitate simultaneous multi-panel UL transmission for higher UL throughput/reliability, focusing on FR2 and multi-TRP, assuming up to 2 TRPs and up to 2 panels, targeting CPE/FWA/vehicle/industrial devices (if applicable) - UL precoding indication for PUSCH, where no new codebook is introduced for multi-panel simultaneous transmission - The total number of layers is up to four across all panels and total number of codewords is up to two across all panels, considering single DCI and multi-DCI based multi-TRP operation. - UL beam indication for PUCCH/PUSCH, where unified TCI framework extension in objective 2 is assumed, considering single DCI and multi-DCI based multi-TRP operation - For the case of multi-DCI based multi-TRP operation, only PUSCH+PUSCH, or PUCCH+PUCCH is transmitted across two panels in a same CC. - Study, and if justified, specify the following - Two TAs for UL multi-DCI for multi-TRP operation - Power control for UL single DCI for multi-TRP operation where unified TCI framework extension in objective 2 is assumed. For the case of simultaneous UL transmission from multiple panels, the operation will only be limited to the objective 6 scenarios.

- Plan

Based on the contributions from companies [1]-[32], the followings are provided in this document:

- Summary of companies’ views on each of open issues raised by interested companies, where the open issues are categorized as follow:

  - Issue 1 – General framework for unified TCI extension

  - Issue 2 – TCI state update and activation

  - Issue 3 – How to inform UE which indicated TCI state(s) that UE shall apply to target channel/signal

  - Issue 4 – UL power control for UL MTRP

  - Issue 5 – PDSCH-CJT

  - Issue 6 – Beam failure recovery and beam reporting

- Observations and recommended proposals based on the summary of companies’ views

- Contact Person

For potential offline discussion, companies/delegates are encouraged to enter the contact information in the table below:

Table 0 Contact Information

| Company | Point(s) of contact | Email address(es) |
| --- | --- | --- |
| Panasonic | Khalid | khalid.zeineddine@eu.panasonic.com |
| FGI | Cubie | wanchen.lin@fginnov.com |
| Ericsson | Claes | Claes.tidestav@ericsson.com |
| vivo | Yang | songyang@vivo.com |
| Sharp | Taka | fukui.takahisa@sharp.co.jp |
| ZTE | Bo | gao.bo1@ZTE.com.cn |
| OPPO | Jeffrey | caojianfei@oppo.com |
| MediaTek | Darcy | darcy.tsai@mediatek.com |
| MediaTek | Rebecca | rebecca.chen@mediatek.com |
| Google | Alex | alexliou@google.com |
| Qualcomm | Yan | yanzhou@qti.qualcomm.com |
| Futurewei | Zhigang | zrong@futurewei.com |
| NTT DOCOMO | Yuki | yuki.matsumura@docomo-lab.com |
| NTT DOCOMO | Weiqi | sunwq@docomolabs-beijing.com.cn |
| LG | Jaehoon | jhoon.chung@lge.com |
| NEC | Peng | guan_peng@nec.cn |
| CMCC | Yan | liyanwx@chinamobile.com |
| Intel | Avik | avik.sengupta@intel.com |
| Huawei, HiSilicon | Keyvan | Keyvan.zarifi@huawei.com |
| InterDigital | Jonghyun | jonghyun.park@interdigital.com |
| Samsung | Dalin | dalin.zhu@samsung.com |
| Xiaomi | Mingju LI | limingju@xiaomi.com |
| Fujitsu | Jian | zhangjian1288@fujitsu.com |
| Fraunhofer IIS/HHI | Sutharshun | sutharshun.varatharaajan@iis.fraunhofer.de |
| Apple | Hong | hhe5@apple.com |
| Spreadtrum | Qiyishu Li | qiyishu.li@unisoc.com |
| Lenovo | Bingchao Liu | liubc2@lenovo.com |
| Hyundai | Jeongsu Lee | Jeongsu.lee@hyundai.com |
| CEWiT | Vishakha Singh | Vish@cewit.org.in |

- Proposals to be discussed in the online session

Proposal 3.2: On unified TCI framework extension for S-DCI based MTRP, a 2-bit [TCI selection field] can be configured by RRC to be present in a DCI format 1_1/1_2 that schedules/activates PDSCH reception (including dynamic PDSCH and SPS PDSCH) according to the followings:

- If the DCI format 1_1/1_2 indicates codepoint "00" for the [TCI selection field], the UE shall apply the first one of two indicated joint/DL TCI states to all PDSCH DMRS port(s) of all PDSCH transmission occasions(s) scheduled/activated by the DCI format 1_1/1_2

- If the DCI format 1_1/1_2 indicates codepoint "01" for the [TCI selection field], the UE shall apply the second one of two indicated joint/DL TCI states to all PDSCH DMRS port(s) of all PDSCH transmission occasions(s) scheduled/activated by the DCI format 1_1/1_2

- If the DCI format 1_1/1_2 indicates codepoint "10" for the [TCI selection field], the UE shall apply both indicated joint/DL TCI states to the PDSCH reception scheduled/activated by the DCI format 1_1/1_2

- FFS: Whether and how to use the codepoint "11" of the [TCI selection field]

Above applies at least if the offset between the reception of the scheduling DCI format 1_1/1_2 and the scheduled/activated PDSCH reception is equal to or larger than a threshold

FFS: Regardless the [TCI selection field] is present or not present, how to apply the indicated joint/DL TCI state(s) to the scheduled/activated PDSCH reception if the offset between the reception of the scheduling DCI format 1_1/1_2 and the scheduled/activated PDSCH reception is less than a threshold (if the threshold is needed)

If the UE is in FR1, or the UE supports the capability of two default beams for S-DCI based MTRP in FR2, above apply to all PDSCH reception(s) scheduled/activated by the DCI format 1_1/1_2.

- Note: If the UE supports the capability of two default beams for S-DCI based MTRP in FR2, UE uses both indicated joint/DL TCI states to buffer the received signal before a threshold.

If the UE doesn’t support the capability of two default beams for S-DCI based MTRP in FR2, above apply to the scheduled/activated PDSCH reception when the offset between the reception of the scheduling DCI format 1_1/1_2 and the scheduled/activated PDSCH reception is equal to or larger than a threshold

- FFS: How to apply the indicated joint/DL TCI state(s) to the scheduled/activated PDSCH reception if the offset between the reception of the scheduling DCI format 1_1/1_2 and the scheduled/activated PDSCH reception is less than a threshold in FR2

FFS: Detail of the capability of two default beams for S-DCI based MTRP

FFS: The threshold value

Proposal 3.4: On unified TCI framework extension for S-DCI based MTRP, when two SRS resource sets for CB/NCB are configured, support the followings for PUSCH transmission scheduled/activated by a DCI format 0_1/0_2 (including DG and Type2 CG):

- If the DCI format 0_1/0_2 indicates codepoint "00" for the existing SRS resource set indicator, the UE shall apply the first indicated joint/UL TCI state to all PUSCH antenna port(s) of all PUSCH transmission occasions(s)

- If the DCI format 0_1/0_2 indicates codepoint "01" for the existing SRS resource set indicator, the UE shall apply the second indicated joint/UL TCI state to all PUSCH antenna port(s) of all PUSCH transmission occasions(s)

- If the DCI format 0_1/0_2 indicates codepoint "10" or “11” for the existing SRS resource set indicator:

  - For TDM based PUSCH Tx scheme, the UE shall apply the first indicated joint/UL TCI state to the PUSCH transmission occasions(s) associated with the first SRS resource set for CB/NCB, and the second indicated joint/UL TCI state to the PUSCH transmission occasions(s) associated with the second SRS resource set for CB/NCB (note: the association between an SRS resource set for CB/NCB and PUSCH transmission occasions(s) is defined according to TS 38.214)

  - FFS: SDM and SFN based PUSCH Tx schemes

FFS: The case that the spatial Tx filter(s) determined from the indicated joint/UL TCI state(s) applied to a PUSCH transmission is different from the spatial Tx filter(s) used for the SRS transmission corresponding to the SRS resource(s) indicated to the PUSCH transmission

Proposal 2.1: On unified TCI framework extension, support the followings for S-DCI based MTRP operation:

- For a serving cell configured with joint DL/UL TCI mode, a full-set or any sub-set of {1st joint TCI state, 2nd joint TCI state} can be mapped to a TCI codepoint of the existing TCI field in a DCI format 1_1/1_2 (with or without DL assignment)

  - If the UE receives a TCI codepoint mapped with a full-set of {1st joint TCI state, 2nd joint TCI state}, the UE shall update the corresponding joint TCI states in the full-set

  - If the UE receives a TCI codepoint mapped with a sub-set of {1st joint TCI state, 2nd joint TCI state}, the UE shall update the corresponding joint TCI state in the sub-set, and keep current joint TCI state not in the sub-set

  - TCI state activation command (MAC-CE) should indicate that each activated joint TCI state in the TCI state activation command is mapped to the 1st or 2nd joint TCI state of a TCI codepoint, and how to indicate is up to RAN2 design

  - Note: The 1st joint TCI sate corresponds to one TRP, and the 2nd joint TCI state corresponds to another TRP

- For a serving cell configured with separate DL/UL TCI mode, a full-set or any sub-set of {1st DL TCI state, 1st UL TCI state, 2nd DL TCI state, 2nd UL TCI state} can be mapped to a TCI codepoint of the existing TCI field in a DCI format 1_1/1_2 (with or without DL assignment)

  - If the UE receives a TCI codepoint mapped with a full-set of {1st DL TCI state, 1st UL TCI state, 2nd DL TCI state, 2nd UL TCI state}, the UE shall update the corresponding DL/UL TCI states in the full-set.

  - If the UE receives a TCI codepoint mapped with a sub-set of {1st DL TCI state, 1st UL TCI state, 2nd DL TCI state, 2nd UL TCI state}, the UE shall update the corresponding DL/UL TCI state(s) in the sub-set, and keep current DL/UL TCI state(s) not in the sub-set

  - TCI state activation command (MAC-CE) should indicate that each activated DL/UL TCI state in the TCI state activation command is mapped to the 1st DL TCI state, 1st UL TCI state, 2nd DL TCI state, or 2nd UL TCI state of a TCI codepoint, and how to indicate is up to RAN2 design

  - Note: The 1st DL and UL TCI sates correspond to one TRP, and the 2nd DL and UL TCI states correspond to another TRP

[FFS: How to determine that one indicated joint/DL/UL TCI state or two indicated joint/DL/UL states should be maintained by the UE, e.g., based on RRC configuration or MAC-CE TCI state activation?]

- Support the UE behaviors in this proposal: IDC, Google, MTK, Huawei, HiSilicon, CMCC, OPPO, LG, Spreadtrum, Apple, Sharp, ZTE, Apple, CATT, Panasonic

- Not support the UE behaviors in this proposal: QC, Lenovo, vivo

Proposal 4.1: On unified TCI framework extension, if an indicated joint/UL TCI state(s) applies to a PUSCH/PUCCH/SRS transmission occasion(s) or antenna port(s), the UE shall determine UL Tx power for the PUSCH/PUCCH/SRS transmission occasion(s) or antenna port(s) based on the UL PC parameter setting for PUSCH/PUCCH/SRS, if any, and the PL-RS included in the indicated joint/UL TCI state

- FFS: For STxMP, the maximum Tx power when the UE determines UL Tx power for the PUSCH/PUCCH transmission occasion(s) or antenna port(s) (discussed after receiving RAN4 reply on UE power limitation for STxMP in FR2)

- FFS: Default UL PC parameter setting(s) if one or both of indicated joint/UL TCI states applied to PUSCH/PUCCH/SRS transmission occasion(s) or antenna port(s) does/do not include the UL PC parameter setting(s) for PUCCH/PUSCH/SRS

Proposal 3.7: On unified TCI framework extension for M-DCI based MTRP, down-select at least one alternative from the followings for PUCCH transmission:

- Alt1: A coresetPoolIndex value is provided per PUCCH resource/resource group, and the UE shall apply the indicated joint/UL TCI state specific to the coresetPoolIndex value to the corresponding PUCCH transmission

- Alt2: An RRC configuration is provided per PUCCH resource/resource group to inform that the UE shall apply the first or the second indicated joint/UL TCI state to the corresponding PUCCH transmission, where the first and the second indicated joint/DL TCI states correspond to the indicated joint/UL TCI states specific to coresetPoolIndex value 0 and value 1, respectively.

- Alt3: For a PUCCH transmission scheduled by a DCI with DL assignment on a CORESET when the UE is not provided with ackNackFeedbackMode = joint, the UE shall apply the indicated joint/UL TCI state specific to a coresetPoolIndex value to the PUCCH transmission triggered by PDCCH on a CORESET, where the coresetPoolIndex value is determined from the one associated with the CORESET. Otherwise, Alt1 or Alt2 is adopted.

- Alt4: For a PUCCH transmission with an LLR trigged for either the first BFD-RS set (q0,0) or the second BFD-RS set (q0,1) when the UE is provided only one schedulingRequestID-BFR configuration, the UE shall apply the indicated joint/UL TCI state specific to a coresetPoolIndex value to the PUCCH transmission, where the coresetPoolIndex value is 1 when the LRR is trigged for the first BFD-RS set (q0,0) and the coresetPoolIndex value is 0 when the LRR is trigged for the second BFD-RS set (q0,1) . Otherwise, Alt1 or Alt2 is adopted.

- Discussion

- Issue 1 – General framework for unified TCI extension

- Issue 2 – TCI state update and activation

Table 2-1 Summary for Issue 2

| # | Issue | Companies’ view and Recommended Proposal |
| --- | --- | --- |
| 2.1 | Combinations of joint/DL/UL TCI states that can be mapped to a TCI codepoint of the existing TCI field for S-DCI based MTRP, and corresponding UE behaviors | Proposal 2.1: On unified TCI framework extension, support the followings for S-DCI based MTRP operation: - For a serving cell configured with joint DL/UL TCI mode, a full-set or any sub-set of {1st joint TCI state, 2nd joint TCI state} can be mapped to a TCI codepoint of the existing TCI field in a DCI format 1_1/1_2 (with or without DL assignment) - If the UE receives a TCI codepoint mapped with a full-set of {1st joint TCI state, 2nd joint TCI state}, the UE shall update the corresponding joint TCI states in the full-set - If the UE receives a TCI codepoint mapped with a sub-set of {1st joint TCI state, 2nd joint TCI state}, the UE shall update the corresponding joint TCI state in the sub-set, and keep current joint TCI state not in the sub-set - TCI state activation command (MAC-CE) should indicate that each activated joint TCI state in the TCI state activation command is mapped to the 1st or 2nd joint TCI state of a TCI codepoint, and how to indicate is up to RAN2 design - Note: The 1st joint TCI sate corresponds to the 1st TRP, and the 2nd joint TCI state corresponds to the 2nd TRP - For a serving cell configured with separate DL/UL TCI mode, a full-set or any sub-set of {1st DL TCI state, 1st UL TCI state, 2nd DL TCI state, 2nd UL TCI state} can be mapped to a TCI codepoint of the existing TCI field in a DCI format 1_1/1_2 (with or without DL assignment) - If the UE receives a TCI codepoint mapped with a full-set of {1st DL TCI state, 1st UL TCI state, 2nd DL TCI state, 2nd UL TCI state}, the UE shall update the corresponding DL/UL TCI states in the full-set. - If the UE receives a TCI codepoint mapped with a sub-set of {1st DL TCI state, 1st UL TCI state, 2nd DL TCI state, 2nd UL TCI state}, the UE shall update the corresponding DL/UL TCI state(s) in the sub-set, and keep current DL/UL TCI state(s) not in the sub-set - TCI state activation command (MAC-CE) should indicate that each activated DL/UL TCI state in the TCI state activation command is mapped to the 1st DL TCI state, 1st UL TCI state, 2nd DL TCI state, or 2nd UL TCI state of a TCI codepoint, and how to indicate is up to RAN2 design - Note: The 1st DL and UL TCI sates correspond to the 1st TRP, and the 2nd DL and UL TCI states correspond to the 2nd TRP [FFS: How to determine that one indicated joint/DL/UL TCI state or two indicated joint/DL/UL states should be maintained by the UE, e.g., based on RRC configuration or MAC-CE TCI state activation?] Support the UE behaviors in this proposal: IDC, Google, MTK, Huawei, HiSilicon, CMCC, OPPO, LG, Spreadtrum, Apple, Sharp, ZTE, Apple, CATT, Panasonic Not support the UE behaviors in this proposal: QC, Lenovo, vivo FL note: Based on the previous agreement, it should be able to use the existing TCI field in DCI format 1_1/1_2 to indicate joint/DL/UL TCI state(s) for one of the two TRPs or both TRPs. Thus, Proposal 2.1 is recommended accordingly. FL note: Regarding the last FFS item, it is proposed to address concern from company that how UE understand that it is operated under Rel-17 unified TCI framework with single indicated joint/DL/UL TCI state or Rel-18 unified TCI framework extension with two indicated joint/DL/UL TCI states. However, I think this may be clarified by the main bullet that Rel-18 unified TCI framework extension is operated when the UE is configured with S-DCI based MTRP operation. |
| 2.2 | Support of inter-cell MTRP in unified TCI extension | FL note: Based on the following specification, it seems inter-cell MTRP can be naturally supported in unified TCI extension. Please comment if companies have different view. TS 38.214 If the UE is configured with SSB-MTC-AddtionalPCI and with PDCCH-Config that contains two different values of coresetPoolIndex in ControlResourceSet, the UE receives an activation command for CORESET associated with each coresetPoolIndex, as described in clause 6.1.3.14 of [10, TS 38.321], used to map up to 8 TCI states to the codepoints of the DCI field 'Transmission Configuration Indication' in one CC/DL BWP. When a set of TCI state IDs are activated for a coresetPoolIndex, the activated TCI states corresponding to one coresetPoolIndex can be associated with one physical cell ID and activated TCI states corresponding to another coresetPoolIndex can be associated with another physical cell ID. |
| 2.3 | Common TCI state ID activation/update for the set of CCs configured in the same CC list | Question 1: Whether a CC list can be comprised of a mix of STRP CC(s) and MTRP CC(s)? - Yes: OPPO, Spreadtrum, IDC, Huawei, Hyundai, Google, Docomo, vivo, CATT, LG - No: Xiaomi, QC, NEC, CMCC, ZTE, FGI Question 2: Whether a CC list can be comprised of a mix of S-DCI based MTRP CC(s) and M-DCI based MTRP CC(s)? - Yes: OPPO, IDC, Huawei, Hyundai, Google, Docomo, CATT - No: Xiaomi, Spreadtrum, QC, NEC, CMCC, ZTE, vivo, FGI |
| 2.4 | Timeline to apply the indicated joint/DL/UL TCI state(s) to channels/signals | Question 1: In Rel-18 unified TCI framework extension for MTRP operation, whether the Rel-17 timeline for updating the indicated joint/DL/UL TCI state(s) is retained, i.e., the indicated joint/DL/UL TCI state(s) applied to the DL reception or UL transmission is updated based on the Rel-17 beam application time? - Yes: Xiaomi, Google, Docomo, OPPO, IDC, QC, Apple, ZTE, Panasonic, Sharp, CEWiT, Huawei - No: |
| 2.5 | Support of inter-cell S-DCI based MTRP | Question 1: In Rel-18 unified TCI framework extension for MTRP operation, whether to support inter-cell S-DCI based MTRP? - Yes: Samsung - No: |

Table 2-2 Company input for Issue 2

| Company | Input |
| --- | --- |
| Mod V00 | - Please share your view to the updated Proposal 2.1 - No update to Proposal 2.2 |
| Xiaomi | As for the FFS, could FL provide more explanation about it? Since the red parts has defined the UE behavior that UE need to maintain two joint/DL/UL TCI states, what does “how to determine……” mean? |
| Panasonic | Proposal 2.1: We support. Just for clarification, in our understanding, how RAN2 designs the TCI state activation command (MAC-CE) might have an impact on the beam indication DCI. For example, if two activation commands, one per TRP (similar to coresetPoolIndex) are sent (one possible design), then another DCI field might be needed to fulfill the above in RAN1. Should RAN1 discuss putting some constraints regarding any future DCI impact? Issue 2.4 – Question 1: YES |
| Lenovo | We don’t support the update. Similar with MDCI MTRP in Rel-16, if only one TCI state is indicated for a TCI codepoint in the MAC CE, the UE shall update the single indicated TCI state for the TCI codepoint. |
| QC | For Proposal 2.1, not support. To justify the benefit, the FFS should be answered first, including the MAC-CE codepoint design - We don’t think the scheme will save any codepoint if all activated TCI codepoints can be for both 1 and 2 maintained TCIs. For example, in case of 2 active TCIs per TRP, the total codepoint # is 9 to signal all sTRP and mTRP TCI combinations, e.g. 4 codepoints for sTRP, at least one codepoint for mTRP with 2 TCIs per codepoint, 4 codepoints for mTRP with 1 TCI per codepoint. However, if DCI is used to switch between 1 and 2 maintained TCIs, only 8 codepoints are needed, i.e. 4 codepoints for sTRP, and 4 codepoints for mTRPs with 2 TCIs per codepoint. - So DCI based switch between 1 and 2 maintained TCIs may need less codepoints, depending on the FFS. In addition, DCI based switch has shorter application time and simpler design with MAC-CE format similar to R16 |
| Huawei, HiSilicon | Proposal 2.1: Support except the last FFS. We think the FFS is already clarified in the two red bullets added to the proposal. UE will always maintain two joint/DL TCI state if a serving cell configured with joint DL/UL TCI mode and maintain the full set {1st DL TCI state, 1st UL TCI state, 2nd DL TCI state, 2nd UL TCI state} if it is configured with separate DL/UL TCI mode. Proposal 2.2: Support Issue 2.3: Same view as Round 0 Issue 2.4: Yes. |
| Sharp | Proposal 2.1: We are generally fine with the proposal. |
| vivo | Proposal 2.1: Comments as follows: - We still prefer previous version to FFS the behavior of indicating 1 TCI state. - Besides, the Note: “The 1st joint TCI sate corresponds to the 1st TRP, and the 2nd joint TCI state corresponds to the 2nd TRP” seems not very precise, because we cannot tell which TRP is the 1st TRP or 2nd TRP. |
| ZTE | Proposal 2.1: Same concerns as the last FFS Issue 2.2: Current spec seems fine. |
| Hyundai | Proposal 2.2: We are sorry to repose to the proposal lately. We have a one question for clear understanding, Based on our understanding, unlike the intra cell MTRP case, the CORESET pool index from each TRPs could be either same or not. That is, the one of CORESET index from 1st TRP does not depend on the other CORESET index from 2nd TRP? We will appreciate if anyone tell me the answer. Issue 2.4: We are sorry to repose to the proposal lately too. We want to know how does UE do if scheduled physical channels such as PUSCH/PDSCH/PDCCH/SRS/PUCCH is overlapped with BAT (e.g. Beam application time) ? we think that there is no specific description about the behaviors of UE/gNB for that case. furthermore, if UE can change its beams directly after BAT ? We think there is no description in the current specification. So, we think further study on BAT should be considered. |
| OPPO | On the updated Proposal 2.1, we are supportive. |
| Samsung | Proposal 2.1: Support in principle. We think SDCI based MTRP operation being configured/used should be explicitly captured in this proposal – this is the scope of TCI codepoint mapping. Proposal 2.1: On unified TCI framework extension, support the followings for when S-DCI based MTRP operation(s) is configured/applied: Proposal 2.2: We think discussions on unified TCI framework extension for SDCI based MTRP are needed. Proposal 2.x: On unified TCI framework extension for S-DCI based MTRP, when the UE is configured with SSB-MTC-AdditionalPCI, the indicated joint/DL TCI states mapped to a TCI codepoint can be associated with the serving cell PCI and a PCI other than the serving cell PCI. |
| Fujitsu | Proposal 2.1: Regarding the FFS, just a question for clarification, does the FFS imply how to determine STRP or MTRP? |
| CEWiT | Proposal 2.1: Support in principle. Issue 2.4 – Question 1: YES |
| Google | Proposal 2.1: Generally fine. But could FL explain more on the FFL for the purpose. |

- Issue 3 – How to inform UE which indicated TCI state(s) that UE shall apply to target channel/signal

A brief summary of the TCI selection scheme for each target channel/signal according to current agreements is provided in Table 3-1 as reference, including both S-DCI and M-DCI operation.

Table 3-1 Summary of TCI selection scheme for each target channel/signal in S-DCI/M-DCI based MTRP operation

| S-DCI based MTRP operation |  |  |
| --- | --- | --- |
| Channel/signal | Conclusion | TCI selection scheme |
| PDCCH | Yes | RRC configuration per CORESET (Detail to be discussed in Issue 3.1) |
| PDSCH scheduled/activated by DCI format 1_1/1_2 if TCI selection field is present in DCI format 1_1/1_2 | Yes | An RRC-configurable new indicator field “TCI selection field” in DCI format 1_1/1_2 (Detail to be discussed in Issue 3.2) |
| PDSCH scheduled/activated by DCI format 1_1/1_2 if TCI selection field is not present in DCI format 1_1/1_2 | No | To be discussed in Issue 3.3 |
| PDSCH scheduled/activated by DCI format 1_0 | No | To be discussed in Issue 3.6 |
| PUSCH scheduled/activated by DCI format 0_1/0_2 (including DG-PUSCH and Type2 CG-PUSCH) | Yes | An indicator field in DCI format 0_1/0_2 (Using the existing or a new DCI field and detail to be discussed in Issue 3.4) |
| PUSCH scheduled/activated by DCI format 1_0 (including DG and Type2 CG) | No | To be discussed in Issue 3.6 |
| Type1 CG-PUSCH | No |  |
| PUCCH | Yes | RRC configuration per PUCCH resource/resource group |
| AP CSI-RS for CSI/BM | No | To be discussed in Issue 3.8 |
| SRS for CB/NCB | No |  |
| SRS for antenna switching | No |  |
| AP SRS for BM | No |  |
| M-DCI based MTRP operation |  |  |
| Channel/signal | Conclusion | TCI selection scheme |
| PDCCH | Yes | According to coresetPoolIndex value |
| PDSCH scheduled/activated by DCI format 1_0/1_1/1_2 | Yes | According to coresetPoolIndex value corresponding to scheduling PDCCH |
| PUSCH scheduled/activated by DCI format 0_0/0_1/0_2 (including DG-PUSCH and Type2 CG-PUSCH) | Yes | According to coresetPoolIndex value corresponding to scheduling PDCCH |
| Type1 CG-PUSCH | No |  |
| PUCCH | No | To be discussed in Issue 3.7 |
| AP CSI-RS for CSI/BM | No | To be discussed in Issue 3.9 |
| SRS for CB/NCB | No | To be discussed in Issue 3.10 |
| SRS for antenna switching | No |  |
| AP SRS for BM | No |  |

Table 3-2 Summary for Issue 3

| # | Issue | Companies’ view and Recommended Proposal |
| --- | --- | --- |
| 3.1 | PDCCH reception in S-DCI based MTRP operation, whether Rel-17 rule is reused when provide the RRC configuration for TCI selection | Proposal 3.1: On unified TCI framework extension for S-DCI based MTRP: - If a CORESET other than a CORESET with index 0 is associated only with USS sets and/or Type3-PDCCH CSS sets, the CORESET is configured by RRC to apply the first indicated joint/DL TCI state, the second indicated joint/DL TCI state, or both first and second indicated joint/DL TCI states to PDCCH reception on the CORESET - If a CORESET other than a CORESET with index 0 is associated at least with CSS sets other than Type3-PDCCH CSS sets and followUnifiedTCIstate = 'enabled' is configured for the CORESET, the CORESET is configured by RRC to apply the first indicated joint/DL TCI state, the second indicated joint/DL TCI state, or both first and second indicated joint/DL TCI states to PDCCH reception on the CORESET - If a CORESET with index 0 is configured with followUnifiedTCIstate = 'enabled': - If the CORESET is associated with SS#0 for Type 0/0A/2 CSS sets, the CORESET is configured by RRC to apply the first indicated joint/DL TCI state or the second indicated joint/DL TCI state to PDCCH reception on the CORESET - Otherwise, the CORESET is configured by RRC to apply the first indicated joint/DL TCI state, the second indicated joint/DL TCI state, or both first and second indicated joint/DL TCI states to PDCCH reception on the CORESET FL note: Regarding CORESET#0, applying both indicated joint/DL TCI states is precluded due to the following Rel-17 agreement for PDCCH-SFN: Agreement UE does not expect CORESET#0 to be activated with two TCI states when it is associated with SS#0 for Type 0/0A/2 CSS - Send an LS to inform RAN2 about this agreement - The final LS in R1-2208203 |
| 3.2 | PDSCH reception in S-DCI based MTRP, detail design of TCI selection field | Proposal 3.2: On unified TCI framework extension for S-DCI based MTRP, a 2-bit [TCI selection field] can be configured by RRC to be present in a DCI format 1_1/1_2 that schedules/activates PDSCH reception (including dynamic PDSCH and SPS PDSCH) according to the followings: - If the DCI format 1_1/1_2 indicates codepoint "00" for the [TCI selection field], the UE shall apply the first one of two indicated joint/DL TCI states to all PDSCH DMRS port(s) of all PDSCH transmission occasions(s) scheduled/activated by the DCI format 1_1/1_2 - If the DCI format 1_1/1_2 indicates codepoint "01" for the [TCI selection field], the UE shall apply the second one of two indicated joint/DL TCI states to all PDSCH DMRS port(s) of all PDSCH transmission occasions(s) scheduled/activated by the DCI format 1_1/1_2 - If the DCI format 1_1/1_2 indicates codepoint "10" for the [TCI selection field], the UE shall apply both indicated joint/DL TCI states to the PDSCH reception scheduled/activated by the DCI format 1_1/1_2 - FFS: Whether and how to use the codepoint "11" of the [TCI selection field] If the UE is in FR1, or the UE supports the capability of two default beams for S-DCI based MTRP in FR2, above apply to all PDSCH reception(s) scheduled/activated by the DCI format 1_1/1_2. If the UE doesn’t support the capability of two default beams for S-DCI based MTRP in FR2, above apply to the scheduled/activated PDSCH reception when the offset between the reception of the scheduling DCI format 1_1/1_2 and the scheduled/activated PDSCH reception is equal to or larger than a threshold - FFS: How to apply the indicated joint/DL TCI state(s) to the scheduled/activated PDSCH reception if the offset between the reception of the scheduling DCI format 1_1/1_2 and the scheduled/activated PDSCH reception is less than a threshold in FR2 FFS: Detail of the capability of two default beams for S-DCI based MTRP FL note: Regarding whether the threshold is needed, this issue has been captured in our previous agreement as follows. Thus, I would suggest to copy the FFS in previous agreement to make sure that this issue is still open even the proposal is agreed. Agreement On unified TCI framework extension for S-DCI based MTRP, a DCI field in DCI format 1_1/1_2 that schedules/activates PDSCH reception is used to determine which one or both of the indicated joint/DL TCI states shall be applied to the scheduled/activated PDSCH reception - The presence of the DCI field is configurable by RRC; when the DCI field is not present in DCI format 1_1/1_2, the UE shall apply the default indicated joint/DL TCI state(s) to PDSCH reception - FFS: Details on the default indicated joint/DL TCI state(s) to PDSCH reception - FFS: The DCI field is a new indicator field or an existing field (e.g., the existing TCI field) - FFS: Regardless the DCI field is present or not present, how to apply the indicated joint/DL TCI state(s) to PDSCH reception if the offset between the reception of the DCI format 1_1/1_2 and the corresponding PDSCH reception is less than a threshold FFS: How to apply the indicated joint/DL TCI state(s) to PDSCH reception scheduled/activated by DCI format 1_0. Above applies for the case where PDSCHs scheduled by the same DCI. FL note: Based on my observation from contributions, threshold may be still needed for the case if the UE cannot support applying two indicated joint/DL TCI states “simultaneously”. Note that even a UE supports MTRP operation, support of simultaneous reception is not mandated, e.g., the UE may support only TDM Tx scheme. Only if UE supports the capability of applying two TCI states simultaneously (e.g., based on FG16-2b-0), the threshold may not be needed. |
| 3.3 | PDSCH reception in S-DCI based MTRP operation, the default indicated joint/DL TCI state(s) for PDSCH reception scheduled/ activated by DCI format 1_1/1_2 w/o TCI selection field | Proposal 3.3: On unified TCI framework extension for S-DCI based MTRP, down-select one of the followings for PDSCH reception scheduled/activated by DCI format 1_1/1_2 configured w/o the [TCI selection field]: - Alt1: Using RRC configuration to inform that the UE shall apply the first one, the second one, or both of two indicated joint/DL TCI states to the scheduled/activated PDSCH reception - Alt2: The UE shall apply the first one of two indicated joint/DL TCI states to the scheduled/activated PDSCH reception - Alt3: The UE shall apply both of two indicated joint/DL TCI states to the scheduled/activated PDSCH reception - Alt4: The UE shall apply the same joint/DL TCI state(s) that is applied to the PDCCH reception with the scheduling/activation DCI to the scheduled/activated PDSCH reception - Alt5: Which indicated joint/DL TCI state(s) is/are applied to the scheduled/activated PDSCH reception is determined according to the existing TCI field Above applies at least if the offset between the reception of the scheduling DCI format 1_1/1_2 and the scheduled/activated PDSCH reception is equal to or larger than a threshold |
| 3.4 | PUSCH transmission in S-DCI based MTRP operation, detail design of the indicator field in a DCI format 0_1/0_2 for informing which joint/UL TCI state(s) indicated by MAC-CE/DCI the UE shall apply to PUSCH transmission scheduled/activated by the DCI format 0_1/0_2 | Proposal 3.4: On unified TCI framework extension for S-DCI based MTRP, when two SRS resource sets for CB/NCB are configured, support the followings for PUSCH transmission scheduled/activated by a DCI format 0_1/0_2 (including DG and Type2 CG): - If the DCI format 0_1/0_2 indicates codepoint "00" for the existing SRS resource set indicator, the UE shall apply the first indicated joint/UL TCI state to all PUSCH antenna port(s) of all PUSCH transmission occasions(s) - If the DCI format 0_1/0_2 indicates codepoint "01" for the existing SRS resource set indicator, the UE shall apply the second indicated joint/UL TCI state to all PUSCH antenna port(s) of all PUSCH transmission occasions(s) - If the DCI format 0_1/0_2 indicates codepoint "10" or “11” for the existing SRS resource set indicator: - For TDM based PUSCH Tx scheme, the UE shall apply the first indicated joint/UL TCI state to the PUSCH transmission occasions(s) associated with the first SRS resource set for CB/NCB, and the second indicated joint/UL TCI state to the PUSCH transmission occasions(s) associated with the second SRS resource set for CB/NCB (note: the association between an SRS resource set for CB/NCB and PUSCH transmission occasions(s) is defined according to TS 38.214) - FFS: SDM and SFN based PUSCH Tx schemes FFS: The case that the spatial Tx filter(s) determined from the indicated joint/UL TCI state(s) applied to a PUSCH transmission is different from the spatial Tx filter(s) used for the SRS transmission corresponding to the SRS resource(s) indicated to the PUSCH transmission |
| 3.5 | Beam misalignment between a PUSCH transmission and the SRS indicated to the PUSCH transmission | Proposal 3.5: On unified TCI framework extension, study how to handle the case if the spatial Tx filter(s) determined from the indicated joint/UL TCI state(s) applied to a PUSCH transmission is not aligned with the spatial Tx filter(s) used for the SRS transmission corresponding to the SRS resource(s) indicated to the PUSCH transmission |
| 3.6 | PDSCH/PUSCH scheduled/activated by fallback DCI (DCI format 1_0/0_1) in S-DCI based MTRP operation | Proposal 3.6: On unified TCI framework extension for S-DCI based MTRP: - The UE shall apply the first indicated joint/DL TCI state to PDSCH reception scheduled/activated by DCI format 1_0 (including DG and SPS) if the UE is not configured with sfnSchemePdsch - The UE shall apply both first and second indicated joint/DL TCI states to PDSCH reception scheduled/activated by DCI format 1_0 (including DG and SPS) if the UE is configured with sfnSchemePdsch - The UE shall apply the first indicated joint/UL TCI state to PUSCH transmission scheduled/activated by DCI format 0_0 (including DG and Type2 CG) |
| 3.7 | PUCCH transmission in M-DCI based MTRP operation | Proposal 3.7: On unified TCI framework extension for M-DCI based MTRP, down-select at least one alternative from the followings for PUCCH transmission: - Alt1: A coresetPoolIndex value is provided per PUCCH resource/resource group, and the UE shall apply the indicated joint/UL TCI state specific to the coresetPoolIndex value to the corresponding PUCCH transmission - Alt2: An RRC configuration is provided per PUCCH resource/resource group to inform that the UE shall apply the first or the second indicated joint/UL TCI state to the corresponding PUCCH transmission, where the first and the second indicated joint/DL TCI states correspond to the indicated joint/UL TCI states specific to coresetPoolIndex value 0 and value 1, respectively. - Alt3: For a PUCCH transmission scheduled by a DCI with DL assignment on a CORESET when the UE is not provided with ackNackFeedbackMode = joint, the UE shall apply the indicated joint/UL TCI state specific to a coresetPoolIndex value to the PUCCH transmission triggered by PDCCH on a CORESET, where the coresetPoolIndex value is determined from the one associated with the CORESET. Otherwise, Alt1 or Alt2 is adopted. - Alt4: For a PUCCH transmission with an LLR trigged for either the first BFD-RS set (q0,0) or the second BFD-RS set (q0,1) when the UE is provided only one schedulingRequestID-BFR configuration, the UE shall apply the indicated joint/UL TCI state specific to a coresetPoolIndex value to the PUCCH transmission, where the coresetPoolIndex value is 1 when the LRR is trigged for the first BFD-RS set (q0,0) and the coresetPoolIndex value is 0 when the LRR is trigged for the second BFD-RS set (q0,1) . Otherwise, Alt1 or Alt2 is adopted. |
| 3.8 | AP CSI-RS for CSI/BM in S-DCI based MTRP | Proposal 3.8: On unified TCI framework extension for S-DCI based MTRP, if QCL-Info is absent in CSI-AssociatedReportConfigInfo of CSI-AperiodicTriggerState for an aperiodic CSI-RS resource set configured for CSI/BM, down-select one alternative from the followings: - Alt1: An RRC configuration is provided in CSI-AssociatedReportConfigInfo of CSI-AperiodicTriggerState for the aperiodic CSI-RS resource set to inform that the UE shall apply the first or the second indicated joint/DL TCI state to the aperiodic CSI-RS resource set - Alt2: On aperiodic CSI-RS for enhanced group-based beam reporting or NCJT CSI measurement, the indicated joint/DL TCI state applied to the aperiodic CSI-RS is determined based on a fixed rule. Otherwise, Alt1 is adopted. |
| 3.9 | AP CSI-RS for CSI/BM in M-DCI based MTRP | Proposal 3.9: On unified TCI framework extension for M-DCI based MTRP, if QCL-Info is absent in CSI-AssociatedReportConfigInfo of CSI-AperiodicTriggerState for an aperiodic CSI-RS resource set configured for CSI/BM, down-select at least one alternative from the followings: - Alt1: A coresetPoolIndex value is provided in CSI-AssociatedReportConfigInfo of CSI-AperiodicTriggerState for the aperiodic CSI-RS resource set, and the UE shall apply the indicated joint/DL TCI state specific to the coresetPoolIndex value to the aperiodic CSI-RS resource set - Alt2: An RRC configuration is provided in CSI-AssociatedReportConfigInfo of CSI-AperiodicTriggerState for the aperiodic CSI-RS resource set to inform that the UE shall apply the first or the second indicated joint/DL TCI state to the aperiodic CSI-RS resource set, where the first and the second indicated joint/DL TCI states correspond to the indicated joint/DL TCI states specific to coresetPoolIndex value 0 and value 1, respectively. - Alt3: The UE shall apply the indicated joint/DL TCI state specific to a coresetPoolIndex value to the aperiodic CSI-RS resource set triggered by PDCCH on a CORESET, where the coresetPoolIndex value is determined from the one associated with the CORESET - Alt4: On aperiodic CSI-RS for enhanced group-based beam reporting, the indicated joint/DL TCI state applied to the aperiodic CSI-RS is determined based on a fixed rule. Otherwise, Alt1 or Alt2 is adopted. |
| 3.10 | SRS for CB/NCB in M-DCI based MTRP operation | Proposal 3.10: On unified TCI framework extension for M-DCI based MTRP, when two SRS resource sets for CB/NCB are configured for PUSCH+PUSCH STxMP, the UE shall apply the indicated joint/UL TCI state specific to a coresetPoolIndex value to an SRS resource set for CB/NCB associated with the same coresetPoolIndex value if the SRS resource set for CB/NCB is configured to follow the indicated joint/UL TCI state by followUnifiedTCIState - Note: Associations between SRS resource sets for CB/NCB and coresetPoolIndex values are defined in AI 9.1.4.1 - FFS: The case if one single SRS resource set for CB/NCB is configured FFS: SRS for antenna switching and SRS for BM FL note: According to the following agreement from AI 9.1.4.1, when two SRS resource sets for CB/NCB are configured (for PUSCH STxMP), there are implicit associations between the SRS resource sets for CB/NCB and coresetPoolIndex values. Thus, Proposal 3.10 is recommended for this case accordingly. Other cases can be further discussed, e.g., only one single SRS resource set for CB/NCB is configured, SRS for AS and BM. Agreement from AI 9.1.4.1 in RAN1#111 - For multi-DCI based STxMP, to schedule a PUSCH for STxMP PUSCH+PUSCH transmission, - Alt1: The first SRS resource set is associated with coresetPoolIndex value 0 and the other SRS resource set is associated with coresetPoolIndex value 1 - The PUSCH is associated with SRS resource set with the same value of coresetPoolIndex - FFS: Which is the first SRS resource set, e.g., the set with lower set ID. - For Type 1 CG-PUSCH, one SRS_resource_set_index value is configured in RRC in ConfiguredGrantConfig and the srs-ResourceIndicator/precodingAndNumberOfLayers correspond to the SRS resource set |
| 3.11 | PDSCH default beam in M-DCI based MTRP | Proposal 3.11: On unified TCI framework extension for M-DCI based MTRP, investigate whether/how to apply the indicated joint/DL TCI state(s) to the scheduled/activated PDSCH reception if the offset between the reception of the scheduling DCI format 1_1/1_2 and the scheduled/activated PDSCH reception is less than a threshold at least for UE does not support FG 16-2a-6 |

Table 3-3 Company input for Issue 3

| Company | Input |
| --- | --- |
| Mod V00 | - Please check the updated Proposal 3.2: - In this proposal, let’s focus on the design of the [TCI selection field]. For the details of the mapping order and QCL types/assumptions when two indicated joint/DL TCI states are applied to PDSCH reception (if the [TCI selection field] indicates codepoint “01”), we can further discuss. - Regarding the last bullet, I think everyone has common understanding that this proposal should be applied “at least” to scheduling offset >= threshold. If this is true, let’s focus on this case in this proposal. Whether the proposal can be extended to scheduling offset < threshold, it can be discussed separately. Note that we already have one agreement capturing the related issue. - Regarding whether to add “if two joint/DL TCI states are indicated” in the main bullet, at least I don’t see the need since all the UE behaviors in the sub-bullets are defined under the case if there are two indicated joint/DL TCI states. - Please check the updated Proposal 3.3: - Regarding the last bullet, I think everyone has common understanding that this proposal should be applied “at least” to scheduling offset >= threshold. If this is true, let’s focus on this case in this proposal. Whether the proposal can be extended to scheduling offset < threshold, it can be discussed separately. Note that we already have one agreement capturing the related issue. - Regarding whether to add “if two joint/DL TCI states are indicated” in the main bullet, at least I don’t see the need since all the UE behaviors in the alternatives are defined under the case if there are two indicated joint/DL TCI states. - Please check the updated Proposal 3.4, 3.7, 3.8, 3.9 Agreement On unified TCI framework extension for S-DCI based MTRP, a DCI field in DCI format 1_1/1_2 that schedules/activates PDSCH reception is used to determine which one or both of the indicated joint/DL TCI states shall be applied to the scheduled/activated PDSCH reception - The presence of the DCI field is configurable by RRC; when the DCI field is not present in DCI format 1_1/1_2, the UE shall apply the default indicated joint/DL TCI state(s) to PDSCH reception - FFS: Details on the default indicated joint/DL TCI state(s) to PDSCH reception - FFS: The DCI field is a new indicator field or an existing field (e.g., the existing TCI field) - FFS: Regardless the DCI field is present or not present, how to apply the indicated joint/DL TCI state(s) to PDSCH reception if the offset between the reception of the DCI format 1_1/1_2 and the corresponding PDSCH reception is less than a threshold FFS: How to apply the indicated joint/DL TCI state(s) to PDSCH reception scheduled/activated by DCI format 1_0. Above applies for the case where PDSCHs scheduled by the same DCI. |
| Xiaomi | Proposal 3.2 Support Proposal 3.3 First we would like to clarify that only one common option will be selected for different PDSCH transmission schemes or different option can be selected for different PDSCH transmission scheme. If common option will be selected for different PDSCH transmission schemes, Alt 2 need to be removed since it will be not suitable for UE configured with PDSCH-SFN and dynamic SFN is not supported. Second, in legacy system, DCI format without TCI field apply same rule for determination of default beam. It means that DCI format 1_0 should be discussed together. Third one, just a typo in Alt 5, update “PDCCH” to “PDSCH”. Proposal 3.7 For Alt 3, if the UE is provided with ackNackFeedbackMode = joint, how to apply the indicated TCI state to the PUCCH resource if not apply the indicated TCI state of the corresponding CORESET? Apply two TCI states or apply the one different of the corresponding CORESET? |
| Panasonic | Proposal 3.1: Support Proposal 3.2: We do not agree to the phrase “Above applies at least if the offset between the reception of the scheduling DCI format 1_1/1_2 and the scheduled/activated PDSCH reception is equal to or larger than a threshold” The offset should be related to whether the UE had enough time to switch to the beams either indicated by the scheduling DCI or an earlier beam indication DCI. The TCI field of the scheduling DCI does not necessarily indicate new beams in our understanding. Proposal 3.3: We do not agree to adding Alt 5 - Alt5: Which indicated joint/DL TCI state(s) is/are applied to the scheduled/activated PDSCH reception is determined according to the existing TCI field I think this way to design was abandoned when we agreed to a new indicator field in the DCI. Moreover, the UE is indicated with one or two TCI states. If only one, then the UE uses that TCI state. If two, then the UE needs to know whether it is single or multiple TRP operation in first place. In this sense, Alt1 and Alt4 provide this flexibility. Alt 2 assumes single TRP operation, and Alt 3 always assumes two TRP operation. Proposal 3.4: We support, although we prefer having an agreement for every PUSCH scheme separately to improve clarity. |
| Lenovo | Proposal 3.2: We understand that the 2-bit [TCI selection field] only useful when two TCI states are indicated by the TCI field or be indicated by the MAC CE, thus the case when only one TCI state is indicated/activated, but the 2-bit [TCI selection field] is configured for the UE should also be considered in this proposal. Proposal 3.3: More clarification for Alt3 is needed: Does Alt.3 means that only MTRP based PDSCH scheme can be scheduled for this case? |
| QC | For both proposal 3.2 and 3.3, suggest to have FFS on offset less than threshold at least when UE only supports a single default beam, as well as the mDCI case 16-2b-0 Two default beams for single-DCI based multi-TRP 1. Support of default QCL assumption with two TCI states 16-2c defaultQCL-TwoTCI-r16 16-2a-6 Default QCL enhancement for multi-DCI based multi-TRP 1. Support of default QCL assumption per CORESETPoolIndex 16-2a and 16-2c defaultQCL-PerCORESETPoolIndex-r16 For Proposal 3.5, suggest to add “whether”, On unified TCI framework extension, study “whether/” how to handle |
| Huawei, HiSilicon | Proposal 3.1: Support Proposal 3.2: Support Proposal 3.3: We suggest to reduce the number of alternatives. We are not sure we understand Alt5. We assume the “PDCCH” should be “PDSCH”. Besides, we don’t think it is reasonable to rely on the TCI field to determine the TCI state to be applied for the scheduled PDSCH. In unified TCI framework, TCI field is used for TCI state update, rather than TCI state indication for the scheduled PDSCH. In addition, the TCI field itself may be not present. This would make the situation even more complicated, as we need to further study the case where the TCI field is not present if Alt 5 is supported. Proposal 3.4: Support Proposal 3.5: Support Proposal 3.6: CJT-PDSCH may also be scheduled using DCI 1_0 and should be considered in the proposal. The UE behavior can be similar to SFN. We suggest the following modification: Proposal 3.6 (Modified): On unified TCI framework extension for S-DCI based MTRP: - The UE shall apply the first indicated joint/DL TCI state to PDSCH reception scheduled/activated by DCI format 1_0 (including DG and SPS) if the UE is not configured with sfnSchemePdsch or if the UE is configured with CJT-PDSCH and UE supports 1 indicated joint TCI states for PDSCH-CJT. - The UE shall apply both first and second indicated joint/DL TCI states to PDSCH reception scheduled/activated by DCI format 1_0 (including DG and SPS) if the UE is configured with sfnSchemePdsch or if the UE is configured with CJT-PDSCH and UE supports 2 indicated joint TCI states for PDSCH-CJT. The UE shall apply the first indicated joint/UL TCI state to PUSCH transmission scheduled/activated by DCI format 0_0 (including DG and Type2 CG) Proposal 3.7: Support in principle. In our understanding, these alternatives are not mutually exclusive and more than one of them can be supported. Alt 3 and Alt 4 are rule based solutions for special cases where there is a natural association between the PUCCH and a TRP (and the corresponding TCI state) and Alt 1 and Alt 2 are two competing mechanisms used for other cases. As such, we suggest to add the following Note at the bottom of the Proposal: Note: If Alt3 and/or Alt4 are down-selected to cover their corresponding special cases, one of Alt1 or Alt2 should also be selected to cover other cases. We can support Alt 3 and Alt 4 as dedicated rules for the special cases and Alt 2 as a general mechanism for other PUCCH cases. Comparing to Alt 1, Alt 2 supports reusing the new RRC parameter for TCI selection for PUCCH in SDCI case as agreed in the last meeting: Agreement On unified TCI framework extension for S-DCI based MTRP, use RRC configuration to inform that the UE shall apply the first one, the second one, or both of the indicated joint/UL TCI states to a PUCCH resource/group - Note: Detail of the RRC configuration is left to RAN2 design. Proposal 3.8: Support in principle. For the cases that are included in Alt2, there is a natural association between CSI-RS and a TRP (or corresponding TCI state) and there is no need that such association is determined by RRC (further details in the bottom of this comment). For all other cases, Alt1 should be used. In view of this, we suggest to modify the proposal as follows: Proposal 3.8 (modified): On unified TCI framework extension for S-DCI based MTRP, if QCL-Info is absent in CSI-AssociatedReportConfigInfo of CSI-AperiodicTriggerState for an aperiodic CSI-RS resource set configured for CSI/BM, down-select at least one alternative from the followings: - Alt1: An RRC configuration is provided in CSI-AssociatedReportConfigInfo of CSI-AperiodicTriggerState for the aperiodic CSI-RS resource set to inform that the UE shall apply the first or the second indicated joint/DL TCI state to the aperiodic CSI-RS resource set - Alt2: The indicated joint/DL TCI state applied to aperiodic CSI-RS for enhanced group-based beam reporting or NCJT CSI measurement is determined based on a fixed rule. For all other cases, Alt1 applies. Following is further explanation for Alt2: For enhanced group-based beam reporting, two aperiodic CSI-RS resource sets can be associated with an aperiodic reportConfig which is configured with groupBasedBeamReporting-r17. CSI-RS resources in different resource sets correspond to different TRPs and, thus, should adopt different indicated joint/DL TCI states. In particular, if the CSI-RS resource(s) in the first aperiodic CSI-RS resource set are not provided with QCL-Info, the first indicated joint/DL TCI state in the sDCI case or the indicated joint/DL TCI state specific to coresetPoolIndex 0 in the mDCI case should be applied for the CSI-RS resource(s). In turn, if the CSI-RS resource(s) in the second aperiodic CSI-RS resource sets are not provided with QCL-Info, the second indicated joint/DL TCI state in the sDCI case or the joint/DL TCI state specific to coresetPoolIndex 1 in the mDCI case should be adopted for the CSI-RS resource(s). For NCJT CSI measurement, one or more resource pairs can be configured by cmrGroupingAndPairing-r17 in a resource set where each resource pair includes two CSI-RS resources. Since the two CSI-RS resources are used for channel measurement of different TRPs, they should adopt different indicated joint/DL TCI states. Therefore, Alt1 cannot apply in this case as it applies a single TCI state to all resources in the same resource set. We think, if the first CSI-RS resource in a resource pair is not provided with QCL-Info, the first indicated joint/DL TCI state is applied to the CSI-RS resource. Similarly, if the second CSI-RS resource in a resource pair is not provided with QCL-Info, the second indicated joint/DL TCI state is applied to the CSI-RS resource. Proposal 3.9: Regarding Alt4, we think NCJT CSI measurement corresponds to the sDCI case only and should be removed from Alt4. For mDCI based enhanced group-based beam reporting, there is a natural association between CSI-RS and a TRP (or corresponding TCI state) and there is no need that such association is determined by a RRC parameter or coresetpoolIndex configuration (further details in the bottom of this comment). For all other cases, we think Alt2 or Alt3 can be used. In view of this, we suggest to modify the proposal as follows: Proposal 3.9 (modified): On unified TCI framework extension for M-DCI based MTRP, if QCL-Info is absent in CSI-AssociatedReportConfigInfo of CSI-AperiodicTriggerState for an aperiodic CSI-RS resource set configured for CSI/BM, down-select at least one alternative from the followings: - Alt1: A coresetPoolIndex value is provided in CSI-AssociatedReportConfigInfo of CSI-AperiodicTriggerState for the aperiodic CSI-RS resource set, and the UE shall apply the indicated joint/DL TCI state specific to the coresetPoolIndex value to the aperiodic CSI-RS resource set - Alt2: An RRC configuration is provided in CSI-AssociatedReportConfigInfo of CSI-AperiodicTriggerState for the aperiodic CSI-RS resource set to inform that the UE shall apply the first or the second indicated joint/DL TCI state to the aperiodic CSI-RS resource set, where the first and the second indicated joint/DL TCI states correspond to the indicated joint/DL TCI states specific to coresetPoolIndex value 0 and value 1, respectively. - Alt3: The UE shall apply the indicated joint/DL TCI state specific to a coresetPoolIndex value to the aperiodic CSI-RS resource set triggered by PDCCH on a CORESET, where the coresetPoolIndex value is determined from the one associated with the CORESET Alt4: The indicated joint/DL TCI state applied to aperiodic CSI-RS for enhanced group-based beam reporting or NCJT CSI measurement is determined based on a fixed rule, for all other cases, Alt2 or Alt3 applies. For enhanced group-based beam reporting, two aperiodic CSI-RS resource sets can be associated with an aperiodic reportConfig which is configured with groupBasedBeamReporting-r17. CSI-RS resources in different resource sets correspond to different TRPs and, thus, should adopt different indicated joint/DL TCI states. In particular, if the CSI-RS resource(s) in the first aperiodic CSI-RS resource set are not provided with QCL-Info, the first indicated joint/DL TCI state in the sDCI case or the indicated joint/DL TCI state specific to coresetPoolIndex 0 in the mDCI case should be applied for the CSI-RS resource(s). In turn, if the CSI-RS resource(s) in the second aperiodic CSI-RS resource sets are not provided with QCL-Info, the second indicated joint/DL TCI state in the sDCI case or the joint/DL TCI state specific to coresetPoolIndex 1 in the mDCI case should be adopted for the CSI-RS resource(s). Proposal 3.10: OK in principle Suggest to use a unified terminology as in the agreement in 9.1.4.1: Proposal 3.10: (modified) On unified TCI framework extension for M-DCI based MTRP, when two SRS resource sets for CB/NCB are configured (for PUSCH STxMP) for STxMP PUSCH+PUSCH transmission, the UE shall apply the indicated joint/UL TCI state specific to a coresetPoolIndex value to an SRS resource set for CB/NCB associated with the same coresetPoolIndex value if the SRS resource set for CB/NCB is configured to follow the indicated joint/UL TCI state by followUnifiedTCIState - Note: Associations between SRS resource sets for CB/NCB and coresetPoolIndex values are defined in AI 9.1.4.1 - FFS: The case if one single SRS resource set for CB/NCB is configured FFS: SRS for antenna switching and SRS for BM |
| Sharp | Proposal 3.2: Support Proposal 3.3: Support Proposal 3.4: Support Proposal 3.7: Support Proposal 3.8: Support Proposal 3.9: Support |
| vivo | Proposal 3.1: Support Proposal 3.2: Support Proposal 3.3: We still think the always-on [TCI selection field] is the fallback alternative if no Alt is agreed. Proposal 3.4: Fine with the proposal. The FFS part is not needed in our view, the behavior of such a case is clear. Proposal 3.7: In Alt3, we don’t know why joint HARQ-ACK feedback is separately treated. Alt4 seems not necessary, because the PUCCH-SR is associated with a coresetPoolIndex anyway. Proposal 3.8: Support in principle. The solution in Alt2 needs FFS. Proposal 3.9: Support in principle. The solution in Alt4 needs FFS. Proposal 3.10: Propose to remove for PUSCH STxMP |
| ZTE | Proposal 3.2: Some update for red part? Proposal 3.7: For Alt3, we do not identify the necessity since we already have the dynamic indication for PUCCH resource. Then, for Alt4, we are wondering whether it should be discussed in Issue-6. As a normal case, PUCCH beam/PC update has not bee supported for mTRP-BFR even for mDCI-mTRP. Proposal 3.8: We support Alt1. |
| OPPO | On Proposal 3.2, we are in general supportive. We also think the threshold between scheduling DCI containing the [TCI selection field] and the scheduled PDSCH should be studied anyway. But we failed to foreseen issue for the case when [TCI selection field] is not presented. For this case, UE just applies the indicated DL/joint TCI state(s), there seems no dynamic factor to impact UE reception on PDSCH. On Proposal 3.4, we are also supportive to the main bullets. Regarding the FFS, we understand that as a timeline issue between indicated UL/joint TCI state(s) for PUSCH and the rule in spec that PUSCH ports always following previously transmitted SRS port(s). If that’s the case, we think this issue also exist in Rel.17 STRP case. It would be better to address issue for both STRP and MTRP or for none of them. |
| Mod V15 | Please check the updated Proposal 3.2 |
| Fujitsu | Proposal 3.2: We are fine with the updated proposal. And in our understanding, the FFS is mainly related to the time threshold needed for DCI decoding and possible beam switching, if so, we think timeDurationForQCL is a natural choice for the threshold. Proposal 3.4: Support. |
| CEWiT | Proposal 3.2: Support Proposal 3.3: Support Proposal 3.4: Support Proposal 3.7: Support |
| Google | Proposal 3.2: Support FL’s updated proposal. Proposal 3.3: We suggest removing Alt5. If we can rely on existing TCI field, what’s the point of introducing a new field? And we also think discussion here should consider case with/without SFN PDSCH as Proposal 3.6. Proposal 3.4: Generally OK. But for the last FFS, we share similar views with some companies in offline that we should not reopen the door of long debate. Proposal 3.7: Alt3 and Alt4 are not complete solutions, which only handle some special cases. If we want to discuss these cases, we may have separate discussions. |

- Issue 4 – UL power control for UL MTRP

Table 4-1 Summary for Issue 4

| # | Issue | Companies’ view and Recommended Proposal |
| --- | --- | --- |
| 4.1 | Extend the previous agreement to all PUSCH/PUCCH Tx schemes (including S-DCI and M-DCI based MTRP operation) and SRS | Proposal 4.1: On unified TCI framework extension, if an indicated joint/UL TCI state(s) applies to a PUSCH/PUCCH/SRS transmission occasion(s) or antenna port(s), the UE shall determine UL Tx power for the PUSCH/PUCCH/SRS transmission occasion(s) or antenna port(s) based on the UL PC parameter setting for PUSCH/PUCCH/SRS, if any, and the PL-RS included in the indicated joint/UL TCI state - FFS: For STxMP, the maximum Tx power when the UE determines UL Tx power for the PUSCH/PUCCH transmission occasion(s) or antenna port(s) (discussed after receiving RAN4 reply on UE power limitation for STxMP in FR2) - FFS: Default UL PC parameter setting(s) if one or both of indicated joint/UL TCI states applied to PUSCH/PUCCH/SRS transmission occasion(s) or antenna port(s) does/do not include the UL PC parameter setting(s) for PUCCH/PUSCH/SRS Agreement from 9.1.1.1 in RAN1#109 On unified TCI framework extension, if an indicated joint or UL TCI state applies to a PUSCH/PUCCH transmission occasion at least for S-DCI based PUSCH/PUCCH repetition with TDM and the indicated joint or UL TCI state is associated with an UL PC parameter setting for PUSCH/PUCCH (including P0, alpha for PUSCH, and closed loop index) and a PL-RS, the UE should apply the UL PC parameter setting and the PL-RS for the PUSCH /PUCCH transmission occasion. - FFS: How to extend to other Rel-18 MTRP scheme(s) with STxMP, if supported - FFS: UL PC enhancement for CB and non-CB SRS in above case FFS: The applied UL PC parameter setting if one or both indicated joint or UL TCI state(s) is not associated with an UL PC parameter setting (including P0, alpha for PUSCH, and closed loop index) for PUCCH/PUSCH |
| 4.2 | How to determine the UL PC parameter setting(s) if one or both indicated joint/UL TCI state(s) is not associated with an UL PC parameter setting (including P0, alpha for PUSCH, and closed loop index) for PUCCH/PUSCH | Proposal 4.2: On unified TCI framework extension, if one or both of indicated joint/UL TCI states applied to PUSCH/PUCCH/SRS transmission occasion(s) or antenna port(s) does/do not include the UL PC parameter setting(s) for PUSCH/PUCCH/SRS, down-select one alternative from the followings: - Alt1: Support two default UL PC parameter settings for PUSCH/PUCCH/SRS configured in BWP-UplinkDedicated, and the UE should apply the one or two default UL PC parameter settings for PUSCH/PUCCH/SRS configured in the corresponding UL BWP - FFS: 1-to-1 association between an indicated joint/UL TCI state and a default UL PC parameter setting - Alt2: No change from Rel-17 unified TCI framework |

Table 4-2 Company input for Issue 4

| Company | Input |
| --- | --- |
| Mod V00 | - No update to Proposal 4.1 and 4.2 |
| Huawei, HiSilicon | Proposal 4.1: OK Proposal 4.2: OK and support Alt1. |
| ZTE | Proposal 4.1: OK if the condition of misalignment between SRS and PUCCH is clarified. Proposal 4.2: Alt2, and we are also fine for no consensus. |
| OPPO | On Proposal 4.1: Support. |
| Google | Proposal 4.1: Support |

- Issue 5 – PDSCH-CJT

Table 5-1 Summary for Issue 5

| # | Issue | Companies’ view and Recommended Proposal |
| --- | --- | --- |
| 5.1 | QCL type(s)/assumption(s) if two indicated joint TCI states are applied to PDSCH-CJT | Alt1: PDSCH DMRS port(s) is QCLed with the DL RSs of both indicated joint TCI states with respect to QCL-TypeA - Support: Huawei, Spreadtrum, OPPO, Ericsson, Xiaomi, CATT, Qualcomm, Nokia, Docomo, CMCC, Lenovo, NEC, LG, Intel, Samsung - Concern: ZTE Alt2: PDSCH DMRS port(s) is QCLed with the DL RSs of both indicated joint TCI states with respect to QCL-TypeA except for QCL parameters {Doppler shift, Doppler spread} of the second indicated joint TCI state - Support: Huawei, ZTE, Ericsson, Xiaomi, Qualcomm, Docomo, CMCC, Samsung - Concern: Alt3: PDSCH DMRS port(s) is QCLed with the DL RS of the first indicated joint TCI state with respect to QCL-TypeA and QCLed with the DL RS of the second indicated joint TCI state with respect to QCL-TypeB - Support: ZTE - Concern: Huawei, QC, Samsung Proposal 5.1: On unified TCI framework extension for S-DCI based MTRP, the following two alternatives are supported for PDSCH-CJT applying both indicated joint TCI states (if the UE supports two indicated joint/DL states for PDSCH-CJT): - Alt1: PDSCH DMRS port(s) is QCLed with the DL RSs of both indicated joint TCI states with respect to QCL-TypeA - Alt2: PDSCH DMRS port(s) is QCLed with the DL RSs of both indicated joint TCI states with respect to QCL-TypeA except for QCL parameters {Doppler shift, Doppler spread} of the second indicated joint TCI state |
| 5.2 | Switching between CJT Tx scheme and other S-DCI based PDSCH Tx scheme(s) | Question 1: Whether to support dynamic switching between PDSCH-CJT and S-DCI based PDSCH SDM/FDM/TDM/SFN Tx schemes - Yes: ZTE - No: Xiaomi, QC, Google, Huawei, HiSilicon, Docomo, Lenovo, vivo, NEC, CATT OPPO, Fraunhofer, Intel, Samsung |

Table 5-2 Company input for Issue 5

| Company | Input |
| --- | --- |
| Mod V00 | - Please share your view, if any, on Proposal 5.1 |
| Huawei, HiSilicon | Proposal 5.1: Support Question 1: No. |
| ZTE | Proposal 5.1: Not support, we still have not identified the usage of Alt1. Could any companies provide any simulation results of proving the performance benefits of having that? That is, greater than Alt3? Question 1: One more question: can we conclude that dynamic switching between PDSCH-CJT and STRP should be supported. |

- Issue 6 – Beam failure recovery and beam reporting

Table 6-1 Summary for Issue 6

| # | Issue | Companies’ view and Recommended Proposal |
| --- | --- | --- |
| 6.1 | Implicit BFD-RS determination for S-DCI based MTRP | Proposal 6.1: On unified TCI framework extension for S-DCI based MTRP, if the UE is provided the first candidate beam RS list (q1,0) and the second candidate beam RS set (q1,1) but not explicitly provided the first BFD-RS set (q0,0) and the second BFD-RS set (q0,1) for TRP-specific BFR and if both first and second indicated joint/DL TCI states are configured by RRC to be applied to CORESETs for PDCCH reception, the UE determines the BFD-RS for the first and second BFD-RS sets from the first and second indicated joint/DL TCI states, respectively. - FFS: The case if any CORESET is configured to apply both first and second indicated joint/DL TCI states for PDCCH-SFN - FFS: Whether and how to handle the case if one or both of the first and second indicated joint/DL TCI states is/are NOT configured by RRC to be applied to CORESET(s) for PDCCH reception Current TS 38.213 for link recovery procedures If the UE is not provided q0,0 and q0,1 for a BWP of the serving cell, the UE determines the set q0,0 and q0,1 to include periodic CSI-RS resource configuration indexes with same values as the RS indexes in the RS sets indicated by TCI-State for first and second CORESETs that the UE uses for monitoring PDCCH, respectively, where the UE is provided two coresetPoolIndex values 0 and 1 for the first and second CORESETs, or is not provided coresetPoolIndex value for the first CORESETs and is provided coresetPoolIndex value of 1 for the second CORESETs, respectively. |

Table 6-2 Company input for Issue 6

| Company | Input |
| --- | --- |
| Mod V00 | - Please check the updated Proposal 6.1 |
| Xiaomi | Just repeat our comments in Round 0 to add the following FFS - FFS: If CORESET is not configured to follow unified TCI state, how to decide the RS from the TCI state(s) of such CORESET to be the BFD-RS of the first and/or second BFD-RS sets. |
| Huawei, HiSilicon | Proposal 6.1: Support. |

- Appendix: Agreements/conclusions before/in RAN1#112

RAN1#112

RAN1#111

Agreement On unified TCI framework extension for S-DCI based MTRP, in one beam indication instance, the existing TCI field in DCI format 1_1/1_2 (with or without DL assignment) can indicate joint/DL/UL TCI state(s) for one or both of the two TRPs in a CC/BWP or a set of CCs/BWPs in a CC list - FFS: Increase on the size of the TCI field - Note: The term TRP is used only for discussion purpose in RAN1 and whether/how to capture this is FFS Agreement On unified TCI framework extension for S-DCI based MTRP, a DCI field in DCI format 1_1/1_2 that schedules/activates PDSCH reception is used to determine which one or both of the indicated joint/DL TCI states shall be applied to the scheduled/activated PDSCH reception - The presence of the DCI field is configurable by RRC; when the DCI field is not present in DCI format 1_1/1_2, the UE shall apply the default indicated joint/DL TCI state(s) to PDSCH reception - FFS: Details on the default indicated joint/DL TCI state(s) to PDSCH reception - FFS: The DCI field is a new indicator field or an existing field (e.g., the existing TCI field) - FFS: Regardless the DCI field is present or not present, how to apply the indicated joint/DL TCI state(s) to PDSCH reception if the offset between the reception of the DCI format 1_1/1_2 and the corresponding PDSCH reception is less than a threshold FFS: How to apply the indicated joint/DL TCI state(s) to PDSCH reception scheduled/activated by DCI format 1_0. Above applies for the case where PDSCHs scheduled by the same DCI. Agreement On unified TCI framework extension for S-DCI based MTRP, use RRC configuration to inform that the UE shall apply the first one, the second one, or both of the indicated joint/UL TCI states to a PUCCH resource/group - Note: Detail of the RRC configuration is left to RAN2 design Agreement On unified TCI framework extension, PDSCH-CJT is supported as a S-DCI based MTRP scheme Note: Above does not preclude discussions specific to PDSCH-CJT design in the unified TCI framework Agreement On unified TCI framework extension for S-DCI based MTRP, use an indicator field (could be reusing an existing DCI field or introducing a new DCI field) in the DCI format 0_1/0_2 to inform which joint/UL TCI state(s) indicated by MAC-CE/DCI the UE shall apply to PUSCH transmission scheduled/activated by the DCI format 0_1/0_2 Agreement On unified TCI framework extension, down-select at least one of the following alternatives for PDSCH-CJT applying both indicated joint TCI states (if the UE supports two indicated joint/DL states for PDSCH-CJT): - Alt1: PDSCH DMRS port(s) is QCLed with the DL RSs of both indicated joint TCI states with respect to QCL-TypeA - Alt2: PDSCH DMRS port(s) is QCLed with the DL RSs of both indicated joint TCI states with respect to QCL-TypeA except for QCL parameters {Doppler shift, Doppler spread} of the second indicated joint TCI state - Alt3: PDSCH DMRS port(s) is QCLed with the DL RS of the first indicated joint TCI state with respect to QCL-TypeA and QCLed with the DL RS of the second indicated joint TCI state with respect to QCL-TypeB Agreement On unified TCI framework extension for M-DCI based MTRP, the same configuration/rule used in Rel-17 unified TCI framework (for determining whether the UE shall apply the indicated joint/DL TCI state to PDCCH on a CORESET and respective PDSCH) is reused to determine whether the UE shall apply the indicated joint/DL TCI state specific to a coresetPoolIndex value to PDCCH on a CORESET associated with the same coresetPoolIndex value and PDSCH scheduled/activated by the PDCCH. Agreement On unified TCI framework extension for M-DCI based MTRP, the UE shall apply the indicated joint/UL TCI state specific to a coresetPoolIndex value to PUSCH transmission scheduled/activated by PDCCH (including DG-PUSCH and Type2 CG-PUSCH) on a CORESET that is associated with the same coresetPoolIndex value Agreement On unified TCI framework extension for S-DCI based MTRP, a new indicator field is supported as the DCI field in DCI format 1_1/1_2 that schedules/activates PDSCH reception to determine which one or both of the indicated joint/DL TCI states shall be applied to the scheduled/activated PDSCH reception - FFS: Detail design of the new indicator field

RAN1#110b-e

Conclusion On unified TCI framework extension in Rel-18, there is no consensus to support simultaneous configuration of both joint and separate DL/UL TCI modes in a serving cell Conclusion On unified TCI framework extension in Rel-18, there is no consensus to support separate RRC-configured TCI state list(s) for each of TRPs Agreement On unified TCI framework extension for M-DCI based MTRP: - The existing TCI field in a DCI format 1_1/1_2 (with or without DL assignment) associated with one coresetPoolIndex value can indicate the joint/DL/UL TCI state(s) specific to the same coresetPoolIndex value - FFS: The UE shall apply the indicated joint/DL/UL TCI state(s) specific to a coresetPoolIndex value to channel(s)/signal(s) that have explicit or implicit association with the same coresetPoolIndex value - A coresetPoolIndex value field is included in TCI state activation command (MAC-CE) to indicate that the mapping between the activated TCI state(s) and the TCI codepoint(s) is specific to which coresetPoolIndex value Agreement On unified TCI framework extension for S-DCI based MTRP, to inform the association with the joint/DL TCI state(s) indicated by DCI/MAC-CE for PDCCH repetition, PDCCH-SFN, and PDCCH w/o repetition/SFN, support the following: - Use RRC configuration to inform that the UE shall apply the first one, the second one, both, or none of the joint/DL TCI states indicated by DCI/MAC-CE to a CORESET or a group of CORESETs (if CORESET group configuration is supported) Agreement On unified TCI framework extension for M-DCI based MTRP: - For a serving cell configured with joint DL/UL TCI mode, one joint TCI state can be mapped to a TCI codepoint of the existing TCI field in a DCI format 1_1/1_2 (with or without DL assignment) - For a serving cell configured with separate DL/UL TCI mode, a DL TCI state, an UL TCI state, or a pair of DL and UL TCI states can be mapped to a TCI codepoint of the existing TCI field in a DCI format 1_1/1_2 (with or without DL assignment) Agreement On unified TCI framework extension for S-DCI based MTRP, down-select one alternative from the followings in RAN1#111 for PUSCH transmission scheduled/activated by a DCI format 0_1/0_2: - Alt1: Use an indicator field (could be reusing an existing DCI field or introducing a new DCI field) in the DCI format 0_1/0_2 to inform which joint/UL TCI state(s) indicated by MAC-CE/DCI the UE shall apply to PUSCH transmission scheduled/activated by the DCI format 0_1/0_2 - Alt2: PUSCH transmission scheduled/activated by the DCI format 0_1/0_2 follows the spatial domain transmission filter(s) used for the SRS resource(s) indicated by the DCI format 0_1/0_2 - FFS: PL-RS(s), and UL PC parameter setting(s) (including P0, alpha, and closed loop index) for the PUSCH Agreement On unified TCI framework extension for S-DCI based MTRP, down-select one alternative from the followings in RAN1#111 for PUCCH transmission: - Alt1: Use RRC configuration to inform the association between the indicated joint/UL TCI state(s) and a PUCCH resource/ group - Alt2: Use RRC configuration to inform the association between a CORESET group and a PUCCH resource/group, and the indicated joint/UL TCI state(s) associated with the CORESET group applies to the PUCCH resource/group associated with the same CORESET group - Alt3: Use MAC-CE to inform the association between the indicated joint/UL TCI state(s) and a PUCCH resource/group - Note: the association indicates whether the UE shall apply the first one, the second one, or both of the joint/UL TCI states indicated by DCI/MAC-CE to a PUCCH resource/group Agreement On unified TCI framework extension, up to 2 joint TCI states can be indicated by MAC-CE/DCI and applied to CJT-based PDSCH reception (PDSCH-CJT) in a BWP/CC configured with joint DL/UL TCI mode - Support of 1 or 2 indicated joint TCI states for PDSCH-CJT is up to UE capability - FFS: QCL type(s)/assumption(s) of the indicated joint TCI state(s) applied to PDSCH-CJT - Note: On how to inform UE to apply which indicated joint TCI state(s) to target channel(s)/signal(s) in the BWP/CC, it is discussed individually in AI 9.1.1.1 Agreement On unified TCI framework extension for M-DCI based MTRP: - The UE shall apply the indicated joint/DL TCI state specific to a coresetPoolIndex value to PDCCH on a CORESET that is associated with the same coresetPoolIndex value - The UE shall apply the indicated joint/DL TCI state specific to a coresetPoolIndex value to PDSCH scheduled/activated by PDCCH on a CORESET that is associated with the same coresetPoolIndex value - FFS: Other channel(s)/signal(s) that has explicit or implicit association with a coresetPoolIndex value - FFS: Other channel(s)/signal(s) that doesn’t have association with a coresetPoolIndex value Above are applicable to the CORESET(s) that is configured/allowed to follow the indicated joint/DL TCI state FFS: The configuration/rule to configure/allow CORESET(s) to follow the indicated joint/DL TCI state, including the option to reuse the same configuration/rule as in Rel-17 unified TCI framework Agreement On unified TCI framework extension, study the following enhancements for TRP-specific BFR: - Implicit BFD-RS determination based on the indicated joint/DL TCI states for S-DCI based MTRP - Enhancement to beam update after NW response to TRP-specific BFR request Agreement On unified TCI framework extension for S-DCI based MTRP, down-select one alternative from the followings in RAN1#111: - Alt1: In one beam indication instance, the existing TCI field in DCI format 1_1/1_2 (with or without DL assignment) can indicate joint/DL /UL TCI state(s) for one of the two TRPs or both TRPs in a CC/BWP or a set of CCs/BWPs in a CC list - Alt2: In one beam indication instance, the existing TCI field in DCI format 1_1/1_2 (with or without DL assignment) can indicate joint/DL /UL TCI state(s) only specific to one of the two TRPs in a CC/BWP or a set of CCs/BWPs in a CC list - Note: According to the agreement in RAN1#109-e, support of one additional TCI field or a field associating the TCI field to the TRP(s) is not precluded Note: It has been agreed to use the existing TCI field for TCI state indication for S-DCI based MTRP in RAN1#109e Note: The term TRP is used only for discussion purpose in RAN1 and whether/how to capture this is FFS FFS: The behavior if the UE receives a beam indication DCI that indicates joint/DL/UL TCI state(s) for one TRP

RAN1#110

Agreement On unified TCI framework extension, for the target use cases agreed in RAN1#109-e in AI 9.1.1.1, up to 4 TCI states can be indicated in a CC/BWP or a set of CCs/BWPs in a CC list to DL receptions and/or UL transmissions, where these TCI states are indicated/updated by MAC-CE/DCI with the necessary MAC-CE based TCI state activation - FFS: The possible combination(s) of joint/DL/UL TCI states that can be indicated to DL receptions and/or UL transmissions in a BWP/CC/TRP - Note: This agreement does not imply that there will be more than 2 DL or UL or joint TCI states indicated in a CC/BWP for the target use cases agreed in RAN1#109-e in AI 9.1.1.1 - Note: The maximum number of TCI states that can be indicated to each of the target use cases agreed in RAN1#109-e in AI 9.1.1.1 is remained the same as in Rel-16/17 Note: The maximum number of TCI states that can be indicated simultaneously to CJT-based PDSCH reception and the required type(s) of TCI states (i.e., DL /UL/joint) are independently discussed in this AI Agreement On unified TCI framework extension for S-DCI based MTRP, to inform the association with the joint/DL TCI state(s) indicated by DCI/MAC-CE for PDCCH repetition, PDCCH-SFN, and PDCCH w/o repetition/SFN, down-selection at least one alternative from the followings: - Alt1-1: Use RRC parameter(s) in a CORESET configuration to inform the UE whether and/or which indicated joint/DL TCI state(s) shall be applied to the corresponding PDCCH receptions on the CORESET - FFS: Whether only the CORESET(s) that always/can share the unified TCI state as defined in Rel-17 unified TCI framework can be associated with the joint/DL TCI state(s) indicated by DCI/MAC-CE - Alt1-2: Use an RRC parameter in a CORESET configuration to inform that the CORESET belongs to which CORESET group(s), and the indicated joint/DL TCI state(s) is associated with each CORESET group - FFS: Whether only the CORESET(s) that always/can share the unified TCI state as defined in Rel-17 unified TCI framework can be associated with the CORESET group(s) - FFS: How to associate the indicated joint/DL TCI state(s) with each CORESET group - FFS: The UE applies the indicated joint/DL TCI state(s) to a CORESET according to the CORESET group(s) the CORESET belongs to, or the UE applies the indicated joint/DL TCI state(s) associated with the CORESET group(s) in which the beam indication DCI is received to all PDCCH receptions - Alt2: The association between a CORESET and the indicated joint/DL TCI state(s) is determined based on a fixed rule, and the UE shall apply the indicated joint/DL TCI state(s) to the corresponding PDCCH receptions on the CORESET - FFS: Whether only the CORESET(s) that always/can share the unified TCI state as defined in Rel-17 unified TCI framework can be associated with the joint/DL TCI state(s) indicated by DCI/MAC-CE - Alt3: Use MAC-CE to inform the UE whether and/or which indicated joint/DL TCI state(s) shall be applied to the corresponding PDCCH receptions on a CORESET - FFS: Whether only the CORESET(s) that always/can share the unified TCI state as defined in Rel-17 unified TCI framework can be associated with the joint/DL TCI state(s) indicated by DCI/MAC-CE Switching between multi-TRP and single TRP operation is not precluded Agreement On unified TCI framework extension for S-DCI based MTRP, for PUSCH transmission scheduled/activated by a DCI format 0_1/0_2, down-selection one alternative from the followings: - Alt1: Use an indicator field (could be reusing an existing DCI field or introducing a new DCI field) in a DCI format 0_1/0_2 to inform which joint/UL TCI state(s) indicated by MAC-CE/DCI the UE shall apply to PUSCH transmission scheduled/activated by the DCI format 0_1/0_2 - Alt2: PUSCH transmission scheduled/activated by a DCI format 0_1/0_2 follows the spatial domain transmission filter(s) used for the SRS resource(s) indicated by the DCI format 0_1/0_2 - Alt3: Use an RRC parameter in a CORESET configuration to inform that the CORESET belongs to which CORESET group(s), and the indicated joint/UL TCI state(s) is associated with each CORESET group. When a scheduling/activation DCI format 0_1/0_2 is received in a CORESET group, the indicated joint/UL TCI state(s) associated with the CORESET group is applied to PUSCH transmission scheduled/activated by the DCI format 0_1/0_2 - FFS: Details of CORESET group(s) FFS: PUSCH transmission scheduled/activated by a DCI format 0_0 and Type-1 CG-PUSCH Agreement On unified TCI framework extension for S-DCI based MTRP, to inform the association with joint/UL TCI state(s) indicated by DCI/MAC-CE for PUCCH transmission, down-selection at least one alternative from the followings: - Alt1: Use RRC configuration to inform the association between the indicated joint/UL TCI state(s) and a PUCCH resource/ group - Alt2: Use RRC configuration to inform the association between a CORESET group and a PUCCH resource/group, and the indicated joint/UL TCI state(s) associated with the CORESET group applies to the PUCCH resource/group - Alt3: Use MAC-CE to inform the association between the indicated joint/UL TCI state(s) and a PUCCH resource/group - Alt4: Use DCI to inform the association between the indicated joint/UL TCI state(s) and a PUCCH resource/group

RAN1#109e

Agreement On unified TCI framework extension, consider all the intra and inter-cell MTRP schemes specified in Rel-16 and Rel-17 - Consider, if STxMP is supported, Rel-18 MTRP scheme(s) with STxMP Agreement On unified TCI framework extension at least for single-DCI based MTRP, the existing TCI field in DCI format 1_1/1_2 (with or without DL assignment) can indicate multiple joint/DL/UL TCI states in a CC/BWP or a set of CCs/BWPs in a CC list - FFS: Detail of mapping joint/DL/UL TCI state ID(s) to a TCI codepoint, e.g., possible combinations of joint, DL, and/or UL TCI state IDs that can be mapped to a TCI codepoint - FFS: Whether to increase the max number of MAC CE activated TCI codepoints, i.e., more than 8 codepoints - FFS: Whether to increase the max number of TCI field bits, i.e., more than 3 bits - Note: This doesn't imply that support of one additional TCI field or a field associating the TCI field to the TRP(s) is precluded Note: The term TRP is used only for the purposes of discussions in RAN1 and whether/how to capture this is FFS Agreement On unified TCI framework extension for M-DCI based MTRP, consider the following alternatives for TCI state update: - Alt1: Reuse the same TCI state update scheme for S-DCI based MTRP - Atl2: Use the existing TCI field in the DCI format 1_1/1_2 (with or without DL assignment) associated with one of CORESETPoolIndex values to indicate the joint/DL/UL TCI state(s) corresponding to the same CORESETPoolIndex value - Alt3: Use the existing TCI field in any DCI format 1_1/1_2 (with or without DL assignment) to indicate all joint/DL/UL TCI states corresponding to both CORESETPoolIndex values - Study the association between the indicated joint/DL/UL TCI state(s) and a CORESETPoolIndex value - Alt4: Use the existing TCI field in the DCI format 1_1/1_2 (with or without DL assignment) associated with one of CORESETPoolIndex values to indicate joint/DL/UL TCI state(s) corresponding to the same or different CORESETPoolIndex value. - Study whether the indicated joint/DL/UL TCI state(s) applies to the channels/signals associated with the same CORESETPoolIndex value or different CORESETPoolIndex value is indicated by DCI Agreement On unified TCI framework extension for S-DCI based MTRP, consider at least the following alternatives to map/associate a joint/DL TCI state to PDCCH reception(s) - Atl1: Use RRC configuration to inform the mapping/association between a configured or indicated joint/DL TCI state and a CORESET or a CORESET group - Alt2: Use RRC configuration to inform the mapping/association between a configured or indicated joint/DL TCI state and a search space set - Alt3: Use MAC-CE to inform the mapping/association between an activated or indicated joint/DL TCI state and a CORESET or a CORESET group - Alt4: Use DCI to inform the mapping/association between an indicated joint/DL TCI state and a CORESET or a CORESET group - Alt5: Based on a fixed mapping/association rule, e.g., the first indicated joint/DL TCI state always applies to PDCCH receptions Consider above alternatives for PDCCH repetition, PDCCH-SFN, PDCCH w/o repetition/SFN, and potential support of dynamic switching between S-TRP and M-TRP for PDCCH. It is not precluded to adopt one single alternative or multiple alternatives to support these cases. Agreement On unified TCI framework extension, if an indicated joint or UL TCI state applies to a PUSCH/PUCCH transmission occasion at least for S-DCI based PUSCH/PUCCH repetition with TDM and the indicated joint or UL TCI state is associated with an UL PC parameter setting for PUSCH/PUCCH (including P0, alpha for PUSCH, and closed loop index) and a PL-RS, the UE should apply the UL PC parameter setting and the PL-RS for the PUSCH /PUCCH transmission occasion. - FFS: How to extend to other Rel-18 MTRP scheme(s) with STxMP, if supported - FFS: UL PC enhancement for CB and non-CB SRS in above case FFS: The applied UL PC parameter setting if one or both indicated joint or UL TCI state(s) is not associated with an UL PC parameter setting (including P0, alpha for PUSCH, and closed loop index) for PUCCH/PUSCH Agreement On UE power limitation for STxMP for FR2, send LS to RAN4 to check the followings: - Whether it is feasible to assume power limitation per panel for STxMP (Assumption 1) - Whether it is feasible to assume a total power limitation per UE over all UE panels used for STxMP (Assumption 2) - In either of Assumption1 or Assumption 2, whether the total power limitation per UE over all UE panels used for STxMP or the sum of per-panel power limitation for STxMP can be different from (greater than) the existing power limitation for a given power class? - If both Assumption 1 and Assumption 2 are feasible, whether both assumptions can be applied to a same UE, and what is the relationship between the per-panel power limitation and total power limitation if both are applied (e.g., the sum of per-panel power limitation can be larger than the total power limitation per UE, or should be always the same)? FFS: Detail of exact LS if agreed Note: Scenarios of above include at least single carrier scenario for FR2 Note: Above power limitation includes both total radiated power and EIRP LS to RAN4 is endorsed in R1-2205639.

- References

| 1 | R1-2300600 | Discussion on unified TCI framework extension for multi-TRP operation | TCL Communication Ltd. |
| --- | --- | --- | --- |
| 2 | R1-2300740 | Discussion on unified TCI framework extension for MTRP | Fujitsu |
| 3 | R1-2300650 | Further discussions on unified TCI framework extension for multi-TRP operation | CATT |
| 4 | R1-2300436 | Further discussion on unified TCI framework extension for multi-TRP | vivo |
| 5 | R1-2300333 | Unified TCI framework extension for multi-TRP | Ericsson |
| 6 | R1-2300327 | Unified TCI framework extension for multi-TRP | Panasonic |
| 7 | R1-2300510 | Discussion of unified TCI framework for multi-TRP | Lenovo |
| 8 | R1-2300522 | Unified TCI framework extension for multi-TRP/panel | LG Electronics |
| 9 | R1-2300545 | Unified TCI framework extension for multi-TRP | xiaomi |
| 10 | R1-2300203 | Discussion on unified TCI framework extension for multi-TRP | Spreadtrum |
| 11 | R1-2300248 | Unified TCI framework extension for multi-TRP | OPPO |
| 12 | R1-2300181 | Enhancements on unified TCI framework extension for multi-TRP | ZTE |
| 13 | R1-2300156 | Details on Unified TCI Extension for MTRP | InterDigital, Inc. |
| 14 | R1-2300093 | Discussion on unified TCI framework extension for multi-TRP | Huawei, HiSilicon |
| 15 | R1-2300048 | Unified TCI framework extension for multi-TRP | FUTUREWEI |
| 16 | R1-2301165 | Discussion on unified TCI framework extension for multi-TRP | Google |
| 17 | R1-2300503 | Multi-TRP enhancements for the unified TCI framework | Fraunhofer |
| 18 | R1-2300487 | Discussion on unified TCI framework extension for multi-TRP | FGI |
| 19 | R1-2301245 | Views on unified TCI extension focusing on m-TRP | Samsung |
| 20 | R1-2300820 | Discussion on unified TCI framework extension for multi-TRP | NEC |
| 21 | R1-2300982 | Discussion on unified TCI framework extension for multi-TRP | CMCC |
| 22 | R1-2300899 | Discussion on unified TCI framework extension for multi-TRP | Hyundai Motor Company |
| 23 | R1-2300847 | Unified TCI framework extension for multi-TRP | Sharp |
| 24 | R1-2300865 | Discussion on unified TCI framework extension for multi-TRP | Sony |
| 25 | R1-2300932 | Unified TCI Framework for Multi-TRP | Intel Corporation |
| 26 | R1-2301318 | Discussion on unified TCI framework extension for multi-TRP | ITRI |
| 27 | R1-2301329 | Unified TCI framework Extension for mTRP | Apple |
| 28 | R1-2301395 | Extension of unified TCI framework for mTRP | Qualcomm Incorporated |
| 29 | R1-2301477 | Discussion on unified TCI framework extension for multi-TRP | NTT DOCOMO, INC. |
| 30 | R1-2301461 | Discussion on sDCI mTRP PDSCH regarding eUTCI framework | ASUSTeK |
| 31 | R1-2301580 | Unified TCI framework extension for multi-TRP | MediaTek Inc. |
| 32 | R1-2301686 | Discussion on Unified TCI framework extension for multi-TRP | CEWiT |
| 33 | R1-2301642 | Unified TCI framework extension for multi-TRP | Nokia, Nokia Shanghai Bell |