3GPP TSG-RAN WG1 Meeting #112	R1-2302181

Athens, Greece, February 27th - March 3rd, 2023

Revision of R1-2302018

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.213 | CR | 1435 | rev | 1 | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on corrections to eMTC support for IoT NTN in TDD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator (Sony), OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN-Core |  |  |  |  |  | Date: |  |  | 2023-02-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As TDD was not treated during the Rel17 IoT NTN WI, TDD clauses in the spec should not be changed because of NTN. All references to Koffset should be removed from TDD clauses. This CR removes some newly identified references to Koffset from the TDD clauses for this reason. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clause 7.3.2.1, 8.0, 10.1.3, 10.1.3.1: Remove all references to Koffset from the TDD clauses. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Rel-17 IoT NTN does not support TDD so clauses would wrongly imply that it does and potentially confuse implementers. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.2.1, 8.0, 10.1.3, 10.1.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 7.3.2.1	TDD HARQ-ACK reporting procedure for same UL/DL configuration

<Unchanged parts are omitted>

For TDD and a BL/CE UE,

-	if the UE is configured with ce-PDSCH-MultiTB-Config, and multiple TBs are scheduled by a single DCI

-	the UE is not expected to receive any other PDSCH transmission(s) or MPDCCH indicating downlink SPS release, corresponding to which the UE shall report HARQ-ACK in any subframe(s) in which HARQ-ACKs are reported for the multiple TBs scheduled by the single DCI, according to clause 10.2

-	the UE behaviour for HARQ-ACK reporting is the same as that of a BL/CE UE with FDD, except:

-	PUCCH resource(s) is (are) determined according to Clause 10.1.3.1; and

-	PUCCH(s) is (are) transmitted in a set of BL/CE UL subframe(s) according to Clause 10.2 for TDD and BL/CE UEs;

-	else if the UE is configured with csi-NumRepetitionCE equal to 1 and mPDCCH-NumRepetition equal to 1,

-	the UE behaviour for HARQ-ACK reporting is the same as that of a non-BL/CE UE with TDD, except:

-	PDCCH/EPDCCH is replaced by MPDCCH; and

-	DCI format 1/1A/1B/1D/2/2A/2B/2C/2D is replaced by DCI format 6-1A; and

-	DCI format 0/4 is replaced by DCI format 6-0A; and

-	PUCCH is transmitted in a set of BL/CE UL subframe(s) according to Clause 10.2 for TDD and BL/CE UEs;

-	else

-	the UE is not expected to receive more than one PDSCH transmission, or more than one of PDSCH and MPDCCH indicating downlink SPS releases, with transmission ending within subframe(s) n-k-Koffset where  and  is defined in Table 10.1.3.1-1 intended for the UE;

-	The UE behavior for HARQ-ACK reporting is the same as that of a BL/CE UE with FDD, except:

-	PUCCH resource is determined according to Clause 10.1.3.1; and

-	PUCCH is transmitted in a set of BL/CE UL subframe(s) according to Clause 10.2 for TDD and BL/CE UEs.

If the BL/CE UE is configured in CEModeA, and if the PDSCH is assigned by or semi-statically scheduled by a MPDCCH with DCI format 6-1A, the UE shall assume no PDSCH repetition if the higher layer parameter csi-NumRepetitionCE-r13 indicates one subframe.

<Unchanged parts are omitted>

## 8.0	UE procedure for transmitting the physical uplink shared channel

<Unchanged parts are omitted>

For TDD and a BL/CE UE configured with CEModeA, if an MPDCCH with CSI request field set to trigger an aperiodic CSI report, as described in Clause 7.2.1, is detected by a UE on subframe n, then on subframe n+k UCI is mapped on the corresponding PUSCH transmission, including all subframe repetitions of the PUSCH transmission, where k is given by Table 8-2.

<Unchanged parts are omitted>

### 10.1.3	TDD HARQ-ACK feedback procedures

For TDD and a UE that does not support aggregating more than one serving cell with frame structure type 2, two HARQ-ACK feedback modes are supported by higher layer configuration.

- HARQ-ACK bundling and

- HARQ-ACK multiplexing

For TDD and a BL/CE UE, if multiple TBs are not scheduled by a single DCI,

-	if the UE is configured with csi-NumRepetitionCE equal to 1 and mPDCCH-NumRepetition equal to 1,

-	the UE may be configured with HARQ-ACK bundling or HARQ-ACK multiplexing;

-	HARQ-ACK multiplexing can be configured only if pucch-NumRepetitionCE-format1 equal 1 and HARQ-ACK multiplexing is performed according to the set of Tables 10.1.3-5/6/7

-	else

-	the UE is not expected to receive more than one PDSCH transmission, or more than one of PDSCH and MPDCCH indicating downlink SPS releases, with transmission ending within subframe(s) n-k-Koffset, where  and  is defined in Table 10.1.3.1-1 intended for the UE;

<Unchanged parts are omitted>

#### 10.1.3.1	TDD HARQ-ACK procedure for one configured serving cell

<Unchanged parts are omitted>

-	For a BL/CE UE, if there is only a PDSCH transmission within one or more consecutive BL/CE downlink subframe(s) where subframe n-k-Koffset, is the last subframe in which the PDSCH is transmitted where and  is defined in Table 10.1.3.1-1 and there is not a corresponding MPDCCH, the UE shall use PUCCH format 1a and PUCCH resource  where the value of is determined according to higher layer configuration and Table 9.2-2.

-	If there is PDSCH transmission indicated by the detection of corresponding MPDCCH or there is MPDCCH indicating downlink SPS release within subframe(s) n-k-Koffset, where  and  (defined in Table 10.1.3.1-1) is a set of M elements  depending on the subframe n and the UL/DL configuration (defined in Table 4.2-2 in [3]) and subframe n-km-Koffset is the last subframe in which the PDSCH or MPDCCH indicating downlink SPS release is transmitted and there is no  where  and subframe n-km-Koffset is the last subframe in which a PDSCH indicated by the detection of corresponding MPDCCH or MPDCCH indicating downlink SPS release is transmitted, the UE shall use

-	if MPDCCH-PRB-set is configured for distributed transmission

-	if MPDCCH-PRB-set is configured for localized transmission

for antenna port , where  is the number of the first ECCE (i.e. lowest ECCE index used to construct the MPDCCH) used for transmission of the corresponding DCI assignment in MPDCCH-PRB-set ,  for MPDCCH-PRB-set  is configured

-	by the higher layer parameter n1PUCCH-AN-r13, if configured; otherwise:

-	by the higher layer parameter n1PUCCH-AN-InfoList-r13 for the corresponding CE level,

for MPDCCH-PRB-set  is given in Clause 6.8A.1 in [3], is determined from the antenna port used for the MPDCCH transmission which is described in Clause 6.8A.5 in [3]. If ,  is determined from the HARQ-ACK resource offset field in the DCI format of the corresponding MPDCCH as given in Table 10.1.2.1-1. If ,  is determined from the HARQ-ACK resource offset field in the DCI format of the corresponding MPDCCH as given in Table 10.1.3.1-2. If subframe n-ki1-Koffset is a BL/CE downlink subframe,  is equal to the number of ECCEs in MPDCCH-PRB-set configured for that UE in subframe n-ki1-Koffset. If subframe n-ki1-Koffset is not a BL/CE downlink subframe,  is equal to 0. If subframe n-ki1-Koffset is a BL/CE downlink special subframe in which MPDCCH is not supported, the UE shall calculate  by assuming for normal cyclic prefix and for extended cyclic prefix. If an MPDCCH-PRB-set is 2+4, then . When an MPDCCH-PRB-set is 2+4, following procedures is applied.

-	if the detected MPDCCH is located within 2 PRB set, is obtained by above procedure.

-	if the detected MPDCCH is located within 4 PRB set, is the sum between and the value obtained by above procedure.

-	if the detected MPDCCH is MPDCCH format 5, is obtained by the above procedure with  = 0.

<Unchanged parts are omitted>

-	For a BL/CE UE, for a PDSCH transmission detected in subframe n-ki-Koffset without a corresponding MPDCCH, the value of  is determined according to higher layer configuration and Table 9.2-2.

-	For a BL/CE UE, for a PDSCH transmission in sub-frame n-ki-Koffset where  indicated by the detection of corresponding MPDCCH or a MPDCCH indicating downlink SPS release in sub-frame n-ki-Koffset where , the UE shall use

-	if MPDCCH-PRB-set is configured for distributed transmission

-	if MPDCCH-PRB-set is configured for localized transmission

where  is the number of the first ECCE (i.e. lowest ECCE index used to construct the MPDCCH) used for transmission of the corresponding DCI assignment in MPDCCH-PRB-set ,  for MPDCCH-PRB-set  is configured

-	by the higher layer parameter n1PUCCH-AN-r13, if configured; otherwise:

-	by the higher layer parameter n1PUCCH-AN-InfoList-r13 for the corresponding CE level,

for MPDCCH-PRB-set  is given in Clause 6.8A.1 in [3], is determined from the antenna port used for the MPDCCH transmission which is described in Clause 6.8A.5 in [3]. If ,  is determined from the HARQ-ACK resource offset field in the DCI format of the corresponding MPDCCH as given in Table 10.1.2.1-1. If ,  is determined from the HARQ-ACK resource offset field in the DCI format of the corresponding MPDCCH as given in Table 10.1.3.1-2, where the variable in the table is substituted with . If subframe n-ki1-Koffset is a BL/CE downlink subframe,  is equal to the number of ECCEs in MPDCCH-PRB-set configured for that UE in subframe n-ki1-Koffset. If subframe n-ki1-Koffset is not a BL/CE downlink subframe,  is equal to 0. If subframe n-ki1-Koffset is a BL/CE downlink special subframe in which MPDCCH is not supported, the UE shall calculate  by assuming for normal cyclic prefix and for extended cyclic prefix. If an MPDCCH-PRB-set is 2+4, then . When an MPDCCH-PRB-set is 2+4, following procedures is applied.

-	if the detected MPDCCH is located within 2 PRB set, is obtained by above procedure.

-	if the detected MPDCCH is located within 4 PRB set, is the sum between and the value obtained by above procedure.

-	if the detected MPDCCH is MPDCCH format 5, is obtained by the above procedure with  = 0.

<Unchanged parts are omitted>