3GPP TSG-RAN WG1 Meeting #112be     	R1-2302397

Online, April 17 – April 26, 2023

Source:	Panasonic

Title: 	UL Precoding for Multi-panel Transmission

Agenda Item:	9.1.4.1

Document for:	Discussion

# Introduction

During RAN#94e, a new WID for Rel-18 MIMO evolution for DL and UL was agreed [1]. This contribution discusses the following objective:

- Study, and if needed, specify the following items to facilitate simultaneous multi-panel UL transmission for higher UL throughput/reliability, focusing on FR2 and multi-TRP, assuming up to 2 TRPs and up to 2 panels, targeting CPE/FWA/vehicle/industrial devices (if applicable) - UL precoding indication for PUSCH, where no new codebook is introduced for multi-panel simultaneous transmission - The total number of layers is up to four across all panels and total number of codewords is up to two across all panels, considering single DCI and multi-DCI based multi-TRP operation. - UL beam indication for PUCCH/PUSCH, where unified TCI framework extension in objective 2 is assumed, considering single DCI and multi-DCI based multi-TRP operation For the case of multi-DCI based multi-TRP operation, only PUSCH+PUSCH, or PUCCH+PUCCH is transmitted across two panels in a same CC.

# Discussion

The WID targets CPE/FWA/vehicle/industrial devices which have higher transmission power levels than handheld UE devices. RAN4 spec defines the maximum output power for various UE types in FR2, but RAN1 should further specify technical assumptions regarding the targeted devices (CPE/FWA/vehicle/industrial), for example, power class definition, panel switching latency among a pool of panels, antenna ports coherency between panels, and total UE power constraint etc.

Table 1: 38.101-2 assumption of UE Types (Table 6.2.1.0-1)

| UE Power class | UE type |
| --- | --- |
| 1 | Fixed wireless access (FWA) UE |
| 2 | Vehicular UE |
| 3 | Handheld UE |
| 4 | High power non-handheld UE |
| 5 | Fixed wireless access (FWA) UE |
| 6 | High Speed Train Roof-Mounted UE |
| 7 | RedCap UE |

Moreover, the gNB should be able to determine whether two beams from different UE panels can be transmitted simultaneously from the UE. One way to do this is to use group-based beam reporting by UE similar to what is done on the DL, but additionally reporting the number of SRS ports or rank associated with that beam. Also note that the issue of compatibility of two beams being simultaneously transmitted is manageable when one node is doing the scheduling. This issue becomes more pronounced in multi-DCI STxMP where perhaps only semi-static coordination exists between the schedulers of the two TRPs.

Study enhancements to Rel-17 group-based beam reporting and UE capability index reporting to inform gNB whether the UE is able to transmit simultaneously from two panels, when the panels may have same maximum number of SRS ports.

## Single-DCI based STxMP SDM PUSCH Scheme

RAN1 had the following agreement:

Working Assumption (RAN1#112)

For dynamic switching between STxMP SDM scheme and sTRP transmission, support the following:

- For sTRP transmission: The maximal number of layers of sTRP transmission is configured by the maxRank (or Lmax) as in current spec (i.e., Option 1)

- For SDM scheme: configure one single maximal number of layers (separate from maxRank (or Lmax) for sTRP) that is applied to the first SRS resource set and the second SRS resource set, separately (i.e., Alt1)

- FFS: Whether/How to enable that the total number of used PUSCH antenna ports for the SDM and sTRP is the same.

  - Note: This corresponds to the case that digital ports are shared between the panels

Note: RAN1 supports both implementations that digital ports are shared or separate among panels

The underlying assumption here is that RAN1 will only support panels with symmetric capabilities. This simplifies the STxMP design. In general, the association of a beam with a panel is not fixed. For example, if the UE is equipped with 4 panels with different capabilities, and 2 panels are used at a time for STxMP, the beam-panel capability association might change and perhaps RRC reconfiguration would be needed. This is not the case with symmetric capabilities. This simplification has an implication on other issues being discussed.

First the following was agreed in RAN1 #110-bis-e:

Agreement (RAN1#110bis-e)

For SDM scheme of single-DCI based STxMP PUSCH

- Configure two SRS resource sets for CB or NCB.

  - FFS: These two SRS resource sets can have different number of SRS resources for codebook -based or non-codebook based.

- For codebook-based PUSCH, DCI indicates two TPMI fields, and each TPMI field separately indicates the precoding information and the number of layers conveyed over the SRS ports of the indicated SRS resource in each SRS resource set.

- For non-codebook based PUSCH and codebook-based PUSCH, DCI indicates two SRI fields, and each field indicates SRS resource(s) for each SRS resource set separately.

  - FFS: For codebook-based PUSCH, the two SRS resources indicated by the two SRI fields can have different number of SRS ports

Regarding the last FFS above, for codebook-based PUSCH, since only UE panels with same capabilities are supported, the number of SRS ports in SRS resources belonging to different SRS resource sets is the same. The same argument applies to the first FFS, for both codebook-based and non-codebook-based PUSCH. These two SRS resource sets can have the same number of SRS resources.

On the two SRS resource sets configured for single-DCI based STxMP SDM scheme:

- For CB/NCB PUSCH, the number of SRS resources in these two SRS resource sets is the same.

- For CB PUSCH, the two SRS resources indicated by the DCI for SDM scheme have same number of ports.

Second, RAN1 needs to discuss how to interpret each codepoint of “SRS resource set indicator” field. Also, it needs to discuss how to interpret the TPMI field for CB PUSCH or SRI field for NCB PUSCH when dynamically switching between SDM STxMP and single TRP transmission.

In R17 single DCI mTRP transmission for PUSCH, SRS resource set indicator can be configured in DCI format 0_1/0_2 as follows to dynamically indicate sTRP or mTRP operation for the scheduled/activated PUSCH.

Table 2: SRS resource set indicator for release 17 sDCI mTRP PUSCH repetition scheme

| Codepoint | SRS resource set(s) | SRI (for both CB and NCB)/TPMI (CB only) field(s) |
| --- | --- | --- |
| 00 | s-TRP mode with 1st SRS resource set (TRP1) | 1st SRI/TPMI field (2nd field is unused) |
| 01 | s-TRP mode with 2nd SRS resource set (TRP2) | 1st SRI/TPMI field (2nd field is unused) |
| 10 | m-TRP mode with (TRP1,TRP2 order) 1st SRI/TPMI field: 1st SRS resource set 2nd SRI/TPMI field: 2nd SRS resource set | Both 1st and 2nd SRI/TPMI fields |
| 11 | m-TRP mode with (TRP2,TRP1 order) 1st SRI/TPMI field: 1st SRS resource set 2nd SRI/TPMI field: 2nd SRS resource set | Both 1st and 2nd SRI/TPMI fields |

The indices ‘10’ and ’11’ of SRS resource set indicator field are used for TRP order switching. However, for STxMP operation, PUSCH is simultaneously transmitted over the SRS ports associated with the SRS resource sets and, hence, panel order switching is not needed for STxMP SDM scheme. Therefore, one codepoint is enough to indicate SDM STxMP, and bit field ‘11’ of SRS resource set indicator field can be reserved. See table 3 below.

Table 3: Possible interpretation of SRS resource set indicator for release 17 sDCI mTRP PUSCH repetition scheme

| Codepoint | SRS resource set(s) | SRI (for both CB and NCB)/TPMI (CB only) field(s) |
| --- | --- | --- |
| 00 | s-TRP mode with 1st SRS resource set | - 1st SRI/TPMI field (2nd field is unused) or - Both 1st and 2nd SRI/TPMI fields are used for indication of one SRI/TPMI |
| 01 | s-TRP mode with 2nd SRS resource set | - 1st SRI/TPMI field (2nd field is unused) or - Both 1st and 2nd SRI/TPMI fields are used for indication of one SRI/TPMI |
| 10 | STxMP SDM scheme | Both 1st and 2nd SRI/TPMI fields |
| 11 | Reserved |  |

The codepoints of “SRS resource set indicator” in DCI for dynamic switching between STxMP SDM and sTRP transmission are interpreted as follows:

  - SRS resource set indicator codepoints 00 and 01 indicate sTRP transmission with 1st SRS resource set and 2nd SRS resource set respectively.

  - SRS resource set indicator codepoint 10 indicates SDM with 1st SRS resource set and 2nd SRS resource set. The 1st TPMI or SRI field and 2nd TPMI or SRI field indicates the precoding/rank/SRI for the 1st SRS resource set and 2nd SRS resource set for CB or NCB respectively.

  - The codepoint 11 of SRS resource set indicator is reserved/unused.

For codepoints 00, and 01, it is necessary to determine how to interpret the TPMI field for CB PUSCH or SRI field for NCB PUSCH when dynamically switching between SDM STxMP and single TRP transmission. We have two valid implementations for STxMP. In Case 1, the number of available digital ports is per panel, and therefore the maximum number of PUSCH layers associated with one SRS resource set is the same irrespective of whether the PUSCH is associated with one SRS resource set (sTRP) or two SRS resource sets (STxMP). In Case 2, the digital ports are shared across multiple panels, and therefore, the maximum number of PUSCH layers associated with one SRS resource set depends on whether the PUSCH is associated with one SRS resource set (sTRP) or two SRS resource sets (STxMP). We think that both cases should be supported. Therefore, we support the following interpretation for TPMI/SRI fields.

For SDM, the size of the 1st TPMI or SRI fields and the 2nd TPMI or SRI fields are determined by the maximal number of layers of the 1st and 2nd SRS resource set respectively for CB or NCB.

For SRS resource set indicator codepoint 00 and 01 that indicate sTRP transmission with 1st SRS resource set and 2nd SRS resource set respectively, we support:

- concatenating the 1st TPMI and the 2nd TPMI to indicate precoding/rank for CB PUSCH or concatenating   1st SRI and 2nd SRI fields to indicate precoding/rank for NCB PUSCH for sTRP (i.e., using both 1st and 2nd field for sTRP transmission)

- Zero padding is used to align the DCI size or the last few bits in the 2nd TPMI/SRI field not used by sTRP are reserved.

## Single-DCI based STxMP SFN PUSCH scheme

Similar to SDM scheme single-DCI based STxMP transmission, it was agreed in RAN1#110bis-e to support SFN scheme of single-DCI based STxMP PUSCH. Moreover, it was also agreed to allow DCI-based dynamic switching between SFN scheme of single-DCI based STxMP PUSCH and sTRP transmission and to use the DCI field “SRS resource set indicator” to indicate the switching between SFN scheme and sTRP transmission. No consensus was reached to support DCI-based dynamic switching between SFN scheme and SDM scheme.

The difference between the SDM and SFN single-DCI based STxMP transmission schemes, is that the number of layers for SFN STxMP is indicated only by SRI/TPMI associated with one of the SRS resource sets. For CB-PUSCH, the second TPMI field only contains TPMIs corresponding to the indicated rank of the first TPMI field. For non-codebook PUSCH, the second SRI field only contains the SRI(s) combinations corresponding to the indicated rank of the first SRI field.

One interpretation of the SRS resource set indication 2-bit field for SFN scheme of single-DCI based STxMP PUSCH is shown in table 4.

Table 4: Possible interpretation of SRS resource set indicaton 2-bit field for SFN scheme of single-DCI based STxMP PUSCH

| Codepoint | SRS resource set(s) | SRI (for both CB and NCB)/TPMI (CB only) field(s) |
| --- | --- | --- |
| 00 | s-TRP mode with 1st SRS resource set | - 1st SRI/TPMI field (2nd field is unused) or - Both 1st and 2nd SRI/TPMI fields are used for indication of one SRI/TPMI |
| 01 | s-TRP mode with 2nd SRS resource set | - 1st SRI/TPMI field (2nd field is unused) or - Both 1st and 2nd SRI/TPMI fields are used for indication of one SRI/TPMI |
| 10 | STxMP SFN scheme | Both 1st and 2nd SRI/TPMI fields |
| 11 | Reserved |  |

The codepoints of “SRS resource set indicator” in DCI for dynamic switching between STxMP SFN and sTRP transmission are interpreted as follows:

  - The codepoints 00 and 01 indicate sTRP transmission where codepoint 00 indicates the first SRS resource set and codepoint 01 indicates the second SRS resource set.

  - The codepoints 10 indicate STxMP SFN transmission with the first and second SRS resource set.

  - The codepoint 11 is reserved.

We support using Rel-17 design for mTRP PUSCH TDM scheme to interpret the TPMI/SRI fields of the DCI scheduling STxMP SFN transmission.

For STxMP SFN transmission (codepoint 10), follow the same the design as Rel-17 mTRP PUSCH TDM scheme, where the 1st TPMI/SRI fields indicate the rank and precoding/SRI for the first SRS resource set and the 2nd TPMI/SRI fields indicate the precoding/SRI for the second SRS resource set.

Moreover, similar to SDM STxMP, we support the following:

For sTRP CB PUSCH, concatenating the 1st and 2nd TPMI fields indicates the precoding/rank and the 1st SRI field indicates the SRS resource. For NCB PUSCH, concatenating the 1st and 2nd SRI fields indicates the precoding/rank.

Moreover, the maximum number of layers in SFN scheme needs to be determined. In RAN1#112, the following was agreed, hence configuring a separate parameter for the maximal number of layers for STxMP SFN.

Agreement (RAN1#112)

On dynamic switching between STxMP SFN scheme and sTRP transmission:

- The legacy maxRank/Lmax is applied to sTRP transmission

- For configuration of SFN,

  - Alt2: Configure a separate parameter for the maximal number of layers for STxMP SFN

Therefore, we propose to have a maximum 2 layers per panel for STxMP PUSCH which is aligned with the WID as well.

SFN-based transmission scheme for STxMP PUSCH supports a maximum 2 layers per panel.

# Conclusions

Based on the discussion above, we propose the following:

- Study enhancements to Rel-17 group-based beam reporting and UE capability index reporting to inform gNB whether the UE is able to transmit simultaneously from two panels, when the panels may have same maximum number of SRS ports.

On the two SRS resource sets configured for single-DCI based STxMP SDM scheme:

- For CB/NCB PUSCH, the number of SRS resources in these two SRS resource sets is the same.

- For CB PUSCH, the two SRS resources indicated by the DCI for SDM scheme have same number of ports.

The codepoints of “SRS resource set indicator” in DCI for dynamic switching between STxMP SDM and sTRP transmission are interpreted as follows:

  - SRS resource set indicator codepoints 00 and 01 indicate sTRP transmission with 1st SRS resource set and 2nd SRS resource set respectively.

  - SRS resource set indicator codepoint 10 indicates SDM with 1st SRS resource set and 2nd SRS resource set. The 1st TPMI or SRI field and 2nd TPMI or SRI field indicates the precoding/rank/SRI for the 1st SRS resource set and 2nd SRS resource set for CB or NCB respectively.

  - The codepoint 11 of SRS resource set indicator is reserved/unused.

For SDM, the size of the 1st TPMI or SRI fields and the 2nd TPMI or SRI fields are determined by the maximal number of layers of the 1st and 2nd SRS resource set respectively for CB or NCB.

For SRS resource set indicator codepoint 00 and 01 that indicate sTRP transmission with 1st SRS resource set and 2nd SRS resource set respectively, we support:

- concatenating the 1st TPMI and the 2nd TPMI to indicate precoding/rank for CB PUSCH or concatenating   1st SRI and 2nd SRI fields to indicate precoding/rank for NCB PUSCH for sTRP (i.e., using both 1st and 2nd field for sTRP transmission)

- Zero padding is used to align the DCI size or the last few bits in the 2nd TPMI/SRI field not used by sTRP are reserved.

The codepoints of “SRS resource set indicator” in DCI for dynamic switching between STxMP SFN and sTRP transmission are interpreted as follows:

  - The codepoints 00 and 01 indicate sTRP transmission where codepoint 00 indicates the first SRS resource set and codepoint 01 indicates the second SRS resource set.

  - The codepoints 10 indicate STxMP SFN transmission with the first and second SRS resource set.

  - The codepoint 11 is reserved.

For STxMP SFN transmission (codepoint 10), follow the same the design as Rel-17 mTRP PUSCH TDM scheme, where the 1st TPMI/SRI fields indicate the rank and precoding/SRI for the first SRS resource set and the 2nd TPMI/SRI fields indicate the precoding/SRI for the second SRS resource set.

For sTRP CB PUSCH, concatenating the 1st and 2nd TPMI fields indicates the precoding/rank and the 1st SRI field indicates the SRS resource. For NCB PUSCH, concatenating the 1st and 2nd SRI fields indicates the precoding/rank.

SFN-based transmission scheme for STxMP PUSCH supports a maximum 2 layers per panel.

# References

- [1]		RP-213598, Revised WID: MIMO evolution for downlink and uplink, Samsung, RAN#94-e, Dec 2021

Older Agreements

During RAN1#111 meeting, the following agreements were made:

Agreement 1 (RAN1#111)

For the DMRS port indication for SDM scheme single-DCI based STxMP transmission, support Alt2:

- Alt2: the DMRS ports associated with two TPMI/SRI fields can be in same or different CDM groups.

Agreement 2 (RAN1#111)

Support CG PUSCH + CG PUSCH in multi-DCI based STxMP PUSCH+PUSCH transmission.

Agreement 3 (RAN1#111)

Support the SFN scheme for single-DCI based STxMP PUCCH transmission

Conclusion

There is no consensus on the support STxMP PUCCH+PUCCH in multi-DCI based mTRP system

Agreement 4 (RAN1#111)

- For multi-DCI based STxMP, to schedule a PUSCH for STxMP PUSCH+PUSCH transmission,

  - Alt1: The first SRS resource set is associated with coresetPoolIndex value 0 and the other SRS resource set is associated with coresetPoolIndex value 1

    - The PUSCH is associated with SRS resource set with the same value of coresetPoolIndex

    - FFS: Which is the first SRS resource set, e.g., the set with lower set ID.

- Regarding how to interpret the SRI/TPMI field in DCI:

  - For DG-PUSCH, the indicated SRI/TPMI field corresponds to the SRS resource set associated with same coresetPoolIndex value of the CORESET where scheduling DCI format 0_1 or 0_2 is received

  - For Type 2 CG-PUSCH, the indicated SRI/TPMI field corresponds to the SRS resource set associated with same coresetPoolIndex value of the CORESET where activation DCI is received.

- For Type 1 CG-PUSCH, one SRS_resource_set_index value is configured in RRC in ConfiguredGrantConfig and the srs-ResourceIndicator/precodingAndNumberOfLayers correspond to the SRS resource set

Agreement 5 (RAN1#111)

- Support DCI-based dynamic switching between SFN scheme of single-DCI based STxMP PUSCH and sTRP transmission

  - The DCI field “SRS resource set indicator” is used to indicate the switching between SFN scheme and sTRP transmission.

- (Conclusion) There is no consensus to support DCI-based dynamic switching between SFN scheme and SDM scheme

Agreement 6 (RAN1#111)

For the SFN scheme of single-DCI based STxMP PUSCH:

- Configure two SRS resource sets for CB or NCB.

  - FFS: Number of SRS resources of SRS resource set, and number of SRS ports of SRS resource

- The DCI indicates two SRI fields and TPMI fields for SFN transmission,

- On the indication of number of layers for CB and NCB PUSCH:

  - Alt1: Similar to rel-17 mTRP TDM scheme, the number of layers is indicated by the first SRI field (for NCB PUSCH) or the first TPMI field (for CB PUSCH)

Agreement 7 (RAN1#111)

For dynamic switching between SDM scheme of single-DCI based STxMP PUSCH and sTRP transmission:

- Use the 2-bit “SRS resource set indicator” DCI field to dynamically indicate the sTRP or SDM transmission.

- FFS: how to interpret each codepoint of “SRS resource set indicator”.

- For the maximal number of layers for sTRP transmission, down-select:

  - Option1: The maximal number of layers of sTRP transmission is configured by the maxRank (or Lmax) in current spec

  - Option2:  Configuring one additional maximal numbers of layers for sTRP transmission, in addition to maxRank in current spec.

- Down-select one from the following for maximal number of layers of SDM transmission:

  - Alt1: Configure one single maximal number of layers (separate from the maximal number(s) of layers for sTRP), that is applied to the first SRS resource set and the second SRS resource set, separately.

  - Alt1a: The maxRank (or Lmax) in current spec is also applied to the first SRS resource set and the second SRS resource set, separately.

  - Alt2: Configure separate maximal numbers of layers for the first SRS resource set and the second SRS resource set

  - Alt3: no dedicated configuration for SDM. The maximal number(s) of layers of sTRP and the UE capability reporting for SDM are used to determine the maximal number of layers of SDM transmission.

  - Alt4: The maximal number(s) of layers in above Option1/2 also applied to the first SRS resource set and the second SRS resource set, separately.

- FFS:  To ensure the same size of DCI for sTRP and SDM cases, how to use/interpret the TPMI/SRI field(s) and whether to do reserved bit or zero padding.

Agreement 8 (RAN1#111)

For SDM scheme single-DCI based STxMP transmission, when L1 and L2 layers are indicated/determined by two TPMI fields of CB PUSCH or two SRI fields of NCB PUSCH respectively:

- The first L1 indicated DMRS ports correspond to the L1 layers indicated by the first TPMI or SRI field

- The remaining L2 indicated DMRS ports correspond to the L2 layers indicated by the second TMPI or SRI field

- Support at least one of the following options for indication of layer combination {1+2}:

  - Option 1: new entry is added to DMRS table, e.g., {0, 2, 3}, {2, 0, 1}.

  - Option 2: use DCI field (e.g., SRS resource set indicator) to indicate that for layer combination {1+2}, the first two indicated DMRS ports correspond to the 2 layers indicated by the second TPMI or SRI field and the rest one indicated DMRS port correspond to the layer indicated by the first TPMI or SRI field.

  - Option 3: For layer combination of {1+2}, the DMRS port in the CDM group with only one port is mapped to the SRI/TPMI field indicating one layer, and the DMRS ports in the CDM group with 2 ports are mapped to the SRI/TPMI field indicating 2 layers

  - Other options are not precluded

Conclusion

There is no consensus to support layer combinations {1+3} and {3+1} in SDM scheme of single-DCI based STxMP PUSCH.

During RAN1#112 meeting, the following agreements were made:

Conclusion 1 (RAN1#112)

There is no consensus to support dynamic switching between STxMP SDM/SFN scheme and Rel-17 mTRP PUSCH TDM scheme

Conclusion 2 (RAN1#112)

There is no consensus to support 2 CWs in single-DCI based STxMP SDM scheme

Agreement 1 (RAN1#112)

Confirm the working assumption with the following updates:

Support the following scheme for STxMP PUSCH transmission in single-DCI based mTRP system in Rel-18:

- SDM scheme with one CW

- In RAN1#110bis-e, RAN1 will only consider SFN based transmission scheme to support in addition to the above. Decision to support or not to be made in RAN1#110bis-e.

Agreement 2 (RAN1#112)

When max 2 PTRS ports are configured for SDM scheme of single-DCI based STxMP PUSCH:

- Actual number of PTRS ports in SDM is 2 and 2-bit “PTRS-DMRS association” DCI field is used to indicate the PTRS-DMRS association for the DMRS ports associated with two TMPI/SRI fields.

  - The MSB indicates the association between PTRS port 0 and the DMRS port(s) associated with the first TPMI/SRI field.

  - The LSB indicates the association between PTRS port 1 and the DMRS port(s) associated with the second TPMI/SRI field.

- Regarding the “ptrs-PortIndex” configured to SRS resource for NCB PUSCH of SDM scheme, the UE ignores the configuration of “ptrs-PortIndex” per SRS resource.

FFS: Whether additional RRC configuration is needed for the max number of PTRS ports for SDM transmission

Working Assumption  1 (RAN1#112)

For dynamic switching between STxMP SDM scheme and sTRP transmission, support the following:

- For sTRP transmission: The maximal number of layers of sTRP transmission is configured by the maxRank (or Lmax) as in current spec (i.e., Option 1)

- For SDM scheme: configure one single maximal number of layers (separate from maxRank (or Lmax) for sTRP) that is applied to the first SRS resource set and the second SRS resource set, separately (i.e., Alt1)

- FFS: Whether/How to enable that the total number of used PUSCH antenna ports for the SDM and sTRP is the same.

  - Note: This corresponds to the case that digital ports are shared between the panels

  - Note: RAN1 supports both implementations that digital ports are shared or separate among panels

Agreement 3 (RAN1#112)

For multi-DCI based STxMP PUSCH+PUSCH, study enhancements of the UCI multiplexing rule to address the case that one PUCCH overlaps with two overlapped PUSCHs of STxMP PUSCH+PUSCH:

Agreement 4 (RAN1#112)

On dynamic switching between STxMP SFN scheme and sTRP transmission:

- The legacy maxRank/Lmax is applied to sTRP transmission

- For configuration of SFN,

  - Alt2: Configure a separate parameter for the maximal number of layers for STxMP SFN

Agreement 5 (RAN1#112)

To support indicating DMRS ports in different CDM groups for layer combination {1+2} in SDM

- Add new entry {0, 2, 3} to the DMRS table for the layer combination {1+2};

- This is optional UE capability for UE that supports sDCI based STxMP SDM

Agreement 6 (RAN1#112)

For single-DCI based STxMP SFN scheme,

- Alt2: When maxNrofPorts = 2 is configured for PTRS in SFN scheme, the actual number of PTRS port(s) in SFN is determined by the 1st TPMI field for CB or 1st SRI field for NCB

  - Each PTRS port is transmitted in SFN manner

Agreement 7 (RAN1#112)

Among the two SRS resource sets configured for multi-DCI based STxMP PUSCH+PUSCH, the SRS resource set with lower set ID is the first SRS resource set.