3GPP TSG RAN WG1 #112bis-e	R1-2302437

# e-Meeting, April 17th – April 26th, 2023

Source:	ZTE

Title:	Evaluation on AI CSI feedback enhancement

Agenda Item:	9.2.2.1

Document for:	Discussion and Decision

- Introduction

According to discussion in 3GPP RAN1#112 meeting [1], some progress has been made on evaluation methodology for CSI feedback enhancement. In this contribution, we provide our views on further details for evaluation methodology and share some evaluation results. In our companion contribution [2], potential specification impacts are discussed accordingly.

- Evaluation methodology on CSI compression in spatial and frequency domain

  - CSI training dataset and inference dataset separation

- Questions in RAN1#110 Question: For the evaluation of the AI/ML based CSI compression sub use cases, do you think there is a need to align the method to avoid correlation of samples between the training dataset and inference dataset?

In RAN1#110 meeting [3], the issue of CSI training dataset and inference dataset separation was raised for discussion, since inappropriate dataset separation for AI training/inference may cause an unconvincing conclusion. For example, if the training data and test data show a strong correlation, the model may show over-estimated performances.

Considering dataset construction for training and inference, there are at least two methods can be applied.

- Method 1: Training samples are generated for only one TTI from multiple drops, which brings abundant channel diversity across drops.

- Method 2: Training samples are generated for multiple TTIs from the same drop (or a few drops), which may introduce strong time correlation across different TTIs of the same drop.

Two different dataset construction types would have different performance gains, wherein AI model has real performance gains with Method 1, while over-estimated performance gains are obtained using Method 2. From our perspective, Method 1 can be adopted for better dataset separation, i.e. dataset should be generated in diverse drops rather than in diverse TTIs.

For CSI dataset construction, dataset should be generated in diverse drops rather than in diverse TTIs to ensure dataset diversity e.g., training samples are generated from various drops and only one TTI is used in a single drop.

- Remaining issues on the template for CSI compression

  - CSI feedback payload

In RAN1#112 meeting [1], an agreement and some working assumptions were achieved as following.

- In RAN1#112 meeting Agreement #1 The CSI feedback overhead is calculated as the weighted average of CSI payload per rank and the distribution of ranks reported by the UE. - For AI/ML based solutions: The above-mentioned “CSI feedback overhead” is calculated as max allowed bits at the given rank. - For legacy Type II CB: Option 2b is mandatorily reported by companies, while Option 2a can be optionally reported up to companies if partial NZC report is assumed for the legacy Type II CB - Option 2a: The above-mentioned “CSI feedback overhead” is calculated as each CSI reported payload with a given rank - Option 2b: The above-mentioned “CSI feedback overhead” is calculated as max allowed bits at the given rank Working Assumption #1 ... SGCS of benchmark, [layer 1]; CSI feedback payload X. CSI feedback payload Y. CSI feedback payload Z. SGCS of benchmark, [layer 2]; CSI feedback payload X. CSI feedback payload Y. CSI feedback payload Z. ... Gain for Mean UPT (for a specific CSI feedback overhead); [CSI feedback payload X*Max rank value]. [CSI feedback payload Y*Max rank value]. [CSI feedback payload Z*Max rank value]. Gain for 5% UPT; [CSI feedback payload X*Max rank value]. [CSI feedback payload Y*Max rank value]. [CSI feedback payload Z*Max rank value]. Working Assumption #2 For the initial template for AI/ML-based CSI compression without generalization/scalability verification achieved in the working assumption in the RAN1#111 meeting, X, Y and Z are determined as: - X is <=80bits - Y is 100bits-140bits - Z is >=230bits Working Assumption #3 X, Y and Z are applicable for per layer

Clarification on the terminologies of CSI feedback payload, CSI feedback overhead and CSI payload

In addition to the terminologies of CSI feedback overhead and CSI payload mentioned in Agreement #1, the terminology of CSI feedback payload appears in the Working Assumption #1. The three mentioned terminologies for overhead or payload calculation are ambiguous. For example, for a rank-common model, it’s hard to differentiate the CSI feedback payload per layer. Instead, it should be an averaged value per layer based on total CSI feedback overhead of a given max rank value. Hence, it is necessary to add a note for the template to clarify them, e.g., the CSI feedback payload X/Y/Z for intermediate KPI is an averaged value per layer based on the total CSI feedback payload of a given max rank value.

It is necessary to add a note for the template: the CSI feedback payload X/Y/Z for intermediate KPI is an averaged value per layer based on the total CSI feedback payload of a given max rank value.

- CSI feedback payload for Rank 3/4

According to Working Assumption #2&3, the CSI feedback payload size X, Y and Z are determined. However, the Z value is too large for Rank 3/4, since there are no parameter combinations of Rel-16 Type II compatible with the CSI feedback payload size Z. For legacy Rel-16 Type II CB, the maximum number of non-zero coefficients for Rank 2 is twice as that of Rank 1. So, the CSI feedback payload for Rank 2 is about twice as that of Rank 1. In addition, the maximum number of non-zero coefficients for Rank 3/4 in Rel-16 Type II keeps the same as Rank2. Like the legacy mechanism, we propose a solution for total CSI feedback payload under Rank 3/4 as follows:

- CSI payload with X is <=200 bits

- CSI payload with Y is 240bits-320bits

- CSI payload with Z is >= 500

Therefore, the CSI feedback payload per layer for a max rank value of 3/4 can be calculated as:

- X is <=200/vbits

- Y is 240/vbits-320/vbits

- Z is >=500/vbits

where v is a given max rank value.

The X, Y and Z values for CSI feedback payload per layer are determined as:

For a max rank value of 1/2

- X is <=80bits

- Y is 100bits-140bits

- Z is >=230bits

For a max rank value of 3/4

- X is <=200/vbits

- Y is 240/vbits-320/vbits

- Z is >=500/vbits

where v is a given max rank value.

Clarification on CSI feedback overhead in the template

In Working Assumption #1, CSI feedback overhead for UPT is not yet confirmed. To our understanding, the tentative calculation method, i.e., ‘CSI feedback payload X/Y/Z*Max rank value’, may have some problems. As cited in Agreement#1, the CSI feedback overhead is calculated as the weighted average of CSI payload per rank and the distribution of ranks reported by the UE. Therefore, the CSI feedback overhead for UPT will be smaller than the CSI feedback payload X/Y/Z*Max rank value due to various rank distributions. Our suggestion is that CSI feedback overhead for UPT should be determined according to the given values of X0, Y0, Z0 selected by each company for CSI feedback payload for intermediate results.

Therefore, it is necessary to add a note for the template to clarify the CSI feedback overhead for UPT, which is selected as follows:

- Value range#1: CSI feedback overhead <= CSI feedback payload X0*Max rank value;

- Value range#2: CSI feedback payload X0*Max rank value < CSI feedback overhead <= CSI feedback payload Y0*Max rank value;

- Value range#3: CSI feedback payload Y0*Max rank value < CSI feedback overhead <= CSI feedback payload Z0*Max rank value;

where the values of X0, Y0, Z0 are selected by each company for CSI feedback payload for intermediate results.

It is necessary to add a note for the template to clarify the CSI feedback overhead for UPT, which is selected as follows:

- Value range#1: CSI feedback overhead <= CSI feedback payload X0*Max rank value;

- Value range#2: CSI feedback payload X0*Max rank value < CSI feedback overhead <= CSI feedback payload Y0*Max rank value;

- Value range#3: CSI feedback payload Y0*Max rank value < CSI feedback overhead <= CSI feedback payload Z0*Max rank value;

- where the values of X0, Y0, Z0 are selected by each company for CSI feedback payload for intermediate results.

  - CSI overhead reduction and UPT Gain

After discussion in the RAN1#112 meeting, how to capture CSI overhead reduction and gain for UPT to the template is still pending. To our understanding, the following two methods can be considered,

- Method 1: For a given overhead for AI scheme, calculate the overhead reduction between the AI scheme and benchmark scheme in which the benchmark scheme has the same throughput value as the AI scheme. Then, calculate UPT gain based on the throughput values of AI scheme and benchmark scheme in the given overhead.

- Method 2: For a given overhead for benchmark scheme, calculate the overhead reduction between the benchmark scheme and AI scheme in which the AI scheme has the same throughput value as the benchmark scheme. Then, calculate UPT gain based on the throughput values of AI scheme and benchmark scheme in the given overhead.

Figure 1 shows the above two methods of CSI overhead reduction, where Method 1 is illustrated in Figure 1(a), and Method 2 is shown in Figure 1(b). Considering the benchmark scheme is used for a baseline, Method 2 with a given overhead for benchmark scheme should be adopted for calculating CSI overhead reduction and UPT gain.

In addition, the results of CSI overhead reduction should be captured in the template for different CSI feedback payload values X/Y/Z, respectively. This is due to that CSI overhead reductions are different for different CSI feedback payload values X/Y/Z.

Figure 1(a) Method 1 for CSI overhead reduction           Figure 1(b) Method 2 for CSI overhead reduction

and Gain for UPT                                     and Gain for UPT

The CSI overhead reduction and UPT gain are calculated based on:

Method 2: For a given overhead for benchmark scheme, calculate the overhead reduction between the benchmark scheme and AI scheme in which the AI scheme has the same throughput value as the benchmark scheme. Then, calculate UPT gain based on the throughput values of AI scheme and benchmark scheme in the given overhead.

The results of CSI overhead reduction should be captured in the template for different CSI feedback payload values X/Y/Z, respectively.

- Evaluation of the high-resolution quantization of the ground-truth CSI

In RAN1#110bis-e meeting [9], several high-resolution quantization methods were mentioned, e.g., Float32, Float16, R16 Type II-like method with new parameter combinations. Several companies showed that R16 Type II-like method with new parameter combinations can achieve attractive performances with low overhead [10]-[13]. This section shares our evaluation for R16 Type II-like method with new parameter combinations, wherein two parameter combinations mentioned in [10] are used, and float32 format is used as an ideal performance baseline. For convenience, the two parameter combinations are respectively labeled eType-II PC9 and eType-II PC10, and the details are as following:

- eType-II PC9: L = 8, pv=0.75, β=0.31, Reference amplitude = 8 bits, Differential amplitude = 4 bits, Phase = 6 bits

- eType-II PC10: L = 10, pv=0.9, β=0.31, Reference amplitude = 8 bits, Differential amplitude = 4 bits, Phase = 6 bits

To achieve a higher resolution, the two parameter combinations of eType-II PC9 and eType-II PC10 have larger L values and larger pv values than legacy eType-II PCs, to achieve a higher resolution. Table 4-1 shows the maximum overhead and SGCS for the ground-truth CSI quantization in eType-II PC8, eType-II PC9, eType-II PC10 and float32 for rank 2. The overheads of the eType-II PCs are compressed to only 1.2% to 3.8% compared to float32. Both two layer’s SGCS of eType II PC8 are below 0.9, both two layer’s SGCS of eType II PC10 above 0.9, and the first layer’s SGCS of eType II PC9 is above 0.9 with its second layer’s SGCS below 0.9.

Table 4-1: Maximum overhead and SGCS for the ground-truth CSI quantization methods for Rank = 2

|  | eType II PC8 | eType II PC9 | eType II PC10 | float 32 |
| --- | --- | --- | --- | --- |
| Maximum overhead (bits) | 635(1.2%) | 1358(2.5%) | 2011(3.8%) | 53248(100%) |
| SGCS | 0.8792/0.8037 | 0.9334 /0.8852 | 0.9561 /0.9249 | / |

Table 4-2 shows SGCS performance for models trained on data in different quantization methods, where the test data applies float32 format. Compared to float 32 format, the maximum loss of the SGCS for model trained on eType II PC8, eType II PC9 and eType II PC10 are about 0.05, 0.02 and 0.004 respectively

Tabel 4-2: SGCS for output of model trained on quantized dataset

| Feedback Overhead(bit) | model trained on eType II PC8 dataset | model trained on eType II PC9 dataset | model trained on eType II PC10 dataset | model trained on float32 dataset |
| --- | --- | --- | --- | --- |
| 120 bits | 0.6870/0.5538 | 0.7025/0.5842 | 0.7157//0.5866 | 0.7191/0.5886 |
| 644 bits | 0.8711/0.7951 | 0.9083/0.8458 | 0.9182/0.8603 | 0.9218/0.8661 |

The R16 Type II method with larger L values and larger pv values than legacy eType-II PCs has the possibility to achieve high resolution quantization with low overhead.

New parameters combinations for enhanced R16 Type II method should be supported/evaluated to achieve high resolution CSI with acceptable overhead for ground-truth CSI collection

- Evaluation of CQI determination

An agreement was achieved in last meeting [1], some CQI determination methods were mentioned as following.

- In RAN1#112 meeting Agreement In CSI compression using two-sided model use case, further study the following options for CQI determination in CSI report, if CQI in CSI report is configured. - Option 1: CQI is NOT calculated based on the output of CSI reconstruction part from the realistic channel estimation, including - Option 1a: CQI is calculated based on target CSI with realistic channel measurement - Option 1b: CQI is calculated based on target CSI with realistic channel measurement and potential adjustment - Option 1c: CQI is calculated based on legacy codebook - Option 2: CQI is calculated based on the output of CSI reconstruction part from the realistic channel estimation, including - Option 2a: CQI is calculated based on CSI reconstruction output, if CSI reconstruction model is available at the UE and UE can perform reconstruction model inference with potential adjustment - Note: CSI reconstruction part at the UE can be different comparing to the actual CSI reconstruction part used at the NW. - Option 2b: CQI is calculated using two stage approach, UE derive CQI using precoded CSI-RS transmitted with a reconstructed precoder. - Other options are not precluded - Note1: feasibility of different options should be evaluated - Note2: Gap analyses between the UE side CQI calculation results and the NW side results, as well as the impact on the scheduling performance should be evaluated - Note3: Complexity of CQI calculation needs to be evaluated, including the computing complexity and potential RS/signaling overhead

To facilitate discussion, Option 2a is further categorized into two sub options:

- Option 2a-1: CQI is calculated based on CSI reconstruction output, where CSI reconstruction part at the UE is the same as the actual CSI reconstruction part used at the NW.

- Option 2a-2: CQI is calculated based on CSI reconstruction output assumed at UE side, where CSI reconstruction part at the UE is different from the actual CSI reconstruction part used at the NW.

We have provided our analysis of pros and cons for different options in our companion contribution [2]. In this contribution, the performance of Option 1a, Option 2a-1 and Option 2a-2 for CQI determination are evaluated in SLS. For Option 2a-2, more details can be found in our companion contribution [2]. In Figure 2, the performance of the different CQI determination options compared to eType-II scheme and Ideal CSI scheme are shown. With regard to average sector throughput gain, Option 2a-1 and Option 2a-2 have similar performance. The average sector throughput gains of Option 2a-1 and Option 2a-2 are obviously higher than Option 1a and eType-II scheme. Option 1a has worse performance than eType-II scheme in low CSI feedback overhead, and better performance than eType-II scheme in high CSI feedback overhead, which is due to that the mismatch on PMI and CQI is hard to be adjusted by gNB side.

Figure 2 Average sector throughput gain

However, for Option 2a-1, it is an issue that CSI reconstruction model is not available at the UE, which can be an upper-bound to compare different options. For Option 2a-2, the working mode is similar as model monitoring as we discussed in [2] and section 6. Therefore, for further study on CQI determination, the feasibility of practical deployment should be considered for different options.

Option 2a-1 is not practical in reality, but it can provide a performance upper-bound to compare different options.

With regard to average sector throughput gain, Option 2a-1 and Option 2a-2 have similar performance. The average sector throughput gains of Option 2a-1 and Option 2a-2 are obviously higher than Option 1a and eType-II scheme.

Option 1a has worse performance than eType-II scheme in low CSI feedback overhead, and better performance than eType-II scheme in high CSI feedback overhead, which is due to that the mismatch on PMI and CQI is hard to be adjusted by gNB side.

Companies should disclose its CQI calculation assumptions when submitting the simulation results.

Further study and evaluate different CQI calculation options and draw necessary conclusions (including the performance and feasibility) in Rel-18.

- Evaluation of model performance monitoring

- In RAN1#112 meeting Agreement In CSI compression using two-sided model use case, further study the necessity, feasibility, and potential specification impact for intermediate KPIs based monitoring including at least: - NW-side monitoring based on the target CSI with realistic channel estimation associated to the CSI report, reported by the UE or obtained from the UE-side. - UE-side monitoring based on the output of the CSI reconstruction model, subject to the aligned format, associated to the CSI report, indicated by the NW or obtained from the network side. - Network may configure a threshold criterion to facilitate UE to perform model monitoring. - UE-side monitoring based on the output of the CSI reconstruction model at the UE-side - Note: CSI reconstruction model at the UE-side can be the same or different comparing to the actual CSI reconstruction model used at the NW-side. - Network may configure a threshold criterion to facilitate UE to perform model monitoring. - FFS: Other solutions, e.g., UE-side uses a model that directly outputs intermediate KPI. Network-side monitoring based on target CSI measured via SRS from the UE. Note: Monitoring approaches not based on intermediate KPI are not precluded Note: the study of intermediate KPIs based monitoring should take into account the monitoring reliability (accuracy), overhead, complexity, and latency.

Model performance monitoring was discussed for several meetings, several monitoring methods were mentioned [1][4][5], and the issue of evaluating these monitoring methods was raised in RAN1#111 meeting [6]. In evaluation, performance, complexity, overhead, latency of methods should be considered, where performance evaluation is considered to be the main challenge in all factors. Considering that companies have different views on how to evaluate model monitoring based on data distribution and eventual KPIs, intermediate KPI (e.g., SGCS) based monitoring can be a starting point of evaluation for model performance monitoring.

Intermediate KPI based monitoring can be a starting point for evaluation on model performance monitoring.

Case 1: UE-side monitoring based on the output of the CSI reconstruction model assumed at the UE side

Intermediate KPIs are calculated by UE based on the output of the CSI reconstruction model assumed at the UE side. As shown in Figure 3, UE has its own reference reconstruction part in CSI generation model which is not the same as the actual CSI reconstruction model used at the NW side. Hence, CSI generation model has two output modules. The first one is feedback part, which is used for model input of CSI reconstruction model. The second output (i.e., assumed output) is for model monitoring. As shown in the Figure 3, the assumed output data is trained to imitate reconstruction model output as much as possible via knowledge distillation technology [7]. By doing this, UE can monitor the loss2 to check the situation of loss1. If the monitoring metrics between input and assumed output cannot meet a target requirement, so as the monitoring metrics between input and actual output.

Figure 3 Intermediate KPIs calculated by UE based on the output of the CSI reconstruction model at the UE side

AI model performance is considered invalid when the SGCS between the reconstructed CSI and the target CSI is less than a threshold, otherwise valid. In this method, UE calculates the SGCS between the monitored CSI and the target CSI. AI model performance is invalid when the SGCS between the assumed output CSI and the target CSI is less than the threshold, otherwise valid. False detections are observed since UE only relies on the assumed output CSI to do model monitoring. As is shown from Table 6-1, for the SGCS thresholds of 0.7 and 0.8, the monitoring accuracy is higher than 96%.

Table 6-1: Performance of model performance monitoring at UE side

| Threshold | Monitoring Accuracy |
| --- | --- |
| 0.7 | 98.22% |
| 0.8 | 96.11% |

For model performance monitoring at UE side, assumed output CSI at UE can be used as CSI reconstruction part output to achieve a good monitoring accuracy.

Case 2: NW-side monitoring based on the target CSI with realistic channel estimation associated to the CSI report, reported by the UE or obtained from the UE-side

Intermediate KPIs are calculated by NW based on traditional CSI and CSI reconstruction model output. As shown in Figure 4, due to the fact that network cannot directly obtain the ground-truth label to calculate the monitoring metrics, UE should report ground-truth CSI to for network to calculate the monitoring metrics. In order to improve the performance of network-based model monitoring, a higher resolution ground-truth label needs to be reported by UE.

Figure 4 Intermediate KPIs calculated by NW based on traditional CSI and CSI reconstruction model output

AI model performance is considered invalid when the SGCS between the reconstructed CSI and the target CSI is less than a threshold, otherwise valid. With the model monitoring method, UE reports a reference CSI, and gNB calculates the SGCS between the reconstructed CSI and the reference CSI. gNB determines AI model performance is invalid when the SGCS between the reconstructed CSI and the reference CSI is less than the threshold, otherwise valid. Sometimes wrong determination will occur.

In this evaluation, the feedback overhead of monitored CSI is equal to that of eType-II PC8, the mean SGCS of monitored CSI is 0.9218, and the CSI of eType-II PC8 and the CSI of eType-II PC10 are used as the reference CSI respectively. Thresholds of 0.7 and 0.8 are used respectively. For convenience, the details of eType-II PC10 [8] are followed:

- eType-II PC10: L = 10, pv=0.9, β=0.31, Reference amplitude = 8 bits, Differential amplitude = 4 bits, Phase = 6 bits

Table 6-2 shows the evaluation result for Rank 1 in terms of monitoring accuracy. As be shown from Table 6-2, under a high feedback overhead of AI model, for the SGCS threshold of 0.8, with the reference CSI of eType-II PC8, the monitoring accuracy is lower than 90%, while for others the monitoring accuracy is higher than 95%.

Table 6-2: Performance of model performance monitoring methods with different reference CSIs

| Reference CSI: eType-II PC8 |  |
| --- | --- |
| Threshold | Monitoring Accuracy |
| 0.7 | 96.63% |
| 0.8 | 88.31% |
| eType-II PC10 |  |
| Threshold | Monitoring Accuracy |
| 0.7 | 99.32% |
| 0.8 | 97.61% |

For NW-side monitoring based on the target CSI with realistic channel estimation associated to the CSI report, reported by the UE or obtained from the UE-side, the monitoring accuracy is lower than 90% based on the reference CSI of eType-II PC8 under the case of SGCS threshold of 0.8, while for other cases, the monitoring accuracy is higher than 95%.

For NW-side monitoring based on the target CSI with realistic channel estimation associated to the CSI report, reported by the UE or obtained from the UE-side, eType II CSI with new parameter combination can be used as ground-truth CSI at NW to achieve a good monitoring accuracy.

For model performance monitoring, further study and evaluate the following two cases:

Case 1: UE-side monitoring based on the output of the CSI reconstruction model assumed at the UE side

Case 2: NW-side monitoring based on the target CSI with realistic channel estimation associated to the CSI report, reported by the UE or obtained from the UE-side

Study necessary KPIs for model performance monitoring, e.g., threshold to determine the monitoring accuracy.

- Evaluation Results of CSI compression

We perform a preliminary simulation on spatial-frequency domain CSI compression sub-use case to evaluate the performance of AI models, and the Rel-16 eTypeII is also simulated as baseline. In SLS, we further compare the throughput between AI-based feedback and ideal CSI feedback to see how much performance margin that AI-based approach has in comparison with an upper bound, where ideal eigenvector feedback is used as ideal CSI feedback. For Rel-16 eTypeII, 8 parameter combinations are used, and wherein paramCombination 7 and paramCombination 8 are directly extended for Rank=3 and Rank=4. In order to fairly compare the SLS performance, AI-based approach has the same overhead as that of Rel-16 eTypeII, i.e., corresponding to the 8 parameter combinations. For SLS evaluation metric, two types (Option 1 and Option 2b as mentioned in Section 3.3) of overhead calculation are adopted, wherein Option 1 marked as Maximum overhead, and Option 2b is Weighted Avg. overhead. The detailed simulation assumptions are shown in Table 8-1. In addition, detailed AI model training parameters are listed in Table 8-2. The system-level channel data is generated from 3000 simulation drops and 210 UEs per simulation drop, which results in 630K samples in total. Then, the datasets are randomly divided into three parts which are training, validation, and testing datasets with 600K, 10K, and 20K samples respectively. In our evaluation, ideal sub-band eigenvectors are used as the input of AI models for training, validation and testing. The number of parameters for a model ranges from about 9M-11M, where the CSI generation part is about 4M-5M and the CSI reconstruction part is 5M-6M. In addition, the FLOPs of CSI generation part and CSI reconstruction part are approximately 25M-26M and 27M-29M, respectively.

Based on the excel templates, our evaluation results for CSI compression are collected in 4 excels in the attachment as following:

- Excel 1. 1-on-1 joint training

- Excel 2. Evaluation results for CSI compression with model generalization

- Excel 3. Evaluation results for CSI compression with model scalability

- Excel 5. separate training

  - Rank 1

In this section, we evaluate the intermediate KPIs, e.g. SGCS, between our AI approach and the Rel-16 eType II. In order to explore the eventual UPT performance of our AI model, the SLS is performed for the case of up to 4 layers MU scheduling under FTP3 traffic model with 70% RU. These results are shown in Excel 1. 1-on-1 joint training (1st sheet: FTP, Max rank = 1). According to the Excel 1, we have following observations:

For Rank 1, AI-based CSI reconstruction completely outperforms the Rel-16 eType II in term of SGCS. With the low feedback overhead, AI-based CSI recovery can obtain up to 6.43% SGCS gains; with the medium feedback overhead, AI-based CSI recovery can obtain up to 6.44% SGCS gains; with the high feedback overhead, AI-based CSI recovery can obtain up to 7% SGCS gains.

For Rank 1, AI-based CSI reconstruction achieves minor average UPT gain, where less than 1% average UPT gain is obtained over Rel-16 eTypeII with the same feedback overhead.

For Rank 1, for 5% tail UPT, AI-based CSI reconstruction can achieve up to 20.43% performance gain with the low feedback overhead, up to 10.13% performance gain with the medium feedback overhead, and up to 0.85% performance gain with the high feedback overhead.

For Rank 1, the margin between AI-based CSI recovery and ideal CSI feedback is quite small for average UPT, as AI-based CSI reconstruction and Rel-16 eTypeII can already work well in single layer scheduling.

For Rank 1, the margin between AI-based CSI recovery and ideal CSI feedback is 5.6% for 5% tail UPT.

  - Up to Rank 2

In this section, we evaluate the intermediate KPIs, e.g. SGCS, between our AI approach and the Rel-16 eType II for Rank 2. In order to explore the eventual UPT performance of our AI model, the SLS is performed for the case of up to 8 layers MU scheduling under FTP3 traffic model with 50% and 70% RU. These results are shown in Excel 1. 1-on-1 joint training (2nd sheet: FTP, Max rank = 2). According to the evaluation results, we have following observations:

For Rank 2, AI-based CSI reconstruction completely outperforms the Rel-16 eType II in term of SGCS. With the low feedback overhead, AI-based CSI recovery can obtain up to 4.67% SGCS gains in Layer 1 and 5.92% SGCS gains in Layer 2; with the medium feedback overhead, AI-based CSI recovery can obtain up to 4.04% SGCS gains in Layer 1 and 5.90% SGCS gains in Layer 2; with the high feedback overhead, AI-based CSI recovery can obtain up to 6.44% SGCS gains in Layer 1 and 9.95% SGCS gains in Layer 2.

For up to Rank 2, under the case of 50% RU, AI based CSI reconstruction has about up to 4.09% average UPT gain over the Rel-16 eType II with the low feedback overhead, up to 4.08% average UPT gain over the Rel-16 eType II with the medium feedback overhead, and up to 4.55% average UPT gain over the Rel-16 eType II with the high feedback overhead.

For up to Rank 2, under the case of 50% RU, for 5% tail UPT, AI based CSI reconstruction has about up to 9.72% performance gain over the Rel-16 eType II with the low feedback overhead, up to 20.00% performance gain over the Rel-16 eType II with the medium feedback overhead, and up to 14.37% performance gain over the Rel-16 eType II with the high feedback overhead.

With regard to up to Rank 2, for average UPT, the margin between AI-based CSI recovery and ideal CSI feedback is 5.44% under the case of 50% RU.

With regard to up to Rank 2, for 5% tail UPT, the margin between AI-based CSI recovery and ideal CSI feedback is 24.04% under the case of 50% RU.

For up to Rank 2, under the case of 50% RU, AI-based CSI reconstruction can achieve up to 48.60% overhead reduction with the low feedback overhead,up to 39.80% overhead reduction with the medium feedback overhead, and up to 40.22% overhead reduction with the high feedback overhead.

For up to Rank 2, under the case of 70% RU, AI based CSI reconstruction has about up to 5.30% average UPT gain over the Rel-16 eType II with the low feedback overhead, up to 4.92% average UPT gain over the Rel-16 eType II with the medium feedback overhead, and up to 6.94% average UPT gain over the Rel-16 eType II with the high feedback overhead.

For up to Rank 2, under the case of 70% RU, for 5% tail UPT, AI based CSI reconstruction has about up to 10.26% performance gain over the Rel-16 eType II with the low feedback overhead, up to 15.02% performance gain over the Rel-16 eType II with the medium feedback overhead, and up to 16.26% performance gain over the Rel-16 eType II with the high feedback overhead.

With regard to up to Rank 2, for average UPT, the margin between AI-based CSI recovery and ideal CSI feedback is 9.50% under the case of 70% RU.

With regard to up to Rank 2, for 5% tail UPT, the margin between AI-based CSI recovery and ideal CSI feedback is 28.75% under the case of 70% RU.

For up to Rank 2, under the case of 70% RU, AI-based CSI reconstruction can achieve up to 44.44% overhead reduction with the low feedback overhead,up to 38.12% overhead reduction with the medium feedback overhead, and up to 40.41% overhead reduction with the high feedback overhead.

The SGCS performance for AI/ML-based CSI compression presents positive correlation with the throughput performance for up to Rank 2.

AI/ML approaches show better average throughput performance with heavy traffic load for up to Rank 2.

  - Up to Rank 4

In this section, we evaluate the intermediate KPIs, e.g. SGCS, between our AI approach and the Rel-16 eType II for Rank 4. In order to explore the eventual UPT performance of our AI model, the SLS is performed for the case of up to 8 layers MU scheduling under FTP3 traffic model with 50% and 70% RU, and under full buffer traffic model. These results are shown in Excel 1. 1-on-1 joint training (4th sheet: FTP, Max rank = 4 and 8th sheet: Full buffer, Max rank = 4). According to the evaluation results, we have following observations:

For Rank 4, AI-based CSI reconstruction completely outperforms the Rel-16 eType II in term of SGCS. With the low feedback overhead, AI-based CSI recovery can obtain up to 3.64% SGCS gains in Layer 1,  3.86% SGCS gains in Layer 2, 20.56% SGCS gains in Layer 3, and 53.21% SGCS gains in Layer 4; with the medium feedback overhead, AI-based CSI recovery can obtain up to 3.56% SGCS gains in Layer 1, 3.55% SGCS gains in Layer 2, 11.24% SGCS gains in Layer 3, and 26.99% SGCS gains in Layer 4; with the high feedback overhead, AI-based CSI recovery can obtain up to 5.80% SGCS gains in Layer 1, 5.41% SGCS gains in Layer 2, 13.53% SGCS gains in Layer 3, and 26.46% SGCS gains in Layer 4.

For up to Rank 4, under the case of 50% RU, AI based CSI reconstruction has about up to 12.22% average UPT gain over the Rel-16 eType II with the low feedback overhead, up to 7.04% average UPT gain over the Rel-16 eType II with the medium feedback overhead, and up to 8.19% average UPT gain over the Rel-16 eType II with the high feedback overhead.

For up to Rank 4, under the case of 50% RU, for 5% tail UPT, AI based CSI reconstruction has about up to 11.98% performance gain over the Rel-16 eType II with the low feedback overhead, up to 6.38% performance gain over the Rel-16 eType II with the medium feedback overhead, and up to 8.12% performance gain over the Rel-16 eType II with the high feedback overhead.

With regard to up to Rank 4, for average UPT, the margin between AI-based CSI recovery and ideal CSI feedback is 25.06% under the case of 50% RU.

With regard to up to Rank 4, for 5% tail UPT, the margin between AI-based CSI recovery and ideal CSI feedback is 37.72% under the case of 50% RU.

For up to Rank 4, under the case of 50% RU, AI-based CSI reconstruction can achieve up to 60.76% overhead reduction with the low feedback overhead, up to 34.31% overhead reduction with the medium feedback overhead, and up to 25.40% overhead reduction with the high feedback overhead.

For up to Rank 4, under the case of 70% RU, AI based CSI reconstruction has about up to 14.89% average UPT gain over the Rel-16 eType II with the low feedback overhead, up to 6.64% average UPT gain over the Rel-16 eType II with the medium feedback overhead, and up to 6.88% average UPT gain over the Rel-16 eType II with the high feedback overhead.

For up to Rank 4, under the case of 70% RU, for 5% tail UPT, AI based CSI reconstruction has about up to 20.72% performance gain over the Rel-16 eType II with the low feedback overhead, up to 12.50% performance gain over the Rel-16 eType II with the medium feedback overhead, and up to 14.20% performance gain over the Rel-16 eType II with the high feedback overhead.

With regard to up to Rank 4, for average UPT, the margin between AI-based CSI recovery and ideal CSI feedback is 25.09% under the case of 70% RU.

With regard to up to Rank 4, for 5% tail UPT, the margin between AI-based CSI recovery and ideal CSI feedback is 35.58% under the case of 70% RU.

For up to Rank 4, under the case of 70% RU, AI-based CSI reconstruction can achieve up to 64.38% overhead reduction with the low feedback overhead,up to 34.20% overhead reduction with the medium feedback overhead, and up to 24.81% overhead reduction with the high feedback overhead.

For up to Rank 4, under full buffer traffic, AI based CSI reconstruction has about up to 10.67% average UPT gain over the Rel-16 eType II with the low feedback overhead, up to 7.68% average UPT gain over the Rel-16 eType II with the medium feedback overhead, and up to 9.98% average UPT gain over the Rel-16 eType II with the high feedback overhead.

For up to Rank 4, under full buffer traffic, for 5% tail UPT, AI based CSI reconstruction has about up to 4.91% performance gain over the Rel-16 eType II with the low feedback overhead, up to 3.53% performance gain over the Rel-16 eType II with the medium feedback overhead, and up to 6.15% performance gain over the Rel-16 eType II with the high feedback overhead.

With regard to up to Rank 4, for average UPT, the margin between AI-based CSI recovery and ideal CSI feedback is 19.75% under full buffer traffic.

With regard to up to Rank 4, for 5% tail UPT, the margin between AI-based CSI recovery and ideal CSI feedback is 45.21% under full buffer traffic.

For up to Rank 4, under full buffer traffic, AI-based CSI reconstruction can achieve up to 66.67% overhead reduction with the low feedback overhead,up to 45.48% overhead reduction with the medium feedback overhead, and up to 39.40% overhead reduction with the high feedback overhead.

The SGCS performance for AI/ML-based CSI compression presents positive correlation with the throughput performance for up to Rank 4.

AI/ML approaches show better average throughput performance with heavy traffic load for up to Rank 4.

The case of rank>1 should be prioritized in future study.

  - Model generalization

In this section, we provide some results for the evaluation of model generalization capability.

Various deployment scenarios:

These results are shown in Excel 2. Evaluation results for CSI compression with model generalization (2nd sheet: Deployment scenarios, rank=2,4). According to the evaluation results, we have following observations:

Case 3(AI model trained in mixed deployment scenarios) shows good generalization performance for various scenarios included in the mixed deployment scenarios.

The AI model trained in UMa/UMi scenario shows good generalization performance to InH dataset, while the AI model trained in InH scenario shows generalization degradation to UMa/UMi scenario.

The AI model trained in the scenario of InH and fine-tuned with the dataset from a UMa/UMi scenario shows 1%-5% SGCS performance gain over the AI model trained only in InH scenario and tested in UMa/UMi scenario.

With the increased samples of fine-tuning dataset from testing scenario, the AI model shows better performance in testing scenario.

Various configurations on bandwidth:

We simulate AI generalization for different datasets with bandwidths of 10MHz (52RBs) and 20MHz (104RBs). According to agreements made in previous meetings, 4RBs per sub-band for 10MHz and 8RBs per sub-band for 20MHz are adopted, so the AI input sizes keep the same for both datasets. These results are shown in Excel 3. Evaluation results for CSI compression with model scalability (2nd sheet: Bandwidths, rank=2 or 4). According to the evaluation results, we have following observations:

AI/ML approaches can achieve good generalization performance for the case that the training dataset and testing dataset are generated with different bandwidth configurations but with the same model input size.

Various configurations on antenna port number:

We also simulate AI generalization for various antenna port numbers, where three cases of generalization are considered. These results are shown in Excel 3. Evaluation results for CSI compression with model scalability (4th sheet: Tx ports, rank=2 or 4). According to the evaluation results, we have following observations:

Case 3(AI model trained in mixed configurations of antenna port numbers) shows good generalization performance for various antenna port numbers.

AI model trained in the configuration of 32 antenna ports would suffer from performance degradation in16 antenna ports when input data is zero padding from 16 antenna ports to 32 antenna ports.

Model generalization across layers:

Model generalization across layers is shown in Excel 3. Evaluation results for CSI compression with model scalability (8th sheet: Rank-layer Num, Max rank = 4). According to the evaluation results, we have following observations:

The AI/ML model shows a good generalization capability across layers in the following cases:

Trained with data from all layers and tested for different layers

Trained with data from the first layer and tested for different layers

Trained with data from the first two layers and tested for different layers

  - Performance evaluation for Type 3 sequential training

Performance for Type 3 sequential training is evaluated in this section, where both NW-first training and UE-first training are considered. These results are shown in Excel 5. separate training. According to the evaluation results, we have following observations:

For both Type 3 training with NW-first and Type 3 training with UE-first, two cases have similar performance. Type 3 training with NW-first has nearly the same performance as Type 1 training, while Type 3 training with UE-first has a slightly lower performance than Type 1 training.

- Evaluation Results of CSI Prediction

Based on the templates, our evaluation results for CSI compression are collected in in 2 excels in the attachment as following:

- Excel 6. CSI prediction without generalization

- Excel 7. CSI prediction with generalization

  - CSI prediction without generalization

We perform a preliminary simulation on the sub-use case of CSI prediction to evaluate the performance of AI models. In addition, the nearest historical CSI and a non-AI approach (i.e., Wiener filtering) are adopted for performance comparison. These results are shown in Excel 6. CSI prediction without generalization. According to the evaluation results, we have following observations and proposals:

Both AI-based CSI prediction and Wiener filtering-based CSI prediction can completely outperform the nearest historical CSI. Moreover, Wiener filtering-based non-AI CSI prediction shows even better performance than the AI-based approach when enough historical CSIs are applied.

The prediction accuracy of AI-based approach and Wiener filtering-based approach drops seriously when the predicted time becomes longer due to the channel aging. However, AI-based CSI prediction can maintain the performance for a longer time than Wiener filtering-based CSI prediction.

Further evaluate various scenarios to justify that AI/ML based CSI prediction shows obvious advantages over non-AI/ML based CSI prediction, e.g., reduced length of measurement window, reduced decorrelation distance, high UE speed, low LOS probability.

After eTypeII quantization on the predicated CSI via post-processing, AI/ML based CSI prediction and Wiener filtering-based non-AI CSI prediction have similar SGCS performances, where both the methods completely outperform the CSI predication based on the nearest historical CSI.

The prediction accuracy of both AI-based approach and Wiener filtering-based approach improve with the increased number of historical CSIs as input and presents certain positive correlation with the number of input historical CSIs.

AI-based CSI prediction shows less or even no performance gain over Wiener filtering-based algorithm with the increased number of historical CSIs in model input.

Various lengths of observation window and prediction window should be evaluated to have a fair comparison between AI-based CSI prediction and non-AI based CSI prediction

The prediction accuracy of AI-based approach outperforms non-AI approaches when sub-band eigenvectors are adopted as model input.

The performance of eigenvector prediction for AI-based approach drops dramatically compared with the input of raw channel, and AI-based approach shows marginal performance gain over non-AI algorithms.

Further study the input and output types for the sub-use case of AI/ML based CSI prediction.

  - CSI prediction with generalization

We perform a preliminary simulation on the sub-use case of CSI prediction to evaluate the performance of AI models. In addition, the nearest historical CSI and a non-AI approach (i.e., Wiener filtering) are adopted for performance comparison. These results are shown in Excel 7. CSI prediction with generalization (3rd sheet: UE speeds). According to the evaluation results, we have following observations and proposals:

The model trained with a high UE speed can achieve a good generalization performance to a low UE speed. However, on the contrary, the model trained with a low UE speed can achieve a poor generalization performance to a high UE speed.

Case 3(AI model trained with mixed UE speeds) shows good generalization performance for various UE speeds.

- Conclusion

In this contribution, the evaluations on AI/ML for CSI feedback enhancement are discussed, and simulation results are provided. We have the following observations and proposals.

Proposal 1: For CSI dataset construction, dataset should be generated in diverse drops rather than in diverse TTIs to ensure dataset diversity e.g., training samples are generated from various drops and only one TTI is used in a single drop.

Proposal 2: It is necessary to add a note for the template: the CSI feedback payload X/Y/Z for intermediate KPI is an averaged value per layer based on the total CSI feedback payload of a given max rank value.

Proposal 3: The X, Y and Z values for CSI feedback payload per layer are determined as:

 For a max rank value of 1/2

- X is <=80bits

- Y is 100bits-140bits

- Z is >=230bits

 For a max rank value of 3/4

- X is <=200/vbits

- Y is 240/vbits-320/vbits

- Z is >=500/vbits

where v is a given max rank value.

Proposal 4: It is necessary to add a note for the template to clarify the CSI feedback overhead for UPT, which is selected as follows:

- Value range#1: CSI feedback overhead <= CSI feedback payload X0*Max rank value;

- Value range#2: CSI feedback payload X0*Max rank value < CSI feedback overhead <= CSI feedback payload Y0*Max rank value;

- Value range#3: CSI feedback payload Y0*Max rank value < CSI feedback overhead <= CSI feedback payload Z0*Max rank value;

where the values of X0, Y0, Z0 are selected by each company for CSI feedback payload for intermediate results.

Proposal 5: The CSI overhead reduction and UPT gain are calculated based on:

 Method 2: For a given overhead for benchmark scheme, calculate the overhead reduction between the benchmark scheme and AI scheme in which the AI scheme has the same throughput value as the benchmark scheme. Then, calculate UPT gain based on the throughput values of AI scheme and benchmark scheme in the given overhead.

Proposal 6: The results of CSI overhead reduction should be captured in the template for different CSI feedback payload values X/Y/Z, respectively.

Proposal 7: New parameters combinations for enhanced R16 Type II method should be supported/evaluated to achieve high resolution CSI with acceptable overhead for ground-truth CSI collection

Proposal 8: Companies should disclose its CQI calculation assumptions when submitting the simulation results.

Proposal 9: Further study and evaluate different CQI calculation options and draw necessary conclusions (including the performance and feasibility) in Rel-18.

Proposal 10: Intermediate KPI based monitoring can be a starting point for evaluation on model performance monitoring.

Proposal 11: For NW-side monitoring based on the target CSI with realistic channel estimation associated to the CSI report, reported by the UE or obtained from the UE-side, eType II CSI with new parameter combination can be used as ground-truth CSI at NW to achieve a good monitoring accuracy.

Proposal 12: For model performance monitoring, further study and evaluate the following two cases:

 Case 1: UE-side monitoring based on the output of the CSI reconstruction model assumed at the UE side

 Case 2: NW-side monitoring based on the target CSI with realistic channel estimation associated to the CSI report, reported by the UE or obtained from the UE-side

Proposal 13: Study necessary KPIs for model performance monitoring, e.g., threshold to determine the monitoring accuracy.

Proposal 14: The case of rank>1 should be prioritized in future study.

Proposal 15: Further evaluate various scenarios to justify that AI/ML based CSI prediction shows obvious advantages over non-AI/ML based CSI prediction, e.g., reduced length of measurement window, reduced decorrelation distance, high UE speed, low LOS probability.

Proposal 16: Various lengths of observation window and prediction window should be evaluated to have a fair comparison between AI-based CSI prediction and non-AI based CSI prediction

Proposal 17: Further study the input and output types for the sub-use case of AI/ML based CSI prediction.

Observation 1: The R16 Type II method with larger L values and larger pv values than legacy eType-II PCs has the possibility to achieve high resolution quantization with low overhead.

Observation 2: Option 2a-1 is not practical in reality, but it can provide a performance upper-bound to compare different options.

Observation 3: With regard to average sector throughput gain, Option 2a-1 and Option 2a-2 have similar performance. The average sector throughput gains of Option 2a-1 and Option 2a-2 are obviously higher than Option 1a and eType-II scheme.

Observation 4: Option 1a has worse performance than eType-II scheme in low CSI feedback overhead, and better performance than eType-II scheme in high CSI feedback overhead, which is due to that the mismatch on PMI and CQI is hard to be adjusted by gNB side.

Observation 5: For model performance monitoring at UE side, assumed output CSI at UE can be used as CSI reconstruction part output to achieve a good monitoring accuracy.

Observation 6: For NW-side monitoring based on the target CSI with realistic channel estimation associated to the CSI report, reported by the UE or obtained from the UE-side, the monitoring accuracy is lower than 90% based on  the reference CSI of eType-II PC8 under the case of SGCS threshold of 0.8, while for other cases, the monitoring accuracy is higher than 95%.

.Observation 7: For Rank 1, AI-based CSI reconstruction completely outperforms the Rel-16 eType II in term of SGCS. With the low feedback overhead, AI-based CSI recovery can obtain up to 6.43% SGCS gains; with the medium feedback overhead, AI-based CSI recovery can obtain up to 6.44% SGCS gains; with the high feedback overhead, AI-based CSI recovery can obtain up to 7% SGCS gains.

Observation 8: For Rank 1, AI-based CSI reconstruction achieves minor average UPT gain, where less than 1% average UPT gain is obtained over Rel-16 eTypeII with the same feedback overhead.

Observation 9: For Rank 1, for 5% tail UPT, AI-based CSI reconstruction can achieve up to 20.43% performance gain with the low feedback overhead, up to 10.13% performance gain with the medium feedback overhead, and up to 0.85% performance gain with the high feedback overhead.

Observation 10: For Rank 1, the margin between AI-based CSI recovery and ideal CSI feedback is quite small for average UPT, as AI-based CSI reconstruction and Rel-16 eTypeII can already work well in single layer scheduling.

Observation 11: For Rank 1, the margin between AI-based CSI recovery and ideal CSI feedback is 5.6% for 5% tail UPT.

Observation 12: For Rank 2, AI-based CSI reconstruction completely outperforms the Rel-16 eType II in term of SGCS. With the low feedback overhead, AI-based CSI recovery can obtain up to 4.67% SGCS gains in Layer 1 and 5.92% SGCS gains in Layer 2; with the medium feedback overhead, AI-based CSI recovery can obtain up to 4.04% SGCS gains in Layer 1 and 5.90% SGCS gains in Layer 2; with the high feedback overhead, AI-based CSI recovery can obtain up to 6.44% SGCS gains in Layer 1 and 9.95% SGCS gains in Layer 2.

Observation 13: For up to Rank 2, under the case of 50% RU, AI based CSI reconstruction has about up to 4.09% average UPT gain over the Rel-16 eType II with the low feedback overhead, up to 4.08% average UPT gain over the Rel-16 eType II with the medium feedback overhead, and up to 4.55% average UPT gain over the Rel-16 eType II with the high feedback overhead.

Observation 14: For up to Rank 2, under the case of 50% RU, for 5% tail UPT, AI based CSI reconstruction has about up to 9.72% performance gain over the Rel-16 eType II with the low feedback overhead, up to 20.00% performance gain over the Rel-16 eType II with the medium feedback overhead, and up to 14.37% performance gain over the Rel-16 eType II with the high feedback overhead.

Observation 15: With regard to up to Rank 2, for average UPT, the margin between AI-based CSI recovery and ideal CSI feedback is 5.44% under the case of 50% RU.

Observation 16: With regard to up to Rank 2, for 5% tail UPT, the margin between AI-based CSI recovery and ideal CSI feedback is 24.04% under the case of 50% RU.

Observation 17: For up to Rank 2, under the case of 50% RU, AI-based CSI reconstruction can achieve up to 48.60% overhead reduction with the low feedback overhead,up to 39.80% overhead reduction with the medium feedback overhead, and up to 40.22% overhead reduction with the high feedback overhead.

Observation 18: For up to Rank 2, under the case of 70% RU, AI based CSI reconstruction has about up to 5.30% average UPT gain over the Rel-16 eType II with the low feedback overhead, up to 4.92% average UPT gain over the Rel-16 eType II with the medium feedback overhead, and up to 6.94% average UPT gain over the Rel-16 eType II with the high feedback overhead.

Observation 19: For up to Rank 2, under the case of 70% RU, for 5% tail UPT, AI based CSI reconstruction has about up to 10.26% performance gain over the Rel-16 eType II with the low feedback overhead, up to 15.02% performance gain over the Rel-16 eType II with the medium feedback overhead, and up to 16.26% performance gain over the Rel-16 eType II with the high feedback overhead.

Observation 20: With regard to up to Rank 2, for average UPT, the margin between AI-based CSI recovery and ideal CSI feedback is 9.50% under the case of 70% RU.

Observation 21: With regard to up to Rank 2, for 5% tail UPT, the margin between AI-based CSI recovery and ideal CSI feedback is 28.75% under the case of 70% RU.

Observation 22: For up to Rank 2, under the case of 70% RU, AI-based CSI reconstruction can achieve up to 44.44% overhead reduction with the low feedback overhead,up to 38.12% overhead reduction with the medium feedback overhead, and up to 40.41% overhead reduction with the high feedback overhead.

Observation 23: The SGCS performance for AI/ML-based CSI compression presents positive correlation with the throughput performance for up to Rank 2.

Observation 24: AI/ML approaches show better average throughput performance with heavy traffic load for up to Rank 2.

Observation 25: For Rank 4, AI-based CSI reconstruction completely outperforms the Rel-16 eType II in term of SGCS. With the low feedback overhead, AI-based CSI recovery can obtain up to 3.64% SGCS gains in Layer 1,  3.86% SGCS gains in Layer 2, 20.56% SGCS gains in Layer 3, and 53.21% SGCS gains in Layer 4; with the medium feedback overhead, AI-based CSI recovery can obtain up to 3.56% SGCS gains in Layer 1, 3.55% SGCS gains in Layer 2, 11.24% SGCS gains in Layer 3, and 26.99% SGCS gains in Layer 4; with the high feedback overhead, AI-based CSI recovery can obtain up to 5.80% SGCS gains in Layer 1, 5.41% SGCS gains in Layer 2, 13.53% SGCS gains in Layer 3, and 26.46% SGCS gains in Layer 4.

Observation 26: For up to Rank 4, under the case of 50% RU, AI based CSI reconstruction has about up to 12.22% average UPT gain over the Rel-16 eType II with the low feedback overhead, up to 7.04% average UPT gain over the Rel-16 eType II with the medium feedback overhead, and up to 8.19% average UPT gain over the Rel-16 eType II with the high feedback overhead.

Observation 27: For up to Rank 4, under the case of 50% RU, for 5% tail UPT, AI based CSI reconstruction has about up to 11.98% performance gain over the Rel-16 eType II with the low feedback overhead, up to 6.38% performance gain over the Rel-16 eType II with the medium feedback overhead, and up to 8.12% performance gain over the Rel-16 eType II with the high feedback overhead.

Observation 28: With regard to up to Rank 4, for average UPT, the margin between AI-based CSI recovery and ideal CSI feedback is 25.06% under the case of 50% RU.

Observation 29: With regard to up to Rank 4, for 5% tail UPT, the margin between AI-based CSI recovery and ideal CSI feedback is 37.72% under the case of 50% RU.

Observation 30: For up to Rank 4, under the case of 50% RU, AI-based CSI reconstruction can achieve up to 60.76% overhead reduction with the low feedback overhead,up to 34.31% overhead reduction with the medium feedback overhead, and up to 25.40% overhead reduction with the high feedback overhead.

Observation 31: For up to Rank 4, under the case of 70% RU, AI based CSI reconstruction has about up to 14.89% average UPT gain over the Rel-16 eType II with the low feedback overhead, up to 6.64% average UPT gain over the Rel-16 eType II with the medium feedback overhead, and up to 6.88% average UPT gain over the Rel-16 eType II with the high feedback overhead.

Observation 32: For up to Rank 4, under the case of 70% RU, for 5% tail UPT, AI based CSI reconstruction has about up to 20.72% performance gain over the Rel-16 eType II with the low feedback overhead, up to 12.50% performance gain over the Rel-16 eType II with the medium feedback overhead, and up to 14.20% performance gain over the Rel-16 eType II with the high feedback overhead.

Observation 33: With regard to up to Rank 4, for average UPT, the margin between AI-based CSI recovery and ideal CSI feedback is 25.09% under the case of 70% RU.

Observation 34: With regard to up to Rank 4, for 5% tail UPT, the margin between AI-based CSI recovery and ideal CSI feedback is 35.58% under the case of 70% RU.

Observation 35: For up to Rank 4, under the case of 70% RU, AI-based CSI reconstruction can achieve up to 64.38% overhead reduction with the low feedback overhead,up to 34.20% overhead reduction with the medium feedback overhead, and up to 24.81% overhead reduction with the high feedback overhead.

Observation 36: For up to Rank 4, under full buffer traffic, AI based CSI reconstruction has about up to 10.67% average UPT gain over the Rel-16 eType II with the low feedback overhead, up to 7.68% average UPT gain over the Rel-16 eType II with the medium feedback overhead, and up to 9.98% average UPT gain over the Rel-16 eType II with the high feedback overhead.

Observation 37: For up to Rank 4, under full buffer traffic, for 5% tail UPT, AI based CSI reconstruction has about up to 4.91% performance gain over the Rel-16 eType II with the low feedback overhead, up to 3.53% performance gain over the Rel-16 eType II with the medium feedback overhead, and up to 6.15% performance gain over the Rel-16 eType II with the high feedback overhead.

Observation 38: With regard to up to Rank 4, for average UPT, the margin between AI-based CSI recovery and ideal CSI feedback is 19.75% under full buffer traffic.

Observation 39: With regard to up to Rank 4, for 5% tail UPT, the margin between AI-based CSI recovery and ideal CSI feedback is 45.21% under full buffer traffic.

Observation 40: For up to Rank 4, under full buffer traffic, AI-based CSI reconstruction can achieve up to 66.67% overhead reduction with the low feedback overhead,up to 45.48% overhead reduction with the medium feedback overhead, and up to 39.40% overhead reduction with the high feedback overhead.

Observation 41: The SGCS performance for AI/ML-based CSI compression presents positive correlation with the throughput performance for up to Rank 4.

Observation 42: AI/ML approaches show better average throughput performance with heavy traffic load for up to Rank 4.

Observation 43: Case 3(AI model trained in mixed deployment scenarios) shows good generalization performance for various scenarios included in the mixed deployment scenarios.

Observation 44: The AI model trained in UMa/UMi scenario shows good generalization performance to InH dataset, while the AI model trained in InH scenario shows generalization degradation to UMa/UMi scenario.

Observation 45: The AI model trained in the scenario of InH and fine-tuned with the dataset from a UMa/UMi scenario shows 1%-5% SGCS performance gain over the AI model trained only in InH scenario and tested in UMa/UMi scenario.

Observation 46: With the increased samples of fine-tuning dataset from testing scenario, the AI model shows better performance in testing scenario.

Observation 47: AI/ML approaches can achieve good generalization performance for the case that the training dataset and testing dataset are generated with different bandwidth configurations but with the same model input size.

Observation 48: Case 3(AI model trained in mixed configurations of antenna port numbers) shows good generalization performance for various antenna port numbers.

Observation 49: AI model trained in the configuration of 32 antenna ports would suffer from performance degradation in16 antenna ports when input data is zero padding from 16 antenna ports to 32 antenna ports.

Observation 50: The AI/ML model shows a good generalization capability across layers in the following cases:

 Trained with data from all layers and tested for different layers

 Trained with data from the first layer and tested for different layers

 Trained with data from the first two layers and tested for different layers

Observation 51: For both Type 3 training with NW-first and Type 3 training with UE-first, two cases have similar performance. Type 3 training with NW-first has nearly the same performance as Type 1 training, while Type 3 training with UE-first has a slightly lower performance than Type 1 training.

Observation 52: Both AI-based CSI prediction and Wiener filtering-based CSI prediction can completely outperform the nearest historical CSI. Moreover, Wiener filtering-based non-AI CSI prediction shows even better performance than the AI-based approach when enough historical CSIs are applied.

Observation 53: The prediction accuracy of AI-based approach and Wiener filtering-based approach drops seriously when the predicted time becomes longer due to the channel aging. However, AI-based CSI prediction can maintain the performance for a longer time than Wiener filtering-based CSI prediction.

Observation 54: After eTypeII quantization on the predicated CSI via post-processing, AI/ML based CSI prediction and Wiener filtering-based non-AI CSI prediction have similar SGCS performances, where both the methods completely outperform the CSI predication based on the nearest historical CSI.

Observation 55: The prediction accuracy of both AI-based approach and Wiener filtering-based approach improve with the increased number of historical CSIs as input and presents certain positive correlation with the number of input historical CSIs.

Observation 56: AI-based CSI prediction shows less or even no performance gain over Wiener filtering-based algorithm with the increased number of historical CSIs in model input.

Observation 57: The prediction accuracy of AI-based approach outperforms non-AI approaches when sub-band eigenvectors are adopted as model input.

Observation 58: The performance of eigenvector prediction for AI-based approach drops dramatically compared with the input of raw channel, and AI-based approach shows marginal performance gain over non-AI algorithms.

Observation 59: The model trained with a high UE speed can achieve a good generalization performance to a low UE speed. However, on the contrary, the model trained with a low UE speed can achieve a poor generalization performance to a high UE speed.

Observation 60: Case 3(AI model trained with mixed UE speeds) shows good generalization performance for various UE speeds.

- References

- Chairman's notes, RAN1 #112.

- R1-2302438, Discussion on other aspects for AI CSI feedback enhancement, ZTE.

- R1-2207840 Summary#3_9.2.2.1_CSI_Eva_v002, RAN1 #110, Moderator.

- R1-2210611, Summary #4 on other aspects of AI/ML for CSI enhancement, RAN1 #110bis-e, Moderator.

- R1-2212909, Summary #4 on other aspects of AI/ML for CSI enhancement, RAN1 #111,Moderator.

- R1-2212673 Summary#5 for CSI evaluation of [111-R18-AI/ML], Moderator.

- Knowledge distillation, https://en.wikipedia.org/wiki/Knowledge_distillation.

- R1-2300171, Evaluation on AI CSI feedback enhancement, ZTE Corporation.

- R1-2210753 Summary#7 of [110bis-e-R18-AIML-02]_CSI_Eva, Moderator.

- R1-2210885, Evaluation on AI/ML for CSI feedback enhancement, Huawei, HiSilicon.

- R1-2210954, Evaluations on AI-CSI, Ericsson.

- R1-2210998, Evaluation on AI/ML for CSI feedback enhancement, vivo.

- R1-2211073, Evaluation on AI/ML for CSI feedback enhancement, Fujitsu.

- Appendix A

Table 11-1 SLS assumptions for AI/ML based CSI feedback

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD, OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Dense Urban (Macro only) |
| Frequency Range |  | FR1 only, 2GHz |
| Inter-BS distance |  | 200m |
| Channel model |  | According to TR 38.901 |
| Antenna setup and port layouts at gNB |  | 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ |
| Antenna setup and port layouts at UE |  | 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for (rank 1-4) |
| BS Tx power |  | 41 dBm for 10MHz |
| BS antenna height |  | 25m |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Modulation |  | Up to 256QAM |
| Coding on PDSCH |  | LDPC Max code-block size=8448bit |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | 15kHz for 2GHz |
| Simulation bandwidth |  | 10MHz |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | SU/MU-MIMO with rank adaptation. |
| MIMO layers |  | Maximum MU 4 layers for max rank=1 Maximum MU 8 layers for max rank=2 |
| CSI feedback |  | Feedback assumption at least for baseline scheme CSI feedback periodicity (full CSI feedback) : 5 ms, Scheduling delay (from CSI feedback to time to apply in scheduling) : 4 ms |
| Overhead |  | 2 OFDM symbols for PDCCH，type 2 for DMRS(24 REs/PRB/slot) CSI-RS overhead(32 REs/PRB/5 slot) |
| Traffic model |  | FTP 3 |
| Traffic load (Resource utilization) |  | RU 50% and 70% |
| UE distribution |  | 80% indoor (3km/h), 20% outdoor (30km/h) |
| UE receiver |  | MMSE-IRC |
| Feedback assumption |  | Realistic |
| Channel estimation |  | Realistic |
| Evaluation Metric |  | Throughput and CSI feedback overhead |
| Baseline for performance evaluation |  | Rel-16 TypeII Codebook |

Table 11-2  Training parameters of AI/ML model for CSI compression

| Parameter | Value |
| --- | --- |
| Backbone | Transformer |
| Parameter type | Real value |
| Input CSI type | Eigenvectors of the ideal channel matrix estimated by UE |
| Output CSI type | Recovered eigenvectors by AI/ML model in gNB |
| Data-processing | Normalization |
| Quantization | Vector quantization |
| CSI generation part: Number of parameters/M | 4-5 |
| CSI generation part: FLOPs/M | 25-26 |
| CSI reconstruction part: Number of parameters/M | 5-6 |
| CSI reconstruction part: FLOPs/M | 27-29 |
| Training dataset | 600K |
| Validation dataset | 10K |
| Testing dataset | 20K |
| Batch size | 400 |
| Optimizer | Adam |
| Loss function | MSE |

Table 11-3  Training parameters of AI/ML model for CSI prediction based on raw channels

| Parameter | Value |
| --- | --- |
| Backbone | ResNet |
| Parameter type | Real value |
| Input CSI type | Historical channel matrices measured by UE |
| Output CSI type | Predicted channel matrix by AI/ML model in UE |
| Model input size | () |
| Model output size | () |
| Training dataset | 80K |
| Validation dataset | 2K |
| Testing dataset | 2K |
| Batch size | 200 |
| Optimizer | Adam |
| Loss function | MSE |

Table 11-4  Training parameters of AI/ML model for CSI prediction based on eigenvectors

| Parameter | Value |
| --- | --- |
| Backbone | ResNet |
| Parameter type | Real value |
| Input CSI type | Sub-band eigenvectors |
| Output CSI type | Predicted eigenvectors by AI/ML model in UE |
| Model input size | () |
| Model output size | () |
| Training dataset | 80K |
| Validation dataset | 2K |
| Testing dataset | 2K |
| Batch size | 200 |
| Optimizer | Adam |
| Loss function | MSE |